#

#	20171117

#	One day one of Inter cards became not working.

#

#	Block device /dev/nvme0 was absent.

#	After cycle power of and on via iDRAC, this block device became visible.

#

#	This issue was resolved by Mangasev Alan.

#	Was opened case at Dell technical support website.

#	How it looks like.

[root@ds-gp-prd-sdw35 ~]# less /var/log/messages

...

Nov 17 10:12:26 sdw35 kernel: nvme 0000:04:00.0: Aborting I/O 0 QID 5

Nov 17 10:12:26 sdw35 kernel: nvme 0000:04:00.0: Aborting I/O 1 QID 5

Nov 17 10:12:26 sdw35 kernel: nvme 0000:04:00.0: Aborting I/O 0 QID 11

Nov 17 10:12:26 sdw35 kernel: nvme 0000:04:00.0: Aborting I/O 0 QID 20

Nov 17 10:12:28 sdw35 mfsmount[5358]: write file error, inode: 2564155, index: 4 - Timeout after 74665 ms (Timeout)(try counter: 1)

Nov 17 10:12:37 sdw35 mfsmount[5358]: write file error, inode: 2564769, index: 0 - Timeout after 66651 ms (Timeout)(try counter: 1)

Nov 17 10:12:46 sdw35 mfsmount[5358]: write file error, inode: 2558587, index: 8 - Timeout after 60824 ms (Timeout)(try counter: 1)

Nov 17 10:13:26 sdw35 kernel: nvme 0000:04:00.0: I/O 4 QID 0 timeout, reset controller

Nov 17 10:13:47 sdw35 mfsmount[5358]: write file error, inode: 2558587, index: 9 - Timeout after 59300 ms (Timeout)(try counter: 1)

Nov 17 10:14:15 sdw35 kernel: INFO: task xfslogd/10:4417 blocked for more than 120 seconds.

Nov 17 10:14:15 sdw35 kernel:      Not tainted 2.6.32-696.el6.x86_64 #1

Nov 17 10:14:15 sdw35 kernel: "echo 0 > /proc/sys/kernel/hung_task_timeout_secs" disables this message.

Nov 17 10:14:15 sdw35 kernel: xfslogd/10    D 000000000000000a     0  4417      2 0x00000000

Nov 17 10:14:15 sdw35 kernel: ffff88283737faa0 0000000000000046 ffff8828377e52c0 ffff88283aae4040

Nov 17 10:14:15 sdw35 kernel: ffff88283737fa20 ffffffffa006a981 ffff882836dc0000 ffff882836dc0038

Nov 17 10:14:15 sdw35 kernel: 0000000000000000 ffff8828377e52c0 ffff88283aae45f8 ffff88283737ffd8

Nov 17 10:14:15 sdw35 kernel: Call Trace:

Nov 17 10:14:15 sdw35 kernel: [<ffffffffa006a981>] ? nvme_submit_iod+0x81/0x240 [nvme]

Nov 17 10:14:15 sdw35 kernel: [<ffffffff8154b5b5>] schedule_timeout+0x215/0x2e0

Nov 17 10:14:15 sdw35 kernel: [<ffffffff8127d6d0>] ? generic_make_request+0x240/0x5a0

Nov 17 10:14:15 sdw35 kernel: [<ffffffff81130cd5>] ? mempool_alloc_slab+0x15/0x20

Nov 17 10:14:15 sdw35 kernel: [<ffffffff8154b213>] wait_for_common+0x123/0x180

Nov 17 10:14:15 sdw35 kernel: [<ffffffff8106c480>] ? default_wake_function+0x0/0x20

Nov 17 10:14:15 sdw35 kernel: [<ffffffff8127daa0>] ? submit_bio+0x70/0x120

...

Nov 17 10:14:15 sdw35 kernel: [<ffffffff8109f9d0>] ? worker_thread+0x0/0x2a0

Nov 17 10:14:15 sdw35 kernel: [<ffffffff810a63ae>] kthread+0x9e/0xc0

Nov 17 10:14:15 sdw35 kernel: [<ffffffff8100c28a>] child_rip+0xa/0x20

Nov 17 10:14:15 sdw35 kernel: [<ffffffff810a6310>] ? kthread+0x0/0xc0

Nov 17 10:14:15 sdw35 kernel: [<ffffffff8100c280>] ? child_rip+0x0/0x20

Nov 17 10:14:15 sdw35 kernel: INFO: task xfssyncd/nvme0n:4625 blocked for more than 120 seconds.

Nov 17 10:14:15 sdw35 kernel:      Not tainted 2.6.32-696.el6.x86_64 #1

Nov 17 10:14:15 sdw35 kernel: "echo 0 > /proc/sys/kernel/hung_task_timeout_secs" disables this message.

Nov 17 10:14:15 sdw35 kernel: xfssyncd/nvme D 0000000000000007     0  4625      2 0x00000000

Nov 17 10:14:15 sdw35 kernel: ffff88283b99bb50 0000000000000046 0000000000000000 ffffffffa0350bb8

Nov 17 10:14:15 sdw35 kernel: ffff88502c3251e0 ffff883e0be4f038 0000cb986c501b0d ffff883e0be4f000

Nov 17 10:14:15 sdw35 kernel: ffff88283b99bb30 000000010d508268 ffff88283bc2dad8 ffff88283b99bfd8

Nov 17 10:14:15 sdw35 kernel: Call Trace:

Nov 17 10:14:15 sdw35 kernel: [<ffffffffa0350bb8>] ? xfs_iextents_copy+0x68/0xc0 [xfs]

Nov 17 10:14:15 sdw35 kernel: [<ffffffff8154b5b5>] schedule_timeout+0x215/0x2e0

Nov 17 10:14:15 sdw35 kernel: [<ffffffff8154b213>] wait_for_common+0x123/0x180

Nov 17 10:14:15 sdw35 kernel: [<ffffffff8106c480>] ? default_wake_function+0x0/0x20

...

Nov 17 10:14:47 sdw35 kernel: nvme 0000:04:00.0: Device not ready; aborting reset

...

#	It helps us to identify this nvme card as Inter (not as HGST).

[root@ds-gp-prd-sdw35 ~]# lspci -v -s $(lspci | grep -ie ssd | awk '{print $1}')

04:00.0 Non-Volatile memory controller: Intel Corporation PCIe Data Center SSD (rev 01) (prog-if 02 [NVM Express])

Subsystem: Intel Corporation Device 00e4

Flags: bus master, fast devsel, latency 0, IRQ 42

Memory at 96100000 (64-bit, non-prefetchable) [size=16K]

Expansion ROM at 91d00000 [disabled] [size=64K]

Capabilities: [40] Power Management version 3

Capabilities: [50] MSI-X: Enable+ Count=32 Masked-

Capabilities: [60] Express Endpoint, MSI 00

Capabilities: [100] Advanced Error Reporting

Capabilities: [150] Virtual Channel

Capabilities: [180] Power Budgeting <?>

Capabilities: [190] Alternative Routing-ID Interpretation (ARI)

Capabilities: [270] Device Serial Number 55-cd-2e-41-4d-4e-e3-2e

Capabilities: [2a0] #19

Kernel driver in use: nvme

Kernel modules: nvme

[root@ds-gp-prd-sdw35 ~]#

#	Below I use special utiltiy to see state of this Intel NVMe card.

[root@ds-gp-prd-sdw35 ~]# isdct show -intelssd

- Intel SSD DC P3700 Series PHFT6424006P2P0EGN -

Bootloader : 8B1B0133

DevicePath : /dev/nvme0

DeviceStatus : Selected drive is in a disable logical state.

Firmware : 8DV1DP05

FirmwareUpdateAvailable : Please contact Intel Customer Support for further assistance at the following website: http://www.intel.com/go/ssdsupport.

Index : 0

ModelNumber : INTEL SSDPEDMD020T4D HHHL NVMe 2000GB

ProductFamily : Intel SSD DC P3700 Series

SerialNumber : PHFT6424006P2P0EGN

[root@ds-gp-prd-sdw35 ~]#

[root@ds-gp-prd-sdw35 ~]# isdct show -a -intelssd

- Intel SSD DC P3700 Series PHFT6424006P2P0EGN -

AggregationThreshold : Selected drive is in a disable logical state.

AggregationTime : Selected drive is in a disable logical state.

ArbitrationBurst : Selected drive is in a disable logical state.

Bootloader : 8B1B0133

CoalescingDisable : Selected drive is in a disable logical state.

DevicePath : /dev/nvme0

DeviceStatus : Selected drive is in a disable logical state.

DirectivesSupported : False

DynamicMMIOEnabled : The selected drive does not support this feature.

EnduranceAnalyzer : Selected drive is in a disable logical state.

ErrorString : *XASSERT_405574F8 00

Firmware : 8DV1DP05

FirmwareUpdateAvailable : Please contact Intel Customer Support for further assistance at the following website: http://www.intel.com/go/ssdsupport.

HighPriorityWeightArbitration : Selected drive is in a disable logical state.

IOCompletionQueuesRequested : Selected drive is in a disable logical state.

IOSubmissionQueuesRequested : Selected drive is in a disable logical state.

Index : 0

Intel : True

IntelGen3SATA : False

IntelNVMe : True

InterruptVector : Selected drive is in a disable logical state.

LatencyTrackingEnabled : Selected drive is in a disable logical state.

LowPriorityWeightArbitration : Selected drive is in a disable logical state.

MediumPriorityWeightArbitration : Selected drive is in a disable logical state.

ModelNumber : INTEL SSDPEDMD020T4D HHHL NVMe 2000GB

NVMeControllerID : 0

NVMeMajorVersion : 1

NVMeMinorVersion : 0

NVMePowerState : Selected drive is in a disable logical state.

NVMeTertiaryVersion : 0

NamespaceId : 4294967295

NamespaceManagementSupported : False

NativeMaxLBA : Selected drive is in a disable logical state.

NumErrorLogPageEntries : 63

OEM : Dell

PCILinkGenSpeed : 3

PCILinkWidth : 4

PLITestTimeInterval : The selected drive does not support this feature.

PhySpeed : The selected drive does not support this feature.

PhysicalSectorSize : The selected drive does not support this feature.

PowerGovernorAveragePower : Selected drive is in a disable logical state.

PowerGovernorBurstPower : Selected drive is in a disable logical state.

PowerGovernorMode : Selected drive is in a disable logical state.

Product : Fultondale

ProductFamily : Intel SSD DC P3700 Series

ProductProtocol : NVME

ReadErrorRecoveryTimer : Selected drive is in a disable logical state.

SMARTEnabled : True

SMARTHealthCriticalWarningsConfiguration : Selected drive is in a disable logical state.

SMBusAddress : Selected drive is in a disable logical state.

SectorSize : 512

SerialNumber : PHFT6424006P2P0EGN

TCGSupported : False

TempThreshold : Selected drive is in a disable logical state.

TemperatureLoggingInterval : The selected drive does not support this feature.

TimeLimitedErrorRecovery : Selected drive is in a disable logical state.

TrimSupported : True

VolatileWriteCacheEnabled : Selected drive is in a disable logical state.

WriteAtomicityDisableNormal : Selected drive is in a disable logical state.

WriteCacheReorderingStateEnabled : The selected drive does not support this feature.

WriteCacheState : The selected drive does not support this feature.

WriteErrorRecoveryTimer : Selected drive is in a disable logical state.

[root@ds-gp-prd-sdw35 ~]#

#	P3700

#	20170815

#	This is several ways to use utility for Intel PCIe SSD card.

#

[root@m1-2-gp-prd-sdw1 ~]# yum localinstall /tmp/isdct-3.0.6.400-17.x86_64.rpm

[root@m1-2-gp-prd-sdw1 ~]# isdct show -intelssd

- Intel SSD DC P3700 Series PHFT642400KL2P0EGN -

Bootloader : 8B1B0133

DevicePath : /dev/nvme0n1

DeviceStatus : Healthy

Firmware : 8DV1DP05

FirmwareUpdateAvailable : Please contact your Intel representative about firmware update for this drive.

Index : 0

ModelNumber : INTEL SSDPEDMD020T4D HHHL NVMe 2000GB

ProductFamily : Intel SSD DC P3700 Series

SerialNumber : PHFT642400KL2P0EGN

[root@m1-2-gp-prd-sdw1 ~]#

[root@m1-2-gp-prd-sdw1 ~]# isdct show -a -intelssd

- Intel SSD DC P3700 Series PHFT642400KL2P0EGN -

AggregationThreshold : 0

AggregationTime : 0

ArbitrationBurst : 0

Bootloader : 8B1B0133

CoalescingDisable : 1

DevicePath : /dev/nvme0n1

DeviceStatus : Healthy

DirectivesSupported : False

DynamicMMIOEnabled : The selected drive does not support this feature.

EndToEndDataProtCapabilities : 17

EnduranceAnalyzer : Media Workload Indicators have reset values. Run 60+ minute workload prior to running the endurance analyzer.

ErrorString :

Firmware : 8DV1DP05

FirmwareUpdateAvailable : Please contact your Intel representative about firmware update for this drive.

HighPriorityWeightArbitration : 0

IOCompletionQueuesRequested : 30

IOSubmissionQueuesRequested : 30

Index : 0

Intel : True

IntelGen3SATA : False

IntelNVMe : True

InterruptVector : 0

LBAFormat : 0

LatencyTrackingEnabled : False

LowPriorityWeightArbitration : 0

MaximumLBA : 3907029167

MediumPriorityWeightArbitration : 0

MetadataSetting : 0

ModelNumber : INTEL SSDPEDMD020T4D HHHL NVMe 2000GB

NVMeControllerID : 0

NVMeMajorVersion : 1

NVMeMinorVersion : 0

NVMePowerState : 0

NVMeTertiaryVersion : 0

NamespaceId : 1

NamespaceManagementSupported : False

NativeMaxLBA : 3907029167

NumErrorLogPageEntries : 63

NumLBAFormats : 6

OEM : Dell

PCILinkGenSpeed : 3

PCILinkWidth : 4

PLITestTimeInterval : The selected drive does not support this feature.

PhySpeed : The selected drive does not support this feature.

PhysicalSectorSize : The selected drive does not support this feature.

PhysicalSize : 2000398934016

PowerGovernorAveragePower : The desired feature is not supported.

PowerGovernorBurstPower : The desired feature is not supported.

PowerGovernorMode : 0 25W

Product : Fultondale

ProductFamily : Intel SSD DC P3700 Series

ProductProtocol : NVME

ProtectionInformation : 0

ProtectionInformationLocation : 0

ReadErrorRecoveryTimer : Device does not support this command set.

SMARTEnabled : True

SMARTHealthCriticalWarningsConfiguration : 0

SMBusAddress : 128

SectorSize : 512

SerialNumber : PHFT642400KL2P0EGN

TCGSupported : False

TempThreshold : 85

TemperatureLoggingInterval : The selected drive does not support this feature.

TimeLimitedErrorRecovery : 0

TrimSupported : True

VolatileWriteCacheEnabled : False

WriteAtomicityDisableNormal : 0

WriteCacheReorderingStateEnabled : The selected drive does not support this feature.

WriteCacheState : The selected drive does not support this feature.

WriteErrorRecoveryTimer : Device does not support this command set.

[root@m1-2-gp-prd-sdw1 ~]#

#

#	20170815

#	We can use for nagios checks, for Intel PCIe SSD cards, same script as we use for GHST PCIe SSD cards.

[root@m1-2-gp-prd-sdw1 ~]# cat /etc/nrpe.d/check_nvme.cfg

command[check_nvme]=/usr/lib64/nagios/plugins/check_nvme.sh

[root@m1-2-gp-prd-sdw1 ~]# /usr/lib64/nagios/plugins/check_nvme.sh

OK - nvme0n1

[root@m1-2-gp-prd-sdw1 ~]#

From: Belogrud Sergey Vladimirovich

Sent: Tuesday, August 15, 2017 12:45 PM

To: Chernyh Aleksandr Vladimirovich <a.chernyh@tinkoff.ru>

Subject: Утилита для PCIe SSD карт Intel 3700

Добрый день.

https://downloadcenter.intel.com/product/80095/SSD-Software

Сейчас rpm-пакет лежит здесь.

[root@m1-2-gp-prd-mdw 20170814-1720]# pwd ; ls isdct-3.0.6.400-17.x86_64.rpm

/root/work/20170814-1720

isdct-3.0.6.400-17.x86_64.rpm

[root@m1-2-gp-prd-mdw 20170814-1720]#

[root@m1-2-gp-prd-sdw1 ~]# yum localinstall /tmp/isdct-3.0.6.400-17.x86_64.rpm

[root@m1-2-gp-prd-sdw1 ~]# isdct show -intelssd

- Intel SSD DC P3700 Series PHFT642400KL2P0EGN -

Bootloader : 8B1B0133

DevicePath : /dev/nvme0n1

DeviceStatus : Healthy

Firmware : 8DV1DP05

FirmwareUpdateAvailable : Please contact your Intel representative about firmware update for this drive.

Index : 0

ModelNumber : INTEL SSDPEDMD020T4D HHHL NVMe 2000GB

ProductFamily : Intel SSD DC P3700 Series

SerialNumber : PHFT642400KL2P0EGN

[root@m1-2-gp-prd-sdw1 ~]#

Имеющаяся утилита и скрипт проверки с PCIe SSD картами работает тоже.

[root@m1-2-gp-prd-sdw1 ~]# cat /etc/nrpe.d/check_nvme.cfg

command[check_nvme]=/usr/lib64/nagios/plugins/check_nvme.sh

[root@m1-2-gp-prd-sdw1 ~]# /usr/lib64/nagios/plugins/check_nvme.sh

OK - nvme0n1

[root@m1-2-gp-prd-sdw1 ~]#

С уважением,

Сергей Белогруд

Старший системный администратор

#

#	Here is a nagios script: /usr/lib64/nagios/plugins/check_nvme.sh

#

#!/bin/bash

#

# Simple Nagios check for nvme using nvme-cli

# Author: Sam McLeod https://smcleod.net

#

# Requirements:

# nvme-cli - git clone https://github.com/linux-nvme/nvme-cli

#

# Usage:

# ./check_nvme.sh

DISKS=$(lsblk -e 11,253 -dn -o NAME | grep nvme)

CRIT=false

MESSAGE=""

for DISK in $DISKS ; do

# Check for critical_warning

$(nvme smart-log /dev/$DISK | awk 'FNR == 2 && $3 != 0 {exit 1}')

if [ $? == 1 ]; then

CRIT=true

MESSAGE="$MESSAGE $DISK has critical warning "

fi

# Check media_errors

$(nvme smart-log /dev/$DISK | awk 'FNR == 15 && $3 != 0 {exit 1}')

if [ $? == 1 ]; then

CRIT=true

MESSAGE="$MESSAGE $DISK has media errors "

fi

# Check num_err_log_entries

$(nvme smart-log /dev/$DISK | awk 'FNR == 16 && $3 != 0 {exit 1}')

if [ $? == 1 ]; then

CRIT=true

MESSAGE="$MESSAGE $DISK has errors logged "

fi

done

if [ $CRIT = "true" ]; then

echo "CRITICAL: $MESSAGE"

exit 2

else

echo "OK - $(echo $DISKS | tr -d '\n')"

exit 0

fi

#	Alternative nagios scripts.

https://github.com/vorlon/check_nvme/blob/master/check_nvme.sh

https://communities.intel.com/thread/77666

#

#	20170815

#	To download drivers or other software.

#

https://downloadcenter.intel.com/product/80095/SSD-Software

https://downloadcenter.intel.com/download/26970/Intel-SSD-Data-Center-Tool?product=80095

#

#	Release Notes pdf document contains follow.

...

https://www.intel.com/content/www/us/en/products/memory-storage/solid-state-drives/data-center-ssds/dc-p3700-series.html

https://www.intel.ru/content/www/ru/ru/solid-state-drives/intel-ssd-dc-family-for-pcie.html

https://www.intel.com/content/www/us/en/products/memory-storage/solid-state-drives/data-center-ssds/dc-p3700-series/dc-p3700-2tb-aic-20nm.html

https://downloadcenter.intel.com/ru/download/23931/-Intel-SSD-Data-Center-Tool?product=79624

http://www.thg.ru/storage/obzor_intel_ssd_dc_p3700/

...