https://support.hpe.com/hpsc/swd/public/detail?swItemId=MTX-76bfa460792d4f7cbc0aac234e&hprpt_id=HPGL_ALERTS_2048949&jumpid=em_alerts_us-us_Sep19_xbu_all_all_1990228_2048949_SynergyServersSoftware_routine__&DIMID=EMID_5116BF80068C4CCEF3473502D691DADD/#tab3

This is a set of sample XML scripts used to manage the configuration of Integrated Lights-Out (iLO) management processors and to control servers in which iLO devices are in use. Use LOCFG.PL (available in this bundle) to run the XML.

Драйверы и программное обеспечение

HPE Lights-Out XML PERL Scripting Sample for Linux

Примечание.  Для использования некоторых программ необходимо наличие действующей гарантии, контракта на поддержку с Hewlett Packard Enterprise или оплаченная лицензия.

| Тип: | Программное обеспечение - Управление Lights-Out |  |
| --- | --- | --- |
| Версия: | 5.10.0(9 сен 2019) |  |
| Операционная система(ы): | Red Hat Enterprise Linux 5 Server (x86) Red Hat Enterprise Linux 5 Server (x86-64). Red Hat Enterprise Linux 6 Server (x86). Red Hat Enterprise Linux 6 Server (x86-64). Red Hat Enterprise Linux 7 Server. Red Hat Enterprise Linux 8 Server. SUSE Linux Enterprise Server 10 (AMD64/EM64T). SUSE Linux Enterprise Server 10 (x86). SUSE Linux Enterprise Server 11 (AMD64/EM64T). SUSE Linux Enterprise Server 11 (x86). SUSE Linux Enterprise Server 12. SUSE Linux Enterprise Server 15. VMware ESX/ESXi 4.0. VMware ESX/ESXi 4.1. VMware ESXi 5.0. |
| Имя файла: | linux-LOsamplescripts5.10.0.tgz (58 KB) |  |

To ensure the integrity of your download, HPE recommends verifying your results with this SHA-256 Checksum value:

| ea145b41d2d2a996bec0ac390c7bca81ec21b776a9dbff4f914502196b40f6cb | linux-LOsamplescripts5.10.0.tgz |
| --- | --- |

Reboot Requirement:

Reboot is not required after installation for updates to take effect and hardware stability to be maintained.

Installation:

Unzip/Untar linux-LOsamplescripts5.10.0.tgz to a subdirectory. This file contains the following scripts:

- Abort_Directory_test.xml - This is a sample XML script to abort ongoing directory test.

- Add_Federation_Group.xml - This is a sample XML script to add a Federation group membership and grant associated privileges

- Add_SSO_Rec.xml - Add an HPE SIM Single Sign-On (SSO) record to the end of the database.

- Add_User.xml - Add a user to the Lights-Out device user database.

- Administrator_reset_pw.xml- Reset the default administrator account password using HPONCFG.

- Cert_Request.xml - Request a Certificate Signing Request from Lights-Out device.

- Change_Password.xml - Change the password of an existing Light-Out device user.

- Clear_AHS_Data.xml - Clear the AHS data.

- Clear_EventLog.xml - Clear the Light-Out device Event Log.

- Clear_IML.xml - Clear the server Integrated Management Log (IML).

- Clear_Power_On_Time.xml - Clear the power-on time counter.

- Computer_Lock_Config.xml - Change the values for Computer Lock settings.

- Delete_Federation_Group.xml - This is a sample XML script to delete a Federation group membership

- Delete_SSH_Key.xml - This is a sample XML script that deletes existing SSH key.

- Delete_SSO_Rec.xml - Delete an HPE SIM Single Sign-On (SSO) record by index.

- Delete_User.xml - Delete a user from the Light-Out device user database.

- Eject_Virtual_Media.xml - Ejects the image of the specified Virtual Media device.

- ERS_AHS_Submit.xml - This is a sample XML script to initiate AHS data submission to IRS.

- ERS_DC_CompleteRegistration.xml - This is a sample XML script to complete the server registration.

- ERS_DC_RegisterDevice.xml - This is a sample XML script to connect ERS to Insight Online and perform the 1st step of server registration.

- ERS_DC_SetWebProxy.xml - This is a sample XML script to modify the Web Proxy information required by  ERS to communicate with Insight Online.

- ERS_Disable.xml - This is a sample XML script to disable iLO ERS functionality and un-register the server.

- ERS_Get_Settings.xml - This is a sample XML script to retrieve current ERS settings.

- ERS_IRS_Enable.xml - This is a sample XML script to connect ERS to Insight Remote Support server (IRS) and register the server.

- ERS_L2_Collection_Submit.xml - This is a sample XML script to initiate a L2 data collection submission to IRS.

- ERS_Test_Event_Submit.xml - This is a sample XML script to initiate a test service event submission to IRS.

- Factory_Defaults.xml - Sets the Lights-out device to factory default settings.

- Get_AHS_Status.xml - This is a sample XML script to find out if AHS is enabled or disabled.

- Get_All_Languages.xml - This is a sample XML script to read all languages that are supported on iLO.

- Get_All_Licenses.xml - This is a sample XML script to get the license information.

- Get_All_User_Info.xml - Return the configuration details for all users.

- Get_All_Users.xml - Return a list of all user accounts on Lights-Out device.

- Get_Asset_Tag.xml - This is a sample XML script to get the asset tag.

- Get_Boot_Mode.xml - This is a sample XML script to get the pending boot mode.

- Get_Current_Boot_Mode.xml - This is a sample XML script to get the current boot mode.

- Get_Directory.xml - Return the current directory configuration of an Lights-Out device.

- Get_Directory_Test_Results.xml - This is a sample XML script to get directory test results.

- Get_discovery_services.xml - This is a sample XML script to read location discovery Services data.

- Get_EmHealth.xml - Return the health information of the server.

- Get_Embedded_health.xml - Retrieves health information of the server.

- Get_Encrypt.xml - This is a sample XML script to return current encryption settings

- Get_Federation_Group.xml - This is a sample XML script to retrieve the privileges granted to a specified Federation

- Get_Federation_All_Groups.xml - This is a sample XML script to retrieve a list of all Federation group memberships

- Get_Federation_All_Groups_Info.xml - This is a sample XML script to retrieve a list of all Federation group memberships and granted privileges

- Get_Federation_Multicast_Options.xml - This is a sample XML script to retrieve the current Federation multicast options.

- Get_FIPS_Status.xml - Return the status of FIPs mode.

- Get_FW_Version.xml - Return the version and date/time of the Lights-Out device firmware.

- Get_Global.xml - Return global settings.

- Get_Host_APO.xml - Retrieve server automatic power on and power on delay settings.

- Get_Host_CTRO.xml - To get the critical temperature shutdown behavior of a server.

- Get_Host_Data.xml - Retrieve SMBIOS data from Server Information page.

- Get_Host_Power.xml - Return the current power state of the host server.

- Get_Host_Power_Saver.xml - Return the current Power Regulator setting.

- Get_Host_Pwr_Micro_Ver.xml - Power Micro Firmware revision information of the server.

- Get_Hotkey_Config.xml - Retrieve the Remote Console hotkeys.

- Get_iLO_Log.xml - Retrieve the records in the Lights-Out device Event Log.

- Get_IML.xml - Retrieve the records in the Integrated Management Log.

- Get_language.xml - This is a sample XML script to get the default language setting and to retrieve the list of installed languages.

- Get_LDAP_CA_Cert_Status.xml - Return LDAP CA certificate status.

- Get_Network.xml - Return network settings.

- Get_OA_Info.xml - Return enclosure data for c-Class Blade Infrastructure.

- Get_One_Time_Boot_Order.xml - This is a sample XML script to retrieve the current State of the One Time Boot.

- Get_Persistent_Boot_Order.xml - This is a sample XML script to retrieve the current State of the Boot Order.

- Get_Persmouse_Status.xml  - Get the persistent mouse and keyboard status.

- Get_Power_On_Time.xml - Return the number of minutes the server has been powered on and running.

- Get_Power_Readings.xml: Return the power readings from the server.

- Get_PowerCap.xml - Return Power Cap setting.

- Get_Product_name.xml -  Get the product name.

- Get_PS_Data.xml -  Return the power supply information of the server.

- Get_Pwreg_Alert_Threshold.xml - Return the current power alert threshold settings.

- Get_Rack_Settings.xml - This is a sample XML script to return enclosure data for c-Class blade infrastructure

- Get_Security_Msg.xml - This is a sample XML script to retrieve the current security text message set in the iLO Login Banner.

- Get_Server_Name.xml - Return the host server name.

- Get_Server_FQDN.xml - This is a sample XML script to get the host server FQDN.

- Get_SNMP_IM.xml - Return the current SNMP and Insight Manager security settings.

- Get_SSO_Settings.xml - Retrieve the HPE SIM Single Sign-On (SSO) settings.

- Get_Supported_Boot_Mode.xml - This is a sample XML script to get the supported boot mode(s).

- Get_TPM_Status.xml - This is a sample XML script to return the status of TPM

- Get_UID_Status.xml - Return the current state of the server's Unit ID (UID) on the host server.

- Get_User.xml - Return the configuration details for a single user.

- Get_VM_Status.xml - Return the status of the specified Virtual Media device.

- HD_Zone_Defaults.xml - Reset the Hard Drive Zoning to factory defaults for XL servers that support hard drive zoning.

- HD_Zone_ReadBackplaneInfo.xml - Read the Hard Drive Backplane Information from XL servers that support hard drive zoning.

- HD_Zone_ReadZoneTable.xml - Read the Hard Drive Zoning for XL servers that support hard drive zoning.

- HD_Zone_Write_Zone.xml - Write the Hard Drive Zoning Information to XL servers that support hard drive zoning.

- Hotkey_Config.xml - Change the values for Remote Console hotkeys.

- hponcfg_reformat.xsl - The style-sheet (.xsl) converts the iLO configuration captured using hponcfgcap.xml to a format that can be used to restore the configuration of iLO.

- hponcfgcap.xml - Capture the current iLO configuration such as Global Settings,Network Settings, Users Information and Directory Settings to a file using HPONCFG.

- ilo_perl_xml.pdf - Document PERL script usage.

- Import_Cert.xml - Import a signed certificate into a Lights-Out device.

- Import_LDAP_CA_Cert.xml - Import LDAP CA certificate into a Lights-Out device.

- Import_SSH_Key.xml - Imports SSH key into a Lights-Out device.

- Insert_Virtual_Media.xml - Inserts the Virtual Media image via specified URL.

- License.xml - Add the iLO Advanced Pack license key for a server.

- locfg.pl - Sample PERL script to send XML commands.

- Lock_Configuration.xml - Enable/disable the datacenter configuration lock.

- Mod_Directory.xml - Change the directory settings of an Lights-Out device.

- Mod_Encrypt_Settings - This is a sample XML script to configure the encryption Settings

- Mod_Federation_Group.xml - This is a sample XML script to modify a Federation group membership and associated privileges

- Mod_Global_Settings.xml - Change the Lights-Out device Global Settings.

- Mod_Kerberos_Config.xml - To configure kerberos configuration.

- Mod_Network_Settings.xml - Change the Lights-Out device Network Settings.

- Mod_Schemaless_Directory.xml - Change the schemaless directory settings of an iLO.

- Mod_SNMP_IM_Settings.xml - Change the Lights-Out device SNMP and Insight Manager Settings.

- Mod_SSO_Settings.xml - Configure the HPE SIM Single Sign-On (SSO) settings.

- Mod_User.xml  - Change the login id, display name, password and/or privileges for an existing user account on a Lights-Out device.

- Mod_VLAN.xml - Configure virtual LAN settings on a Light-Out device.

- Mod_VM_Port_Settings.xml - This is a sample XML script to configure the virtual media port functionality on iLO.

- Profile_Apply.xml - This is a sample XML script to set the Profile Descriptor.

- Profile_Apply_Get_Results.xml - This is a sample XML script to get the results from the Profile_Apply.

- Profile_Desc_Delete.xml - This is a sample XML script to delete the Profile Descriptor.

- Profile_Desc_Download.xml - This is a sample XML script to write a profile Description, download a specific blob and write the blob to the blobstore.

- Profile_Desc_List.xml -  This is a sample XML script to list all the Profile Descriptors and the data stored in the Profile Descriptor.

- RBSU_POST_IP.xml - Configure Lights-Out device to display the IP address at POST.

- Reset_RIB.xml - Restart the Lights-Out Management Processor.

- Reset_Server.xml - Immediately power off and back on the host server.

- Set_AHS_Status.xml - This is a sample XML script to enable or disable AHS logging.

- Set_Asset_tag.xml - This is a sample XML script to set the asset tag, asset tag limited to 32 characters.

- Set_Boot_Mode.xml - This is a sample XML script to set the pending boot mode.

- Set_Brownout.xml - This is a sample XML script to enable or disable brownout support.

- Set_Federation_Multicast_Options.xml - This is a sample XML script to set the Federation multicast options.

- Set_FIPS_Enable.xml - This is a sample XML script to enable FIPs mode.

- Set_Host_APO.xml - Change server automatic power on and power on delay settings.

- Set_Host_CTRO.xml - To set the critical temperature shutdown behavior of a server.

- Set_Host_Power.xml - Power on/off the host server.

- Set_Host_Power_Saver.xml - Change the Power Regulator setting.

- Set_Language.xml - This is a sample XML script to set the default language on iLO.

- Set_One_Time_Boot_Order.xml - This is a sample XML script to Set One Time Boot Order.

- Set_Persistent_Boot_Order.xml - This is a sample XML script to Set Persistent Boot Order.

- Set_Persmouse_Status.xml - Set the persistent mouse and keyboard.

- Set_PowerCap.xml - Change the Power Cap setting.

- Set_Pwreg_Alert_Threshold.xml - Change the power alert threshold settings.

- Set_Security_Msg.xml - This is a sample XML script to configure a security text message in the iLO Login Banner.

- Set_Server_Name.xml - Set the host server name.

- Set_Server_FQDN.xml - This is a sample XML script to set the host server FQDN.

- Set_Virtual_Power_BTN.xml - This is a sample XML script to simulate physical press of the server power button, cold boot and warm boot.

- Set_VM_Status.xml - This is a sample XML script to configure the virtual media boot options.

- Shared_Network_Port.xml - Configure Lights-Out device to pass network traffic on the shared host network port on supported platforms.

- Start_Directory_Test.xml - This is a sample XML script to start directory tests.

- UID_Control.xml - Turn on/off the Unit ID (UID) light on the host server.

- Update_Firmware.xml - Flash the Lights-Out device firmware to a new version.

- Update_Language.xml - Script to update the language pack.

- vspconfig.pl - Set the Virtual Serial Port Configuration for Linux.

- linux-LOsamplescripts5.10.0.txt - README file.

A valid iLO user account and password is required for each XML script when using LOCFG.PL. The user account must have appropriate lights-out privileges to process the request.

The locfg.pl script connects to the iLO management processor using a Secure Sockets Layer (SSL) connection where:

perl locfg.pl -s {servername|ipaddress}[:port] [-l logfilename] -f input_filename [-u username -p password] [-ilo3|-ilo4|-ilo5]

- -s servername is the DNS name of target server.

- -s ipaddress is the IP Address of the target server. If a port is not specified, the port will default to :443. DO NOT USE this switch if launching from System Insight Manager.

- -l logfilename is the name of the file to log all output to. A default file with the server name/ipaddress is created if this option is not specified. DO NOT USE this switch if launching from System Insight Manager.

- -f input_filename is the filename containing the RIB Commands

- -u username is the iLO user account name

- -p password  Command line user name and password override those which are in the script file.

Note: Use -u and -p with caution as command line options are visible on Linux.

- -ilo3|-ilo4|ilo5 - Specifies the type of device on the server.  Note: locfg.pl detects the device automatically if this option is not present.

#

#	20190923

#

[s.belogrud@TCS4357]# cp /drives/c/Users/s.belogrud/Downloads/linux-LOsamplescripts5.10.0.tgz /drives/c/share/

[support@centos work]$ mkdir 20190923-1345.ilo.samples.scripts

[support@centos work]$ cd 20190923-1345.ilo.samples.scripts

[support@centos 20190923-1345.ilo.samples.scripts]$ cp /mnt/share/linux-LOsamplescripts5.10.0.tgz .

[support@centos 20190923-1345.ilo.samples.scripts]$ ll

total 60

-rwxrwxr-x 1 support support 59894 Sep 23 13:46 linux-LOsamplescripts5.10.0.tgz

[support@centos 20190923-1345.ilo.samples.scripts]$ tar -xvzf linux-LOsamplescripts5.10.0.tgz

[support@centos 20190923-1345.ilo.samples.scripts]$ samples/locfg.pl -s 10.219.104.202 -i samples/Get_FW_Version.xml -u s.belogrud@tcsbank.ru -p $(pass common/windows-current)

Can't locate Socket6.pm in @INC (@INC contains: /usr/local/lib64/perl5 /usr/local/share/perl5 /usr/lib64/perl5/vendor_perl /usr/share/perl5/vendor_perl /usr/lib64/perl5 /usr/share/perl5 .) at samples/locfg.pl line 23.

BEGIN failed--compilation aborted at samples/locfg.pl line 23.

[support@centos 20190923-1345.ilo.samples.scripts]$

[root@centos ~]# yum install perl-Socket6

…

Resolving Dependencies

--> Running transaction check

---> Package perl-Socket6.x86_64 0:0.23-15.el7 will be installed

--> Finished Dependency Resolution

Dependencies Resolved

===========================================================================================================================================================================================================

Package                                           Arch                                        Version                                          Repository                                            Size

===========================================================================================================================================================================================================

Installing:

perl-Socket6                                      x86_64                                      0.23-15.el7                                      tcs-centos-base                                       27 k

Transaction Summary

===========================================================================================================================================================================================================

Install  1 Package

Total download size: 27 k

Installed size: 50 k

Is this ok [y/d/N]: y

Downloading packages:

perl-Socket6-0.23-15.el7.x86_64.rpm                                                                                                                                                 |  27 kB  00:00:00

Running transaction check

Running transaction test

Transaction test succeeded

Running transaction

Installing : perl-Socket6-0.23-15.el7.x86_64                                                                                                                                                         1/1

Verifying  : perl-Socket6-0.23-15.el7.x86_64                                                                                                                                                         1/1

Installed:

perl-Socket6.x86_64 0:0.23-15.el7

Complete!

[root@centos ~]#

[support@centos 20190923-1345.ilo.samples.scripts]$ samples/locfg.pl -s 10.219.104.202 -i samples/Get_FW_Version.xml -u s.belogrud@tcsbank.ru -p $(pass common/windows-current)

Usage: perl locfg.pl -s server -f inputfile [options]

perl locfg.pl -s ipV4Address -f inputfile [options]

perl locfg.pl -s ipV4Address:portNumber -f inputfile [options]

perl locfg.pl -s ipV6Address -f inputfile [options]

perl locfg.pl -s [ipV6Address] -f inputfile [options]

perl locfg.pl -s [ipV6Address]:portNumber -f inputfile [options]

perl locfg.pl -s DnsName:portnumber -f inputfile [options]

-l logfile         log file

-v                 enable verbose mode

-t                 substitute variables with values specified(ab=xy,c=z)

-i                 entering username and password interactively

-u username        username

-p password        password

-ilo3|-ilo4|-ilo5  target is iLO 3, iLO 4 or iLO 5

Note: Use -u and -p with caution as command line options are

visible on Linux. The '-i' option is for entering the

username and password interactively.

[support@centos 20190923-1345.ilo.samples.scripts]$

[support@centos 20190923-1345.ilo.samples.scripts]$ perl samples/locfg.pl -s 10.219.104.202 -f samples/Get_FW_Version.xml -u s.belogrud@tcsbank.ru -p $(pass common/windows-current)

<?xml version="1.0"?>

<RIBCL VERSION="2.23">

<RESPONSE

STATUS="0x0000"

MESSAGE='No error'

/>

</RIBCL>

<?xml version="1.0"?>

<RIBCL VERSION="2.23">

<RESPONSE

STATUS="0x0000"

MESSAGE='No error'

/>

</RIBCL>

<?xml version="1.0"?>

<RIBCL VERSION="2.23">

<RESPONSE

STATUS="0x0000"

MESSAGE='No error'

/>

</RIBCL>

<?xml version="1.0"?>

<RIBCL VERSION="2.23">

<RESPONSE

STATUS="0x0000"

MESSAGE='No error'

/>

<GET_FW_VERSION

FIRMWARE_VERSION = "1.45"

FIRMWARE_DATE = "Jun 27 2019"

MANAGEMENT_PROCESSOR = "iLO5"

LICENSE_TYPE = "iLO Advanced"

/>

</RIBCL>

<?xml version="1.0"?>

<RIBCL VERSION="2.23">

<RESPONSE

STATUS="0x0000"

MESSAGE='No error'

/>

</RIBCL>

<?xml version="1.0"?>

<RIBCL VERSION="2.23">

<RESPONSE

STATUS="0x0000"

MESSAGE='No error'

/>

</RIBCL>

...Script Succeeded...

[support@centos 20190923-1345.ilo.samples.scripts]$

[support@centos 20190923-1345.ilo.samples.scripts]$ perl samples/locfg.pl -s 10.219.104.202 -f samples/Get_Network.xml -u s.belogrud@tcsbank.ru -p $(pass common/windows-current)

<?xml version="1.0"?>

<RIBCL VERSION="2.23">

<RESPONSE

STATUS="0x0000"

MESSAGE='No error'

/>

</RIBCL>

<?xml version="1.0"?>

<RIBCL VERSION="2.23">

<RESPONSE

STATUS="0x0000"

MESSAGE='No error'

/>

</RIBCL>

<?xml version="1.0"?>

<RIBCL VERSION="2.23">

<RESPONSE

STATUS="0x0000"

MESSAGE='No error'

/>

</RIBCL>

<?xml version="1.0"?>

<RIBCL VERSION="2.23">

<RESPONSE

STATUS="0x0000"

MESSAGE='No error'

/>

<GET_NETWORK_SETTINGS>

<ENABLE_NIC VALUE="Y"/>

<SHARED_NETWORK_PORT VALUE="N"/>

<VLAN_ENABLED VALUE="N"/>

<VLAN_ID VALUE="0"/>

<SPEED_AUTOSELECT VALUE="Y"/>

<NIC_SPEED VALUE="Automatic"/>

<FULL_DUPLEX VALUE="Automatic"/>

<DHCP_ENABLE VALUE="N"/>

<DHCP_GATEWAY VALUE="Y"/>

<DHCP_DNS_SERVER VALUE="Y"/>

<DHCP_WINS_SERVER VALUE="Y"/>

<DHCP_STATIC_ROUTE VALUE="Y"/>

<DHCP_DOMAIN_NAME VALUE="Y"/>

<DHCP_SNTP_SETTINGS VALUE="N"/>

<REG_WINS_SERVER VALUE="N"/>

<REG_DDNS_SERVER VALUE="N"/>

<PING_GATEWAY VALUE="Y"/>

<MAC_ADDRESS VALUE="08:f1:ea:6c:69:b8"/>

<IP_ADDRESS VALUE="10.219.104.202"/>

<SUBNET_MASK VALUE="255.255.254.0"/>

<GATEWAY_IP_ADDRESS VALUE="10.219.105.254"/>

<DNS_NAME VALUE="CZJ92404W0"/>

<DOMAIN_NAME VALUE="mgmt.tcsbank.ru"/>

<PRIM_DNS_SERVER VALUE="10.219.50.4"/>

<SEC_DNS_SERVER VALUE="10.219.50.5"/>

<TER_DNS_SERVER VALUE="0.0.0.0"/>

<PRIM_WINS_SERVER VALUE="0.0.0.0"/>

<SEC_WINS_SERVER VALUE="0.0.0.0"/>

<SNTP_SERVER1 VALUE="ntp01.tcsbank.ru"/>

<SNTP_SERVER2 VALUE="ntp02.tcsbank.ru"/>

<TIMEZONE VALUE="Asia/Kuwait"/>

<STATIC_ROUTE_1 DEST="0.0.0.0"

MASK="0.0.0.0"

GATEWAY="0.0.0.0"/>

<STATIC_ROUTE_2 DEST="0.0.0.0"

MASK="0.0.0.0"

GATEWAY="0.0.0.0"/>

<STATIC_ROUTE_3 DEST="0.0.0.0"

MASK="0.0.0.0"

GATEWAY="0.0.0.0"/>

<IPV6_ADDRESS   VALUE="fe80::af1:eaff:fe6c:69b8"

PREFIXLEN="64"

ADDR_SOURCE="SLAAC"

ADDR_STATUS="ACTIVE"/>

<IPV6_STATIC_ROUTE_1

IPV6_DEST="::"

PREFIXLEN="0"

IPV6_GATEWAY="::"

ADDR_STATUS="INACTIVE"/>

<IPV6_STATIC_ROUTE_2

IPV6_DEST="::"

PREFIXLEN="0"

IPV6_GATEWAY="::"

ADDR_STATUS="INACTIVE"/>

<IPV6_STATIC_ROUTE_3

IPV6_DEST="::"

PREFIXLEN="0"

IPV6_GATEWAY="::"

ADDR_STATUS="INACTIVE"/>

<IPV6_PRIM_DNS_SERVER VALUE="::"/>

<IPV6_SEC_DNS_SERVER VALUE="::"/>

<IPV6_TER_DNS_SERVER VALUE="::"/>

<IPV6_DEFAULT_GATEWAY VALUE="::"/>

<IPV6_PREFERRED_PROTOCOL VALUE="N"/>

<IPV6_ADDR_AUTOCFG VALUE="N"/>

<IPV6_REG_DDNS_SERVER VALUE="N"/>

<DHCPV6_STATELESS_ENABLE VALUE="N"/>

<DHCPV6_STATEFUL_ENABLE VALUE="N"/>

<DHCPV6_RAPID_COMMIT VALUE="N"/>

<DHCPV6_DOMAIN_NAME VALUE="Y"/>

<DHCPV6_SNTP_SETTINGS VALUE="N"/>

<DHCPV6_DNS_SERVER VALUE="Y"/>

<ILO_NIC_AUTO_SELECT VALUE="DISABLED"/>

<ILO_NIC_AUTO_SNP_SCAN VALUE="0"/>

<ILO_NIC_AUTO_DELAY VALUE="90"/>

<ILO_NIC_FAIL_OVER VALUE="DISABLED"/>

<ILO_NIC_FAIL_OVER_DELAY VALUE="300"/>

<SNP_PORT VALUE="1"/>

</GET_NETWORK_SETTINGS>

</RIBCL>

<?xml version="1.0"?>

<RIBCL VERSION="2.23">

<RESPONSE

STATUS="0x0000"

MESSAGE='No error'

/>

</RIBCL>

<?xml version="1.0"?>

<RIBCL VERSION="2.23">

<RESPONSE

STATUS="0x0000"

MESSAGE='No error'

/>

</RIBCL>

<?xml version="1.0"?>

<RIBCL VERSION="2.23">

<RESPONSE

STATUS="0x0000"

MESSAGE='No error'

/>

</RIBCL>

...Script Succeeded...

[support@centos 20190923-1345.ilo.samples.scripts]$

[support@centos 20190923-1345.ilo.samples.scripts]$ cp samples/Mod_Network_Settings.xml 20190923-1435.Mod_Network_Setting.set_new_dns.xml

[support@centos 20190923-1345.ilo.samples.scripts]$ cat 20190923-1435.Mod_Network_Setting.set_new_dns.xml

<!--         RIBCL Sample Script for HPE Lights-Out Products         -->

<!--  Copyright 2003,2017 Hewlett Packard Enterprise Development LP  -->

<!-- Description:  This is a sample XML script to configure the      -->

<!--               network settings for Integrated Lights-Out(iLO).  -->

<!-- NOTE:  You will need to replace the USER_LOGIN and PASSWORD     -->

<!--        and other values inside the quotation marks with values  -->

<!--        that are appropriate for your environment.               -->

<!--        Double check all values before executing this script.    -->

<!--        Incorrect or mismatched network settings may cause you   -->

<!--        to lose the ability to connect to your Lights-Out device.-->

<!--        Use HPQLOCFG.EXE v1.00 or greater with this script.      -->

<!--        See "iLO Scripting and Command Line Guide" for more      -->

<!--        information on scripting and the syntax of the RIBCL     -->

<!--        XML.                                                     -->

<!--        Firmware support information for this script:            -->

<!--            iLO 5 - All versions                                 -->

<!--            iLO 4 - All versions.                                -->

<!--            iLO 3 - All versions.                                -->

<RIBCL VERSION="2.0">

<LOGIN USER_LOGIN="adminname" PASSWORD="password">

<RIB_INFO MODE="write">

<MOD_NETWORK_SETTINGS>

<PRIM_DNS_SERVER value="10.219.50.4"/>

<SEC_DNS_SERVER value="10.219.50.5"/>

</MOD_NETWORK_SETTINGS>

</RIB_INFO>

</LOGIN>

</RIBCL>

[support@centos 20190923-1345.ilo.samples.scripts]$

[support@centos 20190923-1345.ilo.samples.scripts]$ ll

total 76

-r-------- 1 support support  1678 Sep 23 14:38 20190923-1435.Mod_Network_Setting.set_new_dns.xml

-rwxrwxr-x 1 support support 59894 Sep 23 13:46 linux-LOsamplescripts5.10.0.tgz

drwxrwxr-x 2 support support  8192 Sep 23 14:05 samples

[support@centos 20190923-1345.ilo.samples.scripts]$ chmod 666 20190923-1435.Mod_Network_Setting.set_new_dns.xml

[support@centos 20190923-1345.ilo.samples.scripts]$ ll

total 76

-rw-rw-rw- 1 support support  1678 Sep 23 14:38 20190923-1435.Mod_Network_Setting.set_new_dns.xml

-rwxrwxr-x 1 support support 59894 Sep 23 13:46 linux-LOsamplescripts5.10.0.tgz

drwxrwxr-x 2 support support  8192 Sep 23 14:05 samples

[support@centos 20190923-1345.ilo.samples.scripts]$