#

#	Troubled firmware version.

#

root@ds-devops-prod-mon-2:~# ethtool -i eno5 | grep -ie "firmware-version"

firmware-version: 14.26.1040 (HP_2690110034)

root@ds-devops-prod-mon-2:~# ethtool -i eno6 | grep -ie "firmware-version"

firmware-version: 14.26.1040 (HP_2690110034)

root@ds-devops-prod-mon-2:~#

Исправил глюки с сетевой картой

Mellanox Technologies MT27710 Family [ConnectX-4 Lx]

на сервере m1-soc-ls1.

Сменил версию firmware на ней.

#

#	https://support.hpe.com/hpsc/swd/public/detail?swItemId=MTX_d0e63d0a446746b2bc8d096096#tab4

[s.belogrud@TCS4357]# scp /drives/c/Users/s.belogrud/Downloads/firmware-nic-mellanox-ethernet-only-1.0.10-1.1.x86_64.rpm root@m1-soc-ls1:/tmp/

firmware-nic-mellanox-ethernet-only-1.0.10-1.1.x86_64.rpm                                                                                                         100% 8042KB  15.1MB/s   00:00

[s.belogrud@TCS4357]#

[root@m1-soc-ls1 work]# mkdir 20200414-2043

[root@m1-soc-ls1 work]# cd 20200414-2043

[root@m1-soc-ls1 20200414-2043]# cp /tmp/firmware-nic-mellanox-ethernet-only-1.0.10-1.1.x86_64.rpm .

[root@m1-soc-ls1 20200414-2043]# rpm -ivh firmware-nic-mellanox-ethernet-only-1.0.10-1.1.x86_64.rpm

warning: firmware-nic-mellanox-ethernet-only-1.0.10-1.1.x86_64.rpm: Header V3 RSA/SHA256 Signature, key ID 26c2b797: NOKEY

Verifying...                          ################################# [100%]

Preparing...                          ################################# [100%]

Updating / installing...

1:firmware-nic-mellanox-ethernet-on################################# [100%]

[root@m1-soc-ls1 20200414-2043]#

[root@m1-soc-ls1 20200414-2043]# /usr/lib/x86_64-linux-gnu/firmware-nic-mellanox-ethernet-only-1.0.10-1.1/setup

##################################################################################################

HPE Mellanox InfiniBand Online Firmware Upgrade Utility for Linux

Copyright (c) 2011 Hewlett-Packard Enterprise Development Company, L.P.

##################################################################################################

SecureBoot is disabled.

List of Network Adapters detected on the Server.................

[0] 0000:12:00.0 Broadcom Inc. and subsidiaries

[1] 0000:5d:00.0 Mellanox Technologies

If PSID or FW_Version is not found for some interfaces, please check /tmp/datanjDv8b

Interface 0000:12:00.0 is not Mellanox one.

Mellanox card info for 0000:5d:00.0 = FW_VERSION 14.26.1040, BUS_INFO 0000:5d:00.0, PSID HP_2690110034

0000:5d:00.0

DEVICE INFO---------->15b3 1015 1590 d3

pciIdString-------------->15B3-1015-1590-00D3-HP_2690110034

Current Firmware Version is 14.26.1040 on 0000:5d:00.0

Repository has these firmware versions............[1] Image Version  14.24.9006

Repository doesn't have newer firmware version, Installed firmware version is the latest one..........Please Quit the updation.

Would you like to flash the firmware?

y/n/q (n):y

Current FW version on flash:  14.26.1040

New FW version:               14.24.9006

Note: The new FW version is older than the current FW version on flash.

Do you want to continue ? (y/n) [n] : y

Initializing image partition -   OK

Writing Boot image component -   OK

-I- To load new FW run mlxfwreset or reboot machine.

Firmware Flashed:       SUCCESS  for interface 0000:5d:00.0

Please Reboot node for new image to be loaded into silicon

[root@m1-soc-ls1 20200414-2043]#

#

#	And then - reboot the server.

#

[root@m1-soc-ls1 ~]# dmidecode -t system

# dmidecode 3.2

Getting SMBIOS data from sysfs.

SMBIOS 3.2.1 present.

# SMBIOS implementations newer than version 3.2.0 are not

# fully supported by this version of dmidecode.

Handle 0x0089, DMI type 1, 27 bytes

System Information

Manufacturer: HPE

Product Name: ProLiant DL360 Gen10

Version: Not Specified

Serial Number: CZJ00204R7

UUID: 37393150-3636-5a43-4a30-303230345237

Wake-up Type: Power Switch

SKU Number: P19766-B21

Family: ProLiant

Handle 0x00AA, DMI type 32, 11 bytes

System Boot Information

Status: No errors detected

[root@m1-soc-ls1 ~]#

[root@m1-soc-ls1 ~]# tail -n 5 /var/log/messages

Apr 14 17:52:38 m1-soc-ls1 kernel: bond0: An illegal loopback occurred on adapter (eno5)#012Check the configuration to verify that all adapters are connected to 802.3ad compliant switch ports

Apr 14 17:52:39 m1-soc-ls1 kernel: bond0: An illegal loopback occurred on adapter (eno5)#012Check the configuration to verify that all adapters are connected to 802.3ad compliant switch ports

Apr 14 17:52:40 m1-soc-ls1 kernel: bond0: An illegal loopback occurred on adapter (eno5)#012Check the configuration to verify that all adapters are connected to 802.3ad compliant switch ports

Apr 14 17:52:41 m1-soc-ls1 kernel: bond0: An illegal loopback occurred on adapter (eno5)#012Check the configuration to verify that all adapters are connected to 802.3ad compliant switch ports

Apr 14 17:52:42 m1-soc-ls1 kernel: bond0: An illegal loopback occurred on adapter (eno5)#012Check the configuration to verify that all adapters are connected to 802.3ad compliant switch ports

[root@m1-soc-ls1 ~]#

#	https://access.redhat.com/solutions/56604

#	https://access.redhat.com/solutions/4863081

https://www.mellanox.com/support/firmware/hpe

https://www.mellanox.com/support/firmware/identification

https://www.mellanox.com/support/firmware/firmware-downloads

https://www.mellanox.com/support/firmware/connectx4lxen

#	Here I’ve no found PSID for our Mellanox network card (HP_2690110034). Only MT_...

[root@m1-soc-ls1 ~]# ethtool -i eno5

driver: mlx5_core

version: 5.0-0

firmware-version: 14.26.1040 (HP_2690110034)

expansion-rom-version:

bus-info: 0000:5d:00.0

supports-statistics: yes

supports-test: yes

supports-eeprom-access: no

supports-register-dump: no

supports-priv-flags: yes

[root@m1-soc-ls1 ~]# lspci -s 0000:5d:00.0

5d:00.0 Ethernet controller: Mellanox Technologies MT27710 Family [ConnectX-4 Lx]

[root@m1-soc-ls1 ~]# lspci -s 0000:5d:00.1

5d:00.1 Ethernet controller: Mellanox Technologies MT27710 Family [ConnectX-4 Lx]

[root@m1-soc-ls1 ~]#

[root@m1-soc-ls1 ~]# ethtool eno5

Settings for eno5:

Supported ports: [ Backplane ]

Supported link modes:   1000baseKX/Full

10000baseKR/Full

25000baseCR/Full

25000baseKR/Full

25000baseSR/Full

Supported pause frame use: Symmetric

Supports auto-negotiation: Yes

Supported FEC modes: None BaseR

Advertised link modes:  1000baseKX/Full

10000baseKR/Full

25000baseCR/Full

25000baseKR/Full

25000baseSR/Full

Advertised pause frame use: Symmetric

Advertised auto-negotiation: Yes

Advertised FEC modes: None

Speed: 10000Mb/s

Duplex: Full

Port: Direct Attach Copper

PHYAD: 0

Transceiver: internal

Auto-negotiation: on

Supports Wake-on: g

Wake-on: g

Current message level: 0x00000004 (4)

link

Link detected: yes

[root@m1-soc-ls1 ~]# ethtool eno6

Settings for eno6:

Supported ports: [ FIBRE ]

Supported link modes:   1000baseKX/Full

10000baseKR/Full

25000baseCR/Full

25000baseKR/Full

25000baseSR/Full

Supported pause frame use: Symmetric

Supports auto-negotiation: Yes

Supported FEC modes: None BaseR

Advertised link modes:  1000baseKX/Full

10000baseKR/Full

25000baseCR/Full

25000baseKR/Full

25000baseSR/Full

Advertised pause frame use: Symmetric

Advertised auto-negotiation: Yes

Advertised FEC modes: None

Speed: 10000Mb/s

Duplex: Full

Port: FIBRE

PHYAD: 0

Transceiver: internal

Auto-negotiation: on

Supports Wake-on: g

Wake-on: d

Current message level: 0x00000004 (4)

link

Link detected: yes

[root@m1-soc-ls1 ~]#

[root@m1-soc-ls1 ~]# cat >> /etc/yum.repos.d/tcs-nexus-centos-8-base.repo

[tcs-nexus-centos-8-base]

name=Nexus - CentOS-8 - Base

baseurl=http://repo-linux.tcsbank.ru/repository/centos/centos/8/BaseOS/$basearch/os/

gpgcheck=1

gpgkey=file:///etc/pki/rpm-gpg/RPM-GPG-KEY-CentOS-8

skip_if_unavailable=True

proxy=_none_

[root@m1-soc-ls1 ~]# yum repolist

Nexus - CentOS-8 - Base                                                                                                                                             2.6 MB/s | 6.0 MB     00:02

Last metadata expiration check: 0:00:01 ago on Tue 14 Apr 2020 06:13:28 PM MSK.

Modular dependency problems:

Problem 1: conflicting requests

- nothing provides module(perl:5.26) needed by module perl-DBD-MySQL:4.046:8010020191114030811:073fa5fe-0.x86_64

Problem 2: conflicting requests

- nothing provides module(perl:5.26) needed by module perl-DBI:1.641:8010020191113222731:16b3ab4d-0.x86_64

repo id                                                                                        repo name                                                                                      status

tcs-nexus-centos-8-base                                                                        Nexus - CentOS-8 - Base                                                                        2,233

[root@m1-soc-ls1 ~]#

[root@m1-soc-ls1 ~]# cat /etc/yum.repos.d/tcs-nexus-centos-8-centosplus.repo

[tcs-nexus-centos-8-centosplus]

name=Nexus - CentOS-$releasever - Plus

mirrorlist=http://repo-linux.tcsbank.ru/repository/centos/?release=$releasever&arch=$basearch&repo=centosplus&infra=$infra

#baseurl=http://mirror.centos.org/$contentdir/$releasever/centosplus/$basearch/os/

gpgcheck=1

enabled=0

gpgkey=file:///etc/pki/rpm-gpg/RPM-GPG-KEY-centosofficial

[root@m1-soc-ls1 ~]#

#

#	Ansible doesn't work on just installed Centos 8.

#

(env) [support@centos msp-scm]$ ansible-playbook playbook-dco-bootstrap.yml -i list_of_hosts --become --extra-vars "ansible_user=root ansible_password=$(pass common/linux-default) ansible_sudo_pass=$(pass common/linux-default) ansible_ssh_common_args='-oStrictHostKeyChecking=no -oPubkeyAuthentication=no -oUserKnownHostsFile=/dev/null'"

PLAY [all[0]] *****************************************************************************************************************************************************************************************

TASK [assert] *****************************************************************************************************************************************************************************************

Tuesday 14 April 2020  18:07:40 +0300 (0:00:00.361)       0:00:00.361 *********

ok: [m1-soc-ls1.tcsbank.ru] => {

"changed": false

}

MSG:

All assertions passed

PLAY [all:!localhost] *********************************************************************************************************************************************************************************

TASK [Gathering Facts] ********************************************************************************************************************************************************************************

Tuesday 14 April 2020  18:07:40 +0300 (0:00:00.093)       0:00:00.455 *********

fatal: [m1-soc-ls1.tcsbank.ru]: FAILED! => {

"changed": false,

"rc": 127

}

MSG:

MODULE FAILURE

See stdout/stderr for the exact error

MODULE_STDOUT:

/bin/sh: /usr/bin/python: No such file or directory

MODULE_STDERR:

Shared connection to m1-soc-ls1.tcsbank.ru closed.

NO MORE HOSTS LEFT ************************************************************************************************************************************************************************************

NO MORE HOSTS LEFT ************************************************************************************************************************************************************************************

PLAY RECAP ********************************************************************************************************************************************************************************************

m1-soc-ls1.tcsbank.ru      : ok=1    changed=0    unreachable=0    failed=1

Tuesday 14 April 2020  18:07:47 +0300 (0:00:06.743)       0:00:07.199 *********

===============================================================================

Gathering Facts -------------------------------------------------------------------------------------------------------------------------------------------------------------------------------- 6.74s

assert ----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------- 0.09s

(env) [support@centos msp-scm]$

#

#	This is solution!!!

#	https://www.toptechskills.com/ansible-tutorials-courses/how-to-fix-usr-bin-python-not-found-error-tutorial/

#

ln --symbolic /usr/bin/python3 /usr/bin/python

#

#	But anyway for Centos 8 not prepaired appropriate repos in ansible playbook!

#

fatal: [m1-soc-ls1.tcsbank.ru]: FAILED! => {

"changed": false,

"rc": 1,

"results": []

}

MSG:

Failed to synchronize repodata: Failed to download metadata for repo 'CentOS-Base'

NO MORE HOSTS LEFT ************************************************************************************************************************************************************************************

NO MORE HOSTS LEFT ************************************************************************************************************************************************************************************

PLAY RECAP ********************************************************************************************************************************************************************************************

m1-soc-ls1.tcsbank.ru      : ok=24   changed=8    unreachable=0    failed=1

[root@m1-soc-ls1 yum.repos.d]# cat tcs-nexus-epel.repo

[tcs-nexus-epel]

name=Nexus - Extra Packages for Enterprise Linux

baseurl=http://repo-linux.tcsbank.ru/repository/epel/$releasever/$basearch/

enabled=1

gpgcheck=0

repo_gpgcheck=0

skip_if_unavailable=True

proxy=_none_

[root@m1-soc-ls1 yum.repos.d]#

#

#	Still unavailable to find package flint or msflint.

#

#	https://centos.pkgs.org/8/centos-appstream-x86_64/mstflint-4.12.0-1.el8.x86_64.rpm.html

[s.belogrud@TCS4357]# wget http://mirror.centos.org/centos/8/AppStream/x86_64/os/Packages/mstflint-4.12.0-1.el8.x86_64.rpm

--2020-04-14 19:44:54--  http://mirror.centos.org/centos/8/AppStream/x86_64/os/Packages/mstflint-4.12.0-1.el8.x86_64.rpm

Resolving mirror.centos.org... 81.171.33.194

Connecting to mirror.centos.org|81.171.33.194|:80... connected.

HTTP request sent, awaiting response... 200 OK

Length: 3582276 (3.4M) [application/x-rpm]

Saving to: `mstflint-4.12.0-1.el8.x86_64.rpm'

100%[==========================================================================================================================================================>] 3,582,276   7.23M/s   in 0.5s

2020-04-14 19:44:55 (7.23 MB/s) - `mstflint-4.12.0-1.el8.x86_64.rpm' saved [3582276/3582276]

[s.belogrud@TCS4357]# scp mstflint-4.12.0-1.el8.x86_64.rpm root@m1-soc-ls1:/tmp/

mstflint-4.12.0-1.el8.x86_64.rpm                                                                                                                                  100% 3498KB  16.1MB/s   00:00

[s.belogrud@TCS4357]#

[root@m1-soc-ls1 work]# yum localinstall 20200414-1941/mstflint-4.12.0-1.el8.x86_64.rpm

Last metadata expiration check: 0:08:05 ago on Tue 14 Apr 2020 07:38:50 PM MSK.

Modular dependency problems:

Problem 1: conflicting requests

- nothing provides module(perl:5.26) needed by module perl-DBD-MySQL:4.046:8010020191114030811:073fa5fe-0.x86_64

Problem 2: conflicting requests

- nothing provides module(perl:5.26) needed by module perl-DBI:1.641:8010020191113222731:16b3ab4d-0.x86_64

Error:

Problem: conflicting requests

- nothing provides libboost_regex.so.1.66.0()(64bit) needed by mstflint-4.12.0-1.el8.x86_64

(try to add '--skip-broken' to skip uninstallable packages or '--nobest' to use not only best candidate packages)

[root@m1-soc-ls1 work]#

[root@m1-soc-ls1 work]#

[root@m1-soc-ls1 work]# yum localinstall --skip-broken 20200414-1941/mstflint-4.12.0-1.el8.x86_64.rpm

Last metadata expiration check: 0:10:10 ago on Tue 14 Apr 2020 07:38:50 PM MSK.

Modular dependency problems:

Problem 1: conflicting requests

- nothing provides module(perl:5.26) needed by module perl-DBD-MySQL:4.046:8010020191114030811:073fa5fe-0.x86_64

Problem 2: conflicting requests

- nothing provides module(perl:5.26) needed by module perl-DBI:1.641:8010020191113222731:16b3ab4d-0.x86_64

Dependencies resolved.

Nothing to do.

Complete!

[root@m1-soc-ls1 work]#

[s.belogrud@TCS4357]# wget ftp://ftp.pbone.net/mirror/ftp.centos.org/8-stream/AppStream/x86_64/os/Packages/boost-regex-1.66.0-6.el8.x86_64.rpm

--2020-04-14 19:55:42--  ftp://ftp.pbone.net/mirror/ftp.centos.org/8-stream/AppStream/x86_64/os/Packages/boost-regex-1.66.0-6.el8.x86_64.rpm

=> `boost-regex-1.66.0-6.el8.x86_64.rpm'

Resolving ftp.pbone.net... 93.179.225.212

Connecting to ftp.pbone.net|93.179.225.212|:21... connected.

Logging in as anonymous ... Logged in!

==> SYST ... done.    ==> PWD ... done.

==> TYPE I ... done.  ==> CWD (1) /mirror/ftp.centos.org/8-stream/AppStream/x86_64/os/Packages ... done.

==> SIZE boost-regex-1.66.0-6.el8.x86_64.rpm ... 292624

==> PASV ... done.    ==> RETR boost-regex-1.66.0-6.el8.x86_64.rpm ... done.

Length: 292624 (286K) (unauthoritative)

100%[==========================================================================================================================================================>] 292,624      689K/s   in 0.4s

2020-04-14 19:55:44 (689 KB/s) - `boost-regex-1.66.0-6.el8.x86_64.rpm' saved [292624]

[s.belogrud@TCS4357]#

[s.belogrud@TCS4357]# scp boost-regex-1.66.0-6.el8.x86_64.rpm root@m1-soc-ls1:/tmp/

boost-regex-1.66.0-6.el8.x86_64.rpm                                                                                                                               100%  286KB  14.8MB/s   00:00

[s.belogrud@TCS4357]#

[root@m1-soc-ls1 20200414-1941]# yum localinstall mstflint-4.12.0-1.el8.x86_64.rpm boost-regex-1.66.0-6.el8.x86_64.rpm

Last metadata expiration check: 0:17:22 ago on Tue 14 Apr 2020 07:38:50 PM MSK.

Modular dependency problems:

Problem 1: conflicting requests

- nothing provides module(perl:5.26) needed by module perl-DBD-MySQL:4.046:8010020191114030811:073fa5fe-0.x86_64

Problem 2: conflicting requests

- nothing provides module(perl:5.26) needed by module perl-DBI:1.641:8010020191113222731:16b3ab4d-0.x86_64

Dependencies resolved.

====================================================================================================================================================================================================

Package                                         Architecture                               Version                                          Repository                                        Size

====================================================================================================================================================================================================

Installing:

mstflint                                        x86_64                                     4.12.0-1.el8                                     @commandline                                     3.4 M

boost-regex                                     x86_64                                     1.66.0-6.el8                                     @commandline                                     286 k

Transaction Summary

====================================================================================================================================================================================================

Install  2 Packages

Total size: 3.7 M

Installed size: 18 M

Is this ok [y/N]: y

Downloading Packages:

Running transaction check

Transaction check succeeded.

Running transaction test

Transaction test succeeded.

Running transaction

Preparing        :                                                                                                                                                                            1/1

Installing       : boost-regex-1.66.0-6.el8.x86_64                                                                                                                                            1/2

Running scriptlet: boost-regex-1.66.0-6.el8.x86_64                                                                                                                                            1/2

Installing       : mstflint-4.12.0-1.el8.x86_64                                                                                                                                               2/2

Running scriptlet: mstflint-4.12.0-1.el8.x86_64                                                                                                                                               2/2

Verifying        : mstflint-4.12.0-1.el8.x86_64                                                                                                                                               1/2

Verifying        : boost-regex-1.66.0-6.el8.x86_64                                                                                                                                            2/2

Installed:

mstflint-4.12.0-1.el8.x86_64                                                                    boost-regex-1.66.0-6.el8.x86_64

Complete!

[root@m1-soc-ls1 20200414-1941]#

................................................................................

#

#	https://www.mellanox.com/support/firmware/nic

#

[s.belogrud@TCS4357]# wget --no-check-certificate https://www.openfabrics.org/downloads/mstflint/mstflint-4.4.0.tar.gz

--2020-04-14 18:46:45--  https://www.openfabrics.org/downloads/mstflint/mstflint-4.4.0.tar.gz

Resolving www.openfabrics.org... 69.55.231.76

Connecting to www.openfabrics.org|69.55.231.76|:443... connected.

WARNING: cannot verify www.openfabrics.org's certificate, issued by `/C=RU/ST=Moscow/L=Moscow/O=Tinkoff Bank/CN=SI':

Unable to locally verify the issuer's authority.

HTTP request sent, awaiting response... 200 OK

Length: 1348758 (1.3M) [application/x-gzip]

Saving to: `mstflint-4.4.0.tar.gz'

100%[==========================================================================================================================================================>] 1,348,758    967K/s   in 1.4s

2020-04-14 18:46:48 (967 KB/s) - `mstflint-4.4.0.tar.gz' saved [1348758/1348758]

[s.belogrud@TCS4357]#

[s.belogrud@TCS4357]# scp mstflint-4.4.0.tar.gz root@m1-soc-ls1:/tmp/

mstflint-4.4.0.tar.gz                                                                                                                                             100% 1317KB  15.8MB/s   00:00

[s.belogrud@TCS4357]#

[root@m1-soc-ls1 yum.repos.d]# mkdir -p ~/work/20200414-1848

[root@m1-soc-ls1 yum.repos.d]# cp /tmp/mstflint-4.4.0.tar.gz ~/work/20200414-1848/

[root@m1-soc-ls1 yum.repos.d]# cd ~/work/20200414-1848/

[root@m1-soc-ls1 20200414-1848]# gunzip mstflint-4.4.0.tar.gz

[root@m1-soc-ls1 20200414-1848]#

[root@m1-soc-ls1 20200414-1848]# tar -xf mstflint-4.4.0.tar

[root@m1-soc-ls1 20200414-1848]# ll

total 6376

drwxr-xr-x 23 3189  101    4096 May 24  2016 mstflint-4.4.0

-rw-r--r--  1 root root 6522880 Apr 14 18:47 mstflint-4.4.0.tar

[root@m1-soc-ls1 20200414-1848]#

[root@m1-soc-ls1 mstflint-4.4.0]# ./configure --disable-dc --disable-inband

[root@m1-soc-ls1 mstflint-4.4.0]# yum install make

Nexus - Extra Packages for Enterprise Linux                                                                                                                         353 kB/s | 2.0 kB     00:00

Failed to download metadata for repo 'tcs-nexus-epel'

Ignoring repositories: tcs-nexus-epel

Last metadata expiration check: 0:20:18 ago on Tue 14 Apr 2020 06:31:26 PM MSK.

Modular dependency problems:

Problem 1: conflicting requests

- nothing provides module(perl:5.26) needed by module perl-DBD-MySQL:4.046:8010020191114030811:073fa5fe-0.x86_64

Problem 2: conflicting requests

- nothing provides module(perl:5.26) needed by module perl-DBI:1.641:8010020191113222731:16b3ab4d-0.x86_64

Dependencies resolved.

====================================================================================================================================================================================================

Package                                Architecture                             Version                                            Repository                                                 Size

====================================================================================================================================================================================================

Installing:

make                                   x86_64                                   1:4.2.1-9.el8                                      tcs-nexus-centos-8-base                                   498 k

Transaction Summary

====================================================================================================================================================================================================

Install  1 Package

Total download size: 498 k

Installed size: 1.4 M

Is this ok [y/N]: y

Downloading Packages:

make-4.2.1-9.el8.x86_64.rpm                                                                                                                                         512 kB/s | 498 kB     00:00

----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Total                                                                                                                                                               511 kB/s | 498 kB     00:00

warning: /var/cache/dnf/tcs-nexus-centos-8-base-1363f4b2d5422939/packages/make-4.2.1-9.el8.x86_64.rpm: Header V3 RSA/SHA256 Signature, key ID 8483c65d: NOKEY

Nexus - CentOS-8 - Base                                                                                                                                             0.0  B/s |   0  B     00:00

Curl error (37): Couldn't read a file:// file for file:///etc/pki/rpm-gpg/RPM-GPG-KEY-CentOS-8 [Couldn't open file /etc/pki/rpm-gpg/RPM-GPG-KEY-CentOS-8]

The downloaded packages were saved in cache until the next successful transaction.

You can remove cached packages by executing 'dnf clean packages'.

[root@m1-soc-ls1 mstflint-4.4.0]#

[root@m1-soc-ls1 mstflint-4.4.0]# dnf clean packages

2 files removed

[root@m1-soc-ls1 mstflint-4.4.0]#

#

#	I've turned off gpgcheck.

#

[root@m1-soc-ls1 mstflint-4.4.0]# vim /etc/yum.repos.d/tcs-nexus-centos-8-base.repo

[root@m1-soc-ls1 mstflint-4.4.0]# yum install make

Nexus - CentOS-8 - Base                                                                                                                                             798 kB/s | 3.8 kB     00:00

Nexus - Extra Packages for Enterprise Linux                                                                                                                         444 kB/s | 2.0 kB     00:00

Failed to download metadata for repo 'tcs-nexus-epel'

Ignoring repositories: tcs-nexus-epel

Modular dependency problems:

Problem 1: conflicting requests

- nothing provides module(perl:5.26) needed by module perl-DBD-MySQL:4.046:8010020191114030811:073fa5fe-0.x86_64

Problem 2: conflicting requests

- nothing provides module(perl:5.26) needed by module perl-DBI:1.641:8010020191113222731:16b3ab4d-0.x86_64

Dependencies resolved.

====================================================================================================================================================================================================

Package                                Architecture                             Version                                            Repository                                                 Size

====================================================================================================================================================================================================

Installing:

make                                   x86_64                                   1:4.2.1-9.el8                                      tcs-nexus-centos-8-base                                   498 k

Transaction Summary

====================================================================================================================================================================================================

Install  1 Package

Total size: 498 k

Installed size: 1.4 M

Is this ok [y/N]: y

Downloading Packages:

[SKIPPED] make-4.2.1-9.el8.x86_64.rpm: Already downloaded

Running transaction check

Transaction check succeeded.

Running transaction test

Transaction test succeeded.

Running transaction

Preparing        :                                                                                                                                                                            1/1

Installing       : make-1:4.2.1-9.el8.x86_64                                                                                                                                                  1/1

Running scriptlet: make-1:4.2.1-9.el8.x86_64                                                                                                                                                  1/1

Verifying        : make-1:4.2.1-9.el8.x86_64                                                                                                                                                  1/1

Installed:

make-1:4.2.1-9.el8.x86_64

Complete!

[root@m1-soc-ls1 mstflint-4.4.0]#

[root@m1-soc-ls1 mstflint-4.4.0]# make

make  all-recursive

make[1]: Entering directory '/root/work/20200414-1848/mstflint-4.4.0'

Making all in common

make[2]: Entering directory '/root/work/20200414-1848/mstflint-4.4.0/common'

make[2]: Nothing to be done for 'all'.

make[2]: Leaving directory '/root/work/20200414-1848/mstflint-4.4.0/common'

Making all in mft_utils

make[2]: Entering directory '/root/work/20200414-1848/mstflint-4.4.0/mft_utils'

source='mft_sig_handler.c' object='mft_sig_handler.o' libtool=no \

DEPDIR=.deps depmode=none /bin/sh ../config/depcomp \

gcc -DHAVE_CONFIG_H -I. -I..  -I. -I../common  -MD -pipe -Wall -W -Werror -g -O2 -Werror -DMST_UL -c mft_sig_handler.c

source='errmsg.cpp' object='errmsg.o' libtool=no \

DEPDIR=.deps depmode=none /bin/sh ../config/depcomp \

g++ -DHAVE_CONFIG_H -I. -I..  -I. -I../common   -DHAVE_TERMIOS_H -Werror -DMST_UL -c -o errmsg.o errmsg.cpp

../config/depcomp: line 772: exec: g++: not found

make[2]: *** [Makefile:428: errmsg.o] Error 127

make[2]: Leaving directory '/root/work/20200414-1848/mstflint-4.4.0/mft_utils'

make[1]: *** [Makefile:508: all-recursive] Error 1

make[1]: Leaving directory '/root/work/20200414-1848/mstflint-4.4.0'

make: *** [Makefile:394: all] Error 2

[root@m1-soc-ls1 mstflint-4.4.0]#

[root@m1-soc-ls1 20200414-1941]# mstflint -d 0000:5d:00.0 q

Image type:            FS3

FW Version:            14.26.1040

FW Release Date:       26.9.2019

Product Version:       14.26.1040

Rom Info:              type=UEFI version=14.19.14 cpu=AMD64

type=PXE version=3.5.803 cpu=AMD64

Description:           UID                GuidsNumber

Base GUID:             9440c9ffff71e164        16

Base MAC:              9440c971e164            16

Image VSD:             N/A

Device VSD:            N/A

PSID:                  HP_2690110034

Security Attributes:   N/A

[root@m1-soc-ls1 20200414-1941]# mstflint -d 0000:5d:00.1 q

Image type:            FS3

FW Version:            14.26.1040

FW Release Date:       26.9.2019

Product Version:       14.26.1040

Rom Info:              type=UEFI version=14.19.14 cpu=AMD64

type=PXE version=3.5.803 cpu=AMD64

Description:           UID                GuidsNumber

Base GUID:             9440c9ffff71e164        16

Base MAC:              9440c971e164            16

Image VSD:             N/A

Device VSD:            N/A

PSID:                  HP_2690110034

Security Attributes:   N/A

[root@m1-soc-ls1 20200414-1941]#

................................................................................

#

#	https://support.hpe.com/hpsc/swd/public/detail?swItemId=MTX_c93c3bae8f7c4b3a9c7b5e026b#tab3

#

[s.belogrud@TCS4357]# scp /drives/c/Users/s.belogrud/Downloads/firmware-* root@m1-soc-ls1:/tmp/

firmware-nic-mellanox-ethernet-only-1.0.11-1.1.x86_64.compsig                                                                                                     100% 2153   291.0KB/s   00:00

firmware-nic-mellanox-ethernet-only-1.0.11-1.1.x86_64.rpm                                                                                                         100% 7492KB  16.7MB/s   00:00

[s.belogrud@TCS4357]#

[root@m1-soc-ls1 20200414-2021]# rpm -qi firmware-nic-mellanox-ethernet-only-1.0.11-1.1.x86_64.rpm

warning: firmware-nic-mellanox-ethernet-only-1.0.11-1.1.x86_64.rpm: Header V3 RSA/SHA256 Signature, key ID 26c2b797: NOKEY

Name        : firmware-nic-mellanox-ethernet-only

Version     : 1.0.11

Release     : 1.1

Architecture: x86_64

Install Date: (not installed)

Group       : Applications/System

Size        : 57598839

License     : 2012 Hewlett Packard Enterprise Development LP

Signature   : RSA/SHA256, Thu 31 Oct 2019 12:55:30 PM MSK, Key ID c208adde26c2b797

Source RPM  : firmware-nic-mellanox-ethernet-only-1.0.11-1.1.src.rpm

Build Date  : Thu 31 Oct 2019 12:35:05 PM MSK

Build Host  : localhost

Relocations : (not relocatable)

Packager    : Hewlett Packard Enterprise <Linux_SWdeliverables@groups.ext.hpe.com>

Vendor      : Hewlett Packard Enterprise

URL         : http://www.hpe.com/info/proliantlinux

Summary     : Online Firmware Upgrade Utility (Linux x86_64) for HPE Mellanox Ethernet only adapters firmware

Description :

This package contains the Online Firmware Upgrade Utility (Linux x86_64) for HPE Mellanox Ethernet only adapters firmware

[root@m1-soc-ls1 20200414-2021]# rpm -ivh firmware-nic-mellanox-ethernet-only-1.0.11-1.1.x86_64.rpm

warning: firmware-nic-mellanox-ethernet-only-1.0.11-1.1.x86_64.rpm: Header V3 RSA/SHA256 Signature, key ID 26c2b797: NOKEY

Verifying...                          ################################# [100%]

Preparing...                          ################################# [100%]

Updating / installing...

1:firmware-nic-mellanox-ethernet-on################################# [100%]

[root@m1-soc-ls1 20200414-2021]#

[root@m1-soc-ls1 20200414-2021]# rpm -qpl firmware-nic-mellanox-ethernet-only-1.0.11-1.1.x86_64.rpm

warning: firmware-nic-mellanox-ethernet-only-1.0.11-1.1.x86_64.rpm: Header V3 RSA/SHA256 Signature, key ID 26c2b797: NOKEY

/usr/lib/x86_64-linux-gnu/firmware-nic-mellanox-ethernet-only-1.0.11-1.1

/usr/lib/x86_64-linux-gnu/firmware-nic-mellanox-ethernet-only-1.0.11-1.1/.cpq_package.inc

/usr/lib/x86_64-linux-gnu/firmware-nic-mellanox-ethernet-only-1.0.11-1.1/.setup

/usr/lib/x86_64-linux-gnu/firmware-nic-mellanox-ethernet-only-1.0.11-1.1/CP040051.xml

/usr/lib/x86_64-linux-gnu/firmware-nic-mellanox-ethernet-only-1.0.11-1.1/getmlnxinfo

/usr/lib/x86_64-linux-gnu/firmware-nic-mellanox-ethernet-only-1.0.11-1.1/hpsetup

/usr/lib/x86_64-linux-gnu/firmware-nic-mellanox-ethernet-only-1.0.11-1.1/nic_fw

/usr/lib/x86_64-linux-gnu/firmware-nic-mellanox-ethernet-only-1.0.11-1.1/nic_fw/NICFWData.xml

/usr/lib/x86_64-linux-gnu/firmware-nic-mellanox-ethernet-only-1.0.11-1.1/nic_fw/fw-ConnectX3Pro-full-rel-2_42_5044-779793-B21_0A-CLP-8025-UEFI-14.11.48-FlexBoot-3.4.754.bin

/usr/lib/x86_64-linux-gnu/firmware-nic-mellanox-ethernet-only-1.0.11-1.1/nic_fw/fw-ConnectX3Pro-full-rel-2_42_5044-779799-B21_Ax-CLP-8025-UEFI-14.11.48-FlexBoot-3.4.754.bin

/usr/lib/x86_64-linux-gnu/firmware-nic-mellanox-ethernet-only-1.0.11-1.1/nic_fw/fw-ConnectX4Lx-rel-14_26_1040-817749-B21_Ax-UEFI-14.19.14-FlexBoot-3.5.803.bin

/usr/lib/x86_64-linux-gnu/firmware-nic-mellanox-ethernet-only-1.0.11-1.1/nic_fw/fw-ConnectX4Lx-rel-14_26_1040-817753-B21_Ax-UEFI-14.19.14-FlexBoot-3.5.803.bin

/usr/lib/x86_64-linux-gnu/firmware-nic-mellanox-ethernet-only-1.0.11-1.1/nic_fw/fw-ConnectX5-rel-16_26_1040-874253-B21_Ax-UEFI-14.19.14-FlexBoot-3.5.803.signed.bin

/usr/lib/x86_64-linux-gnu/firmware-nic-mellanox-ethernet-only-1.0.11-1.1/nic_fw/mstflint

/usr/lib/x86_64-linux-gnu/firmware-nic-mellanox-ethernet-only-1.0.11-1.1/payload.json

/usr/lib/x86_64-linux-gnu/firmware-nic-mellanox-ethernet-only-1.0.11-1.1/setup

/usr/lib/x86_64-linux-gnu/hp-scexe-compat

/usr/lib/x86_64-linux-gnu/hp-scexe-compat/CP040051.scexe

/usr/lib/x86_64-linux-gnu/scexe-compat

/usr/lib/x86_64-linux-gnu/scexe-compat/CP040051.scexe

[root@m1-soc-ls1 20200414-2021]#

[root@m1-soc-ls1 20200414-2021]# /usr/lib/x86_64-linux-gnu/firmware-nic-mellanox-ethernet-only-1.0.11-1.1/setup

##################################################################################################

HPE Mellanox InfiniBand Online Firmware Upgrade Utility for Linux

Copyright (c) 2011 Hewlett-Packard Enterprise Development Company, L.P.

##################################################################################################

SecureBoot is disabled.

List of Network Adapters detected on the Server.................

[0] 0000:12:00.0 Broadcom Inc. and subsidiaries

[1] 0000:5d:00.0 Mellanox Technologies

If PSID or FW_Version is not found for some interfaces, please check /tmp/datazL530h

Interface 0000:12:00.0 is not Mellanox one.

Mellanox card info for 0000:5d:00.0 = FW_VERSION 14.26.1040, BUS_INFO 0000:5d:00.0, PSID HP_2690110034

0000:5d:00.0

DEVICE INFO---------->15b3 1015 1590 d3

pciIdString-------------->15B3-1015-1590-00D3-HP_2690110034

Current Firmware Version is 14.26.1040 on 0000:5d:00.0

Repository has these firmware versions............[1] Image Version  14.26.1040

Repository doesn't have newer firmware version, Installed firmware version is the latest one..........Please Quit the updation.

Would you like to flash the firmware?

y/n/q (n):y

Current FW version on flash:  14.26.1040

New FW version:               14.26.1040

Note: The new FW version is the same as the current FW version on flash.

Do you want to continue ? (y/n) [n] : y

Initializing image partition -   OK

Writing Boot image component -   OK

-I- To load new FW run mstfwreset or reboot machine.

Firmware Flashed:       SUCCESS  for interface 0000:5d:00.0

Please Reboot node for new image to be loaded into silicon

[root@m1-soc-ls1 20200414-2021]#

#

#	There is no results.

#	Shit continue drops.

#

#

#	On this server no problem with Mellanox card.

#	Version firmware is 14.24.9006

[root@ds-soc-ls1 ~]# ethtool -i eno5

driver: mlx5_core

version: 5.0-0

firmware-version: 14.24.9006 (HP_2690110034)

expansion-rom-version:

bus-info: 0000:5d:00.0

supports-statistics: yes

supports-test: yes

supports-eeprom-access: no

supports-register-dump: no

supports-priv-flags: yes

[root@ds-soc-ls1 ~]# lspci -s 0000:5d:00.0

5d:00.0 Ethernet controller: Mellanox Technologies MT27710 Family [ConnectX-4 Lx]

[root@ds-soc-ls1 ~]#

#

#	https://support.hpe.com/hpsc/swd/public/detail?swItemId=MTX_d0e63d0a446746b2bc8d096096#tab4

#

[s.belogrud@TCS4357]# scp /drives/c/Users/s.belogrud/Downloads/firmware-nic-mellanox-ethernet-only-1.0.10-1.1.x86_64.rpm root@m1-soc-ls1:/tmp/

firmware-nic-mellanox-ethernet-only-1.0.10-1.1.x86_64.rpm                                                                                                         100% 8042KB  15.1MB/s   00:00

[s.belogrud@TCS4357]#

[root@m1-soc-ls1 work]# mkdir 20200414-2043

[root@m1-soc-ls1 work]# cd 20200414-2043

[root@m1-soc-ls1 20200414-2043]# cp /tmp/firmware-nic-mellanox-ethernet-only-1.0.10-1.1.x86_64.rpm .

[root@m1-soc-ls1 20200414-2043]# rpm -ivh firmware-nic-mellanox-ethernet-only-1.0.10-1.1.x86_64.rpm

warning: firmware-nic-mellanox-ethernet-only-1.0.10-1.1.x86_64.rpm: Header V3 RSA/SHA256 Signature, key ID 26c2b797: NOKEY

Verifying...                          ################################# [100%]

Preparing...                          ################################# [100%]

package firmware-nic-mellanox-ethernet-only-1.0.11-1.1.x86_64 (which is newer than firmware-nic-mellanox-ethernet-only-1.0.10-1.1.x86_64) is already installed

[root@m1-soc-ls1 20200414-2043]# rpm -qpl firmware-nic-mellanox-ethernet-only-1.0.10-1.1.x86_64.rpm

warning: firmware-nic-mellanox-ethernet-only-1.0.10-1.1.x86_64.rpm: Header V3 RSA/SHA256 Signature, key ID 26c2b797: NOKEY

/usr/lib/x86_64-linux-gnu/firmware-nic-mellanox-ethernet-only-1.0.10-1.1

/usr/lib/x86_64-linux-gnu/firmware-nic-mellanox-ethernet-only-1.0.10-1.1/.cpq_package.inc

/usr/lib/x86_64-linux-gnu/firmware-nic-mellanox-ethernet-only-1.0.10-1.1/.setup

/usr/lib/x86_64-linux-gnu/firmware-nic-mellanox-ethernet-only-1.0.10-1.1/CP038539.xml

/usr/lib/x86_64-linux-gnu/firmware-nic-mellanox-ethernet-only-1.0.10-1.1/getmlnxinfo

/usr/lib/x86_64-linux-gnu/firmware-nic-mellanox-ethernet-only-1.0.10-1.1/hpsetup

/usr/lib/x86_64-linux-gnu/firmware-nic-mellanox-ethernet-only-1.0.10-1.1/nic_fw

/usr/lib/x86_64-linux-gnu/firmware-nic-mellanox-ethernet-only-1.0.10-1.1/nic_fw/NICFWData.xml

/usr/lib/x86_64-linux-gnu/firmware-nic-mellanox-ethernet-only-1.0.10-1.1/nic_fw/fw-ConnectX3Pro-full-rel-2_42_5044-779793-B21_0A-CLP-8025-UEFI-14.11.48-FlexBoot-3.4.754.bin

/usr/lib/x86_64-linux-gnu/firmware-nic-mellanox-ethernet-only-1.0.10-1.1/nic_fw/fw-ConnectX3Pro-full-rel-2_42_5044-779799-B21_Ax-CLP-8025-UEFI-14.11.48-FlexBoot-3.4.754.bin

/usr/lib/x86_64-linux-gnu/firmware-nic-mellanox-ethernet-only-1.0.10-1.1/nic_fw/fw-ConnectX4-rel-12_24_9000-868779-B21_Ax-UEFI-14.17.13-FlexBoot-3.5.603.signed.bin

/usr/lib/x86_64-linux-gnu/firmware-nic-mellanox-ethernet-only-1.0.10-1.1/nic_fw/fw-ConnectX4Lx-rel-14_24_9000-817753-B21_Ax-UEFI-14.17.13-FlexBoot-3.5.603.bin

/usr/lib/x86_64-linux-gnu/firmware-nic-mellanox-ethernet-only-1.0.10-1.1/nic_fw/fw-ConnectX4Lx-rel-14_24_9006-817749-B21_Ax-UEFI-14.17.13-FlexBoot-3.5.603.bin

/usr/lib/x86_64-linux-gnu/firmware-nic-mellanox-ethernet-only-1.0.10-1.1/nic_fw/fw-ConnectX5-rel-16_24_9000-874253-B21_Ax-UEFI-14.17.13-FlexBoot-3.5.603.signed.bin

/usr/lib/x86_64-linux-gnu/firmware-nic-mellanox-ethernet-only-1.0.10-1.1/nic_fw/mstflint

/usr/lib/x86_64-linux-gnu/firmware-nic-mellanox-ethernet-only-1.0.10-1.1/payload.json

/usr/lib/x86_64-linux-gnu/firmware-nic-mellanox-ethernet-only-1.0.10-1.1/setup

/usr/lib/x86_64-linux-gnu/hp-scexe-compat

/usr/lib/x86_64-linux-gnu/hp-scexe-compat/CP038539.scexe

/usr/lib/x86_64-linux-gnu/scexe-compat

/usr/lib/x86_64-linux-gnu/scexe-compat/CP038539.scexe

[root@m1-soc-ls1 20200414-2043]#

[root@m1-soc-ls1 20200414-2043]# yum remove firmware-nic-mellanox-ethernet-only-1.0.11-1.1.x86_64

Modular dependency problems:

Problem 1: conflicting requests

- nothing provides module(perl:5.26) needed by module perl-DBD-MySQL:4.046:8010020191114030811:073fa5fe-0.x86_64

Problem 2: conflicting requests

- nothing provides module(perl:5.26) needed by module perl-DBI:1.641:8010020191113222731:16b3ab4d-0.x86_64

Dependencies resolved.

====================================================================================================================================================================================================

Package                                                            Architecture                          Version                                      Repository                              Size

====================================================================================================================================================================================================

Removing:

firmware-nic-mellanox-ethernet-only                                x86_64                                1.0.11-1.1                                   @System                                 55 M

Transaction Summary

====================================================================================================================================================================================================

Remove  1 Package

Freed space: 55 M

Is this ok [y/N]: y

Running transaction check

Transaction check succeeded.

Running transaction test

Transaction test succeeded.

Running transaction

Preparing        :                                                                                                                                                                            1/1

Erasing          : firmware-nic-mellanox-ethernet-only-1.0.11-1.1.x86_64                                                                                                                      1/1

Running scriptlet: firmware-nic-mellanox-ethernet-only-1.0.11-1.1.x86_64                                                                                                                      1/1

Verifying        : firmware-nic-mellanox-ethernet-only-1.0.11-1.1.x86_64                                                                                                                      1/1

Removed:

firmware-nic-mellanox-ethernet-only-1.0.11-1.1.x86_64

Complete!

[root@m1-soc-ls1 20200414-2043]#

[root@m1-soc-ls1 20200414-2043]# rpm -ivh firmware-nic-mellanox-ethernet-only-1.0.10-1.1.x86_64.rpm

warning: firmware-nic-mellanox-ethernet-only-1.0.10-1.1.x86_64.rpm: Header V3 RSA/SHA256 Signature, key ID 26c2b797: NOKEY

Verifying...                          ################################# [100%]

Preparing...                          ################################# [100%]

Updating / installing...

1:firmware-nic-mellanox-ethernet-on################################# [100%]

[root@m1-soc-ls1 20200414-2043]#

[root@m1-soc-ls1 20200414-2043]# /usr/lib/x86_64-linux-gnu/firmware-nic-mellanox-ethernet-only-1.0.10-1.1/setup

##################################################################################################

HPE Mellanox InfiniBand Online Firmware Upgrade Utility for Linux

Copyright (c) 2011 Hewlett-Packard Enterprise Development Company, L.P.

##################################################################################################

SecureBoot is disabled.

List of Network Adapters detected on the Server.................

[0] 0000:12:00.0 Broadcom Inc. and subsidiaries

[1] 0000:5d:00.0 Mellanox Technologies

If PSID or FW_Version is not found for some interfaces, please check /tmp/datanjDv8b

Interface 0000:12:00.0 is not Mellanox one.

Mellanox card info for 0000:5d:00.0 = FW_VERSION 14.26.1040, BUS_INFO 0000:5d:00.0, PSID HP_2690110034

0000:5d:00.0

DEVICE INFO---------->15b3 1015 1590 d3

pciIdString-------------->15B3-1015-1590-00D3-HP_2690110034

Current Firmware Version is 14.26.1040 on 0000:5d:00.0

Repository has these firmware versions............[1] Image Version  14.24.9006

Repository doesn't have newer firmware version, Installed firmware version is the latest one..........Please Quit the updation.

Would you like to flash the firmware?

y/n/q (n):y

Current FW version on flash:  14.26.1040

New FW version:               14.24.9006

Note: The new FW version is older than the current FW version on flash.

Do you want to continue ? (y/n) [n] : y

Initializing image partition -   OK

Writing Boot image component -   OK

-I- To load new FW run mlxfwreset or reboot machine.

Firmware Flashed:       SUCCESS  for interface 0000:5d:00.0

Please Reboot node for new image to be loaded into silicon

[root@m1-soc-ls1 20200414-2043]#

................................................................................

#

#	20200525

#	Here is not tested idea to apply firmware update on non Rhel systems. On Ubuntu, for example.

#

#

#	Idea is extract firmware and utility from rpm to files. Compress them to tgz..

#	Copy extracted archive to target server.

#	Extract archive and run update utility.

#	It was not tested, yet. I’ve asked Ivanaev Artyom to downgrade firmware with special HP web-service that controls firmware on modern HP servers.

#

[s.belogrud@TCS4357]# scp /drives/c/Users/s.belogrud/Downloads/firmware-nic-mellanox-ethernet-only-1.0.10-1.1.x86_64.rpm bsv@m1-soc-ls1:/tmp/

firmware-nic-mellanox-ethernet-only-1.0.10-1.1.x86_64.rpm                                                                100% 8042KB  27.2MB/s   00:00

[s.belogrud@TCS4357]#

[root@m1-soc-ls1 ~]# mkdir ~/work/20200525-1240

[root@m1-soc-ls1 ~]# cp /tmp/firmware-nic-mellanox-ethernet-only-1.0.10-1.1.x86_64.rpm ~/work/20200525-1240/

[root@m1-soc-ls1 ~]# cd ~/work/20200525-1240/

[root@m1-soc-ls1 20200525-1240]#

[root@m1-soc-ls1 20200525-1240]# rpm2cpio firmware-nic-mellanox-ethernet-only-1.0.10-1.1.x86_64.rpm | cpio -id

138327 blocks

[root@m1-soc-ls1 20200525-1240]# ll

total 8048

-rwx------. 1 root root 8235307 May 25 12:40 firmware-nic-mellanox-ethernet-only-1.0.10-1.1.x86_64.rpm

drwxr-xr-x. 3 root root    4096 May 25 12:42 usr

[root@m1-soc-ls1 20200525-1240]#

[root@m1-soc-ls1 20200525-1240]# tar cvzf firmware-nic-mellanox-ethernet-only-1.0.10-1.1.x86_64.tgz usr/

[root@m1-soc-ls1 20200525-1240]# cp firmware-nic-mellanox-ethernet-only-1.0.10-1.1.x86_64.tgz /tmp

[s.belogrud@TCS4357]# scp bsv@m1-soc-ls1:/tmp/firmware-nic-mellanox-ethernet-only-1.0.10-1.1.x86_64.tgz /drives/c/Users/s.belogrud/Downloads/

firmware-nic-mellanox-ethernet-only-1.0.10-1.1.x86_64.tgz                                                                100% 9662KB 126.4KB/s   01:16

[s.belogrud@TCS4357]#

#

#	20210907

#	Another task with this trouble.

*. Задача SD "На сервере ds-pr-paym-es-w-metal-01 моргает bond интерфейс" (https://forge.tcsbank.ru/case/5961850/info)("other20210901 SD#5961850 Network interface bond0 produce messages in log on server ds-pr-paym-es-w-metal-01").

Создал канал в Слаке #sd5961850 для коммуникации по задаче.

https://forge.tcsbank.ru/case/5961850/info

................................................................................

https://forge.tcsbank.ru/case/5961850/info

Дополнительная информация:

На сервере ds-pr-paym-es-w-metal-01.tcsbank.ru что-то с bond интерфейсом, периодически пропадает связь ```[29562756.735500] bond0: An illegal loopback occurred on adapter (eno5)

Check the configuration to verify that all adapters are connected to 802.3ad compliant switch ports

[29562757.932698] bond0: An illegal loopback occurred on adapter (eno5)

Check the configuration to verify that all adapters are connected to 802.3ad compliant switch ports

[29562758.830586] bond0: An illegal loopback occurred on adapter (eno5)

Check the configuration to verify that all adapters are connected to 802.3ad compliant switch ports```

Сведения о сотруднике:

ФИО: Лобачев Андрей Николаевич

Domain: TCSBANK

Login: a.lobachev

Руководитель: Корнев Василий Тимофеевич

Внутренний номер: 37781

Почта для нотификации: a.lobachev@tinkoff.ru

................................................................................

root@ds-pr-paym-es-w-metal-01 ~$ cat /etc/redhat-release

CentOS Linux release 7.8.2003 (Core)

root@ds-pr-paym-es-w-metal-01 ~$ cat /proc/net/bonding/bond0

Ethernet Channel Bonding Driver: v3.7.1 (April 27, 2011)

Bonding Mode: IEEE 802.3ad Dynamic link aggregation

Transmit Hash Policy: layer3+4 (1)

MII Status: up

MII Polling Interval (ms): 100

Up Delay (ms): 0

Down Delay (ms): 0

802.3ad info

LACP rate: fast

Min links: 0

Aggregator selection policy (ad_select): stable

System priority: 65535

System MAC address: 94:40:c9:86:da:30

Active Aggregator Info:

Aggregator ID: 7

Number of ports: 2

Actor Key: 21

Partner Key: 5

Partner Mac Address: 03:31:32:04:04:00

Slave Interface: eno5

MII Status: up

Speed: 25000 Mbps

Duplex: full

Link Failure Count: 2

Permanent HW addr: 94:40:c9:86:da:30

Slave queue ID: 0

Aggregator ID: 7

Actor Churn State: none

Partner Churn State: none

Actor Churned Count: 0

Partner Churned Count: 0

details actor lacp pdu:

system priority: 65535

system mac address: 94:40:c9:86:da:30

port key: 21

port priority: 255

port number: 1

port state: 63

details partner lacp pdu:

system priority: 127

system mac address: 03:31:32:04:04:00

oper key: 5

port priority: 127

port number: 4

port state: 63

Slave Interface: eno6

MII Status: up

Speed: 25000 Mbps

Duplex: full

Link Failure Count: 2

Permanent HW addr: 94:40:c9:86:da:31

Slave queue ID: 0

Aggregator ID: 7

Actor Churn State: none

Partner Churn State: none

Actor Churned Count: 0

Partner Churned Count: 0

details actor lacp pdu:

system priority: 65535

system mac address: 94:40:c9:86:da:30

port key: 21

port priority: 255

port number: 2

port state: 63

details partner lacp pdu:

system priority: 127

system mac address: 03:31:32:04:04:00

oper key: 5

port priority: 127

port number: 5

port state: 63

root@ds-pr-paym-es-w-metal-01 ~$

root@ds-pr-paym-es-w-metal-01 ~$ ip a | grep -w inet

inet 127.0.0.1/8 scope host lo

inet 169.254.0.1/16 brd 169.254.255.255 scope global docker0

inet 10.213.90.1/24 brd 10.213.90.255 scope global bond0.1390

root@ds-pr-paym-es-w-metal-01 ~$ ip route

default via 10.213.90.254 dev bond0.1390

10.213.90.0/24 dev bond0.1390 proto kernel scope link src 10.213.90.1

169.254.0.0/16 dev docker0 proto kernel scope link src 169.254.0.1

169.254.0.0/16 dev bond0 scope link metric 1009

169.254.0.0/16 dev bond0.1390 scope link metric 1014

root@ds-pr-paym-es-w-metal-01 ~$

................................................................................

root@ds-pr-paym-es-w-metal-01 ~$ ls -Alh /var/log/messages*

-rw------- 1 root root  68M Sep  1 17:20 /var/log/messages

-rw------- 1 root root 134M Aug  8 03:11 /var/log/messages-20210808

-rw------- 1 root root 134M Aug 15 03:34 /var/log/messages-20210815

-rw------- 1 root root 134M Aug 22 03:19 /var/log/messages-20210822

-rw------- 1 root root 135M Aug 29 03:43 /var/log/messages-20210829

root@ds-pr-paym-es-w-metal-01 ~$

root@ds-pr-paym-es-w-metal-01 ~$ grep -ie "bond" /var/log/messages-20210808 | head

Aug  1 03:17:02 ds-pr-paym-es-w-metal-01 kernel: bond0: An illegal loopback occurred on adapter (eno5)#012Check the configuration to verify that all adapters are connected to 802.3ad compliant switch ports

Aug  1 03:17:03 ds-pr-paym-es-w-metal-01 kernel: bond0: An illegal loopback occurred on adapter (eno5)#012Check the configuration to verify that all adapters are connected to 802.3ad compliant switch ports

Aug  1 03:17:04 ds-pr-paym-es-w-metal-01 kernel: bond0: An illegal loopback occurred on adapter (eno5)#012Check the configuration to verify that all adapters are connected to 802.3ad compliant switch ports

Aug  1 03:17:05 ds-pr-paym-es-w-metal-01 kernel: bond0: An illegal loopback occurred on adapter (eno5)#012Check the configuration to verify that all adapters are connected to 802.3ad compliant switch ports

Aug  1 03:17:06 ds-pr-paym-es-w-metal-01 kernel: bond0: An illegal loopback occurred on adapter (eno5)#012Check the configuration to verify that all adapters are connected to 802.3ad compliant switch ports

Aug  1 03:17:07 ds-pr-paym-es-w-metal-01 kernel: bond0: An illegal loopback occurred on adapter (eno5)#012Check the configuration to verify that all adapters are connected to 802.3ad compliant switch ports

Aug  1 03:17:08 ds-pr-paym-es-w-metal-01 kernel: bond0: An illegal loopback occurred on adapter (eno5)#012Check the configuration to verify that all adapters are connected to 802.3ad compliant switch ports

Aug  1 03:17:09 ds-pr-paym-es-w-metal-01 kernel: bond0: An illegal loopback occurred on adapter (eno5)#012Check the configuration to verify that all adapters are connected to 802.3ad compliant switch ports

Aug  1 03:17:10 ds-pr-paym-es-w-metal-01 kernel: bond0: An illegal loopback occurred on adapter (eno5)#012Check the configuration to verify that all adapters are connected to 802.3ad compliant switch ports

Aug  1 03:17:11 ds-pr-paym-es-w-metal-01 kernel: bond0: An illegal loopback occurred on adapter (eno5)#012Check the configuration to verify that all adapters are connected to 802.3ad compliant switch ports

root@ds-pr-paym-es-w-metal-01 ~$

root@ds-pr-paym-es-w-metal-01 ~$ grep -ie "bond" /var/log/messages | tail -n 2

Sep  1 17:25:49 ds-pr-paym-es-w-metal-01 kernel: bond0: An illegal loopback occurred on adapter (eno5)#012Check the configuration to verify that all adapters are connected to 802.3ad compliant switch ports

Sep  1 17:25:50 ds-pr-paym-es-w-metal-01 kernel: bond0: An illegal loopback occurred on adapter (eno5)#012Check the configuration to verify that all adapters are connected to 802.3ad compliant switch ports

root@ds-pr-paym-es-w-metal-01 ~$

................................................................................

#

#	20210901

#

Белогруд Сергей Владимирович

•  Сегодня 17:41

@net-admins

Коллеги, проверьте пожалуйста, что есть с вашей стороны (настройки портов к которым подключен сервер, логи, прочее, что может помочь).

Самые давние записи в логе фиксируют сообщения с 01.08.2021 о том, что "heck the configuration to verify that all adapters are connected to 802.3ad compliant switch ports".

Сообщения фиксируются по настоящий момент.

root@ds-pr-paym-es-w-metal-01 ~$ grep -ie "bond" /var/log/messages-20210808 | head

Aug  1 03:17:02 ds-pr-paym-es-w-metal-01 kernel: bond0: An illegal loopback occurred on adapter (eno5)#012Check the configuration to verify that all adapters are connected to 802.3ad compliant switch ports

Aug  1 03:17:03 ds-pr-paym-es-w-metal-01 kernel: bond0: An illegal loopback occurred on adapter (eno5)#012Check the configuration to verify that all adapters are connected to 802.3ad compliant switch ports

Неполадок на сервере (OS, iLO) не обнаружено. Все оборудование в рабочем исправном состоянии.

root@ds-pr-paym-es-w-metal-01 ~$ ip a | grep -w inet

inet 127.0.0.1/8 scope host lo

inet 169.254.0.1/16 brd 169.254.255.255 scope global docker0

inet 10.213.90.1/24 brd 10.213.90.255 scope global bond0.1390

root@ds-pr-paym-es-w-metal-01 ~$ ip route

default via 10.213.90.254 dev bond0.1390

10.213.90.0/24 dev bond0.1390 proto kernel scope link src 10.213.90.1

169.254.0.0/16 dev docker0 proto kernel scope link src 169.254.0.1

169.254.0.0/16 dev bond0 scope link metric 1009

169.254.0.0/16 dev bond0.1390 scope link metric 1014

root@ds-pr-paym-es-w-metal-01 ~$

root@ds-pr-paym-es-w-metal-01 ~$ cat /etc/redhat-release

CentOS Linux release 7.8.2003 (Core)

root@ds-pr-paym-es-w-metal-01 ~$ cat /proc/net/bonding/bond0

Ethernet Channel Bonding Driver: v3.7.1 (April 27, 2011)

Bonding Mode: IEEE 802.3ad Dynamic link aggregation

Transmit Hash Policy: layer3+4 (1)

MII Status: up

MII Polling Interval (ms): 100

Up Delay (ms): 0

Down Delay (ms): 0

802.3ad info

LACP rate: fast

Min links: 0

Aggregator selection policy (ad_select): stable

System priority: 65535

System MAC address: 94:40:c9:86:da:30

Active Aggregator Info:

Aggregator ID: 7

Number of ports: 2

Actor Key: 21

Partner Key: 5

Partner Mac Address: 03:31:32:04:04:00

Slave Interface: eno5

MII Status: up

Speed: 25000 Mbps

Duplex: full

Link Failure Count: 2

Permanent HW addr: 94:40:c9:86:da:30

Slave queue ID: 0

Aggregator ID: 7

Actor Churn State: none

Partner Churn State: none

Actor Churned Count: 0

Partner Churned Count: 0

details actor lacp pdu:

system priority: 65535

system mac address: 94:40:c9:86:da:30

port key: 21

port priority: 255

port number: 1

port state: 63

details partner lacp pdu:

system priority: 127

system mac address: 03:31:32:04:04:00

oper key: 5

port priority: 127

port number: 4

port state: 63

Slave Interface: eno6

MII Status: up

Speed: 25000 Mbps

Duplex: full

Link Failure Count: 2

Permanent HW addr: 94:40:c9:86:da:31

Slave queue ID: 0

Aggregator ID: 7

Actor Churn State: none

Partner Churn State: none

Actor Churned Count: 0

Partner Churned Count: 0

details actor lacp pdu:

system priority: 65535

system mac address: 94:40:c9:86:da:30

port key: 21

port priority: 255

port number: 2

port state: 63

details partner lacp pdu:

system priority: 127

system mac address: 03:31:32:04:04:00

oper key: 5

port priority: 127

port number: 5

port state: 63

root@ds-pr-paym-es-w-metal-01 ~$

................................................................................

Мирошниченко Дмитрий Сергеевич

•  Сегодня 13:53

с MLAG проблем не вижу, конфиг на коммутаторах корректнен, lacp на текущий момент собран успешно.

Беглый гуглинг по проблеме говорит что есть такие ошибки на карточках mellanox, проверьте корреляцию пожалуйста с текущими на сервере.

................................................................................

#

#	20210906

root@ds-pr-paym-es-w-metal-01 ~$ ethtool -i eno5

driver: mlx5_core

version: 5.0-0

firmware-version: 14.26.1040 (HP_2690110034)

expansion-rom-version:

bus-info: 0000:5d:00.0

supports-statistics: yes

supports-test: yes

supports-eeprom-access: no

supports-register-dump: no

supports-priv-flags: yes

root@ds-pr-paym-es-w-metal-01 ~$ ethtool -i eno6

driver: mlx5_core

version: 5.0-0

firmware-version: 14.26.1040 (HP_2690110034)

expansion-rom-version:

bus-info: 0000:5d:00.1

supports-statistics: yes

supports-test: yes

supports-eeprom-access: no

supports-register-dump: no

supports-priv-flags: yes

root@ds-pr-paym-es-w-metal-01 ~$

root@ds-pr-paym-es-w-metal-01 ~$ lspci -s 0000:5d:00.0

5d:00.0 Ethernet controller: Mellanox Technologies MT27710 Family [ConnectX-4 Lx]

root@ds-pr-paym-es-w-metal-01 ~$ lspci -s 0000:5d:00.1

5d:00.1 Ethernet controller: Mellanox Technologies MT27710 Family [ConnectX-4 Lx]

root@ds-pr-paym-es-w-metal-01 ~$

root@ds-pr-paym-es-w-metal-01 ~$ lspci -s 0000:5d:00.0 -v

5d:00.0 Ethernet controller: Mellanox Technologies MT27710 Family [ConnectX-4 Lx]

Subsystem: Hewlett Packard Enterprise Device 00d3

Flags: bus master, fast devsel, latency 0, IRQ 24, NUMA node 0

Memory at de000000 (64-bit, prefetchable) [size=32M]

[virtual] Expansion ROM at e0200000 [disabled] [size=1M]

Capabilities: [60] Express Endpoint, MSI 00

Capabilities: [48] Vital Product Data

Capabilities: [9c] MSI-X: Enable+ Count=64 Masked-

Capabilities: [c0] Vendor Specific Information: Len=18 <?>

Capabilities: [40] Power Management version 3

Capabilities: [100] Advanced Error Reporting

Capabilities: [150] Alternative Routing-ID Interpretation (ARI)

Capabilities: [180] Single Root I/O Virtualization (SR-IOV)

Capabilities: [1c0] #19

Capabilities: [230] Access Control Services

Kernel driver in use: mlx5_core

Kernel modules: mlx5_core

root@ds-pr-paym-es-w-metal-01 ~$

................................................................................

https://www.mellanox.com/files/doc-2020/pb-connectx-4-lx-en-card.pdf

https://access.redhat.com/solutions/4863081

https://access.redhat.com/solutions/56604

................................................................................

Белогруд Сергей Владимирович

•  Сегодня 19:34

Нужно запланировать downtime сервера (потребуется его перезагрузка).

Для обновления firmware сетевой карты Mellanox.

................................................................................

https://downloads.linux.hpe.com/SDR/repo/spp-gen9/RHEL/7.8/x86_64/current/

https://downloads.linux.hpe.com/SDR/repo/spp-gen9/RHEL/7.8/x86_64/current/firmware-nic-mellanox-ethernet-only-1.0.10-1.1.x86_64.rpm

https://downloads.linux.hpe.com/SDR/repo/spp-gen9/RHEL/7.8/x86_64/current/firmware-nic-mellanox-ethernet-only-1.0.14-1.1.x86_64.rpm

................................................................................

#

#	20210907

#

[s.belogrud@nb2339]# scp /drives/c/Users/s.belogrud/Downloads/firmware-nic-mellanox-ethernet-only-1.0.14-1.1.x86_64.rpm bsv@ds-pr-paym-es-w-metal-01:/tmp

firmware-nic-mellanox-ethernet-only-1.0.14-1.1.x86_64.rpm                                                              100% 8133KB   3.9MB/s   00:02

[s.belogrud@nb2339]# scp /drives/c/Users/s.belogrud/Downloads/firmware-nic-mellanox-ethernet-only-1.0.10-1.1.x86_64.rpm bsv@ds-pr-paym-es-w-metal-01:/tmp

firmware-nic-mellanox-ethernet-only-1.0.10-1.1.x86_64.rpm                                                              100% 8042KB   3.9MB/s   00:02

[s.belogrud@nb2339]#

[s.belogrud@nb2339]# ssh bsv@ds-pr-paym-es-w-metal-01

Last login: Mon Sep  6 19:08:23 2021 from 172.30.50.21

bsv@ds-pr-paym-es-w-metal-01 ~$ sudo -i

[sudo] password for bsv:

root@ds-pr-paym-es-w-metal-01 ~$ mkdir -p ~/work/20210907-1159

root@ds-pr-paym-es-w-metal-01 ~$ cp /tmp/firmware-nic-mellanox-ethernet-only-1.0.1*.rpm ~/work/20210907-1159/

root@ds-pr-paym-es-w-metal-01 ~$ cd ~/work/20210907-1159/

root@ds-pr-paym-es-w-metal-01 work/20210907-1159$

root@ds-pr-paym-es-w-metal-01 work/20210907-1159$ ethtool -i eno5 | grep -ie "firmware-version"

firmware-version: 14.26.1040 (HP_2690110034)

root@ds-pr-paym-es-w-metal-01 work/20210907-1159$ ethtool -i eno6 | grep -ie "firmware-version"

firmware-version: 14.26.1040 (HP_2690110034)

root@ds-pr-paym-es-w-metal-01 work/20210907-1159$

root@ds-pr-paym-es-w-metal-01 work/20210907-1159$ ll

total 16180

-rwx------ 1 root root 8235307 Sep  7 11:59 firmware-nic-mellanox-ethernet-only-1.0.10-1.1.x86_64.rpm

-rwx------ 1 root root 8328427 Sep  7 11:59 firmware-nic-mellanox-ethernet-only-1.0.14-1.1.x86_64.rpm

root@ds-pr-paym-es-w-metal-01 work/20210907-1159$

root@ds-pr-paym-es-w-metal-01 work/20210907-1159$ rpm -ivh ./firmware-nic-mellanox-ethernet-only-1.0.14-1.1.x86_64.rpm

warning: ./firmware-nic-mellanox-ethernet-only-1.0.14-1.1.x86_64.rpm: Header V3 RSA/SHA256 Signature, key ID 26c2b797: NOKEY

Preparing...                          ################################# [100%]

Updating / installing...

1:firmware-nic-mellanox-ethernet-on################################# [100%]

root@ds-pr-paym-es-w-metal-01 work/20210907-1159$

root@ds-pr-paym-es-w-metal-01 work/20210907-1159$ /usr/lib/x86_64-linux-gnu/firmware-nic-mellanox-ethernet-only-1.0.14-1.1/setup

##################################################################################################

HPE Mellanox InfiniBand Online Firmware Upgrade Utility for Linux

Copyright (c) 2011 Hewlett-Packard Enterprise Development Company, L.P.

##################################################################################################

SecureBoot is disabled.

List of Network Adapters detected on the Server.................

[0] 0000:02:00.0 Broadcom Inc. and subsidiaries

[1] 0000:5d:00.0 Mellanox Technologies

If PSID or FW_Version is not found for some interfaces, please check /tmp/dataGDdUPN

Interface 0000:02:00.0 is not Mellanox one.

Mellanox card info for 0000:5d:00.0 = FW_VERSION 14.26.1040, BUS_INFO 0000:5d:00.0, PSID HP_2690110034

0000:5d:00.0

DEVICE INFO---------->15b3 1015 1590 d3

pciIdString-------------->15B3-1015-1590-00D3-HP_2690110034

Repository has New firmware version for interface 0000:5d:00.0.................Please flash the newer version

Current Firmware Version is 14.26.1040 on 0000:5d:00.0

Repository has these firmware versions............[1] Image Version  14.29.1016

Would you like to flash the firmware?

y/n/q (y):y

Current FW version on flash:  14.26.1040

New FW version:               14.29.1016

Initializing image partition -   OK

Writing Boot image component -   4%

...

Writing Boot image component -   OK

-I- To load new FW run mstfwreset or reboot machine.

Firmware Flashed:       SUCCESS  for interface 0000:5d:00.0

Please Reboot node for new image to be loaded into silicon

root@ds-pr-paym-es-w-metal-01 work/20210907-1159$

root@ds-pr-paym-es-w-metal-01 work/20210907-1159$ shutdown -r now

Connection to ds-pr-paym-es-w-metal-01 closed by remote host.

Connection to ds-pr-paym-es-w-metal-01 closed.

[s.belogrud@nb2339]#

[s.belogrud@nb2339]# ssh bsv@ds-pr-paym-es-w-metal-01

Last login: Tue Sep  7 11:58:26 2021 from 172.30.46.87

bsv@ds-pr-paym-es-w-metal-01 ~$ sudo -i

[sudo] password for bsv:

root@ds-pr-paym-es-w-metal-01 ~$ (ethtool -i eno5 ; ethtool -i eno6) | grep -ie "firmware-version"

firmware-version: 14.29.1016 (HP_2690110034)

firmware-version: 14.29.1016 (HP_2690110034)

root@ds-pr-paym-es-w-metal-01 ~$

root@ds-pr-paym-es-w-metal-01 ~$ tail -f /var/log/messages

Sep  7 12:29:26 ds-pr-paym-es-w-metal-01 consul: 2021/09/07 12:29:26 [WARN] agent: Check "paym-es-w-tcp-9200-check" socket connection failed: dial tcp 10.213.90.1:9201: connect: connection refused

Sep  7 12:29:26 ds-pr-paym-es-w-metal-01 consul[2178]: agent: Check "paym-es-w-tcp-9200-check" socket connection failed: dial tcp 10.213.90.1:9201: connect: connection refused

Sep  7 12:29:41 ds-pr-paym-es-w-metal-01 consul: 2021/09/07 12:29:41 [WARN] agent: Check "paym-es-w-tcp-9200-check" socket connection failed: dial tcp 10.213.90.1:9201: connect: connection refused

Sep  7 12:29:41 ds-pr-paym-es-w-metal-01 consul[2178]: agent: Check "paym-es-w-tcp-9200-check" socket connection failed: dial tcp 10.213.90.1:9201: connect: connection refused

Sep  7 12:30:01 ds-pr-paym-es-w-metal-01 systemd: Created slice User Slice of root.

Sep  7 12:30:01 ds-pr-paym-es-w-metal-01 systemd: Started Session 3 of user root.

Sep  7 12:30:01 ds-pr-paym-es-w-metal-01 systemd: Removed slice User Slice of root.

Sep  7 12:31:01 ds-pr-paym-es-w-metal-01 systemd: Created slice User Slice of root.

Sep  7 12:31:01 ds-pr-paym-es-w-metal-01 systemd: Started Session 4 of user root.

Sep  7 12:31:01 ds-pr-paym-es-w-metal-01 systemd: Removed slice User Slice of root.

root@ds-pr-paym-es-w-metal-01 ~$

#

#	Задачу по этой же теме по другим серверам "Проверить firmware сетевой карты на bare metal ЕЛК" (https://jira.tcsbank.ru/browse/IN-1465) набросило на Курбанова Константина.

Oct 13, 2021

#	Та же задачка по очередному серверу.

#	Задача "обновить прошивку на сервере" (https://jira.tcsbank.ru/browse/IN-1629) [Третьяков Артём].

#	https://tinkoff.slack.com/archives/C482SKHJP/p1634020936192400

#	Oct 18, 2021 Задача "обновить прошивку на сервере" (https://jira.tcsbank.ru/browse/IN-1657) [Третьяков Артём]

#

#	Make Nagios downtime.

#	And collect information about server via ansible playbook.

#

[s.belogrud@nb2339]# scp /drives/c/Users/s.belogrud/Downloads/firmware-nic-mellanox-ethernet-only-1.0.14-1.1.x86_64.rpm s_s.belogrud@ds-pr-paym-es-w-meta

l-02.tcsbank.ru:/tmp/

firmware-nic-mellanox-ethernet-only-1.0.14-1.1.x86_64.rpm                                                              100% 8133KB   3.9MB/s   00:02

[s.belogrud@nb2339]#

root@ds-pr-paym-es-w-metal-02 ~$ mkdir -p ~/work/20211013-1121

root@ds-pr-paym-es-w-metal-02 ~$ cp /tmp/firmware-nic-mellanox-ethernet-only-1.0.14-1.1.x86_64.rpm ~/work/20211013-1121/

root@ds-pr-paym-es-w-metal-02 ~$ cat /proc/net/bonding/bond0 | grep -ie "interface"

Slave Interface: eno5

Slave Interface: eno6

root@ds-pr-paym-es-w-metal-02 ~$ (ethtool -i eno5 ; ethtool -i eno6) | grep -ie "firmware-version"

firmware-version: 14.26.1040 (HP_2690110034)

firmware-version: 14.26.1040 (HP_2690110034)

root@ds-pr-paym-es-w-metal-02 ~$

root@ds-pr-paym-es-w-metal-02 ~$ rpm -ivh ~/work/20211013-1121/firmware-nic-mellanox-ethernet-only-1.0.14-1.1.x86_64.rpm

warning: /root/work/20211013-1121/firmware-nic-mellanox-ethernet-only-1.0.14-1.1.x86_64.rpm: Header V3 RSA/SHA256 Signature, key ID 26c2b797: NOKEY

Preparing...                          ################################# [100%]

package firmware-nic-mellanox-ethernet-only-1.0.14-1.1.x86_64 is already installed

root@ds-pr-paym-es-w-metal-02 ~$

root@ds-pr-paym-es-w-metal-02 ~$ /usr/lib/x86_64-linux-gnu/firmware-nic-mellanox-ethernet-only-1.0.14-1.1/setup

##################################################################################################

HPE Mellanox InfiniBand Online Firmware Upgrade Utility for Linux

Copyright (c) 2011 Hewlett-Packard Enterprise Development Company, L.P.

##################################################################################################

SecureBoot is disabled.

List of Network Adapters detected on the Server.................

[0] 0000:02:00.0 Broadcom Inc. and subsidiaries

[1] 0000:5d:00.0 Mellanox Technologies

If PSID or FW_Version is not found for some interfaces, please check /tmp/datalDHG2B

Interface 0000:02:00.0 is not Mellanox one.

Mellanox card info for 0000:5d:00.0 = FW_VERSION 14.26.1040, BUS_INFO 0000:5d:00.0, PSID HP_2690110034

0000:5d:00.0

DEVICE INFO---------->15b3 1015 1590 d3

pciIdString-------------->15B3-1015-1590-00D3-HP_2690110034

Repository has New firmware version for interface 0000:5d:00.0.................Please flash the newer version

Current Firmware Version is 14.26.1040 on 0000:5d:00.0

Repository has these firmware versions............[1] Image Version  14.29.1016

Would you like to flash the firmware?

y/n/q (y):y

Current FW version on flash:  14.26.1040

New FW version:               14.29.1016

Initializing image partition -   OK

Writing Boot image component -  11%

...

Initializing image partition -   OK

Writing Boot image component -   OK

-I- To load new FW run mstfwreset or reboot machine.

Firmware Flashed:       SUCCESS  for interface 0000:5d:00.0

Please Reboot node for new image to be loaded into silicon

root@ds-pr-paym-es-w-metal-02 ~$

root@ds-pr-paym-es-w-metal-02 ~$ shutdown -r now

Connection to ds-pr-paym-es-w-metal-02.tcsbank.ru closed by remote host.

Connection to ds-pr-paym-es-w-metal-02.tcsbank.ru closed.

[s.belogrud@nb2339]#

[s.belogrud@nb2339]# ssh s_s.belogrud@ds-pr-paym-es-w-metal-02.tcsbank.ru

Last login: Wed Oct 13 11:20:26 2021 from 172.30.8.114

s_s.belogrud@ds-pr-paym-es-w-metal-02 ~$ sudo -i

[sudo] password for s_s.belogrud:

root@ds-pr-paym-es-w-metal-02 ~$ (ethtool -i eno5 ; ethtool -i eno6) | grep -ie "firmware-version"

firmware-version: 14.29.1016 (HP_2690110034)

firmware-version: 14.29.1016 (HP_2690110034)

root@ds-pr-paym-es-w-metal-02 ~$