20151014

#	If command line utility racadm doesn’t work on some server.

[root@ds-hadoop-wk09p ~]# racadm getniccfg

ERROR: RACADM is unable to process the requested subcommand because there is no

local RAC configuration to communicate with.

Local RACADM subcommand execution requires the following:

1. A Remote Access Controller (RAC) must be present on the managed server

2. Appropriate managed node software must be installed and running on the

server

[root@ds-hadoop-wk09p ~]#

[root@ds-hadoop-wk09p ~]# rpm -qa | grep -i srvadmin-idracadm

[root@ds-hadoop-wk09p ~]#

[root@ds-hadoop-wk09p ~]# yum install srvadmin-idracadm

#	Still not working.

[root@ds-hadoop-wk09p ~]# racadm getniccfg

NOTE:

The current version of Integrated Dell Remote Access Controller (iDRAC)

that do not support the execution of RACADM commands.

Contact Dell customer service to upgrade to iDRAC Express or iDRAC Enterprise

The installed version of RACADM client executable that do not match the

generation of the server.

[root@ds-hadoop-wk09p ~]#

[root@ds-hadoop-wk09p ~]# yum install srvadmin-idracadm7

#	After installed follow package the racadm utility starts working well.

[root@ds-hadoop-wk09p ~]# racadm getniccfg | head

IPv4 settings:

NIC Enabled          = 1

IPv4 Enabled         = 1

DHCP Enabled         = 0

IP Address           = 10.219.11.61

Subnet Mask          = 255.255.255.0

Gateway              = 10.219.11.254

IPv6 settings:

[root@ds-hadoop-wk09p ~]#

#	In assume - we should have installed two package.

[root@ds-gp-prd-sdw1 ~]# rpm -qa | grep -i srvadmin-idracadm

srvadmin-idracadm-8.1.0-4.4.7.el6.x86_64

srvadmin-idracadm7-8.1.0-4.69.4.el6.x86_64

[root@ds-gp-prd-sdw1 ~]#

20151008

[root@ds-gp-prd-sdw7 ~]# racadm storage get vdisks -o -p layout,status

Disk.Virtual.0:RAID.Integrated.1-1

Layout                           = Raid-1

Status                           = Ok

Disk.Virtual.0:RAID.Slot.3-1

Layout                           = Raid-10

Status                           = Warning

Disk.Virtual.1:RAID.Integrated.1-1

Layout                           = Raid-10

Status                           = Ok

[root@ds-gp-prd-sdw7 ~]#

[root@ds-gp-prd-sdw7 ~]# racadm storage get pdisks -o -p status

Disk.Bay.0:Enclosure.Internal.0-1:RAID.Integrated.1-1

Status                           = Ok

Disk.Bay.1:Enclosure.Internal.0-1:RAID.Integrated.1-1

Status                           = Ok

Disk.Bay.2:Enclosure.Internal.0-1:RAID.Integrated.1-1

Status                           = Ok

Disk.Bay.3:Enclosure.Internal.0-1:RAID.Integrated.1-1

Status                           = Ok

Disk.Bay.4:Enclosure.Internal.0-1:RAID.Integrated.1-1

Status                           = Ok

Disk.Bay.5:Enclosure.Internal.0-1:RAID.Integrated.1-1

Status                           = Ok

Disk.Bay.6:Enclosure.Internal.0-1:RAID.Integrated.1-1

Status                           = Ok

Disk.Bay.7:Enclosure.Internal.0-1:RAID.Integrated.1-1

Status                           = Ok

Disk.Bay.8:Enclosure.Internal.0-1:RAID.Integrated.1-1

Status                           = Ok

Disk.Bay.12:Enclosure.Internal.0-1:RAID.Slot.3-1

Status                           = Ok

Disk.Bay.13:Enclosure.Internal.0-1:RAID.Slot.3-1

Status                           = Failed

Disk.Bay.14:Enclosure.Internal.0-1:RAID.Slot.3-1

Status                           = Ok

Disk.Bay.15:Enclosure.Internal.0-1:RAID.Slot.3-1

Status                           = Ok

Disk.Bay.16:Enclosure.Internal.0-1:RAID.Slot.3-1

Status                           = Ok

Disk.Bay.17:Enclosure.Internal.0-1:RAID.Slot.3-1

Status                           = Ok

Disk.Bay.18:Enclosure.Internal.0-1:RAID.Slot.3-1

Status                           = Ok

Disk.Bay.19:Enclosure.Internal.0-1:RAID.Slot.3-1

Status                           = Ok

Disk.Bay.9:Enclosure.Internal.0-1:RAID.Integrated.1-1

Status                           = Ok

[root@ds-gp-prd-sdw7 ~]#

[root@ds-gp-prd-sdw7 ~]# racadm storage get pdisks:Disk.Bay.13:Enclosure.Internal.0-1:RAID.Slot.3-1

Disk.Bay.13:Enclosure.Internal.0-1:RAID.Slot.3-1

Status                           = Failed

DeviceDescription                = Disk 13 in Backplane 1 of RAID Controller in Slot 3

RollupStatus                     = Failed

Name                             = Physical Disk 0:1:13

State                            = Failed

OperationState                   = Not Applicable

PowerStatus                      = Spun-Up

Size                             = 558.38 GB

FailurePredicted                 = NO

RemainingRatedWriteEndurance     = Not Applicable

SecurityStatus                   = Not Capable

BusProtocol                      = SAS

MediaType                        = HDD

UsedRaidDiskSpace                = 558.38 GB

AvailableRaidDiskSpace           = 558.38 GB

Hotspare                         = NO

Manufacturer                     = TOSHIBA

ProductId                        = AL13SEB600

Revision                         = DE11

SerialNumber                     = 6590A03LFRD3

PartNumber                       = PH05TFDD7557156CFKXJA06

NegotiatedSpeed                  = 6.0 Gb/s

ManufacturedDay                  = 6

ManufacturedWeek                 = 24

ManufacturedYear                 = 2015

SasAddress                       = 0x500003965808297E

FormFactor                       = 2.5 Inch

RaidNominalMediumRotationRate    = 10000

T10PICapability                  = Not Capable

BlockSizeInBytes                 = 512

MaxCapableSpeed                  = 6 Gb/s

[root@ds-gp-prd-sdw7 ~]#

[root@ds-gp-prd-sdw7 ~]# racadm storage get pdisks -o -p name,status

Disk.Bay.0:Enclosure.Internal.0-1:RAID.Integrated.1-1

Name                             = Physical Disk 0:1:0

Status                           = Ok

Disk.Bay.1:Enclosure.Internal.0-1:RAID.Integrated.1-1

Name                             = Physical Disk 0:1:1

Status                           = Ok

Disk.Bay.2:Enclosure.Internal.0-1:RAID.Integrated.1-1

Name                             = Physical Disk 0:1:2

Status                           = Ok

Disk.Bay.3:Enclosure.Internal.0-1:RAID.Integrated.1-1

Name                             = Physical Disk 0:1:3

Status                           = Ok

Disk.Bay.4:Enclosure.Internal.0-1:RAID.Integrated.1-1

Name                             = Physical Disk 0:1:4

Status                           = Ok

Disk.Bay.5:Enclosure.Internal.0-1:RAID.Integrated.1-1

Name                             = Physical Disk 0:1:5

Status                           = Ok

Disk.Bay.6:Enclosure.Internal.0-1:RAID.Integrated.1-1

Name                             = Physical Disk 0:1:6

Status                           = Ok

Disk.Bay.7:Enclosure.Internal.0-1:RAID.Integrated.1-1

Name                             = Physical Disk 0:1:7

Status                           = Ok

Disk.Bay.8:Enclosure.Internal.0-1:RAID.Integrated.1-1

Name                             = Physical Disk 0:1:8

Status                           = Ok

Disk.Bay.12:Enclosure.Internal.0-1:RAID.Slot.3-1

Name                             = Physical Disk 0:1:12

Status                           = Ok

Disk.Bay.13:Enclosure.Internal.0-1:RAID.Slot.3-1

Name                             = Physical Disk 0:1:13

Status                           = Failed

Disk.Bay.14:Enclosure.Internal.0-1:RAID.Slot.3-1

Name                             = Physical Disk 0:1:14

Status                           = Ok

Disk.Bay.15:Enclosure.Internal.0-1:RAID.Slot.3-1

Name                             = Physical Disk 0:1:15

Status                           = Ok

Disk.Bay.16:Enclosure.Internal.0-1:RAID.Slot.3-1

Name                             = Physical Disk 0:1:16

Status                           = Ok

Disk.Bay.17:Enclosure.Internal.0-1:RAID.Slot.3-1

Name                             = Physical Disk 0:1:17

Status                           = Ok

Disk.Bay.18:Enclosure.Internal.0-1:RAID.Slot.3-1

Name                             = Physical Disk 0:1:18

Status                           = Ok

Disk.Bay.19:Enclosure.Internal.0-1:RAID.Slot.3-1

Name                             = Physical Disk 0:1:19

Status                           = Ok

Disk.Bay.9:Enclosure.Internal.0-1:RAID.Integrated.1-1

Name                             = Physical Disk 0:1:9

Status                           = Ok

[root@ds-gp-prd-sdw7 ~]#

[root@ds-gp-prd-sdw7 ~]# racadm storage get controllers -o -p name,status

RAID.Integrated.1-1

Name                             = PERC H730P Mini (Embedded)

Status                           = Ok

RAID.Slot.3-1

Name                             = PERC H730P Adapter (PCI Slot 3)

Status                           = Ok

[root@ds-gp-prd-sdw7 ~]#

[root@ds-gp-prd-sdw7 ~]# racadm help raid

Storage monitoring, inventory and configuration of hardware RAID connected to the server.

Usage :

racadm storage get status

racadm storage get <Object type I>

racadm storage get <Object type I> -o

racadm storage get <Object type I> -o -p <property names separated by comma>

racadm storage get <Object type I>:<FQDD's of Object type separated by comma>

-p <property names separated by comma>

racadm storage get <Object type I>:<FQDD's of Object type separated by comma>

racadm storage get <Object type II> --refkey <reference keys separated by comma>

racadm storage get <Object type II> --refkey <reference keys separated by comma> -o

racadm storage get <Object type II> --refkey <reference keys separated by comma> -o

-p <property names separated by comma>

racadm storage <Object type III>:<FQDD>

racadm storage hotspare:<Physical Disk FQDD> -assign yes  -type dhs -vdkey:<FQDD of VD>

racadm storage init:<VD FQDD> -speed fast }

racadm storage createvd:<Controller FQDD> -rl {r0|r1|r5|r6|r10|r50|r60} [-wp {wt|wb}] [-rp {nra|ra|ara}]

[-ss {1k|2k|4k|8k|16k|32k|64k|128k|256k|512k|1M|2M|4M|8M|16M}] -pdkey:<comma separated PD FQDD>

[-dcp {enabled|disabled|default}] [-name <VD name>] [-size <VD size>{b|k|m|g|t}]

racadm storage createsecuritykey:<Controller FQDD> -key <Key id> -passwd <passphrase>

racadm storage modifysecuritykey:<Controller FQDD> -key <Key id>

-oldpasswd <old passphrase> -newpasswd <new passphrase>

racadm storage blink:<PD FQDD>

racadm storage unblink:<PD FQDD>

racadm storage preparetoremove:<PCIeSSD FQDD>

racadm storage secureerase:<PCIeSSD FQDD>

racadm storage patrolread:<Controller FQDD> -state start

--------------------------------------------------------------------------------

Valid Options:

Object type I      : controllers, batteries, vdisks, pdisks, fans, emms, temppro

bes, psus, enclosures.

Object type II     : batteries, vdisks, pdisks, fans, emms, tempprobes, psus, enclosures.

Object type III    : resetconfig, clearconfig, importconfig, deletesecuritykey, encryptvd,

deletevd, converttoraid, converttononraid, ccheck, cancelcheck

-o                 : Displays all the properties of the selected Key or Object.

-p                 : Displays the property names with filter.

FQDD's             : Displays all the properties of the FQDD's Key.

NOTE: Maximum Property names can be specified in -p option is = 10.

NOTE: Maximum FQDD's can be specified is = 3.

--------------------------------------------------------------------------------

Usage Examples :

racadm storage get controllers

racadm storage get controllers -o

racadm storage get controllers -o -p name,status

racadm storage get vdisks -o -p layout,status

racadm storage get controllers:RAID.Integrated.1-1

racadm storage get controllers:RAID.Integrated.1-1 -p status

racadm storage resetconfig:RAID.Integrated.1-1

racadm storage deletevd:Disk.Virtual.0:RAID.Integrated.1-1

racadm storage clearconfig:RAID.Integrated.1-1

racadm storage patrolread:RAID.Integrated.1-1 -state start

racadm storage importconfig:RAID.Integrated.1-1

racadm storage hotspare:Disk.Bay.0:Enclosure.Internal.0-0:RAID.Integrated.1-1 -assign yes -type dhs -vdkey:Disk.Virtual.0:RAID.Integrated.1-1

racadm storage init:Disk.Virtual.0:RAID.Integrated.1-1 -speed fast }

racadm storage createvd:RAID.Integrated.1-1 -rl r0 -wp wt -rp nra -name raid_0 -pdkey:Disk.Bay.0:Enclosure.Internal.0-0:RAID.Integrated.1-1,Disk.Bay.1:Enclosure.Internal.0-0:RAID.Integrated.1-1

racadm storage createsecuritykey:RAID.Integrated.1-1 -key <Key id> -passwd <passphrase>

racadm storage modifysecuritykey:RAID.Integrated.1-1 -key <Key id> -oldpasswd <old passphrase> -newpasswd <new passphrase>

racadm storage deletesecuritykey:RAID.Integrated.1-1

racadm storage encryptvd:Disk.Virtual.0:RAID.Integrated.1-1

racadm storage converttoraid:Disk.Bay.0:Enclosure.Internal.0-0:RAID.Integrated.1-1

racadm storage converttononraid:Disk.Bay.0:Enclosure.Internal.0-0:RAID.Integrated.1-1

racadm storage blink:Disk.Bay.0:Enclosure.Internal.0-0:RAID.Integrated.1-1

racadm storage unblink:Disk.Bay.0:Enclosure.Internal.0-0:RAID.Integrated.1-1

racadm storage preparetoremove:Disk.Bay.6:Enclosure.Internal.0-1:PCIeExtender.Slot.1

racadm storage secureerase:Disk.Bay.6:Enclosure.Internal.0-1:PCIeExtender.Slot.1

racadm storage ccheck:Disk.Virtual.0:RAID.Integrated.1-1

racadm storage cancelcheck:Disk.Virtual.0:RAID.Integrated.1-1

For help on viewing the properties of a group, run the command - racadm help get

For help on configuring the properties of a group, run the command - racadm help set

-----------------------------------------------------------------------

[root@ds-gp-prd-sdw7 ~]#

[root@ds-gp-prd-sdw7 ~]# racadm storage get controllers:RAID.Slot.3-1

RAID.Slot.3-1

Status                           = Ok

DeviceDescription                = RAID Controller in Slot 3

RollupStatus                     = Warning

Name                             = PERC H730P Adapter (PCI Slot 3)

PciSlot                          = 3

FirmwareVersion                  = 25.3.0.0016

DriverVersion                    = --NA--

RebuildRate                      = 30

BgiRate                          = 30

CheckConsistencyRate             = 30

ReconstructRate                  = 30

PatrolReadRate                   = 30

PatrolReadMode                   = Automatic

PatrolReadState                  = Stopped

CheckConsistencyMode             = Normal

LoadBalanceSetting               = Auto

CopybackMode                     = ON

PreservedCache                   = Not Present

CacheMemorySize                  = 2048 MB

PersistHotspare                  = Disabled

SpindownUnconfiguredDrives       = Disabled

SpindownHotspare                 = Disabled

Timeintervalforspindown          = 30 (Minutes)

SecurityStatus                   = Encryption Capable

EncryptionMode                   = None

SasAddress                       = 0x544A842031B66100

PciDeviceId                      = 0x5d

PciSubdeviceId                   = 0x1f42

PciVendorId                      = 0x1000

PciSubvendorId                   = 0x1028

PciBus                           = 0x83

PciDevice                        = 0x0

PciFunction                      = 0x0

BusWidth                         = 8x or x8

SlotLength                       = Long Length

SlotType                         = PCI Express Gen3 x16

MaxCapableSpeed                  = 12.0 Gb/s

LearnMode                        = Not supported

T10PICapability                  = Capable

SupportRAID10UnevenSpans         = Supported

SupportEnhancedAutoForeignImport = Supported

EnhancedAutoImportForeignConfig  = Disabled

SupportControllerBootMode        = Supported

ControllerBootMode               = Continue Boot On Error

RealtimeConfigurationCapability  = Capable

CurrentControllerMode            = RAID

[root@ds-gp-prd-sdw7 ~]#

http://web.mit.edu/cron/documentation/dell-server-admin/en/idrac1/chap05.htm

...

The default user name and password are root and calvin, respectively.

…

http://www.dell.com/support/

http://www.dell.com/support/contents/us/en/19/article/Product-Support/Self-support-Knowledgebase/locate-service-tag/Server-Storage

# Servers and storage products

How to find the service tag on a Dell server or storage system.

The Dell service tag is a seven character identifier that is unique to your product. In addition to the service tag, there is an Express Service Code which is a 10-digit numeric version of the service tag which can be typed into a telephone for call routing. While most Dell products have a service tag, some accessories and peripherals do not. As a general rule, desktop and notebook computers, servers and storage devices, printers, and mobile devices (such as phones or tablets) will have a service tag.

Enterprise server and storage computer systems include these model names: PowerEdge Server, PowerVault, and DellEMC Storage.

The service tag label will usually be found on the back panel of the case.

http://en.community.dell.com/techcenter/systems-management/w/wiki/3205.racadm-command-line-interface-for-drac

## RACADM Command Line Interface for DRAC

#### SYSTEMS MANAGEMENT - WIKI

# Overview

The Dell RACADM (Remote Access Controller Admin) utility is a command line tool that allows for remote or local management of Dell Servers via the iDRAC or DRAC. RACADM provides similar functionality to the iDRAC/DRAC Graphical User Interface (GUI). The Dell Chassis Management Controller (CMC) can also be managed remotely with RACADM.

RACADM commands can be run remotely from a management station and/or locally on the managed system.

RACADM commands allow you to view managed system information, perform power operations on the managed system, perform firmware updates, configure settings and more. Because RACADM is run from a command line interface (CLI), system administrators can create scripts that control and update Dell systems in a one-to-many fashion. See the links to the RACADM documentation at the end of this article for a complete listing of features for your product version.

# Terminology

Managed System - The Dell PowerEdge Server containing the iDRAC or DRAC, or the Dell Blade Chassis containing the Chassis Management Controller (CMC).

Management Station - A computer used for accessing the iDRAC, DRAC, or CMC remotely.

iDRAC - Integrated Dell Remote Access Controller. Available on newer and current generations of Dell PowerEdge Servers (11G and 12G).

DRAC - Dell Remote Access Controller. Available on older generations of Dell PowerEdge Servers.

# The Different RACADM Utilities

The following RACADM utilities are available:

- Remote RACADM - utility is installed on a management station.

- Local RACADM - utility is installed on the managed system.

- SSH/Telnet/Serial RACADM (also known as Firmware RACADM) - utility is built into the iDRAC, DRAC, and CMC firmware.

NOTES: Local RACADM is not required if you will be managing your server from a management station via Remote RACADM. Remote RACADM and Local RACADM cannot be installed at the same time on the same system.

# Locating and Installing RACADM Utilities

## Remote RACADM

Remote RACADM supports executing RACADM commands from a remote management station such as a laptop or desktop computer. It is available on the Dell OpenManage Systems Management Tools and Documentation DVD or at support.dell.com.

To download Remote RACADM:

- Go to http://support.dell.com

- If prompted, select Start Here under Small Businesses or Enterprise IT.

- Enter the Service Tag of the managed system and Submit, or use one of the Choose options to locate your managed system model.

- Select Drivers and Downloads > select the Operating System of your Management Station from the drop-down menu. Note: The drop-down menu only contains supported server operating systems. If you are running a supported Windows management station OS (Windows 7 for example), choose a Windows Server version from the drop-down menu instead.

- Expand Systems Management

  - For Windows Operating Systems, locate the DRAC Tools application.

  - For Linux operating systems, locate the Linux Remote Access Utilities application.

- Click Download File.

Remote RACADM Download - Windows Operating System

Remote RACADM Download - Linux Operating System

iDRAC 10.219.9.57 - 10.219.9.64

10.219.9.60 - PowerEdge R730xd

[support@centos00 ~]$ ssh root@10.219.9.60

The authenticity of host '10.219.9.60 (10.219.9.60)' can't be established.

RSA key fingerprint is 6d:02:dd:b3:92:c6:32:02:63:83:4b:51:7f:0c:36:44.

Are you sure you want to continue connecting (yes/no)? yes

Warning: Permanently added '10.219.9.60' (RSA) to the list of known hosts.

root@10.219.9.60's password:

WARNING: Default password is configured. Dell highly recommends changing user root's password immediately

/admin1-> help

[Usage]

show   [<options>] [<target>] [<properties>]

[<propertyname>== <propertyvalue>]

set    [<options>] [<target>] <propertyname>=<value>

cd     [<options>] [<target>]

create [<options>] <target> [<property of new target>=<value>]

[<property of new target>=<value>]

delete [<options>] <target>

exit   [<options>]

reset  [<options>] [<target>]

start  [<options>] [<target>]

stop   [<options>] [<target>]

version [<options>]

help   [<options>] [<help topics>]

load -source <URI> [<options>] [<target>]

dump -destination <URI> [<options>] [<target>]

/admin1->

/admin1-> help dump

Description:

The dump command is used to take a binary image from an ME and sent it

to a specific location.

Syntax:

dump -destination <URI> [<options>] [<target>]

Options:

-destination <URI>

The option destination tells the implementation the target to which it

will transfer the binary image. The desired destination is specified as

the argument to the destination option.

-x, -examine

The examine option causes the Command Processor to examine

command syntax only and provide feedback to the user on the

command's validity and correctness.

-f, -force

Instructs verb to ignore warning condition that would prevent execution

by default.

-h, -help

The help option causes the Command Processor to return text

describing the proper use of the verb entered as the first term on the

command line.

-k, -keep

The keep option is used to request the Command Processor to retain

status information for the job spawned in response to the command

with which the option was included. In order to use the keep option to

request that status for a command be preserved, it must be included as

part of the initial command request.

-o, -output

The output option controls the format of output returned by the

Command Processor to the Client. This option has no effect on the

execution of the command or the Managed Elements affected by the

command.

-v, -version

The version option causes the Command Processor to return the

version of  the command verb that apprears as the first term on the

command line.

-w, -wait

The wait option causes the Command Processor to not return the

control immediately to the Client, instead the Command Processor will

withhold all Command Response data and command input control

until all jobs related to this command have completed. The wait option

has no effect on the success or  failure of the command or spawned

jobs.

/admin1->

/admin1-> show -a

/admin1

associations

targets

hdwr1

profiles1

system1

verbs

cd

show

help

version

/admin1->

/admin1-> version -v

version of version is: 1.0.2

/admin1->

/admin1-> racadm

racadm>>help

racadm help

racadm help

help [subcommand]    -- display usage summary for a subcommand

arp                  -- display the networking ARP table

autoupdatescheduler  -- Automatic Platform Update of the devices on the server.

clearasrscreen       -- clear the last ASR (crash) screen

clearpending         -- clear pending attribute(s) value of a Device Class

closessn             -- close a session

clrsel               -- clear the System Event Log (SEL)

config               -- Deprecated: modify RAC configuration properties

coredump             -- display the last RAC coredump

coredumpdelete       -- delete the last RAC coredump

debug                -- Field Service Debug Authorization facility commands

eventfilters         -- Alerts configuration commands

fwupdate             -- update the RAC firmware

get                  -- display RAC configuration properties

getconfig            -- Deprecated: display RAC configuration properties

gethostnetworkinterfaces -- Display host network interface details

getled               -- Get the state of the LED on a module.

getniccfg            -- display current network settings

getraclog            -- display the RAC log

getractime           -- display the current RAC time

getsel               -- display records from the System Event Log (SEL)

getsensorinfo        -- display system sensors

getssninfo           -- display session information

getsvctag            -- display service tag information

getsysinfo           -- display general RAC and system information

gettracelog          -- display the RAC diagnostic trace log

getuscversion        -- Deprecated: display the current USC version details

getversion           -- display the current version details

ifconfig             -- display network interface information

inlettemphistory     -- inlet temperature history operations

license              -- License Manager commands

lclog                -- LCLog operations

frontpanelerror      -- hide LCD errors - color amber to blue

netstat              -- display routing table and network statistics

ping                 -- send ICMP echo packets on the network

ping6                -- send ICMP echo packets on the network

racdump              -- display RAC diagnostic information

racreset             -- perform a RAC reset operation

racresetcfg          -- restore the RAC configuration to factory defaults

remoteimage          -- make a remote ISO image available to the server

rollback             -- Rollback firmware to its previous version.

serveraction         -- perform system power management operations

set                  -- modify RAC configuration properties

setled               -- Set the state of the LED on a module.

setniccfg            -- modify network configuration properties

sshpkauth            -- manage SSH PK authentication keys on the RAC

sslcertdelete        -- delete an SSL certificate on the iDRAC

sslcertview          -- view SSL certificate information

sslcsrgen            -- generate a certificate CSR from the RAC

sslEncryptionStrength -- Deprecated: Display or modify the SSL Encryption strength.

sslresetcfg          -- Reset iDRAC to apply new certificate. Until iDRAC is reset old certificate will be active.

swinventory          -- Display the list of S/W Installed on the server.

systemconfig         -- Backup &/or Restore of iDRAC Config and Firmware

systemerase          -- Performs system erase on a selected component.

testemail            -- test RAC e-mail notifications

testtrap             -- test RAC SNMP trap notifications

testalert            -- test RAC SNMP - FQDN trap notifications

traceroute           -- print the route packets trace to network host

traceroute6          -- print the route packets trace to network host

techsupreport        -- Tech Support Report operations.

usercertview         -- view user certificate information

vflashpartition      -- manage partitions on the vFlash SD card

vflashsd             -- perform vFlash SD Card initialization

vmdisconnect         -- disconnect Virtual Media connections

raid                 -- Monitoring and Inventory of H/W RAID connected to the server.

storage              -- Monitoring and Inventory of H/W RAID connected to the server.

hwinventory          -- Monitoring and Inventory of H/W NICs connected to the server.

nicstatistics        -- Statistics for NICs connected to the server.

fcstatistics         -- Statistics for FCs connected to the server.

update               -- Platform Update of the devices on the server

jobqueue             -- Jobqueue of of the jobs currently scheduled

sensorsettings       -- Set the sensor threshold levels.

diagnostics          -- Remote Diagnostic commands

SystemPerfStatistics -- Display or Modify System Performance Statistics

Groups

BIOS                -- Configuration of BIOS attributes

iDRAC               -- Configuration of iDRAC attributes

LifecycleController -- Configuration of LifecycleController attributes

Nic                 -- Configuration of NIC attributes

Storage             -- Configuration of Storage attributes

System              -- Configuration of System attributes

FC                  -- Configuration of Fiber Channel attributes

For Help on configuring the properties of a group - racadm help set

-----------------------------------------------------------------------

racadm>>

racadm>>raid get vdisks

racadm raid get vdisks

Disk.Virtual.0:RAID.Integrated.1-1

racadm>>raid get pdisks

racadm raid get pdisks

Disk.Bay.0:Enclosure.Internal.0-1:RAID.Integrated.1-1

Disk.Bay.1:Enclosure.Internal.0-1:RAID.Integrated.1-1

Disk.Bay.2:Enclosure.Internal.0-1:RAID.Integrated.1-1

Disk.Bay.3:Enclosure.Internal.0-1:RAID.Integrated.1-1

Disk.Bay.4:Enclosure.Internal.0-1:RAID.Integrated.1-1

Disk.Bay.5:Enclosure.Internal.0-1:RAID.Integrated.1-1

Disk.Bay.6:Enclosure.Internal.0-1:RAID.Integrated.1-1

Disk.Bay.7:Enclosure.Internal.0-1:RAID.Integrated.1-1

Disk.Bay.8:Enclosure.Internal.0-1:RAID.Integrated.1-1

Disk.Bay.9:Enclosure.Internal.0-1:RAID.Integrated.1-1

Disk.Bay.10:Enclosure.Internal.0-1:RAID.Integrated.1-1

Disk.Bay.11:Enclosure.Internal.0-1:RAID.Integrated.1-1

Disk.Bay.12:Enclosure.Internal.0-1:RAID.Integrated.1-1

Disk.Bay.13:Enclosure.Internal.0-1:RAID.Integrated.1-1

racadm>>

racadm>>getconfig -h

racadm getconfig -h

idRacInfo            -- Information about iDRAC being queried

cfgRemoteHosts       -- Properties for configuration of the SMTP server

cfgUserAdmin         -- Information about iDRAC users

cfgEmailAlert        -- Parameters to configure e-mail alerting capabilities

cfgSessionManagement -- Information of the session Properties

cfgSerial            -- Provides configuration parameters for the iDRAC

cfgOobSnmp           -- Configuration of the SNMP agent and trap capabilities

cfgRacTuning         -- Configuration for various iDRAC properties.

ifcRacManagedNodeOs  -- Properties of the managed server OS

cfgRacSecurity       -- Configure SSL certificate signing request settings

cfgRacVirtual        -- Configuration Properties for iDRAC Virtual Media

cfgActiveDirectory   -- Configuration of the iDRAC Active Directory feature

cfgLDAP              -- Configuration properties for LDAP settings

cfgLdapRoleGroup     -- Configuration of role groups for LDAP

cfgLogging           -- Group Description for group cfgLogging

cfgStandardSchema    -- Configuration of AD standard schema settings

cfgIpmiSerial        -- Properties to configure the IPMI serial interface

cfgIpmiSol           -- Configuration the SOL capabilities of the system

cfgIpmiLan           -- Configuration the IPMI over LAN of the system

cfgIpmiPef           -- Configuration the platform event filters

cfgServerPower       -- Provides power management features

cfgServerPowerSupply -- Provides information related to the power supplies

cfgVFlashSD          -- Configure the properties for the vFlash SD card

cfgVFlashPartition   -- Configure partitions on the vFlash SD Card

cfgUserDomain        -- Configure the Active Directory user domain names

cfgSmartCard         -- Properties to access iDRAC using a smart card

cfgServerInfo        -- Configuration of first boot device

cfgSensorRedundancy  -- Configure the power supply redundancy

cfgLanNetworking     -- Parameters to configure the iDRAC NIC

cfgStaticLanNetworking -- Parameters to configure the iDRAC NIC

cfgNetTuning         -- Group Description for group cfgNetTuning

cfgIPv6LanNetworking -- Configuration of the IPv6 over LAN networking

cfgIPv6StaticLanNetworking -- Configuration of the IPv6 over LAN networking

cfgIPv6URL           -- Configuration of the iDRAC IPv6 URL.

racadm>>

racadm getconfig -g  cfgServerInfo

cfgServerBootOnce=1

cfgServerFirstBootDevice=No-Override

RAC1168: The RACADM "getconfig" command will be deprecated in a

future version of iDRAC firmware. Run the RACADM

"racadm get" command to retrieve the iDRAC configuration parameters.

For more information on the get command, run the RACADM command

"racadm help get".

racadm>>

racadm>>help get

racadm help get

get -- display RAC configuration properties

Usage:

racadm get <fqdd alias>.<group>

racadm get <fqdd alias>.<group>.<object>

racadm get <fqdd alias>.<group>.<index>.<object>

racadm get -f <filename> -t <filetype> -u <username> -p <password>

-l <CIFS share> -c <FQDD>

racadm get -f <filename> -t <filetype> -l <NFS share> -c <FQDD>

-------------------------------------------------------------------------------

Valid Options:

-f : save RAC configuration to a file

-u : Username of the remote share to where the file must be exported.

-p : Password for the remote share to where the file must be exported.

-l : Network share location to where the file must be exported.

For more information on NFS or CIFS share, see section on Usage examples.

-c : Specifies the FQDD of the components for which the configurations

are to be exported. If this option is not specified,

configuration related to all the components are exported.

-t : Specifies the file type to be exported. Valid values are "xml" and "ini".

These are case-insensitive. ini exports the legacy configuration file.

The ini file cannot be exported to a remote share.  If -t is not specified,

the legacy file is exported.

--clone   : Export Cloned configuration file. Supports xml file format only.

Configuration file can be exported to remote or local share.

VD Creation Action and Size will be uncommented.

--replace : Export Replaced configuration file. Supports xml file format only.

Configuration file can be exported to remote or local share.

VD Creation Action and Size will be uncommented, along with server I/O

identity like NIC/FC properties are uncommented.

--includero : Include read only attributes in the configuration XML file.

--includeph : Include password hash attributes in the configuration XML file.

-------------------------------------------------------------------------------

Usage Examples:

- Display an entire group, in this case the topology configuration:

racadm get system.location

- Display a single object from a particular group:

racadm get system.location.rack.name

- Display an indexed group:

racadm get system.power.supply.1

- Export the xml configuration to a CIFS share:

racadm get -f file -t xml -u myuser -p mypass -l //10.1.12.13/share

- Export the xml configuration of the iDRAC component to a CIFS share:

racadm get -f file -t xml -u myuser -p mypass -l //10.1.12.13/share

-c iDRAC.Embedded.1

- Export the xml configuration to an NFS share:

racadm get -f file -t xml -l 10.1.12.13:/myshare

-------------------------------------------------------------------------------

For help on viewing the properties of a group, run the command - racadm help get

For help on configuring the properties of a group, run the command - racadm help set

-----------------------------------------------------------------------

racadm>>

racadm>>get system

racadm get system

Backplane

LCD

Location

Power

Power.Supply

QuickSync

ServerOS

ThermalConfig

ThermalSettings

racadm>>

[support@centos00 ~]$ sshpass -p "${IDRACPASS}" ssh root@10.219.9.60 racadm getconfig -g cfgServerInfo

cfgServerBootOnce=1

cfgServerFirstBootDevice=No-Override

RAC1168: The RACADM "getconfig" command will be deprecated in a

future version of iDRAC firmware. Run the RACADM

"racadm get" command to retrieve the iDRAC configuration parameters.

For more information on the get command, run the RACADM command

"racadm help get".

WARNING: Default password is configured. Dell highly recommends changing user root's password immediately

[support@centos00 ~]$

[support@centos00 ~]$ sshpass -p "${IDRACPASS}" ssh root@10.219.9.60 racadm get system.serveros

[Key=system.Embedded.1#ServerOS.1]

HostName=

OSName=

#OSVersion=

#ServerPoweredOnTime=525311

WARNING: Default password is configured. Dell highly recommends changing user root's password immediately

[support@centos00 ~]$

[support@centos00 ~]$ sshpass -p "${IDRACPASS}" ssh root@10.219.9.60 racadm getsysinfo

RAC Information:

RAC Date/Time           = Wed Aug  5 16:40:34 2015

Firmware Version        = 2.15.10.10

Firmware Build          = 08

Last Firmware Update    = 06/30/2015 17:40:49

Hardware Version        = 0.01

MAC Address             = 18:FB:7B:9D:AC:3F

Common settings:

Register DNS RAC Name   = 0

DNS RAC Name            = idrac

Current DNS Domain      =

Domain Name from DHCP   = Disabled

IPv4 settings:

Enabled                 = 1

Current IP Address      = 10.219.9.60

Current IP Gateway      = 10.219.9.254

Current IP Netmask      = 255.255.255.0

DHCP Enabled            = 0

Current DNS Server 1    = 0.0.0.0

Current DNS Server 2    = 0.0.0.0

DNS Servers from DHCP   = Disabled

IPv6 settings:

Enabled                 = 0

Current IP Address 1    = ::

Current IP Gateway      = ::

Autoconfig              = 1

Link Local IP Address   = ::

Current IP Address 2    = ::

Current IP Address 3    = ::

Current IP Address 4    = ::

Current IP Address 5    = ::

Current IP Address 6    = ::

Current IP Address 7    = ::

Current IP Address 8    = ::

Current IP Address 9    = ::

Current IP Address 10   = ::

Current IP Address 11   = ::

Current IP Address 12   = ::

Current IP Address 13   = ::

Current IP Address 14   = ::

Current IP Address 15   = ::

DNS Servers from DHCPv6 = Disabled

Current DNS Server 1    = ::

Current DNS Server 2    = ::

System Information:

System Model            = PowerEdge R730xd

System Revision         = I

System BIOS Version     = 1.3.6

Service Tag             = 4P7V762

Express Svc Code        = 10231996682

Host Name               =

OS Name                 =

OS Version              =

Power Status            = ON

Fresh Air Capable       = Yes

Watchdog Information:

Recovery Action         = None

Present countdown value = 15 seconds

Initial countdown value = 15 seconds

Embedded NIC MAC Addresses:

NIC.Integrated.1-1-1    Ethernet                = 44:A8:42:32:7F:EE

iSCSI                   = 44:A8:42:32:7F:EF

FIP                     = 44:A8:42:32:7F:EF

WWN                     = 20:00:44:A8:42:32:7F:EF

WWPN                    = 20:01:44:A8:42:32:7F:EF

NIC.Integrated.1-2-1    Ethernet                = 44:A8:42:32:7F:F0

iSCSI                   = 44:A8:42:32:7F:F1

FIP                     = 44:A8:42:32:7F:F1

WWN                     = 20:00:44:A8:42:32:7F:F1

WWPN                    = 20:01:44:A8:42:32:7F:F1

NIC.Integrated.1-3-1    Ethernet                = 44:A8:42:32:7F:F2

iSCSI                   = 44:A8:42:32:7F:F3

FIP                     = 44:A8:42:32:7F:F3

WWN                     = 20:00:44:A8:42:32:7F:F3

WWPN                    = 20:01:44:A8:42:32:7F:F3

NIC.Integrated.1-4-1    Ethernet                = 44:A8:42:32:7F:F4

iSCSI                   = 44:A8:42:32:7F:F5

FIP                     = 44:A8:42:32:7F:F5

WWN                     = 20:00:44:A8:42:32:7F:F5

WWPN                    = 20:01:44:A8:42:32:7F:F5

WARNING: Default password is configured. Dell highly recommends changing user root's password immediately

[support@centos00 ~]$

[support@centos00 ~]$ sshpass -p "${IDRACPASS}" ssh root@10.219.9.60 racadm get idrac.info

[Key=idrac.Embedded.1#Info.1]

#Build=08

#Description=This system component provides a complete set of remote management functions for Dell PowerEdge Servers

#Name=idrac

#Product=Integrated Dell Remote Access Controller

#ServerGen=13G

#Type=32

#Version=2.15.10.10

WARNING: Default password is configured. Dell highly recommends changing user root's password immediately

[support@centos00 ~]$

[support@centos00 6_init_bkp]$ sshpass -p "${IDRACPASS}" ssh root@10.219.9.60 racadm getsvctag

4P7V762

[support@centos00 6_init_bkp]$

[support@centos00 6_init_bkp]$ sshpass -p "${IDRACPASS}" ssh root@10.219.9.60 racadm racdump

===============================================================================

General System/RAC Information

===============================================================================

RAC Information:

RAC Date/Time           = Thu Aug  6 14:19:16 2015

Firmware Version        = 2.15.10.10

Firmware Build          = 08

Last Firmware Update    = 06/30/2015 17:40:49

Hardware Version        = 0.01

MAC Address             = 18:FB:7B:9D:AC:3F

Common settings:

Register DNS RAC Name   = 0

DNS RAC Name            = idrac

Current DNS Domain      =

Domain Name from DHCP   = Disabled

IPv4 settings:

Enabled                 = 1

Current IP Address      = 10.219.9.60

Current IP Gateway      = 10.219.9.254

Current IP Netmask      = 255.255.255.0

1852 root      7308 S    /usr/sbin/sfcbd -d

1908 root     20316 S    /sbin/avct_server

1911 root      7152 S    /sbin/vkvm_pm

1922 root      6488 S    /sbin/sshd -g 60

2044 root     17432 S    /bin/maserserver

2047 root      3752 S    {cfgbkup.sh} /bin/sh /etc/sysapps_script/cfgscripts/

2051 root      9232 S    /bin/mctpd

2074 root      4700 S    /usr/sbin/mrcached

2095 root      3752 S    /sbin/crond -b

2116 root     30296 S    /usr/local/bin/appweb --config /var/run/appweb.conf

2216 root     29868 R    /usr/sbin/ipmiextd

2221 root     23344 S    /usr/sbin/dsm_sa_popproc iracpop

2323 root     16688 S    /usr/sbin/dsm_sa_popproc ienvpop

2340 root     15464 S    /usr/sbin/dsm_sa_popproc hwipop

2346 root      8884 S    /usr/sbin/dsm_sa_popproc jcpop

2351 root     23232 S    /usr/sbin/dsm_sa_popproc hiipop

2357 root     21536 S    /usr/sbin/dsm_sa_popproc raidpop

2374 root     12560 S    /usr/sbin/dsm_sa_popproc proxypop

2379 root     14720 S    /usr/sbin/dsm_sa_popproc reutlpop

2513 root      8336 S    /usr/sbin/snmpd -p /var/run/snmpd.pid -c /var/run/sn

2637 root      3824 S    {mut_collect_nig} /bin/sh /etc/sysapps_script/mut_co

2640 root      4944 S    /usr/bin/cimmond

2646 root     11548 S    /bin/usb_app

2659 root      6128 S    /bin/ttymonitor

2679 root      3752 S    getty -L ttySC3 57600 vt100 -H AVOCENT

3849 root      7524 S    /usr/bin/idracmonitor

5674 root     10220 S    /sbin/avct_wsman -r0 -m

5676 root     15544 S    /sbin/avct_wsman -r0 -m

7005 root     27380 S    /usr/sbin/sfcbd -d

10551 root     17656 S    /usr/sbin/sfcbd -d

14754 root         0 SW   [kworker/0:0]

18914 root     34960 S    /usr/local/bin/guiDataServer

19881 root         0 SW   [kworker/0:2]

21129 root      4944 S    /usr/bin/cimmond

23206 root      3692 S    sleep 3600

24591 root         0 SW   [kworker/u:1]

25600 root         0 SW   [kworker/u:3]

26466 root     19504 S    sshd: racuser@notty

26486 root     11612 S    clpd -c racadm racdump

26491 root     11172 S    /usr/sbin/sfcbd -d

26495 root     33088 S    /bin/racadm racdump

26506 root      3620 S    sleep 5

26513 root      3692 S    sleep 1

26514 root      3752 S    sh -c /bin/ps

26515 root      3756 R    /bin/ps

===============================================================================

RAC Firmware Build Log

===============================================================================

BLD_TAG=idracfw_bldtag_2.15.10.10_691231_1800_00

BLD_VERSION=2.15.10.10

BLD_NUMBER=69.12.31

BLD_DATE=2.10.10.10.300

BLD_TYPE=idrac

BLD_KERNEL=ZIMAGE

[support@centos00 6_init_bkp]$

1852 root      7308 S    /usr/sbin/sfcbd -d

1908 root     20316 S    /sbin/avct_server

1911 root      7152 S    /sbin/vkvm_pm

1922 root      6488 S    /sbin/sshd -g 60

2044 root     17432 S    /bin/maserserver

2047 root      3752 S    {cfgbkup.sh} /bin/sh /etc/sysapps_script/cfgscripts/

2051 root      9232 S    /bin/mctpd

2074 root      4700 S    /usr/sbin/mrcached

2095 root      3752 S    /sbin/crond -b

2116 root     30296 S    /usr/local/bin/appweb --config /var/run/appweb.conf

2216 root     29868 R    /usr/sbin/ipmiextd

2221 root     23344 S    /usr/sbin/dsm_sa_popproc iracpop

2323 root     16688 S    /usr/sbin/dsm_sa_popproc ienvpop

2340 root     15464 S    /usr/sbin/dsm_sa_popproc hwipop

2346 root      8884 S    /usr/sbin/dsm_sa_popproc jcpop

2351 root     23232 S    /usr/sbin/dsm_sa_popproc hiipop

2357 root     21536 S    /usr/sbin/dsm_sa_popproc raidpop

2374 root     12560 S    /usr/sbin/dsm_sa_popproc proxypop

2379 root     14720 S    /usr/sbin/dsm_sa_popproc reutlpop

2513 root      8336 S    /usr/sbin/snmpd -p /var/run/snmpd.pid -c /var/run/sn

2637 root      3824 S    {mut_collect_nig} /bin/sh /etc/sysapps_script/mut_co

2640 root      4944 S    /usr/bin/cimmond

2646 root     11548 S    /bin/usb_app

2659 root      6128 S    /bin/ttymonitor

2679 root      3752 S    getty -L ttySC3 57600 vt100 -H AVOCENT

3849 root      7524 S    /usr/bin/idracmonitor

5674 root     10220 S    /sbin/avct_wsman -r0 -m

5676 root     15544 S    /sbin/avct_wsman -r0 -m

7005 root     27380 S    /usr/sbin/sfcbd -d

10551 root     17656 S    /usr/sbin/sfcbd -d

14754 root         0 SW   [kworker/0:0]

18914 root     34960 S    /usr/local/bin/guiDataServer

19881 root         0 SW   [kworker/0:2]

21129 root      4944 S    /usr/bin/cimmond

23206 root      3692 S    sleep 3600

24591 root         0 SW   [kworker/u:1]

25600 root         0 SW   [kworker/u:3]

26466 root     19504 S    sshd: racuser@notty

26486 root     11612 S    clpd -c racadm racdump

26491 root     11172 S    /usr/sbin/sfcbd -d

26495 root     33088 S    /bin/racadm racdump

26506 root      3620 S    sleep 5

26513 root      3692 S    sleep 1

26514 root      3752 S    sh -c /bin/ps

26515 root      3756 R    /bin/ps

===============================================================================

RAC Firmware Build Log

===============================================================================

BLD_TAG=idracfw_bldtag_2.15.10.10_691231_1800_00

BLD_VERSION=2.15.10.10

BLD_NUMBER=69.12.31

BLD_DATE=2.10.10.10.300

BLD_TYPE=idrac

BLD_KERNEL=ZIMAGE

[support@centos00 6_init_bkp]$

[support@centos00 6_init_bkp]$ sshpass -p "${IDRACPASS}" ssh root@10.219.9.60 racadm getraclog

SeqNumber       = 144

Message ID      = USR0030

Category        = Audit

AgentID         = RACLOG

Severity        = Information

Timestamp       = 2015-08-06 14:36:33

Message         = Successfully logged in using root, from 172.16.28.74 and SSH.

Message Arg   1 = root

Message Arg   2 = 172.16.28.74

Message Arg   3 = SSH

FQDD            = iDRAC.Embedded.1

--------------------------------------------------------------------------------

...

[support@centos00 6_init_bkp]$ sshpass -p "${IDRACPASS}" ssh root@10.219.9.60 racadm gettracelog

Record:      1

Date/Time:   Aug  5 20:09:24

Source:      syslogd 1.4.2

Description: restart.

-------------------------------------------------------------------------------

Record:      2

Date/Time:   Aug  6 07:45:55

Source:      appweb

Description: pam_ldap_manager(webgui:auth): Got username root ----------->

-------------------------------------------------------------------------------

...

-------------------------------------------------------------------------------

Record:      186

Date/Time:   Aug  6 14:36:45

Source:      sshd[29463]

Description: Received disconnect from 172.16.28.74: 11: disconnected by user

-------------------------------------------------------------------------------

Record:      187

Date/Time:   Aug  6 14:36:45

Source:      sshd[29463]

Description: pam_auth_status(sshd:session): unknown option: sessiontype=SSH

-------------------------------------------------------------------------------

Record:      188

Date/Time:   Aug  6 14:36:45

Source:      sshd[29463]

Description: pam_auth_status(sshd:session): User=root closing session from host=172.16.28.74

-------------------------------------------------------------------------------

Record:      189

Date/Time:   Aug  6 14:36:45

Source:      aim[1036]

Description: removeChildSessionFromParent() - could not open /tmp/aim/dynamic/sessions/0

-------------------------------------------------------------------------------

[support@centos00 6_init_bkp]$

[support@centos00 6_init_bkp]$ sshpass -p "${IDRACPASS}" ssh root@10.219.9.60 racadm sslcertview -t 1

Serial Number            : 9F4E16FBB8853F8F

Subject Information:

Country Code (CC)        : US

State (S)                : Texas

Locality (L)             : Round Rock

Organization (O)         : Dell Inc.

Organizational Unit (OU) : Remote Access Group

Common Name (CN)         : idrac

Issuer Information:

Country Code (CC)        : US

State (S)                : Texas

Locality (L)             : Round Rock

Organization (O)         : Dell Inc.

Organizational Unit (OU) : Remote Access Group

Common Name (CN)         : idrac

Valid From               : Jun 29 22:42:00 2015 GMT

Valid To                 : Jun 30 22:42:00 2025 GMT

[support@centos00 6_init_bkp]$

[root@centos00 yum.repos.d]# curl http://repo-linux.tcsbank.ru/tcs-dell.repo.sh | sh

% Total    % Received % Xferd  Average Speed   Time    Time     Time  Current

Dload  Upload   Total   Spent    Left  Speed

153   153  153   153    0     0   4616      0 --:--:-- --:--:-- --:--:--  9000

[root@centos00 yum.repos.d]#

[root@centos00 yum.repos.d]# yum search all racadm

Loaded plugins: downloadonly, refresh-packagekit, security

============================================================================================= Matched: racadm ==============================================================================================

srvadmin-idracadm.x86_64 : iDRAC6 Command Interface, 8.1.0

srvadmin-idracadm7.x86_64 : iDRAC7 Command Interface, 8.1.0

srvadmin-racadm4.x86_64 : RAC Command Interface, 7.1.0

srvadmin-racadm5.x86_64 : RAC5 Command Interface, 7.1.0

[root@centos00 yum.repos.d]#

[root@centos00 yum.repos.d]# yum install srvadmin-idracadm7.x86_64

Loaded plugins: downloadonly, refresh-packagekit, security

Setting up Install Process

Resolving Dependencies

--> Running transaction check

---> Package srvadmin-idracadm7.x86_64 0:8.1.0-4.69.4.el6 will be installed

--> Processing Dependency: srvadmin-omilcore for package: srvadmin-idracadm7-8.1.0-4.69.4.el6.x86_64

--> Processing Dependency: libargtable2.so.0()(64bit) for package: srvadmin-idracadm7-8.1.0-4.69.4.el6.x86_64

--> Running transaction check

---> Package srvadmin-argtable2.x86_64 0:8.1.0-4.5.1.el6 will be installed

---> Package srvadmin-omilcore.x86_64 0:8.1.0-4.85.1.el6 will be installed

--> Processing Dependency: smbios-utils-bin for package: srvadmin-omilcore-8.1.0-4.85.1.el6.x86_64

--> Running transaction check

---> Package smbios-utils-bin.x86_64 0:2.2.27-4.4.1.el6 will be installed

--> Processing Dependency: libsmbios = 2.2.27-4.4.1.el6 for package: smbios-utils-bin-2.2.27-4.4.1.el6.x86_64

--> Processing Dependency: libsmbios_c.so.2()(64bit) for package: smbios-utils-bin-2.2.27-4.4.1.el6.x86_64

--> Processing Dependency: libsmbios.so.2()(64bit) for package: smbios-utils-bin-2.2.27-4.4.1.el6.x86_64

--> Running transaction check

---> Package libsmbios.x86_64 0:2.2.27-4.4.1.el6 will be installed

--> Finished Dependency Resolution

Dependencies Resolved

============================================================================================================================================================================================================

Package                                                Arch                                       Version                                               Repository                                    Size

============================================================================================================================================================================================================

Installing:

srvadmin-idracadm7                                     x86_64                                     8.1.0-4.69.4.el6                                      tcs-dell                                     701 k

Installing for dependencies:

libsmbios                                              x86_64                                     2.2.27-4.4.1.el6                                      tcs-dell                                     2.0 M

smbios-utils-bin                                       x86_64                                     2.2.27-4.4.1.el6                                      tcs-dell                                     119 k

srvadmin-argtable2                                     x86_64                                     8.1.0-4.5.1.el6                                       tcs-dell                                      58 k

srvadmin-omilcore                                      x86_64                                     8.1.0-4.85.1.el6                                      tcs-dell                                      29 k

Transaction Summary

============================================================================================================================================================================================================

Install       5 Package(s)

Total download size: 2.9 M

Installed size: 11 M

Is this ok [y/N]: y

Downloading Packages:

(1/5): libsmbios-2.2.27-4.4.1.el6.x86_64.rpm                                                                                                                                         | 2.0 MB     00:00

(2/5): smbios-utils-bin-2.2.27-4.4.1.el6.x86_64.rpm                                                                                                                                  | 119 kB     00:00

(3/5): srvadmin-argtable2-8.1.0-4.5.1.el6.x86_64.rpm                                                                                                                                 |  58 kB     00:00

(4/5): srvadmin-idracadm7-8.1.0-4.69.4.el6.x86_64.rpm                                                                                                                                | 701 kB     00:00

(5/5): srvadmin-omilcore-8.1.0-4.85.1.el6.x86_64.rpm                                                                                                                                 |  29 kB     00:00

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Total

Running rpm_check_debug

Running Transaction Test

Transaction Test Succeeded

Running Transaction

Installing : srvadmin-argtable2-8.1.0-4.5.1.el6.x86_64                                                                                                                                                1/5

Installing : libsmbios-2.2.27-4.4.1.el6.x86_64                                                                                                                                                        2/5

Installing : smbios-utils-bin-2.2.27-4.4.1.el6.x86_64                                                                                                                                                 3/5

Installing : srvadmin-omilcore-8.1.0-4.85.1.el6.x86_64                                                                                                                                                4/5

**********************************************************

After the install process completes, you may need

to log out and then log in again to reset the PATH

variable to access the Server Administrator CLI utilities

**********************************************************

Installing : srvadmin-idracadm7-8.1.0-4.69.4.el6.x86_64                                                                                                                                               5/5

Verifying  : srvadmin-idracadm7-8.1.0-4.69.4.el6.x86_64                                                                                                                                               1/5

Verifying  : srvadmin-omilcore-8.1.0-4.85.1.el6.x86_64                                                                                                                                                2/5

Verifying  : smbios-utils-bin-2.2.27-4.4.1.el6.x86_64                                                                                                                                                 3/5

Verifying  : libsmbios-2.2.27-4.4.1.el6.x86_64                                                                                                                                                        4/5

Verifying  : srvadmin-argtable2-8.1.0-4.5.1.el6.x86_64                                                                                                                                                5/5

Installed:

srvadmin-idracadm7.x86_64 0:8.1.0-4.69.4.el6

Dependency Installed:

libsmbios.x86_64 0:2.2.27-4.4.1.el6          smbios-utils-bin.x86_64 0:2.2.27-4.4.1.el6          srvadmin-argtable2.x86_64 0:8.1.0-4.5.1.el6          srvadmin-omilcore.x86_64 0:8.1.0-4.85.1.el6

Complete!

[root@centos00 yum.repos.d]#

[root@centos00 yum.repos.d]# rpm -ql srvadmin-idracadm7

/etc/ld.so.conf.d/srvadmin-idrac7.conf

/opt/dell/srvadmin/bin/idracadm7

/opt/dell/srvadmin/sbin/racadm-wrapper-idrac7

[root@centos00 yum.repos.d]#

[support@centos00 ~]$ /opt/dell/srvadmin/bin/idracadm7 -r 10.219.9.60 -u root -p ${IDRACPASS} getsysinfo

Security Alert: Certificate is invalid - self signed certificate

Continuing execution. Use -S option for racadm to stop execution on certificate-related errors.

SEC0701: Warning: Default username and password are currently in use.

It is strongly recommended to change the default password immediately.

To change the password, run the following racadm command:

"racadm set iDRAC.Users.<index>.Password <Password>".

Alternatively, to avoid this warning message run the following racadm command:

"racadm set iDRAC.Tuning.DefaultCredentialWarning Disabled".

RAC Information:

RAC Date/Time           = Wed Aug  5 17:12:30 2015

Firmware Version        = 2.15.10.10

Firmware Build          = 08

Last Firmware Update    = 06/30/2015 17:40:49

Hardware Version        = 0.01

MAC Address             = 18:FB:7B:9D:AC:3F

...

Displays RAC log entries.

[support@centos00 6_init_bkp]$ /opt/dell/srvadmin/bin/idracadm7 -r 10.219.9.60 -u root -p ${IDRACPASS} getraclog

Security Alert: Certificate is invalid - self signed certificate

Continuing execution. Use -S option for racadm to stop execution on certificate-related errors.

ERROR: Failed to send the command to the firmware.

SeqNumber       = 174

Message ID      = USR0030

Category        = Audit

AgentID         = RACLOG

Severity        = Information

Timestamp       = 2015-08-06 18:02:46

Message         = Successfully logged in using root, from 172.16.28.74 and RACADM.

Message Arg   1 = root

Message Arg   2 = 172.16.28.74

Message Arg   3 = RACADM

FQDD            = iDRAC.Embedded.1

--------------------------------------------------------------------------------

SeqNumber       = 173

Message ID      = USR0032

Category        = Audit

AgentID         = RACLOG

Severity        = Information

Timestamp       = 2015-08-06 17:59:11

Message         = The session for root from 172.16.28.74 using RACADM is logged off.

Message Arg   1 = root

Message Arg   2 = 172.16.28.74

Message Arg   3 = RACADM

FQDD            = iDRAC.Embedded.1

--------------------------------------------------------------------------------

...

--------------------------------------------------------------------------------

SeqNumber       = 3

Message ID      = LOG007

Category        = Audit

AgentID         = DE

Severity        = Information

Timestamp       = 2015-06-30 21:25:23

Message         = The previous log entry was repeated 1 times.

Message Arg   1 = 1

--------------------------------------------------------------------------------

SeqNumber       = 2

Message ID      = SYS1003

Category        = Audit

AgentID         = DE

Severity        = Information

Timestamp       = 2015-06-30 21:24:46

Message         = System CPU Resetting.

FQDD            = iDRAC.Embedded.1#HostPowerCtrl

--------------------------------------------------------------------------------

[support@centos00 6_init_bkp]$

Displays System Event Log entries.

[support@centos00 6_init_bkp]$ /opt/dell/srvadmin/bin/idracadm7 -r 10.219.9.60 -u root -p ${IDRACPASS} getsel

Continuing execution. Use -S option for racadm to stop execution on certificate-related errors.

Record:      1

Date/Time:   06/30/2015 21:19:31

Source:      system

Severity:    Ok

Description: Log cleared.

-------------------------------------------------------------------------------

Record:      2

Date/Time:   06/30/2015 21:28:29

Source:      system

Severity:    Ok

Description: C: boot completed.

-------------------------------------------------------------------------------

Record:      3

Date/Time:   06/30/2015 21:28:29

Source:      system

Severity:    Ok

Description: OEM software event.

-------------------------------------------------------------------------------

[support@centos00 6_init_bkp]$

Displays Trace Log entries

[support@centos00 6_init_bkp]$ /opt/dell/srvadmin/bin/idracadm7 -r 10.219.9.60 -u root -p ${IDRACPASS} gettracelog

Security Alert: Certificate is invalid - self signed certificate

Continuing execution. Use -S option for racadm to stop execution on certificate-related errors.

Record:      1

Date/Time:   Aug  6 14:19:18

Source:      syslogd 1.4.2

Description: restart.

-------------------------------------------------------------------------------

Record:      2

Date/Time:   Aug  6 14:19:18

Source:      sshd[26466]

Description: Received disconnect from 172.16.28.74: 11: disconnected by user

-------------------------------------------------------------------------------

...

-------------------------------------------------------------------------------

Record:      187

Date/Time:   Aug  6 17:59:07

Source:      os[1081]

Description: processLoginSuccess() login succeeded, removing login failed entries in the queue for 172.16.28.74

-------------------------------------------------------------------------------

Record:      188

Date/Time:   Aug  6 17:59:07

Source:      ipmiextd[2216]

Description: pam_session_manager(racadm:session): license_feature_check: Can't get RHOST_SOCKET from pam_putenv

-------------------------------------------------------------------------------

[support@centos00 6_init_bkp]$

[support@centos00 6_init_bkp]$ /opt/dell/srvadmin/bin/idracadm7 -r 10.219.9.60 -u root -p ${IDRACPASS} help fwupdate

Security Alert: Certificate is invalid - self signed certificate

Continuing execution. Use -S option for racadm to stop execution on certificate-related errors.

fwupdate -- update the RAC firmware

Usage:

racadm fwupdate -p -u -d <path> [--clearcfg]

racadm fwupdate -f <ftp server ip address or FQDN> <username> <password> -d <path>

racadm fwupdate -g -u -a <ip address> [-d <path>] [--clearcfg]

racadm fwupdate -s

racadm fwupdate -r

-------------------------------------------------------------------------------

Valid Options:

-p : upload firmware image from the client (RACADM)

-g : use TFTP download process for firmware update (use with -u)

-u : perform firmware update operation (used with -p and -g)

-a : specifies TFTP server IP address used for firmware image (used with -g)

-d : specifies source path where firmware image exists (default is local)

-f : use FTP download process for firmware update

-s : query for current firmware update process status

-r : rollback to the standby firmware

--clearcfg : after firmware update, removes the previous idrac configuration

-------------------------------------------------------------------------------

Usage Examples:

- Upload a firmware image from the client and start firmware update:

racadm fwupdate -p -u -d /tmp/images

- Upload firmware image from FTP server and start firmware update:

racadm fwupdate -f 192.168.0.10 test test -d firmimg.d7

- Upload firmware image from TFTP server and start firmware update:

racadm fwupdate -g -u -a 192.168.0.100 -d /tmp/images

- Query the current status of the firmware update process:

racadm fwupdate -s

- Rollback to the standby firmware:

racadm fwupdate -r

- Upload firmware image from TFTP server, start firmware update. After firmware update is complete, delete previous iDRAC configuration:

racadm fwupdate -g -u -a 192.168.0.100 -d /tmp/images --clearcfg

-------------------------------------------------------------------------------

For help on viewing the properties of a group, run the command - racadm help get

For help on configuring the properties of a group, run the command - racadm help set

-----------------------------------------------------------------------

[support@centos00 6_init_bkp]$

[support@centos00 20150805-1830]$ /opt/dell/srvadmin/bin/idracadm7 -r 10.219.9.60 -u root -p ${IDRACPASS} getconfig -f 20150806-1913.racadm.getconfig.xml

Security Alert: Certificate is invalid - self signed certificate

Continuing execution. Use -S option for racadm to stop execution on certificate-related errors.

RAC configuration saved to file successfully

RAC1168: The RACADM "getconfig" command will be deprecated in a

future version of iDRAC firmware. Run the RACADM

"racadm get" command to retrieve the iDRAC configuration parameters.

For more information on the get command, run the RACADM command

"racadm help get".

[support@centos00 20150805-1830]$

[support@centos00 20150805-1830]$ head 20150806-1913.racadm.getconfig.xml

[idRacInfo]

# idRacType=32

# idRacProductInfo=Integrated Dell Remote Access Controller

# idRacDescriptionInfo=This system component provides a complete set of remote management functions for Dell PowerEdge Servers

# idRacVersionInfo=2.15.10.10

# idRacBuildInfo=08

# idRacName=idrac

[support@centos00 20150805-1830]$ /opt/dell/srvadmin/bin/idracadm7 -r 10.219.9.60 -u root -p ${IDRACPASS} getssninfo

Security Alert: Certificate is invalid - self signed certificate

Continuing execution. Use -S option for racadm to stop execution on certificate-related errors.

SSNID Type              User                                          IP Address Login Date/Time

---------------------------------------------------------------------------

57    RACADM            root                                        172.16.28.74 08/06/2015 20:06:42

[support@centos00 20150805-1830]

[support@centos00 20150805-1830]$ /opt/dell/srvadmin/bin/idracadm7 -r 10.219.9.60 -u root -p ${IDRACPASS} getsysinfo -d

Security Alert: Certificate is invalid - self signed certificate

Continuing execution. Use -S option for racadm to stop execution on certificate-related errors.

RAC Information:

RAC Date/Time           = Thu Aug  6 20:10:25 2015

Firmware Version        = 2.15.10.10

Firmware Build          = 08

Last Firmware Update    = 06/30/2015 17:40:49

Hardware Version        = 0.01

MAC Address             = 18:FB:7B:9D:AC:3F

[support@centos00 20150805-1830]$

[support@centos00 20150805-1830]$ /opt/dell/srvadmin/bin/idracadm7 -r 10.219.9.60 -u root -p ${IDRACPASS} getractime

Security Alert: Certificate is invalid - self signed certificate

Continuing execution. Use -S option for racadm to stop execution on certificate-related errors.

Thu Aug  6 20:11:33 2015

[support@centos00 20150805-1830]$

support@centos00 20150805-1830]$ /opt/dell/srvadmin/bin/idracadm7 -r 10.219.9.60 -u root -p ${IDRACPASS} help

Security Alert: Certificate is invalid - self signed certificate

Continuing execution. Use -S option for racadm to stop execution on certificate-related errors.

help [subcommand]    -- display usage summary for a subcommand

arp                  -- display the networking ARP table

autoupdatescheduler  -- Automatic Platform Update of the devices on the server.

clearasrscreen       -- clear the last ASR (crash) screen

clearpending         -- clear pending attribute(s) value of a Device Class

closessn             -- close a session

clrsel               -- clear the System Event Log (SEL)

config               -- Deprecated: modify RAC configuration properties

coredump             -- display the last RAC coredump

coredumpdelete       -- delete the last RAC coredump

debug                -- Field Service Debug Authorization facility commands

eventfilters         -- Alerts configuration commands

fwupdate             -- update the RAC firmware

get                  -- display RAC configuration properties

getconfig            -- Deprecated: display RAC configuration properties

gethostnetworkinterfaces -- Display host network interface details

getled               -- Get the state of the LED on a module.

getniccfg            -- display current network settings

getraclog            -- display the RAC log

getractime           -- display the current RAC time

getsel               -- display records from the System Event Log (SEL)

getsensorinfo        -- display system sensors

getssninfo           -- display session information

getsvctag            -- display service tag information

getsysinfo           -- display general RAC and system information

gettracelog          -- display the RAC diagnostic trace log

getuscversion        -- Deprecated: display the current USC version details

getversion           -- display the current version details

ifconfig             -- display network interface information

inlettemphistory     -- inlet temperature history operations

krbkeytabupload      -- upload kerberose keytab file to the RAC

license              -- License Manager commands

lclog                -- LCLog operations

frontpanelerror      -- hide LCD errors - color amber to blue

netstat              -- display routing table and network statistics

ping                 -- send ICMP echo packets on the network

ping6                -- send ICMP echo packets on the network

racdump              -- display RAC diagnostic information

racreset             -- perform a RAC reset operation

racresetcfg          -- restore the RAC configuration to factory defaults

remoteimage          -- make a remote ISO image available to the server

rollback             -- Rollback firmware to its previous version.

serveraction         -- perform system power management operations

set                  -- modify RAC configuration properties

setled               -- Set the state of the LED on a module.

setniccfg            -- modify network configuration properties

sshpkauth            -- manage SSH PK authentication keys on the RAC

sslcertupload        -- upload an SSL certificate to the RAC

sslcertdelete        -- delete an SSL certificate on the iDRAC

sslcertdownload      -- download an SSL certificate from the RAC

sslcertview          -- view SSL certificate information

sslcsrgen            -- generate a certificate CSR from the RAC

sslEncryptionStrength -- Deprecated: Display or modify the SSL Encryption strength.

sslkeyupload         -- upload an SSL key to the RAC

sslresetcfg          -- Reset iDRAC to apply new certificate. Until iDRAC is reset old certificate will be active.

swinventory          -- Display the list of S/W Installed on the server.

systemconfig         -- Backup &/or Restore of iDRAC Config and Firmware

systemerase          -- Performs system erase on a selected component.

testemail            -- test RAC e-mail notifications

testtrap             -- test RAC SNMP trap notifications

testalert            -- test RAC SNMP - FQDN trap notifications

traceroute           -- print the route packets trace to network host

traceroute6          -- print the route packets trace to network host

techsupreport        -- Tech Support Report operations.

usercertupload       -- upload an user certificate to the DRAC

usercertview         -- view user certificate information

vflashpartition      -- manage partitions on the vFlash SD card

vflashsd             -- perform vFlash SD Card initialization

vmdisconnect         -- disconnect Virtual Media connections

raid                 -- Monitoring and Inventory of H/W RAID connected to the server.

storage              -- Monitoring and Inventory of H/W RAID connected to the server.

hwinventory          -- Monitoring and Inventory of H/W NICs connected to the server.

nicstatistics        -- Statistics for NICs connected to the server.

fcstatistics         -- Statistics for FCs connected to the server.

update               -- Platform Update of the devices on the server

jobqueue             -- Jobqueue of of the jobs currently scheduled

sensorsettings       -- Set the sensor threshold levels.

diagnostics          -- Remote Diagnostic commands

SystemPerfStatistics -- Display or Modify System Performance Statistics

Groups

BIOS                -- Configuration of BIOS attributes

iDRAC               -- Configuration of iDRAC attributes

LifecycleController -- Configuration of LifecycleController attributes

Nic                 -- Configuration of NIC attributes

Storage             -- Configuration of Storage attributes

System              -- Configuration of System attributes

FC                  -- Configuration of Fiber Channel attributes

For Help on configuring the properties of a group - racadm help set

-----------------------------------------------------------------------

[support@centos00 20150805-1830]$

[support@centos00 20150805-1830]$ /opt/dell/srvadmin/bin/idracadm7 -r 10.219.9.60 -u root -p ${IDRACPASS} help set

Security Alert: Certificate is invalid - self signed certificate

Continuing execution. Use -S option for racadm to stop execution on certificate-related errors.

set -- modify RAC configuration properties

Usage:

racadm set <fqdd alias>.<group>.<object> <value>

racadm set <fqdd alias>.<group>.<index>.<object> <value>

racadm set -f <filename> [--continue]

racadm set -f <filename> -t xml

racadm set -f <filename> -t ini

racadm set -f <filename> -t xml -u myuser -p mypass -l //10.1.2.3

-------------------------------------------------------------------------------

Valid Options:

-f : save RAC configuration to a file

-u : Username of the remote share from where the file must be imported.

-p : Password for the remote share from where the file must be imported.

-l : Network share location from where the file must be imported.

For more information on NFS or CIFS share, see section on Usage examples.

-t : Specifies the file type to be imported. Valid values are "xml" and "ini".

These are case-insensitive. ini imports the legacy configuration file.

The ini file cannot be imported from a remote share. If -t is not specified

the legacy file is imported.

-b : Specifies the shutdown type for the host after the import operation

completes. The parameters are "Graceful", "Forced" and "NoReboot"

for graceful, forced shutdown and no reboot respectively. If this parameter

is not specified, graceful shutdown is taken as the default.

-w : Maximum time to wait for the graceful shut down to occur.

This has to be given in seconds. The maximum accepted value is

3600 seconds ( one hour ). The default value is 300 seconds ( 5 minutes ).

-s : Power state of the host when the import operation completes.

The parameters are "On" and "Off" for Powered ON and Powered OFF

respectively. If this parameter is not specified, power ON is taken as

default.

-c : Specifies the FQDD of the components for which the configurations

are to be imported. If this option is not specified,

configuration related to all the components are imported.

--preview : Perform the preview operation and does not configure the RAC.

Specify --preview along with set operation.

If this parameter is not specified, it will not perform preview.

--continue : If configuration through file fails for a group,

configuration continues with the next group in the file.

-------------------------------------------------------------------------------

Usage Examples:

- Configure rack name for server:

racadm set system.location.rack.name rack1

- Configure a RAC from an XML configuration file located on a remote CIFS share.

racadm set -f myfile.xml -t xml -u myuser -p mypass -l //10.1.2.3/myshare

- Preview RAC from an XML configuration file located on a remote CIFS share.

racadm set -f myfile.xml -t xml -u myuser -p mypass -l //10.1.2.3/myshare --preview

- Configure a RAC from an XML configuration file located on a remote NFS share.

racadm set -f myfile.xml -t xml -l 10.1.2.3:/myshare

- To reset the values set using set operation use racadm command

racadm help clearpending

- To clear out a string attribute, pass in double quotes for the value.

Ex: set system.location.datacenter ""

- Configure a RAC from an XML file, with a wait time of ten minutes,

shutdown type graceful and end host type power on:

racadm set -f myfile.xml -t xml -b graceful -w 600 -s ON

- Configure a RAC from a configuration file:

racadm set -f config.txt

- Configure a RAC from a configuration file and continue if a group

fails to get configured:

racadm set -f config.txt --continue

- Configure a RAC from an XML file:

racadm set -f myfile.xml -t xml

- Configure the iDRAC component on a RAC from an XML file:

racadm set -f myfile.xml -t xml -c iDRAC.Embedded.1

- Configure a RAC from an INI file:

racadm set -f myfile.txt -t ini

-------------------------------------------------------------------------------

For help on viewing the properties of a group, run the command - racadm help get

For help on configuring the properties of a group, run the command - racadm help set

-----------------------------------------------------------------------

[support@centos00 20150805-1830]$ /opt/dell/srvadmin/bin/idracadm7 -r 10.219.9.60 -u root -p ${IDRACPASS} help get

Security Alert: Certificate is invalid - self signed certificate

Continuing execution. Use -S option for racadm to stop execution on certificate-related errors.

get -- display RAC configuration properties

Usage:

racadm get <fqdd alias>.<group>

racadm get <fqdd alias>.<group>.<object>

racadm get <fqdd alias>.<group>.<index>.<object>

racadm get -f <filename>

racadm get -f <filename> -t <filetype> -u <username> -p <password>

-l <CIFS share> -c <FQDD>

racadm get -f <filename> -t <filetype> -l <NFS share> -c <FQDD>

-------------------------------------------------------------------------------

Valid Options:

-f : save RAC configuration to a file

-u : Username of the remote share to where the file must be exported.

-p : Password for the remote share to where the file must be exported.

-l : Network share location to where the file must be exported.

For more information on NFS or CIFS share, see section on Usage examples.

-c : Specifies the FQDD of the components for which the configurations

are to be exported. If this option is not specified,

configuration related to all the components are exported.

-t : Specifies the file type to be exported. Valid values are "xml" and "ini".

These are case-insensitive. ini exports the legacy configuration file.

The ini file cannot be exported to a remote share.  If -t is not specified,

the legacy file is exported.

--clone   : Export Cloned configuration file. Supports xml file format only.

Configuration file can be exported to remote or local share.

VD Creation Action and Size will be uncommented.

--replace : Export Replaced configuration file. Supports xml file format only.

Configuration file can be exported to remote or local share.

VD Creation Action and Size will be uncommented, along with server I/O

identity like NIC/FC properties are uncommented.

--includero : Include read only attributes in the configuration XML file.

--includeph : Include password hash attributes in the configuration XML file.

-------------------------------------------------------------------------------

Usage Examples:

- Display an entire group, in this case the topology configuration:

racadm get system.location

- Display a single object from a particular group:

racadm get system.location.rack.name

- Display an indexed group:

racadm get system.power.supply.1

- Export the xml configuration to a CIFS share:

racadm get -f file -t xml -u myuser -p mypass -l //10.1.12.13/share

- Export the xml configuration of the iDRAC component to a CIFS share:

racadm get -f file -t xml -u myuser -p mypass -l //10.1.12.13/share

-c iDRAC.Embedded.1

- Export the xml configuration to an NFS share:

racadm get -f file -t xml -l 10.1.12.13:/myshare

- Save RAC configuration to a file:

racadm get -f config.txt

-------------------------------------------------------------------------------

For help on viewing the properties of a group, run the command - racadm help get

For help on configuring the properties of a group, run the command - racadm help set

-----------------------------------------------------------------------

[support@centos00 20150805-1830]$

[support@centos00 20150805-1830]$ /opt/dell/srvadmin/bin/idracadm7 -r 10.219.9.60 -u root -p ${IDRACPASS} get bios

Security Alert: Certificate is invalid - self signed certificate

Continuing execution. Use -S option for racadm to stop execution on certificate-related errors.

BiosBootSettings

IntegratedDevices

IscsiDev1Con1Settings

IscsiDev1Con2Settings

IscsiDev1Settings

MemSettings

MiscSettings

NetworkSettings

OneTimeBoot

ProcSettings

PxeDev1Settings

PxeDev2Settings

PxeDev3Settings

PxeDev4Settings

SataSettings

SerialCommSettings

SlotDisablement

SysInformation

SysProfileSettings

SysSecurity

[support@centos00 20150805-1830]$

[support@centos00 20150805-1830]$ sshpass -p "${IDRACPASS}" ssh root@10.219.9.60 racadm raid get pdisks -o

Disk.Bay.0:Enclosure.Internal.0-1:RAID.Integrated.1-1

Status                           = Ok

DeviceDescription                = Disk 0 in Backplane 1 of Integrated RAID Controller 1

RollupStatus                     = Ok

Name                             = Physical Disk 0:1:0

State                            = Non-Raid

OperationState                   = Not Applicable

PowerStatus                      = Spun-Up

Size                             = 3725.50 GB

FailurePredicted                 = NO

RemainingRatedWriteEndurance     = Not Applicable

SecurityStatus                   = Not Capable

BusProtocol                      = SAS

MediaType                        = HDD

UsedRaidDiskSpace                = 0.00 GB

AvailableRaidDiskSpace           = 3725.50 GB

Hotspare                         = NO

Manufacturer                     = SEAGATE

ProductId                        = ST4000NM0023

Revision                         =

SerialNumber                     =

PartNumber                       =

NegotiatedSpeed                  = 6.0 Gb/s

ManufacturedDay                  = 0

ManufacturedWeek                 = 0

ManufacturedYear                 = 0

SasAddress                       = 0x5000C50088B5FC5D

FormFactor                       = Unknown

RaidNominalMediumRotationRate    = 7200

T10PICapability                  = Not Capable

BlockSizeInBytes                 = 512

MaxCapableSpeed                  = 6 Gb/s

Disk.Bay.1:Enclosure.Internal.0-1:RAID.Integrated.1-1

Status                           = Ok

DeviceDescription                = Disk 1 in Backplane 1 of Integrated RAID Controller 1

RollupStatus                     = Ok

Name                             = Physical Disk 0:1:1

State                            = Non-Raid

OperationState                   = Not Applicable

PowerStatus                      = Spun-Up

Size                             = 3725.50 GB

FailurePredicted                 = NO

RemainingRatedWriteEndurance     = Not Applicable

SecurityStatus                   = Not Capable

BusProtocol                      = SAS

MediaType                        = HDD

UsedRaidDiskSpace                = 0.00 GB

AvailableRaidDiskSpace           = 3725.50 GB

Hotspare                         = NO

Manufacturer                     = SEAGATE

ProductId                        = ST4000NM0023

Revision                         =

SerialNumber                     =

PartNumber                       =

NegotiatedSpeed                  = 6.0 Gb/s

ManufacturedDay                  = 0

ManufacturedWeek                 = 0

ManufacturedYear                 = 0

SasAddress                       = 0x5000C50088A84ECD

FormFactor                       = Unknown

RaidNominalMediumRotationRate    = 7200

T10PICapability                  = Not Capable

BlockSizeInBytes                 = 512

MaxCapableSpeed                  = 6 Gb/s

…

Disk.Bay.12:Enclosure.Internal.0-1:RAID.Integrated.1-1

Status                           = Ok

DeviceDescription                = Disk 12 in Backplane 1 of Integrated RAID Controller 1

RollupStatus                     = Ok

Name                             = Physical Disk 0:1:12

State                            = Online

OperationState                   = Not Applicable

PowerStatus                      = Spun-Up

Size                             = 465.25 GB

FailurePredicted                 = NO

RemainingRatedWriteEndurance     = Not Applicable

SecurityStatus                   = Not Capable

BusProtocol                      = SAS

MediaType                        = HDD

UsedRaidDiskSpace                = 465.25 GB

AvailableRaidDiskSpace           = 0.00 GB

Hotspare                         = NO

Manufacturer                     = SEAGATE

ProductId                        = ST9500620SS

Revision                         = AS0B

SerialNumber                     = 9XF3LNW8

PartNumber                       = TH055RMX2123354M002MA08

NegotiatedSpeed                  = 6.0 Gb/s

ManufacturedDay                  = 6

ManufacturedWeek                 = 17

ManufacturedYear                 = 2015

SasAddress                       = 0x5000C500838DC9F5

FormFactor                       = 2.5 Inch

RaidNominalMediumRotationRate    = 7200

T10PICapability                  = Not Capable

BlockSizeInBytes                 = 512

MaxCapableSpeed                  = 6 Gb/s

Disk.Bay.13:Enclosure.Internal.0-1:RAID.Integrated.1-1

Status                           = Ok

DeviceDescription                = Disk 13 in Backplane 1 of Integrated RAID Controller 1

RollupStatus                     = Ok

Name                             = Physical Disk 0:1:13

State                            = Online

OperationState                   = Not Applicable

PowerStatus                      = Spun-Up

Size                             = 465.25 GB

FailurePredicted                 = NO

RemainingRatedWriteEndurance     = Not Applicable

SecurityStatus                   = Not Capable

BusProtocol                      = SAS

MediaType                        = HDD

UsedRaidDiskSpace                = 465.25 GB

AvailableRaidDiskSpace           = 0.00 GB

Hotspare                         = NO

Manufacturer                     = SEAGATE

ProductId                        = ST9500620SS

Revision                         = AS0B

SerialNumber                     = 9XF3MT76

PartNumber                       = TH055RMX2123354M002PA08

NegotiatedSpeed                  = 6.0 Gb/s

ManufacturedDay                  = 6

ManufacturedWeek                 = 17

ManufacturedYear                 = 2015

SasAddress                       = 0x5000C500838DE765

FormFactor                       = 2.5 Inch

RaidNominalMediumRotationRate    = 7200

T10PICapability                  = Not Capable

BlockSizeInBytes                 = 512

MaxCapableSpeed                  = 6 Gb/s

[support@centos00 20150805-1830]$

[support@centos00 20150805-1830]$ sshpass -p "${IDRACPASS}" ssh root@10.219.9.60 racadm raid get vdisks -o

Disk.Virtual.0:RAID.Integrated.1-1

Status                           = Ok

DeviceDescription                = Virtual Disk 0 on Integrated RAID Controller 1

Name                             = Virtual Disk 0

RollupStatus                     = Ok

State                            = Online

OperationalState                 = Not applicable

Layout                           = Raid-1

Size                             = 465.25 GB

SpanDepth                        = 1

AvailableProtocols               = SAS

MediaType                        = HDD

ReadPolicy                       = Read Ahead

WritePolicy                      = Write Back

StripeSize                       = 64K

DiskCachePolicy                  = Default

BadBlocksFound                   = NO

Secured                          = NO

RemainingRedundancy              = 1

EnhancedCache                    = Not Applicable

T10PIStatus                      = Disabled

BlockSizeInBytes                 = 512

[support@centos00 20150805-1830]$

[support@centos00 20150805-1830]$ for var1 in $(cat) ; do /opt/dell/srvadmin/bin/idracadm7 -r 10.219.9.60 -u root -p ${IDRACPASS} get bios.${var1} ; done

BiosBootSettings

IntegratedDevices

IscsiDev1Con1Settings

IscsiDev1Con2Settings

IscsiDev1Settings

MemSettings

MiscSettings

NetworkSettings

OneTimeBoot

ProcSettings

PxeDev1Settings

PxeDev2Settings

PxeDev3Settings

PxeDev4Settings

SataSettings

SerialCommSettings

SlotDisablement

SysInformation

SysProfileSettings

SysSecurity

Security Alert: Certificate is invalid - self signed certificate

Continuing execution. Use -S option for racadm to stop execution on certificate-related errors.

[Key=BIOS.Setup.1-1#BiosBootSettings]

BootMode=Bios

BootSeq=HardDisk.List.1-1,NIC.Integrated.1-1-1,Floppy.iDRACVirtual.1-1,Optical.iDRACVirtual.1-1

BootSeqRetry=Enabled

HddFailover=Disabled

HddSeq=RAID.Integrated.1-1

SetBootOrderFqdd1=

SetBootOrderFqdd10=

SetBootOrderFqdd11=

SetBootOrderFqdd12=

SetBootOrderFqdd13=

SetBootOrderFqdd14=

SetBootOrderFqdd15=

SetBootOrderFqdd16=

SetBootOrderFqdd2=

SetBootOrderFqdd3=

SetBootOrderFqdd4=

SetBootOrderFqdd5=

SetBootOrderFqdd6=

SetBootOrderFqdd7=

SetBootOrderFqdd8=

SetBootOrderFqdd9=

SetLegacyHddOrderFqdd1=

SetLegacyHddOrderFqdd10=

SetLegacyHddOrderFqdd11=

SetLegacyHddOrderFqdd12=

SetLegacyHddOrderFqdd13=

SetLegacyHddOrderFqdd14=

SetLegacyHddOrderFqdd15=

SetLegacyHddOrderFqdd16=

SetLegacyHddOrderFqdd2=

SetLegacyHddOrderFqdd3=

SetLegacyHddOrderFqdd4=

SetLegacyHddOrderFqdd5=

SetLegacyHddOrderFqdd6=

SetLegacyHddOrderFqdd7=

SetLegacyHddOrderFqdd8=

SetLegacyHddOrderFqdd9=

UefiBootSeq=

Security Alert: Certificate is invalid - self signed certificate

Continuing execution. Use -S option for racadm to stop execution on certificate-related errors.

[Key=BIOS.Setup.1-1#IntegratedDevices]

#CurrentEmbVideoState=Enabled

EmbVideo=Enabled

IntegratedNetwork1=Enabled

IntegratedRaid=Enabled

InternalUsb=On

IoatEngine=Disabled

MmioAbove4Gb=Enabled

OsWatchdogTimer=Disabled

SriovGlobalEnable=Disabled

Usb3Setting=Disabled

UsbPorts=AllOn

Security Alert: Certificate is invalid - self signed certificate

Continuing execution. Use -S option for racadm to stop execution on certificate-related errors.

[Key=BIOS.Setup.1-1#IscsiDev1Con1Settings]

Security Alert: Certificate is invalid - self signed certificate

Continuing execution. Use -S option for racadm to stop execution on certificate-related errors.

[Key=BIOS.Setup.1-1#IscsiDev1Con2Settings]

Security Alert: Certificate is invalid - self signed certificate

Continuing execution. Use -S option for racadm to stop execution on certificate-related errors.

[Key=BIOS.Setup.1-1#IscsiDev1Settings]

Security Alert: Certificate is invalid - self signed certificate

Continuing execution. Use -S option for racadm to stop execution on certificate-related errors.

[Key=BIOS.Setup.1-1#MemSettings]

CorrEccSmi=Enabled

MemOpMode=OptimizerMode

MemTest=Disabled

NodeInterleave=Disabled

SnoopMode=EarlySnoop

#SysMemSize=256 GB

#SysMemSpeed=1866 Mhz

#SysMemType=ECC DDR4

#SysMemVolt=1.20 V

#VideoMem=16 MB

Security Alert: Certificate is invalid - self signed certificate

Continuing execution. Use -S option for racadm to stop execution on certificate-related errors.

[Key=BIOS.Setup.1-1#MiscSettings]

AssetTag=

ErrPrompt=Enabled

ForceInt10=Disabled

InSystemCharacterization=FastBoot

NumLock=On

Security Alert: Certificate is invalid - self signed certificate

Continuing execution. Use -S option for racadm to stop execution on certificate-related errors.

[Key=BIOS.Setup.1-1#NetworkSettings]

Security Alert: Certificate is invalid - self signed certificate

Continuing execution. Use -S option for racadm to stop execution on certificate-related errors.

[Key=BIOS.Setup.1-1#OneTimeBoot]

OneTimeBootMode=Disabled

#OneTimeBootSeqDev=HardDisk.List.1-1

#OneTimeHddSeqDev=RAID.Integrated.1-1

Security Alert: Certificate is invalid - self signed certificate

Continuing execution. Use -S option for racadm to stop execution on certificate-related errors.

[Key=BIOS.Setup.1-1#ProcSettings]

ControlledTurbo=Disabled

DcuIpPrefetcher=Enabled

DcuStreamerPrefetcher=Enabled

DynamicCoreAllocation=Disabled

LogicalProc=Enabled

#Proc1Brand=Intel(R) Xeon(R) CPU E5-2640 v3 @ 2.60GHz

#Proc1Id=6-3F-2

#Proc1L2Cache=8x256 KB

#Proc1L3Cache=20 MB

#Proc1NumCores=8

#Proc2Brand=Intel(R) Xeon(R) CPU E5-2640 v3 @ 2.60GHz

#Proc2Id=6-3F-2

#Proc2L2Cache=8x256 KB

#Proc2L3Cache=20 MB

#Proc2NumCores=8

#Proc64bit=Yes

ProcAdjCacheLine=Enabled

ProcAts=Enabled

#ProcBusSpeed=8.0 GT/s

ProcConfigTdp=Nominal

#ProcCoreSpeed=2.60 GHz

ProcCores=All

ProcHwPrefetcher=Enabled

ProcVirtualization=Enabled

ProcX2Apic=Disabled

QpiSpeed=MaxDataRate

RtidSetting=Disabled

Security Alert: Certificate is invalid - self signed certificate

Continuing execution. Use -S option for racadm to stop execution on certificate-related errors.

[Key=BIOS.Setup.1-1#PxeDev1Settings]

Security Alert: Certificate is invalid - self signed certificate

Continuing execution. Use -S option for racadm to stop execution on certificate-related errors.

[Key=BIOS.Setup.1-1#PxeDev2Settings]

Security Alert: Certificate is invalid - self signed certificate

Continuing execution. Use -S option for racadm to stop execution on certificate-related errors.

[Key=BIOS.Setup.1-1#PxeDev3Settings]

Security Alert: Certificate is invalid - self signed certificate

Continuing execution. Use -S option for racadm to stop execution on certificate-related errors.

[Key=BIOS.Setup.1-1#PxeDev4Settings]

Security Alert: Certificate is invalid - self signed certificate

Continuing execution. Use -S option for racadm to stop execution on certificate-related errors.

[Key=BIOS.Setup.1-1#SataSettings]

EmbSata=AhciMode

#SataPortA=Auto

#SataPortACapacity=N/A

#SataPortADriveType=Not Enumerated

#SataPortAModel=Not Enumerated

#SataPortB=Auto

#SataPortBCapacity=N/A

#SataPortBDriveType=Not Enumerated

#SataPortBModel=Not Enumerated

SecurityFreezeLock=Enabled

WriteCache=Disabled

Security Alert: Certificate is invalid - self signed certificate

Continuing execution. Use -S option for racadm to stop execution on certificate-related errors.

[Key=BIOS.Setup.1-1#SerialCommSettings]

ConTermType=Vt100Vt220

ExtSerialConnector=Serial1

FailSafeBaud=115200

RedirAfterBoot=Enabled

SerialComm=OnConRedirAuto

SerialPortAddress=Serial1Com2Serial2Com1

Security Alert: Certificate is invalid - self signed certificate

Continuing execution. Use -S option for racadm to stop execution on certificate-related errors.

[Key=BIOS.Setup.1-1#SlotDisablement]

Slot1=Enabled

Slot2=Enabled

Slot3=Enabled

Slot4=Enabled

Slot5=Enabled

Slot6=Enabled

Security Alert: Certificate is invalid - self signed certificate

Continuing execution. Use -S option for racadm to stop execution on certificate-related errors.

[Key=BIOS.Setup.1-1#SysInformation]

#SysMfrContactInfo=www.dell.com

#SystemBiosVersion=1.3.6

#SystemCpldVersion=1.0.1

#SystemManufacturer=Dell Inc.

#SystemMeVersion=3.0.7.164

#SystemModelName=PowerEdge R730xd

#SystemServiceTag=4P7V762

#UefiComplianceVersion=2.3.1

Security Alert: Certificate is invalid - self signed certificate

Continuing execution. Use -S option for racadm to stop execution on certificate-related errors.

[Key=BIOS.Setup.1-1#SysProfileSettings]

#CollaborativeCpuPerfCtrl=Disabled

#EnergyEfficientTurbo=Disabled

#EnergyPerformanceBias=MaxPower

#MemFrequency=MaxPerf

#MemPatrolScrub=Standard

#MemRefreshRate=1x

#MonitorMwait=Enabled

PowerSaver=Disabled

#Proc1TurboCoreNum=All

#Proc2TurboCoreNum=All

#ProcC1E=Disabled

#ProcCStates=Disabled

#ProcPwrPerf=MaxPerf

#ProcTurboMode=Enabled

SysProfile=PerfOptimized

#UncoreFrequency=MaxUFS

WorkloadProfile=NotAvailable

Security Alert: Certificate is invalid - self signed certificate

Continuing execution. Use -S option for racadm to stop execution on certificate-related errors.

[Key=BIOS.Setup.1-1#SysSecurity]

AcPwrRcvry=Last

AcPwrRcvryDelay=Immediate

#AcPwrRcvryUserDelay=60

#AesNi=Enabled

#IntelTxt=Off

NmiButton=Disabled

PasswordStatus=Unlocked

PwrButton=Enabled

SHA256SetupPassword=

SHA256SetupPasswordSalt=

SHA256SystemPassword=

SHA256SystemPasswordSalt=

#SecureBoot=Disabled

#SecureBootPolicy=Standard

SetupPassword=******** (Write-Only)

SysPassword=******** (Write-Only)

#TpmCommand=None

#TpmInfo=No TPM present

#TpmSecurity=Off

#TpmStatus=Unknown

UefiVariableAccess=Standard

[support@centos00 20150805-1830]$

[support@centos00 20150805-1830]$ /opt/dell/srvadmin/bin/idracadm7 -r 10.219.9.60 -u root -p ${IDRACPASS} getsensorinfo

Security Alert: Certificate is invalid - self signed certificate

Continuing execution. Use -S option for racadm to stop execution on certificate-related errors.

Sensor Type : POWER

<Sensor Name>                   <Status>                 <Type>

PS1 Status                      Present                  AC

PS2 Status                      Present                  AC

Sensor Type : TEMPERATURE

<Sensor Name>            <Status>    <Reading> <lc> <uc>  <lnc>[R/W]  <unc>[R/W]

[Key = iDRAC.Embedded.1#SystemBoardInletTemp]

System Board Inlet Temp       Ok      23C      -7C  47C     3C [Y]      42C [Y]

[Key = iDRAC.Embedded.1#SystemBoardExhaustTemp]

System Board Exhaust Temp     Ok      48C      0C   75C     0C [N]      70C [N]

[Key = iDRAC.Embedded.1#CPU1Temp]

CPU1 Temp                     Ok      57C      3C   87C     8C [N]      82C [N]

[Key = iDRAC.Embedded.1#CPU2Temp]

CPU2 Temp                     Ok      63C      3C   87C     8C [N]      82C [N]

Sensor Type : FAN

<Sensor Name>                   <Status>    <Reading>   <lc>        <uc>        <PWM %>

System Board Fan1 RPM           Ok          5280RPM     360RPM      NA          25%

System Board Fan2 RPM           Ok          5400RPM     360RPM      NA          25%

System Board Fan3 RPM           Ok          5400RPM     360RPM      NA          25%

System Board Fan4 RPM           Ok          5160RPM     360RPM      NA          24%

System Board Fan5 RPM           Ok          5640RPM     360RPM      NA          27%

System Board Fan6 RPM           Ok          5280RPM     360RPM      NA          24%

Sensor Type : VOLTAGE

<Sensor Name>                   <Status>    <Reading>   <lc>        <uc>

CPU1 VCORE PG                   Ok          Good                NA          NA

CPU2 VCORE PG                   Ok          Good                NA          NA

System Board 3.3V PG            Ok          Good                NA          NA

System Board 5V AUX PG          Ok          Good                NA          NA

CPU2 M23 VPP PG                 Ok          Good                NA          NA

CPU1 M23 VPP PG                 Ok          Good                NA          NA

System Board 1.05V PG           Ok          Good                NA          NA

System Board BP0 5V PG          Ok          Good                NA          NA

CPU1 M23 VDDQ PG                Ok          Good                NA          NA

CPU1 M23 VTT PG                 Ok          Good                NA          NA

System Board 5V SWITCH PG       Ok          Good                NA          NA

System Board DIMM PG            Ok          Good                NA          NA

System Board VCCIO PG           Ok          Good                NA          NA

CPU2 M01 VDDQ PG                Ok          Good                NA          NA

CPU1 M01 VDDQ PG                Ok          Good                NA          NA

CPU2 M23 VTT PG                 Ok          Good                NA          NA

CPU2 M01 VTT PG                 Ok          Good                NA          NA

System Board NDC PG             Ok          Good                NA          NA

CPU2 M01 VPP PG                 Ok          Good                NA          NA

CPU1 M01 VPP PG                 Ok          Good                NA          NA

CPU2 M23 VDDQ PG                Ok          Good                NA          NA

System Board 1.5V PG            Ok          Good                NA          NA

PS2 PG Fail                     Ok          Good                NA          NA

System Board PS1 PG Fail        Ok          Good                NA          NA

System Board BP1 5V PG          Ok          Good                NA          NA

System Board 1.5V AUX PG        Ok          Good                NA          NA

CPU1 M01 VTT PG                 Ok          Good                NA          NA

PS1 Voltage 1                   Ok          230V                NA          NA

CPU1 FIVR PG                    Ok          Good                NA          NA

CPU2 FIVR PG                    Ok          Good                NA          NA

System Board 2.5V AUX PG        Ok          Good                NA          NA

PS2 Voltage 2                   Ok          230V                NA          NA

Sensor Type : CURRENT

<Sensor Name>            <Status>    <Reading> <lc> <uc>  <lnc>[R/W]  <unc>[R/W]

[Key = iDRAC.Embedded.1#PS1Current1]

PS1 Current 1                 Ok      1.0Amps  NA   NA      0Amps [N]      0Amps [N]

[Key = iDRAC.Embedded.1#SystemBoardPwrConsumption]

System Board Pwr Consumption  Ok      238Watts NA   980Watts0Watts [N]      896Watts [Y]

[Key = iDRAC.Embedded.1#PS2Current2]

PS2 Current 2                 Ok      0.2Amps  NA   NA      0Amps [N]      0Amps [N]

Sensor Type : PROCESSOR

<Sensor Name>                   <Status>    <State>             <lc>        <uc>

CPU1 Status                     Ok          Presence Detected   NA          NA

CPU2 Status                     Ok          Presence Detected   NA          NA

Sensor Type : MEMORY

<Sensor Name>                   <Status>    <State>             <lc>        <uc>

DIMM A1                         Ok          Presence Detected   NA          NA

DIMM A2                         Ok          Presence Detected   NA          NA

DIMM A3                         Ok          Presence Detected   NA          NA

DIMM A4                         Ok          Presence Detected   NA          NA

DIMM A5                         Ok          Presence Detected   NA          NA

DIMM A6                         Ok          Presence Detected   NA          NA

DIMM A7                         Ok          Presence Detected   NA          NA

DIMM A8                         Ok          Presence Detected   NA          NA

DIMM A9                         N/A         Absent              NA          NA

DIMM A10                        N/A         Absent              NA          NA

DIMM A11                        N/A         Absent              NA          NA

DIMM A12                        N/A         Absent              NA          NA

DIMM B1                         Ok          Presence Detected   NA          NA

DIMM B2                         Ok          Presence Detected   NA          NA

DIMM B3                         Ok          Presence Detected   NA          NA

DIMM B4                         Ok          Presence Detected   NA          NA

DIMM B5                         Ok          Presence Detected   NA          NA

DIMM B6                         Ok          Presence Detected   NA          NA

DIMM B7                         Ok          Presence Detected   NA          NA

DIMM B8                         Ok          Presence Detected   NA          NA

DIMM B9                         N/A         Absent              NA          NA

DIMM B10                        N/A         Absent              NA          NA

DIMM B11                        N/A         Absent              NA          NA

DIMM B12                        N/A         Absent              NA          NA

Sensor Type : BATTERY

<Sensor Name>                   <Status>    <Reading>   <lc>        <uc>

System Board CMOS Battery       Ok          Present             NA          NA

PERC1 ROMB Battery              Ok          Present             NA          NA

Sensor Type : PERFORMANCE

<Sensor Name>                   <Status>    <State>        <lc>      <uc>

System Board Power Optimized    Ok          Not Degraded   NA        NA

Sensor Type : INTRUSION

<Sensor Name>                   <Intrusion>    <Status>

System Board Intrusion          Closed         Power ON

Sensor Type : REDUNDANCY

<Sensor Name>                   <Status>                 <Type>

System Board Fan Redundancy     Full Redundant           Fan

System Board PS Redundancy      Full Redundant           PSU

Sensor Type : SYSTEM PERFORMANCE

<Sensor Name>            <Status>    <Reading> <lc> <uc>  <lnc>[R/W]  <unc>[R/W]

[Key = iDRAC.Embedded.1#SystemBoardCPUUsage]

System Board CPU Usage   Ok             2%      NA  NA    NA  [N]      101%[Y]

[Key = iDRAC.Embedded.1#SystemBoardIOUsage]

System Board IO Usage    Ok             0%      NA  NA    NA  [N]      101%[Y]

[Key = iDRAC.Embedded.1#SystemBoardMEMUsage]

System Board MEM Usage   Ok             0%      NA  NA    NA  [N]      101%[Y]

[Key = iDRAC.Embedded.1#SystemBoardSYSUsage]

System Board SYS Usage   Ok             3%      NA  NA    NA  [N]      101%[Y]

[support@centos00 20150805-1830]$

[support@centos00 20150805-1830]$ /opt/dell/srvadmin/bin/idracadm7 -r 10.219.9.60 -u root -p ${IDRACPASS} get system.power

Security Alert: Certificate is invalid - self signed certificate

Continuing execution. Use -S option for racadm to stop execution on certificate-related errors.

#Avg.LastDay=244 W | 833 Btu/hr

#Avg.LastHour=249 W | 850 Btu/hr

#Avg.LastWeek=268 W | 915 Btu/hr

#Cap.ActivePolicy.BtuHr=N/A

#Cap.ActivePolicy.Name=N/A

#Cap.ActivePolicy.Watts=N/A

Cap.BtuHr=2010 btu/hr

Cap.Enable=Disabled

#Cap.MaxThreshold=591 W | 2018 Btu/hr

#Cap.MinThreshold=402 W | 1371 Btu/hr

Cap.Percent=99

Cap.Watts=589 W

#EnergyConsumption=50.389 KWh | 171978 Btu

#EnergyConsumption.Clear=******** (Write-Only)

#EnergyConsumption.StarttimeStamp=Tue Jun 30 11:45:46 2015

HotSpare.Enable=Enabled

HotSpare.PrimaryPSU=PSU1

#Max.Amps=62.7 Amps

#Max.Amps.Timestamp=Thu Aug 06 06:37:16 2015

#Max.Headroom=177 W | 604 Btu/hr

#Max.LastDay=455 W | 1553 Btu/hr

#Max.LastDay.Timestamp=Fri Aug 07 08:59:13 2015

#Max.LastHour=265 W | 904 Btu/hr

#Max.LastHour.Timestamp=Fri Aug 07 09:36:33 2015

#Max.LastWeek=723 W | 2468 Btu/hr

#Max.LastWeek.Timestamp=Thu Aug 06 04:20:27 2015

#Max.Power=723 W | 2468 Btu/hr

#Max.Power.Timestamp=Thu Aug  6 03:19:58 2015

#Max.PowerClear=******** (Write-Only)

#Min.LastDay=234 W | 799 Btu/hr

#Min.LastDay.Timestamp=Fri Aug 07 09:37:47 2015

#Min.LastHour=234 W | 799 Btu/hr

#Min.LastHour.Timestamp=Fri Aug 07 09:37:47 2015

#Min.LastWeek=5 W | 17 Btu/hr

#Min.LastWeek.Timestamp=Mon Aug 03 05:46:59 2015

PFCEnable=Disabled

#Realtime.Amps=1.1 Amps

#Realtime.Headroom=650 W | 2218 Btu/hr

#Realtime.Power=250 W | 853 Btu/hr

#RedundancyCapabilities=Not Redundant,Input Power Redundant

RedundancyPolicy=Input Power Redundant

#Status=1

[support@centos00 20150805-1830]$

Last recommendation for these parameters are set it to:

system.power.HotSpare.Enable=Disable

system.power.RedundancyPolicy=Input Power Redundant

https://wiki.tcsbank.ru/pages/viewpage.action?pageId=41077249

[support@centos00 20150805-1830]$ sshpass -p "${IDRACPASS}" ssh root@10.219.9.60 racadm get system.power.HotSpare.Enable

Enabled

[support@centos00 20150805-1830]$ sshpass -p "${IDRACPASS}" ssh root@10.219.9.60 racadm get system.power.RedundancyPolicy

Input Power Redundant

[support@centos00 20150805-1830]$

sshpass -p "${IDRACPASS}" ssh root@10.219.9.60 racadm get system.power.HotSpare.Enable Disable

[support@centos00 20150805-1830]$ /opt/dell/srvadmin/bin/idracadm7 -r 10.219.9.60 -u root -p ${IDRACPASS} help storage

Security Alert: Certificate is invalid - self signed certificate

Continuing execution. Use -S option for racadm to stop execution on certificate-related errors.

Storage monitoring, inventory and configuration of hardware RAID connected to the server.

Usage :

racadm storage get status

racadm storage get <Object type I>

racadm storage get <Object type I> -o

racadm storage get <Object type I> -o -p <property names separated by comma>

racadm storage get <Object type I>:<FQDD's of Object type separated by comma>

-p <property names separated by comma>

racadm storage get <Object type I>:<FQDD's of Object type separated by comma>

racadm storage get <Object type II> --refkey <reference keys separated by comma>

racadm storage get <Object type II> --refkey <reference keys separated by comma> -o

racadm storage get <Object type II> --refkey <reference keys separated by comma> -o

-p <property names separated by comma>

racadm storage <Object type III>:<FQDD>

racadm storage hotspare:<Physical Disk FQDD> -assign yes  -type dhs -vdkey:<FQDD of VD>

racadm storage init:<VD FQDD> -speed fast }

racadm storage createvd:<Controller FQDD> -rl {r0|r1|r5|r6|r10|r50|r60} [-wp {wt|wb}] [-rp {nra|ra|ara}]

[-ss {1k|2k|4k|8k|16k|32k|64k|128k|256k|512k|1M|2M|4M|8M|16M}] -pdkey:<comma separated PD FQDD>

[-dcp {enabled|disabled|default}] [-name <VD name>] [-size <VD size>{b|k|m|g|t}]

racadm storage createsecuritykey:<Controller FQDD> -key <Key id> -passwd <passphrase>

racadm storage modifysecuritykey:<Controller FQDD> -key <Key id>

-oldpasswd <old passphrase> -newpasswd <new passphrase>

racadm storage blink:<PD FQDD>

racadm storage unblink:<PD FQDD>

racadm storage preparetoremove:<PCIeSSD FQDD>

racadm storage secureerase:<PCIeSSD FQDD>

racadm storage patrolread:<Controller FQDD> -state start

--------------------------------------------------------------------------------

Valid Options:

Object type I      : controllers, batteries, vdisks, pdisks, fans, emms, temppro

bes, psus, enclosures.

Object type II     : batteries, vdisks, pdisks, fans, emms, tempprobes, psus, enclosures.

Object type III    : resetconfig, clearconfig, importconfig, deletesecuritykey, encryptvd,

deletevd, converttoraid, converttononraid, ccheck, cancelcheck

-o                 : Displays all the properties of the selected Key or Object.

-p                 : Displays the property names with filter.

FQDD's             : Displays all the properties of the FQDD's Key.

NOTE: Maximum Property names can be specified in -p option is = 10.

NOTE: Maximum FQDD's can be specified is = 3.

--------------------------------------------------------------------------------

Usage Examples :

racadm storage get controllers

racadm storage get controllers -o

racadm storage get controllers -o -p name,status

racadm storage get vdisks -o -p layout,status

racadm storage get controllers:RAID.Integrated.1-1

racadm storage get controllers:RAID.Integrated.1-1 -p status

racadm storage resetconfig:RAID.Integrated.1-1

racadm storage deletevd:Disk.Virtual.0:RAID.Integrated.1-1

racadm storage clearconfig:RAID.Integrated.1-1

racadm storage patrolread:RAID.Integrated.1-1 -state start

racadm storage importconfig:RAID.Integrated.1-1

racadm storage hotspare:Disk.Bay.0:Enclosure.Internal.0-0:RAID.Integrated.1-1 -assign yes -type dhs -vdkey:Disk.Virtual.0:RAID.Integrated.1-1

racadm storage init:Disk.Virtual.0:RAID.Integrated.1-1 -speed fast }

racadm storage createvd:RAID.Integrated.1-1 -rl r0 -wp wt -rp nra -name raid_0 -pdkey:Disk.Bay.0:Enclosure.Internal.0-0:RAID.Integrated.1-1,Disk.Bay.1:Enclosure.Internal.0-0:RAID.Integrated.1-1

racadm storage createsecuritykey:RAID.Integrated.1-1 -key <Key id> -passwd <passphrase>

racadm storage modifysecuritykey:RAID.Integrated.1-1 -key <Key id> -oldpasswd <old passphrase> -newpasswd <new passphrase>

racadm storage deletesecuritykey:RAID.Integrated.1-1

racadm storage encryptvd:Disk.Virtual.0:RAID.Integrated.1-1

racadm storage converttoraid:Disk.Bay.0:Enclosure.Internal.0-0:RAID.Integrated.1-1

racadm storage converttononraid:Disk.Bay.0:Enclosure.Internal.0-0:RAID.Integrated.1-1

racadm storage blink:Disk.Bay.0:Enclosure.Internal.0-0:RAID.Integrated.1-1

racadm storage unblink:Disk.Bay.0:Enclosure.Internal.0-0:RAID.Integrated.1-1

racadm storage preparetoremove:Disk.Bay.6:Enclosure.Internal.0-1:PCIeExtender.Slot.1

racadm storage secureerase:Disk.Bay.6:Enclosure.Internal.0-1:PCIeExtender.Slot.1

racadm storage ccheck:Disk.Virtual.0:RAID.Integrated.1-1

racadm storage cancelcheck:Disk.Virtual.0:RAID.Integrated.1-1

For help on viewing the properties of a group, run the command - racadm help get

For help on configuring the properties of a group, run the command - racadm help set

-----------------------------------------------------------------------

[support@centos00 20150805-1830]$

[support@centos00 20150805-1830]$ /opt/dell/srvadmin/bin/idracadm7 -r 10.219.9.60 -u root -p ${IDRACPASS} systemperfstatistics view

Security Alert: Certificate is invalid - self signed certificate

Continuing execution. Use -S option for racadm to stop execution on certificate-related errors.

[key = iDRAC.Embedded.1#SystemBoardCPUUsageStat]

[key = iDRAC.Embedded.1#SystemBoardIOUsageStat]

[key = iDRAC.Embedded.1#SystemBoardMEMUsageStat]

[key = iDRAC.Embedded.1#SystemBoardSYSUsageStat]

[support@centos00 20150805-1830]$ /opt/dell/srvadmin/bin/idracadm7 -r 10.219.9.60 -u root -p ${IDRACPASS} systemperfstatistics iDRAC.Embedded.1#SystemBoardIOUsageStat

Security Alert: Certificate is invalid - self signed certificate

Continuing execution. Use -S option for racadm to stop execution on certificate-related errors.

Minimum Readings

Last Hour    = 3% [At Fri, 07 Aug 2015 18:01:06]

Last Day     = 0% [At Thu, 30 Jul 2015 12:51:31]

Last Week    = 0% [At Wed, 31 Dec 1969 18:00:00]

Maximum Readings

Last Hour    = 4% [At Fri, 07 Aug 2015 18:02:19]

Last Day     = 4% [At Fri, 07 Aug 2015 16:39:23]

Last Week    = 4% [At Fri, 07 Aug 2015 16:39:23]

Average Readings

Last Hour    = 3%

Last Day     = 0%

Last Week    = 0%

Peak Readings

Last Week    4% [At Fri, 07 Aug 2015 16:39:23]

[support@centos00 20150805-1830]$ /opt/dell/srvadmin/bin/idracadm7 -r 10.219.9.60 -u root -p ${IDRACPASS} systemperfstatistics iDRAC.Embedded.1#SystemBoardMEMUsageStat

Security Alert: Certificate is invalid - self signed certificate

Continuing execution. Use -S option for racadm to stop execution on certificate-related errors.

Minimum Readings

Last Hour    = 5% [At Fri, 07 Aug 2015 18:01:06]

Last Day     = 0% [At Thu, 30 Jul 2015 12:51:31]

Last Week    = 0% [At Wed, 31 Dec 1969 18:00:00]

Maximum Readings

Last Hour    = 6% [At Fri, 07 Aug 2015 17:58:40]

Last Day     = 6% [At Fri, 07 Aug 2015 16:52:48]

Last Week    = 6% [At Fri, 07 Aug 2015 16:52:48]

Average Readings

Last Hour    = 5%

Last Day     = 0%

Last Week    = 0%

Peak Readings

Last Week    6% [At Fri, 07 Aug 2015 16:52:48]

[support@centos00 20150805-1830]$

[root@vm-vol-nag01t nagios]# /usr/lib64/nagios/plugins/check_openmanage --help

Usage: check_openmanage [OPTION]...

GENERAL OPTIONS:

-f, --config         Specify configuration file

-p, --perfdata       Output performance data [default=no]

-t, --timeout        Plugin timeout in seconds [default=30]

-c, --critical       Custom temperature critical limits

-w, --warning        Custom temperature warning limits

-F, --fahrenheit     Use Fahrenheit as temperature unit

-d, --debug          Debug output, reports everything

-h, --help           Display this help text

-V, --version        Display version info

SNMP OPTIONS:

-H, --hostname       Hostname or IP (required for SNMP)

-C, --community      SNMP community string [default=public]

-P, --protocol       SNMP protocol version [default=2c]

--port               SNMP port number [default=161]

-6, --ipv6           Use IPv6 instead of IPv4 [default=no]

--tcp                Use TCP instead of UDP [default=no]

OUTPUT OPTIONS:

-i, --info           Prefix any alerts with the service tag

-e, --extinfo        Append system info to alerts

-s, --state          Prefix alerts with alert state

-S, --short-state    Prefix alerts with alert state abbreviated

-o, --okinfo         Verbosity when check result is OK

-B, --show-blacklist Show blacklistings in OK output

-I, --htmlinfo       HTML output with clickable links

CHECK CONTROL AND BLACKLISTING:

-a, --all            Check everything, even log content

-b, --blacklist      Blacklist missing and/or failed components

--only               Only check a certain component or alert type

--check              Fine-tune which components are checked

--no-storage         Don't check storage

--vdisk-critical     Make any alerts on virtual disks critical

For more information and advanced options, see the manual page or URL:

http://folk.uio.no/trondham/software/check_openmanage.html

[root@vm-vol-nag01t nagios]#

[root@vm-vol-nag01t nagios]# /usr/lib64/nagios/plugins/check_openmanage -H m1-hadoop-wk01t -C tcspub -P 1 -p -d

System:      PowerEdge R730xd         OMSA version:    8.1.0

ServiceTag:  1J7V762                  Plugin version:  3.7.12

BIOS/date:   1.3.6 06/03/2015         Checking mode:   SNMPv1 UDP/IPv4

-----------------------------------------------------------------------------

Storage Components

=============================================================================

STATE  |    ID    |  MESSAGE TEXT

---------+----------+--------------------------------------------------------

OK |        0 | Controller 0 [PERC H730P Mini] is Ready

OK |  0:0:1:0 | Physical Disk 0:1:0 [SAS-HDD 4.0TB] on ctrl 0 is Non-RAID

OK |  0:0:1:1 | Physical Disk 0:1:1 [SAS-HDD 4.0TB] on ctrl 0 is Non-RAID

OK |  0:0:1:2 | Physical Disk 0:1:2 [SAS-HDD 4.0TB] on ctrl 0 is Non-RAID

OK |  0:0:1:3 | Physical Disk 0:1:3 [SAS-HDD 4.0TB] on ctrl 0 is Non-RAID

OK |  0:0:1:4 | Physical Disk 0:1:4 [SAS-HDD 4.0TB] on ctrl 0 is Non-RAID

OK |  0:0:1:5 | Physical Disk 0:1:5 [SAS-HDD 4.0TB] on ctrl 0 is Non-RAID

OK |  0:0:1:6 | Physical Disk 0:1:6 [SAS-HDD 4.0TB] on ctrl 0 is Non-RAID

OK |  0:0:1:7 | Physical Disk 0:1:7 [SAS-HDD 4.0TB] on ctrl 0 is Non-RAID

OK |  0:0:1:8 | Physical Disk 0:1:8 [SAS-HDD 4.0TB] on ctrl 0 is Non-RAID

OK |  0:0:1:9 | Physical Disk 0:1:9 [SAS-HDD 4.0TB] on ctrl 0 is Non-RAID

OK | 0:0:1:10 | Physical Disk 0:1:10 [SAS-HDD 4.0TB] on ctrl 0 is Non-RAID

OK | 0:0:1:11 | Physical Disk 0:1:11 [SAS-HDD 4.0TB] on ctrl 0 is Non-RAID

OK | 0:0:1:12 | Physical Disk 0:1:12 [SAS-HDD 500GB] on ctrl 0 is Online

OK | 0:0:1:13 | Physical Disk 0:1:13 [SAS-HDD 500GB] on ctrl 0 is Online

OK |      0:0 | Logical Drive '/dev/sdm' [RAID-1, 465.25 GB] is Ready

OK |      0:0 | Cache Battery 0 in controller 0 is Ready

OK |      0:0 | Connector 0 [SAS] on controller 0 is Ready

OK |    0:0:1 | Enclosure 0:0:1 [Backplane] on controller 0 is Ready

-----------------------------------------------------------------------------

Chassis Components

=============================================================================

STATE  |  ID  |  MESSAGE TEXT

---------+------+------------------------------------------------------------

OK |    0 | Memory module 0 [A1, 16384 MB] is Ok

OK |    1 | Memory module 1 [A2, 16384 MB] is Ok

OK |    2 | Memory module 2 [A3, 16384 MB] is Ok

OK |    3 | Memory module 3 [A4, 16384 MB] is Ok

OK |    4 | Memory module 4 [A5, 16384 MB] is Ok

OK |    5 | Memory module 5 [A6, 16384 MB] is Ok

OK |    6 | Memory module 6 [A7, 16384 MB] is Ok

OK |    7 | Memory module 7 [A8, 16384 MB] is Ok

OK |    8 | Memory module 8 [B1, 16384 MB] is Ok

OK |    9 | Memory module 9 [B2, 16384 MB] is Ok

OK |   10 | Memory module 10 [B3, 16384 MB] is Ok

OK |   11 | Memory module 11 [B4, 16384 MB] is Ok

OK |   12 | Memory module 12 [B5, 16384 MB] is Ok

OK |   13 | Memory module 13 [B6, 16384 MB] is Ok

OK |   14 | Memory module 14 [B7, 16384 MB] is Ok

OK |   15 | Memory module 15 [B8, 16384 MB] is Ok

OK |    0 | Chassis fan 0 [System Board Fan1 RPM] reading: 6840 RPM

OK |    1 | Chassis fan 1 [System Board Fan2 RPM] reading: 6840 RPM

OK |    2 | Chassis fan 2 [System Board Fan3 RPM] reading: 6960 RPM

OK |    3 | Chassis fan 3 [System Board Fan4 RPM] reading: 6480 RPM

OK |    4 | Chassis fan 4 [System Board Fan5 RPM] reading: 7200 RPM

OK |    5 | Chassis fan 5 [System Board Fan6 RPM] reading: 6600 RPM

OK |    0 | Power Supply 0 [AC]: Presence detected

OK |    1 | Power Supply 1 [AC]: Presence detected

OK |    0 | Temperature Probe 0 [System Board Inlet Temp] reads 24 C (min=3/-7, max=42/47)

OK |    1 | Temperature Probe 1 [System Board Exhaust Temp] reads 48 C (max=70/75)

OK |    2 | Temperature Probe 2 [CPU1 Temp] reads 62 C (min=8/3, max=82/87)

OK |    3 | Temperature Probe 3 [CPU2 Temp] reads 65 C (min=8/3, max=82/87)

OK |    0 | Processor 0 [Intel Xeon E5-2640 v3 2.60GHz] is Present

OK |    1 | Processor 1 [Intel Xeon E5-2640 v3 2.60GHz] is Present

OK |    0 | Voltage sensor 0 [CPU1 VCORE PG] is Good

OK |    1 | Voltage sensor 1 [CPU2 VCORE PG] is Good

OK |    2 | Voltage sensor 2 [System Board 3.3V PG] is Good

OK |    3 | Voltage sensor 3 [System Board 5V AUX PG] is Good

OK |    4 | Voltage sensor 4 [CPU2 M23 VPP PG] is Good

OK |    5 | Voltage sensor 5 [CPU1 M23 VPP PG] is Good

OK |    6 | Voltage sensor 6 [System Board 1.05V PG] is Good

OK |    7 | Voltage sensor 7 [System Board BP0 5V PG] is Good

OK |    8 | Voltage sensor 8 [CPU1 M23 VDDQ PG] is Good

OK |    9 | Voltage sensor 9 [CPU1 M23 VTT PG] is Good

OK |   10 | Voltage sensor 10 [System Board 5V SWITCH PG] is Good

OK |   11 | Voltage sensor 11 [System Board DIMM PG] is Good

OK |   12 | Voltage sensor 12 [System Board VCCIO PG] is Good

OK |   13 | Voltage sensor 13 [CPU2 M01 VDDQ PG] is Good

OK |   14 | Voltage sensor 14 [CPU1 M01 VDDQ PG] is Good

OK |   15 | Voltage sensor 15 [CPU2 M23 VTT PG] is Good

OK |   16 | Voltage sensor 16 [CPU2 M01 VTT PG] is Good

OK |   17 | Voltage sensor 17 [System Board NDC PG] is Good

OK |   18 | Voltage sensor 18 [CPU2 M01 VPP PG] is Good

OK |   19 | Voltage sensor 19 [CPU1 M01 VPP PG] is Good

OK |   20 | Voltage sensor 20 [CPU2 M23 VDDQ PG] is Good

OK |   21 | Voltage sensor 21 [System Board 1.5V PG] is Good

OK |   22 | Voltage sensor 22 [PS2 PG Fail] is Good

OK |   23 | Voltage sensor 23 [System Board PS1 PG Fail] is Good

OK |   24 | Voltage sensor 24 [System Board BP1 5V PG] is Good

OK |   25 | Voltage sensor 25 [System Board 1.5V AUX PG] is Good

OK |   26 | Voltage sensor 26 [CPU1 M01 VTT PG] is Good

OK |   27 | Voltage sensor 27 [PS1 Voltage 1] reads 230 V

OK |   28 | Voltage sensor 28 [PS2 Voltage 2] reads 230 V

OK |   29 | Voltage sensor 29 [CPU1 FIVR PG] is Good

OK |   30 | Voltage sensor 30 [CPU2 FIVR PG] is Good

OK |   31 | Voltage sensor 31 [System Board 2.5V AUX PG] is Good

OK |    0 | Battery probe 0 [System Board CMOS Battery] is Presence Detected

OK |    0 | Amperage probe 0 [PS1 Current 1] reads 1.4 A

OK |    1 | Amperage probe 1 [PS2 Current 2] reads 0.2 A

OK |    2 | Amperage probe 2 [System Board Pwr Consumption] reads 322 W

OK |    0 | Chassis intrusion 0 detection: Ok (Not Breached)

OK |    0 | SD Card 0 [vFlash] is Absent

-----------------------------------------------------------------------------

Other messages

=============================================================================

STATE  |  MESSAGE TEXT

---------+-------------------------------------------------------------------

OK | ESM log health is Ok (less than 80% full)

OK | Chassis Service Tag is sane

[root@vm-vol-nag01t nagios]#

http://en.community.dell.com/techcenter/extras/m/white_papers/20118781

[root@centos00 yum.repos.d]# yum search all wsmancli

Loaded plugins: downloadonly, refresh-packagekit, security

============================================================================================ Matched: wsmancli =============================================================================================

wsmancli.x86_64 : WS-Management-Command line Interface

[root@centos00 yum.repos.d]# yum install wsmancli.x86_64

Loaded plugins: downloadonly, refresh-packagekit, security

Setting up Install Process

Resolving Dependencies

--> Running transaction check

---> Package wsmancli.x86_64 0:2.2.3-1.el6 will be installed

--> Processing Dependency: openwsman for package: wsmancli-2.2.3-1.el6.x86_64

--> Processing Dependency: libwsman_curl_client_transport.so.1()(64bit) for package: wsmancli-2.2.3-1.el6.x86_64

--> Processing Dependency: libwsman_client.so.1()(64bit) for package: wsmancli-2.2.3-1.el6.x86_64

--> Processing Dependency: libwsman.so.1()(64bit) for package: wsmancli-2.2.3-1.el6.x86_64

--> Running transaction check

---> Package libwsman1.x86_64 0:2.2.3.9-4.1.2.el6 will be installed

--> Finished Dependency Resolution

Dependencies Resolved

============================================================================================================================================================================================================

Package                                     Arch                                     Version                                                Repository                                                Size

============================================================================================================================================================================================================

Installing:

wsmancli                                    x86_64                                   2.2.3-1.el6                                            tcs-centos-6-os-x86_64                                    21 k

Installing for dependencies:

libwsman1                                   x86_64                                   2.2.3.9-4.1.2.el6                                      tcs-dell                                                 335 k

Transaction Summary

============================================================================================================================================================================================================

Install       2 Package(s)

Total download size: 356 k

Installed size: 946 k

Is this ok [y/N]: y

Downloading Packages:

(1/2): libwsman1-2.2.3.9-4.1.2.el6.x86_64.rpm                                                                                                                                        | 335 kB     00:00

(2/2): wsmancli-2.2.3-1.el6.x86_64.rpm                                                                                                                                               |  21 kB     00:00

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Total                                                                                                                                                                       4.7 MB/s | 356 kB     00:00

Running rpm_check_debug

Running Transaction Test

Transaction Test Succeeded

Running Transaction

Installing : libwsman1-2.2.3.9-4.1.2.el6.x86_64                                                                                                                                                       1/2

Installing : wsmancli-2.2.3-1.el6.x86_64                                                                                                                                                              2/2

Verifying  : wsmancli-2.2.3-1.el6.x86_64                                                                                                                                                              1/2

Verifying  : libwsman1-2.2.3.9-4.1.2.el6.x86_64                                                                                                                                                       2/2

Installed:

wsmancli.x86_64 0:2.2.3-1.el6

Dependency Installed:

libwsman1.x86_64 0:2.2.3.9-4.1.2.el6

Complete!

[root@centos00 yum.repos.d]#

[support@centos00 20150805-1830]$ cp /media/sf_share/dell/idrac_attribs.zip .

[support@centos00 20150805-1830]$ unzip idrac_attribs.zip

Archive:  idrac_attribs.zip

creating: idrac/

inflating: idrac/IDRAC0.01.xml

inflating: idrac/pullattr.py

inflating: idrac/pulliDRAC.sh

inflating: idrac/pushattr.py

inflating: idrac/pushiDRAC.sh

inflating: idrac/README.idrac.txt

extracting: Archive created by free jZip.url

[support@centos00 20150805-1830]$

[support@centos00 20150805-1830]$ cd idrac

[support@centos00 idrac]$ . ./pull

pullattr.py   pulliDRAC.sh

[support@centos00 idrac]$ . ./pulliDRAC.sh

IP address (ENTER for default: ) ? 10.219.9.60

User name (ENTER for default: ) ? root

User password (ENTER for default: ) ? calvin

Building the Order Attributes Template File...

Getting the Attributes ....

pullattr.py:222: DeprecationWarning: os.popen3 is deprecated.  Use the subprocess module.

(fin, fout, ferr) = os.popen3(command, "t")

Creating the .ini file ....

File Created: 10.219.9.60_idrac.ini

Adding Attributes to the file...

[support@centos00 idrac]$

| Subcommand | Definition |
| --- | --- |
| serveraction | Executes a managed system reset or power-on/off/cycle. |

### Synopsis

racadm serveraction <action>

### Description

The serveraction subcommand enables users to perform power management operations on the host system. Table A-28 describes the serveraction power control options.

Table A-28. serveraction Subcommand Options

| String | Definition |
| --- | --- |
| <action> | Specifies the action. The options for the <action> string are: - powerdown — Powers down the managed system. - powerup — Powers up the managed system. - powercycle — Issues a power-cycle operation on the managed system. This action is similar to pressing the power button on the system's front panel to power down and then power up the system. - powerstatus — Displays the current power status of the server ("ON", or "OFF") - hardreset — Performs a reset (reboot) operation on the managed system. |

racreset

Legacy Scripts

http://en.community.dell.com/techcenter/systems-management/w/wiki/7654.legacy-scripts

How to setup and manage your iDRAC7 and Lifecycle Controller for Dell PowerEdge 12G server

http://www.dell.com/support/article/us/en/19/SLN114824/EN

RACADM Subcommand Overview

http://public.support.unisys.com/pcproducts/esx/docs/DellDocs5.4/en/drac5/racugaa.htm#51808

http://www.gooksu.com/2015/04/27/racadm-quick-dirty-cheatsheet/

racadm quick dirty cheatsheet

Posted on April 27, 2015 by jlim0930

iDRAC racadm quick and dirty cheatsheet. racadm command can be issues via iDRAC/CMC/OS if svradmin-racadm is installed. Also you can specify -h option to access remote servers RAC as long as you have network access. Also if you are having problems with racadm “Failed to initialize transport” install openssl-devel. full documentation for iDRAC7 can be found here.

% Get all iDRAC settings in a file

racadm get -f config.txt

If you like you can change the contents of config.txt and apply it back to iDRAC

racadm set -f config.txt

% Set password for root user

racadm set iDRAC.Users.2.Password PASSWORD"

% List all ssh keys for root user

racadm sshpkauth -i 2 -v -k all

% Add ssh key to root user

racadm sshpkauth -i 2 -k 1 "CONTENTS OF PUBLIC KEY"

% Delete ssh key for root user

racadm sshpkauth -i 2 -d -k 1

% Get iDRAC IP config

racadm getniccfg

racadm get iDRAC.NIC

% set iDRAC IP

Using config command:

racadm config -g cfgLanNetworking -o cfgNicEnable 1

racadm config -g cfgLanNetworking -o cfgNicIpAddress x.x.x.x

racadm config -g cfgLanNetworking -o cfgNicNetmask 255.255.255.0

racadm config -g cfgLanNetworking -o cfgNicGateway x.x.x.x

racadm config -g cfgLanNetworking -o cfgNicUseDHCP 0

racadm config -g cfgLanNetworking -o cfgDNSServersFromDHCP 0

racadm config -g cfgLanNetworking -o cfgDNSServer1 y.y.y.y

racadm config -g cfgLanNetworking -o cfgDNSServer2 y.y.y.y

• Using set command:

racadm set iDRAC.Nic.Enable 1

racadm set iDRAC.IPv4.Address x.x.x.x

racadm set iDRAC.IPv4.Netmask 255.255.255.0

racadm set iDRAC.IPv4.Gateway x.x.x.x

racadm set iDRAC.IPv4.DHCPEnable 0

racadm set iDRAC.IPv4.DNSFromDHCP 0

racadm set iDRAC.IPv4.DNS1 y.y.y.y

racadm set iDRAC.IPv4.DNS2 y.y.y.y

% Set iDRAC DNS Name

racadm set iDRAC.NIC.DNSRacName iDRACNAME

% Set iDRAC domain name

racadm set iDRAC.NIC.DNSDomainName DOMAIN.NAME

% Set iDRAC DNS Server

racadm config -g cfgLanNetworking -o cfgDNSServer1 x.x.x.x

racadm config -g cfgLanNetworking -o cfgDNSServer2 y.y.y.y

% Set Front LCD to hostname

racadm set System.LCD.Configuration 16

% Reset iDRAC to factory defaults

racadm racresetcfg

% Reset/Reboot iDRAC

racadm racreset OPTIONS

Options : soft, hard, cold

or

racadm serveraction powercycle

% Get Serial number (service tag)

racadm getsvctag

% Get current system information

racadm getsysinfo

% Configure one-time-boot to PXE

racadm set BIOS.OneTimeBoot.OneTimeBootMode OneTimeBootSeq

racadm set BIOS.OneTimeBoot.OneTimeBootSeqDev NIC.Integrated.1-1-1

% Configure persistent Boot Device

racadm config -g cfgServerInfo -o cfgServerBootOnce 0

racadm config -g cfgServerInfo -i cfgServerFirstBootDevice HDD

% Check boot order list

racadm get BIOS.BiosBootSettings.bootseq

% Disable HyperThreading

racadm set BIOS.ProcSettings.LogicalProc Disabled

% Disable OS to iDRAC pass-thru for iDRAC service module (automatically create a pseudo NIC in OS)

racadm set iDRAC.OS-BMC.AdminState Disabled

% Change SNMP public community string

racadm set iDRAC.SNMP.AgentCommunity NEW STRING

% Disable ASR

racadm config -g cfgRacTuning -o cfgRacTuneAsrEnable 0

% Configure Serial redirection

racadm config -g cfgSerial -o cfgSerialConsoleEnable 1

racadm config -g cfgSerial -o cfgSerialBaudRate 115200

racadm config -g cfgSerial -o cfgSerialCom2RedirEnable 1

racadm config -g cfgSerial -o cfgSerialTelnetEnable 0

racadm config -g cfgSerial -o cfgSerialSshEnable 1

to access console via ssh console com2

% Disable Serial On Lan

racadm config -g cfgImpiSol -o cfgIpmiSolEnable 0

% Change Power Profile

racadm set BIOS.SysProfileSettings PerfPerWattOptimizedOs

% Set AC Power Recovery

racadm set BIOS.SysSecurity.AcPwdRcvry Last

racadm set BIOS.SysSecurity.AcPwdRcvryDelay Immediate

% Get RAID physical Disk information

racadm raid get pdisks

racadm raid get pdisks -o (all information)

racadm raid get pdisks -o -p state,size (specific information)

% Get RAID Virtual Disk Information

Racadm raid get vdisks

Be Sociable, Share!

iDRAC 7 CheatSheet

http://www.gnali.org/?p=435

# Get all iDRAC settings in a file

?

| 1 | racadm get -f config.txt |
| --- | --- |

# Set password for root user

?

| 1 | racadm set iDRAC.Users.2.Password "password" |
| --- | --- |

# List all ssh key for root user

?

| 1 | racadm sshpkauth -i 2 -v -k all |
| --- | --- |

# Add ssh key to root user

?

| 1 | racadm sshpkauth -i 2 -k 1 "contents of the public key file" |
| --- | --- |

# Delete ssh key for root user

?

| 1 | racadm sshpkauth -i 2 -d -k 1 |
| --- | --- |

# Get iDRAC IP config

?

| 1 2 | racadm getniccfg racadm get iDRAC.NIC |
| --- | --- |

# Set iDRAC DNS Name

?

| 1 | racadm set iDRAC.NIC.DNSRacName rougerie-rsc |
| --- | --- |

# Set iDRAC domain Name

?

| 1 | racadm set iDRAC.NIC.DNSDomainName gnali.org |
| --- | --- |

# Set iDRAC DNS server

?

| 1 2 | racadm config -g cfgLanNetworking -o cfgDNSServer1 191.254.31.175 racadm config -g cfgLanNetworking -o cfgDNSServer2 184.7.12.44 |
| --- | --- |

## Set FrontLCD to hostname

?

| 1 | racadm set System.LCD.Configuration 16 |
| --- | --- |

# Reset iDRAC to factory defaults

?

| 1 | racadm racresetcfg |
| --- | --- |

#Reset iDRAC

?

| 1 | racadm racreset |
| --- | --- |

# Get serial number (service tag)

?

| 1 | racadm getsvctag |
| --- | --- |

# Get current system information

?

| 1 | racadm getsysinfo |
| --- | --- |

# Configure One-Time-Boot

?

| 1 2 | racadm set BIOS.OneTimeBoot.OneTimeBootMode OneTimeBootSeq racadm set BIOS.OneTimeBoot.OneTimeBootSeqDev NIC.Integrated.1-1-1 |
| --- | --- |

# Create Commit and Reboot jobqueue to apply BIOS or NIC Configuration

?

| 1 | racadm jobqueue create BIOS.Setup.1-1 -r pwrcycle -s TIME_NOW -e TIME_NA |
| --- | --- |

# Create Commit jobqueue without Reboot

?

| 1 | racadm jobqueue create BIOS.Setup.1-1 |
| --- | --- |

# Delete a jobqueue

?

| 1 | racadm jobqueue delete -i &lt;job_ID&gt; |
| --- | --- |

# List all Jobqueue

?

| 1 | racadm jobqueue view |
| --- | --- |

# Check Boot Order List

?

| 1 | racadm get BIOS.BiosBootSettings.bootseq |
| --- | --- |

# Disable HyperThreadings

?

| 1 | racadm set BIOS.ProcSettings.LogicalProc Disabled |
| --- | --- |

# Disabled OS to iDRAC Pass-through for iDRAC Service Module (automatically create a pseudo NIC in OS)

?

| 1 | racadm set iDRAC.OS-BMC.AdminState Disabled |
| --- | --- |

# Change SNMP public community

?

| 1 | racadm set iDRAC.SNMP.AgentCommunity unixplop |
| --- | --- |

# Disable ASR

?

| 1 | racadm config -g cfgRacTuning -o cfgRacTuneAsrEnable 0 |
| --- | --- |

# Configure Serial redirection

?

| 1 2 3 4 5 | racadm config -g cfgSerial -o cfgSerialConsoleEnable 1 racadm config -g cfgSerial -o cfgSerialBaudRate 115200 racadm config -g cfgSerial -o cfgSerialCom2RedirEnable 1 racadm config -g cfgSerial -o cfgSerialTelnetEnable 0 racadm config -g cfgSerial -o cfgSerialSshEnable 1 |
| --- | --- |

# Access ssh console

?

| 1 2 | ssh root@rouerie-rsc console com2 |
| --- | --- |

# Disable Serial On Lan

?

| 1 | racadm config -g cfgIpmiSol -o cfgIpmiSolEnable 0 |
| --- | --- |

# Help on specific section

?

| 1 | racadm help BIOS.SysProfileSettings |
| --- | --- |

# Change Power Profile

?

| 1 | racadm set BIOS.SysProfileSettings PerfPerWattOptimizedOs |
| --- | --- |

# Set AC Power Recovery

?

| 1 2 | racadm set BIOS.SysSecurity.AcPwrRcvry Last racadm set BIOS.SysSecurity.AcPwrRcvryDelay Immediate |
| --- | --- |

# Get RAID physical Disk information

?

| 1 2 3 | racadm raid get pdisks racadm raid get pdisks -o (all information) racadm raid get pdisks -o -p state,size (specific information) |
| --- | --- |

# Get RAID Virtual Disk Information

?

| 1 | racadm raid get vdisks |
| --- | --- |

# Configuration Web Virtual console to use JAVA

[support@centos00 20150805-1830]$ /opt/dell/srvadmin/bin/idracadm7 -r 10.219.9.60 -u root -p ${IDRACPASS} get BIOS.SysProfileSettings

Security Alert: Certificate is invalid - self signed certificate

Continuing execution. Use -S option for racadm to stop execution on certificate-related errors.

[Key=BIOS.Setup.1-1#SysProfileSettings]

#CollaborativeCpuPerfCtrl=Disabled

#EnergyEfficientTurbo=Disabled

#EnergyPerformanceBias=MaxPower

#MemFrequency=MaxPerf

#MemPatrolScrub=Standard

#MemRefreshRate=1x

#MonitorMwait=Enabled

PowerSaver=Disabled

#Proc1TurboCoreNum=All

#Proc2TurboCoreNum=All

#ProcC1E=Disabled

#ProcCStates=Disabled

#ProcPwrPerf=MaxPerf

#ProcTurboMode=Enabled

SysProfile=PerfOptimized

#UncoreFrequency=MaxUFS

WorkloadProfile=NotAvailable

[support@centos00 20150805-1830]$

20150805-2024.10.219.9.60.server.profile.backup.img

[s.belogrud.tcs4500] ➤ ls -Alh /drives/d/share/temp/20150805-2008/

total 60040

-rwxrwxrwx    1 ▒▒▒▒▒▒▒▒ UsersGrp  117.3M Aug  5 20:38 20150805-2024.10.219.9.60.server.profile.backup.img

Repository

http://linux.dell.com/repo/community/repositories/isv:/dell:/community/el6-x86_64/x86_64/

https://frednotes.wordpress.com/2012/11/13/reset-dell-idrac-using-ssh/

## Reset DELL idrac using SSH

Posted on 2012/11/13by Fred

How to reboot an DELL idrac when web page refuse access:

Connect to idrac IP using ssh with the password refused by web page

$ ssh root@192.168.0.120
root@192.168.0.120's password:

The magic DELL tool is here : radadm

/admin1-> racadm
===============================================================================
 RACADM version 1.80 (Build 17)
 Copyright (c) 2003-2010 Dell, Inc.
 All Rights Reserved
 ===============================================================================
RACADM usage syntax:
racadm <subcommand> <options>
Examples:
racadm getsysinfo
 racadm getsysinfo -d
 racadm getniccfg
 racadm setniccfg -d
 racadm setniccfg -s 192.168.0.120 255.255.255.0 192.168.0.1
 racadm getconfig -g cfgLanNetworking
Display a list of available subcommands for the RAC:
racadm help
Display more detailed help for a specific subcommand:
racadm help <subcommand>
------------------------------------------------------------------------------

So to reboot :

/admin1-> racadm racreset soft
RAC reset operation initiated successfully. It may take up to a minute
for the RAC to come back online again.
/admin1-> Connection to 192.168.0.120 closed by remote host.
Connection to 192.168.0.120 closed.

http://thornelabs.net/2014/09/03/reconfigure-a-dell-raid-perc-controller-with-racadm.html

# Reconfigure a Dell RAID (PERC) Controller with racadm

Wednesday, September 3, 2014

While setting up an OpenStack Swift cluster, I had five Dell R720 servers with incorrectly configured RAID arrays. The two operating system hard drives were configured in a RAID1 array, as expected, but the 24 data hard drives were configured in two RAID5 arrays. The data hard drives in the OpenStack Swift storage nodes should not be in a RAID array; Swift simply wants each hard drive on its own.

You can reboot most of Dell’s enterprise servers into the Dell RAID (PERC) Controller to get access to a GUI to re-configure the RAID arrays. But, with five servers and 24 disks on each, using the GUI would have been a very tedious and time consuming task. So, I began looking into what the racadm command was capable of.

You can SSH to the IP address of a Dell iDRAC and use the racadm command to run all sorts of commands. Luckily, racadm has a set of commands that allow you to reconfigure the RAID arrays.

Aftering SSH’ing into the Dell iDRAC, I needed to get the names of the existing virtual disks:

racadm raid get vdisks

The command returned the following output:

Disk.Virtual.0:RAID.Integrated.1-1
Disk.Virtual.1:RAID.Integrated.1-1
Disk.Virtual.2:RAID.Integrated.1-1

Disk.Virtual.0:RAID.Integrated.1-1 was the RAID1 array containing the two operating system hard drives. That did not need to be changed. The other two virtual disks were the incorrectly configured RAID5 arrays. Those two virtual disks needed to be deleted:

racadm raid deletevd:Disk.Virtual.1:RAID.Integrated.1-1
racadm raid deletevd:Disk.Virtual.2:RAID.Integrated.1-1

Even though I issued the delete command, nothing had actually happened. For those commands to actually run, I needed to put them into a job:

racadm jobqueue create RAID.Integrated.1-1

With the job created, I verified there was actually a job pending:

racadm jobqueue view

Then, to run the job, I power cycled the server:

racadm serveraction powercycle

During the boot process, the Lifecycle Controller GUI came up and performed the tasks within the job. Once the Lifecycle Controller GUI closed, the job was completed.

When I re-ran the racadm raid get vdisks command, I only had one virtual disk, as expected:

Disk.Virtual.0:RAID.Integrated.1-1

Next, I needed to reconfigure the 24 hard drives, now not tied to any virtual disks.

First, I needed the names of the physical disks:

racadm raid get pdisks

The command returned the following output:

Disk.Bay.0:Enclosure.Internal.0-1:RAID.Integrated.1-1
Disk.Bay.1:Enclosure.Internal.0-1:RAID.Integrated.1-1
Disk.Bay.2:Enclosure.Internal.0-1:RAID.Integrated.1-1
... truncated ...
Disk.Bay.23:Enclosure.Internal.0-1:RAID.Integrated.1-1
Disk.Bay.24:Enclosure.Internal.0-1:RAID.Integrated.1-1
Disk.Bay.25:Enclosure.Internal.0-1:RAID.Integrated.1-1

In this particular case, Disk.Bay.24 and Disk.Bay.25 were the two operating system drives in the RAID1 array. Just like earlier, those hard drives did not need to be reconfigured. However, Disk.Bay.0 through Disk.Bay.23 did need to be reconfigured.

For whatever reason, The Dell RAID (PERC) Controller does not allow you to create a JBOD, or in other words, have each hard drive exist on its own outside of a virtual disk. If I left the hard drives as they were, not part of any virtual disks, the Dell RAID (PERC) Controller did not present them to the operating system. So, my only option was to create 24 virtual disks with 1 disk in each in a RAID0. This was done with the following commands:

racadm raid createvd:RAID.Integrated.1-1 -rl r0 -pdkey:Disk.Bay.0:Enclosure.Internal.0-1:RAID.Integrated.1-1
racadm raid createvd:RAID.Integrated.1-1 -rl r0 -pdkey:Disk.Bay.1:Enclosure.Internal.0-1:RAID.Integrated.1-1
racadm raid createvd:RAID.Integrated.1-1 -rl r0 -pdkey:Disk.Bay.2:Enclosure.Internal.0-1:RAID.Integrated.1-1
... truncated ...
racadm raid createvd:RAID.Integrated.1-1 -rl r0 -pdkey:Disk.Bay.21:Enclosure.Internal.0-1:RAID.Integrated.1-1
racadm raid createvd:RAID.Integrated.1-1 -rl r0 -pdkey:Disk.Bay.22:Enclosure.Internal.0-1:RAID.Integrated.1-1
racadm raid createvd:RAID.Integrated.1-1 -rl r0 -pdkey:Disk.Bay.23:Enclosure.Internal.0-1:RAID.Integrated.1-1

Once again, even though I issued the create command, nothing had actually happened. For those commands to actually run, I needed to put them into another job:

racadm jobqueue create RAID.Integrated.1-1

Again, with the job created, I verified there was actually a job pending:

racadm jobqueue view

And, to run the job, I power cycled the server:

racadm serveraction powercycle

Once again, during the boot process, the Lifecycle Controller GUI came up and performed the tasks within the job. Once the Lifecycle Controller GUI closed, the job was completed.

Now, when I ran racadm raid get vdisks I had 25 virtual disks:

Disk.Virtual.0:RAID.Integrated.1-1
Disk.Virtual.1:RAID.Integrated.1-1
Disk.Virtual.2:RAID.Integrated.1-1
... truncated ...
Disk.Virtual.22:RAID.Integrated.1-1
Disk.Virtual.23:RAID.Integrated.1-1
Disk.Virtual.24:RAID.Integrated.1-1

When I logged into the operating system, in this particular case RHEL 6.5, and ran fdisk -l, I saw /dev/sda through/dev/sdy.

Because /dev/sda contained the operating system, it had already been partitioned and formatted. But, with all the of RAID re-configuration, I was then able to partition and format /dev/sdb through /dev/sdy as needed.

http://thornelabs.net/2014/04/16/dell-idrac-racadm-commands-and-scripts.html

# Dell iDRAC racadm Commands and Scripts

Wednesday, April 16, 2014

The Dell iDRAC web interface has always been cumbersome to deal with. Luckily, you can login to a Dell iDRAC via SSH and use the racadm command to configure it and perform actions against the physical server. With this functionality, it is possible to automate actions that would otherwise take too long to do via the web interface.

This will be an ever growing list of Dell iDRAC racadm commands and ways to automate repetitive tasks.

## Set Server to PXE Boot Once and Power Cycle

Login to the Dell iDRAC via SSH:

ssh root@IP_ADDRESS

Configure the server to PXE boot once and reboot immediately:

racadm config -g cfgServerInfo -o cfgServerBootOnce 1
racadm config -g cfgServerInfo -o cfgServerFirstBootDevice PXE
racadm serveraction powercycle

## Set Persistent Boot Device

Login to the Dell iDRAC via SSH:

ssh root@IP_ADDRESS

Configure the server to boot to the local hard drive every time:

racadm config -g cfgServerInfo -o cfgServerBootOnce 0
racadm config -g cfgServerInfo -o cfgServerFirstBootDevice HDD

## Change the Dell iDRAC7 System Host Name

If you need to change the Host Name field as reported by command racadm getsysinfo and the System Host Name field as shown in the Dell iDRAC7 web GUI in the following screenshot, you must install Dell OpenManage on the particular server:

The attribute that contains this value in the Dell iDRAC7 is read only and cannot be changed by the user as shown with the following command:

/admin1-> racadm config -g ifcRacManagedNodeOs -o ifcRacMnOsHostname build01
ERROR: The specified object is READ ONLY and cannot be modified.

You can install Dell OpenManage on CentOS and RHEL by following the instructions here and on Ubuntu by following the instructions here.

## Reset Dell iDRAC

If you ever run into the following error when trying to login to the Dell iDRAC via the web interface:

Dell DRAC: RAC0218: The maximum number of user sessions is reached

Login to the Dell iDRAC via SSH:

ssh root@IP_ADDRESS

Reset the Dell iDRAC (it may take a few minutes to come back online):

racadm racreset

After a few minutes you should be able to login to the web interface again.

## racadm Scripts

### Set Many Servers to PXE Boot Once and Power Cycle

If you have a handful of servers that you need to set to PXE boot once and then power cycle the server, you can do so with the following script. Be sure to input the correct password, otherwise you will be locked out of the Dell iDRAC via SSH for about 5 minutes. In addition, whatever server you run the script from, the sshpass program will need to be installed for this to work.

#!/bin/bash
echo -n "Enter iDRAC root password (password will not be displayed): "
read -s DRACPASS
echo

for IP in IP_ADDRESS_1 IP_ADDRESS_2 IP_ADDRESS_3 IP_ADDRESS_N
do
echo "Setting $IP to boot once"
sshpass -p "$DRACPASS" ssh root@$IP racadm config -g cfgServerInfo -o cfgServerBootOnce 1
echo "Setting $IP boot device to PXE"
sshpass -p "$DRACPASS" ssh root@$IP racadm config -g cfgServerInfo -o cfgServerFirstBootDevice PXE
echo "Power cycling $IP now"
sshpass -p "$DRACPASS" ssh root@$IP racadm serveraction powercycle
done

## References

Dell DRAC: RAC0218: The maximum number of user sessions is reached

http://aftershox.com/2013/07/11/how-to-configure-a-dell-idrac-card-using-the-racadm-command-line-tool/

# How to configure a Dell iDRAC card using the Racadm command line tool

Posted by Tariq Ahmed on Jul 11, 2013 in I.T Systems |  1 comment

### Background

As the Director of Technology, my teams include Software Development, Business Intelligence, and managed I.T Infrastructure services.

Having recently taken over an existing site, we’re in the process of conducting various house cleaning efforts. We came across a server that had a mis-configured iDRAC card which prevented managing it remotely or to run hardware diagnostics.

One option is to reboot the server, boot into BIOS, and configure the iDRAC while in front of the machine. However this would require a drive out the data center, and the additional maintenance window outage.

We did find a solution to all of this with the installation of Dell DRAC Tools which includes a command line tool called Racadm. When installed on the machine you want to configure, Racadm allows a Systems Administrator to modify the iDRAC settings from a command-line without requiring a reboot.

Here are the commands:

?

| 1 2 | racadm getniccfg racadm setniccfg –s 172.17.2.124 255.255.252.0 172.17.0.5 |
| --- | --- |

-OR-

?

| 1 2 3 4 5 6 7 8 | racadm getconfig -g cfgLanNetworking racadm config -g cfgLanNetworking -o cfgNicIpAddress 172.17.2.124 racadm config -g cfgLanNetworking -o cfgNicNetmask 255.255.252.0 racadm config -g cfgLanNetworking -o cfgNicGateway 172.17.0.7 racadm config -g cfgLanNetworking -o cfgDNSServer1 172.17.0.6 racadm config -g cfgLanNetworking -o cfgDNSServer2 172.17.0.5 racadm config -g cfgLanNetworking -o cfgDNSRacName ServerName-DRAC racadm config -g cfgLanNetworking -o cfgDNSDomainName corp.company.com |
| --- | --- |

Kudos to Jeremy Moreira for this information

### DRAC Tools, includes Racadm (32bit), v7.1:

The Dell Remote Access Controller (DRAC) console is management station software designed to provide remote management capabilities for the Dell systems. You can remotely connect to the DRAC hardware and access the DRAC features either by using a web browser or the RACADM Command Line Interface (CLI). RACADM CLI is the command line user interface to the DRAC.

Download DRAC Tools (v7.1):  http://www.dell.com/support/drivers/us/en/19/driverdetails?driverid=HR1V5

- Filename:OM-DRAC-Dell-Web-WIN-7.1.0-5304_A00.exe

- Compatibility: Windows Server 2003, Windows 2003 x64, Windows 2008 x86, Windows 2008 x64, Windows 2008 R2, Windows Server 2012

### RACADM Subcommand Overview:

The getconfig subcommand allows you to retrieve iDRAC configuration parameters individually, or all the iDRAC configuration groups may be retrieved and saved into a file.

#### Example:

?

| 1 | racadm config -g cfgLanNetworking -o cfgNicIpAddress 10.35.10.110 |
| --- | --- |

Sets the cfgNicIpAddress configuration parameter (object) to the value 10.35.10.110. This IP address object is contained in the group cfgLanNetworking.