20160324

#	After I turned on all snmp alerts on all serves, I start receiving a lot of messages from one of servers.

#	As I have understood, It’s mean, that some device is in usb port at front panel of server. And this device consume some current.

#	This is unsecure.

--------------------------------------------------------------------------------

SeqNumber       = 1481240

Message ID      = USR0175

Category        = Audit

AgentID         = iDRAC

Severity        = Warning

Timestamp       = 2016-03-24 10:56:24

Message         = The Front Panel USB Port Over Current is detected for the attached device on Disk.USBFront.1.

Message Arg   1 = 1

--------------------------------------------------------------------------------

#	I turned off this message sending.

#	This record helps me to find what check-box in iDRAC I should use to turn off sending these messages.

#	I thought follow way: this is “Alert”, this is “UserTracking”, and this is “Audit” category.

--------------------------------------------------------------------------------

SeqNumber       = 1481240

Message ID      = USR0175

Category        = Audit

AgentID         = iDRAC

Severity        = Warning

Timestamp       = 2016-03-24 10:56:24

Message         = The Front Panel USB Port Over Current is detected for the attached device on Disk.USBFront.1.

Message Arg   1 = 1

--------------------------------------------------------------------------------

#	Related messages into /var/log/messages were found and looks like below:

#

#   It's was taken with vim selection

[root@ds-hadoop-wk14p ~]# cat /tmp/20160324-1154.from.messages

Mar  4 17:56:07 ds-hadoop-wk14p snmpd[3379]: [smux_parse_var] Panic: type 48

Mar  4 17:57:42 ds-hadoop-wk14p dsm_ism_srvmgrd: 4893 ISM0007 [iSM@674.10892.2 EventID="8194" EventCategory="Audit" EventSeverity="warn" IsPastEvent="false" language="en-US"] The iDRAC Service Module communication with iDRAC has ended.

Mar  4 17:58:05 ds-hadoop-wk14p kernel: usb 1-1.6: USB disconnect, device number 3

Mar  4 17:58:05 ds-hadoop-wk14p kernel: usb 1-1.6.3: USB disconnect, device number 4

Mar  4 17:58:05 ds-hadoop-wk14p kernel: cdc_ether 1-1.6.3:1.0: idrac: unregister 'cdc_ether' usb-0000:00:1a.0-1.6.3, CDC Ethernet Device

Mar  4 17:58:06 ds-hadoop-wk14p ntpd[3403]: Deleting interface #8 idrac, fe80::46a8:42ff:fe35:f50f#123, interface stats: received=0, sent=0, dropped=0, active_time=10352164 secs

Mar  4 17:58:06 ds-hadoop-wk14p ntpd[3403]: Deleting interface #7 idrac, 169.254.0.2#123, interface stats: received=0, sent=0, dropped=0, active_time=10352164 secs

Mar  4 17:58:06 ds-hadoop-wk14p ntpd[3403]: peers refreshed

Mar  4 17:58:29 ds-hadoop-wk14p kernel: usb 1-1.6: new high speed USB device number 7 using ehci_hcd

Mar  4 17:58:29 ds-hadoop-wk14p kernel: usb 1-1.6: New USB device found, idVendor=413c, idProduct=a001

Mar  4 17:58:29 ds-hadoop-wk14p kernel: usb 1-1.6: New USB device strings: Mfr=1, Product=2, SerialNumber=3

Mar  4 17:58:29 ds-hadoop-wk14p kernel: usb 1-1.6: Product: Gadget USB HUB

Mar  4 17:58:29 ds-hadoop-wk14p kernel: usb 1-1.6: Manufacturer: no manufacturer

Mar  4 17:58:29 ds-hadoop-wk14p kernel: usb 1-1.6: SerialNumber: 0123456789

Mar  4 17:58:29 ds-hadoop-wk14p kernel: usb 1-1.6: configuration #1 chosen from 1 choice

Mar  4 17:58:29 ds-hadoop-wk14p kernel: hub 1-1.6:1.0: USB hub found

Mar  4 17:58:29 ds-hadoop-wk14p kernel: hub 1-1.6:1.0: 6 ports detected

Mar  4 17:58:59 ds-hadoop-wk14p kernel: usb 1-1.6.1: new high speed USB device number 8 using ehci_hcd

Mar  4 17:58:59 ds-hadoop-wk14p kernel: usb 1-1.6.1: New USB device found, idVendor=0624, idProduct=0249

Mar  4 17:58:59 ds-hadoop-wk14p kernel: usb 1-1.6.1: New USB device strings: Mfr=4, Product=5, SerialNumber=6

Mar  4 17:58:59 ds-hadoop-wk14p kernel: usb 1-1.6.1: Product: Keyboard/Mouse Function

Mar  4 17:58:59 ds-hadoop-wk14p kernel: usb 1-1.6.1: Manufacturer: Avocent

Mar  4 17:58:59 ds-hadoop-wk14p kernel: usb 1-1.6.1: SerialNumber: 20121018

Mar  4 17:58:59 ds-hadoop-wk14p kernel: usb 1-1.6.1: configuration #1 chosen from 1 choice

Mar  4 17:58:59 ds-hadoop-wk14p kernel: input: Avocent Keyboard/Mouse Function as /devices/pci0000:00/0000:00:1a.0/usb1/1-1/1-1.6/1-1.6.1/1-1.6.1:1.0/input/input2

Mar  4 17:58:59 ds-hadoop-wk14p kernel: generic-usb 0003:0624:0249.0001: input,hidraw0: USB HID v1.00 Keyboard [Avocent Keyboard/Mouse Function] on usb-0000:00:1a.0-1.6.1/input0

Mar  4 17:58:59 ds-hadoop-wk14p kernel: input: Avocent Keyboard/Mouse Function as /devices/pci0000:00/0000:00:1a.0/usb1/1-1/1-1.6/1-1.6.1/1-1.6.1:1.1/input/input3

Mar  4 17:58:59 ds-hadoop-wk14p kernel: generic-usb 0003:0624:0249.0002: input,hidraw1: USB HID v1.00 Mouse [Avocent Keyboard/Mouse Function] on usb-0000:00:1a.0-1.6.1/input1

Mar  4 17:58:59 ds-hadoop-wk14p kernel: input: Avocent Keyboard/Mouse Function as /devices/pci0000:00/0000:00:1a.0/usb1/1-1/1-1.6/1-1.6.1/1-1.6.1:1.2/input/input4

Mar  4 17:58:59 ds-hadoop-wk14p kernel: generic-usb 0003:0624:0249.0003: input,hidraw2: USB HID v1.00 Mouse [Avocent Keyboard/Mouse Function] on usb-0000:00:1a.0-1.6.1/input2

Mar  4 17:59:00 ds-hadoop-wk14p kernel: usb 1-1.6.3: new high speed USB device number 9 using ehci_hcd

Mar  4 17:59:00 ds-hadoop-wk14p kernel: usb 1-1.6.3: New USB device found, idVendor=413c, idProduct=a102

Mar  4 17:59:00 ds-hadoop-wk14p kernel: usb 1-1.6.3: New USB device strings: Mfr=1, Product=2, SerialNumber=0

Mar  4 17:59:00 ds-hadoop-wk14p kernel: usb 1-1.6.3: Product: iDRAC Virtual NIC USB Device

Mar  4 17:59:00 ds-hadoop-wk14p kernel: usb 1-1.6.3: Manufacturer: Dell(TM)

Mar  4 17:59:00 ds-hadoop-wk14p kernel: usb 1-1.6.3: configuration #1 chosen from 2 choices

Mar  4 17:59:00 ds-hadoop-wk14p kernel: cdc_ether 1-1.6.3:1.0: usb0: register 'cdc_ether' at usb-0000:00:1a.0-1.6.3, CDC Ethernet Device, ae:4a:6b:19:d7:39

Mar  4 17:59:01 ds-hadoop-wk14p kernel: udev: renamed network interface usb0 to idrac

Mar  4 17:59:02 ds-hadoop-wk14p snmpd[3379]: IfIndex of an interface changed. Such interfaces will appear multiple times in IF-MIB.

Mar  4 17:59:06 ds-hadoop-wk14p kernel: usb 1-1.6.3: USB disconnect, device number 9

Mar  4 17:59:06 ds-hadoop-wk14p kernel: cdc_ether 1-1.6.3:1.0: idrac: unregister 'cdc_ether' usb-0000:00:1a.0-1.6.3, CDC Ethernet Device

Mar  4 17:59:18 ds-hadoop-wk14p kernel: usb 1-1.6.3: new high speed USB device number 10 using ehci_hcd

Mar  4 17:59:18 ds-hadoop-wk14p kernel: usb 1-1.6.3: New USB device found, idVendor=413c, idProduct=a102

Mar  4 17:59:18 ds-hadoop-wk14p kernel: usb 1-1.6.3: New USB device strings: Mfr=1, Product=2, SerialNumber=0

Mar  4 17:59:18 ds-hadoop-wk14p kernel: usb 1-1.6.3: Product: iDRAC Virtual NIC USB Device

Mar  4 17:59:18 ds-hadoop-wk14p kernel: usb 1-1.6.3: Manufacturer: Dell(TM)

Mar  4 17:59:18 ds-hadoop-wk14p kernel: usb 1-1.6.3: configuration #1 chosen from 2 choices

Mar  4 17:59:18 ds-hadoop-wk14p kernel: cdc_ether 1-1.6.3:1.0: eth0: register 'cdc_ether' at usb-0000:00:1a.0-1.6.3, CDC Ethernet Device, 44:a8:42:35:f5:0f

Mar  4 17:59:18 ds-hadoop-wk14p kernel: udev: renamed network interface eth0 to idrac

Mar  4 17:59:21 ds-hadoop-wk14p kernel: usb 1-1.6.1: USB disconnect, device number 8

Mar  4 18:00:34 ds-hadoop-wk14p ntpd[3403]: Listen normally on 9 idrac 169.254.0.2 UDP 123

Mar  4 18:00:34 ds-hadoop-wk14p ntpd[3403]: Listen normally on 10 idrac fe80::46a8:42ff:fe35:f50f UDP 123

Mar  4 18:00:34 ds-hadoop-wk14p ntpd[3403]: peers refreshed

Mar  4 18:00:36 ds-hadoop-wk14p dsm_ism_srvmgrd: 4893 ISM0005 [iSM@674.10892.2 EventID="8196" EventCategory="Audit" EventSeverity="info" IsPastEvent="false" language="en-US"] The iDRAC Service Module has successfully restarted communication with iDRAC.

Mar  4 18:02:35 ds-hadoop-wk14p snmpd[3379]: [smux_parse_var] Panic: type 48

[root@ds-hadoop-wk14p ~]#

#	Appropriate dmesg log:

[root@ds-hadoop-wk14p ~]# dmesg

...

audit: audit_backlog=321 > audit_backlog_limit=320

audit: audit_lost=2836 audit_rate_limit=0 audit_backlog_limit=320

audit: backlog limit exceeded

NOHZ: local_softirq_pending 100

usb 1-1.6: USB disconnect, device number 3

usb 1-1.6.3: USB disconnect, device number 4

cdc_ether 1-1.6.3:1.0: idrac: unregister 'cdc_ether' usb-0000:00:1a.0-1.6.3, CDC Ethernet Device

usb 1-1.6: new high speed USB device number 7 using ehci_hcd

usb 1-1.6: New USB device found, idVendor=413c, idProduct=a001

usb 1-1.6: New USB device strings: Mfr=1, Product=2, SerialNumber=3

usb 1-1.6: Product: Gadget USB HUB

usb 1-1.6: Manufacturer: no manufacturer

usb 1-1.6: SerialNumber: 0123456789

usb 1-1.6: configuration #1 chosen from 1 choice

hub 1-1.6:1.0: USB hub found

hub 1-1.6:1.0: 6 ports detected

usb 1-1.6.1: new high speed USB device number 8 using ehci_hcd

usb 1-1.6.1: New USB device found, idVendor=0624, idProduct=0249

usb 1-1.6.1: New USB device strings: Mfr=4, Product=5, SerialNumber=6

usb 1-1.6.1: Product: Keyboard/Mouse Function

usb 1-1.6.1: Manufacturer: Avocent

usb 1-1.6.1: SerialNumber: 20121018

usb 1-1.6.1: configuration #1 chosen from 1 choice

input: Avocent Keyboard/Mouse Function as /devices/pci0000:00/0000:00:1a.0/usb1/1-1/1-1.6/1-1.6.1/1-1.6.1:1.0/input/input2

generic-usb 0003:0624:0249.0001: input,hidraw0: USB HID v1.00 Keyboard [Avocent Keyboard/Mouse Function] on usb-0000:00:1a.0-1.6.1/input0

input: Avocent Keyboard/Mouse Function as /devices/pci0000:00/0000:00:1a.0/usb1/1-1/1-1.6/1-1.6.1/1-1.6.1:1.1/input/input3

generic-usb 0003:0624:0249.0002: input,hidraw1: USB HID v1.00 Mouse [Avocent Keyboard/Mouse Function] on usb-0000:00:1a.0-1.6.1/input1

input: Avocent Keyboard/Mouse Function as /devices/pci0000:00/0000:00:1a.0/usb1/1-1/1-1.6/1-1.6.1/1-1.6.1:1.2/input/input4

generic-usb 0003:0624:0249.0003: input,hidraw2: USB HID v1.00 Mouse [Avocent Keyboard/Mouse Function] on usb-0000:00:1a.0-1.6.1/input2

usb 1-1.6.3: new high speed USB device number 9 using ehci_hcd

usb 1-1.6.3: New USB device found, idVendor=413c, idProduct=a102

usb 1-1.6.3: New USB device strings: Mfr=1, Product=2, SerialNumber=0

usb 1-1.6.3: Product: iDRAC Virtual NIC USB Device

usb 1-1.6.3: Manufacturer: Dell(TM)

usb 1-1.6.3: configuration #1 chosen from 2 choices

cdc_ether 1-1.6.3:1.0: usb0: register 'cdc_ether' at usb-0000:00:1a.0-1.6.3, CDC Ethernet Device, ae:4a:6b:19:d7:39

udev: renamed network interface usb0 to idrac

usb 1-1.6.3: USB disconnect, device number 9

cdc_ether 1-1.6.3:1.0: idrac: unregister 'cdc_ether' usb-0000:00:1a.0-1.6.3, CDC Ethernet Device

usb 1-1.6.3: new high speed USB device number 10 using ehci_hcd

usb 1-1.6.3: New USB device found, idVendor=413c, idProduct=a102

usb 1-1.6.3: New USB device strings: Mfr=1, Product=2, SerialNumber=0

usb 1-1.6.3: Product: iDRAC Virtual NIC USB Device

usb 1-1.6.3: Manufacturer: Dell(TM)

usb 1-1.6.3: configuration #1 chosen from 2 choices

cdc_ether 1-1.6.3:1.0: eth0: register 'cdc_ether' at usb-0000:00:1a.0-1.6.3, CDC Ethernet Device, 44:a8:42:35:f5:0f

udev: renamed network interface eth0 to idrac

usb 1-1.6.1: USB disconnect, device number 8

idrac: no IPv6 routers present

[root@ds-hadoop-wk14p ~]#

#	In hardware log was not found some interesting.

#

[support@centos00 ~]$ ssh tcsbank.ru\\s.belogrud@HS6V762.mgmt.tcsbank.ru racadm getsel

The authenticity of host 'hs6v762.mgmt.tcsbank.ru (10.219.11.66)' can't be established.

RSA key fingerprint is 69:65:33:50:73:6f:2f:da:30:c7:57:3c:04:b3:89:44.

Are you sure you want to continue connecting (yes/no)? yes

Warning: Permanently added 'hs6v762.mgmt.tcsbank.ru,10.219.11.66' (RSA) to the list of known hosts.

tcsbank.ru\s.belogrud@hs6v762.mgmt.tcsbank.ru's password:

Record:      1

Date/Time:   06/27/2015 03:47:49

Source:      system

Severity:    Ok

Description: Log cleared.

-------------------------------------------------------------------------------

Record:      2

Date/Time:   06/27/2015 04:02:57

Source:      system

Severity:    Ok

Description: C: boot completed.

-------------------------------------------------------------------------------

Record:      3

Date/Time:   06/27/2015 04:02:57

Source:      system

Severity:    Ok

Description: OEM software event.

-------------------------------------------------------------------------------

Record:      4

Date/Time:   08/06/2015 11:41:33

Source:      system

Severity:    Critical

Description: The chassis is open while the power is off.

-------------------------------------------------------------------------------

Record:      5

Date/Time:   08/06/2015 11:41:38

Source:      system

Severity:    Ok

Description: The chassis is closed while the power is off.

-------------------------------------------------------------------------------

Record:      6

Date/Time:   08/11/2015 12:31:45

Source:      system

Severity:    Critical

Description: Power supply redundancy is lost.

-------------------------------------------------------------------------------

Record:      7

Date/Time:   08/11/2015 12:31:45

Source:      system

Severity:    Critical

Description: The power input for power supply 2 is lost.

-------------------------------------------------------------------------------

Record:      8

Date/Time:   08/11/2015 12:34:34

Source:      system

Severity:    Ok

Description: The input power for power supply 2 has been restored.

-------------------------------------------------------------------------------

Record:      9

Date/Time:   08/11/2015 12:34:39

Source:      system

Severity:    Ok

Description: The power supplies are redundant.

-------------------------------------------------------------------------------

Record:      10

Date/Time:   08/11/2015 12:35:20

Source:      system

Severity:    Critical

Description: Power supply redundancy is lost.

-------------------------------------------------------------------------------

Record:      11

Date/Time:   08/11/2015 12:35:25

Source:      system

Severity:    Critical

Description: The power input for power supply 1 is lost.

-------------------------------------------------------------------------------

Record:      12

Date/Time:   08/11/2015 12:41:35

Source:      system

Severity:    Ok

Description: The input power for power supply 1 has been restored.

-------------------------------------------------------------------------------

Record:      13

Date/Time:   08/11/2015 12:41:41

Source:      system

Severity:    Ok

Description: The power supplies are redundant.

-------------------------------------------------------------------------------

[support@centos00 ~]$

[support@centos00 ~]$ ssh tcsbank.ru\\s.belogrud@HS6V762.mgmt.tcsbank.ru racadm getraclog | tee -a /tmp/20160324-1233.getraclog.ds-hadoop-wk14p.log

SeqNumber       = 1481826

Message ID      = USR0030

Category        = Audit

AgentID         = RACLOG

Severity        = Information

Timestamp       = 2016-03-24 12:34:21

Message         = Successfully logged in using s.belogrud@tcsbank.ru, from 172.16.28.74 and SSH.

Message Arg   1 = s.belogrud@tcsbank.ru

Message Arg   2 = 172.16.28.74

Message Arg   3 = SSH

FQDD            = iDRAC.Embedded.1

--------------------------------------------------------------------------------

SeqNumber       = 1481825

Message ID      = USR0176

Category        = Audit

AgentID         = iDRAC

Severity        = Information

Timestamp       = 2016-03-24 12:33:27

Message         = The Front Panel USB Port Over Current condition is cleared for the attached device Disk.USBFront.1.

Message Arg   1 = 1

...

--------------------------------------------------------------------------------

SeqNumber       = 1481246

Message ID      = USR0175

Category        = Audit

AgentID         = iDRAC

Severity        = Warning

Timestamp       = 2016-03-24 10:56:38

Message         = The Front Panel USB Port Over Current is detected for the attached device on Disk.USBFront.1.

Message Arg   1 = 1

--------------------------------------------------------------------------------

SeqNumber       = 1481245

Message ID      = LOG007

Category        = Audit

AgentID         = DE

Severity        = Information

Timestamp       = 2016-03-24 10:56:38

Message         = The previous log entry was repeated 4 times.

Message Arg   1 = 4

--------------------------------------------------------------------------------

SeqNumber       = 1481244

Message ID      = USR0030

Category        = Audit

AgentID         = RACLOG

Severity        = Information

Timestamp       = 2016-03-24 10:56:33

Message         = Successfully logged in using root, from 10.216.43.159 and WS-MAN.

Message Arg   1 = root

Message Arg   2 = 10.216.43.159

Message Arg   3 = WS-MAN

FQDD            = iDRAC.Embedded.1

--------------------------------------------------------------------------------

SeqNumber       = 1481243

Message ID      = USR0176

Category        = Audit

AgentID         = iDRAC

Severity        = Information

Timestamp       = 2016-03-24 10:56:28

Message         = The Front Panel USB Port Over Current condition is cleared for the attached device Disk.USBFront.1.

Message Arg   1 = 1

--------------------------------------------------------------------------------

SeqNumber       = 1481242

Message ID      = USR0175

Category        = Audit

AgentID         = iDRAC

Severity        = Warning

Timestamp       = 2016-03-24 10:56:24

Message         = The Front Panel USB Port Over Current is detected for the attached device on Disk.USBFront.1.

Message Arg   1 = 1

--------------------------------------------------------------------------------

SeqNumber       = 1481241

Message ID      = USR0030

Category        = Audit

AgentID         = RACLOG

Severity        = Information

Timestamp       = 2016-03-24 10:56:24

Message         = Successfully logged in using root, from 10.216.43.159 and WS-MAN.

Message Arg   1 = root

Message Arg   2 = 10.216.43.159

Message Arg   3 = WS-MAN

FQDD            = iDRAC.Embedded.1

--------------------------------------------------------------------------------

SeqNumber       = 1481240

Message ID      = USR0175

Category        = Audit

AgentID         = iDRAC

Severity        = Warning

Timestamp       = 2016-03-24 10:56:24

Message         = The Front Panel USB Port Over Current is detected for the attached device on Disk.USBFront.1.

Message Arg   1 = 1

--------------------------------------------------------------------------------

SeqNumber       = 1481239

Message ID      = LOG007

Category        = Audit

AgentID         = DE

Severity        = Information

Timestamp       = 2016-03-24 10:56:24

Message         = The previous log entry was repeated 9 times.

Message Arg   1 = 9

--------------------------------------------------------------------------------

SeqNumber       = 1481238

Message ID      = USR0030

Category        = Audit

AgentID         = RACLOG

Severity        = Information

Timestamp       = 2016-03-24 10:56:16

Message         = Successfully logged in using root, from 10.216.43.159 and WS-MAN.

Message Arg   1 = root

Message Arg   2 = 10.216.43.159

Message Arg   3 = WS-MAN

FQDD            = iDRAC.Embedded.1

--------------------------------------------------------------------------------

SeqNumber       = 1481237

Message ID      = USR0176

Category        = Audit

AgentID         = iDRAC

Severity        = Information

Timestamp       = 2016-03-24 10:56:15

Message         = The Front Panel USB Port Over Current condition is cleared for the attached device Disk.USBFront.1.

Message Arg   1 = 1

--------------------------------------------------------------------------------

Timeout, server not responding.

[support@centos00 ~]$