20150921

#	Here is an example of test for PCIe SSD drive.

[root@ds-db-tst 20150921-1635]# time timeout 1m fio --name=fio-job --filename=/dev/fioa --blocksize=4k --rw=randwrite --direct=1 --buffered=0 --ioengine=libaio --iodepth=16

fio-job: (g=0): rw=randwrite, bs=4K-4K/4K-4K/4K-4K, ioengine=libaio, iodepth=16

fio-2.0.13

Starting 1 process

Jobs: 1 (f=1): [w] [0.2% done] [0K/254.7M/0K /s] [0 /65.2K/0  iops] [eta 06h:48m:23s]

fio: terminating on signal 15

fio-job: (groupid=0, jobs=1): err= 0: pid=61144: Mon Sep 21 17:55:09 2015

write: io=14871MB, bw=256026KB/s, iops=64006 , runt= 59477msec

slat (usec): min=3 , max=474 , avg=13.82, stdev= 4.78

clat (usec): min=34 , max=1059 , avg=234.81, stdev=22.25

lat (usec): min=46 , max=1103 , avg=248.76, stdev=23.05

clat percentiles (usec):

|  1.00th=[  199],  5.00th=[  213], 10.00th=[  217], 20.00th=[  221],

| 30.00th=[  225], 40.00th=[  229], 50.00th=[  233], 60.00th=[  237],

| 70.00th=[  241], 80.00th=[  247], 90.00th=[  255], 95.00th=[  266],

| 99.00th=[  298], 99.50th=[  322], 99.90th=[  524], 99.95th=[  548],

| 99.99th=[  604]

bw (KB/s)  : min=226568, max=295248, per=99.98%, avg=255985.56, stdev=10109.17

lat (usec) : 50=0.01%, 100=0.01%, 250=84.89%, 500=14.95%, 750=0.16%

lat (usec) : 1000=0.01%

lat (msec) : 2=0.01%

cpu          : usr=17.25%, sys=81.95%, ctx=54113, majf=0, minf=23

IO depths    : 1=0.1%, 2=0.1%, 4=0.1%, 8=0.1%, 16=100.0%, 32=0.0%, >=64=0.0%

submit    : 0=0.0%, 4=100.0%, 8=0.0%, 16=0.0%, 32=0.0%, 64=0.0%, >=64=0.0%

complete  : 0=0.0%, 4=100.0%, 8=0.0%, 16=0.1%, 32=0.0%, 64=0.0%, >=64=0.0%

issued    : total=r=0/w=3806911/d=0, short=r=0/w=0/d=0

Run status group 0 (all jobs):

WRITE: io=14871MB, aggrb=256025KB/s, minb=256025KB/s, maxb=256025KB/s, mint=59477msec, maxt=59477msec

Disk stats (read/write):

fioa: ios=0/8587318, merge=0/0, ticks=0/262137, in_queue=257947, util=100.00%

real    1m0.208s

user    0m10.533s

sys     0m49.050s

[root@ds-db-tst 20150921-1635]#

20150921

#	Here is an example of test for PCIe SSD drive.

[root@ds-db-tst 20150921-1635]# echo "/dev/fioa1" > ./$(date +%Y%m%d-%H%M)_fioa1.lun ; ./orion_linux_x86-64 -run simple -testname ./$(date +%Y%m%d-%H%M)_fioa1 -num_disks 1

ORION: ORacle IO Numbers -- Version 11.1.0.7.0

./20150921-1644_fioa1_20150921_1644

Test will take approximately 9 minutes

Larger caches may take longer

[root@ds-db-tst 20150921-1635]#

[root@ds-db-tst 20150921-1635]# ll

total 45288

-rw-r--r-- 1 root root      128 Sep 21 16:51 20150921-1644_fioa1_20150921_1644_iops.csv

-rw-r--r-- 1 root root      128 Sep 21 16:51 20150921-1644_fioa1_20150921_1644_lat.csv

-rw-r--r-- 1 root root      102 Sep 21 16:51 20150921-1644_fioa1_20150921_1644_mbps.csv

-rw-r--r-- 1 root root      624 Sep 21 16:51 20150921-1644_fioa1_20150921_1644_summary.txt

-rw-r--r-- 1 root root     3026 Sep 21 16:51 20150921-1644_fioa1_20150921_1644_trace.txt

-rw-r--r-- 1 root root       11 Sep 21 16:44 20150921-1644_fioa1.lun

-rwxr-x--- 1 root root 46350238 Sep 21 16:35 orion_linux_x86-64

[root@ds-db-tst 20150921-1635]#

[root@ds-db-tst 20150921-1635]# cat 20150921-1644_fioa1_20150921_1644_summary.txt

ORION VERSION 11.1.0.7.0

Commandline:

-run simple -testname ./20150921-1644_fioa1 -num_disks 1

This maps to this test:

Test: ./20150921-1644_fioa1

Small IO size: 8 KB

Large IO size: 1024 KB

IO Types: Small Random IOs, Large Random IOs

Simulated Array Type: CONCAT

Write: 0%

Cache Size: Not Entered

Duration for each Data Point: 60 seconds

Small Columns:,      0

Large Columns:,      0,      1,      2

Total Data Points: 8

Name: /dev/fioa1        Size: 6399998296064

1 FILEs found.

Maximum Large MBPS=2850.77 @ Small=0 and Large=2

Maximum Small IOPS=83042 @ Small=5 and Large=0

Minimum Small Latency=0.03 @ Small=1 and Large=0

[root@ds-db-tst 20150921-1635]#

From: Chernyh Aleksandr Vladimirovich

Sent: Monday, September 21, 2015 11:23 AM

To: Belogrud Sergey Vladimirovich

Subject: тесты

Orion можешь взять с [root@m1-gfs2-tst01 ~]#       /root/

2 теста на блочное устройство

1)

./orion_linux_x86-64 -run simple -testname test_raid10 -num_disks 8

[root@m1-gfs2-tst01 ~]# cat test_raid10.lun

/dev/mapper/stripe_group_raid10-data_array_raid10

2)  	Запустить на 20 мин например – убрать прерыванием

fio ./fio-raid10.ini

[root@m1-gfs2-tst01 ~]# cat fio-raid10.ini

[writetest]

blocksize=4k

filename=/dev/mapper/stripe_group_raid10-data_array_raid10

rw=randwrite

direct=1

buffered=0

ioengine=libaio

iodepth=16

Тест файликов

bonnie++ -d /mnt/raid10 -r 10G -s 20G -n 5:5242880:2:100 -u root

пример вывода

| Sequential Output |  |  |  |  |  | Sequential Input |  |  |  | Random Seeks |  |  |  | Sequential Create |  |  |  |  |  |  |  | Random Create |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Block |  | Rewrite |  | Per Char |  |  | Block |  | Create |  |  | Read |  | Delete |  | Create |  | Read |  |  | Delete |  |  |  |  |  |
| K/sec | % CPU | K/sec | % CPU | K/sec | % CPU |  | K/sec | % CPU | /sec |  | % CPU | /sec | % CPU |  | /sec | % CPU | /sec | % CPU | /sec | % CPU | /sec |  | % CPU | /sec | % CPU |  |
| 270633 | 24 | 291902 | 20 | 3000 | 105 |  | 3219654 | 99 | 2508 |  | 111 | 99 | 22 |  | 661 | 99 | 1999 | 98 | 94 | 27 | 1071 |  | 99 | 1938 | 97 |  |
| 318us |  | 275us |  | 3292us |  |  | 274us |  | 2456us |  |  | 14070us |  |  | 5409us |  | 2059us |  | 19031us |  | 3126us |  |  | 2897us |  |  |

С уважением,

Александр Черных

Главный Системный Администратор