20160729

http://prefetch.net/blog/index.php/2011/07/22/measuring-hard-drive-seek-times/

## Measuring hard drive seek times

I’ve been fascinated with I/O and file system performance for years, and chose prefetch.net as my domain name after reading about pre-fetching algorithms in the book UNIX file systems (a great book that I need to read again). Since most applications access data that is not laid out sequentially on a hard drive platter, seek times come into play when you start to look at getting the most bang for your buck on random I/O workloads.

This past week I saw this first hand. I was playing around with some disk drives and started to wonder how many seeks I could get per second. I was also curious what the access time of each seek was, so I started poking around for a tool that displays this information. After a quick search, I came across seeker.

When seeker is run with the path to a block device, it will initiate reads to a device and measure the number of seeks per second, the average time of each seek, and the starting and ending offsets that were used to randomly access blocks on the disk. Here is a sample run:

$ seeker /dev/sda

Seeker v3.0, 2009-06-17, http://www.linuxinsight.com/how_fast_is_your_disk.html
Benchmarking /dev/sda [156250000 blocks, 80000000000 bytes, 74 GB, 76293 MB, 80 GiB, 80000 MiB]
[512 logical sector size, 512 physical sector size]
[1 threads]
Wait 30 seconds..............................
Results: 50 seeks/second, 19.881 ms random access time (128677785 < offsets < 79965549334)

If you want to see the effects of multiple threads accessing a disk, you can pass a thread count to seeker:

$ seeker /dev/sda 32

Seeker v3.0, 2009-06-17, http://www.linuxinsight.com/how_fast_is_your_disk.html
Benchmarking /dev/sda [156250000 blocks, 80000000000 bytes, 74 GB, 76293 MB, 80 GiB, 80000 MiB]
[512 logical sector size, 512 physical sector size]
[32 threads]
Wait 30 seconds..............................
Results: 53 seeks/second, 18.587 ms random access time (36588683 < offsets < 79946851693)

In the example above 32 threads would try to access data at random locations and the results from these disk accesses would be measured and printed. Seeker is a super handy utility, and I would put it right up there with blktrace.

20160729

http://prefetch.net/blog/index.php/2009/02/16/tracing-block-io-operations-on-linux-hosts-with-blktrace/

## Tracing block I/O operations on Linux hosts with blktrace

I’ve been spending a bunch of time with Linux lately, and have found some really nifty tools to help me better manage the systems I support. One of these tools is blktrace, which allows you to view block I/O operations to the devices connected to your system. To see just how useful this utility is, I present to you with the following example:

disarm:~# blktrace -d /dev/sda -o – | blkparse -i –

8,0    0        1     0.000000000   783  A   W 36744519 + 8 <- (8,1) 36744456
  8,0    0        2     0.000000472   783  Q   W 36744519 + 8 [kjournald]
  8,0    0        3     0.000012413   783  G   W 36744519 + 8 [kjournald]
  8,0    0        4     0.000016614   783  P   N [kjournald]
  8,0    0        5     0.000018272   783  I   W 36744519 + 8 [kjournald]
  8,0    0        6     0.000026492   783  A   W 36744527 + 8 <- (8,1) 36744464
  8,0    0        7     0.000026784   783  Q   W 36744527 + 8 [kjournald]
  8,0    0        8     0.000028877   783  M   W 36744527 + 8 [kjournald]
  8,0    0        9     0.000031539   783  A   W 36744535 + 8 <- (8,1) 36744472
  8,0    0       10     0.000031752   783  Q   W 36744535 + 8 [kjournald]
  8,0    0       11     0.000032234   783  M   W 36744535 + 8 [kjournald]
  8,0    0       12     0.000033151   783  A   W 36744543 + 8 <- (8,1) 36744480
  8,0    0       13     0.000033343   783  Q   W 36744543 + 8 [kjournald]
  8,0    0       14     0.000033758   783  M   W 36744543 + 8 [kjournald]
  8,0    0       15     0.000034477   783  A   W 36744551 + 8 <- (8,1) 36744488
  8,0    0       16     0.000034670   783  Q   W 36744551 + 8 [kjournald]
  8,0    0       17     0.000035082   783  M   W 36744551 + 8 [kjournald]
  8,0    0       18     0.000035786   783  A   W 36744559 + 8 <- (8,1) 36744496
  8,0    0       19     0.000035972   783  Q   W 36744559 + 8 [kjournald]
  8,0    0       20     0.000036381   783  M   W 36744559 + 8 [kjournald]
  8,0    0       21     0.000037124   783  A   W 36744567 + 8 <- (8,1) 36744504

  < ..... >

^CCPU0 (8,0):
 Reads Queued:           0,        0KiB  Writes Queued:          13,       52KiB
 Read Dispatches:        0,        0KiB  Write Dispatches:        2,       52KiB
 Reads Requeued:         0               Writes Requeued:         0
 Reads Completed:        0,        0KiB  Writes Completed:        2,       52KiB
 Read Merges:            0,        0KiB  Write Merges:           11,       44KiB
 Read depth:             0               Write depth:             1
 IO unplugs:             2               Timer unplugs:           0

Throughput (R/W): 0KiB/s / 52000KiB/s
Events (8,0): 49 entries
Skips: 0 forward (0 -   0.0%)

As you can see from the above output, each I/O is printed along with a summary of the operations and how they were processed by the I/O scheduler. This is super information, which can be used to figure out I/O patterns (random reads, random writes, etc.), the size of the I/O operations hitting physical devices in a system and the type of workload on a system. I used to think that Solaris (w/ DTrace) had a serious leg up on Linux, but I’m starting to realize that you can understand Linux performance just as well if you install one or more third party tools. I’ve come across a slew of nifty utilities over the past month, and will start posting regularly with example output and links to each utility.

## 7 Comments

#### Shane  on February 16th, 2009

Great find!! I’ll be using blktrace this week. I’m looking forward to what other tools you have come across.

#### jeff wasilko  on February 16th, 2009

Thanks for the pointer! I’ve got a mix of solaris and linux, and i’m working on trying to understand the workload on one of my big linux farms at this point….hoping some flash disks might help with a bottleneck.

Did you get my mail about the nfstrace ‘bug’?

#### Bill Hathaway  on February 17th, 2009

Have you seen iotop? http://guichaz.free.fr/iotop/

#### Jeff Schroeder  on February 17th, 2009

Thanks matty, you might also look at the block_dump feature of newer kernels.

I wrote up a hacky top-disk-users script to give you an idea:

http://www.digitalprognosis.com/opensource/scripts/top-disk-users

#### Kostas Georgiou  on March 8th, 2009

You probably want to look at seekwatcher that creates some nice plots/movies from blktrace.

#### luqmanux  on March 20th, 2009

I have installed the package, whenever I execute the command you posted abovw, I have the following output:

/sys/kernel/debug does not appear to be a debug filesystem

Any clue?

#### Greg Varga  on June 30th, 2009

luqmanux, you need to mount the debug filesystem:

mount -t debugfs debugfs /sys/kernel/debug