[root@sdw7 ~]# dmidecode -t system

# dmidecode 2.11

SMBIOS 2.7 present.

Handle 0x0100, DMI type 1, 27 bytes

System Information

Manufacturer: HP

Product Name: ProLiant DL380p Gen8

Version: Not Specified

Serial Number: CZ23290BNN

UUID: 32333536-3030-5A43-3233-323930424E4E

Wake-up Type: Power Switch

SKU Number: 653200-B21

Family: ProLiant

Handle 0x2000, DMI type 32, 11 bytes

System Boot Information

Status: No errors detected

[root@sdw7 ~]#

[root@mdw_prod ~]# ssh sdw7

Last login: Thu Feb  6 16:53:10 2014 from mdw

[root@sdw7 ~]# hpacucli ctrl all show status

Smart Array P420 in Slot 2

Controller Status: OK

Cache Status: OK

Battery/Capacitor Status: OK

Smart Array P420i in Slot 0 (Embedded)

Controller Status: OK

Cache Status: OK

Battery/Capacitor Status: OK

[root@sdw7 ~]# hpacucli ctrl slot=2 ld all show status

logicaldrive 1 (3.8 TB, RAID 5): Interim Recovery Mode

[root@sdw7 ~]#

[root@sdw7 ~]# hpacucli ctrl all show config

Smart Array P420 in Slot 2                (sn: PDSXK0ARH4W2KV)

array A (SAS, Unused Space: 0  MB)

logicaldrive 1 (3.8 TB, RAID 5, Interim Recovery Mode)

physicaldrive 1I:1:1 (port 1I:box 1:bay 1, SAS, 600 GB, OK)

physicaldrive 1I:1:2 (port 1I:box 1:bay 2, SAS, 600 GB, Failed)

physicaldrive 1I:1:3 (port 1I:box 1:bay 3, SAS, 600 GB, OK)

physicaldrive 1I:1:4 (port 1I:box 1:bay 4, SAS, 600 GB, OK)

physicaldrive 2I:1:5 (port 2I:box 1:bay 5, SAS, 600 GB, OK)

physicaldrive 2I:1:6 (port 2I:box 1:bay 6, SAS, 600 GB, OK)

physicaldrive 2I:1:7 (port 2I:box 1:bay 7, SAS, 600 GB, OK)

physicaldrive 2I:1:8 (port 2I:box 1:bay 8, SAS, 600 GB, OK)

SEP (Vendor ID PMCSIERA, Model SRCv8x6G) 380 (WWID: 50014380276306AF)

Smart Array P420i in Slot 0 (Embedded)    (sn: 00143802641BB90)

array A (SAS, Unused Space: 0  MB)

logicaldrive 1 (3.8 TB, RAID 5, OK)

physicaldrive 1I:2:1 (port 1I:box 2:bay 1, SAS, 600 GB, OK)

physicaldrive 1I:2:2 (port 1I:box 2:bay 2, SAS, 600 GB, OK)

physicaldrive 1I:2:3 (port 1I:box 2:bay 3, SAS, 600 GB, OK)

physicaldrive 1I:2:4 (port 1I:box 2:bay 4, SAS, 600 GB, OK)

physicaldrive 2I:2:5 (port 2I:box 2:bay 5, SAS, 600 GB, OK)

physicaldrive 2I:2:6 (port 2I:box 2:bay 6, SAS, 600 GB, OK)

physicaldrive 2I:2:7 (port 2I:box 2:bay 7, SAS, 600 GB, OK)

physicaldrive 2I:2:8 (port 2I:box 2:bay 8, SAS, 600 GB, OK)

SEP (Vendor ID PMCSIERA, Model SRCv8x6G) 380 (WWID: 500143802641BB9F)

[root@sdw7 ~]#

[root@sdw7 ~]# hpacucli controller slot=2 physicaldrive 1I:1:2 show status

physicaldrive 1I:1:2 (port 1I:box 1:bay 2, 600 GB): Failed

[root@sdw7 ~]# hpacucli controller slot=2 physicaldrive 1I:1:2 show detail

Smart Array P420 in Slot 2

array A

physicaldrive 1I:1:2

Port: 1I

Box: 1

Bay: 2

Status: Failed

Drive Type: Data Drive

Interface Type: SAS

Size: 600 GB

Rotational Speed: 10000

Firmware Revision: HPD9

Serial Number:         KNH229HF

Model: HP      EG0600FBVFP

Maximum Temperature (C): 40

PHY Count: 2

PHY Transfer Rate: 6.0Gbps, Unknown

Drive Authentication Status: Not Applicable

[root@sdw7 ~]#

[root@sdw7 ~]# hpacucli ctrl all show detail

Smart Array P420 in Slot 2

Bus Interface: PCI

Slot: 2

Serial Number: PDSXK0ARH4W2KV

Cache Serial Number: PBKUC0BRH4V0WM

RAID 6 (ADG) Status: Enabled

Controller Status: OK

Hardware Revision: B

Firmware Version: 4.68

Rebuild Priority: Medium

Expand Priority: Medium

Surface Scan Delay: 3 secs

Surface Scan Mode: Idle

Queue Depth: Automatic

Monitor and Performance Delay: 60  min

Elevator Sort: Enabled

Degraded Performance Optimization: Disabled

Inconsistency Repair Policy: Disabled

Wait for Cache Room: Disabled

Surface Analysis Inconsistency Notification: Disabled

Post Prompt Timeout: 15 secs

Cache Board Present: True

Cache Status: OK

Cache Ratio: 0% Read / 100% Write

Drive Write Cache: Enabled

Total Cache Size: 1024 MB

Total Cache Memory Available: 816 MB

No-Battery Write Cache: Disabled

Cache Backup Power Source: Capacitors

Battery/Capacitor Count: 1

Battery/Capacitor Status: OK

SATA NCQ Supported: True

Spare Activation Mode: Activate on drive failure

Controller Temperature (C): 88

Cache Module Temperature (C): 44

Capacitor Temperature  (C): 21

Smart Array P420i in Slot 0 (Embedded)

Bus Interface: PCI

Slot: 0

Serial Number: 00143802641BB90

Cache Serial Number: PBKUC0BRH4V1LL

RAID 6 (ADG) Status: Enabled

Controller Status: OK

Hardware Revision: B

Firmware Version: 4.68

Rebuild Priority: Medium

Expand Priority: Medium

Surface Scan Delay: 3 secs

Surface Scan Mode: Idle

Queue Depth: Automatic

Monitor and Performance Delay: 60  min

Elevator Sort: Enabled

Degraded Performance Optimization: Disabled

Inconsistency Repair Policy: Disabled

Wait for Cache Room: Disabled

Surface Analysis Inconsistency Notification: Disabled

Post Prompt Timeout: 15 secs

Cache Board Present: True

Cache Status: OK

Cache Ratio: 0% Read / 100% Write

Drive Write Cache: Enabled

Total Cache Size: 1024 MB

Total Cache Memory Available: 816 MB

No-Battery Write Cache: Disabled

Cache Backup Power Source: Capacitors

Battery/Capacitor Count: 1

Battery/Capacitor Status: OK

SATA NCQ Supported: True

Spare Activation Mode: Activate on drive failure

Controller Temperature (C): 78

Cache Module Temperature (C): 37

Capacitor Temperature  (C): 25

[root@sdw7 ~]#

[root@sdw7 tmp]# tar czvf /tmp/20140206.ds-gp-prod-sdw7.hpadureport.tgz /var/log/hpadureport*

tar: Removing leading `/' from member names

/var/log/hpadureport.zip-20140128

/var/log/hpadureport.zip-20140129

/var/log/hpadureport.zip-20140130

/var/log/hpadureport.zip-20140131

/var/log/hpadureport.zip-20140201

/var/log/hpadureport.zip-20140202

/var/log/hpadureport.zip-20140203

/var/log/hpadureport.zip-20140204

/var/log/hpadureport.zip-20140205

/var/log/hpadureport.zip-20140206

[root@sdw7 tmp]#

[root@mdw_prod ~]# cd /tmp/ ; scp sdw7:/tmp/20140206.ds-gp-prod-sdw7.hpadureport.tgz .

20140206.ds-gp-prod-sdw7.hpadureport.tgz                                                                                               100% 2040KB   2.0MB/s   00:00

[root@mdw_prod tmp]#

[support@centos00 20140206]$ scp bsv@ds-gp-prod-m:/tmp/20140206.ds-gp-prod-sdw7.hpadureport.tgz .

bsv@ds-gp-prod-m's password:

20140206.ds-gp-prod-sdw7.hpadureport.tgz                                                                                               100% 2040KB   2.0MB/s   00:00

[support@centos00 20140206]$

Your case was successfully submitted. Please note your Case ID: 4647424439 for future reference. An e-mail confirmation will be sent to the case contact. HP will contact you to begin work on your problem based on your contract or warranty coverage.

* Note: Your case was submitted using an unlinked warranty. This case will not be available for case reporting unless you link it to your account.

Link support agreements Link HP Care Packs Link warranties

Add attachments to case:

Click this link to send attachment to hpsupport_global@hp.com (Maximum size is 5 Mb)

Case information

| Case ID: |  | 4647424439 |
| --- | --- | --- |
| Case title: |  | Hard drive failure |
| Severity |  | 1-Critical Down |
| Product serial number: |  | CZ23290BNN |
| Product number: |  | 653200-B21 |
| Submitted: |  | 2/6/2014 2:20:57 PM |
| Last updated: |  | 2/6/2014 2:20:57 PM |
| Source: |  | Web |
| Case status: |  | Received by HP |

Case details

| Operating system/version: | Red Hat Enterprise Linux Server release 6.3 (Santiago) |
| --- | --- |
| Product: | HP ProLiant DL Servers |
| Product vendor: |  |

Problem description:

Добрый день. Вышел из строя жесткий диск. Просим заменить его на исправный. [root@sdw7 ~]# hpacucli ctrl all show config Smart Array P420 in Slot 2 (sn: PDSXK0ARH4W2KV) array A (SAS, Unused Space: 0 MB) logicaldrive 1 (3.8 TB, RAID 5, Interim Recovery Mode) physicaldrive 1I:1:1 (port 1I:box 1:bay 1, SAS, 600 GB, OK) physicaldrive 1I:1:2 (port 1I:box 1:bay 2, SAS, 600 GB, Failed) physicaldrive 1I:1:3 (port 1I:box 1:bay 3, SAS, 600 GB, OK) physicaldrive 1I:1:4 (port 1I:box 1:bay 4, SAS, 600 GB, OK) physicaldrive 2I:1:5 (port 2I:box 1:bay 5, SAS, 600 GB, OK) physicaldrive 2I:1:6 (port 2I:box 1:bay 6, SAS, 600 GB, OK) physicaldrive 2I:1:7 (port 2I:box 1:bay 7, SAS, 600 GB, OK) physicaldrive 2I:1:8 (port 2I:box 1:bay 8, SAS, 600 GB, OK) SEP (Vendor ID PMCSIERA, Model SRCv8x6G) 380 (WWID: 50014380276306AF) Smart Array P420i in Slot 0 (Embedded) (sn: 00143802641BB90) array A (SAS, Unused Space: 0 MB) logicaldrive 1 (3.8 TB, RAID 5, OK) physicaldrive 1I:2:1 (port 1I:box 2:bay 1, SAS, 600 GB, OK) physicaldrive 1I:2:2 (port 1I:box 2:bay 2, SAS, 600 GB, OK) physicaldrive 1I:2:3 (port 1I:box 2:bay 3, SAS, 600 GB, OK) physicaldrive 1I:2:4 (port 1I:box 2:bay 4, SAS, 600 GB, OK) physicaldrive 2I:2:5 (port 2I:box 2:bay 5, SAS, 600 GB, OK) physicaldrive 2I:2:6 (port 2I:box 2:bay 6, SAS, 600 GB, OK) physicaldrive 2I:2:7 (port 2I:box 2:bay 7, SAS, 600 GB, OK) physicaldrive 2I:2:8 (port 2I:box 2:bay 8, SAS, 600 GB, OK) SEP (Vendor ID PMCSIERA, Model SRCv8x6G) 380 (WWID: 500143802641BB9F) [root@sdw7 ~]# [root@sdw7 ~]# hpacucli controller slot=2 physicaldrive 1I:1:2 show status physicaldrive 1I:1:2 (port 1I:box 1:bay 2, 600 GB): Failed [root@sdw7 ~]# hpacucli controller slot=2 physicaldrive 1I:1:2 show detail Smart Array P420 in Slot 2 array A physicaldrive 1I:1:2 Port: 1I Box: 1 Bay: 2 Status: Failed Drive Type: Data Drive Interface Type: SAS Size: 600 GB Rotational Speed: 10000 Firmware Revision: HPD9 Serial Number: KNH229HF Model: HP EG0600FBVFP Maximum Temperature (C): 40 PHY Count: 2 PHY Transfer Rate: 6.0Gbps, Unknown Drive Authentication Status: Not Applicable [root@sdw7 ~]#

Troubleshooting steps taken:

Contact & equipment location information

| Contact |  | Equipment location |  |
| --- | --- | --- | --- |
| First name: | Sergey | Equipment contact name: | Sergey Belogrud |
| Last name: | Belogrud | Equipment contact phone: | +79150464278 |
| Phone number: | +74956481000 2173 | Company name: | TCS Bank |
| Alternate Phone: | +79150464278 | Address line 1: | 1-st Volokolamskiy proezd, building 10, structure 1 |
| Email address: | s.belogrud@tcsbank.ru |  |  |
|  |  | City/Town: | Moscow |
|  |  | State/Province: | Moscow region |
|  |  | Zip/Postal code: | 123040 |
|  |  | Country/Region: | RUSSIA |

iLO

ds-gp-prod-sdw7 https://10.219.10.142/

https://cmdb.tcsbank.ru/new/index.php?part=devices&action=edit&id=214

Device Rack: DS-R/F-8

Device Rack First Unit: 24

Device Rack Last Unit: 25

Device model: HP DL 380p Gen8

Device height: 2

Device length: full length

Serial Number: CZ23290BNN

From: Belogrud Sergey Vladimirovich

Sent: Friday, February 07, 2014 1:32 PM

To: Myalin Dmitriy

Cc: unix-admins; greenplum_admins

Subject: RE: Согласование проведения работ по замене неисправного HDD на сервере ds-gp-prod-sdw7

Хорошо.

Тогда более подробный план работ следующий.

1. В субботу вечером. Ответственный: Гусь Константин.

1.1. Завершается работа ПО GreenPlum.

2. В воскресенье утром, ориентировочно в 6.00. Ответственный: Мялин Дмитрий.

2.1.  Заменяется неисправный диск на сервере ds-gp-prod-sdw7.

3. В воскресенье, позже по времени. Ответственный: Гусь Константин.

3.1. Перезапускаются серверы-сегменты контура ds-gp-pord.

3.2. Проверяется исправность работы всех дисков на сервере ds-gp-prod-sdw7.

3.3. Запускается ПО GreenPlum.

3.4. Проверяется исправность работы ПО GreenPlum и контура в целом.

From: Belogrud Sergey Vladimirovich

Sent: Friday, February 07, 2014 1:11 PM

To: Gus Konstantin Mikhaylovich

Cc: unix-admins; greenplum_admins

Subject: Согласование проведения работ по замене неисправного HDD на сервере ds-gp-prod-sdw7

Добрый день.

В воскресенье 9.02.2014 в 6.00 утра предлагается  запланировать проведение работ по замене неисправного HDD-диска в контуре GreenPlum ds-gp-prod на сервере-сегменте sdw7.

Прошу согласовать разрешение на проведение работ у всех заинтересованных лиц.

Информация по месту расположения сервера, в котором будет производиться замена диска, следующая.

iLO

ds-gp-prod-sdw7 https://10.219.10.142/

https://cmdb.tcsbank.ru/new/index.php?part=devices&action=edit&id=214

Device Rack: DS-R/F-8

Device Rack First Unit: 24

Device Rack Last Unit: 25

Device model: HP DL 380p Gen8

Device height: 2

Device length: full length

Serial Number: CZ23290BNN

20140210

[root@sdw7 ~]# hpacucli ctrl slot=2 ld all show status

logicaldrive 1 (3.8 TB, RAID 5): OK

[root@sdw7 ~]# hpacucli ctrl all show config

Smart Array P420 in Slot 2                (sn: PDSXK0ARH4W2KV)

array A (SAS, Unused Space: 0  MB)

logicaldrive 1 (3.8 TB, RAID 5, OK)

physicaldrive 1I:1:1 (port 1I:box 1:bay 1, SAS, 600 GB, OK)

physicaldrive 1I:1:2 (port 1I:box 1:bay 2, SAS, 600 GB, OK)

physicaldrive 1I:1:3 (port 1I:box 1:bay 3, SAS, 600 GB, OK)

physicaldrive 1I:1:4 (port 1I:box 1:bay 4, SAS, 600 GB, OK)

physicaldrive 2I:1:5 (port 2I:box 1:bay 5, SAS, 600 GB, OK)

physicaldrive 2I:1:6 (port 2I:box 1:bay 6, SAS, 600 GB, OK)

physicaldrive 2I:1:7 (port 2I:box 1:bay 7, SAS, 600 GB, OK)

physicaldrive 2I:1:8 (port 2I:box 1:bay 8, SAS, 600 GB, OK)

SEP (Vendor ID PMCSIERA, Model SRCv8x6G) 380 (WWID: 50014380276306AF)

Smart Array P420i in Slot 0 (Embedded)    (sn: 00143802641BB90)

array A (SAS, Unused Space: 0  MB)

logicaldrive 1 (3.8 TB, RAID 5, OK)

physicaldrive 1I:2:1 (port 1I:box 2:bay 1, SAS, 600 GB, OK)

physicaldrive 1I:2:2 (port 1I:box 2:bay 2, SAS, 600 GB, OK)

physicaldrive 1I:2:3 (port 1I:box 2:bay 3, SAS, 600 GB, OK)

physicaldrive 1I:2:4 (port 1I:box 2:bay 4, SAS, 600 GB, OK)

physicaldrive 2I:2:5 (port 2I:box 2:bay 5, SAS, 600 GB, OK)

physicaldrive 2I:2:6 (port 2I:box 2:bay 6, SAS, 600 GB, OK)

physicaldrive 2I:2:7 (port 2I:box 2:bay 7, SAS, 600 GB, OK)

physicaldrive 2I:2:8 (port 2I:box 2:bay 8, SAS, 600 GB, OK)

SEP (Vendor ID PMCSIERA, Model SRCv8x6G) 380 (WWID: 500143802641BB9F)

[root@sdw7 ~]#

[root@sdw7 tmp]# grep -E 'Smart Array.*Serial SCSI Physical Drive Error Log.*|Errors Logged.*' ADUReport.txt

Smart Array P420i in Embedded Slot : Internal Drive Cage at Port 1I : Box 2 : Drive Cage on Port 1I : Physical Drive (600 GB SAS) 1I:2:1 : Serial SCSI Physical Drive Error Log

Errors Logged                        40 (0x00000028)

Smart Array P420i in Embedded Slot : Internal Drive Cage at Port 1I : Box 2 : Drive Cage on Port 1I : Physical Drive (600 GB SAS) 1I:2:2 : Serial SCSI Physical Drive Error Log

Errors Logged                        40 (0x00000028)

Smart Array P420i in Embedded Slot : Internal Drive Cage at Port 1I : Box 2 : Drive Cage on Port 1I : Physical Drive (600 GB SAS) 1I:2:3 : Serial SCSI Physical Drive Error Log

Errors Logged                        40 (0x00000028)

Smart Array P420i in Embedded Slot : Internal Drive Cage at Port 1I : Box 2 : Drive Cage on Port 1I : Physical Drive (600 GB SAS) 1I:2:4 : Serial SCSI Physical Drive Error Log

Errors Logged                        40 (0x00000028)

Smart Array P420i in Embedded Slot : Internal Drive Cage at Port 2I : Box 2 : Drive Cage on Port 2I : Physical Drive (600 GB SAS) 2I:2:5 : Serial SCSI Physical Drive Error Log

Errors Logged                        40 (0x00000028)

Smart Array P420i in Embedded Slot : Internal Drive Cage at Port 2I : Box 2 : Drive Cage on Port 2I : Physical Drive (600 GB SAS) 2I:2:6 : Serial SCSI Physical Drive Error Log

Errors Logged                        40 (0x00000028)

Smart Array P420i in Embedded Slot : Internal Drive Cage at Port 2I : Box 2 : Drive Cage on Port 2I : Physical Drive (600 GB SAS) 2I:2:7 : Serial SCSI Physical Drive Error Log

Errors Logged                        40 (0x00000028)

Smart Array P420i in Embedded Slot : Internal Drive Cage at Port 2I : Box 2 : Drive Cage on Port 2I : Physical Drive (600 GB SAS) 2I:2:8 : Serial SCSI Physical Drive Error Log

Errors Logged                        40 (0x00000028)

Smart Array P420 in slot 2 : Internal Drive Cage at Port 1I : Box 1 : Drive Cage on Port 1I : Physical Drive (600 GB SAS) 1I:1:1 : Serial SCSI Physical Drive Error Log

Errors Logged                        40 (0x00000028)

Smart Array P420 in slot 2 : Internal Drive Cage at Port 1I : Box 1 : Drive Cage on Port 1I : Physical Drive (600 GB SAS) 1I:1:2 : Serial SCSI Physical Drive Error Log

Errors Logged                        4 (0x00000004)

Smart Array P420 in slot 2 : Internal Drive Cage at Port 1I : Box 1 : Drive Cage on Port 1I : Physical Drive (600 GB SAS) 1I:1:3 : Serial SCSI Physical Drive Error Log

Errors Logged                        40 (0x00000028)

Smart Array P420 in slot 2 : Internal Drive Cage at Port 1I : Box 1 : Drive Cage on Port 1I : Physical Drive (600 GB SAS) 1I:1:4 : Serial SCSI Physical Drive Error Log

Errors Logged                        40 (0x00000028)

Smart Array P420 in slot 2 : Internal Drive Cage at Port 2I : Box 1 : Drive Cage on Port 2I : Physical Drive (600 GB SAS) 2I:1:5 : Serial SCSI Physical Drive Error Log

Errors Logged                        40 (0x00000028)

Smart Array P420 in slot 2 : Internal Drive Cage at Port 2I : Box 1 : Drive Cage on Port 2I : Physical Drive (600 GB SAS) 2I:1:6 : Serial SCSI Physical Drive Error Log

Errors Logged                        40 (0x00000028)

Smart Array P420 in slot 2 : Internal Drive Cage at Port 2I : Box 1 : Drive Cage on Port 2I : Physical Drive (600 GB SAS) 2I:1:7 : Serial SCSI Physical Drive Error Log

Errors Logged                        40 (0x00000028)

Smart Array P420 in slot 2 : Internal Drive Cage at Port 2I : Box 1 : Drive Cage on Port 2I : Physical Drive (600 GB SAS) 2I:1:8 : Serial SCSI Physical Drive Error Log

Errors Logged                        40 (0x00000028)

[root@sdw7 tmp]#