#

#	20200408

#	It’s how to show link absent on one network interface.

#

root@ds-sage-soc-eswarm-prod01:~# ethtool eno5 | egrep -e "Speed:|Link detected:"

Speed: Unknown!

Link detected: no

root@ds-sage-soc-eswarm-prod01:~# ethtool eno6 | egrep -e "Speed:|Link detected:"

Speed: 25000Mb/s

Link detected: yes

root@ds-sage-soc-eswarm-prod01:~#

Можно использовать команду ip link show чтобы быстро глянуть статус интерфейсов (поднят или нет).

eeepc901 ~ # ip link show

1: lo: <LOOPBACK,UP,LOWER_UP> mtu 16436 qdisc noqueue state UNKNOWN

link/loopback 00:00:00:00:00:00 brd 00:00:00:00:00:00

2: eth0: <NO-CARRIER,BROADCAST,MULTICAST,UP> mtu 1500 qdisc pfifo_fast state DOWN qlen 1000

link/ether 00:22:15:97:3b:e6 brd ff:ff:ff:ff:ff:ff

3: wlan0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc mq state UP qlen 1000

link/ether 00:15:af:ed:ad:53 brd ff:ff:ff:ff:ff:ff

4: vboxnet0: <BROADCAST,MULTICAST> mtu 1500 qdisc noop state DOWN qlen 1000

link/ether 0a:00:27:00:00:00 brd ff:ff:ff:ff:ff:ff

root@ds-pr-pg-amber01 ~$ ethtool -i ens2f0

driver: qlcnic

version: 5.3.66

firmware-version: 4.14.26

expansion-rom-version:

bus-info: 0000:0b:00.0

supports-statistics: yes

supports-test: yes

supports-eeprom-access: yes

supports-register-dump: yes

supports-priv-flags: no

root@ds-pr-pg-amber01 ~$

root@ds-pr-pg-amber01 ~$ lspci -s 0000:0b:00.0

0b:00.0 Ethernet controller: QLogic Corp. cLOM8214 1/10GbE Controller (rev 54)

root@ds-pr-pg-amber01 ~$

root@ds-pr-pg-amber01 ~$ lspci -v -s 0000:0b:00.0

0b:00.0 Ethernet controller: QLogic Corp. cLOM8214 1/10GbE Controller (rev 54)

Subsystem: Hewlett-Packard Company CN1000Q Dual Port Converged Network Adapter

Physical Slot: 2

Flags: bus master, fast devsel, latency 0, IRQ 44

Memory at fbe00000 (64-bit, non-prefetchable) [size=2M]

Memory at fbdf0000 (64-bit, non-prefetchable) [size=64K]

[virtual] Expansion ROM at fb100000 [disabled] [size=256K]

Capabilities: [40] MSI-X: Enable+ Count=32 Masked-

Capabilities: [80] Power Management version 3

Capabilities: [88] Vital Product Data

Capabilities: [a0] MSI: Enable- Count=1/32 Maskable- 64bit+

Capabilities: [c0] Express Endpoint, MSI 00

Capabilities: [100] Advanced Error Reporting

Capabilities: [140] Device Serial Number fe-54-61-d0-2c-27-d7-ff

Kernel driver in use: qlcnic

Kernel modules: qlcnic

root@ds-pr-pg-amber01 ~$

root@ds-pr-pg-amber01 ~$ ethtool enp4s0f0                                                                                                                                                               [49/691]

Settings for enp4s0f0:

Supported ports: [ TP ]

Supported link modes:   10baseT/Half 10baseT/Full

100baseT/Half 100baseT/Full

1000baseT/Full

Supported pause frame use: No

Supports auto-negotiation: Yes

Supported FEC modes: Not reported

Advertised link modes:  10baseT/Half 10baseT/Full

100baseT/Half 100baseT/Full

1000baseT/Full

Advertised pause frame use: No

Advertised auto-negotiation: Yes

Advertised FEC modes: Not reported

Speed: Unknown!

Duplex: Unknown! (255)

Port: Twisted Pair

PHYAD: 1

Transceiver: internal

Auto-negotiation: on

MDI-X: Unknown

Supports Wake-on: g

Wake-on: g

Link detected: no

root@ds-pr-pg-amber01 ~$

root@ds-pr-pg-amber01 ~$ ethtool -S enp4s0f0                                                                                                                                                            [24/691]

NIC statistics:

rx_bytes: 0

rx_error_bytes: 0

tx_bytes: 0

tx_error_bytes: 0

rx_ucast_packets: 0

rx_mcast_packets: 0

rx_bcast_packets: 0

tx_ucast_packets: 0

tx_mcast_packets: 0

tx_bcast_packets: 0

tx_mac_errors: 0

tx_carrier_errors: 0

rx_crc_errors: 0

rx_align_errors: 0

tx_single_collisions: 0

tx_multi_collisions: 0

tx_deferred: 0

tx_excess_collisions: 0

tx_late_collisions: 0

tx_total_collisions: 0

rx_fragments: 0

rx_jabbers: 0

rx_undersize_packets: 0

rx_oversize_packets: 0

rx_oversize_packets: 0

rx_64_byte_packets: 0

rx_65_to_127_byte_packets: 0

rx_128_to_255_byte_packets: 0

rx_256_to_511_byte_packets: 0

rx_512_to_1023_byte_packets: 0

rx_1024_to_1522_byte_packets: 0

rx_1523_to_9022_byte_packets: 0

tx_64_byte_packets: 0

tx_65_to_127_byte_packets: 0

tx_128_to_255_byte_packets: 0

tx_256_to_511_byte_packets: 0

tx_512_to_1023_byte_packets: 0

tx_1024_to_1522_byte_packets: 0

tx_1523_to_9022_byte_packets: 0

rx_xon_frames: 0

rx_xoff_frames: 0

tx_xon_frames: 0

tx_xoff_frames: 0

rx_mac_ctrl_frames: 0

rx_filtered_packets: 0

rx_ftq_discards: 0

rx_discards: 0

rx_fw_discards: 0

root@ds-pr-pg-amber01 ~$

#

#	But it works only for 1Gbps interface.

#

root@ds-pr-pg-amber01 ~$ mii-tool enp4s0f0

enp4s0f0: no link

root@ds-pr-pg-amber01 ~$

root@ds-pr-pg-amber01 ~$ mii-tool -v enp4s0f0

enp4s0f0: no link

product info: vendor 00:50:ef, model 60 rev 8

basic mode:   autonegotiation enabled

basic status: no link

capabilities: 1000baseT-FD 100baseTx-FD 100baseTx-HD 10baseT-FD 10baseT-HD

advertising:  1000baseT-FD 100baseTx-FD 100baseTx-HD 10baseT-FD 10baseT-HD flow-control

root@ds-pr-pg-amber01 ~$

root@ds-pr-pg-amber01 ~$ mii-tool -v

No interface specified

usage: mii-tool [-VvRrwl] [-A media,... | -F media] [-p addr] <interface ...>

-V, --version               display version information

-v, --verbose               more verbose output

-R, --reset                 reset MII to poweron state

-r, --restart               restart autonegotiation

-w, --watch                 monitor for link status changes

-l, --log                   with -w, write events to syslog

-A, --advertise=media,...   advertise only specified media

-F, --force=media           force specified media technology

-p, --phy=addr              set PHY (MII address) to report

media: 1000baseTx-HD, 1000baseTx-FD,

100baseT4, 100baseTx-FD, 100baseTx-HD,

10baseT-FD, 10baseT-HD,

(to advertise both HD and FD) 1000baseTx, 100baseTx, 10baseT

root@ds-pr-pg-amber01 ~$

#

#	For 10Gbps interface it’s not support.

#

root@ds-pr-pg-amber01 ~$ mii-tool ens2f0

SIOCGMIIPHY on 'ens2f0' failed: Operation not supported

root@ds-pr-pg-amber01 ~$ mii-tool -v ens2f0

SIOCGMIIPHY on 'ens2f0' failed: Operation not supported

root@ds-pr-pg-amber01 ~$

root@ds-pr-pg-amber01 ~$ lspci -s 0000:0b:00

0b:00.0 Ethernet controller: QLogic Corp. cLOM8214 1/10GbE Controller (rev 54)

0b:00.1 Ethernet controller: QLogic Corp. cLOM8214 1/10GbE Controller (rev 54)

0b:00.6 Fibre Channel: QLogic Corp. 8200 Series 10GbE Converged Network Adapter (FCoE) (rev 54)

0b:00.7 Fibre Channel: QLogic Corp. 8200 Series 10GbE Converged Network Adapter (FCoE) (rev 54)

root@ds-pr-pg-amber01 ~$

root@ds-pr-pg-amber01 ~$ ethtool -i ens2f0

driver: qlcnic

version: 5.3.66

firmware-version: 4.14.26

expansion-rom-version:

bus-info: 0000:0b:00.0

supports-statistics: yes

supports-test: yes

supports-eeprom-access: yes

supports-register-dump: yes

supports-priv-flags: no

root@ds-pr-pg-amber01 ~$

root@ds-pr-pg-amber01 ~$ lspci -s 0000:04:00.

04:00.0 Ethernet controller: Broadcom Inc. and subsidiaries NetXtreme II BCM5709 Gigabit Ethernet (rev 20)

04:00.1 Ethernet controller: Broadcom Inc. and subsidiaries NetXtreme II BCM5709 Gigabit Ethernet (rev 20)

root@ds-pr-pg-amber01 ~$

root@ds-pr-pg-amber01 ~$ ethtool -i enp4s0f0

driver: bnx2

version: 2.2.6

firmware-version: bc 5.2.3

expansion-rom-version:

bus-info: 0000:04:00.0

supports-statistics: yes

supports-test: yes

supports-eeprom-access: yes

supports-register-dump: yes

supports-priv-flags: no

root@ds-pr-pg-amber01 ~$