20140902

By email from ossec I have received message like this one.

-----Original Message-----

From: OSSEC HIDS [mailto:ossec@ds-gp-prod-sm.tcsbank.ru]

Sent: Tuesday, September 02, 2014 11:20 AM

To: ossec@localhost.tcsbank.ru

Subject: OSSEC Notification - (ds-gp-prod-sdw7) 10.216.2.7 - Alert level 2

OSSEC HIDS Notification.

2014 Sep 02 11:19:23

Received From: (ds-gp-prod-sdw7) 10.216.2.7->/var/log/messages

Rule: 1002 fired (level 2) -> "Unknown problem somewhere in the system."

Portion of the log(s):

Sep  2 11:17:11 sdw7 kernel: hpsa 0000:07:00.0: FAILED abort on device C3:B0:T0:L0

--END OF NOTIFICATION

Into system log it looks like this.

[root@sdw7 tmp.24rBSJHPqC]# tail -n 20 /var/log/messages

Sep  2 04:25:06 sdw7 auditd[2543]: Audit daemon rotating log files

Sep  2 08:40:55 sdw7 kernel: hpsa 0000:07:00.0: Abort request on C3:B0:T0:L0

Sep  2 08:40:55 sdw7 kernel: hpsa 0000:07:00.0: cp ffff88007ee5f000 is reported invalid (probably means target device no longer present)

Sep  2 08:40:55 sdw7 kernel: hpsa 0000:07:00.0: cp ffff88007ee5f000 is reported invalid (probably means target device no longer present)

Sep  2 08:40:55 sdw7 kernel: hpsa 0000:07:00.0: FAILED abort on device C3:B0:T0:L0

Sep  2 08:40:55 sdw7 kernel: hpsa 0000:07:00.0: resetting device 3:0:0:0

Sep  2 08:40:55 sdw7 kernel: hpsa 0000:07:00.0: device is ready.

Sep  2 09:52:14 sdw7 auditd[2543]: Audit daemon rotating log files

Sep  2 11:15:28 sdw7 kernel: hpsa 0000:07:00.0: Abort request on C3:B0:T0:L0

Sep  2 11:15:28 sdw7 kernel: hpsa 0000:07:00.0: cp ffff88000ba3d000 is reported invalid (probably means target device no longer present)

Sep  2 11:15:28 sdw7 kernel: hpsa 0000:07:00.0: cp ffff88000ba3d000 is reported invalid (probably means target device no longer present)

Sep  2 11:15:28 sdw7 kernel: hpsa 0000:07:00.0: FAILED abort on device C3:B0:T0:L0

Sep  2 11:15:28 sdw7 kernel: hpsa 0000:07:00.0: resetting device 3:0:0:0

Sep  2 11:15:28 sdw7 kernel: hpsa 0000:07:00.0: device is ready.

Sep  2 11:17:11 sdw7 kernel: hpsa 0000:07:00.0: Abort request on C3:B0:T0:L0

Sep  2 11:17:11 sdw7 kernel: hpsa 0000:07:00.0: cp ffff88008f15d000 is reported invalid (probably means target device no longer present)

Sep  2 11:17:11 sdw7 kernel: hpsa 0000:07:00.0: cp ffff88008f15d000 is reported invalid (probably means target device no longer present)

Sep  2 11:17:11 sdw7 kernel: hpsa 0000:07:00.0: FAILED abort on device C3:B0:T0:L0

Sep  2 11:17:11 sdw7 kernel: hpsa 0000:07:00.0: resetting device 3:0:0:0

Sep  2 11:17:44 sdw7 kernel: hpsa 0000:07:00.0: device is ready.

[root@sdw7 tmp.24rBSJHPqC]#

But unfortunately info from logs can help us to identify only controller and we cannot identify hard disk drive from it.

Sep  2 11:17:11 sdw7 kernel: hpsa 0000:07:00.0: FAILED abort on device C3:B0:T0:L0

Here we can see - this is external Raid Controller (it’s absent “i” caracter).

[root@sdw7 tmp.24rBSJHPqC]# lspci -s 0000:07:00.0

07:00.0 RAID bus controller: Hewlett-Packard Company Device 323b (rev 01)

[root@sdw7 tmp.24rBSJHPqC]# lspci -v -s 0000:07:00.0

07:00.0 RAID bus controller: Hewlett-Packard Company Device 323b (rev 01)

Subsystem: Hewlett-Packard Company Device 3351

Physical Slot: 2

Flags: bus master, fast devsel, latency 0, IRQ 26

Memory at f7f00000 (64-bit, non-prefetchable) [size=1M]

Memory at f7ef0000 (64-bit, non-prefetchable) [size=1K]

I/O ports at 6000 [size=256]

[virtual] Expansion ROM at f4000000 [disabled] [size=512K]

Capabilities: [80] Power Management version 3

Capabilities: [90] MSI: Enable- Count=1/32 Maskable+ 64bit+

Capabilities: [b0] MSI-X: Enable+ Count=64 Masked-

Capabilities: [c0] Express Endpoint, MSI 00

Capabilities: [100] Advanced Error Reporting

Capabilities: [300] #19

Kernel driver in use: hpsa

Kernel modules: hpsa

[root@sdw7 tmp.24rBSJHPqC]#

And from this parameter we still have only receive controller, but not hard drive.

Sep  2 11:17:11 sdw7 kernel: hpsa 0000:07:00.0: FAILED abort on device C3:B0:T0:L0

It’s understands as Controller, Bus, Target, Lun numbers.

[root@sdw7 tmp.24rBSJHPqC]# cat /sys/class/scsi_disk/3:0:0:0/device/unique_id

600508B1001CF30011419FD5D576C9F2

[root@sdw7 tmp.24rBSJHPqC]# cat /sys/class/scsi_disk/3:0:0:0/device/lunid

0x0000004000000000

[root@sdw7 tmp.24rBSJHPqC]# cat /sys/class/scsi_disk/3:0:0:0/device/raid_level

RAID 5

[root@sdw7 tmp.24rBSJHPqC]#

https://github.com/alibaba/taobao-kernel/blob/master/redhat-kernel-source/linux-2.6.32/Documentation/scsi/hpsa.txt

HPSA - Hewlett Packard Smart Array driver

-----------------------------------------

This file describes the hpsa SCSI driver for HP Smart Array controllers.

The hpsa driver is intended to supplant the cciss driver for newer

Smart Array controllers.  The hpsa driver is a SCSI driver, while the

cciss driver is a "block" driver.  Actually cciss is both a block

driver (for logical drives) AND a SCSI driver (for tape drives). This

"split-brained" design of the cciss driver is a source of excess

complexity and eliminating that complexity is one of the reasons

for hpsa to exist.

Supported devices:

------------------

Smart Array P212

Smart Array P410

Smart Array P410i

Smart Array P411

Smart Array P812

Smart Array P712m

Smart Array P711m

StorageWorks P1210m

Additionally, older Smart Arrays may work with the hpsa driver if the kernel

boot parameter "hpsa_allow_any=1" is specified, however these are not tested

nor supported by HP with this driver.  For older Smart Arrays, the cciss

driver should still be used.

The "hpsa_simple_mode=1" boot parameter may be used to prevent the driver from

putting the controller into "performant" mode.  The difference is that with simple

mode, each command completion requires an interrupt, while with "performant mode"

(the default, and ordinarily better performing) it is possible to have multiple

command completions indicated by a single interrupt.

HPSA specific entries in /sys

-----------------------------

In addition to the generic SCSI attributes available in /sys, hpsa supports

the following attributes:

HPSA specific host attributes:

------------------------------

/sys/class/scsi_host/host*/rescan

/sys/class/scsi_host/host*/firmware_revision

/sys/class/scsi_host/host*/resettable

/sys/class/scsi_host/host*/transport_mode

/sys/class/scsi_host/host*/heartbeat

the host "rescan" attribute is a write only attribute.  Writing to this

attribute will cause the driver to scan for new, changed, or removed devices

(e.g. hot-plugged tape drives, or newly configured or deleted logical drives,

etc.) and notify the SCSI midlayer of any changes detected.  Normally this is

triggered automatically by HP's Array Configuration Utility (either the GUI or

command line variety) so for logical drive changes, the user should not

normally have to use this.  It may be useful when hot plugging devices like

tape drives, or entire storage boxes containing pre-configured logical drives.

The "firmware_revision" attribute contains the firmware version of the Smart Array.

For example:

root@host:/sys/class/scsi_host/host4# cat firmware_revision

7.14

The transport_mode indicates whether the controller is in "performant"

or "simple" mode.  This is controlled by the "hpsa_simple_mode" module

parameter.

The "resettable" read-only attribute indicates whether a particular

controller is able to honor the "reset_devices" kernel parameter.  If the

device is resettable, this file will contain a "1", otherwise, a "0".  This

parameter is used by kdump, for example, to reset the controller at driver

load time to eliminate any outstanding commands on the controller and get the

controller into a known state so that the kdump initiated i/o will work right

and not be disrupted in any way by stale commands or other stale state

remaining on the controller from the previous kernel.  This attribute enables

kexec tools to warn the user if they attempt to designate a device which is

unable to honor the reset_devices kernel parameter as a dump device.

The "heartbeat" read-only attribute returns the value of a heartbeat

counter register on a Smart Array controller as a 32 bit unsigned

hexadecimal integer (e.g: "0x12345678").  The value should change

periodically, not less than once per second.  If this value fails to

change for a period longer than one second, it means something has

gone wrong (e.g. Smart Array controller firmware has locked up.)

HPSA specific disk attributes:

------------------------------

/sys/class/scsi_disk/c:b:t:l/device/unique_id

/sys/class/scsi_disk/c:b:t:l/device/raid_level

/sys/class/scsi_disk/c:b:t:l/device/lunid

(where c:b:t:l are the controller, bus, target and lun of the device)

For example:

root@host:/sys/class/scsi_disk/4:0:0:0/device# cat unique_id

600508B1001044395355323037570F77

root@host:/sys/class/scsi_disk/4:0:0:0/device# cat lunid

0x0000004000000000

root@host:/sys/class/scsi_disk/4:0:0:0/device# cat raid_level

RAID 0

HPSA specific ioctls:

---------------------

For compatibility with applications written for the cciss driver, many, but

not all of the ioctls supported by the cciss driver are also supported by the

hpsa driver.  The data structures used by these are described in

include/linux/cciss_ioctl.h

CCISS_DEREGDISK

CCISS_REGNEWDISK

CCISS_REGNEWD

The above three ioctls all do exactly the same thing, which is to cause the driver

to rescan for new devices.  This does exactly the same thing as writing to the

hpsa specific host "rescan" attribute.

CCISS_GETPCIINFO

Returns PCI domain, bus, device and function and "board ID" (PCI subsystem ID).

CCISS_GETDRIVVER

Returns driver version in three bytes encoded as:

(major_version << 16) | (minor_version << 8) | (subminor_version)

CCISS_PASSTHRU

CCISS_BIG_PASSTHRU

Allows "BMIC" and "CISS" commands to be passed through to the Smart Array.

These are used extensively by the HP Array Configuration Utility, SNMP storage

agents, etc.  See cciss_vol_status at http://cciss.sf.net for some examples.

[support@centos00 tmp]$ yumdownloader lsscsi

Loaded plugins: refresh-packagekit

tcs-atomic-centos-6-x86_64                                                                                                                                                            | 2.9 kB     00:00

tcs-atomic-centos-6-x86_64/primary_db                                                                                                                                                 | 481 kB     00:00

tcs-centos-6-centosplus-x86_64                                                                                                                                                        | 2.9 kB     00:00

tcs-centos-6-centosplus-x86_64/primary_db                                                                                                                                             | 2.4 MB     00:00

tcs-centos-6-extras-x86_64                                                                                                                                                            | 2.9 kB     00:00

tcs-centos-6-extras-x86_64/primary_db                                                                                                                                                 |  19 kB     00:00

tcs-centos-6-os-x86_64                                                                                                                                                                | 2.9 kB     00:00

tcs-centos-6-os-x86_64/primary_db                                                                                                                                                     | 4.0 MB     00:00

tcs-centos-6-updates-x86_64                                                                                                                                                           | 2.9 kB     00:00

tcs-centos-6-updates-x86_64/primary_db                                                                                                                                                | 4.8 MB     00:00

tcs-centos-6.4-x86_64-iso-dvd1                                                                                                                                                        | 3.8 kB     00:00

tcs-centos-6.4-x86_64-iso-dvd1/primary_db                                                                                                                                             | 3.0 MB     00:00

tcs-centos-6.4-x86_64-iso-dvd2                                                                                                                                                        | 2.9 kB     00:00

tcs-centos-6.4-x86_64-iso-dvd2/primary_db                                                                                                                                             | 1.1 MB     00:00

tcs-epel-6-x86_64                                                                                                                                                                     | 2.9 kB     00:00

tcs-epel-6-x86_64/primary_db                                                                                                                                                          | 5.1 MB     00:00

tcs-hp-hpsum                                                                                                                                                                          | 2.9 kB     00:00

tcs-hp-hpsum/primary_db                                                                                                                                                               | 111 kB     00:00

tcs-hp-spp                                                                                                                                                                            | 2.9 kB     00:00

tcs-hp-spp/primary_db                                                                                                                                                                 | 458 kB     00:00

tcs-hp-stk                                                                                                                                                                            | 2.9 kB     00:00

tcs-hp-stk/primary_db                                                                                                                                                                 |  11 kB     00:00

tcs-lldpad                                                                                                                                                                            | 2.9 kB     00:00

tcs-lldpad/primary_db                                                                                                                                                                 | 6.4 kB     00:00

tcs-remi-enterprise-6-remi-x86_64                                                                                                                                                     | 2.9 kB     00:00

tcs-remi-enterprise-6-remi-x86_64/primary_db                                                                                                                                          | 868 kB     00:00

tcs-rpmforge-redhat-el6-en-x86_64                                                                                                                                                     | 2.9 kB     00:00

tcs-rpmforge-redhat-el6-en-x86_64/primary_db                                                                                                                                          | 2.4 MB     00:00

lsscsi-0.23-2.el6.x86_64.rpm                                                                                                                                                          |  38 kB     00:00

[support@centos00 tmp]$ cd -

/home/support/work/20140826-1930

[support@centos00 20140826-1930]$ scp /tmp/lsscsi-0.23-2.el6.x86_64.rpm bsv@ds-gp-prod-sm:/tmp/

bsv@ds-gp-prod-sm's password:

lsscsi-0.23-2.el6.x86_64.rpm                                                                                                                                               100%   38KB  38.1KB/s   00:00

[support@centos00 20140826-1930]$

[root@sdw7 ~]# rpm -Uvh /tmp/lsscsi-0.23-2.el6.x86_64.rpm

warning: /tmp/lsscsi-0.23-2.el6.x86_64.rpm: Header V3 RSA/SHA256 Signature, key ID c105b9de: NOKEY

Preparing...                ########################################### [100%]

1:lsscsi                 ########################################### [100%]

[root@sdw7 ~]#

[root@sdw7 ~]# lsscsi -g

[2:0:0:0]    disk    HP       LOGICAL VOLUME   5.42  /dev/sda   /dev/sg1

[2:3:0:0]    storage HP       P420i            5.42  -         /dev/sg0

[3:0:0:0]    disk    HP       LOGICAL VOLUME   5.42  /dev/sdb   /dev/sg3

[3:3:0:0]    storage HP       P420             5.42  -         /dev/sg2

[root@sdw7 ~]# lsscsi

[2:0:0:0]    disk    HP       LOGICAL VOLUME   5.42  /dev/sda

[2:3:0:0]    storage HP       P420i            5.42  -

[3:0:0:0]    disk    HP       LOGICAL VOLUME   5.42  /dev/sdb

[3:3:0:0]    storage HP       P420             5.42  -

[root@sdw7 ~]#

[root@sdw7 ~]# lsscsi --help

Usage: lsscsi   [--classic] [--device] [--generic] [--help] [--hosts]

[--kname] [--list] [--long] [--protection] [--sysfsroot=PATH]

[--transport] [--verbose] [--version] [<h:c:t:l>]

where:

--classic|-c      alternate output similar to 'cat /proc/scsi/scsi'

--device|-d       show device node's major + minor numbers

--generic|-g      show scsi generic device name

--help|-h         this usage information

--hosts|-H        lists scsi hosts rather than scsi devices

--kname|-k        show kernel name instead of device node name

--list|-L         additional information output one

attribute=value per line

--long|-l         additional information output

--protection|-p   show data integrity (protection) information

--sysfsroot=PATH|-y PATH    set sysfs mount point to PATH (def: /sys)

--transport|-t    transport information for target or, if '--hosts'

given, for initiator

--verbose|-v      output path names where data is found

--version|-V      output version string and exit

<h:c:t:l>         filter output list (def: '- - - -' (all))

List SCSI devices or hosts, optionally with additional information

[root@sdw7 ~]# lsscsi -c

Attached devices:

Host: scsi2 Channel: 00 Target: 00 Lun: 00

Vendor: HP       Model: LOGICAL VOLUME   Rev: 5.42

Type:   Direct-Access                    ANSI SCSI revision: 05

Host: scsi2 Channel: 03 Target: 00 Lun: 00

Vendor: HP       Model: P420i            Rev: 5.42

Type:   Storage array                    ANSI SCSI revision: 05

Host: scsi3 Channel: 00 Target: 00 Lun: 00

Vendor: HP       Model: LOGICAL VOLUME   Rev: 5.42

Type:   Direct-Access                    ANSI SCSI revision: 05

Host: scsi3 Channel: 03 Target: 00 Lun: 00

Vendor: HP       Model: P420             Rev: 5.42

Type:   Storage array                    ANSI SCSI revision: 05

[root@sdw7 ~]#

[root@sdw7 ~]# lsscsi -d

[2:0:0:0]    disk    HP       LOGICAL VOLUME   5.42  /dev/sda [8:0]

[2:3:0:0]    storage HP       P420i            5.42  -

[3:0:0:0]    disk    HP       LOGICAL VOLUME   5.42  /dev/sdb [8:16]

[3:3:0:0]    storage HP       P420             5.42  -

[root@sdw7 ~]# lsscsi -g

[2:0:0:0]    disk    HP       LOGICAL VOLUME   5.42  /dev/sda   /dev/sg1

[2:3:0:0]    storage HP       P420i            5.42  -         /dev/sg0

[3:0:0:0]    disk    HP       LOGICAL VOLUME   5.42  /dev/sdb   /dev/sg3

[3:3:0:0]    storage HP       P420             5.42  -         /dev/sg2

[root@sdw7 ~]# lsscsi -H

[0]    ata_piix

[1]    ata_piix

[2]    hpsa

[3]    hpsa

[4]    be2iscsi

[5]    be2iscsi

[root@sdw7 ~]# lsscsi -k

[2:0:0:0]    disk    HP       LOGICAL VOLUME   5.42  /dev/sda

[2:3:0:0]    storage HP       P420i            5.42  -

[3:0:0:0]    disk    HP       LOGICAL VOLUME   5.42  /dev/sdb

[3:3:0:0]    storage HP       P420             5.42  -

[root@sdw7 ~]# lsscsi -L

[2:0:0:0]    disk    HP       LOGICAL VOLUME   5.42  /dev/sda

device_blocked=0

iocounterbits=32

iodone_cnt=0x7c325ca8

ioerr_cnt=0x0

iorequest_cnt=0x7c325caa

queue_depth=1020

queue_type=none

scsi_level=6

state=running

timeout=30

type=0

[2:3:0:0]    storage HP       P420i            5.42  -

device_blocked=0

iocounterbits=32

iodone_cnt=0x2

ioerr_cnt=0x0

iorequest_cnt=0x2

queue_depth=1020

queue_type=none

scsi_level=6

state=running

timeout=0

type=12

[3:0:0:0]    disk    HP       LOGICAL VOLUME   5.42  /dev/sdb

device_blocked=0

iocounterbits=32

iodone_cnt=0x76c1073c

ioerr_cnt=0x0

iorequest_cnt=0x76c10755

queue_depth=1020

queue_type=none

scsi_level=6

state=running

timeout=30

type=0

[3:3:0:0]    storage HP       P420             5.42  -

device_blocked=0

iocounterbits=32

iodone_cnt=0x2

ioerr_cnt=0x0

iorequest_cnt=0x2

queue_depth=1020

queue_type=none

scsi_level=6

state=running

timeout=0

type=12

[root@sdw7 ~]#

[root@sdw7 ~]# lsscsi -l

[2:0:0:0]    disk    HP       LOGICAL VOLUME   5.42  /dev/sda

state=running queue_depth=1020 scsi_level=6 type=0 device_blocked=0 timeout=30

[2:3:0:0]    storage HP       P420i            5.42  -

state=running queue_depth=1020 scsi_level=6 type=12 device_blocked=0 timeout=0

[3:0:0:0]    disk    HP       LOGICAL VOLUME   5.42  /dev/sdb

state=running queue_depth=1020 scsi_level=6 type=0 device_blocked=0 timeout=30

[3:3:0:0]    storage HP       P420             5.42  -

state=running queue_depth=1020 scsi_level=6 type=12 device_blocked=0 timeout=0

[root@sdw7 ~]# lsscsi -p

[2:0:0:0]    disk    HP       LOGICAL VOLUME   5.42  /dev/sda

[2:3:0:0]    storage HP       P420i            5.42  -         -

[3:0:0:0]    disk    HP       LOGICAL VOLUME   5.42  /dev/sdb

[3:3:0:0]    storage HP       P420             5.42  -         -

[root@sdw7 ~]# lsscsi -t

[2:0:0:0]    disk                                    /dev/sda

[2:3:0:0]    storage                                 -

[3:0:0:0]    disk                                    /dev/sdb

[3:3:0:0]    storage                                 -

[root@sdw7 ~]# lsscsi -v

[2:0:0:0]    disk    HP       LOGICAL VOLUME   5.42  /dev/sda

dir: /sys/bus/scsi/devices/2:0:0:0  [/sys/devices/pci0000:00/0000:00:02.2/0000:02:00.0/host2/target2:0:0/2:0:0:0]

[2:3:0:0]    storage HP       P420i            5.42  -

dir: /sys/bus/scsi/devices/2:3:0:0  [/sys/devices/pci0000:00/0000:00:02.2/0000:02:00.0/host2/target2:3:0/2:3:0:0]

[3:0:0:0]    disk    HP       LOGICAL VOLUME   5.42  /dev/sdb

dir: /sys/bus/scsi/devices/3:0:0:0  [/sys/devices/pci0000:00/0000:00:01.0/0000:07:00.0/host3/target3:0:0/3:0:0:0]

[3:3:0:0]    storage HP       P420             5.42  -

dir: /sys/bus/scsi/devices/3:3:0:0  [/sys/devices/pci0000:00/0000:00:01.0/0000:07:00.0/host3/target3:3:0/3:3:0:0]

[root@sdw7 ~]#

[root@sdw7 ~]# lsscsi 3:0:0:0

[3:0:0:0]    disk    HP       LOGICAL VOLUME   5.42  /dev/sdb

[root@sdw7 ~]# lsscsi -v 3:0:0:0

[3:0:0:0]    disk    HP       LOGICAL VOLUME   5.42  /dev/sdb

dir: /sys/bus/scsi/devices/3:0:0:0  [/sys/devices/pci0000:00/0000:00:01.0/0000:07:00.0/host3/target3:0:0/3:0:0:0]

[root@sdw7 ~]#

[root@sdw7 ~]# lsscsi -g 3:0:0:0

[3:0:0:0]    disk    HP       LOGICAL VOLUME   5.42  /dev/sdb   /dev/sg3

[root@sdw7 ~]#

[root@sdw7 ~]# smartctl --help

smartctl 5.39.1 2010-01-28 r3054 [x86_64-redhat-linux-gnu] (local build)

Copyright (C) 2002-10 by Bruce Allen, http://smartmontools.sourceforge.net

Usage: smartctl [options] device

============================================ SHOW INFORMATION OPTIONS =====

-h, --help, --usage

Display this help and exit

-V, --version, --copyright, --license

Print license, copyright, and version information and exit

-i, --info

Show identity information for device

-a, --all

Show all SMART information for device

-x, --xall

Show all information for device

================================== SMARTCTL RUN-TIME BEHAVIOR OPTIONS =====

-q TYPE, --quietmode=TYPE                                           (ATA)

Set smartctl quiet mode to one of: errorsonly, silent, noserial

-d TYPE, --device=TYPE

Specify device type to one of: ata, scsi, sat[,N][+TYPE], usbcypress[,X], usbjmicron[,x][,N], usbsunplus, marvell, areca,N, 3ware,N, hpt,L/M/N, megaraid,N, cciss,N, test

-T TYPE, --tolerance=TYPE                                           (ATA)

Tolerance: normal, conservative, permissive, verypermissive

-b TYPE, --badsum=TYPE                                              (ATA)

Set action on bad checksum to one of: warn, exit, ignore

-r TYPE, --report=TYPE

Report transactions (see man page)

-n MODE, --nocheck=MODE                                             (ATA)

No check if: never, sleep, standby, idle (see man page)

============================== DEVICE FEATURE ENABLE/DISABLE COMMANDS =====

-s VALUE, --smart=VALUE

Enable/disable SMART on device (on/off)

-o VALUE, --offlineauto=VALUE                                       (ATA)

Enable/disable automatic offline testing on device (on/off)

-S VALUE, --saveauto=VALUE                                          (ATA)

Enable/disable Attribute autosave on device (on/off)

======================================= READ AND DISPLAY DATA OPTIONS =====

-H, --health

Show device SMART health status

-c, --capabilities                                                  (ATA)

Show device SMART capabilities

-A, --attributes

Show device SMART vendor-specific Attributes and values

-l TYPE, --log=TYPE

Show device log. TYPE: error, selftest, selective, directory[,g|s],

background, sasphy[,reset], sataphy[,reset],

scttemp[sts,hist],

gplog,N[,RANGE], smartlog,N[,RANGE],

xerror[,N][,error], xselftest[,N][,selftest]

-v N,OPTION , --vendorattribute=N,OPTION                            (ATA)

Set display OPTION for vendor Attribute N (see man page)

-F TYPE, --firmwarebug=TYPE                                         (ATA)

Use firmware bug workaround: none, samsung, samsung2,

samsung3, swapid

-P TYPE, --presets=TYPE                                             (ATA)

Drive-specific presets: use, ignore, show, showall

-B [+]FILE, --drivedb=[+]FILE                                       (ATA)

Read and replace [add] drive database from FILE

============================================ DEVICE SELF-TEST OPTIONS =====

-t TEST, --test=TEST

Run test. TEST: offline short long conveyance select,M-N

pending,N afterselect,[on|off] scttempint,N[,p]

-C, --captive

Do test in captive mode (along with -t)

-X, --abort

Abort any non-captive test on device

=================================================== SMARTCTL EXAMPLES =====

smartctl --all /dev/hda                    (Prints all SMART information)

smartctl --smart=on --offlineauto=on --saveauto=on /dev/hda

(Enables SMART on first disk)

smartctl --test=long /dev/hda          (Executes extended disk self-test)

smartctl --attributes --log=selftest --quietmode=errorsonly /dev/hda

(Prints Self-Test & Attribute errors)

smartctl --all --device=3ware,2 /dev/sda

smartctl --all --device=3ware,2 /dev/twe0

smartctl --all --device=3ware,2 /dev/twa0

(Prints all SMART info for 3rd ATA disk on 3ware RAID controller)

smartctl --all --device=hpt,1/1/3 /dev/sda

(Prints all SMART info for the SATA disk attached to the 3rd PMPort

of the 1st channel on the 1st HighPoint RAID controller)

smartctl --all --device=areca,3 /dev/sg2

(Prints all SMART info for 3rd ATA disk on Areca RAID controller)

[root@sdw7 ~]# smartctl /dev/sg3

smartctl 5.39.1 2010-01-28 r3054 [x86_64-redhat-linux-gnu] (local build)

Copyright (C) 2002-10 by Bruce Allen, http://smartmontools.sourceforge.net

[root@sdw7 ~]# smartctl /dev/sg2

smartctl 5.39.1 2010-01-28 r3054 [x86_64-redhat-linux-gnu] (local build)

Copyright (C) 2002-10 by Bruce Allen, http://smartmontools.sourceforge.net

[root@sdw7 ~]# smartctl /dev/sdb

smartctl 5.39.1 2010-01-28 r3054 [x86_64-redhat-linux-gnu] (local build)

Copyright (C) 2002-10 by Bruce Allen, http://smartmontools.sourceforge.net

[root@sdw7 ~]#

[root@sdw7 ~]# smartctl --all /dev/sg2

smartctl 5.39.1 2010-01-28 r3054 [x86_64-redhat-linux-gnu] (local build)

Copyright (C) 2002-10 by Bruce Allen, http://smartmontools.sourceforge.net

Device: HP       P420             Version: 5.42

Serial number: PDSXK0ARH4W2KV

Device type: storage array

Local Time is: Tue Sep  2 12:43:14 2014 MSK

Device does not support SMART

Error Counter logging not supported

Device does not support Self Test logging

[root@sdw7 ~]#

[root@sdw7 ~]# smartctl --all /dev/sdb

smartctl 5.39.1 2010-01-28 r3054 [x86_64-redhat-linux-gnu] (local build)

Copyright (C) 2002-10 by Bruce Allen, http://smartmontools.sourceforge.net

Device: HP       LOGICAL VOLUME   Version: 5.42

Serial number: PDSXK0ARH4W2KV

Device type: disk

Local Time is: Tue Sep  2 12:53:42 2014 MSK

Device supports SMART and is Enabled

Temperature Warning Disabled or Not Supported

SMART Health Status: OK

Error Counter logging not supported

Device does not support Self Test logging

[root@sdw7 ~]#

http://smartmontools.sourceforge.net/man/smartctl.8.html

...

DESCRIPTION

smartctl controls the Self-Monitoring, Analysis and Reporting Technology (SMART) system built into most ATA/SATA and SCSI/SAS hard drives and solid-state drives. The purpose of SMART is to monitor the reliability of the hard drive and predict drive failures, and to carry out different types of drive self-tests. smartctl also supports some features not related to SMART. This version of smartctl is compatible with ACS-2, ATA8-ACS, ATA/ATAPI-7 and earlier standards (see REFERENCES below).

smartctl also provides support for polling TapeAlert messages from SCSI tape drives and changers.

The user must specify the device to be controlled or interrogated as the final argument to smartctl. The command set used by the device is often derived from the device path but may need help with the '-d' option (for more information see the section on "ATA, SCSI command sets and SAT" below). Device paths are as follows:

LINUX:

Use the forms "/dev/hd[a-t]" for IDE/ATA devices, and "/dev/sd[a-z]" for SCSI devices. For SCSI Tape Drives and Changers with TapeAlert support use the devices "/dev/nst*" and "/dev/sg*". For SATA disks accessed with libata, use "/dev/sd[a-z]" and append "-d ata". For disks behind 3ware controllers you may need "/dev/sd[a-z]" or "/dev/twe[0-9]", "/dev/twa[0-9]" or "/dev/twl[0-9]": see details below. For disks behind HighPoint RocketRAID controllers you may need "/dev/sd[a-z]". For disks behind Areca SATA RAID controllers, you need "/dev/sg[2-9]" (note that smartmontools interacts with the Areca controllers via a SCSI generic device which is different than the SCSI device used for reading and writing data)! For HP Smart Array RAID controllers, there are three currently supported drivers: cciss, hpsa, and hpahcisr. For disks accessed via the cciss driver the device nodes are of the form "/dev/cciss/c[0-9]d0". For disks accessed via the hpahcisr and hpsa drivers, the device nodes you need are "/dev/sg[0-9]*". ("lsscsi -g" is helpful in determining which scsi generic device node corresponds to which device.) Use the nodes corresponding to the RAID controllers, not the nodes corresponding to logical drives. See the -d option below, as well.

...