Чтобы прицепить UPS к рэковому серверу по Com порту делаем такие настройки.

1.  Запускаем minicom и устанавливаем в нём параметры порта

/dev/ttyS0

2400 8N1

Остальное оставляем по умолчанию. (Hardware Flow control: Yes например).

Ну я ещё поставил в параметрах "cOnfigure Minicom"->"Modem and dialing"->(Auto bps detect: Yes).

И ещё включил (Add linefeed) и (Lockal echo on/off) в основном меню.

Сохранил это в конфигурацию apc

Вышел из minicom по "eXit and reset"

2.  Втыкаем Com-портовый кабель для APC UPS и перезапускаем сетевой адаптер на

UPS - нажимаем reset.

3.  Запускаем minicom apc

Нажимаем "enter" на всякий случай.

MINICOM(1)							    MINICOM(1)

NAME

minicom - friendly serial communication program

SYNOPSIS

minicom [-somMlwz8] [-c on|off] [-S script] [-d entry]

[-a on|off] [-t term] [-p pty] [-C capturefile] [configuration]

DESCRIPTION

minicom is a communication program which somewhat resembles the	share-

ware  program  TELIX  but  is free with source code and runs under most

unices.	Features include dialing directory with	 auto-redial,  support

for UUCP-style lock files on serial devices, a seperate script language

interpreter, capture to file, multiple users with individual configura-

tions, and more.

COMMAND-LINE

-s   Setup.   Root  edits  the  system-wide defaults in /etc/minirc.dfl

with this option.  When it is used, minicom does  not  initialize,

but	 puts  you  directly into the configuration menu. This is very

handy if minicom refuses to	 start	up  because  your  system  has

changed,  or for the first time you run minicom. For most systems,

reasonable defaults are already compiled in.

-o   Do not initialize. Minicom	will  skip  the	 initialization	 code.

This  option  is  handy if you quitted from minicom without reset-

ting, and then want to restart a session. It is  potentially  dan-

gerous  though:  no check for lock files etc. is made, so a normal

user could interfere with things like uucp... Maybe this  will  be

taken  out	later. For now it is assumed, that users who are given

access to a modem are responsible enough for their actions.

-m   Override command-key with the Meta or ALT key. This is the default

in	1.80  and it can also be configured in one of minicom's menus,

but if you use different terminals all the	time,  of  which  some

don't  have	 a Meta or ALT key, it's handy to set the default com-

mand key to Ctrl-A and use this option when you  have  a  keyboard

supporting	Meta  or  ALT keys. Minicom assumes that your Meta key

sends the ESC prefix, not the other variant that sets the  highest

bit of the character.

-M   Same as -m, but assumes that your Meta key sets the 8th bit of the

character high (sends 128 + character code).

-z   Use terminal status line. This only works on terminals  that  sup-

port it and that have the relevant information in their termcap or

terminfo database entry.

-l   Literal translation of characters with the high bit set. With this

flag on, minicom will not try to translate the IBM line characters

to ASCII, but passes them straight	trough.	 Many  PC-unix	clones

will  display  them correctly without translation (Linux in a spe-

cial mode, Coherent and Sco).

-L   Ditto but assume screen uses an ISO8859 character set.

-w   Turns linewrap on at startup by default.

-a   Attribute usage. Some terminals, notably televideo's, have a  rot-

ten	 attribute  handling (serial instead of parallel). By default,

minicom uses '-a on', but if you are using such a terminal you can

(must!)  supply the option '-a off'. The trailing 'on' or 'off' is

needed.

-t   Terminal type. With this flag, you can  override  the  environment

TERM  variable.   This is handy for use in the MINICOM environment

variable; one can create a special	termcap	 entry	for  use  with

minicom on the console, that initializes the screen to raw mode so

that in conjunction with the -l flag, the IBM line characters  are

displayed untranslated.

-c   Color  usage.  Some	 terminals (such as the Linux console) support

color with the standard ANSI escape sequences.  Because  there  is

apparently	no  termcap  support for color, these escape sequences

are hard-coded into minicom.  Therefore  this  option  is  off  by

default.   You  can	 turn  it  on with '-c on'. This, and the '-m'

option, are good candidates to put into  the  MINICOM  environment

variable.

-S   script.  Run the named script at startup. So far, passing username

and password to a startup script is not supported. If you also use

the	 -d  option to start dialing at startup, the -S script will be

run BEFORE dialing the entries specified with -d.

-d   Dial an entry from the dialing directory on startup. You can spec-

ify	 an  index  number,  but  also	a substring of the name of the

entry. If you specify a name that  has  multiple  entries  in  the

directory,	they  are all tagged for dialing. You can also specify

multiple names or index numbers by separating  them	 with  commas.

The	 dialing  will	start from the first entry specified after all

other program initialization procedures are completed.

-p   Pseudo terminal to use. This overrrides the terminal port  defined

in	the  configuration  files, but only if it is a pseudo tty. The

filename supplied  must  be	 of  the  form	(/dev/)tty[p-z/][0-f],

(/dev/)pts[p-z/][0-f]   or	 (/dev/)pty[p-z/][0-f].	 For  example,

/dev/ttyp1, pts/0 or /dev/ptyp2.

-C   filename.  Open capture file at startup.

-T   Disable the display of the online time in the status bar.

-8   8bit characters pass through without any modification.   'Continu-

ous'  means	 no  locate/attribute  control	sequences are inserted

without real change of locate/attribute. This mode is  to  display

8bit  multibyte  characters	 such as Japanese. Not needed in every

language with 8bit characters.  (For  example  displaying  Finnish

text doesn't need this.)

When  minicom  starts,  it	first searches the MINICOM environment

variable for command-line arguments, which can be  over-ridden  on

the command line.  Thus, if you have done

MINICOM='-m -c on'

export MINICOM

or	the  equivalent,  and  start minicom, minicom will assume that

your terminal has a Meta or <ALT> key and that color is supported.

If	you then log in from a terminal without color support, and you

have set MINICOM in your startup (.profile	or  equivalent)	 file,

and	 don't	want to re-set your environment variable, you can type

'minicom -c off' and run without color support for that session.

configuration

The configuration argument is more interesting. Normally,  minicom

gets  its defaults from a file called "minirc.dfl". If you however

give an argument to minicom, it will try to get its defaults  from

a file called "minirc.configuration".  So it is possible to create

multiple configuration files, for different ports, different users

etc.  Most	sensible  is to use device names, such as tty1, tty64,

sio2 etc. If a user creates his own configuration  file,  it  will

show up in his home directory as '.minirc.dfl'.

USE

Minicom	is window based. To popup a window with the function you want,

press Control-A (from now on, we will use C-A to mean  Control-A),  and

then the function key (a-z or A-Z). By pressing C-A first and then 'z',

a help screen comes up with a  short  summary  of  all  commands.  This

escape  key can be altered when minicom is configured (-s option or C-A

O), but we'll stick to Control-A for now.

For every menu the next keys can be used:

UP     arrow-up or 'k'

DOWN   arrow-down or 'j'

LEFT   arrow-left or 'h'

RIGHT  arrow-right or 'l'

CHOOSE Enter

CANCEL ESCape.

The screen is divided into two portions: the upper  24  lines  are  the

terminal-emulator   screen.  In	this  window,  ANSI  or	 VT100	escape

sequences are interpreted.  If there is a line left at  the  bottom,  a

status  line  is placed there.  If this is not possible the status line

will be showed every time you press C-A. On terminals that have a  spe-

cial  status  line that will be used if the termcap information is com-

plete and the -k flag has been given.

Possible commands are listed next, in alphabetical order.

C-A  Pressing C-A a second time will just send a C-A to the remote sys-

tem.   If  you  have  changed your "escape character" to something

other than C-A, this works analogously for that character.

A    Toggle 'Add Linefeed' on/off. If it is on,	a  linefeed  is	 added

before every carriage return displayed on the screen.

B    Gives  you	a  scroll  back buffer. You can scroll up with u, down

with d, a page up with b, a page down with f, and if you have them

the	 arrow	and  page  up/page down keys can also be used. You can

search for text in the buffer with s (case-sensitive) or S	(case-

insensitive).  N  will  find the next occurrence of the string.  c

will enter citation mode. A text cursor appears  and  you  specify

the	 start	line  by hitting Enter key. Then scroll back mode will

finish and the contents with prefix '>' will be sent.

C    Clears the screen.

D    Dial a number, or go to the dialing directory.

E    Toggle local echo on and off (if your version of minicom  supports

it).

F    A break signal is sent to the modem.

G    Run script (Go). Runs a login script.

H    Hangup.

I    Toggle  the	 type  of  escape  sequence  that the cursor keys send

between normal and applications mode. (See also the comment	 about

the status line below).

J    Jump to a shell. On return, the whole screen will be redrawn.

K    Clears the screen, runs kermit and redraws the screen upon return.

L    Turn Capture file on off. If turned on, all	 output	 sent  to  the

screen will be captured in the file too.

M    Sends  the	modem initialization string. If you are online and the

DCD line setting is on, you are asked for confirmation before  the

modem is initialized.

O    Configure minicom. Puts you in the configuration menu.

P    Communication  Parameters. Allows you to change the bps rate, par-

ity and number of bits.

Q    Exit minicom without resetting the modem. If  macros  changed  and

were not saved, you will have a chance to do so.

R    Receive  files.  Choose  from various protocols (external). If you

have the filename selection window and  the	 prompt	 for  download

directory  enabled, you'll get a selection window for choosing the

directory  for  downloading.  Otherwise  the  download   directory

defined in the Filenames and paths menu will be used.

S    Send  files. Choose the protocol like you do with the receive com-

mand. If you don't have the filename selection window enabled  (in

the	 File  transfer protocols menu), you'll just have to write the

filename(s) in a dialog window. If you have the  selection	window

enabled, a window will pop up showing the filenames in your upload

directory. You can tag and untag filenames by  pressing  spacebar,

and	 move  the cursor up and down with the cursor keys or j/k. The

selected filenames are  shown  highlighted.	 Directory  names  are

shown  [within brackets] and you can move up or down in the direc-

tory tree by pressing the spacebar twice. Finally, send the	 files

by pressing ENTER or quit by pressing ESC.

T    Choose  Terminal  emulation:  Ansi(color)  or vt100.  You can also

change the backspace key here, turn the status line on or off, and

define  delay  (in	milliseconds)  after  each newline if you need

that.

W    Toggle linewrap on/off.

X    Exit minicom, reset modem. If macros changed and were  not	saved,

you will have a chance to do so.

Z    Pop up the help screen.

DIALING DIRECTORY

By pressing C-A D the program puts you in the dialing directory. Select

a  command  by  pressing	 the  capitalized  letter  or  moving	cursor

right/left  with the arrow keys or the h/l keys and pressing Enter. You

can add, delete or edit entries and move them up and down in the direc-

tory  list. By choosing "dial" the phone numbers of the tagged entries,

or if nothing is tagged, the number of the highlighted  entry  will  be

dialed.	While  the  modem  is  dialing, you can press escape to cancel

dialing. Any other key will close the dial window, but won't cancel the

dialing	itself.	 Your  dialing directory will be saved into a the file

".dialdir" in your home directory.  You can scroll up and down with the

arrow  keys,  but  you  can  also scroll complete pages by pressing the

PageUp or PageDown key.	If you don't have those, use Control-B	(Back-

ward)  and Control-F (Forward). You can use the space bar to tag a num-

ber of entries and minicom will rotate trough this list if a connection

can't  be made. A '>' symbol is drawn in the directory before the names

of the tagged entries.

The "edit" menu speaks for itself, but I will discuss it briefly	 here.

A - Name	 The name for this entry

B - Number

and its telephone number.

C - Dial string #

Which	specific dial string you want to use to connect. There

are three different dial strings (prefixes and suffixes) that

can be configured in the Modem and dialing menu.

D - Local echo

can  be on or off for this system (if your version of minicom

supports it).

E - Script

The script that must be executed after a  succesfull  connec-

tion is made (see the manual for runscript)

F - Username

The  username that is passed to the runscript program.	 It is

passed in the environment string "$LOGIN".

G - Password

The password is passed as "$PASS".

H - Terminal Emulation

Use ANSI or VT100 emulation.

I - Backspace key sends

What code (Backspace or Delete) the backspace key sends.

J - Linewrap

Can be on or off.

K - Line settings

Bps rate, bits, parity and number of stop  bits  to  use  for

this  connection.   You  can choose current for the speed, so

that it will use whatever speed is being used at that	moment

(useful if you have multiple modems).

L - Conversion table

You  may  spacify  a  character conversion table to be loaded

whenever this entry answers, before running the login script.

If this field is blank, the conversion table stays unchanged.

The edit menu also shows the latest date and time when you called  this

entry  and  the total number of calls there, but doesn't let you change

them.  They are updated automatically when you connect.

The moVe command lets you move the highlighted entry up or down in  the

dialing	directory  with	 the  up/down  arrow keys or the k and j keys.

Press Enter or ESC to end moving the entry.

CONFIGURATION

By pressing C-A O you will be thrown into the setup menu. Most settings

there can be changed by everyone, but some are restricted to root only.

Those priviliged settings are marked with a star (*) here.

Filenames and paths

This menu defines your default directories.

A - Download directory

where the downloaded files go to.

B - Upload directory

where the uploaded files are read from.

C - Script directory

Where you keep your login scripts.

D - Script program

Which program to use as the script interpreter. Defaults to the

program "runscript", but if you want to use something else (eg,

/bin/sh or "expect") it is possible.  Stdin and stdout are con-

nected to the modem, stderr to the screen.

If  the path is relative (ie, does not start with a slash) then

it's relative to your home directory,  except  for  the	script

interpreter.

E - Kermit program

Where to find the executable for kermit, and it's options. Some

simple macro's can  be  used  on	 the  command  line:  '%l'  is

expanded	 to the complete filename of the dial out-device, '%f'

is expanded to the serial port  file  descriptor	 and  '%b'  is

expanded to the current serial port speed.

F - Logging options

Options to configure the logfile writing.

A - File name

Here  you can enter the name of the logfile. The file will

be written in your home directory, and the	default	 value

is	"minicom.log".	 If you blank the name, all logging is

turned off.

B - Log connects and hangups

This option defines whether or not the logfile is  written

when  the remote end answers the call or hangs up. Or when

you give the hangup	 command  yourself  or	leave  minicom

without hangup while online.

C - Log file transfers

Do you want log entries of receiving and sending files.

The  'log' command in the scripts is not affected by logging options

B and C.  It is always executed, if you just have the	 name  of  the

log file defined.

File Transfer Protocols

Protocols defined here will show up when C-A s/r is pressed.	"Name"

in the beginning of the line is the name that will show  up  in  the

menu.	 "Program"  is	the  path  to  the protocol. "Name" after that

defines if the program needs an argument, eg. a file to be transmit-

ted.	U/D  defines if this entry should show up in the upload or the

download menu.  Fullscr defines  if  the  program  should  run  full

screen,  or that minicom will only show it's stderr in a window. IO-

Red defines if minicom should attach the program's standard  in  and

output  to  the modem port or not. "Multi" tells the filename selec-

tion window whether or not the protocol can send multiple files with

one  command. It has no effect on download protocols, and it is also

ignored with upload protocols if you don't use the  filename	selec-

tion	window. The old sz and rz are not full screen, and have IO-Red

set. However, there are curses based versions of at least rz that do

not  want  their  stdin  and stdout redirected, and run full screen.

All file transfer protocols are run with the UID of  the  user,  and

not  with  UID=root.	'%l', '%f' and '%b' can be used on the command

line as with kermit.	Within this menu you can also  define  if  you

want to use the filename selection window when prompted for files to

upload, and if you like to be prompted for  the  download  directory

every time the automatic download is started. If you leave the down-

load directory prompt disabled, the download	directory  defined  in

the file and directory menu is used.

Serial port setup

*A - Serial device

/dev/tty1  or /dev/ttyS1 for most people.  /dev/cua<n> is still

possible under linux, but  not  recommended  any	 more  because

these  devices  are  obsolete  and many newly installed systems

with kernel 2.2.x or newer don't have them.   Use  /dev/ttyS<n>

instead.	 You may also have /dev/modem as a symlink to the real

device.

If you have modems connected to two or more serial  ports,  you

may  specify  all  of  them  here in a list separated by space,

comma or semicolon. When Minicom starts,	 it  checks  the  list

until  it finds an available modem and uses that one. (However,

you can't specify different init strings to them ..at least not

yet.)

To  use a UNIX socket for communication the device name must be

prefixed with "unix#" following by the full path and the	 file-

name  of	 the socket.  Minicom will then try to connect to this

socket as a client. As long as it cannot connect to the	socket

it  stays  'offline'.  As  soon	as the connection establishes,

minicom goes 'online'. If the server closes the socket, minicom

switches to 'offline' again.

*B - Lock file location

On  most	 systems This should be /usr/spool/uucp. Linux systems

use /var/lock. If this directory does not exist,	 minicom  will

not attempt to use lockfiles.

*C - Callin program

If  you	have  a	 uugetty  or something on your serial port, it

could be that you want a program to be run to switch the	 modem

cq.  port  into dialin/dialout mode. This is the program to get

into dialin mode.

*D - Callout program

And this to get into dialout mode.

E - Bps/Par/Bits

Default parameters at startup.

If one of the entries is left blank, it will not be used. So if  you

don't	 care  about  locking,	and don't have a getty running on your

modemline, entries B - D should  be  left  blank.   Be  warned!  The

callin  and  callout	programs are run with the effective user id of

"root", eg 0!

Modem and Dialing

Here, the parameters for your modem are defined. I will not  explain

this	further because the defaults are for generic Hayes modems, and

should work always. This file is not a Hayes tutorial :-)  The  only

things  worth	 noticing  are	that control characters can be sent by

prefixing them with a '^', in which '^^' means '^' itself,  and  the

'\'  character  must	also  be doubled as '\\', because backslash is

used specially in the	 macro	definitions.   Some  options  however,

don't	 have much to do with the modem but more with the behaviour of

minicom itself:

M - Dial time

The number of seconds before minicom times out if no connection

is established.

N - Delay before redial

Minicom	will  redial  if  no connection was made, but it first

waits some time.

O - Number of tries

Maximum number of times that minicom attempts to dial.

P - Drop DTR time

If you set this to 0, minicom hangs up by sending a  Hayes-type

hangup  sequence.  If  you specify a non-zero value, the hangup

will be done by dropping the DTR line. The value tells in  sec-

onds how long DTR will be kept down.

Q - Auto bps detect

If this is on, minicom tries to match the dialed party's speed.

With most modern modems this is NOT desirable, since the	 modem

buffers the data and converts the speed.

R - Modem has DCD line

If  your	 modem,	 and  your O/S both support the DCD line (that

goes 'high' when a connection is made)  minicom	will  use  it.

When you have this option on, minicom will also NOT start dial-

ing while you are already online.

S - Status line shows DTE speed / line speed

You can toggle the status line to show  either  the  DTE	 speed

(the  speed  which minicom uses to communicate with your modem)

or the line speed (the speed that your modem uses on  the  line

to  communicate	with  the  other  modem). Notice that the line

speed may change during the connection, but you will still only

see  the	 initial  speed that the modems started the connection

with. This is because the modem doesn't tell the program if the

speed is changed. Also, to see the line speed, you need to have

the modem set to show it in the connect string.	Otherwise  you

will only see 0 as the line speed.

T - Multi-line untag

You  can	 toggle	 the feature to untag entries from the dialing

directory when a connection is established to a multi-line BBS.

All the tagged entries that have the same name are untagged.

Note  that	a  special exception is made for this menu: every user

can change all parameters here, but	 some  of  them	 will  not  be

saved.

Screen and keyboard

A - Command key is

the 'Hot Key' that brings you into command mode. If this is set

to 'ALT' or 'meta key', you can directly call commands by  alt-

key instead of HotKey-key.

B - Backspace key sends

There  still  are  some	systems	 that want a VT100 to send DEL

instead of BS. With this option you can enable that  stupidity.

(Eh, it's even on by default...)

C - Status line is

Enabled or disabled. Some slow terminals (for example, X-termi-

nals) cause  the	 status	 line  to  jump	 "up  and  down"  when

scrolling,  so you can turn it off if desired. It will still be

shown in command-mode.

D - Alarm sound

If turned on, minicom will sound an alarm (on the console only)

after  a	 succesfull connection and when up/downloading is com-

plete.

E - Foreground Color (menu)

indicates the foreground color to use for all the configuration

windows in minicom.

F - Background Color (menu)

indicates the background color to use for all the configuration

windows in minicom. Note that minicom will not allow you to set

forground and background colors to the same value.

G - Foreground Color (term)

indicates the foreground color to use in the terminal window.

H - Background Color (term)

indicates  the  background color to use in the terminal window.

Note that minicom will not  allow  you  to  set	forground  and

background colors to the same value.

I - Foreground Color (stat)

indicates the foreground color to use in for the status bar.

J - Background Color (stat)

indicates  the  color  to  use in for the status bar. Note that

minicom will allow you to set the status	 bar's	forground  and

background colors to the same value. This will effectively make

the status bar invisible but  if	 these	are  your  intensions,

please see the option

K - History buffer size

The  number  of	lines  to  keep	 in  the  history  buffer (for

backscrolling).

L - Macros file

is the full path to the file that holds	macros.	 Macros	 allow

you to define a string to be sent when you press a certain key.

In minicom, you may define F1 through F10 to  send  up  to  256

characters  [this  is  set  at  compile time]. The filename you

specify is verified as soon as you hit ENTER.  If  you  do  not

have permissions to create the specified file, an error message

will so indicate and you will be forced to  re-edit  the	 file-

name.  If  you are permitted to create the file, minicom checks

to see if it already exists. If so, it  assumes	it's  a	 macro

file  and reads it in. If it isn't, well, it's your problem :-)

If the file does not exist, the filename is accepted.

M - Edit Macros

opens up a new window which allows you to edit the  F1  through

F10 macros.

N - Macros enabled

-  Yes or No. If macros are disabled, the F1-F10 keys will just

send the VT100/VT220 function key escape sequences.

O - Character conversion

The active conversion table filename is shown here. If you  can

see  no name, no conversion is active. Pressing O, you will see

the conversion table edit menu.

Edit Macros

Here, the macros for F1 through F10 are defined.  The	bottom

of  the  window shows a legend of character combinations that

have special meaning.	They allow you to enter	 special  con-

trol characters with plain text by prefixing them with a '^',

in which '^^' means '^' itself. You can send a 1 second delay

with  the  '^~'  code.	 This is useful when you are trying to

login after ftp'ing or telnet'ing somewhere.	You  can  also

include  your	current	 username  and password from the phone

directory in the macros with '\u' and '\p', respectively.  If

you  need the backslash character in the macro, write it dou-

bled as '\\'.	To edit a macro, press the number  (or	letter

for  F10) and you will be moved to the end of the macro. When

editing the line, you may use the left & right arrows, Home &

End  keys,  Delete & BackSpace, and ESC and RETURN.  ESC can-

cels any changes made while ENTER accepts the changes.

Character conversion

Here you can edit the character conversion table. If you  are

not  an  American,  you know that in many languages there are

characters that are not included in the ASCII character  set,

and  in the old times they may have replaced some less impor-

tant characters in ASCII and now they are  often  represented

with character codes above 127. AND there are various differ-

ent ways to represent them. This is where you may  edit  con-

version tables for systems that use a character set different

from the one on your computer.

A - Load table

You probably guessed it. This command loads a  table  from

the	 disk.	You are asked a file name for the table.  Pre-

defined  tables  .mciso,  .mcpc8  and  .mcsf7  should   be

included  with  the	 program. Table .mciso does no conver-

sion, .mcpc8 is to be used for  connections	 with  systems

that  use  the  8-bit  pc character set, and .mcsf7 is for

compatibility with the systems  that  uses	the  good  old

7-bit  coding  to  replace	the characters {|}[]\ with the

diacritical characters used in Finnish and Swedish.

B - Save table

This one saves the active table on the filename you	 spec-

ify.

C - edit char

This  is  where you can make your own modifications to the

existing table.  First you are asked the  character	 value

(in	 decimal)  whose  conversion  you want to change. Next

you'll say which character you want to see on your	screen

when that character comes from the outside world. And then

you'll be asked what you want to  be  sent	out  when  you

enter that character from your keyboard.

D - next screen

E - prev screen

Yeah, you probably noticed that this screen shows you what

kind of conversions are active. The screen just  is	 (usu-

ally)  too	small  to  show	 the whole table at once in an

easy-to-understand format. This is how you can scroll  the

table left and right.

F - convert capture

Toggles  whether  or not the character conversion table is

used when writing the capture file.

Save setup as dfl

Save the parameters as the default for the next time the program  is

started.  Instead  of	 dfl,  any  other  parameter  name may appear,

depending on which one was used when the program was started.

Save setup as..

Save the parameters  under  a	 special  name.	 Whenever  Minicom  is

started with this name as an argument, it will use these parameters.

This option is of course priviliged to root.

Exit

Escape from this menu without saving.	 This can also	be  done  with

ESC.

Exit from minicom

Only	root  will see this menu entry, if he/she started minicom with

the '-s' option. This way, it is possible to change  the  configura-

tion without actually running minicom.

STATUS LINE

The status line has several indicators, that speak for themselves.  The

mysterious APP or NOR indicator probably needs explanation.  The	 VT100

cursor  keys  can  be  in two modes: applications mode and cursor mode.

This is controlled by an escape sequence. If you find that  the	cursor

keys  do	 not work in, say, vi when you're logged in using minicom then

you can see with this indicator whether the cursor keys are in applica-

tions or cursor mode. You can toggle the two with the C-A I key. If the

cursor keys then work, it's probably an error in	 the  remote  system's

termcap initialization strings (is).

LOCALES

Minicom	has now support for local languages. This means you can change

most of the English messages and other strings to another  language  by

setting	the environment variable LANG. On September 2001 the supported

languages are Brazilian Portuguese, Finnish, Japanese, French,  Polish,

Czech, Russian and Spanish.  Turkish is under construction.

SECURITY ISSUES

Since  Minicom  is run setuid root on some computers, you probably want

to restrict access to it. This is possible  by  using  a	 configuration

file  in	 the  same  directory  as  the	default	 files,	 called "mini-

com.users".  The syntax of this file is as following:

<username> <configuration> [configuration...]

To allow user 'miquels' to use the  default  configuration,  enter  the

following line into "minicom.users":

miquels dfl

If  you	want  users to be able to use more than the default configura-

tions, just add the names of those configurations behind the user name.

If  no configuration is given behind the username, minicom assumes that

the user has access to all configurations.

MISC

If minicom is hung, kill it with SIGTERM . (This	 means	kill  -15,  or

since  sigterm  is  default,  just plain "kill <minicompid>". This will

cause a graceful exit of minicom, doing resets and everything.  You may

kill  minicom  from  a  script  with the command "! killall -9 minicom"

without hanging up the line. Without the -9  parameter,	minicom	 first

hangs up before exiting.

Since  a	 lot of escape sequences begin with ESC (Arrow up is ESC [ A),

Minicom does not know if the escape character it gets is	 you  pressing

the escape key, or part of a sequence.

An  old version of Minicom, V1.2, solved this in a rather crude way: to

get the escape key, you had to press it twice.

As of release 1.3 this has bettered a little: now a 1-second timeout is

builtin, like in vi. For systems that have the select() system call the

timeout is 0.5 seconds. And... surprise: a special Linux-dependant hack

:-)  was	 added.	 Now,  minicom can separate the escape key and escape-

sequences. To see how dirty this was done, look into wkeys.c.   But  it

works like a charm!

FILES

Minicom	keeps  it's  configuration  files  in  one  directory, usually

/var/lib/minicom, /usr/local/etc or /etc.  To  find  out	 what  default

directory  minicom  has	compiled  in,  issue  the  command minicom -h.

You'll probably also find the demo  files  for  runscript(1),  and  the

examples	 of  character conversion tables either there or in the subdi-

rectories of /usr/doc/minicom*. The conversion tables are  named	 some-

thing  like  mc.*  in that directory, but you probably want to copy the

ones you need in your home directory as something beginning with a dot.

minicom.users

minirc.*

$HOME/.minirc.*

$HOME/.dialdir

$HOME/minicom.log

/usr/share/locale/*/LC_MESSAGES/minicom.mo

VERSION

Minicom is now up to version 2.1.

AUTHORS

The   original	author	 of   minicom	is   Miquel   van  Smoorenburg

(miquels@cistron.nl).  He wrote versions up to 1.75.

Jukka  Lahtinen	(walker@netsonic.fi,  jukkal@despammed.com)  has  been

responsible  for	 new versions since 1.78, helped by some other people,

including:

filipg@paranoia.com wrote the History buffer searching to 1.79.

Arnaldo Carvalho de Melo (acme@conectiva.com.br) did the international-

ization and the Brasilian Portuguese translations.

Jim Seymour (jseymour@jimsun.LinxNet.com) wrote the multiple modem sup-

port and the filename selection window used since 1.80.

Tomohiro Kubota (kubota@debian.or.jp) wrote the	Japanese  translations

and the citation facility, and did some fixes.

Gael Queri (gqueri@mail.dotcom.fr) wrote the French translations.

Arkadiusz Miskiewicz (misiek@pld.org.pl) wrote the Polish translations.

Kim Soyoung (nexti@chollian.net) wrote the Korean translations.

Jork Loeser (jork.loeser@inf.tu-dresden.de) provided the socket	exten-

sion.

Most  of	 this  man page is copied, with corrections, from the original

minicom README, but some pieces and the corrections are by  Michael  K.

Johnson.

Jukka  Lahtinen	(walker@netsonic.fi) has added some information of the

changes made after version 1.75.

User's Manual		 $Date: 2003/04/26 07:31:06 $		    MINICOM(1)