| Параметры запроса |  |
| --- | --- |
| [редактировать критичность] |  |

| Номер | 264490 |
| --- | --- |
| Название | №264490 |
| Приоритет | Средний |
| Класс обслуживания | 9x5 |
| Дата и время обращения | 03.02.2014 13:54 |
| Дата регистрации | 03.02.2014 13:54 |
| Тип запроса | Изменение данных пользователя |
| Категория |  |
| Критичность | 1 - 1-10 клиентов |
| Срочность | Низкая |
| Описание запроса Сергей, надо подготовить сервер. Железка – https://cmdb.tcsbank.ru/index.php?do... https://cmdb.tcsbank.ru/new/index.ph... сейчас там винда, можно все сносить. Ставим rhel 6.4 , x64, стандартная установка, puppet, репозитарии, мониторинг, бекапы, драйвера, прошивки и прочее. Будет raid10 из 4 дисков. Разбивка будет / (root) – 16Gb /tmp – 8Gb /var – 16Gb /opt – 520Gb Ip думаю можно оставить – 10.216.43.43. Интерфейсы настроены на trunk vlan, но лучше проверить и сделать это, если не так. // отправил в servicedesk // From: Makhnev Nikita Sent: Friday, January 31, 2014 5:11 PM To: Meagcov Vitalie; Terentev Aleksey Subject: RE: напоминаю про сервак Пилоты в тест пихаем - что бы вообще ничего не прописывать. Проды - вопрос интересный, но чем меньше файерволлов будет на пути - тем лучше -----Original Message----- From: Meagcov Vitalie Sent: Friday, January 31, 2014 05:06 PM Russian Standard Time To: Makhnev Nikita; Terentev Aleksey Subject: RE: напоминаю про сервак Это я говорил про сеть. Назначение серверов общее, надо в одну сеть их помещать. Проще потом доступы прописывать. From: Makhnev Nikita Sent: Friday, January 31, 2014 4:18 PM To: Meagcov Vitalie; Terentev Aleksey Subject: RE: напоминаю про сервак Это понятно - просто сервер под compuware -----Original Message----- From: Meagcov Vitalie Sent: Friday, January 31, 2014 04:02 PM Russian Standard Time To: Makhnev Nikita; Terentev Aleksey Subject: RE: напоминаю про сервак Вообще-то, Нагиос одна из систем мониторинга. From: Makhnev Nikita Sent: Friday, January 31, 2014 1:46 PM To: Terentev Aleksey; Meagcov Vitalie Subject: RE: напоминаю про сервак А при чем тут нагиос? -----Original Message----- From: Terentev Aleksey Sent: Friday, January 31, 2014 12:01 PM Russian Standard Time To: Meagcov Vitalie; Makhnev Nikita Subject: RE: напоминаю про сервак Сейчас в 172.16.1.* , общей серверной сети, в Диапазоне . Куда хотели мигрировать – также общая серверная сеть в ДатаСпейс (AD). Если надо заводить отдельную, надо обоснование-описание и к сетевикам. From: Meagcov Vitalie Sent: Friday, January 31, 2014 11:35 AM To: Terentev Aleksey; Makhnev Nikita Subject: RE: напоминаю про сервак По разбивке нет. Есть пожелания по сети. Как мне видится, надо создавать отдельную IP-сеть для мониторинга. Это упростит открытие доступов при установке новых серверов. У вас Нагиос где живет? From: Terentev Aleksey Sent: Friday, January 31, 2014 11:28 AM To: Makhnev Nikita Cc: Meagcov Vitalie Subject: RE: напоминаю про сервак ProLiant DL380p Gen8 , 288ГБ, 2x10Gb cna, dataspace - один из запланированных под мониторинг. Ставим RHEL6.4. Пожелания по разбивке есть ? Всего 4 диска по 300Гб (EG0300FBDBR). From: Terentev Aleksey Sent: Monday, January 27, 2014 6:11 PM To: Makhnev Nikita Cc: Meagcov Vitalie Subject: RE: напоминаю про сервак Ты хоть напиши, зачем и как будет называться.. Есть пожелания по сетевым соединениям ? Виталий, не против ? From: Makhnev Nikita Sent: Monday, January 27, 2014 11:05 AM To: Terentev Aleksey Subject: напоминаю про сервак 380й, на месяц. -------- Regards, Nikita Makhnev {#} Tinkoff. Credit Systems 39A Leningradskoe shosse, Moscow 125212, Russia BC Olympia Park Desk: +7495 648.1000 #2069 Mobile: +7925 000.4665 Skype: nikita.makhnev Mail: n.makhnev@tcsbank.ru<mailto:n.makhnev@tcsbank.ru> This e-mail (including attachments) is confidential and is intended for the addressee(s) only. Unless you are the addressee, you may not read, copy, use or store this e-mail in any way, and delete immediately. If you have received it in error, please contact ‘Tinkoff Credit Systems’ (Bank ‘TCS’ ZAO) on +7 495 648 1000.. |  |
| Текущее состояние | Принят на обработку |
| Автор | system |
| Ответственный за запрос | Белогруд Сергей / Отдел технической поддержки и информационных систем (группа) |
| Кем решен |  |
| Кем закрыт |  |
| Массовая проблема | нет |
| Количество задач (активных/всего) | 0 / 0 |
| Количество файлов | 1 |

From: Meagcov Vitalie

Sent: Monday, February 03, 2014 2:24 PM

To: Terentev Aleksey; Belogrud Sergey Vladimirovich

Cc: Makhnev Nikita

Subject: RE: напоминаю про сервак

Это тестовая сеть?

From: Terentev Aleksey

Sent: Monday, February 03, 2014 1:53 PM

To: Belogrud Sergey Vladimirovich

Cc: Makhnev Nikita; Meagcov Vitalie

Subject: RE: напоминаю про сервак

Сергей, надо подготовить сервер.

Железка –

https://cmdb.tcsbank.ru/index.php?do=view&sn=CZ224809Z9&type=x86

https://cmdb.tcsbank.ru/new/index.php?part=devices&action=edit&id=38

сейчас там винда, можно все сносить.

Ставим rhel 6.4 , x64, стандартная установка, puppet, репозитарии, мониторинг, бекапы, драйвера, прошивки и прочее.

Будет raid10 из 4 дисков.

Разбивка будет

/ (root) – 16Gb

/tmp – 8Gb

/var – 16Gb

/opt – 520Gb

Ip думаю можно оставить – 10.216.43.43.

Интерфейсы настроены на trunk vlan, но лучше проверить и сделать это, если не так.

// отправил в servicedesk //

compuware

vlan 1643

iLO

https://10.219.10.117/

Firmware Version 1.30

CZ224809Z9.mgmt.tcsbank.ru

#### Information

| Server Name | WIN-3UP1EB74FOJ |
| --- | --- |
| Product Name | ProLiant DL380p Gen8 |
| UUID | 32323636-3735-5A43-3232-343830395A39 |
| UUID (Logical) |  |
| Server Serial Number | CZ224809Z9 |
| Product ID | 662257-421 |
| Chassis Serial Number |  |
| Serial Number (Logical) |  |
| System ROM | P70 12/14/2012 |
| Backup System ROM | 08/20/2012 |
| Integrated Remote Console | .NET Java |
| License Type | iLO 4 Advanced |
| iLO Firmware Version | 1.30 Jul 18 2013 |
| IP Address | 10.219.10.117 |
| Link-Local IPv6 Address | fe80::a2b3:ccff:feea:33d6/64 |
| iLO Hostname | CZ224809Z9.mgmt.tcsbank.ru |

https://h20565.www2.hp.com/portal/site/hpsc/public/psi/home/?sp4ts.oid=5177957

Understand what is server name.

In our case that is:

ds-mon11t

Fill out quiz list.

From: Terentev Aleksey

Sent: Monday, October 07, 2013 7:43 PM

To: net-admins; Sec_infobez; admins

Cc: Shcherbakov Oleg; Smolenko Sergey

Subject: опросный лист, выдача ресурса (сервер)

Коллеги, для выстраивания процесса выдачи и учета ресурсов, мы уже предлагаем инициаторам, которые просят сервера, заполнять следующую информацию.

Думаю ближайшее время вывесить на wiki и посылать туда людей.

Если есть предложения, дополнения – присылайте.

| требования к дополнительной информации для выделения серверных мощностей (сервер linux/windows) |  |
| --- | --- |
| - название проекта | название, ключевые слова, связки - короткое описание, для ссылок и именований |
| - краткое описание | достигаемая техническая цель, связь с бизнес-целью |
| - ответственный сотрудник | сотрудник, который курирует финальные сроки проекта, определяет факт сдачи проекта, либо задает цели и сроки отдельно взятой части проекта |
| - схема потоков данных приложения | какие входные-выходные данные, из каких приложений, систем |
| - схема сетевого взаимодействия | кто, из каких сетей будет обращаться к серверу, направления, протоколы, порты |
| - количественные характеристики | цпу, память, диски - количество, параметры производительности. Желательно в виде требований к ПО и обоснованные. Либо с указанием - "без особых требований", будет развернуто из шаблонов по умолчанию |
| - ответственный пользователь сервера | сотрудник, который отвечает за функционал сервера, за получение оповещений от системы мониторинга, за согласование плановых работ и downtime сервера в случае необходимости |
| - параметры доступности, мониторинга и оповещений | aka SLA - 8x5, 24x7 . То же и про оповещения. Список метрик мониторинга, кроме "по-умолчанию" |
| - решение для DR | решение по "восстановлению после катастрофы" ,с указанием допустимой недоступности сервиса. Например - восстановление из бекапов, кластерное решение из более чем 1-го узла - actice-active, active-backup, балансировщики и т.п. |
| - решение для резервного копирования, восстановления | в случае необратимых логических ошибок в ПО, либо по требованию восстановления на указанную дату , с указанием глубины и времени восстановления. |
| - согласование с сетевым отделом, выделение сетей | создание новых сетей, или размещение в существующих |
| - согласование с руководителем | согласование с непосредственным руководителем |
| - согласование с ИБ | pci dss, доступы по сетевом взаимодействию |

Алексей Терентьев

Руководитель отдела технической поддержки информационных систем.

=Тинькофф Кредитные Системы= Банк (закрытое акционерное общество)

*. Collect information about network (vlan and so on). I have a plan to use it when I will use kickstart.

If it didn’t received before.

*.  See raid array configuration.

Configure it.

*. Check fresh SPP on HP web-site. And roll up it.

Start install firmware manually.

Check HP Insight Diagnostics first.

If possible to save information about system - do it.

Then run firmware update.

*. Install OS.

Reboot server and stop booting here. Meanwhile prepare kickstart file anaconda.

Don’t forget option ssh=1 - it’s allow access to server while kickstart is working.

[support@centos00 kickstart]$ rm floppy.ds-mon11t.img ; mkfs.vfat -C "floppy.ds-mon11t.img" 1440 ; sudo mount -o loop -t vfat /home/support/mnt/share/greenplum/kickstart/floppy.ds-mon11t.img /mnt/floppy ; sudo cp /home/support/mnt/share/greenplum/kickstart/20140203-1659.kickstart.ds-mon11t.cfg /mnt/floppy/ks.cfg ; sudo umount /mnt/floppy

rm: cannot remove `floppy.ds-mon11t.img': No such file or directory

mkfs.vfat 3.0.9 (31 Jan 2010)

[support@centos00 kickstart]$

Ctrl+Alt+F1

Ctrl+Alt+F2

Ctrl+Alt+F3

Ctrl+Alt+F4

Ctrl+Alt+F5

Ctrl+Alt+F6

Ctrl+Alt+F7,F8,F9,F10,F11,F12

How to configure server to connect it via ssh. For automatic ssh configuration it should be use ssh=1 option into kickstart or kernel command line - I’m not sure.

http://www.geekride.com/how-to-generate-ssh-host-keys/

Connecting to server when kickstart is running. It was when kickstart freezed.

[support@centos00 ~]$ ssh root@10.216.43.43

The authenticity of host '10.216.43.43 (10.216.43.43)' can't be established.

RSA key fingerprint is 77:c4:3c:cd:94:a5:7e:3a:32:8d:e7:d2:a3:e6:c0:44.

Are you sure you want to continue connecting (yes/no)? yes

Warning: Permanently added '10.216.43.43' (RSA) to the list of known hosts.

-bash-4.1#

-bash-4.1# ls -Alh /installation_logs.tar

-rw-r--r--. 1 root root 132M Jun 21 00:13 /installation_logs.tar

-bash-4.1#

-bash-4.1# exit

logout

Connection to 10.216.43.43 closed.

[support@centos00 ~]$

[support@centos00 20140203-1751]$ pwd

/home/support/mnt/share/temp/20140203-1751

[support@centos00 20140203-1751]$

[support@centos00 20140203-1751]$ scp root@10.216.43.43:/installation_logs.tar .

installation_logs.tar                                                                                                                  100%  132MB  12.0MB/s   00:11

[support@centos00 20140203-1751]$

[support@centos00 20140203-1751]$ tar xvf installation_logs.tar

# dmesg > /tmp/dmesg.log

# fdisk -l > /tmp/fdisk.log

# parted -sl > /tmp/parted.log

# cat /proc/partitions > /tmp/proc_part.log

# cat /proc/scsi/scsi > /tmp/proc_scsi.log

# tar cvf installation_logs.tar /tmp/*

[anaconda root@localhost /]# tar czvf installation_logs.tgz /tmp/*

[support@centos00 20140203-1751]$ scp root@10.216.43.43:/installation_logs.tgz .

installation_logs.tgz                                                                                                                  100%  118MB  11.8MB/s   00:10

[support@centos00 20140203-1751]$

[support@centos00 20140203-1751]$ ls -Alh

total 250M

-rwxrwxrwx 1 root root 132M Feb  3 17:52 installation_logs.tar..old

-rwxrwxrwx 1 root root 119M Feb  3 18:10 installation_logs.tgz

drwxrwxrwx 1 root root 4.0K Feb  3 17:53 tmp.old

[support@centos00 20140203-1751]$

[support@centos00 20140203-1751]$ tar xzvf installation_logs.tgz

[support@centos00 kickstart]$ vim 20140203-1659.kickstart.ds-mon11t.cfg

...

# Next interesting disk option.

#ignoredisk --drives=sdb

ignoredisk --drives=sda

[support@centos00 kickstart]$ rm floppy.ds-mon11t.img ; mkfs.vfat -C "floppy.ds-mon11t.img" 1440 ; sudo mount -o loop -t vfat /home/support/mnt/share/greenplum/kickstart/floppy.ds-mon11t.img /mnt/floppy ; sudo cp /home/support/mnt/share/greenplum/kickstart/20140203-1659.kickstart.ds-mon11t.cfg /mnt/floppy/ks.cfg ; sudo umount /mnt/floppy

mkfs.vfat 3.0.9 (31 Jan 2010)

[support@centos00 kickstart]$

After server reboot. We check how it access thrue network.

##### Here we got a lot of waiting time. About one minutes.

Working well kickstart file!

[support@centos00 kickstart]$ cat 20140203-1659.kickstart.ds-mon11t.cfg

# Kickstart file automatically generated by anaconda.

# Install OS instead of upgrade

#

#version=DEVEL

install

# Use graphical install

#

# This a default option. We can omit this and result will be same.

graphical

#

# Perform the installation in a completely non-interactive command line mode.

#cmdline

#

# Perform an interactive installation, but use the information in the kickstart file to provide defaults.

#interactive

# Halt the system after the installation has successfully completed. Anaconda displays a message and waits for the user to press akey before rebooting.

# This option is used as the default.

#halt

# Similar to interactive exept it goes to the next screen for you. It is used mostly for debugging and should not be used when deploying asystem because it may disrupt package installation.

#autostep

# Take a screenshot at every step during installation and copy the images over to /root/anaconda-screenshots after installation is complete.

#--autoscreenshot

# This optoin work only if XWindow system was installed. It means that after first boot OS asks for user several first answers.

#firstboot --disable

# Use CDROM installation media

#

# Install from the first optical drive on the system.

cdrom

# This strings was gotten from GreenPlum servers. And it bring to me a lot of troubles, because if this string uncommented, during installation repositories will not be found!

# repo --name="Red Hat Enterprise Linux"  --baseurl=cdrom:sr0 --cost=100

# This will force anaconda to run mediacheck on the installation media. This command requires that installs be attnded, so it is disabled by default.                    #mediacheck

# System language

#

lang en_US.UTF-8

# System keyboard

#

keyboard us

# Network information

#

# This line took from vmware virtual server installation. It has one network interface. It was a template for virtual servers.

#network --onboot yes --device eth0 --bootproto static --ip 10.217.18.218 --netmask 255.255.255.0 --gateway 10.217.18.254 --noipv6 --nameserver 10.216.42.5,10.216.40.4 --hostname rhel64-template

network --onboot yes --device eth0 --bootproto static --ip 10.216.43.43 --netmask 255.255.255.0 --gateway 10.216.43.254 --noipv6 --nameserver 10.216.42.5,10.216.40.4 --hostname ds-mon11t --vlanid 1643 --activate

#

# This string is ideally for virtual server insallation under VirtualBox. I mean this guest host network works behind NAT. I use it for personal purposes.

#network --bootproto=dhcp --device=eth0 --onboot=on

# Activate this device in the installer environment.

#--activate

#--vlanid= — Specifies virtual LAN ID number (802.1q tag).

#--bondslaves= — Specifies which network interfaces will be bonded.

#--bondopts= — a comma-separated list of optional parameters for your bonded interface. For example: network --bondopts=mode=active-backup,primary=eth1

#

# This section have gotten from GreenPlum installed server. These are unconfigurred network interfaces. I don't know how better it should be used. It's for history.

#network --onboot no --device eth0 --noipv4 --noipv6

# Root password

#

# Here is predefined password for root user. Its name is "defoltoviy".

rootpw  --iscrypted $6$33j.jZoE6xBQz08r$qL9BWtoZVoSuKJB3tY13AJLiAF8HHBBpDBFEzkqpCPOYX.rpdh4lUD0z8.wfw9hjdAU3mPD1aeklmbut1w4M2.

# Firewall configuration

#

# Here is rule of firewall iptables configuration that allow ssh connection and privent other connection. It was added than I have installed VirtualBox virtual machine.

# firewall --service=ssh

#

# Here is more prefferable string for fresh and clean server installed. It disables firewall iptables at all.

firewall --disabled

# System authorization information

#

authconfig --enableshadow --passalgo=sha512

# SELinux configuration

#

#selinux --enforcing

selinux --disabled

# System timezone

#

# Better use manual defined type of hardware BIOS clock on physical server - --utc. If server is a virtual server we may omit --utc parameter.

#timezone Europe/Moscow

timezone --utc Europe/Moscow

# System bootloader configuration

#

# For example it possible to use several of drives with predefined order. This string is an example of it. It was taken from GreenPlum servers.

#bootloader --location=mbr --driveorder=sda,sdb --append="crashkernel=auto rhgb quiet"

#bootloader --location=mbr --driveorder=sda --append="crashkernel=auto rhgb quiet"

#

# Here I disabled graphical bar in starting process. I think it useful to view booting process.

bootloader --location=mbr --driveorder=sda --append="crashkernel=auto quiet"

# Installation logging level

#

# This is not mandatory option. It's just for history.

# Specify the minimum level of messages that appear on tty3. All messages will still be sent to the log file.

logging --level=info

# Partition clearing information

#

# The following is the partition information you requested

# Note that any partitions you deleted are not expressed

# here so unless you clear all partitions first, this is

# not guaranteed to work

#clearpart --linux --drives=sda

clearpart --all

# To initialize disk with invalid partition tables and clear theri contents.

#zerombr

# Disk partitioning information

#

# Automatically creates partitions - root partition (1GB or bigger), swap, boot.

#autopart

#

# This size is in Megabytes.

#part / --fstype=ext4 --asprimary --size=5120

# We know - all disks size is about 558.73GB. I have seen in in information from raid controller.

# / root partition size 16GB. (expr 16 \* 1024)

part / --fstype=ext4 --asprimary --size=16384

# /tmp partition size 8GB. (expr 8 \* 1024)

part /tmp --fstype=ext4 --asprimary --size=8192

# /var partition size 16GB. (expr 16 \* 1024)

part /var --fstype=ext4 --asprimary --size=16384

# /opt partition size about 520GB. It takes all remaining disk space - option --grow do that.

part /opt --fstype=ext4 --asprimary --size=1 --grow

#

#part /var --fstype=ext4 --asprimary --size=40960

#part /opt --fstype=ext4 --grow --asprimary --size=200

#

# This strings have taken from GreenPlum server installation anaconda-ks.cfg file.

#

#part /boot --asprimary --fstype="ext4" --ondisk=/dev/sda --size=200

#part / --asprimary --fstype="ext4" --ondisk=/dev/sda --size=47373

#

# These strings was taken from GreenPlum servers. These strings doesn't work, it should be modified before installation.

#

# part /boot --fstype=ext4 --grow --maxsize=1008 --size=1

# part / --fstype=ext4 --size=47373

# part /data1 --fstype=xfs --size=3558262

# part /work1 --fstype=xfs --size=311848

#

# part /data2 --fstype=xfs --size=3558262

# part /work2 --fstype=xfs --size=311848

# part swap --grow --size=200

#

# Next interesting disk option.

#ignoredisk --drives=sdb

ignoredisk --drives=sda

# Specifies a list of disk for the installer to use. All other disks are ignored.

#--only-use=sda

%packages

@base

@client-mgmt-tools

@console-internet

@core

@debugging

@directory-client

@hardware-monitoring

@java-platform

@large-systems

@network-file-system-client

@performance

@perl-runtime

@server-platform

@server-policy

pax

python-dmidecode

oddjob

sgpio

device-mapper-persistent-data

samba-winbind

certmonger

pam_krb5

krb5-workstation

perl-DBD-SQLite

%end

[support@centos00 kickstart]$

[support@centos00 20140203-1751]$ ssh root@10.216.43.43

@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@

@    WARNING: REMOTE HOST IDENTIFICATION HAS CHANGED!     @

@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@

IT IS POSSIBLE THAT SOMEONE IS DOING SOMETHING NASTY!

Someone could be eavesdropping on you right now (man-in-the-middle attack)!

It is also possible that the RSA host key has just been changed.

The fingerprint for the RSA key sent by the remote host is

9d:53:b0:18:2a:c5:84:6a:25:58:89:0a:2d:45:2b:c9.

Please contact your system administrator.

Add correct host key in /home/support/.ssh/known_hosts to get rid of this message.

Offending key in /home/support/.ssh/known_hosts:246

RSA host key for 10.216.43.43 has changed and you have requested strict checking.

Host key verification failed.

[support@centos00 20140203-1751]$ sed -i '246d' /home/support/.ssh/known_hosts

[support@centos00 20140203-1751]$

[support@centos00 20140203-1751]$ ssh root@10.216.43.43

^C

[support@centos00 20140203-1751]$

[support@centos00 20140203-1751]$ ssh root@10.216.43.43

The authenticity of host '10.216.43.43 (10.216.43.43)' can't be established.

RSA key fingerprint is 9d:53:b0:18:2a:c5:84:6a:25:58:89:0a:2d:45:2b:c9.

Are you sure you want to continue connecting (yes/no)? yes

Warning: Permanently added '10.216.43.43' (RSA) to the list of known hosts.

root@10.216.43.43's password:

Last login: Fri Jun 21 01:29:20 2013

[root@ds-mon11t ~]#

Second question.

Why after server was rebooted, I have both of interfaces configured same as each other?

Of course network was not work correctly. Wherefore I should connect to console via iLO and edit configuration file /etc/sysconfig/network-scripts/ifcfg-eth0 and then do command service network restart.

[root@ds-mon11t ~]# cat /etc/sysconfig/network-scripts/ifcfg-eth0

DEVICE="eth0"

#BOOTPROTO="static"

#DNS1="10.216.42.5"

#DNS2="10.216.40.4"

#GATEWAY="10.216.43.254"

HWADDR="2C:27:D7:53:B7:A0"

#IPADDR="10.216.43.43"

IPV6INIT="no"

#NETMASK="255.255.255.0"

NM_CONTROLLED="yes"

ONBOOT="yes"

TYPE="Ethernet"

UUID="7461ba57-80f6-49cc-9256-4849967aaec8"

[root@ds-mon11t ~]#

[root@ds-mon11t ~]# cat /etc/sysconfig/network-scripts/ifcfg-eth0.1643

DEVICE="eth0.1643"

BOOTPROTO="static"

DNS1="10.216.42.5"

DNS2="10.216.40.4"

GATEWAY="10.216.43.254"

HOSTNAME="ds-mon11t"

IPADDR="10.216.43.43"

MTU="1500"

NETMASK="255.255.255.0"

NM_CONTROLLED="yes"

ONBOOT="yes"

TYPE="Vlan"

UUID="066cbd9e-d1d8-4009-97ec-f24ffa02b949"

VLAN="yes"

[root@ds-mon11t ~]#

[root@ds-mon11t ~]# lsblk

NAME   MAJ:MIN RM   SIZE RO TYPE MOUNTPOINT

sr0     11:0    1  1024M  0 rom

sda      8:0    0 558.7G  0 disk

├─sda1   8:1    0    16G  0 part /

├─sda2   8:2    0    16G  0 part /var

├─sda3   8:3    0     8G  0 part /tmp

└─sda4   8:4    0 518.7G  0 part /opt

[root@ds-mon11t ~]#

[root@ds-mon11t ~]# findmnt

TARGET                       SOURCE         FSTYPE      OPTIONS

/                            /dev/sda1      ext4        rw,relatime,barrier=1,stripe=128,data=ordered

├─/proc                      proc           proc        rw,relatime

│ ├─/proc/bus/usb            /proc/bus/usb  usbfs       rw,relatime

│ └─/proc/sys/fs/binfmt_misc                binfmt_misc rw,relatime

├─/sys                       sysfs          sysfs       rw,relatime

├─/dev                       devtmpfs       devtmpfs    rw,relatime,size=148783812k,nr_inodes=37195953,mode=755

│ ├─/dev/pts                 devpts         devpts      rw,relatime,gid=5,mode=620,ptmxmode=000

│ └─/dev/shm                 tmpfs          tmpfs       rw,relatime

├─/opt                       /dev/sda4      ext4        rw,relatime,barrier=1,stripe=128,data=ordered

├─/tmp                       /dev/sda3      ext4        rw,relatime,barrier=1,stripe=128,data=ordered

├─/var                       /dev/sda2      ext4        rw,relatime,barrier=1,stripe=128,data=ordered

│ └─/var/lib/nfs/rpc_pipefs  sunrpc         rpc_pipefs  rw,relatime

├─/misc                      /etc/auto.misc autofs      rw,relatime,fd=7,pgrp=6517,timeout=300,minproto=5,maxproto=5,indirect

└─/net                       -hosts         autofs      rw,relatime,fd=13,pgrp=6517,timeout=300,minproto=5,maxproto=5,indirect

[root@ds-mon11t ~]#

[root@ds-mon11t ~]# df -h

Filesystem            Size  Used Avail Use% Mounted on

/dev/sda1              16G  1.8G   14G  12% /

tmpfs                 142G     0  142G   0% /dev/shm

/dev/sda4             511G  198M  485G   1% /opt

/dev/sda3             7.9G  146M  7.4G   2% /tmp

/dev/sda2              16G  214M   15G   2% /var

[root@ds-mon11t ~]#

*. Send ticket to Network administrator for configure access to new server.

Прошу проверить и предоставить в случае отсутствия доступ с сервера vm-bigdata-bild01 (10.216.22.23) до следующих серверов по следующим портам.

| источник | назначения | порт | тип протокола | описание |
| --- | --- | --- | --- | --- |
| vm-bigdata-bild01 (10.216.22.23) | nag2 (172.16.1.11) | 5666 | tcp | Связь с сервером мониторинга по протоколу nrpe |
|  | monitoring2 (192.168.1.111) |  |  |  |
|  | monitoring3(10.216.40.111) |  |  |  |
|  | monitoring4 (10.216.41.111) |  |  |  |
|  | monitoring5(10.216.42.111) |  |  |  |
|  | monitoring2 (192.168.1.111) | 8140 | tcp | Связь с сервером мониторинга по протоколу puppet |
|  | monitoring3(10.216.40.111) |  |  |  |
|  | monitoring4 (10.216.41.111) |  |  |  |
|  | monitoring5(10.216.42.111) |  |  |  |
|  | nag2 (172.16.1.11) | 161 | udp | Связь с сервером мониторинга по протоколу snmp |
|  | monitoring2 (192.168.1.111) |  |  |  |
|  | monitoring3(10.216.40.111) |  |  |  |
|  | monitoring4 (10.216.41.111) |  |  |  |
|  | monitoring5(10.216.42.111) |  |  |  |
|  | ntp01 (10.219.12.7) | 123 | udp | Связь с сервером точного времени по протоколу ntp |
|  | ntp02 (172.20.16.193) |  |  |  |
|  | ntp03 (172.20.16.194) |  |  |  |

|  | monitoring2 (192.168.1.111) | 80 | tcp | Доступ к репозиториям программ по протоколу http |
| --- | --- | --- | --- | --- |
|  | monitoring3(10.216.40.111) |  |  |  |
|  | monitoring4 (10.216.41.111) |  |  |  |
|  | monitoring5(10.216.42.111) |  |  |  |
|  | nag2 (172.16.1.11) |  | icmp | Эхо запросы проверки доступности хоста по протоколу icmp |
|  | monitoring2 (192.168.1.111) |  |  |  |
|  | monitoring3(10.216.40.111) |  |  |  |
|  | monitoring4 (10.216.41.111) |  |  |  |
|  | monitoring5(10.216.42.111) |  |  |  |

| Дата | Комментарий | Автор |
| --- | --- | --- |
| 04.02.2014 12:05 | Администраторам ЛВС. Прошу проверить и предоставить (в случае отсутствия) доступ с сервера ds-mon11t (10.216.43.43) до следующих серверов по следующим портам. Серверы: nag2 (172.16.1.11), monitoring2 (192.168.1.111), monitoring3(10.216.40.111), monitoring4 (10.216.41.111), monitoring5(10.216.42.111). Порт: 5666; Протокол: tcp; Назначение: Связь с сервером мониторинга по протоколу nrpe. Серверы: monitoring2 (192.168.1.111), monitoring3(10.216.40.111), monitoring4 (10.216.41.111), monitoring5(10.216.42.111). Порт: 8140; Протокол: tcp; Назначение: Связь с сервером мониторинга по протоколу puppet. Серверы: nag2 (172.16.1.11), monitoring2 (192.168.1.111), monitoring3(10.216.40.111), monitoring4 (10.216.41.111), monitoring5(10.216.42.111). Порт: 161; Протокол: udp; Назначение: Связь с сервером мониторинга по протоколу snmp. Серверы: ntp01 (10.219.12.7), ntp02 (172.20.16.193), ntp03 (172.20.16.194). Порт: 123; Протокол: udp; Назначение: Связь с сервером точного времени по протоколу ntp. Серверы: monitoring2 (192.168.1.111), monitoring3(10.216.40.111), monitoring4 (10.216.41.111), monitoring5(10.216.42.111). Порт: 80; Протокол: tcp; Назначение: Доступ к репозиториям пакетов программ по протоколу http. Серверы: nag2 (172.16.1.11), monitoring2 (192.168.1.111), monitoring3(10.216.40.111), monitoring4 (10.216.41.111), monitoring5(10.216.42.111), Протокол: icmp Назначение: Проверка доступности хоста эхо-запросами по протоколу icmp. | Белогруд Сергей В |

*. Create ticket to write DNS-name for server.

| Дата | Комментарий | Автор |
| --- | --- | --- |
| 04.02.2014 10:39 | Администраторам ms Windows. Создать А-запись для хоста. Прошу хосту с ip-адресом 10.216.43.43 присвоить DNS-имя ds-mon11t. После выполнения работ, прошу заявку вернуть на меня. | Белогруд Сергей В |

*. Correct date and time.

[root@ds-mon11t yum.repos.d]# hwclock --show

Fri 21 Jun 2013 08:03:01 PM MSK  -0.922359 seconds

[root@ds-mon11t yum.repos.d]# date 020413242014.00

Tue Feb  4 13:24:00 MSK 2014

[root@ds-mon11t yum.repos.d]#

[root@ds-mon11t yum.repos.d]# hwclock --show

Fri 21 Jun 2013 08:05:26 PM MSK  -0.172337 seconds

[root@ds-mon11t yum.repos.d]# hwclock --systohc

[root@ds-mon11t yum.repos.d]# hwclock --show

Tue 04 Feb 2014 01:25:47 PM MSK  -0.520532 seconds

[root@ds-mon11t yum.repos.d]#

*. Configure timezone.

[root@ds-mon11t yum.repos.d]# echo "UTC=false" >> /etc/sysconfig/clock

[root@ds-mon11t yum.repos.d]# cat !$

cat /etc/sysconfig/clock

ZONE="Europe/Moscow"

UTC=false

[root@ds-mon11t yum.repos.d]#

*. Configre network for using into Intranetwork.

Add “search” and “domain” defines.

[root@ds-mon11t yum.repos.d]# vim /etc/resolv.conf

# Generated by NetworkManager

search tcsbank.ru

domain tcsbank.ru

nameserver 10.216.42.5

nameserver 10.216.40.4

*. Disabling ip6v protocol.

[root@ds-mon11t tmp]# cat > /etc/modprobe.d/disable-ipv6.conf

options ipv6 disable=1

[root@ds-mon11t tmp]#

[root@ds-mon11t tmp]# echo "NETWORKING_IPV6=no" >> /etc/sysconfig/network

[root@ds-mon11t tmp]# cat !$

cat /etc/sysconfig/network

NETWORKING=yes

HOSTNAME=ds-mon11t

GATEWAY=10.216.43.254

NM_BOND_VLAN_ENABLED=yes

NETWORKING_IPV6=no

[root@ds-mon11t tmp]#

[root@ds-mon11t tmp]# grep -i -e IPV6INIT -e IPV6_AUTOCONF /etc/sysconfig/network-scripts/ifcfg-eth*

/etc/sysconfig/network-scripts/ifcfg-eth0:IPV6INIT="no"

[root@ds-mon11t tmp]#

[root@ds-mon11t tmp]# echo "IPV6_AUTOCONF=no" >> /etc/sysconfig/network-scripts/ifcfg-eth0

[root@ds-mon11t tmp]# echo "IPV6_AUTOCONF=no" >> /etc/sysconfig/network-scripts/ifcfg-eth0.1643

[root@ds-mon11t tmp]# echo "IPV6INIT=no" >> /etc/sysconfig/network-scripts/ifcfg-eth0.1643

[root@ds-mon11t tmp]#

[root@ds-mon11t tmp]# cat /etc/sysconfig/network-scripts/ifcfg-eth0

DEVICE="eth0"

#BOOTPROTO="static"

#DNS1="10.216.42.5"

#DNS2="10.216.40.4"

#GATEWAY="10.216.43.254"

HWADDR="2C:27:D7:53:B7:A0"

#IPADDR="10.216.43.43"

IPV6INIT="no"

#NETMASK="255.255.255.0"

NM_CONTROLLED="yes"

ONBOOT="yes"

TYPE="Ethernet"

UUID="7461ba57-80f6-49cc-9256-4849967aaec8"

IPV6_AUTOCONF=no

[root@ds-mon11t tmp]#

[root@ds-mon11t tmp]# cat /etc/sysconfig/network-scripts/ifcfg-eth0.1643

DEVICE="eth0.1643"

BOOTPROTO="static"

DNS1="10.216.42.5"

DNS2="10.216.40.4"

GATEWAY="10.216.43.254"

HOSTNAME="ds-mon11t"

IPADDR="10.216.43.43"

MTU="1500"

NETMASK="255.255.255.0"

NM_CONTROLLED="yes"

ONBOOT="yes"

TYPE="Vlan"

UUID="066cbd9e-d1d8-4009-97ec-f24ffa02b949"

VLAN="yes"

IPV6_AUTOCONF=no

IPV6INIT=no

[root@ds-mon11t tmp]#

[root@ds-mon11t tmp]# chkconfig --list ip6tables

ip6tables       0:off   1:off   2:on    3:on    4:on    5:on    6:off

[root@ds-mon11t tmp]#

[root@ds-mon11t tmp]# chkconfig --level 2345 ip6tables off

[root@ds-mon11t tmp]#

*. Run files with firmware updates that I download before for Bios, Raid controller, Network card.

BIOS update.

[root@ds-mon11t tmp]# ./CP022432.scexe

tar: CP022432.xml: time stamp 2014-01-10 03:56:31 is 17491624.666438431 s in the future

tar: CPQP7012.B94: time stamp 2014-01-08 02:05:47 is 17312180.657180709 s in the future

tar: cpqsfp70.cfg: time stamp 2014-01-10 03:56:16 is 17491609.656373938 s in the future

Flash Engine Version: Linux-1.2.0-3

Name: Online ROM Flash Component for Linux - HP ProLiant DL380p Gen8 (P70) Servers

New Version: 12/20/2013

Current Version: 09/08/2013

The software is installed but is not up to date.

Do you want to upgrade the software to a newer version (y/n) ? y

Flash in progress do not interrupt or your system may become unusable.

Working.......................................................................

The installation procedure completed successfully.

A reboot is required to finish the installation completely.

Do you want to reboot your system now? y

Broadcast message from root@ds-mon11t

(/dev/pts/0) at 17:13 ...

The system is going down for reboot NOW!

[root@ds-mon11t tmp]# Connection to 10.216.43.43 closed by remote host.

Connection to 10.216.43.43 closed.

[support@centos00 20140203-1751]$

After reboot.

[root@ds-mon11t tmp]# dmidecode -t bios

# dmidecode 2.11

SMBIOS 2.8 present.

# SMBIOS implementations newer than version 2.7 are not

# fully supported by this version of dmidecode.

Handle 0x0000, DMI type 0, 24 bytes

BIOS Information

Vendor: HP

Version: P70

Release Date: 12/20/2013

Address: 0xF0000

Runtime Size: 64 kB

ROM Size: 8192 kB

Characteristics:

PCI is supported

PNP is supported

BIOS is upgradeable

BIOS shadowing is allowed

ESCD support is available

Boot from CD is supported

Selectable boot is supported

EDD is supported

5.25"/360 kB floppy services are supported (int 13h)

5.25"/1.2 MB floppy services are supported (int 13h)

3.5"/720 kB floppy services are supported (int 13h)

Print screen service is supported (int 5h)

8042 keyboard services are supported (int 9h)

Serial services are supported (int 14h)

Printer services are supported (int 17h)

CGA/mono video services are supported (int 10h)

ACPI is supported

USB legacy is supported

BIOS boot specification is supported

Function key-initiated network boot is supported

Targeted content distribution is supported

Firmware Revision: 1.30

[root@ds-mon11t tmp]#

Just for it will be.

[root@ds-mon11t tmp]# dmidecode -t system

# dmidecode 2.11

SMBIOS 2.8 present.

# SMBIOS implementations newer than version 2.7 are not

# fully supported by this version of dmidecode.

Handle 0x0100, DMI type 1, 27 bytes

System Information

Manufacturer: HP

Product Name: ProLiant DL380p Gen8

Version: Not Specified

Serial Number: CZ224809Z9

UUID: 32323636-3735-5A43-3232-343830395A39

Wake-up Type: Power Switch

SKU Number: 662257-421

Family: ProLiant

Handle 0x2000, DMI type 32, 11 bytes

System Boot Information

Status: No errors detected

[root@ds-mon11t tmp]#

[support@centos00 DL380p Gen8]$ scp Firmware\ -\ Network/20140203/* root@10.216.43.43:/tmp/

root@10.216.43.43's password:

CP018783.scexe                                                                                                                         100% 1660KB   1.6MB/s   00:00

CP018827.scexe                                                                                                                         100% 3832KB   1.9MB/s   00:02

CP018871.scexe                                                                                                                         100% 5512KB   2.7MB/s   00:02

CP021537.scexe                                                                                                                         100% 4841KB   2.4MB/s   00:02

CP021557.scexe                                                                                                                         100%   11MB   1.8MB/s   00:06

fw-ConnectX3-rel-2_30_8000-649281-B21_Bx-FlexBoot-3.4.151_VPI-CLP-8025.bin                                                             100%  737KB 737.4KB/s   00:00

fw-ConnectX3-rel-2_30_8000-649282-B21_B2-FlexBoot-3.4.151_VPI-CLP-8025.bin                                                             100%  737KB 737.5KB/s   00:00

fw-ConnectX3-rel-2_30_8000-649282-B21_Cx-Dx-FlexBoot-3.4.151_VPI-CLP-8025.bin                                                          100%  738KB 737.5KB/s   00:00

fw-ConnectX3-rel-2_30_8000-649283-B21_B2-FlexBoot-3.4.151_VPI-CLP-8025.bin                                                             100%  737KB 737.4KB/s   00:00

fw-ConnectX3-rel-2_30_8000-649283-B21_Cx-Dx-FlexBoot-3.4.151_VPI-CLP-8025.bin                                                          100%  737KB 737.4KB/s   00:00

readme.txt                                                                                                                             100% 7953     7.8KB/s   00:00

[support@centos00 DL380p Gen8]$

Only one network adapter was found.

[root@ds-mon11t tmp]# ls -Alh

total 30M

-rwxr-xr-x  1 root root 1.7M Jun 21 17:31 CP018783.scexe

-rwxr-xr-x  1 root root 3.8M Jun 21 17:31 CP018827.scexe

-rwxr-xr-x  1 root root 5.4M Jun 21 17:31 CP018871.scexe

-rwxr-xr-x  1 root root 4.8M Jun 21 17:31 CP021537.scexe

-rwxr-xr-x  1 root root  11M Jun 21 17:32 CP021557.scexe

-rwxr-xr-x  1 root root 738K Jun 21 17:32 fw-ConnectX3-rel-2_30_8000-649281-B21_Bx-FlexBoot-3.4.151_VPI-CLP-8025.bin

-rwxr-xr-x  1 root root 738K Jun 21 17:32 fw-ConnectX3-rel-2_30_8000-649282-B21_B2-FlexBoot-3.4.151_VPI-CLP-8025.bin

-rwxr-xr-x  1 root root 738K Jun 21 17:32 fw-ConnectX3-rel-2_30_8000-649282-B21_Cx-Dx-FlexBoot-3.4.151_VPI-CLP-8025.bin

-rwxr-xr-x  1 root root 738K Jun 21 17:32 fw-ConnectX3-rel-2_30_8000-649283-B21_B2-FlexBoot-3.4.151_VPI-CLP-8025.bin

-rwxr-xr-x  1 root root 738K Jun 21 17:32 fw-ConnectX3-rel-2_30_8000-649283-B21_Cx-Dx-FlexBoot-3.4.151_VPI-CLP-8025.bin

drwxrwxrwt  2 root root 4.0K Jun 21 17:17 .ICE-unix

drwx------. 2 root root  16K Jun 21 01:11 lost+found

-rwxr-xr-x  1 root root 7.8K Jun 21 17:32 readme.txt

-rw-------. 1 root root    0 Jun 21 01:12 yum.log

[root@ds-mon11t tmp]# ./CP018783.scexe

##################################################################################################

HP Mellanox InfiniBand Online Firmware Upgrade Utility for Linux

Copyright (c) 2011 Hewlett-Packard Development Company, L.P.

##################################################################################################

List of Network Adapters detected on the Server.................

[0] 24:00.1 QLogic Corp.

[1] 24:00.0 QLogic Corp.

[2] 03:00.1 Broadcom Corporation

[3] 03:00.0 Broadcom Corporation

[4] 07:00.1 QLogic Corp.

[5] 07:00.0 QLogic Corp.

If PSID or FW_Version is not found for some interfaces, please check /tmp/datauss3YZ

Interface 24:00.1 is not Mellanox one.

Interface 24:00.0 is not Mellanox one.

Interface 03:00.1 is not Mellanox one.

Interface 03:00.0 is not Mellanox one.

Interface 07:00.1 is not Mellanox one.

Interface 07:00.0 is not Mellanox one.

NIC firmware update did not complete.  Check log for errors.

[root@ds-mon11t tmp]# ./CP018827.scexe

Copyright (c) 2013 Hewlett-Packard Development Company, L.P.

HP QLogic P2/P3 Online Firmware Upgrade Utility for Linux x86_64

The 'discover' operation was successful

read_discovery_report: Failed to open ./CP018827_disc.xml

Discovery report read failed

NIC firmware update did not complete. Check log for errors.

[root@ds-mon11t tmp]# ./CP018871.scexe

Copyright (c) 2013 Hewlett-Packard Development Company, L.P.

HP Intel Online Firmware Upgrade Utility for Linux x86_64

Vendor has not reported any supported devices!

[root@ds-mon11t tmp]# ./CP021537.scexe

Copyright (c) 2013 Hewlett-Packard Development Company, L.P.

HP Broadcom Online Firmware Upgrade Utility for Linux x86_64

No supported network adapters for firmware upgrade.

[root@ds-mon11t tmp]# ./CP021557.scexe

Copyright (c) 2013 Hewlett-Packard Development Company, L.P.

HP QLogic P3P Online Firmware Upgrade Utility for Linux x86_64

The 'discover' operation was successful

Found CN1000Q HP StorageWorks CN1000Q Dual Port Converged Network Adapter (NIC) MAC: 2C:27:D7:53:B7:A0

*** WARNING *** - Installed firmware is the same version as selected firmware.

Update firmware on eth0 :   ROM 1.13.51 to 1.13.51

y/n/q (Default option is 'n' when you simply press enter):n

Installation was not attempted on the eth0 interface (MAC:2C:27:D7:53:B7:A0) because installed firmware version is same as selected firmware.

Found CN1000Q HP StorageWorks CN1000Q Dual Port Converged Network Adapter (NIC) MAC: 2C:27:D7:53:BA:70

*** WARNING *** - Installed firmware is the same version as selected firmware.

Update firmware on eth2 :   ROM 1.13.51 to 1.13.51

y/n/q (Default option is 'n' when you simply press enter):n

Installation was not attempted on the eth2 interface (MAC:2C:27:D7:53:BA:70) because installed firmware version is same as selected firmware.

Installation was not attempted on the available NIC(s) because they have the same installed firmware version as the selected firmware.

[root@ds-mon11t tmp]#

Raid controller firmware update.

[support@centos00 DL380p Gen8]$ scp Firmware\ -\ Storage\ Controller/20140203/* root@10.216.43.43:/tmp/

root@10.216.43.43's password:

CP020831.scexe                                                                                                                         100% 2572KB   2.5MB/s   00:00

CP020876.scexe                                                                                                                         100% 2850KB   2.8MB/s   00:01

CP021158.scexe                                                                                                                         100% 5455KB   1.8MB/s   00:03

CP021383.scexe                                                                                                                         100% 2693KB   2.6MB/s   00:01

CP021553.scexe                                                                                                                         100% 4633KB   2.3MB/s   00:02

readme.txt                                                                                                                             100% 1340     1.3KB/s   00:00

[support@centos00 DL380p Gen8]$

[root@ds-mon11t tmp]# ls -Alh

total 18M

-rwxr-xr-x  1 root root 2.6M Jun 21 17:41 CP020831.scexe

-rwxr-xr-x  1 root root 2.8M Jun 21 17:41 CP020876.scexe

-rwxr-xr-x  1 root root 5.4M Jun 21 17:41 CP021158.scexe

-rwxr-xr-x  1 root root 2.7M Jun 21 17:41 CP021383.scexe

-rwxr-xr-x  1 root root 4.6M Jun 21 17:41 CP021553.scexe

-rw-------  1 root root    0 Jun 21 17:32 datauss3YZ

drwxrwxrwt  2 root root 4.0K Jun 21 17:17 .ICE-unix

drwx------. 2 root root  16K Jun 21 01:11 lost+found

-rw-r--r--  1 root root    0 Jun 21 17:32 mstflint.log

drwxr-xr-x  2 root root 4.0K Jun 21 17:32 nicfw_backup

-rw-r--r--  1 root root  203 Jun 21 17:34 nicfwupg.log

-rwxr-xr-x  1 root root 1.4K Jun 21 17:41 readme.txt

-rw-------. 1 root root    0 Jun 21 01:12 yum.log

[root@ds-mon11t tmp]# ./CP020831.scexe

tar: CP020831.xml: time stamp 2013-06-29 01:26:23 is 632626.922932172 s in the future

tar: H210i_it.fw: time stamp 2013-06-27 20:10:14 is 527257.920926207 s in the future

tar: H220_it.fw: time stamp 2013-06-27 20:10:40 is 527283.919048716 s in the future

tar: H220i_it.fw: time stamp 2013-06-27 20:10:34 is 527277.917350462 s in the future

tar: H221_it.fw: time stamp 2013-06-27 20:10:46 is 527289.916522908 s in the future

tar: H222_it.fw: time stamp 2013-06-27 20:10:28 is 527271.915660314 s in the future

tar: firmware.cfg: time stamp 2013-06-29 01:26:13 is 632616.915625723 s in the future

tar: hpsetup: time stamp 2013-06-29 01:26:13 is 632616.915596006 s in the future

tar: mptsas2.rom: time stamp 2013-06-27 20:10:58 is 527301.914890754 s in the future

./hpsetup: ./lsiflash: /lib/ld-linux.so.2: bad ELF interpreter: No such file or directory

[root@ds-mon11t tmp]# ./CP020876.scexe

tar: C-238.fuf: time stamp 2013-07-04 00:51:06 is 1062478.362802955 s in the future

tar: ccissflash: time stamp 2013-07-04 00:51:06 is 1062478.353817084 s in the future

tar: Config.xml: time stamp 2013-07-04 00:51:06 is 1062478.353758563 s in the future

tar: CP020876.xml: time stamp 2013-07-04 00:51:06 is 1062478.353678457 s in the future

tar: hpsetup: time stamp 2013-07-04 00:50:35 is 1062447.353647874 s in the future

tar: libcpqimgr.so: time stamp 2013-07-04 00:51:06 is 1062478.351632226 s in the future

./hpsetup: ./ccissflash: /lib/ld-linux.so.2: bad ELF interpreter: No such file or directory

[root@ds-mon11t tmp]# ./CP021158.scexe

tar: Baz-100.fuf: time stamp 2013-07-31 02:40:43 is 3401839.83679661 s in the future

tar: Baz-Mfg-100.bin: time stamp 2013-07-31 02:40:43 is 3401839.834816851 s in the future

tar: ccissflash: time stamp 2013-07-31 02:40:43 is 3401839.829498005 s in the future

tar: Config.xml: time stamp 2013-07-31 02:40:43 is 3401839.829439421 s in the future

tar: CP021158.xml: time stamp 2013-07-31 02:40:43 is 3401839.829292189 s in the future

tar: hpsetup: time stamp 2013-07-31 02:40:13 is 3401809.829268244 s in the future

tar: libcpqimgr.so: time stamp 2013-07-31 02:40:43 is 3401839.827284855 s in the future

tar: nova.bin: time stamp 2013-07-31 02:40:43 is 3401839.820118519 s in the future

./hpsetup: ./ccissflash: /lib/ld-linux.so.2: bad ELF interpreter: No such file or directory

[root@ds-mon11t tmp]# ./CP021383.scexe

tar: B-314.fuf: time stamp 2013-08-09 22:56:58 is 4252404.26780802 s in the future

tar: ccissflash: time stamp 2013-08-09 22:56:58 is 4252404.261565417 s in the future

tar: Config.xml: time stamp 2013-08-09 22:56:58 is 4252404.261515186 s in the future

tar: CP021383.xml: time stamp 2013-08-09 22:56:58 is 4252404.261425505 s in the future

tar: hpsetup: time stamp 2013-08-09 22:56:30 is 4252376.261393867 s in the future

tar: libcpqimgr.so: time stamp 2013-08-09 22:56:58 is 4252404.259479957 s in the future

./hpsetup: ./ccissflash: /lib/ld-linux.so.2: bad ELF interpreter: No such file or directory

[root@ds-mon11t tmp]# ./CP021553.scexe

tar: ccissflash: time stamp 2013-08-20 20:48:32 is 5195087.655218692 s in the future

tar: Config.xml: time stamp 2013-08-20 20:48:32 is 5195087.655027182 s in the future

tar: CP021553.xml: time stamp 2013-08-20 20:48:32 is 5195087.654948088 s in the future

tar: danelectro.bin: time stamp 2013-08-20 20:48:32 is 5195087.646815133 s in the future

tar: hpsetup: time stamp 2013-08-20 20:48:03 is 5195058.646738902 s in the future

tar: libcpqimgr.so: time stamp 2013-08-20 20:48:32 is 5195087.644770264 s in the future

./hpsetup: ./ccissflash: /lib/ld-linux.so.2: bad ELF interpreter: No such file or directory

[root@ds-mon11t tmp]#

[root@ds-mon11t yum.repos.d]# yum install glibc.i686

Loaded plugins: product-id, security, subscription-manager

This system is not registered to Red Hat Subscription Management. You can use subscription-manager to register.

Setting up Install Process

Resolving Dependencies

--> Running transaction check

---> Package glibc.i686 0:2.12-1.107.el6 will be installed

--> Processing Dependency: libfreebl3.so(NSSRAWHASH_3.12.3) for package: glibc-2.12-1.107.el6.i686

--> Processing Dependency: libfreebl3.so for package: glibc-2.12-1.107.el6.i686

--> Running transaction check

---> Package nss-softokn-freebl.i686 0:3.12.9-11.el6 will be installed

--> Finished Dependency Resolution

Dependencies Resolved

=========================================================================================================================================================================

Package                                         Arch                              Version                                     Repository                           Size

=========================================================================================================================================================================

Installing:

glibc                                           i686                              2.12-1.107.el6                              rhel64                              4.3 M

Installing for dependencies:

nss-softokn-freebl                              i686                              3.12.9-11.el6                               rhel64                              116 k

Transaction Summary

=========================================================================================================================================================================

Install       2 Package(s)

Total download size: 4.4 M

Installed size: 13 M

Is this ok [y/N]: y

Downloading Packages:

(1/2): glibc-2.12-1.107.el6.i686.rpm                                                                                                              | 4.3 MB     00:00

(2/2): nss-softokn-freebl-3.12.9-11.el6.i686.rpm                                                                                                  | 116 kB     00:00

-------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Total                                                                                                                                     23 MB/s | 4.4 MB     00:00

Running rpm_check_debug

Running Transaction Test

Transaction Test Succeeded

Running Transaction

Installing : nss-softokn-freebl-3.12.9-11.el6.i686                                                                                                                 1/2

Installing : glibc-2.12-1.107.el6.i686                                                                                                                             2/2

rhel64/productid                                                                                                                                  | 1.7 kB     00:00

Verifying  : glibc-2.12-1.107.el6.i686                                                                                                                             1/2

Verifying  : nss-softokn-freebl-3.12.9-11.el6.i686                                                                                                                 2/2

Installed:

glibc.i686 0:2.12-1.107.el6

Dependency Installed:

nss-softokn-freebl.i686 0:3.12.9-11.el6

Complete!

[root@ds-mon11t yum.repos.d]#

[root@ds-mon11t tmp]# ./CP020831.scexe

tar: CP020831.xml: time stamp 2013-06-29 01:26:23 is 631406.823679262 s in the future

tar: H210i_it.fw: time stamp 2013-06-27 20:10:14 is 526037.821611351 s in the future

tar: H220_it.fw: time stamp 2013-06-27 20:10:40 is 526063.81961031 s in the future

tar: H220i_it.fw: time stamp 2013-06-27 20:10:34 is 526057.817592293 s in the future

tar: H221_it.fw: time stamp 2013-06-27 20:10:46 is 526069.816757414 s in the future

tar: H222_it.fw: time stamp 2013-06-27 20:10:28 is 526051.815892592 s in the future

tar: firmware.cfg: time stamp 2013-06-29 01:26:13 is 631396.815858615 s in the future

tar: hpsetup: time stamp 2013-06-29 01:26:13 is 631396.815826535 s in the future

tar: mptsas2.rom: time stamp 2013-06-27 20:10:58 is 526081.815104338 s in the future

./lsiflash: error while loading shared libraries: libstdc++.so.6: cannot open shared object file: No such file or directory

[root@ds-mon11t tmp]#

[root@ds-mon11t tmp]# yum install libstdc++.i686

Loaded plugins: product-id, security, subscription-manager

This system is not registered to Red Hat Subscription Management. You can use subscription-manager to register.

Setting up Install Process

Resolving Dependencies

--> Running transaction check

---> Package libstdc++.i686 0:4.4.7-3.el6 will be installed

--> Processing Dependency: libgcc_s.so.1(GLIBC_2.0) for package: libstdc++-4.4.7-3.el6.i686

--> Processing Dependency: libgcc_s.so.1(GCC_4.2.0) for package: libstdc++-4.4.7-3.el6.i686

--> Processing Dependency: libgcc_s.so.1(GCC_3.3) for package: libstdc++-4.4.7-3.el6.i686

--> Processing Dependency: libgcc_s.so.1(GCC_3.0) for package: libstdc++-4.4.7-3.el6.i686

--> Processing Dependency: libgcc_s.so.1 for package: libstdc++-4.4.7-3.el6.i686

--> Running transaction check

---> Package libgcc.i686 0:4.4.7-3.el6 will be installed

--> Finished Dependency Resolution

Dependencies Resolved

=========================================================================================================================================================================

Package                                   Arch                                 Version                                     Repository                              Size

=========================================================================================================================================================================

Installing:

libstdc++                                 i686                                 4.4.7-3.el6                                 rhel64                                 300 k

Installing for dependencies:

libgcc                                    i686                                 4.4.7-3.el6                                 rhel64                                 112 k

Transaction Summary

=========================================================================================================================================================================

Install       2 Package(s)

Total download size: 411 k

Installed size: 1.0 M

Is this ok [y/N]: y

Downloading Packages:

(1/2): libgcc-4.4.7-3.el6.i686.rpm                                                                                                                | 112 kB     00:00

(2/2): libstdc++-4.4.7-3.el6.i686.rpm                                                                                                             | 300 kB     00:00

-------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Total                                                                                                                                    3.9 MB/s | 411 kB     00:00

Running rpm_check_debug

Running Transaction Test

Transaction Test Succeeded

Running Transaction

Installing : libgcc-4.4.7-3.el6.i686                                                                                                                               1/2

Installing : libstdc++-4.4.7-3.el6.i686                                                                                                                            2/2

Verifying  : libstdc++-4.4.7-3.el6.i686                                                                                                                            1/2

Verifying  : libgcc-4.4.7-3.el6.i686                                                                                                                               2/2

Installed:

libstdc++.i686 0:4.4.7-3.el6

Dependency Installed:

libgcc.i686 0:4.4.7-3.el6

Complete!

[root@ds-mon11t tmp]#

[root@ds-mon11t tmp]# ./CP020831.scexe

tar: CP020831.xml: time stamp 2013-06-29 01:26:23 is 631211.012055367 s in the future

tar: H210i_it.fw: time stamp 2013-06-27 20:10:14 is 525842.01000464 s in the future

tar: H220_it.fw: time stamp 2013-06-27 20:10:40 is 525868.007333367 s in the future

tar: H220i_it.fw: time stamp 2013-06-27 20:10:34 is 525862.006470634 s in the future

tar: H221_it.fw: time stamp 2013-06-27 20:10:46 is 525874.005649937 s in the future

tar: H222_it.fw: time stamp 2013-06-27 20:10:28 is 525856.004799433 s in the future

tar: firmware.cfg: time stamp 2013-06-29 01:26:13 is 631201.004758338 s in the future

tar: hpsetup: time stamp 2013-06-29 01:26:13 is 631201.004727742 s in the future

tar: mptsas2.rom: time stamp 2013-06-27 20:10:58 is 525886.004022941 s in the future

match:  ERROR: Command Not allowed without an adapter!

[root@ds-mon11t tmp]#

[root@ds-mon11t tmp]# /tmp/CP020876.scexe

tar: C-238.fuf: time stamp 2013-07-04 00:51:06 is 1060855.1773944 s in the future

tar: ccissflash: time stamp 2013-07-04 00:51:06 is 1060855.170679458 s in the future

tar: Config.xml: time stamp 2013-07-04 00:51:06 is 1060855.170617888 s in the future

tar: CP020876.xml: time stamp 2013-07-04 00:51:06 is 1060855.170538111 s in the future

tar: hpsetup: time stamp 2013-07-04 00:50:35 is 1060824.170506864 s in the future

tar: libcpqimgr.so: time stamp 2013-07-04 00:51:06 is 1060855.168145078 s in the future

This program consists of two phases: device discovery and device update.

No device will be updated until you answer.

Do you want to run device discovery?

(yes/no) yes

Finding hardware. This may take a few minutes.

Found 0 devices.

No device will be updated.

No devices updated.

[root@ds-mon11t tmp]#

Only one device was found.

[root@ds-mon11t tmp]# /tmp/CP021158.scexe

tar: Baz-100.fuf: time stamp 2013-07-31 02:40:43 is 3400161.983206726 s in the future

tar: Baz-Mfg-100.bin: time stamp 2013-07-31 02:40:43 is 3400161.977854201 s in the future

tar: ccissflash: time stamp 2013-07-31 02:40:43 is 3400161.971548648 s in the future

tar: Config.xml: time stamp 2013-07-31 02:40:43 is 3400161.971477211 s in the future

tar: CP021158.xml: time stamp 2013-07-31 02:40:43 is 3400161.971296949 s in the future

tar: hpsetup: time stamp 2013-07-31 02:40:13 is 3400131.971263254 s in the future

tar: libcpqimgr.so: time stamp 2013-07-31 02:40:43 is 3400161.969240047 s in the future

tar: nova.bin: time stamp 2013-07-31 02:40:43 is 3400161.960699854 s in the future

This program consists of two phases: device discovery and device update.

No device will be updated until you answer.

Do you want to run device discovery?

(yes/no) yes

Finding hardware. This may take a few minutes.

Found 1 devices.

No devices updated.

[root@ds-mon11t tmp]#

[root@ds-mon11t tmp]# /tmp/CP021383.scexe

tar: B-314.fuf: time stamp 2013-08-09 22:56:58 is 4250708.407182831 s in the future

tar: ccissflash: time stamp 2013-08-09 22:56:58 is 4250708.400803687 s in the future

tar: Config.xml: time stamp 2013-08-09 22:56:58 is 4250708.400738515 s in the future

tar: CP021383.xml: time stamp 2013-08-09 22:56:58 is 4250708.400640605 s in the future

tar: hpsetup: time stamp 2013-08-09 22:56:30 is 4250680.40060869 s in the future

tar: libcpqimgr.so: time stamp 2013-08-09 22:56:58 is 4250708.398322066 s in the future

This program consists of two phases: device discovery and device update.

No device will be updated until you answer.

Do you want to run device discovery?

(yes/no) yes

Finding hardware. This may take a few minutes.

Found 0 devices.

No device will be updated.

No devices updated.

[root@ds-mon11t tmp]#

[root@ds-mon11t tmp]# /tmp/CP021553.scexe

tar: ccissflash: time stamp 2013-08-20 20:48:32 is 5193376.694964965 s in the future

tar: Config.xml: time stamp 2013-08-20 20:48:32 is 5193376.694764617 s in the future

tar: CP021553.xml: time stamp 2013-08-20 20:48:32 is 5193376.694682162 s in the future

tar: danelectro.bin: time stamp 2013-08-20 20:48:32 is 5193376.686470548 s in the future

tar: hpsetup: time stamp 2013-08-20 20:48:03 is 5193347.686397627 s in the future

tar: libcpqimgr.so: time stamp 2013-08-20 20:48:32 is 5193376.684414897 s in the future

This program consists of two phases: device discovery and device update.

No device will be updated until you answer.

Do you want to run device discovery?

(yes/no) yes

Finding hardware. This may take a few minutes.

Found 0 devices.

No device will be updated.

No devices updated.

[root@ds-mon11t tmp]#

*. Connect to repositories. I got commnads from document where was created when I had create vmware templates. Here is basic list of repositories.

[root@ds-mon11t tmp]# cd /etc/yum.repos.d/

[root@ds-mon11t yum.repos.d]# ls -Alh

total 4.0K

-rw-r--r--. 1 root root 529 Jan 29 19:47 rhel-source.repo

[root@ds-mon11t yum.repos.d]#

[root@ds-mon11t yum.repos.d]# curl -O "http://monitoring2/repo/tcs/yum.repos.d/tcs-epel6.repo"

% Total    % Received % Xferd  Average Speed   Time    Time     Time  Current

Dload  Upload   Total   Spent    Left  Speed

180   180  180   180    0     0  11347      0 --:--:-- --:--:-- --:--:-- 18000

[root@ds-mon11t yum.repos.d]#

[root@ds-mon11t yum.repos.d]# curl -O "http://monitoring2/repo/tcs/yum.repos.d/tcs-hp.repo"

% Total    % Received % Xferd  Average Speed   Time    Time     Time  Current

Dload  Upload   Total   Spent    Left  Speed

185   185  185   185    0     0  14464      0 --:--:-- --:--:-- --:--:-- 23125

[root@ds-mon11t yum.repos.d]#

[root@ds-mon11t yum.repos.d]# curl -O "http://monitoring2/repo/tcs/yum.repos.d/tcs-rhel64.repo"

% Total    % Received % Xferd  Average Speed   Time    Time     Time  Current

Dload  Upload   Total   Spent    Left  Speed

127   254  127   254    0     0  22069      0 --:--:-- --:--:-- --:--:-- 42333

[root@ds-mon11t yum.repos.d]#

[root@ds-mon11t yum.repos.d]# yum repolist

Loaded plugins: product-id, security, subscription-manager

This system is not registered to Red Hat Subscription Management. You can use subscription-manager to register.

EPEL                                                                                                                                              | 4.2 kB     00:00

EPEL/primary_db                                                                                                                                   | 5.9 MB     00:00

rhel-hp                                                                                                                                           |  951 B     00:00

rhel-hp/primary                                                                                                                                   |  19 kB     00:00

rhel-hp                                                                                                                                                            35/35

rhel64                                                                                                                                            | 3.9 kB     00:00

rhel64/primary_db                                                                                                                                 | 3.1 MB     00:00

repo id                                              repo name                                                                                                     status

EPEL                                                 TCS EPEL repository                                                                                           10,412

rhel-hp                                              Red Hat Enterprise Linux HP repository 6Server - x86_64 - Source                                                  35

rhel64                                               Red Hat Enterprise Linux 6Server - x86_64 - Source                                                             3,648

repolist: 14,095

[root@ds-mon11t yum.repos.d]#

[root@ds-mon11t yum.repos.d]# curl -O "http://monitoring2/repo/tcs/yum.repos.d/tcs-remi.repo"

% Total    % Received % Xferd  Average Speed   Time    Time     Time  Current

Dload  Upload   Total   Spent    Left  Speed

107   644  107   644    0     0  40217      0 --:--:-- --:--:-- --:--:-- 71555

[root@ds-mon11t yum.repos.d]#

[root@ds-mon11t yum.repos.d]# curl -O "http://monitoring2/repo/tcs/yum.repos.d/tcs-puppetlabs.repo"

% Total    % Received % Xferd  Average Speed   Time    Time     Time  Current

Dload  Upload   Total   Spent    Left  Speed

119   479  119   479    0     0  29120      0 --:--:-- --:--:-- --:--:-- 47900

[root@ds-mon11t yum.repos.d]#

[root@ds-mon11t yum.repos.d]# yum repolist

Loaded plugins: product-id, security, subscription-manager

This system is not registered to Red Hat Subscription Management. You can use subscription-manager to register.

puppetlabs                                                                                                                                        | 1.9 kB     00:00

puppetlabs/primary_db                                                                                                                             | 106 kB     00:00

puppetlabs-dependencies                                                                                                                           | 1.9 kB     00:00

puppetlabs-dependencies/primary_db                                                                                                                |  21 kB     00:00

repo id                                                      repo name                                                                                             status

EPEL                                                         TCS EPEL repository                                                                                   10,412

puppetlabs                                                   Puppet Labs Packages                                                                                     329

puppetlabs-dependencies                                      Puppet Labs Packages Dependencies                                                                         54

rhel-hp                                                      Red Hat Enterprise Linux HP repository 6Server - x86_64 - Source                                          35

rhel64                                                       Red Hat Enterprise Linux 6Server - x86_64 - Source                                                     3,648

repolist: 14,478

[root@ds-mon11t yum.repos.d]#

*. Create administration user accounts and grant them access. Like bsv and so on.

[root@ds-mon11t ~]# useradd -u 525 -c "Belogrud Sergey Vladimirovich,BC Diapazone,2173" -d /home/bsv -m -G root,wheel -s /bin/bash -p '$6$aTOUz./e$V/Svdh3ZepbOGq7wVwMbscFoV2CB7jz5C.l/upLpyD7WmxPwjMJg8cyI4V7sZwKcN7ODLAcmZNHHerELq.bPh0' bsv

[root@ds-mon11t ~]#

[root@ds-mon11t ~]# useradd -u 504 -c 'Nikita Makhnev,BC Diapazone,2069' -d /home/lzn -m -G root,wheel -s /bin/bash -p '$1$acxcWmj6$ipbLi0qgPU607lMEvmTxm1' lzn

[root@ds-mon11t ~]# useradd -u 501 -c 'Aleksey Tertentyev,BC Diapazone,2037' -d /home/tan -m -G root,wheel -s /bin/bash -p '$1$vjkXeATR$Yp20XAZWMNX7q9E5pYth1.' tan

[root@ds-mon11t ~]# useradd -u 503 -c 'Aleksey Aleksandrov,BC Diapazone,2138' -d /home/avk -m -G root,wheel -s /bin/bash -p '$1$rWT5HuLB$B2rz1lJd6PTzr/zAsUVE9.' avk

[root@ds-mon11t ~]# useradd -u 528 -c 'Chernyh Aleksandr,BC Diapazone,2287' -d '/home/che' -m -G root,wheel -s /bin/bash -p '$6$b2mekjgP$8LikVu/Wz.2bPx7shCuvUHM0dGs/.CvyATOBzR43nYgvKpeJws2tSGL43qM1eOXavNO.SBxP5nHeE/5uMMyGZ.' che

[root@ds-mon11t ~]# useradd -u 529 -c 'Zbiranik Evgeniy,BC Diapazone,2502' -d '/home/zbir'  -m -G root,wheel -s /bin/bash -p '$1$1UXDz1eM$fxh64nIOL6KZ6371PItv9/' zbir

[root@ds-mon11t ~]#

Uncomment sudo access for users who member of wheel group.

[root@ds-mon11t tmp]# visudo

...

## Allows people in group wheel to run all commands

%wheel  ALL=(ALL)       ALL

*. Disable ip6v protocol (really). Because after kickstart installation we still have see ip6v protocols.

[root@ds-mon11t ~]# ifconfig | grep inet

inet6 addr: fe80::2e27:d7ff:fe53:b7a0/64 Scope:Link

inet addr:10.216.43.43  Bcast:10.216.43.255  Mask:255.255.255.0

inet6 addr: fe80::2e27:d7ff:fe53:b7a0/64 Scope:Link

inet addr:127.0.0.1  Mask:255.0.0.0

inet6 addr: ::1/128 Scope:Host

[root@ds-mon11t ~]#

Next few points of configuring server for usages.

*. Check security updates and install it.

[root@ds-mon11t ~]# yum --security check-update

Loaded plugins: product-id, security, subscription-manager

This system is not registered to Red Hat Subscription Management. You can use subscription-manager to register.

Limiting package lists to security relevant ones

EPEL/updateinfo                                                                                                                                   | 761 kB     00:00

No packages needed for security; 0 packages available

[root@ds-mon11t ~]#

But maybe it doesn’t work porperly because RHN doesn’t configured and doesn’t connected.

*. Install useful utility.

[root@ds-mon11t ~]# yum install telnet

Loaded plugins: product-id, security, subscription-manager

This system is not registered to Red Hat Subscription Management. You can use subscription-manager to register.

Setting up Install Process

Resolving Dependencies

--> Running transaction check

---> Package telnet.x86_64 1:0.17-47.el6_3.1 will be installed

--> Finished Dependency Resolution

Dependencies Resolved

=========================================================================================================================================================================

Package                              Arch                                 Version                                            Repository                            Size

=========================================================================================================================================================================

Installing:

telnet                               x86_64                               1:0.17-47.el6_3.1                                  rhel64                                58 k

Transaction Summary

=========================================================================================================================================================================

Install       1 Package(s)

Total download size: 58 k

Installed size: 109 k

Is this ok [y/N]: y

Downloading Packages:

telnet-0.17-47.el6_3.1.x86_64.rpm                                                                                                                 |  58 kB     00:00

Running rpm_check_debug

Running Transaction Test

Transaction Test Succeeded

Running Transaction

Installing : 1:telnet-0.17-47.el6_3.1.x86_64                                                                                                                       1/1

Verifying  : 1:telnet-0.17-47.el6_3.1.x86_64                                                                                                                       1/1

Installed:

telnet.x86_64 1:0.17-47.el6_3.1

Complete!

[root@ds-mon11t ~]#

*. Install puppet agent.

[root@ds-mon11t ~]# yum install ruby

Loaded plugins: product-id, security, subscription-manager

This system is not registered to Red Hat Subscription Management. You can use subscription-manager to register.

Setting up Install Process

Resolving Dependencies

--> Running transaction check

---> Package ruby.x86_64 0:1.8.7.352-7.el6_2 will be installed

--> Processing Dependency: ruby-libs = 1.8.7.352-7.el6_2 for package: ruby-1.8.7.352-7.el6_2.x86_64

--> Processing Dependency: libruby.so.1.8()(64bit) for package: ruby-1.8.7.352-7.el6_2.x86_64

--> Running transaction check

---> Package ruby-libs.x86_64 0:1.8.7.352-7.el6_2 will be installed

--> Processing Dependency: libreadline.so.5()(64bit) for package: ruby-libs-1.8.7.352-7.el6_2.x86_64

--> Running transaction check

---> Package compat-readline5.x86_64 0:5.2-17.1.el6 will be installed

--> Finished Dependency Resolution

Dependencies Resolved

=========================================================================================================================================================================

Package                                      Arch                               Version                                        Repository                          Size

=========================================================================================================================================================================

Installing:

ruby                                         x86_64                             1.8.7.352-7.el6_2                              rhel64                             532 k

Installing for dependencies:

compat-readline5                             x86_64                             5.2-17.1.el6                                   rhel64                             129 k

ruby-libs                                    x86_64                             1.8.7.352-7.el6_2                              rhel64                             1.6 M

Transaction Summary

=========================================================================================================================================================================

Install       3 Package(s)

Total download size: 2.3 M

Installed size: 7.8 M

Is this ok [y/N]: y

Downloading Packages:

(1/3): compat-readline5-5.2-17.1.el6.x86_64.rpm                                                                                                   | 129 kB     00:00

(2/3): ruby-1.8.7.352-7.el6_2.x86_64.rpm                                                                                                          | 532 kB     00:00

(3/3): ruby-libs-1.8.7.352-7.el6_2.x86_64.rpm                                                                                                     | 1.6 MB     00:00

-------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Total                                                                                                                                     10 MB/s | 2.3 MB     00:00

Running rpm_check_debug

Running Transaction Test

Transaction Test Succeeded

Running Transaction

Installing : compat-readline5-5.2-17.1.el6.x86_64                                                                                                                  1/3

Installing : ruby-libs-1.8.7.352-7.el6_2.x86_64                                                                                                                    2/3

Installing : ruby-1.8.7.352-7.el6_2.x86_64                                                                                                                         3/3

Verifying  : ruby-1.8.7.352-7.el6_2.x86_64                                                                                                                         1/3

Verifying  : ruby-libs-1.8.7.352-7.el6_2.x86_64                                                                                                                    2/3

Verifying  : compat-readline5-5.2-17.1.el6.x86_64                                                                                                                  3/3

Installed:

ruby.x86_64 0:1.8.7.352-7.el6_2

Dependency Installed:

compat-readline5.x86_64 0:5.2-17.1.el6                                               ruby-libs.x86_64 0:1.8.7.352-7.el6_2

Complete!

[root@ds-mon11t ~]#

[root@ds-mon11t ~]# yum install puppet

Loaded plugins: product-id, security, subscription-manager

This system is not registered to Red Hat Subscription Management. You can use subscription-manager to register.

Setting up Install Process

Resolving Dependencies

--> Running transaction check

---> Package puppet.noarch 0:2.7.23-1.el6 will be installed

--> Processing Dependency: facter < 1:2.0 for package: puppet-2.7.23-1.el6.noarch

--> Processing Dependency: facter >= 1.5 for package: puppet-2.7.23-1.el6.noarch

--> Processing Dependency: ruby-shadow for package: puppet-2.7.23-1.el6.noarch

--> Processing Dependency: ruby-augeas for package: puppet-2.7.23-1.el6.noarch

--> Processing Dependency: ruby(selinux) for package: puppet-2.7.23-1.el6.noarch

--> Running transaction check

---> Package facter.x86_64 0:1.6.18-3.el6 will be installed

---> Package libselinux-ruby.x86_64 0:2.0.94-5.3.el6 will be installed

---> Package ruby-augeas.x86_64 0:0.4.1-1.el6 will be installed

--> Processing Dependency: augeas-libs >= 0.8.0 for package: ruby-augeas-0.4.1-1.el6.x86_64

--> Processing Dependency: libaugeas.so.0(AUGEAS_0.8.0)(64bit) for package: ruby-augeas-0.4.1-1.el6.x86_64

--> Processing Dependency: libaugeas.so.0(AUGEAS_0.12.0)(64bit) for package: ruby-augeas-0.4.1-1.el6.x86_64

--> Processing Dependency: libaugeas.so.0(AUGEAS_0.11.0)(64bit) for package: ruby-augeas-0.4.1-1.el6.x86_64

--> Processing Dependency: libaugeas.so.0(AUGEAS_0.10.0)(64bit) for package: ruby-augeas-0.4.1-1.el6.x86_64

--> Processing Dependency: libaugeas.so.0(AUGEAS_0.1.0)(64bit) for package: ruby-augeas-0.4.1-1.el6.x86_64

--> Processing Dependency: libaugeas.so.0()(64bit) for package: ruby-augeas-0.4.1-1.el6.x86_64

---> Package ruby-shadow.x86_64 0:1.4.1-13.el6 will be installed

--> Running transaction check

---> Package augeas-libs.x86_64 0:0.9.0-4.el6 will be installed

--> Finished Dependency Resolution

Dependencies Resolved

=========================================================================================================================================================================

Package                                      Arch                                Version                                      Repository                           Size

=========================================================================================================================================================================

Installing:

puppet                                       noarch                              2.7.23-1.el6                                 EPEL                                3.0 M

Installing for dependencies:

augeas-libs                                  x86_64                              0.9.0-4.el6                                  rhel64                              317 k

facter                                       x86_64                              1.6.18-3.el6                                 EPEL                                 62 k

libselinux-ruby                              x86_64                              2.0.94-5.3.el6                               rhel64                               99 k

ruby-augeas                                  x86_64                              0.4.1-1.el6                                  EPEL                                 21 k

ruby-shadow                                  x86_64                              1.4.1-13.el6                                 EPEL                                 11 k

Transaction Summary

=========================================================================================================================================================================

Install       6 Package(s)

Total download size: 3.5 M

Installed size: 6.5 M

Is this ok [y/N]: y

Downloading Packages:

(1/6): augeas-libs-0.9.0-4.el6.x86_64.rpm                                                                                                         | 317 kB     00:00

(2/6): facter-1.6.18-3.el6.x86_64.rpm                                                                                                             |  62 kB     00:00

(3/6): libselinux-ruby-2.0.94-5.3.el6.x86_64.rpm                                                                                                  |  99 kB     00:00

(4/6): puppet-2.7.23-1.el6.noarch.rpm                                                                                                             | 3.0 MB     00:00

(5/6): ruby-augeas-0.4.1-1.el6.x86_64.rpm                                                                                                         |  21 kB     00:00

(6/6): ruby-shadow-1.4.1-13.el6.x86_64.rpm                                                                                                        |  11 kB     00:00

-------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Total                                                                                                                                    5.5 MB/s | 3.5 MB     00:00

warning: rpmts_HdrFromFdno: Header V3 RSA/SHA256 Signature, key ID 0608b895: NOKEY

Retrieving key from http://monitoring2.tcsbank.ru/repo/epel/RPM-GPG-KEY-EPEL-6

Importing GPG key 0x0608B895:

Userid: "EPEL (6) <epel@fedoraproject.org>"

From  : http://monitoring2.tcsbank.ru/repo/epel/RPM-GPG-KEY-EPEL-6

Is this ok [y/N]: y

Running rpm_check_debug

Running Transaction Test

Transaction Test Succeeded

Running Transaction

Installing : ruby-shadow-1.4.1-13.el6.x86_64                                                                                                                       1/6

Installing : facter-1.6.18-3.el6.x86_64                                                                                                                            2/6

Installing : libselinux-ruby-2.0.94-5.3.el6.x86_64                                                                                                                 3/6

Installing : augeas-libs-0.9.0-4.el6.x86_64                                                                                                                        4/6

Installing : ruby-augeas-0.4.1-1.el6.x86_64                                                                                                                        5/6

Installing : puppet-2.7.23-1.el6.noarch                                                                                                                            6/6

Verifying  : augeas-libs-0.9.0-4.el6.x86_64                                                                                                                        1/6

Verifying  : ruby-augeas-0.4.1-1.el6.x86_64                                                                                                                        2/6

Verifying  : libselinux-ruby-2.0.94-5.3.el6.x86_64                                                                                                                 3/6

Verifying  : facter-1.6.18-3.el6.x86_64                                                                                                                            4/6

Verifying  : puppet-2.7.23-1.el6.noarch                                                                                                                            5/6

Verifying  : ruby-shadow-1.4.1-13.el6.x86_64                                                                                                                       6/6

Installed:

puppet.noarch 0:2.7.23-1.el6

Dependency Installed:

augeas-libs.x86_64 0:0.9.0-4.el6          facter.x86_64 0:1.6.18-3.el6         libselinux-ruby.x86_64 0:2.0.94-5.3.el6         ruby-augeas.x86_64 0:0.4.1-1.el6

ruby-shadow.x86_64 0:1.4.1-13.el6

Complete!

[root@ds-mon11t ~]#

[root@ds-mon11t ~]# yum list --showduplicates puppet

Loaded plugins: product-id, security, subscription-manager

This system is not registered to Red Hat Subscription Management. You can use subscription-manager to register.

Installed Packages

puppet.noarch                                                                     2.7.23-1.el6                                                                      @EPEL

Available Packages

puppet.noarch                                                                     2.7.23-1.el6                                                                      EPEL

[root@ds-mon11t ~]#

[root@ds-mon11t ~]# telnet monitoring2.tcsbank.ru 8140

Trying 192.168.1.111...

Connected to monitoring2.tcsbank.ru.

Escape character is '^]'.

asdf

sadf

Connection closed by foreign host.

[root@ds-mon11t ~]# puppet agent -t -v --noop

err: Could not retrieve catalog from remote server: getaddrinfo: Name or service not known

warning: Not using cache on failed catalog

err: Could not retrieve catalog; skipping run

err: Could not send report: getaddrinfo: Name or service not known

[root@ds-mon11t ~]#

[root@ds-mon11t yum.repos.d]# yum list duplicates puppet

Loaded plugins: product-id, security, subscription-manager

This system is not registered to Red Hat Subscription Management. You can use subscription-manager to register.

Installed Packages

puppet.noarch                                                                   2.7.23-1.el6                                                                   @EPEL

Available Packages

puppet.noarch                                                                   3.4.2-1.el6                                                                    puppetlabs

[root@ds-mon11t yum.repos.d]#

[root@ds-mon11t yum.repos.d]# yum history

Loaded plugins: product-id, security, subscription-manager

This system is not registered to Red Hat Subscription Management. You can use subscription-manager to register.

ID     | Login user               | Date and time    | Action(s)      | Altered

-------------------------------------------------------------------------------

6 | root <root>              | 2013-06-21 19:05 | Install        |    1

5 | root <root>              | 2013-06-21 18:56 | Install        |    6

4 | root <root>              | 2013-06-21 18:52 | Install        |    3

3 | root <root>              | 2013-06-21 18:05 | Install        |    2

2 | root <root>              | 2013-06-21 18:02 | Install        |    2

1 | System <unset>           | 2013-06-21 01:13 | Install        |  620

history list

[root@ds-mon11t yum.repos.d]# yum history info 5

Loaded plugins: product-id, security, subscription-manager

This system is not registered to Red Hat Subscription Management. You can use subscription-manager to register.

Transaction ID : 5

Begin time     : Fri Jun 21 18:56:06 2013

Begin rpmdb    : 627:8b1cea377f9723f0ee7845c88dcd538e1b6c8dff

End time       :            18:56:08 2013 (2 seconds)

End rpmdb      : 633:32251778fc163479dffb3bf99bcd8a652807377b

User           : root <root>

Return-Code    : Success

Command Line   : install puppet

Transaction performed with:

Installed     rpm-4.8.0-32.el6.x86_64                  @anaconda-RedHatEnterpriseLinux-201301301459.x86_64/6.4

Installed     subscription-manager-1.1.23-1.el6.x86_64 @anaconda-RedHatEnterpriseLinux-201301301459.x86_64/6.4

Installed     yum-3.2.29-40.el6.noarch                 @anaconda-RedHatEnterpriseLinux-201301301459.x86_64/6.4

Installed     yum-metadata-parser-1.1.2-16.el6.x86_64  @anaconda-RedHatEnterpriseLinux-201301301459.x86_64/6.4

Packages Altered:

Dep-Install augeas-libs-0.9.0-4.el6.x86_64        @rhel64

Dep-Install facter-1.6.18-3.el6.x86_64            @EPEL

Dep-Install libselinux-ruby-2.0.94-5.3.el6.x86_64 @rhel64

Install     puppet-2.7.23-1.el6.noarch            @EPEL

Dep-Install ruby-augeas-0.4.1-1.el6.x86_64        @EPEL

Dep-Install ruby-shadow-1.4.1-13.el6.x86_64       @EPEL

history info

[root@ds-mon11t yum.repos.d]#

[root@ds-mon11t yum.repos.d]# yum history undo 5

Loaded plugins: product-id, security, subscription-manager

This system is not registered to Red Hat Subscription Management. You can use subscription-manager to register.

Undoing transaction 5, from Fri Jun 21 18:56:06 2013

Dep-Install augeas-libs-0.9.0-4.el6.x86_64        @rhel64

Dep-Install facter-1.6.18-3.el6.x86_64            @EPEL

Dep-Install libselinux-ruby-2.0.94-5.3.el6.x86_64 @rhel64

Install     puppet-2.7.23-1.el6.noarch            @EPEL

Dep-Install ruby-augeas-0.4.1-1.el6.x86_64        @EPEL

Dep-Install ruby-shadow-1.4.1-13.el6.x86_64       @EPEL

Resolving Dependencies

--> Running transaction check

---> Package augeas-libs.x86_64 0:0.9.0-4.el6 will be erased

---> Package facter.x86_64 0:1.6.18-3.el6 will be erased

---> Package libselinux-ruby.x86_64 0:2.0.94-5.3.el6 will be erased

---> Package puppet.noarch 0:2.7.23-1.el6 will be erased

---> Package ruby-augeas.x86_64 0:0.4.1-1.el6 will be erased

---> Package ruby-shadow.x86_64 0:1.4.1-13.el6 will be erased

--> Finished Dependency Resolution

Dependencies Resolved

=========================================================================================================================================================================

Package                                      Arch                                Version                                     Repository                            Size

=========================================================================================================================================================================

Removing:

augeas-libs                                  x86_64                              0.9.0-4.el6                                 @rhel64                              981 k

facter                                       x86_64                              1.6.18-3.el6                                @EPEL                                146 k

libselinux-ruby                              x86_64                              2.0.94-5.3.el6                              @rhel64                              203 k

puppet                                       noarch                              2.7.23-1.el6                                @EPEL                                5.1 M

ruby-augeas                                  x86_64                              0.4.1-1.el6                                 @EPEL                                 45 k

ruby-shadow                                  x86_64                              1.4.1-13.el6                                @EPEL                                 15 k

Transaction Summary

=========================================================================================================================================================================

Remove        6 Package(s)

Installed size: 6.5 M

Is this ok [y/N]: y

Downloading Packages:

Running rpm_check_debug

Running Transaction Test

Transaction Test Succeeded

Running Transaction

Erasing    : puppet-2.7.23-1.el6.noarch                                                                                                                            1/6

Erasing    : ruby-augeas-0.4.1-1.el6.x86_64                                                                                                                        2/6

Erasing    : facter-1.6.18-3.el6.x86_64                                                                                                                            3/6

Erasing    : augeas-libs-0.9.0-4.el6.x86_64                                                                                                                        4/6

Erasing    : libselinux-ruby-2.0.94-5.3.el6.x86_64                                                                                                                 5/6

Erasing    : ruby-shadow-1.4.1-13.el6.x86_64                                                                                                                       6/6

Verifying  : augeas-libs-0.9.0-4.el6.x86_64                                                                                                                        1/6

Verifying  : ruby-augeas-0.4.1-1.el6.x86_64                                                                                                                        2/6

Verifying  : libselinux-ruby-2.0.94-5.3.el6.x86_64                                                                                                                 3/6

Verifying  : facter-1.6.18-3.el6.x86_64                                                                                                                            4/6

Verifying  : puppet-2.7.23-1.el6.noarch                                                                                                                            5/6

Verifying  : ruby-shadow-1.4.1-13.el6.x86_64                                                                                                                       6/6

Removed:

augeas-libs.x86_64 0:0.9.0-4.el6  facter.x86_64 0:1.6.18-3.el6 libselinux-ruby.x86_64 0:2.0.94-5.3.el6 puppet.noarch 0:2.7.23-1.el6 ruby-augeas.x86_64 0:0.4.1-1.el6

ruby-shadow.x86_64 0:1.4.1-13.el6

Complete!

[root@ds-mon11t yum.repos.d]#

[support@centos00 redhat]$ pwd

/home/support/mnt/share/redhat

[support@centos00 redhat]$

[support@centos00 redhat]$ scp rubygems-1.8.16-1.el6.noarch.rpm root@ds-mon11t:/tmp/

root@ds-mon11t's password:

rubygems-1.8.16-1.el6.noarch.rpm                                                                                                       100%  170KB 170.3KB/s   00:00

[support@centos00 redhat]$

[root@ds-mon11t yum.repos.d]# yum localinstall /tmp/rubygems-1.8.16-1.el6.noarch.rpm

Loaded plugins: product-id, security, subscription-manager

This system is not registered to Red Hat Subscription Management. You can use subscription-manager to register.

Setting up Local Package Process

Examining /tmp/rubygems-1.8.16-1.el6.noarch.rpm: rubygems-1.8.16-1.el6.noarch

Marking /tmp/rubygems-1.8.16-1.el6.noarch.rpm to be installed

Resolving Dependencies

--> Running transaction check

---> Package rubygems.noarch 0:1.8.16-1.el6 will be installed

--> Processing Dependency: ruby(abi) = 1.8 for package: rubygems-1.8.16-1.el6.noarch

--> Processing Dependency: ruby >= 1.8.7 for package: rubygems-1.8.16-1.el6.noarch

--> Processing Dependency: /usr/bin/ruby for package: rubygems-1.8.16-1.el6.noarch

--> Processing Dependency: ruby-rdoc for package: rubygems-1.8.16-1.el6.noarch

--> Running transaction check

---> Package ruby.x86_64 0:1.8.7.352-7.el6_2 will be installed

---> Package ruby-libs.x86_64 0:1.8.7.352-7.el6_2 will be installed

--> Processing Dependency: libreadline.so.5()(64bit) for package: ruby-libs-1.8.7.352-7.el6_2.x86_64

---> Package ruby-rdoc.x86_64 0:1.8.7.352-7.el6_2 will be installed

--> Processing Dependency: ruby-irb = 1.8.7.352-7.el6_2 for package: ruby-rdoc-1.8.7.352-7.el6_2.x86_64

--> Running transaction check

---> Package compat-readline5.x86_64 0:5.2-17.1.el6 will be installed

---> Package ruby-irb.x86_64 0:1.8.7.352-7.el6_2 will be installed

--> Finished Dependency Resolution

Dependencies Resolved

=========================================================================================================================================================================

Package                                Arch                         Version                                   Repository                                           Size

=========================================================================================================================================================================

Installing:

rubygems                               noarch                       1.8.16-1.el6                              /rubygems-1.8.16-1.el6.noarch                       559 k

Installing for dependencies:

compat-readline5                       x86_64                       5.2-17.1.el6                              rhel64                                              129 k

ruby                                   x86_64                       1.8.7.352-7.el6_2                         rhel64                                              532 k

ruby-irb                               x86_64                       1.8.7.352-7.el6_2                         rhel64                                              311 k

ruby-libs                              x86_64                       1.8.7.352-7.el6_2                         rhel64                                              1.6 M

ruby-rdoc                              x86_64                       1.8.7.352-7.el6_2                         rhel64                                              375 k

Transaction Summary

=========================================================================================================================================================================

Install       6 Package(s)

Total size: 3.5 M

Total download size: 3.0 M

Installed size: 11 M

Is this ok [y/N]: y

Downloading Packages:

(1/5): compat-readline5-5.2-17.1.el6.x86_64.rpm                                                                                                   | 129 kB     00:00

(2/5): ruby-1.8.7.352-7.el6_2.x86_64.rpm                                                                                                          | 532 kB     00:00

(3/5): ruby-irb-1.8.7.352-7.el6_2.x86_64.rpm                                                                                                      | 311 kB     00:00

(4/5): ruby-libs-1.8.7.352-7.el6_2.x86_64.rpm                                                                                                     | 1.6 MB     00:00

(5/5): ruby-rdoc-1.8.7.352-7.el6_2.x86_64.rpm                                                                                                     | 375 kB     00:00

-------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Total                                                                                                                                     12 MB/s | 3.0 MB     00:00

Running rpm_check_debug

Running Transaction Test

Transaction Test Succeeded

Running Transaction

Installing : compat-readline5-5.2-17.1.el6.x86_64                                                                                                                  1/6

Installing : ruby-libs-1.8.7.352-7.el6_2.x86_64                                                                                                                    2/6

Installing : ruby-1.8.7.352-7.el6_2.x86_64                                                                                                                         3/6

Installing : ruby-irb-1.8.7.352-7.el6_2.x86_64                                                                                                                     4/6

Installing : ruby-rdoc-1.8.7.352-7.el6_2.x86_64                                                                                                                    5/6

Installing : rubygems-1.8.16-1.el6.noarch                                                                                                                          6/6

Verifying  : ruby-1.8.7.352-7.el6_2.x86_64                                                                                                                         1/6

Verifying  : ruby-irb-1.8.7.352-7.el6_2.x86_64                                                                                                                     2/6

Verifying  : compat-readline5-5.2-17.1.el6.x86_64                                                                                                                  3/6

Verifying  : rubygems-1.8.16-1.el6.noarch                                                                                                                          4/6

Verifying  : ruby-libs-1.8.7.352-7.el6_2.x86_64                                                                                                                    5/6

Verifying  : ruby-rdoc-1.8.7.352-7.el6_2.x86_64                                                                                                                    6/6

Installed:

rubygems.noarch 0:1.8.16-1.el6

Dependency Installed:

compat-readline5.x86_64 0:5.2-17.1.el6       ruby.x86_64 0:1.8.7.352-7.el6_2       ruby-irb.x86_64 0:1.8.7.352-7.el6_2       ruby-libs.x86_64 0:1.8.7.352-7.el6_2

ruby-rdoc.x86_64 0:1.8.7.352-7.el6_2

Complete!

[root@ds-mon11t yum.repos.d]#

[root@ds-mon11t yum.repos.d]# yum install puppet

Loaded plugins: product-id, security, subscription-manager

This system is not registered to Red Hat Subscription Management. You can use subscription-manager to register.

Setting up Install Process

Resolving Dependencies

--> Running transaction check

---> Package puppet.noarch 0:3.4.2-1.el6 will be installed

--> Processing Dependency: facter >= 1.6.11 for package: puppet-3.4.2-1.el6.noarch

--> Processing Dependency: ruby-rgen >= 0.6.5 for package: puppet-3.4.2-1.el6.noarch

--> Processing Dependency: hiera >= 1.0.0 for package: puppet-3.4.2-1.el6.noarch

--> Processing Dependency: ruby-augeas for package: puppet-3.4.2-1.el6.noarch

--> Processing Dependency: ruby-shadow for package: puppet-3.4.2-1.el6.noarch

--> Processing Dependency: ruby(selinux) for package: puppet-3.4.2-1.el6.noarch

--> Running transaction check

---> Package facter.x86_64 1:1.7.4-1.el6 will be installed

---> Package hiera.noarch 0:1.3.1-1.el6 will be installed

--> Processing Dependency: rubygem-json for package: hiera-1.3.1-1.el6.noarch

---> Package libselinux-ruby.x86_64 0:2.0.94-5.3.el6 will be installed

---> Package ruby-augeas.x86_64 0:0.4.1-1.el6 will be installed

--> Processing Dependency: augeas-libs >= 0.8.0 for package: ruby-augeas-0.4.1-1.el6.x86_64

--> Processing Dependency: libaugeas.so.0(AUGEAS_0.8.0)(64bit) for package: ruby-augeas-0.4.1-1.el6.x86_64

--> Processing Dependency: libaugeas.so.0(AUGEAS_0.12.0)(64bit) for package: ruby-augeas-0.4.1-1.el6.x86_64

--> Processing Dependency: libaugeas.so.0(AUGEAS_0.11.0)(64bit) for package: ruby-augeas-0.4.1-1.el6.x86_64

--> Processing Dependency: libaugeas.so.0(AUGEAS_0.10.0)(64bit) for package: ruby-augeas-0.4.1-1.el6.x86_64

--> Processing Dependency: libaugeas.so.0(AUGEAS_0.1.0)(64bit) for package: ruby-augeas-0.4.1-1.el6.x86_64

--> Processing Dependency: libaugeas.so.0()(64bit) for package: ruby-augeas-0.4.1-1.el6.x86_64

---> Package ruby-rgen.noarch 0:0.6.5-1.el6 will be installed

---> Package ruby-shadow.x86_64 0:1.4.1-13.el6 will be installed

--> Running transaction check

---> Package augeas-libs.x86_64 0:0.9.0-4.el6 will be installed

---> Package rubygem-json.x86_64 0:1.5.5-1.el6 will be installed

--> Finished Dependency Resolution

Dependencies Resolved

=========================================================================================================================================================================

Package                                  Arch                            Version                                 Repository                                        Size

=========================================================================================================================================================================

Installing:

puppet                                   noarch                          3.4.2-1.el6                             puppetlabs                                       1.1 M

Installing for dependencies:

augeas-libs                              x86_64                          0.9.0-4.el6                             rhel64                                           317 k

facter                                   x86_64                          1:1.7.4-1.el6                           puppetlabs                                        87 k

hiera                                    noarch                          1.3.1-1.el6                             puppetlabs                                        23 k

libselinux-ruby                          x86_64                          2.0.94-5.3.el6                          rhel64                                            99 k

ruby-augeas                              x86_64                          0.4.1-1.el6                             EPEL                                              21 k

ruby-rgen                                noarch                          0.6.5-1.el6                             puppetlabs-dependencies                           87 k

ruby-shadow                              x86_64                          1.4.1-13.el6                            EPEL                                              11 k

rubygem-json                             x86_64                          1.5.5-1.el6                             puppetlabs-dependencies                          763 k

Transaction Summary

=========================================================================================================================================================================

Install       9 Package(s)

Total download size: 2.5 M

Installed size: 6.4 M

Is this ok [y/N]: y

Downloading Packages:

(1/9): augeas-libs-0.9.0-4.el6.x86_64.rpm                                                                                                         | 317 kB     00:00

(2/9): facter-1.7.4-1.el6.x86_64.rpm                                                                                                              |  87 kB     00:00

(3/9): hiera-1.3.1-1.el6.noarch.rpm                                                                                                               |  23 kB     00:00

(4/9): libselinux-ruby-2.0.94-5.3.el6.x86_64.rpm                                                                                                  |  99 kB     00:00

(5/9): puppet-3.4.2-1.el6.noarch.rpm                                                                                                              | 1.1 MB     00:03

(6/9): ruby-augeas-0.4.1-1.el6.x86_64.rpm                                                                                                         |  21 kB     00:00

(7/9): ruby-rgen-0.6.5-1.el6.noarch.rpm                                                                                                           |  87 kB     00:00

(8/9): ruby-shadow-1.4.1-13.el6.x86_64.rpm                                                                                                        |  11 kB     00:00

(9/9): rubygem-json-1.5.5-1.el6.x86_64.rpm                                                                                                        | 763 kB     00:02

-------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Total                                                                                                                                    290 kB/s | 2.5 MB     00:08

warning: rpmts_HdrFromFdno: Header V4 RSA/SHA512 Signature, key ID 4bd6ec30: NOKEY

Retrieving key from http://monitoring2.tcsbank.ru/repo/puppetlabs/RPM-GPG-KEY-puppetlabs

Importing GPG key 0x4BD6EC30:

Userid: "Puppet Labs Release Key (Puppet Labs Release Key) <info@puppetlabs.com>"

From  : http://monitoring2.tcsbank.ru/repo/puppetlabs/RPM-GPG-KEY-puppetlabs

Is this ok [y/N]: y

Running rpm_check_debug

Running Transaction Test

Transaction Test Succeeded

Running Transaction

Installing : ruby-shadow-1.4.1-13.el6.x86_64                                                                                                                       1/9

Installing : rubygem-json-1.5.5-1.el6.x86_64                                                                                                                       2/9

Installing : hiera-1.3.1-1.el6.noarch                                                                                                                              3/9

Installing : 1:facter-1.7.4-1.el6.x86_64                                                                                                                           4/9

Installing : ruby-rgen-0.6.5-1.el6.noarch                                                                                                                          5/9

Installing : libselinux-ruby-2.0.94-5.3.el6.x86_64                                                                                                                 6/9

Installing : augeas-libs-0.9.0-4.el6.x86_64                                                                                                                        7/9

Installing : ruby-augeas-0.4.1-1.el6.x86_64                                                                                                                        8/9

Installing : puppet-3.4.2-1.el6.noarch                                                                                                                             9/9

Verifying  : augeas-libs-0.9.0-4.el6.x86_64                                                                                                                        1/9

Verifying  : ruby-augeas-0.4.1-1.el6.x86_64                                                                                                                        2/9

Verifying  : libselinux-ruby-2.0.94-5.3.el6.x86_64                                                                                                                 3/9

Verifying  : puppet-3.4.2-1.el6.noarch                                                                                                                             4/9

Verifying  : ruby-rgen-0.6.5-1.el6.noarch                                                                                                                          5/9

Verifying  : 1:facter-1.7.4-1.el6.x86_64                                                                                                                           6/9

Verifying  : rubygem-json-1.5.5-1.el6.x86_64                                                                                                                       7/9

Verifying  : hiera-1.3.1-1.el6.noarch                                                                                                                              8/9

Verifying  : ruby-shadow-1.4.1-13.el6.x86_64                                                                                                                       9/9

Installed:

puppet.noarch 0:3.4.2-1.el6

Dependency Installed:

augeas-libs.x86_64 0:0.9.0-4.el6        facter.x86_64 1:1.7.4-1.el6           hiera.noarch 0:1.3.1-1.el6               libselinux-ruby.x86_64 0:2.0.94-5.3.el6

ruby-augeas.x86_64 0:0.4.1-1.el6        ruby-rgen.noarch 0:0.6.5-1.el6        ruby-shadow.x86_64 0:1.4.1-13.el6        rubygem-json.x86_64 0:1.5.5-1.el6

Complete!

[root@ds-mon11t yum.repos.d]#

[root@ds-mon11t ~]# chkconfig --list puppet

puppet          0:off   1:off   2:off   3:off   4:off   5:off   6:off

[root@ds-mon11t ~]# chkconfig --level 3 puppet on

[root@ds-mon11t ~]# service puppet start

Starting puppet:                                           [  OK  ]

[root@ds-mon11t ~]#

[root@ds-mon11t ~]# puppet agent -t -v --server=monitoring2.tcsbank.ru

info: Caching certificate for ca

info: Creating a new SSL certificate request for ds-mon11t.tcsbank.ru

info: Certificate Request fingerprint (md5): E9:47:1D:27:D3:34:BB:0A:F7:87:16:46:4A:11:28:27

Exiting; no certificate found and waitforcert is disabled

[root@ds-mon11t ~]#

[root@monitoring2 ~]# puppet cert --sign ds-mon11t.tcsbank.ru

Notice: Signed certificate request for ds-mon11t.tcsbank.ru

Notice: Removing file Puppet::SSL::CertificateRequest ds-mon11t.tcsbank.ru at '/var/lib/puppet/ssl/ca/requests/ds-mon11t.tcsbank.ru.pem'

[root@monitoring2 ~]#

[root@monitoring2 ~]# vim /etc/puppet/manifests/site.pp

...

node 'ds-mon11t.tcsbank.ru' {

include puppet

include accounts

realize( User[ "tan", "avk", "bsv", "che", "zbir" ] )

class { 'ntp': servers => [ 'ntp01.tcsbank.ru', 'ntp02.tcsbank.ru', 'ntp03.tcsbank.ru' ], }

}

[root@ds-mon11t ~]# puppet agent -t -v --server=monitoring2.tcsbank.ru

info: Caching certificate for ds-mon11t.tcsbank.ru

info: Caching certificate_revocation_list for ca

err: Could not retrieve catalog from remote server: SSL_connect returned=1 errno=0 state=SSLv3 read server certificate B: certificate verify failed: [CRL is not yet valid for /CN=monitoring2.tcsbank.ru]

warning: Not using cache on failed catalog

err: Could not retrieve catalog; skipping run

err: Could not send report: SSL_connect returned=1 errno=0 state=SSLv3 read server certificate B: certificate verify failed: [CRL is not yet valid for /CN=monitoring2.tcsbank.ru]

[root@ds-mon11t ~]#

[root@ds-mon11t yum.repos.d]# puppet agent -t -v --server=monitoring2.tcsbank.ru

Warning: Unable to fetch my node definition, but the agent run will continue:

Warning: SSL_connect returned=1 errno=0 state=SSLv3 read server certificate B: certificate verify failed: [CRL is not yet valid for /CN=monitoring2.tcsbank.ru]

Info: Retrieving plugin

Error: /File[/var/lib/puppet/lib]: Failed to generate additional resources using 'eval_generate': SSL_connect returned=1 errno=0 state=SSLv3 read server certificate B: certificate verify failed: [CRL is not yet valid for /CN=monitoring2.tcsbank.ru]

Error: /File[/var/lib/puppet/lib]: Could not evaluate: SSL_connect returned=1 errno=0 state=SSLv3 read server certificate B: certificate verify failed: [CRL is not yet valid for /CN=monitoring2.tcsbank.ru] Could not retrieve file metadata for puppet://monitoring2.tcsbank.ru/plugins: SSL_connect returned=1 errno=0 state=SSLv3 read server certificate B: certificate verify failed: [CRL is not yet valid for /CN=monitoring2.tcsbank.ru]

Error: Could not retrieve catalog from remote server: SSL_connect returned=1 errno=0 state=SSLv3 read server certificate B: certificate verify failed: [CRL is not yet valid for /CN=monitoring2.tcsbank.ru]

Warning: Not using cache on failed catalog

Error: Could not retrieve catalog; skipping run

Error: Could not send report: SSL_connect returned=1 errno=0 state=SSLv3 read server certificate B: certificate verify failed: [CRL is not yet valid for /CN=monitoring2.tcsbank.ru]

[root@ds-mon11t yum.repos.d]#

[root@monitoring2 ~]# puppet cert list ds-mon11t.tcsbank.ru

+ "ds-mon11t.tcsbank.ru" (SHA256) 9E:94:36:4A:98:52:39:C2:3D:25:43:DC:E3:E0:73:93:9E:65:3E:94:FB:00:1C:4A:01:CE:40:C7:F5:BA:FA:9E

[root@monitoring2 ~]# puppet cert clean ds-mon11t.tcsbank.ru

Notice: Revoked certificate with serial 607

Notice: Removing file Puppet::SSL::Certificate ds-mon11t.tcsbank.ru at '/var/lib/puppet/ssl/ca/signed/ds-mon11t.tcsbank.ru.pem'

Notice: Removing file Puppet::SSL::Certificate ds-mon11t.tcsbank.ru at '/var/lib/puppet/ssl/certs/ds-mon11t.tcsbank.ru.pem'

[root@monitoring2 ~]# puppet cert list ds-mon11t.tcsbank.ru

Error: Could not find a certificate for ds-mon11t.tcsbank.ru

[root@monitoring2 ~]#

[root@ds-mon11t yum.repos.d]# puppet agent -t -v --server=monitoring2.tcsbank.ru

Warning: Unable to fetch my node definition, but the agent run will continue:

Warning: SSL_connect returned=1 errno=0 state=SSLv3 read server certificate B: certificate verify failed: [CRL is not yet valid for /CN=monitoring2.tcsbank.ru]

Info: Retrieving plugin

Error: /File[/var/lib/puppet/lib]: Failed to generate additional resources using 'eval_generate': SSL_connect returned=1 errno=0 state=SSLv3 read server certificate B: certificate verify failed: [CRL is not yet valid for /CN=monitoring2.tcsbank.ru]

Error: /File[/var/lib/puppet/lib]: Could not evaluate: SSL_connect returned=1 errno=0 state=SSLv3 read server certificate B: certificate verify failed: [CRL is not yet valid for /CN=monitoring2.tcsbank.ru] Could not retrieve file metadata for puppet://monitoring2.tcsbank.ru/plugins: SSL_connect returned=1 errno=0 state=SSLv3 read server certificate B: certificate verify failed: [CRL is not yet valid for /CN=monitoring2.tcsbank.ru]

Error: Could not retrieve catalog from remote server: SSL_connect returned=1 errno=0 state=SSLv3 read server certificate B: certificate verify failed: [CRL is not yet valid for /CN=monitoring2.tcsbank.ru]

Warning: Not using cache on failed catalog

Error: Could not retrieve catalog; skipping run

Error: Could not send report: SSL_connect returned=1 errno=0 state=SSLv3 read server certificate B: certificate verify failed: [CRL is not yet valid for /CN=monitoring2.tcsbank.ru]

[root@ds-mon11t yum.repos.d]#

It was because time and date was wrong.

http://bitcube.co.uk/content/puppet-errors-explained

*. Check is ntpd service work correctly?

[root@ds-mon11t yum.repos.d]# service ntpd status

ntpd (pid  9071) is running...

[root@ds-mon11t yum.repos.d]#

[root@ds-mon11t yum.repos.d]# ntpq -nc pe

remote           refid      st t when poll reach   delay   offset  jitter

==============================================================================

10.219.12.7     194.190.168.1    2 u   29   64    3    4.244  45019.2   2.312

172.20.16.193   194.190.168.1    2 u   27   64    3    2.375  45016.9   1.649

172.20.16.194   172.25.0.1       3 u   25   64    3    2.360  45018.6   1.714

127.127.1.0     .LOCL.          10 l   23   64    3    0.000    0.000   0.000

[root@ds-mon11t yum.repos.d]#

*. Configure for nagios usage.

[root@ds-mon11t yum.repos.d]# yum install nagios-plugins nagios-plugins-disk nagios-plugins-load nagios-plugins-nrpe nagios-plugins-procs nagios-plugins-users nagios-plugins-load nagios-plugins-swap nrpe

Loaded plugins: product-id, security, subscription-manager

This system is not registered to Red Hat Subscription Management. You can use subscription-manager to register.

EPEL                                                                                                                                              | 4.2 kB     00:00

puppetlabs                                                                                                                                        | 1.9 kB     00:00

puppetlabs-dependencies                                                                                                                           | 1.9 kB     00:00

rhel-hp                                                                                                                                           |  951 B     00:00

rhel64                                                                                                                                            | 3.9 kB     00:00

Setting up Install Process

Resolving Dependencies

--> Running transaction check

---> Package nagios-plugins.x86_64 0:1.4.16-10.el6 will be installed

--> Processing Dependency: nagios-common >= 3.3.1-1 for package: nagios-plugins-1.4.16-10.el6.x86_64

---> Package nagios-plugins-disk.x86_64 0:1.4.16-10.el6 will be installed

---> Package nagios-plugins-load.x86_64 0:1.4.16-10.el6 will be installed

---> Package nagios-plugins-nrpe.x86_64 0:2.14-5.el6 will be installed

---> Package nagios-plugins-procs.x86_64 0:1.4.16-10.el6 will be installed

---> Package nagios-plugins-swap.x86_64 0:1.4.16-10.el6 will be installed

---> Package nagios-plugins-users.x86_64 0:1.4.16-10.el6 will be installed

---> Package nrpe.x86_64 0:2.14-5.el6 will be installed

--> Running transaction check

---> Package nagios-common.x86_64 0:3.5.1-1.el6 will be installed

--> Finished Dependency Resolution

Dependencies Resolved

=========================================================================================================================================================================

Package                                          Arch                               Version                                      Repository                        Size

=========================================================================================================================================================================

Installing:

nagios-plugins                                   x86_64                             1.4.16-10.el6                                EPEL                             200 k

nagios-plugins-disk                              x86_64                             1.4.16-10.el6                                EPEL                              69 k

nagios-plugins-load                              x86_64                             1.4.16-10.el6                                EPEL                              31 k

nagios-plugins-nrpe                              x86_64                             2.14-5.el6                                   EPEL                              23 k

nagios-plugins-procs                             x86_64                             1.4.16-10.el6                                EPEL                              67 k

nagios-plugins-swap                              x86_64                             1.4.16-10.el6                                EPEL                              32 k

nagios-plugins-users                             x86_64                             1.4.16-10.el6                                EPEL                              29 k

nrpe                                             x86_64                             2.14-5.el6                                   EPEL                             223 k

Installing for dependencies:

nagios-common                                    x86_64                             3.5.1-1.el6                                  EPEL                              17 k

Transaction Summary

=========================================================================================================================================================================

Install       9 Package(s)

Total download size: 691 k

Installed size: 1.6 M

Is this ok [y/N]: y

Downloading Packages:

(1/9): nagios-common-3.5.1-1.el6.x86_64.rpm                                                                                                       |  17 kB     00:00

(2/9): nagios-plugins-1.4.16-10.el6.x86_64.rpm                                                                                                    | 200 kB     00:00

(3/9): nagios-plugins-disk-1.4.16-10.el6.x86_64.rpm                                                                                               |  69 kB     00:00

(4/9): nagios-plugins-load-1.4.16-10.el6.x86_64.rpm                                                                                               |  31 kB     00:00

(5/9): nagios-plugins-nrpe-2.14-5.el6.x86_64.rpm                                                                                                  |  23 kB     00:00

(6/9): nagios-plugins-procs-1.4.16-10.el6.x86_64.rpm                                                                                              |  67 kB     00:00

(7/9): nagios-plugins-swap-1.4.16-10.el6.x86_64.rpm                                                                                               |  32 kB     00:00

(8/9): nagios-plugins-users-1.4.16-10.el6.x86_64.rpm                                                                                              |  29 kB     00:00

(9/9): nrpe-2.14-5.el6.x86_64.rpm                                                                                                                 | 223 kB     00:00

-------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Total                                                                                                                                    1.9 MB/s | 691 kB     00:00

Running rpm_check_debug

Running Transaction Test

Transaction Test Succeeded

Running Transaction

Installing : nagios-common-3.5.1-1.el6.x86_64                                                                                                                      1/9

Installing : nagios-plugins-1.4.16-10.el6.x86_64                                                                                                                   2/9

Installing : nagios-plugins-procs-1.4.16-10.el6.x86_64                                                                                                             3/9

Installing : nagios-plugins-swap-1.4.16-10.el6.x86_64                                                                                                              4/9

Installing : nagios-plugins-disk-1.4.16-10.el6.x86_64                                                                                                              5/9

Installing : nagios-plugins-users-1.4.16-10.el6.x86_64                                                                                                             6/9

Installing : nagios-plugins-load-1.4.16-10.el6.x86_64                                                                                                              7/9

Installing : nagios-plugins-nrpe-2.14-5.el6.x86_64                                                                                                                 8/9

Installing : nrpe-2.14-5.el6.x86_64                                                                                                                                9/9

Verifying  : nagios-plugins-1.4.16-10.el6.x86_64                                                                                                                   1/9

Verifying  : nagios-plugins-procs-1.4.16-10.el6.x86_64                                                                                                             2/9

Verifying  : nagios-plugins-swap-1.4.16-10.el6.x86_64                                                                                                              3/9

Verifying  : nagios-plugins-disk-1.4.16-10.el6.x86_64                                                                                                              4/9

Verifying  : nagios-plugins-users-1.4.16-10.el6.x86_64                                                                                                             5/9

Verifying  : nagios-plugins-load-1.4.16-10.el6.x86_64                                                                                                              6/9

Verifying  : nagios-common-3.5.1-1.el6.x86_64                                                                                                                      7/9

Verifying  : nagios-plugins-nrpe-2.14-5.el6.x86_64                                                                                                                 8/9

Verifying  : nrpe-2.14-5.el6.x86_64                                                                                                                                9/9

Installed:

nagios-plugins.x86_64 0:1.4.16-10.el6                   nagios-plugins-disk.x86_64 0:1.4.16-10.el6              nagios-plugins-load.x86_64 0:1.4.16-10.el6

nagios-plugins-nrpe.x86_64 0:2.14-5.el6                 nagios-plugins-procs.x86_64 0:1.4.16-10.el6             nagios-plugins-swap.x86_64 0:1.4.16-10.el6

nagios-plugins-users.x86_64 0:1.4.16-10.el6             nrpe.x86_64 0:2.14-5.el6

Dependency Installed:

nagios-common.x86_64 0:3.5.1-1.el6

Complete!

[root@ds-mon11t yum.repos.d]#

[root@ds-mon11t yum.repos.d]# vim /etc/nagios/nrpe.cfg

...

#allowed_hosts=127.0.0.1

allowed_hosts=127.0.0.1,nag2,monitoring1,monitoring2,monitoring3,monitoring4,monitoring5,monitoring6

[support@centos00 nrpe.d]$ scp check_* root@ds-mon11t:/etc/nrpe.d/

root@ds-mon11t's password:

check_disks.cfg                                                                                                                        100%  597     0.6KB/s   00:00

check_load.cfg                                                                                                                         100%  104     0.1KB/s   00:00

check_memory.cfg                                                                                                                       100%  126     0.1KB/s   00:00

check_nofile.cfg                                                                                                                       100%   86     0.1KB/s   00:00

check_ntp.cfg                                                                                                                          100%  469     0.5KB/s   00:00

check_procs.cfg                                                                                                                        100%  161     0.2KB/s   00:00

[support@centos00 nrpe.d]$

[support@centos00 plugins]$ scp check_* root@ds-mon11t:/usr/lib64/nagios/plugins/

root@ds-mon11t's password:

check_dbtest.sh                                                                                                                        100%  359     0.4KB/s   00:00

check_hpsa                                                                                                                             100% 4563     4.5KB/s   00:00

check_mem_linux.pl                                                                                                                     100% 8030     7.8KB/s   00:00

check_ntp_time                                                                                                                         100%   52KB  52.3KB/s   00:00

[support@centos00 plugins]$

[root@ds-mon11t nrpe.d]# chkconfig --list nrpe

nrpe            0:off   1:off   2:off   3:off   4:off   5:off   6:off

[root@ds-mon11t nrpe.d]# chkconfig --level 3 nrpe on

[root@ds-mon11t nrpe.d]# service nrpe start

Starting nrpe:                                             [  OK  ]

[root@ds-mon11t nrpe.d]#

*. Configure snmp.

[root@ds-mon11t nrpe.d]# yum install net-snmp net-snmp-utils

Loaded plugins: product-id, security, subscription-manager

This system is not registered to Red Hat Subscription Management. You can use subscription-manager to register.

Setting up Install Process

Resolving Dependencies

--> Running transaction check

---> Package net-snmp.x86_64 1:5.5-44.el6 will be installed

--> Processing Dependency: net-snmp-libs = 1:5.5-44.el6 for package: 1:net-snmp-5.5-44.el6.x86_64

--> Processing Dependency: libsensors.so.4()(64bit) for package: 1:net-snmp-5.5-44.el6.x86_64

--> Processing Dependency: libnetsnmptrapd.so.20()(64bit) for package: 1:net-snmp-5.5-44.el6.x86_64

--> Processing Dependency: libnetsnmpmibs.so.20()(64bit) for package: 1:net-snmp-5.5-44.el6.x86_64

--> Processing Dependency: libnetsnmphelpers.so.20()(64bit) for package: 1:net-snmp-5.5-44.el6.x86_64

--> Processing Dependency: libnetsnmpagent.so.20()(64bit) for package: 1:net-snmp-5.5-44.el6.x86_64

--> Processing Dependency: libnetsnmp.so.20()(64bit) for package: 1:net-snmp-5.5-44.el6.x86_64

---> Package net-snmp-utils.x86_64 1:5.5-44.el6 will be installed

--> Running transaction check

---> Package lm_sensors-libs.x86_64 0:3.1.1-17.el6 will be installed

---> Package net-snmp-libs.x86_64 1:5.5-44.el6 will be installed

--> Finished Dependency Resolution

Dependencies Resolved

=========================================================================================================================================================================

Package                                      Arch                                Version                                      Repository                           Size

=========================================================================================================================================================================

Installing:

net-snmp                                     x86_64                              1:5.5-44.el6                                 rhel64                              303 k

net-snmp-utils                               x86_64                              1:5.5-44.el6                                 rhel64                              171 k

Installing for dependencies:

lm_sensors-libs                              x86_64                              3.1.1-17.el6                                 rhel64                               38 k

net-snmp-libs                                x86_64                              1:5.5-44.el6                                 rhel64                              1.5 M

Transaction Summary

=========================================================================================================================================================================

Install       4 Package(s)

Total download size: 2.0 M

Installed size: 6.6 M

Is this ok [y/N]: y

Downloading Packages:

(1/4): lm_sensors-libs-3.1.1-17.el6.x86_64.rpm                                                                                                    |  38 kB     00:00

(2/4): net-snmp-5.5-44.el6.x86_64.rpm                                                                                                             | 303 kB     00:00

(3/4): net-snmp-libs-5.5-44.el6.x86_64.rpm                                                                                                        | 1.5 MB     00:00

(4/4): net-snmp-utils-5.5-44.el6.x86_64.rpm                                                                                                       | 171 kB     00:00

-------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Total                                                                                                                                    4.4 MB/s | 2.0 MB     00:00

Running rpm_check_debug

Running Transaction Test

Transaction Test Succeeded

Running Transaction

Installing : lm_sensors-libs-3.1.1-17.el6.x86_64                                                                                                                   1/4

Installing : 1:net-snmp-libs-5.5-44.el6.x86_64                                                                                                                     2/4

Installing : 1:net-snmp-utils-5.5-44.el6.x86_64                                                                                                                    3/4

Installing : 1:net-snmp-5.5-44.el6.x86_64                                                                                                                          4/4

Verifying  : 1:net-snmp-libs-5.5-44.el6.x86_64                                                                                                                     1/4

Verifying  : lm_sensors-libs-3.1.1-17.el6.x86_64                                                                                                                   2/4

Verifying  : 1:net-snmp-utils-5.5-44.el6.x86_64                                                                                                                    3/4

Verifying  : 1:net-snmp-5.5-44.el6.x86_64                                                                                                                          4/4

Installed:

net-snmp.x86_64 1:5.5-44.el6                                                     net-snmp-utils.x86_64 1:5.5-44.el6

Dependency Installed:

lm_sensors-libs.x86_64 0:3.1.1-17.el6                                                 net-snmp-libs.x86_64 1:5.5-44.el6

Complete!

[root@ds-mon11t nrpe.d]#

And comment access with rights with publick community.

[root@ds-mon11t nrpe.d]# vim /etc/snmp/snmpd.conf

...

###############################################################################

# Access Control

###############################################################################

rocommunity tcspub 127.0.0.1

rocommunity tcspub 172.16.1.11

rocommunity tcspub 172.16.1.14

rocommunity tcspub 172.16.1.19

trapcommunity tcspub

trapsink 172.16.1.11 tcspub

...

####

# First, map the community name "public" into a "security name"

#       sec.name  source          community

#com2sec notConfigUser  default       public

[root@ds-mon11t nrpe.d]# chkconfig --list snmpd

snmpd           0:off   1:off   2:off   3:off   4:off   5:off   6:off

[root@ds-mon11t nrpe.d]# chkconfig --level 3 snmpd on

[root@ds-mon11t nrpe.d]# chkconfig --list snmpd

snmpd           0:off   1:off   2:off   3:on    4:off   5:off   6:off

[root@ds-mon11t nrpe.d]# service snmpd start

Starting snmpd:                                            [  OK  ]

[root@ds-mon11t nrpe.d]#

*. Configure ntp.

It was already configured via puppet.

Here it’s just example of manual configuration.

[root@rhel62-template nrpe.d]# vim /etc/ntp.conf

# Use public servers from the pool.ntp.org project.

# Please consider joining the pool (http://www.pool.ntp.org/join.html).

#server 0.rhel.pool.ntp.org

#server 1.rhel.pool.ntp.org

#server 2.rhel.pool.ntp.org

server ntp01.tcsbank.ru iburst

server ntp02.tcsbank.ru iburst

server ntp03.tcsbank.ru iburst

[root@rhel62-template nrpe.d]# chkconfig ntpd on

[root@rhel62-template nrpe.d]#

[root@centos64-template yum.repos.d]# ntpdate ntp01.tcsbank.ru

21 Nov 12:53:57 ntpdate[8741]: adjust time server 10.219.12.7 offset -0.061006 sec

[root@centos64-template yum.repos.d]#

[root@centos64-template yum.repos.d]# service ntpd start

Starting ntpd:                                             [  OK  ]

[root@centos64-template yum.repos.d]# ntpq -pn

remote           refid      st t when poll reach   delay   offset  jitter

==============================================================================

10.219.12.7     194.190.168.1    2 u    2   64    1    2.678  -47.759   2.457

*172.20.16.193   194.190.168.1    2 u    1   64    1    1.470  -51.265   2.016

172.20.16.194   172.25.0.1       3 u    2   64    1    1.376  -56.136   1.419

[root@centos64-template yum.repos.d]#

*. Installing Zabbix agent.

https://www.zabbix.com/documentation/ru/2.0/manual/installation/install_from_packages

[root@ds-mon11t nrpe.d]# rpm -ivh --httpproxy nag2.tcsbank.ru --httpport 3128  http://repo.zabbix.com/zabbix/2.0/rhel/6/x86_64/zabbix-release-2.0-1.el6.noarch.rpm

Retrieving http://repo.zabbix.com/zabbix/2.0/rhel/6/x86_64/zabbix-release-2.0-1.el6.noarch.rpm

warning: /var/tmp/rpm-tmp.xwebdQ: Header V4 DSA/SHA1 Signature, key ID 79ea5ed4: NOKEY

Preparing...                ########################################### [100%]

1:zabbix-release         ########################################### [100%]

[root@ds-mon11t nrpe.d]#

Without proxy for yum we have follow.

[root@ds-mon11t nrpe.d]# yum repolist

Loaded plugins: product-id, security, subscription-manager

This system is not registered to Red Hat Subscription Management. You can use subscription-manager to register.

http://repo.zabbix.com/zabbix/2.0/rhel/6/x86_64/repodata/repomd.xml: [Errno 12] Timeout on http://repo.zabbix.com/zabbix/2.0/rhel/6/x86_64/repodata/repomd.xml: (28, 'connect() timed out!')

Trying other mirror.

repo id                                                      repo name                                                                                             status

EPEL                                                         TCS EPEL repository                                                                                   10,412

puppetlabs                                                   Puppet Labs Packages                                                                                     329

puppetlabs-dependencies                                      Puppet Labs Packages Dependencies                                                                         54

rhel-hp                                                      Red Hat Enterprise Linux HP repository 6Server - x86_64 - Source                                          35

rhel64                                                       Red Hat Enterprise Linux 6Server - x86_64 - Source                                                     3,648

zabbix                                                       Zabbix Official Repository - x86_64                                                                        0

zabbix-non-supported                                         Zabbix Official Repository non-supported - x86_64                                                          0

repolist: 14,478

[root@ds-mon11t nrpe.d]#

[root@ds-mon11t nrpe.d]# vim /etc/yum.conf

...

proxy=http://nag2.tcsbank.ru:3128

And vualla!!!

[root@ds-mon11t nrpe.d]# yum repolist

Loaded plugins: product-id, security, subscription-manager

This system is not registered to Red Hat Subscription Management. You can use subscription-manager to register.

EPEL                                                                                                                                              | 4.2 kB     00:00

puppetlabs                                                                                                                                        | 1.9 kB     00:00

puppetlabs-dependencies                                                                                                                           | 1.9 kB     00:00

rhel-hp                                                                                                                                           |  951 B     00:00

rhel64                                                                                                                                            | 3.9 kB     00:00

zabbix                                                                                                                                            |  951 B     00:00

zabbix/primary                                                                                                                                    |  34 kB     00:00

zabbix                                                                                                                                                           225/225

zabbix-non-supported                                                                                                                              |  951 B     00:00

zabbix-non-supported/primary                                                                                                                      | 2.9 kB     00:00

zabbix-non-supported                                                                                                                                               13/13

repo id                                                      repo name                                                                                             status

EPEL                                                         TCS EPEL repository                                                                                   10,412

puppetlabs                                                   Puppet Labs Packages                                                                                     329

puppetlabs-dependencies                                      Puppet Labs Packages Dependencies                                                                         54

rhel-hp                                                      Red Hat Enterprise Linux HP repository 6Server - x86_64 - Source                                          35

rhel64                                                       Red Hat Enterprise Linux 6Server - x86_64 - Source                                                     3,648

zabbix                                                       Zabbix Official Repository - x86_64                                                                      225

zabbix-non-supported                                         Zabbix Official Repository non-supported - x86_64                                                         13

repolist: 14,716

[root@ds-mon11t nrpe.d]#

[root@ds-mon11t nrpe.d]# yum install zabbix-agent

Loaded plugins: product-id, security, subscription-manager

This system is not registered to Red Hat Subscription Management. You can use subscription-manager to register.

Setting up Install Process

Resolving Dependencies

--> Running transaction check

---> Package zabbix-agent.x86_64 0:2.0.10-1.el6 will be installed

--> Processing Dependency: zabbix = 2.0.10-1.el6 for package: zabbix-agent-2.0.10-1.el6.x86_64

--> Running transaction check

---> Package zabbix.x86_64 0:2.0.10-1.el6 will be installed

--> Finished Dependency Resolution

Dependencies Resolved

=========================================================================================================================================================================

Package                                    Arch                                 Version                                      Repository                            Size

=========================================================================================================================================================================

Installing:

zabbix-agent                               x86_64                               2.0.10-1.el6                                 zabbix                               139 k

Installing for dependencies:

zabbix                                     x86_64                               2.0.10-1.el6                                 zabbix                               122 k

Transaction Summary

=========================================================================================================================================================================

Install       2 Package(s)

Total download size: 261 k

Installed size: 876 k

Is this ok [y/N]: y

Downloading Packages:

(1/2): zabbix-2.0.10-1.el6.x86_64.rpm                                                                                                             | 122 kB     00:00

(2/2): zabbix-agent-2.0.10-1.el6.x86_64.rpm                                                                                                       | 139 kB     00:00

-------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Total                                                                                                                                    331 kB/s | 261 kB     00:00

warning: rpmts_HdrFromFdno: Header V4 DSA/SHA1 Signature, key ID 79ea5ed4: NOKEY

Retrieving key from file:///etc/pki/rpm-gpg/RPM-GPG-KEY-ZABBIX

Importing GPG key 0x79EA5ED4:

Userid : Zabbix SIA <packager@zabbix.com>

Package: zabbix-release-2.0-1.el6.noarch (installed)

From   : /etc/pki/rpm-gpg/RPM-GPG-KEY-ZABBIX

Is this ok [y/N]: y

Running rpm_check_debug

Running Transaction Test

Transaction Test Succeeded

Running Transaction

Warning: RPMDB altered outside of yum.

Installing : zabbix-2.0.10-1.el6.x86_64                                                                                                                            1/2

Installing : zabbix-agent-2.0.10-1.el6.x86_64                                                                                                                      2/2

Verifying  : zabbix-2.0.10-1.el6.x86_64                                                                                                                            1/2

Verifying  : zabbix-agent-2.0.10-1.el6.x86_64                                                                                                                      2/2

Installed:

zabbix-agent.x86_64 0:2.0.10-1.el6

Dependency Installed:

zabbix.x86_64 0:2.0.10-1.el6

Complete!

[root@ds-mon11t nrpe.d]#

[root@ds-mon11t nrpe.d]# vim /etc/zabbix/zabbix_agentd.conf

...

#Server=127.0.0.1

Server=127.0.0.1,monitoring1,monitoring2,monitoring3,monitoring4,monitoring5,monitoring6

[root@ds-mon11t nrpe.d]# chkconfig --list zabbix-agent

zabbix-agent    0:off   1:off   2:off   3:off   4:off   5:off   6:off

[root@ds-mon11t nrpe.d]# chkconfig --level 3 zabbix-agent on

[root@ds-mon11t nrpe.d]# chkconfig --list zabbix-agent

zabbix-agent    0:off   1:off   2:off   3:on    4:off   5:off   6:off

[root@ds-mon11t nrpe.d]# service zabbix-agent start

Starting Zabbix agent:                                     [  OK  ]

[root@ds-mon11t nrpe.d]#

[root@ds-mon11t nrpe.d]# tail /var/log/zabbix/zabbix_agentd.log

12475:20140204:190306.022 Starting Zabbix Agent [Zabbix server]. Zabbix 2.0.10 (revision 40809).

12481:20140204:190306.026 agent #0 started [collector]

12482:20140204:190306.027 agent #1 started [listener]

12483:20140204:190306.027 agent #2 started [listener]

12484:20140204:190306.027 agent #3 started [listener]

12485:20140204:190306.027 agent #4 started [active checks]

12485:20140204:190306.028 active check configuration update from [127.0.0.1:10051] started to fail (cannot connect to [[127.0.0.1]:10051]: [111] Connection refused)

[root@ds-mon11t nrpe.d]#

*. Good idea to add time stamp to history file.

[root@ds-mon11t nrpe.d]# vim /root/.bash_profile

# Get the aliases and functions

if [ -f ~/.bashrc ]; then

. ~/.bashrc

fi

# User specific environment and startup programs

PATH=$PATH:$HOME/bin

export PATH

export HISTTIMEFORMAT='%F %T '

*. Increase number UID and GID for new created users.

Set numbers from 500 to 10000

[root@ds-mon11t nrpe.d]# vim /etc/login.defs

#

# Min/max values for automatic uid selection in useradd

#

#UID_MIN                          500

UID_MIN                 10000

UID_MAX                 60000

#

# Min/max values for automatic gid selection in groupadd

#

#GID_MIN                          500

GID_MIN                 10000

GID_MAX                 60000

*. Here is disabling iptables

Already implemented via kickstart ks.cfg file.

[root@centos64-template ~]# chkconfig iptables off

[root@centos64-template ~]#

*. disabling selinux Enforcing.

Already implemented via kickstart ks.cfg file.

[root@rhel64-template ~]# vim /etc/selinux/config

#SELINUX=enforcing

SELINUX=disabled

*. Install HP utilities on server.

[root@ds-mon11t nrpe.d]# yum install hp-ams hp-fc-enablement hp-health hp-smh-templates hp-snmp-agents hpacucli hpdiags hponcfg hpsmh hpvca

Loaded plugins: product-id, security, subscription-manager

This system is not registered to Red Hat Subscription Management. You can use subscription-manager to register.

EPEL                                                                                                                                              | 4.2 kB     00:00

puppetlabs                                                                                                                                        | 2.5 kB     00:00

puppetlabs/primary_db                                                                                                                             | 110 kB     00:00

puppetlabs-dependencies                                                                                                                           | 2.5 kB     00:00

puppetlabs-dependencies/primary_db                                                                                                                |  21 kB     00:00

rhel-hp                                                                                                                                           |  951 B     00:00

rhel64                                                                                                                                            | 3.9 kB     00:00

zabbix                                                                                                                                            |  951 B     00:00

zabbix-non-supported                                                                                                                              |  951 B     00:00

Setting up Install Process

Resolving Dependencies

--> Running transaction check

---> Package hp-ams.x86_64 0:1.1.0-604.rhel6 will be installed

---> Package hp-fc-enablement.noarch 0:1.2-9 will be installed

---> Package hp-health.x86_64 0:9.1.0.42-54.rhel6 will be installed

---> Package hp-smh-templates.noarch 0:9.1.0.33-32.all will be installed

---> Package hp-snmp-agents.x86_64 0:9.1.0.57-51.rhel6 will be installed

---> Package hpacucli.x86_64 0:9.10-22.0 will be installed

---> Package hpdiags.x86_64 0:9.1.0-301 will be installed

---> Package hponcfg.noarch 0:4.0.0-0 will be installed

---> Package hpsmh.x86_64 0:7.1.0-16 will be installed

---> Package hpvca.i386 0:7.1.0-2 will be installed

--> Finished Dependency Resolution

Dependencies Resolved

=========================================================================================================================================================================

Package                                      Arch                               Version                                       Repository                           Size

=========================================================================================================================================================================

Installing:

hp-ams                                       x86_64                             1.1.0-604.rhel6                               rhel-hp                             504 k

hp-fc-enablement                             noarch                             1.2-9                                         rhel-hp                              23 M

hp-health                                    x86_64                             9.1.0.42-54.rhel6                             rhel-hp                             812 k

hp-smh-templates                             noarch                             9.1.0.33-32.all                               rhel-hp                             3.5 M

hp-snmp-agents                               x86_64                             9.1.0.57-51.rhel6                             rhel-hp                             1.3 M

hpacucli                                     x86_64                             9.10-22.0                                     rhel-hp                             6.0 M

hpdiags                                      x86_64                             9.1.0-301                                     rhel-hp                              12 M

hponcfg                                      noarch                             4.0.0-0                                       rhel-hp                              36 k

hpsmh                                        x86_64                             7.1.0-16                                      rhel-hp                              16 M

hpvca                                        i386                               7.1.0-2                                       rhel-hp                             6.0 M

Transaction Summary

=========================================================================================================================================================================

Install      10 Package(s)

Total download size: 69 M

Installed size: 233 M

Is this ok [y/N]: y

Downloading Packages:

(1/10): hp-ams-1.1.0-604.rhel6.x86_64.rpm                                                                                                         | 504 kB     00:01

(2/10): hp-fc-enablement-1.2-9.noarch.rpm                                                                                                         |  23 MB     00:18

(3/10): hp-health-9.1.0.42-54.rhel6.x86_64.rpm                                                                                                    | 812 kB     00:03

(4/10): hp-smh-templates-9.1.0.33-32.all.noarch.rpm                                                                                               | 3.5 MB     00:05

(5/10): hp-snmp-agents-9.1.0.57-51.rhel6.x86_64.rpm                                                                                               | 1.3 MB     00:01

(6/10): hpacucli-9.10-22.0.x86_64.rpm                                                                                                             | 6.0 MB     00:07

(7/10): hpdiags-9.1.0-301.linux.x86_64.rpm                                                                                                        |  12 MB     00:09

(8/10): hponcfg-4.0.0-0.noarch.rpm                                                                                                                |  36 kB     00:00

(9/10): hpsmh-7.1.0-16.x86_64.rpm                                                                                                                 |  16 MB     00:12

(10/10): hpvca-7.1.0-2.linux.rpm                                                                                                                  | 6.0 MB     00:07

-------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Total                                                                                                                                    967 kB/s |  69 MB     01:13

Running rpm_check_debug

Running Transaction Test

Transaction Test Succeeded

Running Transaction

Installing : hpsmh-7.1.0-16.x86_64                                                                                                                                1/10

Detected Red Hat Enterprise Linux AS/ES/WS/SERVER 6

Created hpsmh user and group...

**********************************************************

* System Management Homepage installed successfully with *

* default configuration values.   To change the default  *

* configuration values,  type the following command at   *

* the root prompt:                                       *

*                                                        *

* /opt/hp/hpsmh/sbin/smhconfig                           *

*                                                        *

**********************************************************

Installing : hp-health-9.1.0.42-54.rhel6.x86_64                                                                                                                   2/10

Please read the Licence Agreement for this software at

/opt/hp/hp-health/hp-health.license

By not removing this package, you are accepting the terms

of the "HP Proliant Essentials Software End User License Agreement".

Using Proliant Standard

IPMI based System Health Monitor

Starting Proliant Standard

IPMI based System Health Monitor (hpasmlited):

OK  ]

Starting HP Advanced Server Recovery Daemon[  OK  ]

The hp-health RPM has installed successfully.

Installing : hp-snmp-agents-9.1.0.57-51.rhel6.x86_64                                                                                                              3/10

This configuration script will configure SNMP to integrate with the HP SIM and

the HP System Management Homepage by editting the snmpd.conf file. The HP-SNMP-Agents can also exist in a more secure

SNMP environment (e.g. VACM) that you have previously configured. See the

hp-snmp-agents(4) man page for specific details on how to configure the VACM entries

in the 'snmpd.conf' file. You may press <ctrl+c> now to exit now if needed.

Do you wish to use an existing snmpd.conf (y/n) (Blank is n): y

Please read the Licence Agreement for this software at

/opt/hp/hp-snmp-agents/hp-snmp-agents.license

By not removing this package, you are accepting the terms

of the "HP Proliant Essentials Software End User License Agreement".

Installing /opt/hp/hp-snmp-agents/nic/etc/HPcmanic.pp SELinux policy module

Starting Health agent (cmahealthd): [  OK  ]

Starting Standard Equipment agent (cmastdeqd): [  OK  ]

Starting Host agent (cmahostd): [  OK  ]

Starting Threshold agent (cmathreshd): [  OK  ]

Starting RIB agent (cmasm2d): [  OK  ]

Starting Performance agent (cmaperfd): [  OK  ]

Starting SNMP Peer (cmapeerd): [  OK  ]

Starting Storage Event Logger (cmaeventd): [  OK  ]

Starting FCA agent (cmafcad): [  OK  ]

Starting SAS agent (cmasasd): [  OK  ]

Starting IDA agent (cmaidad): [  OK  ]

Starting IDE agent (cmaided): [  OK  ]

Starting SCSI agent (cmascsid): [  OK  ]

Starting NIC Agent Daemon (cmanicd): [  OK  ]

WARNING:hp-snmp-agents is not configured.

You must type '/sbin/hpsnmpconfig' as 'root' user to configure agents.

Installing : hp-smh-templates-9.1.0.33-32.all.noarch                                                                                                              4/10

Please read the Licence Agreement for this software at

/opt/hp/hp-smh-templates/hp-smh-templates.license

By not removing this package, you are accepting the terms

of the "HP Proliant Essentials Software End User License Agreement".

========================================================================

NOTE: To activate the software contained in this package, you must type:

/etc/init.d/hpsmhd restart or service hpsmhd restart

as 'root' user.

=========================================================================

Installing : hpdiags-9.1.0-301.x86_64                                                                                                                             5/10

Stopping hpsmhd: [  OK  ]

Starting hpsmhd: [  OK  ]

Installing : hpvca-7.1.0-2.i386                                                                                                                                   6/10

./bin/vcasetup: error while loading shared libraries: libncurses.so.5: cannot open shared object file: No such file or directory

Starting HP Version Control Agent: [  OK  ]

==============================================================================

NOTE: Please run /opt/hp/vcagent/etc/vcaconfig.sh as 'root' user to

configure HP Version Control Agent.

==============================================================================

Installing : hp-fc-enablement-1.2-9.noarch                                                                                                                        7/10

Could not find a bfahbaapi.so for bfa driver 3.0.23.0

Could not find a compatible libdfc library for 8.3.5.86.1p

Copying libqlsdm.so

Modifying /etc/hba.conf

Adding line for x86_64 libqlsdm.so to /etc/hba.conf

Adding options qla2xxx ql2xmaxqdepth=16 ql2xloginretrycount=30 qlport_down_retry=64 to /etc/modprobe.d/fc-hba.conf

Adding options lpfc lpfc_lun_queue_depth=16 lpfc_nodev_tmo=30 lpfc_discovery_threads=32 to /etc/modprobe.d/fc-hba.conf

Creating new initrd - initramfs-2.6.32-358.el6.x86_64.img

Please restart your server for the changes to take effect

Installing : hpacucli-9.10-22.0.x86_64                                                                                                                            8/10

DOWNGRADE NOTE: To downgrade this application to any version prior to 9.10.x.x, the current RPM must be manually uninstalled using the "rpm -e" command before any prior versions can be installed.

LOCKING NOTE: The locking mechanism starting with versions 9.10.X.X, are not compatible with prior versions of the applications. Therefore, mixing older and newer versions of the various applications (ACU, HPACUCLI, HPACUSCRIPTING) is not recommended.

Installing : hp-ams-1.1.0-604.rhel6.x86_64                                                                                                                        9/10

Please read the Licence Agreement for this software at

/opt/hp/hp-ams/hp-ams.license

By not removing this package, you are accepting the terms

of the "HP Proliant Essentials Software End User License Agreement".

Running hpHelper will exhaust all available iLO devices.

Please stop hp-health, hp-snmp-agents or other consumers of iLO devices.

Installing : hponcfg-4.0.0-0.noarch                                                                                                                              10/10

Verifying  : hp-snmp-agents-9.1.0.57-51.rhel6.x86_64                                                                                                              1/10

Verifying  : hp-ams-1.1.0-604.rhel6.x86_64                                                                                                                        2/10

Verifying  : hpdiags-9.1.0-301.x86_64                                                                                                                             3/10

Verifying  : hpvca-7.1.0-2.i386                                                                                                                                   4/10

Verifying  : hp-smh-templates-9.1.0.33-32.all.noarch                                                                                                              5/10

Verifying  : hp-health-9.1.0.42-54.rhel6.x86_64                                                                                                                   6/10

Verifying  : hpacucli-9.10-22.0.x86_64                                                                                                                            7/10

Verifying  : hp-fc-enablement-1.2-9.noarch                                                                                                                        8/10

Verifying  : hpsmh-7.1.0-16.x86_64                                                                                                                                9/10

Verifying  : hponcfg-4.0.0-0.noarch                                                                                                                              10/10

Installed:

hp-ams.x86_64 0:1.1.0-604.rhel6              hp-fc-enablement.noarch 0:1.2-9    hp-health.x86_64 0:9.1.0.42-54.rhel6    hp-smh-templates.noarch 0:9.1.0.33-32.all

hp-snmp-agents.x86_64 0:9.1.0.57-51.rhel6    hpacucli.x86_64 0:9.10-22.0        hpdiags.x86_64 0:9.1.0-301              hponcfg.noarch 0:4.0.0-0

hpsmh.x86_64 0:7.1.0-16                      hpvca.i386 0:7.1.0-2

Complete!

You have mail in /var/spool/mail/root

[root@ds-mon11t nrpe.d]#

[root@ds-mon11t nrpe.d]# vim /etc/snmp/snmpd.conf

...

###############################################################################

# Access Control

###############################################################################

rocommunity tcspub 127.0.0.1

rocommunity tcspub 172.16.1.11

rocommunity tcspub 172.16.1.14

rocommunity tcspub 172.16.1.19

trapcommunity tcspub

trapsink 172.16.1.11 tcspub

[root@ds-mon11t nrpe.d]# chkconfig --list snmpd

snmpd           0:off   1:off   2:on    3:on    4:on    5:on    6:off

[root@ds-mon11t nrpe.d]# service snmpd status

snmpd (pid  12382) is running...

[root@ds-mon11t nrpe.d]#

*. Install lldpcli package.

Finding interfaces that have connected cable. Because ethtool eth2 does not show that link exist.

[support@centos00 plugins]$ scp ~/mnt/share/lldp/lldpd-0.7.3-1.1.x86_64.rpm root@ds-mon11t:/tmp/

root@ds-mon11t's password:

lldpd-0.7.3-1.1.x86_64.rpm                                                                                                             100%  179KB 178.9KB/s   00:00

[support@centos00 plugins]$

[root@ds-mon11t network-scripts]# yum localinstall /tmp/lldpd-0.7.3-1.1.x86_64.rpm

Loaded plugins: product-id, security, subscription-manager

This system is not registered to Red Hat Subscription Management. You can use subscription-manager to register.

Setting up Local Package Process

Examining /tmp/lldpd-0.7.3-1.1.x86_64.rpm: lldpd-0.7.3-1.1.x86_64

Marking /tmp/lldpd-0.7.3-1.1.x86_64.rpm to be installed

Resolving Dependencies

--> Running transaction check

---> Package lldpd.x86_64 0:0.7.3-1.1 will be installed

--> Finished Dependency Resolution

Dependencies Resolved

=========================================================================================================================================================================

Package                           Arch                               Version                                  Repository                                           Size

=========================================================================================================================================================================

Installing:

lldpd                             x86_64                             0.7.3-1.1                                /lldpd-0.7.3-1.1.x86_64                             461 k

Transaction Summary

=========================================================================================================================================================================

Install       1 Package(s)

Total size: 461 k

Installed size: 461 k

Is this ok [y/N]: y

Downloading Packages:

Running rpm_check_debug

Running Transaction Test

Transaction Test Succeeded

Running Transaction

Installing : lldpd-0.7.3-1.1.x86_64                                                                                                                                1/1

Verifying  : lldpd-0.7.3-1.1.x86_64                                                                                                                                1/1

Installed:

lldpd.x86_64 0:0.7.3-1.1

Complete!

[root@ds-mon11t network-scripts]#

[root@ds-mon11t network-scripts]# chkconfig --list lldpd

lldpd           0:off   1:off   2:on    3:on    4:on    5:on    6:off

[root@ds-mon11t network-scripts]# service lldpd start

Starting lldpd:                                            [  OK  ]

[root@ds-mon11t network-scripts]#

[root@ds-mon11t network-scripts]# lldpcli show neighbors

-------------------------------------------------------------------------------

LLDP neighbors:

-------------------------------------------------------------------------------

Interface:    eth0, via: LLDP, RID: 1, Time: 0 day, 00:00:17

Chassis:

ChassisID:    mac ac:4b:c8:6b:46:18

SysName:      DS-QF01

SysDescr:     Juniper Networks, Inc. qfx3500 , version 12.2X50.4 Build date: 2012-11-28 03:15:54 UTC

Capability:   Bridge, on

Capability:   Router, on

Port:

PortID:       local 1214775885

PortDescr:    Rack-E03:xe-0/0/25.0

-------------------------------------------------------------------------------

[root@ds-mon11t network-scripts]#

[root@ds-mon11t network-scripts]# lldpcli show neighbors details

-------------------------------------------------------------------------------

LLDP neighbors:

-------------------------------------------------------------------------------

Interface:    eth0, via: LLDP, RID: 1, Time: 0 day, 00:00:56

Chassis:

ChassisID:    mac ac:4b:c8:6b:46:18

SysName:      DS-QF01

SysDescr:     Juniper Networks, Inc. qfx3500 , version 12.2X50.4 Build date: 2012-11-28 03:15:54 UTC

Capability:   Bridge, on

Capability:   Router, on

Port:

PortID:       local 1214775885

PortDescr:    Rack-E03:xe-0/0/25.0

MFS:          9216

PMD autoneg:  supported: yes, enabled: no

MAU oper type: unknown

VLAN:         1, pvid: yes native---qfabric

VLAN:         1510 v1510---qfabric

VLAN:         1632 v1632---qfabric

VLAN:         1643 v1643---qfabric

VLAN:         1685 v1685---qfabric

-------------------------------------------------------------------------------

[root@ds-mon11t network-scripts]#

[root@ds-mon11t network-scripts]# cat ifcfg-eth1

DEVICE="eth1"

BOOTPROTO="dhcp"

DHCP_HOSTNAME="ds-mon11t"

HWADDR="2C:27:D7:53:B7:A4"

NM_CONTROLLED="yes"

ONBOOT="no"

TYPE="Ethernet"

UUID="f16ace35-9db3-4d6c-8500-10b011e63be7"

[root@ds-mon11t network-scripts]# cat ifcfg-eth2

DEVICE="eth2"

BOOTPROTO="dhcp"

DHCP_HOSTNAME="ds-mon11t"

HWADDR="2C:27:D7:53:BA:70"

NM_CONTROLLED="yes"

ONBOOT="no"

TYPE="Ethernet"

UUID="9174e05f-5f8f-4220-8aa4-a567c21b10f6"

[root@ds-mon11t network-scripts]# ifup eth1

Determining IP information for eth1... failed; no link present.  Check cable?

[root@ds-mon11t network-scripts]#

[root@ds-mon11t network-scripts]# ifup eth2

Determining IP information for eth2... failed.

[root@ds-mon11t network-scripts]# lldpcli show neighbors

-------------------------------------------------------------------------------

LLDP neighbors:

-------------------------------------------------------------------------------

Interface:    eth0, via: LLDP, RID: 1, Time: 0 day, 00:03:13

Chassis:

ChassisID:    mac ac:4b:c8:6b:46:18

SysName:      DS-QF01

SysDescr:     Juniper Networks, Inc. qfx3500 , version 12.2X50.4 Build date: 2012-11-28 03:15:54 UTC

Capability:   Bridge, on

Capability:   Router, on

Port:

PortID:       local 1214775885

PortDescr:    Rack-E03:xe-0/0/25.0

-------------------------------------------------------------------------------

Interface:    eth2, via: LLDP, RID: 1, Time: 0 day, 00:00:26

Chassis:

ChassisID:    mac ac:4b:c8:6b:46:18

SysName:      DS-QF01

SysDescr:     Juniper Networks, Inc. qfx3500 , version 12.2X50.4 Build date: 2012-11-28 03:15:54 UTC

Capability:   Bridge, on

Capability:   Router, on

Port:

PortID:       local 1214776042

PortDescr:    Rack-E04:xe-0/0/25.0

-------------------------------------------------------------------------------

[root@ds-mon11t network-scripts]#

[root@ds-mon11t network-scripts]# lldpcli show neighbors details

-------------------------------------------------------------------------------

LLDP neighbors:

-------------------------------------------------------------------------------

Interface:    eth0, via: LLDP, RID: 1, Time: 0 day, 00:03:40

Chassis:

ChassisID:    mac ac:4b:c8:6b:46:18

SysName:      DS-QF01

SysDescr:     Juniper Networks, Inc. qfx3500 , version 12.2X50.4 Build date: 2012-11-28 03:15:54 UTC

Capability:   Bridge, on

Capability:   Router, on

Port:

PortID:       local 1214775885

PortDescr:    Rack-E03:xe-0/0/25.0

MFS:          9216

PMD autoneg:  supported: yes, enabled: no

MAU oper type: unknown

VLAN:         1, pvid: yes native---qfabric

VLAN:         1510 v1510---qfabric

VLAN:         1632 v1632---qfabric

VLAN:         1643 v1643---qfabric

VLAN:         1685 v1685---qfabric

-------------------------------------------------------------------------------

Interface:    eth2, via: LLDP, RID: 1, Time: 0 day, 00:00:53

Chassis:

ChassisID:    mac ac:4b:c8:6b:46:18

SysName:      DS-QF01

SysDescr:     Juniper Networks, Inc. qfx3500 , version 12.2X50.4 Build date: 2012-11-28 03:15:54 UTC

Capability:   Bridge, on

Capability:   Router, on

Port:

PortID:       local 1214776042

PortDescr:    Rack-E04:xe-0/0/25.0

MFS:          9216

PMD autoneg:  supported: yes, enabled: no

MAU oper type: unknown

VLAN:         1, pvid: yes native---qfabric

VLAN:         1511 v1511---qfabric

VLAN:         1632 v1632---qfabric

VLAN:         1643 v1643---qfabric

VLAN:         1685 v1685---qfabric

-------------------------------------------------------------------------------

[root@ds-mon11t network-scripts]#

[root@ds-mon11t network-scripts]# ifconfig

eth0      Link encap:Ethernet  HWaddr 2C:27:D7:53:B7:A0

UP BROADCAST RUNNING MULTICAST  MTU:1500  Metric:1

RX packets:354040 errors:0 dropped:0 overruns:0 frame:0

TX packets:42282 errors:0 dropped:0 overruns:0 carrier:0

collisions:0 txqueuelen:1000

RX bytes:117974322 (112.5 MiB)  TX bytes:4297281 (4.0 MiB)

Interrupt:111

eth0.1643 Link encap:Ethernet  HWaddr 2C:27:D7:53:B7:A0

inet addr:10.216.43.43  Bcast:10.216.43.255  Mask:255.255.255.0

UP BROADCAST RUNNING MULTICAST  MTU:1500  Metric:1

RX packets:300908 errors:0 dropped:0 overruns:0 frame:0

TX packets:42260 errors:0 dropped:0 overruns:0 carrier:0

collisions:0 txqueuelen:0

RX bytes:109431511 (104.3 MiB)  TX bytes:4291649 (4.0 MiB)

eth1      Link encap:Ethernet  HWaddr 2C:27:D7:53:B7:A4

UP BROADCAST MULTICAST  MTU:1500  Metric:1

RX packets:0 errors:0 dropped:0 overruns:0 frame:0

TX packets:0 errors:0 dropped:0 overruns:0 carrier:0

collisions:0 txqueuelen:1000

RX bytes:0 (0.0 b)  TX bytes:0 (0.0 b)

Interrupt:115

eth2      Link encap:Ethernet  HWaddr 2C:27:D7:53:BA:70

UP BROADCAST RUNNING MULTICAST  MTU:1500  Metric:1

RX packets:1635 errors:0 dropped:0 overruns:0 frame:0

TX packets:24 errors:0 dropped:0 overruns:0 carrier:0

collisions:0 txqueuelen:1000

RX bytes:144472 (141.0 KiB)  TX bytes:6440 (6.2 KiB)

Interrupt:119

lo        Link encap:Local Loopback

inet addr:127.0.0.1  Mask:255.0.0.0

UP LOOPBACK RUNNING  MTU:16436  Metric:1

RX packets:5644 errors:0 dropped:0 overruns:0 frame:0

TX packets:5644 errors:0 dropped:0 overruns:0 carrier:0

collisions:0 txqueuelen:0

RX bytes:386226 (377.1 KiB)  TX bytes:386226 (377.1 KiB)

[root@ds-mon11t network-scripts]# ifdown eth1

[root@ds-mon11t network-scripts]# ifdown eth2

[root@ds-mon11t network-scripts]#

*. Configure bonding for host.

Check that everything is OK.

[root@ds-mon11t network-scripts]# cat /etc/sysconfig/network

NETWORKING=yes

HOSTNAME=ds-mon11t

GATEWAY=10.216.43.254

NM_BOND_VLAN_ENABLED=yes

NETWORKING_IPV6=no

[root@ds-mon11t network-scripts]#

[root@ds-mon11t network-scripts]# cat /etc/resolv.conf

# Generated by NetworkManager

search tcsbank.ru

domain tcsbank.ru

nameserver 10.216.42.5

nameserver 10.216.40.4

[root@ds-mon11t network-scripts]#

Before configuration it was follow.

[root@ds-mon11t ~]# cat ~/backup/20140105-1107/ifcfg-eth0

DEVICE="eth0"

#BOOTPROTO="static"

#DNS1="10.216.42.5"

#DNS2="10.216.40.4"

#GATEWAY="10.216.43.254"

HWADDR="2C:27:D7:53:B7:A0"

#IPADDR="10.216.43.43"

IPV6INIT="no"

#NETMASK="255.255.255.0"

NM_CONTROLLED="yes"

ONBOOT="yes"

TYPE="Ethernet"

UUID="7461ba57-80f6-49cc-9256-4849967aaec8"

IPV6_AUTOCONF=no

[root@ds-mon11t ~]#

[root@ds-mon11t ~]# cat ~/backup/20140105-1107/ifcfg-eth0.1643

DEVICE="eth0.1643"

BOOTPROTO="static"

DNS1="10.216.42.5"

DNS2="10.216.40.4"

GATEWAY="10.216.43.254"

HOSTNAME="ds-mon11t"

IPADDR="10.216.43.43"

MTU="1500"

NETMASK="255.255.255.0"

NM_CONTROLLED="yes"

ONBOOT="yes"

TYPE="Vlan"

UUID="066cbd9e-d1d8-4009-97ec-f24ffa02b949"

VLAN="yes"

IPV6_AUTOCONF=no

IPV6INIT=no

[root@ds-mon11t ~]#

[root@ds-mon11t network-scripts]# vim /etc/modprobe.d/bonding.conf

alias netdev-bond0 bonding

options bonding miimon=100

[root@ds-mon11t network-scripts]# mkdir -p ~/backup/20140105-1107

[root@ds-mon11t network-scripts]# cp ifcfg-eth* !$

cp ifcfg-eth* ~/backup/20140105-1107

[root@ds-mon11t network-scripts]# ls !$

ls ~/backup/20140105-1107

ifcfg-eth0  ifcfg-eth0.1643  ifcfg-eth1  ifcfg-eth2  ifcfg-eth3  ifcfg-eth4  ifcfg-eth5

[root@ds-mon11t network-scripts]# cp ifcfg-eth0.1643 ifcfg-bond0

[root@ds-mon11t network-scripts]#

[root@ds-mon11t ~]# cat /etc/sysconfig/network-scripts/ifcfg-bond0

DEVICE="bond0"

BOOTPROTO="none"

IPADDR="10.216.43.43"

NETMASK="255.255.255.0"

NM_CONTROLLED="yes"

ONBOOT="yes"

IPV6_AUTOCONF=no

IPV6INIT=no

USERCTL=no

NOZEROCONF=yes

BONDING_OPTS="mode=1 miimon=100"

[root@ds-mon11t ~]#

[root@ds-mon11t network-scripts]# cp ifcfg-eth0.1643 ifcfg-eth2.1643

[root@ds-mon11t network-scripts]#

[root@ds-mon11t ~]# cat /etc/sysconfig/network-scripts/ifcfg-eth0.1643

DEVICE="eth0.1643"

BOOTPROTO="none"

NM_CONTROLLED="yes"

ONBOOT="yes"

TYPE="Vlan"

VLAN="yes"

SLAVE=yes

MASTER=bond0

NOZEROCONF=yes

IPV6_AUTOCONF=no

IPV6INIT=no

[root@ds-mon11t ~]#

[root@ds-mon11t ~]# cat /etc/sysconfig/network-scripts/ifcfg-eth2.1643

DEVICE="eth2.1643"

BOOTPROTO="none"

NM_CONTROLLED="yes"

ONBOOT="yes"

TYPE="Vlan"

VLAN="yes"

SLAVE=yes

MASTER=bond0

NOZEROCONF=yes

IPV6_AUTOCONF=no

IPV6INIT=no

[root@ds-mon11t ~]#

[root@ds-mon11t ~]# cat /etc/sysconfig/network-scripts/ifcfg-eth0

DEVICE="eth0"

BOOTPROTO="none"

HWADDR="2C:27:D7:53:B7:A0"

IPV6INIT="no"

NM_CONTROLLED="yes"

ONBOOT="yes"

TYPE="Ethernet"

UUID="7461ba57-80f6-49cc-9256-4849967aaec8"

IPV6_AUTOCONF=no

NOZEROCONF=yes

[root@ds-mon11t ~]#

[root@ds-mon11t ~]# cat /etc/sysconfig/network-scripts/ifcfg-eth2

DEVICE="eth2"

BOOTPROTO="none"

HWADDR="2C:27:D7:53:BA:70"

IPV6INIT="no"

NM_CONTROLLED="yes"

ONBOOT="yes"

TYPE="Ethernet"

UUID="9174e05f-5f8f-4220-8aa4-a567c21b10f6"

IPV6_AUTOCONF=no

NOZEROCONF=yes

[root@ds-mon11t ~]#

Strange thing...

[root@ds-mon11t ~]# cat /etc/modprobe.d/fc-hba.conf

options qla2xxx ql2xmaxqdepth=16 ql2xloginretrycount=30 qlport_down_retry=64

options lpfc lpfc_lun_queue_depth=16 lpfc_nodev_tmo=30 lpfc_discovery_threads=32

[root@ds-mon11t ~]# cat /etc/modprobe.d/fc-hba.conf.old

[root@ds-mon11t ~]#

##### I just have removed empty file.

[root@ds-mon11t ~]# rm /etc/modprobe.d/fc-hba.conf.old

rm: remove regular empty file `/etc/modprobe.d/fc-hba.conf.old'? y

[root@ds-mon11t ~]#

[root@ds-mon11t ~]# tail -n 30 /var/log/messages

...

Feb  5 11:45:01 ds-mon11t kernel: bonding: unable to remove non-existent slave eth0.1643 for bond bond0.

Feb  5 11:45:01 ds-mon11t kernel: bonding: unable to remove non-existent slave eth2.1643 for bond bond0.

Feb  5 11:45:02 ds-mon11t lldpd[28250]: error while receiving frame on eth0: Network is down

Feb  5 11:45:03 ds-mon11t kernel: bonding: bond0: setting mode to active-backup (1).

Feb  5 11:45:03 ds-mon11t kernel: bonding: bond0: Setting MII monitoring interval to 100.

Feb  5 11:45:03 ds-mon11t kernel: bonding: bond0: setting mode to active-backup (1).

Feb  5 11:45:03 ds-mon11t kernel: bonding: bond0: Setting MII monitoring interval to 100.

Feb  5 11:45:03 ds-mon11t kernel: bonding: bond0: Adding slave eth0.1643.

Feb  5 11:45:03 ds-mon11t kernel: New slave device eth0.1643 does not support netpoll

Feb  5 11:45:03 ds-mon11t kernel: Disabling netpoll support for bond0

Feb  5 11:45:03 ds-mon11t kernel: bonding: bond0: enslaving eth0.1643 as a backup interface with a down link.

Feb  5 11:45:04 ds-mon11t kernel: bonding: bond0: Adding slave eth2.1643.

Feb  5 11:45:04 ds-mon11t kernel: device eth2 entered promiscuous mode

Feb  5 11:45:04 ds-mon11t kernel: bonding: bond0: enslaving eth2.1643 as a backup interface with a down link.

Feb  5 11:45:04 ds-mon11t kernel: 8021q: adding VLAN 0 to HW filter on device bond0

Feb  5 11:45:04 ds-mon11t kernel: qlcnic 0000:07:00.0: eth0: NIC Link is up

Feb  5 11:45:04 ds-mon11t kernel: qlcnic 0000:24:00.0: eth2: NIC Link is up

Feb  5 11:45:04 ds-mon11t kernel: bond0: link status definitely up for interface eth0.1643, 10000 Mbps full duplex.

Feb  5 11:45:04 ds-mon11t kernel: bonding: bond0: making interface eth0.1643 the new active one.

Feb  5 11:45:04 ds-mon11t kernel: bonding: bond0: first active interface up!

Feb  5 11:45:04 ds-mon11t kernel: bond0: link status definitely up for interface eth2.1643, 10000 Mbps full duplex.

Feb  5 11:45:04 ds-mon11t ntpd[9071]: Deleting interface #3 bond0, 10.216.43.43#123, interface stats: received=0, sent=0, dropped=0, active_time=665 secs

Feb  5 11:45:09 ds-mon11t ntpd[9071]: Listening on interface #4 bond0, 10.216.43.43#123 Enabled

[root@ds-mon11t ~]#

Here I checked how network restarts.

We see - it works very well!

[root@ds-mon11t ~]# tail -n 30 /var/log/messages

...

Feb  5 12:03:29 ds-mon11t kernel: bonding: bond0: Removing slave eth0.1643

Feb  5 12:03:29 ds-mon11t kernel: bonding: bond0: Warning: the permanent HWaddr of eth0.1643 - 2c:27:d7:53:b7:a0 - is still in use by bond0. Set the HWaddr of eth0.1643 to a different address to avoid conflicts.

Feb  5 12:03:29 ds-mon11t kernel: bonding: bond0: releasing active interface eth0.1643

Feb  5 12:03:29 ds-mon11t kernel: bonding: bond0: making interface eth2.1643 the new active one.

Feb  5 12:03:30 ds-mon11t kernel: bonding: bond0: Removing slave eth2.1643

Feb  5 12:03:30 ds-mon11t kernel: bonding: bond0: releasing active interface eth2.1643

Feb  5 12:03:30 ds-mon11t kernel: device eth2 left promiscuous mode

Feb  5 12:03:32 ds-mon11t lldpd[28250]: error while receiving frame on eth0: Network is down

Feb  5 12:03:32 ds-mon11t lldpd[28250]: error while receiving frame on eth2: Network is down

Feb  5 12:03:32 ds-mon11t ntpd[9071]: Deleting interface #4 bond0, 10.216.43.43#123, interface stats: received=3, sent=3, dropped=0, active_time=1103 secs

Feb  5 12:03:33 ds-mon11t kernel: bonding: bond0: setting mode to active-backup (1).

Feb  5 12:03:33 ds-mon11t kernel: bonding: bond0: Setting MII monitoring interval to 100.

Feb  5 12:03:33 ds-mon11t kernel: bonding: bond0: setting mode to active-backup (1).

Feb  5 12:03:33 ds-mon11t kernel: bonding: bond0: Setting MII monitoring interval to 100.

Feb  5 12:03:33 ds-mon11t kernel: bonding: bond0: Adding slave eth0.1643.

Feb  5 12:03:33 ds-mon11t kernel: bonding: bond0: enslaving eth0.1643 as a backup interface with a down link.

Feb  5 12:03:34 ds-mon11t kernel: bonding: bond0: Adding slave eth2.1643.

Feb  5 12:03:34 ds-mon11t kernel: device eth2 entered promiscuous mode

Feb  5 12:03:34 ds-mon11t kernel: bonding: bond0: enslaving eth2.1643 as a backup interface with a down link.

Feb  5 12:03:34 ds-mon11t kernel: qlcnic 0000:07:00.0: eth0: NIC Link is up

Feb  5 12:03:34 ds-mon11t kernel: 8021q: adding VLAN 0 to HW filter on device bond0

Feb  5 12:03:34 ds-mon11t kernel: bond0: link status definitely up for interface eth0.1643, 10000 Mbps full duplex.

Feb  5 12:03:34 ds-mon11t kernel: bonding: bond0: making interface eth0.1643 the new active one.

Feb  5 12:03:34 ds-mon11t kernel: bonding: bond0: first active interface up!

Feb  5 12:03:34 ds-mon11t kernel: qlcnic 0000:24:00.0: eth2: NIC Link is up

Feb  5 12:03:34 ds-mon11t kernel: bond0: link status definitely up for interface eth2.1643, 10000 Mbps full duplex.

Feb  5 12:03:39 ds-mon11t ntpd[9071]: Listening on interface #5 bond0, 10.216.43.43#123 Enabled

[root@ds-mon11t ~]#

Check how right work bonding with active backup mode. Everythig works well. The ssh link does not broking.

[root@ds-mon11t ~]# ifdown eth2

[root@ds-mon11t ~]# ifconfig

bond0     Link encap:Ethernet  HWaddr 2C:27:D7:53:B7:A0

inet addr:10.216.43.43  Bcast:10.216.43.255  Mask:255.255.255.0

UP BROADCAST RUNNING MASTER MULTICAST  MTU:1500  Metric:1

RX packets:6366 errors:0 dropped:0 overruns:0 frame:0

TX packets:102 errors:0 dropped:0 overruns:0 carrier:0

collisions:0 txqueuelen:0

RX bytes:1333705 (1.2 MiB)  TX bytes:15158 (14.8 KiB)

eth0      Link encap:Ethernet  HWaddr 2C:27:D7:53:B7:A0

UP BROADCAST RUNNING MULTICAST  MTU:1500  Metric:1

RX packets:371909 errors:0 dropped:0 overruns:0 frame:0

TX packets:44131 errors:0 dropped:0 overruns:0 carrier:0

collisions:0 txqueuelen:1000

RX bytes:119550286 (114.0 MiB)  TX bytes:4643077 (4.4 MiB)

Interrupt:111

eth0.1643 Link encap:Ethernet  HWaddr 2C:27:D7:53:B7:A0

UP BROADCAST RUNNING SLAVE MULTICAST  MTU:1500  Metric:1

RX packets:1561 errors:0 dropped:0 overruns:0 frame:0

TX packets:102 errors:0 dropped:0 overruns:0 carrier:0

collisions:0 txqueuelen:0

RX bytes:109933 (107.3 KiB)  TX bytes:15158 (14.8 KiB)

lo        Link encap:Local Loopback

inet addr:127.0.0.1  Mask:255.0.0.0

UP LOOPBACK RUNNING  MTU:16436  Metric:1

RX packets:9835 errors:0 dropped:0 overruns:0 frame:0

TX packets:9835 errors:0 dropped:0 overruns:0 carrier:0

collisions:0 txqueuelen:0

RX bytes:693606 (677.3 KiB)  TX bytes:693606 (677.3 KiB)

[root@ds-mon11t ~]# ifup eth2.1643

[root@ds-mon11t ~]# ifconfig

bond0     Link encap:Ethernet  HWaddr 2C:27:D7:53:B7:A0

inet addr:10.216.43.43  Bcast:10.216.43.255  Mask:255.255.255.0

UP BROADCAST RUNNING MASTER MULTICAST  MTU:1500  Metric:1

RX packets:6542 errors:0 dropped:0 overruns:0 frame:0

TX packets:141 errors:0 dropped:0 overruns:0 carrier:0

collisions:0 txqueuelen:0

RX bytes:1362018 (1.2 MiB)  TX bytes:20800 (20.3 KiB)

eth0      Link encap:Ethernet  HWaddr 2C:27:D7:53:B7:A0

UP BROADCAST RUNNING MULTICAST  MTU:1500  Metric:1

RX packets:372028 errors:0 dropped:0 overruns:0 frame:0

TX packets:44171 errors:0 dropped:0 overruns:0 carrier:0

collisions:0 txqueuelen:1000

RX bytes:119560297 (114.0 MiB)  TX bytes:4648975 (4.4 MiB)

Interrupt:111

eth0.1643 Link encap:Ethernet  HWaddr 2C:27:D7:53:B7:A0

UP BROADCAST RUNNING SLAVE MULTICAST  MTU:1500  Metric:1

RX packets:1655 errors:0 dropped:0 overruns:0 frame:0

TX packets:141 errors:0 dropped:0 overruns:0 carrier:0

collisions:0 txqueuelen:0

RX bytes:116415 (113.6 KiB)  TX bytes:20800 (20.3 KiB)

eth2      Link encap:Ethernet  HWaddr 2C:27:D7:53:BA:70

UP BROADCAST RUNNING MULTICAST  MTU:1500  Metric:1

RX packets:29097 errors:0 dropped:0 overruns:0 frame:0

TX packets:94 errors:0 dropped:0 overruns:0 carrier:0

collisions:0 txqueuelen:1000

RX bytes:5249759 (5.0 MiB)  TX bytes:23896 (23.3 KiB)

Interrupt:119

eth2.1643 Link encap:Ethernet  HWaddr 2C:27:D7:53:B7:A0

UP BROADCAST RUNNING SLAVE MULTICAST  MTU:1500  Metric:1

RX packets:4887 errors:0 dropped:0 overruns:0 frame:0

TX packets:0 errors:0 dropped:0 overruns:0 carrier:0

collisions:0 txqueuelen:0

RX bytes:1245603 (1.1 MiB)  TX bytes:0 (0.0 b)

lo        Link encap:Local Loopback

inet addr:127.0.0.1  Mask:255.0.0.0

UP LOOPBACK RUNNING  MTU:16436  Metric:1

RX packets:9851 errors:0 dropped:0 overruns:0 frame:0

TX packets:9851 errors:0 dropped:0 overruns:0 carrier:0

collisions:0 txqueuelen:0

RX bytes:694744 (678.4 KiB)  TX bytes:694744 (678.4 KiB)

[root@ds-mon11t ~]# ifdown eth0

[root@ds-mon11t ~]# ifconfig

bond0     Link encap:Ethernet  HWaddr 2C:27:D7:53:B7:A0

inet addr:10.216.43.43  Bcast:10.216.43.255  Mask:255.255.255.0

UP BROADCAST RUNNING MASTER MULTICAST  MTU:1500  Metric:1

RX packets:6780 errors:0 dropped:0 overruns:0 frame:0

TX packets:170 errors:0 dropped:1 overruns:0 carrier:0

collisions:0 txqueuelen:0

RX bytes:1413860 (1.3 MiB)  TX bytes:26098 (25.4 KiB)

eth2      Link encap:Ethernet  HWaddr 2C:27:D7:53:BA:70

UP BROADCAST RUNNING MULTICAST  MTU:1500  Metric:1

RX packets:29402 errors:0 dropped:0 overruns:0 frame:0

TX packets:108 errors:0 dropped:0 overruns:0 carrier:0

collisions:0 txqueuelen:1000

RX bytes:5308881 (5.0 MiB)  TX bytes:25360 (24.7 KiB)

Interrupt:119

eth2.1643 Link encap:Ethernet  HWaddr 2C:27:D7:53:B7:A0

UP BROADCAST RUNNING SLAVE MULTICAST  MTU:1500  Metric:1

RX packets:5077 errors:0 dropped:0 overruns:0 frame:0

TX packets:14 errors:0 dropped:0 overruns:0 carrier:0

collisions:0 txqueuelen:0

RX bytes:1294021 (1.2 MiB)  TX bytes:1464 (1.4 KiB)

lo        Link encap:Local Loopback

inet addr:127.0.0.1  Mask:255.0.0.0

UP LOOPBACK RUNNING  MTU:16436  Metric:1

RX packets:9865 errors:0 dropped:0 overruns:0 frame:0

TX packets:9865 errors:0 dropped:0 overruns:0 carrier:0

collisions:0 txqueuelen:0

RX bytes:695782 (679.4 KiB)  TX bytes:695782 (679.4 KiB)

[root@ds-mon11t ~]# ifup eth0.1643

[root@ds-mon11t ~]# ifconfig

bond0     Link encap:Ethernet  HWaddr 2C:27:D7:53:B7:A0

inet addr:10.216.43.43  Bcast:10.216.43.255  Mask:255.255.255.0

UP BROADCAST RUNNING MASTER MULTICAST  MTU:1500  Metric:1

RX packets:6951 errors:0 dropped:0 overruns:0 frame:0

TX packets:197 errors:0 dropped:1 overruns:0 carrier:0

collisions:0 txqueuelen:0

RX bytes:1447739 (1.3 MiB)  TX bytes:30484 (29.7 KiB)

eth0      Link encap:Ethernet  HWaddr 2C:27:D7:53:B7:A0

UP BROADCAST RUNNING MULTICAST  MTU:1500  Metric:1

RX packets:372093 errors:0 dropped:0 overruns:0 frame:0

TX packets:44187 errors:0 dropped:0 overruns:0 carrier:0

collisions:0 txqueuelen:1000

RX bytes:119566039 (114.0 MiB)  TX bytes:4653065 (4.4 MiB)

Interrupt:111

eth0.1643 Link encap:Ethernet  HWaddr 2C:27:D7:53:B7:A0

UP BROADCAST RUNNING SLAVE MULTICAST  MTU:1500  Metric:1

RX packets:1718 errors:0 dropped:0 overruns:0 frame:0

TX packets:156 errors:0 dropped:1 overruns:0 carrier:0

collisions:0 txqueuelen:0

RX bytes:120903 (118.0 KiB)  TX bytes:24634 (24.0 KiB)

eth2      Link encap:Ethernet  HWaddr 2C:27:D7:53:BA:70

UP BROADCAST RUNNING MULTICAST  MTU:1500  Metric:1

RX packets:29644 errors:0 dropped:0 overruns:0 frame:0

TX packets:137 errors:0 dropped:0 overruns:0 carrier:0

collisions:0 txqueuelen:1000

RX bytes:5350233 (5.1 MiB)  TX bytes:30258 (29.5 KiB)

Interrupt:119

eth2.1643 Link encap:Ethernet  HWaddr 2C:27:D7:53:B7:A0

UP BROADCAST RUNNING SLAVE MULTICAST  MTU:1500  Metric:1

RX packets:5233 errors:0 dropped:0 overruns:0 frame:0

TX packets:41 errors:0 dropped:0 overruns:0 carrier:0

collisions:0 txqueuelen:0

RX bytes:1326836 (1.2 MiB)  TX bytes:5850 (5.7 KiB)

lo        Link encap:Local Loopback

inet addr:127.0.0.1  Mask:255.0.0.0

UP LOOPBACK RUNNING  MTU:16436  Metric:1

RX packets:9879 errors:0 dropped:0 overruns:0 frame:0

TX packets:9879 errors:0 dropped:0 overruns:0 carrier:0

collisions:0 txqueuelen:0

RX bytes:696820 (680.4 KiB)  TX bytes:696820 (680.4 KiB)

[root@ds-mon11t ~]#

[root@ds-mon11t ~]# tail -n 50 /var/log/messages

...

Feb  5 12:11:42 ds-mon11t lldpd[28250]: error while receiving frame on eth2: Network is down

Feb  5 12:11:43 ds-mon11t kernel: bonding: bond0: link status definitely down for interface eth2.1643, disabling it

Feb  5 12:11:59 ds-mon11t kernel: device eth2 left promiscuous mode

Feb  5 12:11:59 ds-mon11t kernel: device eth2 entered promiscuous mode

Feb  5 12:11:59 ds-mon11t kernel: qlcnic 0000:24:00.0: eth2: NIC Link is up

Feb  5 12:11:59 ds-mon11t kernel: bond0: link status definitely up for interface eth2.1643, 10000 Mbps full duplex.

Feb  5 12:12:18 ds-mon11t lldpd[28250]: error while receiving frame on eth0: Network is down

Feb  5 12:12:18 ds-mon11t kernel: bonding: bond0: link status definitely down for interface eth0.1643, disabling it

Feb  5 12:12:18 ds-mon11t kernel: bonding: bond0: making interface eth2.1643 the new active one.

Feb  5 12:12:37 ds-mon11t kernel: qlcnic 0000:07:00.0: eth0: NIC Link is up

Feb  5 12:12:37 ds-mon11t kernel: bond0: link status definitely up for interface eth0.1643, 10000 Mbps full duplex.

[root@ds-mon11t ~]#

[root@ds-mon11t ~]# ethtool eth0

Settings for eth0:

Supported ports: [ TP FIBRE ]

Supported link modes:   10000baseT/Full

Supported pause frame use: No

Supports auto-negotiation: No

Advertised link modes:  10000baseT/Full

Advertised pause frame use: No

Advertised auto-negotiation: No

Speed: 10000Mb/s

Duplex: Full

Port: Twisted Pair

PHYAD: 0

Transceiver: external

Auto-negotiation: off

MDI-X: Unknown

Supports Wake-on: g

Wake-on: g

Current message level: 0x00000000 (0)

Link detected: yes

[root@ds-mon11t ~]# ethtool eth2

Settings for eth2:

Supported ports: [ TP FIBRE ]

Supported link modes:   10000baseT/Full

Supported pause frame use: No

Supports auto-negotiation: No

Advertised link modes:  10000baseT/Full

Advertised pause frame use: No

Advertised auto-negotiation: No

Speed: 10000Mb/s

Duplex: Full

Port: Twisted Pair

PHYAD: 0

Transceiver: external

Auto-negotiation: off

MDI-X: Unknown

Supports Wake-on: g

Wake-on: g

Current message level: 0x00000000 (0)

Link detected: yes

[root@ds-mon11t ~]# ethtool -i eth0

driver: qlcnic

version: 5.0.29

firmware-version: 4.14.26

bus-info: 0000:07:00.0

supports-statistics: yes

supports-test: yes

supports-eeprom-access: yes

supports-register-dump: yes

supports-priv-flags: no

[root@ds-mon11t ~]# ethtool -i eth2

driver: qlcnic

version: 5.0.29

firmware-version: 4.14.26

bus-info: 0000:24:00.0

supports-statistics: yes

supports-test: yes

supports-eeprom-access: yes

supports-register-dump: yes

supports-priv-flags: no

[root@ds-mon11t ~]#

*. Tune BIOS configuration for server chassi and for raid controller to optimal performance.

See how was configured cache on raid controller. Tune it to maximum performace.

[root@ds-mon11t ~]# hpacucli ctrl all show detail

Smart Array P420i in Slot 0 (Embedded)

Bus Interface: PCI

Slot: 0

Serial Number: 001438024105D10

Cache Serial Number: PBKUD0ARH2Y1QU

RAID 6 (ADG) Status: Enabled

Controller Status: OK

Hardware Revision: B

Firmware Version: 4.68

Rebuild Priority: Medium

Expand Priority: Medium

Surface Scan Delay: 15 secs

Surface Scan Mode: Idle

Queue Depth: Automatic

Monitor and Performance Delay: 60  min

Elevator Sort: Enabled

Degraded Performance Optimization: Disabled

Inconsistency Repair Policy: Disabled

Wait for Cache Room: Disabled

Surface Analysis Inconsistency Notification: Disabled

Post Prompt Timeout: 0 secs

Cache Board Present: True

Cache Status: OK

Cache Ratio: 10% Read / 90% Write

Drive Write Cache: Disabled

Total Cache Size: 2.0 GB

Total Cache Memory Available: 1.8 GB

No-Battery Write Cache: Disabled

Cache Backup Power Source: Capacitors

Battery/Capacitor Count: 1

Battery/Capacitor Status: OK

SATA NCQ Supported: True

Spare Activation Mode: Activate on drive failure

Controller Temperature (C): 63

Cache Module Temperature (C): 35

Capacitor Temperature  (C): 24

[root@ds-mon11t ~]#

[root@ds-mon11t backup]# hpacucli ctrl slot=0 modify dwc=enable

Warning: Without the proper safety precautions, use of write cache on physical

drives could cause data loss in the event of power failure.  To ensure

data is properly protected, use redundant power supplies and

Uninterruptible Power Supplies. Also, if you have multiple storage

enclosures, all data should be mirrored across them. Use of this

feature is not recommended unless these precautions are followed.

Continue? (y/n) y

[root@ds-mon11t backup]# hpacucli ctrl all show detail

Smart Array P420i in Slot 0 (Embedded)

Bus Interface: PCI

Slot: 0

Serial Number: 001438024105D10

Cache Serial Number: PBKUD0ARH2Y1QU

RAID 6 (ADG) Status: Enabled

Controller Status: OK

Hardware Revision: B

Firmware Version: 4.68

Rebuild Priority: Medium

Expand Priority: Medium

Surface Scan Delay: 15 secs

Surface Scan Mode: Idle

Queue Depth: Automatic

Monitor and Performance Delay: 60  min

Elevator Sort: Enabled

Degraded Performance Optimization: Disabled

Inconsistency Repair Policy: Disabled

Wait for Cache Room: Disabled

Surface Analysis Inconsistency Notification: Disabled

Post Prompt Timeout: 0 secs

Cache Board Present: True

Cache Status: OK

Cache Ratio: 10% Read / 90% Write

Drive Write Cache: Enabled

Total Cache Size: 2.0 GB

Total Cache Memory Available: 1.8 GB

No-Battery Write Cache: Disabled

Cache Backup Power Source: Capacitors

Battery/Capacitor Count: 1

Battery/Capacitor Status: OK

SATA NCQ Supported: True

Spare Activation Mode: Activate on drive failure

Controller Temperature (C): 62

Cache Module Temperature (C): 35

Capacitor Temperature  (C): 24

[root@ds-mon11t backup]#

[support@centos00 plugins]$ scp ~/mnt/share/hp/bios/hp-scripting-toolkit-linux-9.50.tgz root@ds-mon11t:/tmp/

root@ds-mon11t's password:

hp-scripting-toolkit-linux-9.50.tgz                                                                                                    100%  110MB   1.2MB/s   01:30

[support@centos00 plugins]$

[root@ds-mon11t tmp]# tar xvzf /tmp/hp-scripting-toolkit-linux-9.50.tgz -C /

opt/hp/scripting/toolkit/

opt/hp/scripting/toolkit/boot_files/

...

opt/hp/scripting/toolkit/mkusbkey.sh

[root@ds-mon11t tmp]#

[root@ds-mon11t tmp]# ldd /opt/hp/scripting/toolkit/utilities/conrep

linux-gate.so.1 =>  (0x00229000)

libxml2.so.2 => not found

libstdc++.so.6 => /usr/lib/libstdc++.so.6 (0x00a71000)

libm.so.6 => /lib/libm.so.6 (0x009da000)

libgcc_s.so.1 => /lib/libgcc_s.so.1 (0x0032f000)

libc.so.6 => /lib/libc.so.6 (0x0034d000)

/lib/ld-linux.so.2 (0x002b7000)

[root@ds-mon11t tmp]#

[root@ds-mon11t tmp]# yum list duplicates libxml2

Loaded plugins: product-id, security, subscription-manager

This system is not registered to Red Hat Subscription Management. You can use subscription-manager to register.

Installed Packages

libxml2.x86_64                                          2.7.6-8.el6_3.4                                           @anaconda-RedHatEnterpriseLinux-201301301459.x86_64/6.4

Available Packages

libxml2.i686                                            2.7.6-8.el6_3.4                                           rhel64

[root@ds-mon11t tmp]#

[root@ds-mon11t tmp]# yum install libxml2-2.7.6-8.el6_3.4.i686

Loaded plugins: product-id, security, subscription-manager

This system is not registered to Red Hat Subscription Management. You can use subscription-manager to register.

Setting up Install Process

Resolving Dependencies

--> Running transaction check

---> Package libxml2.i686 0:2.7.6-8.el6_3.4 will be installed

--> Processing Dependency: libz.so.1 for package: libxml2-2.7.6-8.el6_3.4.i686

--> Running transaction check

---> Package zlib.i686 0:1.2.3-29.el6 will be installed

--> Finished Dependency Resolution

Dependencies Resolved

=========================================================================================================================================================================

Package                                Arch                                Version                                          Repository                             Size

=========================================================================================================================================================================

Installing:

libxml2                                i686                                2.7.6-8.el6_3.4                                  rhel64                                800 k

Installing for dependencies:

zlib                                   i686                                1.2.3-29.el6                                     rhel64                                 73 k

Transaction Summary

=========================================================================================================================================================================

Install       2 Package(s)

Total download size: 872 k

Installed size: 1.8 M

Is this ok [y/N]: y

Downloading Packages:

(1/2): libxml2-2.7.6-8.el6_3.4.i686.rpm                                                                                                           | 800 kB     00:00

(2/2): zlib-1.2.3-29.el6.i686.rpm                                                                                                                 |  73 kB     00:00

-------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Total                                                                                                                                    2.4 MB/s | 872 kB     00:00

Running rpm_check_debug

Running Transaction Test

Transaction Test Succeeded

Running Transaction

Installing : zlib-1.2.3-29.el6.i686                                                                                                                                1/2

Installing : libxml2-2.7.6-8.el6_3.4.i686                                                                                                                          2/2

Verifying  : libxml2-2.7.6-8.el6_3.4.i686                                                                                                                          1/2

Verifying  : zlib-1.2.3-29.el6.i686                                                                                                                                2/2

Installed:

libxml2.i686 0:2.7.6-8.el6_3.4

Dependency Installed:

zlib.i686 0:1.2.3-29.el6

Complete!

[root@ds-mon11t tmp]#

[root@ds-mon11t tmp]# ldd /opt/hp/scripting/toolkit/utilities/conrep

linux-gate.so.1 =>  (0x00a08000)

libxml2.so.2 => /usr/lib/libxml2.so.2 (0x00692000)

libstdc++.so.6 => /usr/lib/libstdc++.so.6 (0x00347000)

libm.so.6 => /lib/libm.so.6 (0x00b8c000)

libgcc_s.so.1 => /lib/libgcc_s.so.1 (0x00da4000)

libc.so.6 => /lib/libc.so.6 (0x00432000)

libdl.so.2 => /lib/libdl.so.2 (0x00212000)

libz.so.1 => /lib/libz.so.1 (0x002f9000)

/lib/ld-linux.so.2 (0x002b7000)

[root@ds-mon11t tmp]#

[root@ds-mon11t tmp]# mkdir -p ~/backup/$(date +%Y%m%d)-1239

[root@ds-mon11t tmp]#

[root@ds-mon11t tmp]# cd ~/backup/20140205-1239/

[root@ds-mon11t 20140205-1239]#

[root@ds-mon11t 20140205-1239]# /opt/hp/scripting/toolkit/utilities/conrep -s -x/opt/hp/scripting/toolkit/utilities/conrep.xml -f/root/backup/$(date +%Y%m%d)-1239/$(date +%Y%m%d).ds-mon11t.conrep.xml

conrep 3.50 - SmartStart Scripting Toolkit Configuration Replication Program

Copyright (c) 2007-2013 Hewlett-Packard Development Company, L.P.

System Type:                    ProLiant DL380p Gen8

ROM Date   :                    12/20/2013

ROM Family :                    P70

Processor Manufacturer :        Intel

XML System Configuration : /opt/hp/scripting/toolkit/utilities/conrep.xml

Hardware   Configuration : /root/backup/20140205-1239/20140205.ds-mon11t.conrep.xml

File contains global platform restrictions

Global Restriction: [minimumconrepversion            ]        OK

Platform check:

: [ProLiant DL120 G7   ]        no match

: [ProLiant ML110 G7   ]        no match

: [ProLiant DL3        ]        match

: [ProLiant ML3        ]        no match

: [ProLiant DL5        ]        no match

: [ProLiant ML5        ]        no match

: [ProLiant DL7        ]        no match

: [ProLiant DL9        ]        no match

: [ProLiant BL         ]        no match

: [ProLiant XW         ]        no match

: [ProLiant WS         ]        no match

: [ProLiant SE         ]        no match

: [ProLiant SL4        ]        no match

: [ProLiant SL3        ]        no match

: [ProLiant SL2        ]        no match

: [ProLiant DL160 Gen8 ]        no match

: [ProLiant SL160 Gen8 ]        no match

: [ProLiant SL140      ]        no match

: [ProLiant WS460c Gen8 WS Blade]       no match

: [ProLiant MicroServer Gen8]   no match

: [StoreEasy           ]        no match

: [ProLiant ML10       ]        no match

^C

And here it hangs...

I have changed only this option.

After server reboot it starts working.

[root@ds-mon11t ~]# /opt/hp/scripting/toolkit/utilities/conrep -s -x/opt/hp/scripting/toolkit/utilities/conrep.xml -f/root/backup/$(date +%Y%m%d)-1239/$(date +%Y%m%d).ds-mon11t.conrep.xml

conrep 3.50 - SmartStart Scripting Toolkit Configuration Replication Program

Copyright (c) 2007-2013 Hewlett-Packard Development Company, L.P.

System Type:                    ProLiant DL380p Gen8

ROM Date   :                    12/20/2013

ROM Family :                    P70

Processor Manufacturer :        Intel

XML System Configuration : /opt/hp/scripting/toolkit/utilities/conrep.xml

Hardware   Configuration : /root/backup/20140205-1239/20140205.ds-mon11t.conrep.xml

File contains global platform restrictions

Global Restriction: [minimumconrepversion            ]        OK

Platform check:

: [ProLiant DL120 G7   ]        no match

: [ProLiant ML110 G7   ]        no match

: [ProLiant DL3        ]        match

: [ProLiant ML3        ]        no match

: [ProLiant DL5        ]        no match

: [ProLiant ML5        ]        no match

: [ProLiant DL7        ]        no match

: [ProLiant DL9        ]        no match

: [ProLiant BL         ]        no match

: [ProLiant XW         ]        no match

: [ProLiant WS         ]        no match

: [ProLiant SE         ]        no match

: [ProLiant SL4        ]        no match

: [ProLiant SL3        ]        no match

: [ProLiant SL2        ]        no match

: [ProLiant DL160 Gen8 ]        no match

: [ProLiant SL160 Gen8 ]        no match

: [ProLiant SL140      ]        no match

: [ProLiant WS460c Gen8 WS Blade]       no match

: [ProLiant MicroServer Gen8]   no match

: [StoreEasy           ]        no match

: [ProLiant ML10       ]        no match

Saving configuration data to /root/backup/20140205-1239/20140205.ds-mon11t.conrep.xml.

CONREP Return code: 0

[root@ds-mon11t ~]#

[root@ds-mon11t ~]# cat /root/backup/20140205-1239/20140205.ds-mon11t.conrep.xml

<?xml version="1.0" encoding="UTF-8"?>

<!--generated by conrep version 3.50-->

<Conrep version="3.50" originating_platform="ProLiant DL380p Gen8" originating_family="P70" originating_romdate="12/20/2013" originating_processor_manufacturer="Intel">

<Section name="IMD_ServerName" helptext="LCD Display name for this server">

<Line0>ds-mon11t</Line0>

</Section>

<Section name="IPL_Order" helptext="Current Initial ProgramLoad device boot order.">

<Index0>00 </Index0>

<Index1>01 </Index1>

<Index2>03 </Index2>

<Index3>02 </Index3>

<Index4>04 </Index4>

<Index5>ff </Index5>

<Index6>ff </Index6>

<Index7>ff </Index7>

<Index8>ff </Index8>

<Index9>ff </Index9>

<Index10>ff </Index10>

<Index11>ff </Index11>

<Index12>ff </Index12>

<Index13>ff </Index13>

<Index14>ff </Index14>

<Index15>ff </Index15>

</Section>

<Section name="IPL_Order_Size" helptext="Current Initial ProgramLoad device boot order size.">

<Size0>05 </Size0>

</Section>

<Section name="PCI_Devices" helptext="Lists of PCI devices and their interrupts - not displayed if default values are set.">EMPTY_DELETE</Section>

<Section name="Controller_Order" helptext="Lists the current boot controller ordering.">

<Id0>10 3c 33 54 </Id0>

<Slot0>00 </Slot0>

<BusDev0>02 00 </BusDev0>

<Rest0>41 </Rest0>

<Id1>10 3c 18 a9 </Id1>

<Slot1>00 </Slot1>

<BusDev1>00 fa </BusDev1>

<Rest1>41 </Rest1>

<Id2>10 3c 33 48 </Id2>

<Slot2>02 </Slot2>

<BusDev2>07 06 </BusDev2>

<Rest2>41 </Rest2>

<Id3>10 3c 33 48 </Id3>

<Slot3>02 </Slot3>

<BusDev3>07 07 </BusDev3>

<Rest3>41 </Rest3>

<Id4>10 3c 33 48 </Id4>

<Slot4>05 </Slot4>

<BusDev4>24 06 </BusDev4>

<Rest4>41 </Rest4>

<Id5>10 3c 33 48 </Id5>

<Slot5>05 </Slot5>

<BusDev5>24 07 </BusDev5>

<Rest5>41 </Rest5>

</Section>

<Section name="Language" helptext="Defines current language identifier">ENGUSAus  </Section>

<Section name="System_WOL" helptext="System Wake On Lan Capabilities.">Enabled</Section>

<Section name="System_APIC" helptext="System APIC Settings(MPS Table Mode).">Full Table</Section>

<Section name="System_Mouse" helptext="Enable or Disable the onboard mouse port.">Enabled</Section>

<Section name="System_CPU_Serial_Number" helptext="Enables or disabled CPU serial numbers on systems that support it.">Disabled</Section>

<Section name="System_COMA" helptext="Settings for UART 1 on systems that support it.">COM1</Section>

<Section name="System_COMA_IRQ" helptext="Settings for UART 1 on systems that support it.">IRQ4</Section>

<Section name="System_COMB" helptext="Settings for UART 2 on systems that support it.">Disabled</Section>

<Section name="System_COMB_IRQ" helptext="Settings for UART 2 on systems that support it.">Undefined</Section>

<Section name="System_Virtual_Serial_Port" helptext="Virtual Serial Port Settings for systems that support it.">COM2</Section>

<Section name="System_Virtual_Serial_Port_IRQ" helptext="Virtual Serial Port IRQ Settings for systems that support it.">IRQ3</Section>

<Section name="System_LPT" helptext="System parallel port settings for systems that support it.">Disabled</Section>

<Section name="System_LPT_IRQ" helptext="System parallel port settings for systems that support it.">Undefined</Section>

<Section name="System_LPT_Mode" helptext="System parallel port settings for systems that support it.">SPP</Section>

<Section name="System_USB_Control" helptext="Systemwide USB settings.">Enabled</Section>

<Section name="System_USB_EHCI_Controller" helptext="Enables or disabled EHCI mode(USB 2.0) of the system USB controller.">Enabled</Section>

<Section name="Diskette_Write_Control" helptext="Toggles write control of integrated floppy.">Writes_Enabled</Section>

<Section name="POST_F1_Prompt" helptext="Controls display of the F1 POST prompt.">Delayed</Section>

<Section name="Hyperthreading" helptext="Toggles hyperthreading on systems that support it.  This setting is supported on G5 and earlier systems.  For newer systems, use the Intel_Hyperthreading setting.">Enabled</Section>

<Section name="NMI_Debug_Button" helptext="Allows generating NMI(non-maskable interrupt) via NMI button/jumper for hardware debugging.">Enabled</Section>

<Section name="ACPI_Power_Button" helptext="Switches power button to operate in APCI mode.  Enabling permits enhanced power button handling(push and hold to shut off, and single push to shutdown) instead of on/off operation.">Enabled</Section>

<Section name="CPU_Performance" helptext="Allows tuning of system performance goals">Memory</Section>

<Section name="ASR" helptext="Allows enabling/disabling of the Automatic Server Recovery feature.">Enabled</Section>

<Section name="ASR_Timeout" helptext="User Configuration Timeout for Automatic Server Recovery">10 Minutes</Section>

<Section name="Thermal_Shutdown" helptext="Enables shutdown when thermal events occur">Enabled</Section>

<Section name="RBSU_Language" helptext="Language to use for the RBSU menus">01</Section>

<Section name="BIOS_Console" helptext="Used to select the port for the BIOS console">Auto</Section>

<Section name="BIOS_Baud_Rate" helptext="BIOS console communications settings">9600</Section>

<Section name="BIOS_Type" helptext="BIOS console communications settings">VT100</Section>

<Section name="EMS_Console" helptext="Used to select the port for the EMS console">Disabled</Section>

<Section name="BIOS_Interface_Mode" helptext="BIOS console communications settings">Auto Mode</Section>

<Section name="System_Embedded_IDE" helptext="Settings for the embedded IDE controller on systems that support it">Disabled</Section>

<Section name="Virtual_Install_Disk" helptext="Controls the Virtual Install Disk. The Virtual Install Disk may contain drivers specific to this server that an OS may use during installation..">Disabled</Section>

<Section name="HP_Power_Regulator" helptext="Allows tuning of the system power usage">HP_Static_High_Performance_Mode</Section>

<Section name="HW_Prefetch" helptext="System performance tuning entry">Enabled</Section>

<Section name="Adjacent_Sector_Prefetch" helptext="System performance tuning entry">Enabled</Section>

<Section name="ATA_Drive_Write_Cache" helptext="Enables the write cache on attached ATA disks">Disabled</Section>

<Section name="Embedded_SATA_Configuration" helptext="Sets the mode for the embedded SATA controller. Set RAID mode to enable B120i controller.">SATA_Legacy_Enabled</Section>

<Section name="No_Execute_Memory_Protection" helptext="Enables No Execute mode if the system CPU supports it">Enabled</Section>

<Section name="Legacy_SERR">Enabled</Section>

<Section name="CPU_Virtualization" helptext="When enabled, a Virtual Machine Manager supporting this feature can utilize hardware capabilities provided by the server.">Enabled</Section>

<Section name="Embedded_NIC" helptext="This setting allows the enabling/disabling of network boot(PXE) on embedded network controllers in the server.">

<NIC0>01 </NIC0>

<NIC1>00 </NIC1>

<NIC2>00 </NIC2>

<NIC3>00 </NIC3>

<NIC4>00 </NIC4>

<NIC5>00 </NIC5>

<NIC6>00 </NIC6>

<NIC7>00 </NIC7>

<NIC8>00 </NIC8>

<NIC9>00 </NIC9>

<NIC10>00 </NIC10>

<NIC11>00 </NIC11>

</Section>

<Section name="Diskette_Boot">Enabled</Section>

<Section name="NumLock">Off</Section>

<Section name="POST_Speed_Up">Enabled</Section>

<Section name="Integrated_Diskette_Controller">Enabled</Section>

<Section name="PCI_Bus_Reset">Enabled</Section>

<Section name="Hot_Plug_Reservation">Auto Set</Section>

<Section name="Memory_Protection" helptext="Sets the type of memory protection used on the server.  Not all modes are supported on all servers.">Advanced_ECC_Support</Section>

<Section name="Advanced_Memory_Protection_Gen8" helptext="Sets the type of memory protection used on the server.  Not all modes are supported on all servers.">Advanced_ECC_Support</Section>

<Section name="Automatic_Poweron" helptext="For G7 and earlier only. Configures server to automatically power on when A/C power is applied to the system. This setting is not support by blade servers.">Disabled</Section>

<Section name="Automatic_Poweron_Gen8" helptext="For Gen8 and later. Configures server to automatically power on when A/C power is applied to the system. This setting is not support by blade servers.">Restore_Last_Power_State</Section>

<Section name="Power-On_Delay" helptext="Sets the delay for the system to be begin the boot up process">No_Delay</Section>

<Section name="USB_Capability">USB_1.1</Section>

<Section name="PCI_Enumeration_Method" helptext="Controls the PCI enumeration method on G6 servers. Supported on G6 servers only.">Standard</Section>

<Section name="Shared_ILO_Port" helptext="Switches iLo from using its dedicated NIC port to sharing the first NIC of the system for ML310 G5.">Dedicated_Port</Section>

<Section name="Option_ROM_Loading_Sequence" helptext="Choses whether Option ROMs for embedded devices or expansion cards are loaded first.">Embedded_First</Section>

<Section name="Removable_Flash_Media_Boot_Sequence" helptext="Selects which USB or SD card devices will be searched first when enumerating boot devices.">External_Drivekeys_First</Section>

<Section name="USB_Port_Boot_Order_G5" helptext="Allows the USB Port Boot order to be set on G5 or older systems.">Internal</Section>

<Section name="USB_Port_Boot_Order_G6" helptext="Allows the USB Port Boot order to be set on G6 or newer systems.">External</Section>

<Section name="USB_Boot_Support" helptext="Supported on Gen8 only. This option allows the user to prevent the system from booting all USB devices connected to the server. This option will also disable booting iLO virtual media.">Enabled</Section>

<Section name="Low_Power_Halt_state" helptext="Allows the enable/disable of the Low Power Halt state(C1E).">Enabled</Section>

<Section name="Intel_Processor_Core_Disable" helptext="Allows disabling of processor cores using Intel's Core Multi-Processing(CMP) technology.">All_Cores_Enabled</Section>

<Section name="Intel_Processor_Turbo_Mode" helptext="Allows Intel processors to transition to a higher frequency than its rated speed if the processor has available headroom and is within temperature specification.">Enabled</Section>

<Section name="Intel_VT-d2" helptext="When enabled, a Virtual Machine Manager supporting this feature can utilize hardware capabilities provided by Intel's Virtualization Technology for Directed I/O.">Enabled</Section>

<Section name="HP_Power_Profile" helptext="Selects the level of power versus performance for the system.">Custom</Section>

<Section name="Intel_QPI_Link_Power_Management" helptext="Places the Quick Path Interconnect (QPI) links into a low power state to save power.">Enabled</Section>

<Section name="Intel_QPI_Link_Frequency" helptext="This option allows the user to set the QPI Link frequency to a lower speed. Running at a lower frequency may reduce power consumption but may also impact system performance.">Auto</Section>

<Section name="Intel_Minimum_Processor_Idle_Power_State" helptext="Selects the lowest idle power state (C-state) the OS will use. The higher the C-state, the lower the power usage. C6 is the lowest power idle state supported. For G6/G7/Gen8 BIOS 05/05/2011 or later, this is the Power Core State. ">C6_State</Section>

<Section name="Intel_Minimum_Processor_Idle_Power_Package_State" helptext="Selects the lowest idle power package state (C-state). On Gen8 and later servers, Package C3 State corresponds to Package C6(non-retention) State in RBSU.">Package C3 State</Section>

<Section name="Maximum_Memory_Bus_Frequency" helptext="Configures memory system to run memory at a lower maximum speed than what is supported by the installed processor and DIMM configuration.  Setting to Auto configures max speed supported.">Auto</Section>

<Section name="Memory_Interleaving" helptext="Modifies the level of interleaving for which the memory system will be configured.  Typically, higher levels of memory interleaving result in maximum performance, but reducing interleaving level may result in power savings.">Full_Interleaving</Section>

<Section name="Intel_PCI_Express_Generation_20_Support" helptext="For Gen8 servers, this is the same as Intel Maximum PCI Express Speed. Force_PCI-E_Generation_2 is the same as Maximum Supported on Gen8.">Force_PCI-E_Generation_2</Section>

<Section name="Thermal_Configuration" helptext="Selects the fan cooling solution for the system(not for ML350 G6).">Optimal_Cooling</Section>

<Section name="Intel_TPM_Functionality" helptext="For Intel G7. This option allows the enabling of the Trusted Platform Module and BIOS secure startup.">Disabled</Section>

<Section name="Intel_TPM_Visibility" helptext="For Intel G7. This option provides the ability to hide the TPM from the operating system.">Unhide</Section>

<Section name="Intel_TPM_Expansion_ROM_Measuring" helptext="For Intel G7. This option allows enabling the BIOS to measure the PCI/PCI-e expansion (Option) ROM code and store that measurement into the TPM.">Disabled</Section>

<Section name="TPM_Functionality_Gen8" helptext="For Gen8 only. This option allows the enabling of the Trusted Platform Module and BIOS secure startup.">Disabled</Section>

<Section name="TPM_Visibility_Gen8" helptext="For Gen8 only. This option provides the ability to hide the TPM from the operating system.">Unhide</Section>

<Section name="TPM_Expansion_ROM_Measuring_Gen8" helptext="For Gen8 only. This option allows enabling the BIOS to measure the PCI/PCI-e expansion (Option) ROM code and store that measurement into the TPM.">Disabled</Section>

<Section name="Node_Interleaving" helptext="Allows the enabling/disabling of node interleaving on Intel platforms.">Disabled</Section>

<Section name="Intel_Hyperthreading" helptext="Toggles hyperthreading on Intel based G6 and greater systems">Enabled</Section>

<Section name="PCI_Bus_Padding_Option" helptext="Option allows the user to disable the default PCI Bus setting where each expansion slot is provided an extra PCI Bus number. By default, the system ROM will pad one PCI bus per bridge to not affect current bus numbering schemes.">Enabled</Section>

<Section name="PCI_Bus_Padding_Option_Gen8" helptext="For Gen8 servers only. Option allows the user to disable the default PCI Bus setting where each expansion slot is provided an extra PCI Bus number. By default, the system ROM will pad one PCI bus per bridge to not affect current bus numbering schemes.">Enabled</Section>

<Section name="Power-on_Logo" helptext="Allows disabling of HP logo at system boot.">Disabled</Section>

<Section name="Hide_Option_ROM_Messages" helptext="Allows disabling of Hide Option ROM Messages feature.">Enabled</Section>

<Section name="Redundant_Power_Supply_Mode" helptext="Allows configuring how the system will handle redundant power supply configs.  Balanced mode shares the power delivery equally between installed power supplies.">Balanced_Mode</Section>

<Section name="Dynamic_Power_Savings_Mode_Response" helptext="Allows the system ROM to control processor performance and power state depending on the processor workload.">Fast</Section>

<Section name="Memory_Speed_with_2_DIMMs_per_Channel" helptext="Allows system to run DIMMs at 1333 Mhz when up to 2 DIMMs are installed on a memory channel. Function validated with HP DIMMs. HP recommends this setting remain at the default of 1066 MHz if utilizing non HP-DIMMs.">1333MHz_Maximum</Section>

<Section name="Video_Options" helptext="By default, system ROM disables the embedded video controller when an optional video controller is detected.  Option allows embedded video to remain enabled for iLO remote video.">Optional_Primary_Embedded_Disabled</Section>

<Section name="NIC1_Ports" helptext="Option allows individual network ports to be disabled on some G6 systems.">All_Enabled</Section>

<Section name="NIC2_Ports" helptext="Option allows individual network ports to be disabled on some G6 systems.">All_Enabled</Section>

<Section name="Power_Supply_Requirements_Override" helptext="Overrides the number of power supplies required.  This setting will be ignored on the DL580 G7.">Normal</Section>

<Section name="Collaborative_Power_Control" helptext="For OSes that support the Processor Clocking Control Interface, enabling this option allows the OS to request processor frequency changes even when the server has the Power Regulator option configured for Dynamic Power Savings Mode.">Enabled</Section>

<Section name="Processor_Core_Disable_Gen8" helptext="Selects the number of cores that will be enabled per processor socket on Gen8 systems.">All_Cores_Enabled</Section>

<Section name="DCU_Prefetcher" helptext="This setting is called DCU Streamer Prefetcher in RBSU on newer BIOS. System performance tuning entry.">Enabled</Section>

<Section name="DCU_IP_Prefetcher" helptext="This option allows the disabling of a processor prefetch feature. In very limited cases, setting this option to Disabled may improve performance. In the vast majority of cases, the default value of Enabled provides better performance.">Enabled</Section>

<Section name="Data_Reuse" helptext="For G7 and earlier servers only. System performance tuning entry.">Enabled</Section>

<Section name="QPI_Bandwidth_Optimization_Gen8" helptext="The QPI link between 2 processors has been tuned to provide the best performance for all known applications.">Balanced</Section>

<Section name="Intel_Turbo_Boost_Optimization" helptext="Optimize Turbo Boost heuristics for different situations.">Optimized_for_Performance</Section>

<Section name="Intel_Turbo_Boost_Optimization_Gen8" helptext="Optimize Turbo Boost heuristics for different situations. For Gen8 servers only.">Optimized_for_Performance</Section>

<Section name="Intel_PCI_Express_Gen3_Embedded_Storage_Device_Control" helptext="Allows enabling/disabling PCI-E Gen 3.0 embedded storage controller (PMC control) on Intel based Gen8 or later servers.">Enabled</Section>

<Section name="Intel_PCI_Express_Gen3_Embedded_NIC_1_Device_Control" helptext="Allows enabling/disabling PCI-E Gen 3.0 embedded NIC 1 on Intel based Gen8 or later servers.">Enabled</Section>

<Section name="Intel_PCI_Express_Gen3_Embedded_NIC_2_Device_Control" helptext="Allows enabling/disabling PCI-E Gen 3.0 embedded NIC 2 on Intel based Gen8 or later servers.">Disabled</Section>

<Section name="Intel_PCI_Express_Gen3_System_Specific_Device_1_Control" helptext="Allows enabling/disabling PCI-E Gen 3.0 System-Specific Device 1 on Intel based Gen8 or later servers.">Enabled</Section>

<Section name="Intel_PCI_Express_Gen3_System_Specific_Device_2_Control" helptext="Allows enabling/disabling PCI-E Gen 3.0 System-Specific Device 2 on Intel based Gen8 or later servers.">Enabled</Section>

<Section name="Intel_PCI_Express_Gen3_System_Specific_Device_3_Control" helptext="Allows enabling/disabling PCI-E Gen 3.0 System-Specific Device 3 on Intel based Gen8 or later servers.">Enabled</Section>

<Section name="Intel_PCI_Express_Gen3_System_Specific_Device_4_Control" helptext="Allows enabling/disabling PCI-E Gen 3.0 System-Specific Device 4 on Intel based Gen8 or later servers.">Enabled</Section>

<Section name="Intel_PCI_Express_Gen3_System_Specific_Device_5_Control" helptext="Allows enabling/disabling PCI-E Gen 3.0 System-Specific Device 5 on Intel based Gen8 or later servers.">Enabled</Section>

<Section name="Intel_PCI_Express_Gen3_System_Specific_Device_6_Control" helptext="Allows enabling/disabling PCI-E Gen 3.0 System-Specific Device 6 on Intel based Gen8 or later servers.">Enabled</Section>

<Section name="Intel_PCI_Express_Gen3_System_Specific_Device_7_Control" helptext="Allows enabling/disabling PCI-E Gen 3.0 System-Specific Device 7 on Intel based Gen8 or later servers.">Enabled</Section>

<Section name="Intel_PCI_Express_Gen3_System_Specific_Device_8_Control" helptext="Allows enabling/disabling PCI-E Gen 3.0 System-Specific Device 8 on Intel based Gen8 or later servers.">Enabled</Section>

<Section name="Intel_PCI_Express_Gen3_Slot_1_Control" helptext="Allows enabling/disabling PCI-E Gen 3.0 Slot 1 on Intel based Gen8 or later servers.">Enabled</Section>

<Section name="Intel_PCI_Express_Gen3_Slot_2_Control" helptext="Allows enabling/disabling PCI-E Gen 3.0 Slot 2 on Intel based Gen8 or later servers.">Enabled</Section>

<Section name="Intel_PCI_Express_Gen3_Slot_3_Control" helptext="Allows enabling/disabling PCI-E Gen 3.0 Slot 3 on Intel based Gen8 or later servers.">Enabled</Section>

<Section name="Intel_PCI_Express_Gen3_Slot_4_Control" helptext="Allows enabling/disabling PCI-E Gen 3.0 Slot 4 on Intel based Gen8 or later servers.">Enabled</Section>

<Section name="Intel_PCI_Express_Gen3_Slot_5_Control" helptext="Allows enabling/disabling PCI-E Gen 3.0 Slot 5 on Intel based Gen8 or later servers.">Enabled</Section>

<Section name="Intel_PCI_Express_Gen3_Slot_6_Control" helptext="Allows enabling/disabling PCI-E Gen 3.0 Slot 6 on Intel based Gen8 or later servers.">Enabled</Section>

<Section name="Intel_PCI_Express_Gen3_Slot_7_Control" helptext="Allows enabling/disabling PCI-E Gen 3.0 Slot 7 on Intel based Gen8 or later servers.">Enabled</Section>

<Section name="Intel_PCI_Express_Gen3_Slot_8_Control" helptext="Allows enabling/disabling PCI-E Gen 3.0 Slot 8 on Intel based Gen8 or later servers.">Enabled</Section>

<Section name="Intel_PCI_Express_Gen3_Slot_9_Control" helptext="Allows enabling/disabling PCI-E Gen 3.0 Slot 9 on Intel based Gen8 or later servers.">Enabled</Section>

<Section name="Intel_PCI_Express_Gen3_Slot_10_Control" helptext="Allows enabling/disabling PCI-E Gen 3.0 Slot 10 on Intel based Gen8 or later servers.">Enabled</Section>

<Section name="Intel_PCI_Express_Gen3_Slot_11_Control" helptext="Allows enabling/disabling PCI-E Gen 3.0 Slot 11 on Intel based Gen8 or later servers.">Enabled</Section>

<Section name="Intel_PCI_Express_Gen3_Slot_12_Control" helptext="Allows enabling/disabling PCI-E Gen 3.0 Slot 12 on Intel based Gen8 or later servers.">Enabled</Section>

<Section name="Intel_PCI_Express_Gen3_Slot_13_Control" helptext="Allows enabling/disabling PCI-E Gen 3.0 Slot 13 on Intel based Gen8 or later servers.">Enabled</Section>

<Section name="Intel_PCI_Express_Gen3_Slot_14_Control" helptext="Allows enabling/disabling PCI-E Gen 3.0 Slot 14 on Intel based Gen8 or later servers.">Enabled</Section>

<Section name="Intel_PCI_Express_Gen3_Slot_15_Control" helptext="Allows enabling/disabling PCI-E Gen 3.0 Slot 15 on Intel based Gen8 or later servers.">Enabled</Section>

<Section name="Intel_PCI_Express_Gen3_Slot_16_Control" helptext="Allows enabling/disabling PCI-E Gen 3.0 Slot 16 on Intel based Gen8 or later servers.">Enabled</Section>

<Section name="Intel_F11_Boot_Menu_Prompt" helptext="This option allows the disabling of the POST One Time Boot F11 Prompt on Gen8 Servers. ">Enabled</Section>

<Section name="Energy_Performance_Bias" helptext="Sets the mode for the Energy/Performance Bias. ">Balanced_Performance</Section>

<Section name="1333_MHz_support_for_3DPC_PC3-10600H_HP_SmartMemory" helptext="For Gen8 servers. Allows the enabling/disabling of 1333 MHz support for 3 DIMMs per channel for HP SmartMemory.">Disabled</Section>

<Section name="System_Locality_Information_Table_G7" helptext="This ACPI SLIT (System Locality Information Table) describes the relative access times between processors, memory subsystems, and I/O subsystems.">Disabled</Section>

<Section name="System_Locality_Information_Table_Gen8" helptext="This ACPI SLIT (System Locality Information Table) describes the relative access times between processors, memory subsystems, and I/O subsystems.">Disabled</Section>

<Section name="Intel_DIMM_Voltage_Preference" helptext=".">Optimized_for_Power</Section>

<Section name="Intel_Memory_Channel_Mode_Gen8" helptext="Combined Channel Mode provides maximum data protection by allowing multiple-bit memory errors to be corrected in certain instances not possible in normal Advanced ECC Modes.">Independent_Channel_Mode</Section>

<Section name="Dynamic_Power_Capping_Functionality" helptext="This option allows the user to disable the System ROM Power Calibration feature that is executed during the boot process. When disabled, the user can expect faster boot times.">Enabled</Section>

<Section name="SR-IOV" helptext="This option provides the capability to enable or disable SR-IOV(Single Root I/O Virtualization). ">Disabled</Section>

<Section name="Memory_Power_Savings_Mode" helptext=".">Balanced</Section>

<Section name="Network_Boot_Retry_Support" helptext="This option allows the user to disable the Network Boot Retry Support. By default, the System ROM will attempt to boot the Network device (i.e. PXE device) up to 20 times before falling through to the next IPL Device.">Enabled</Section>

<Section name="Reset_on_Boot_Device_Not_Found" helptext="This option controls the System ROM logic of retrying bootable devices listed in the Standard Boot Order (IPL) if no bootable media is found.">Disabled</Section>

<Section name="Intel_NIC_DMA_Channels" helptext="This setting allows the user to enable Intel NIC DMA Channels.">Enabled</Section>

<Section name="Consistent_Device_Naming" helptext="This option selects the level of Consistent Device Naming. On supported operating systems, NIC ports will be named based on their location in the system. &quot;CDN Support for LOMs Only&quot; will only name Embedded NICs and FlexibleLOMs (not NICs in slots).">Disabled</Section>

<Section name="ROM_Selection" helptext="This option allows the server to switch back to a previous ROM image. The backup image will be the ROM image that was being used prior to the last flash event.">Use_Current_ROM</Section>

<Section name="Data_Direct_I/O" helptext="This option controls the ability for the processor to accelerate I/O operations by reducing the number of I/O related memory accesses. This option should increase I/O performance while reducing system power and I/O latency.">Enabled</Section>

<Section name="Intel_Performance_Counter_Monitor" helptext="This option allows the user to unhide certain Performance Counters in the processor. When enabled, the user can use tools such as the Performance Counter Monitor (PCM).">Disabled</Section>

<Section name="A7_Memory_Address_Mode" helptext="This option allows the disabling of a processor memory address mode.  In very limited cases, setting this option to Disabled may improve performance.">Enabled</Section>

</Conrep>

[root@ds-mon11t ~]#

*. If possible, check and configure usage of kdump packages.

[root@ds-mon11t ~]# chkconfig --list kdump

kdump           0:off   1:off   2:off   3:on    4:on    5:on    6:off

[root@ds-mon11t ~]#

[root@ds-mon11t ~]# service kdump status

Kdump is operational

[root@ds-mon11t ~]#

*. Configure nagios monitoring for this host.

[root@nag2 /usr/local/etc/nagios/objects/servers/dc-ds/linux]# cat ds-mon11t.cfg

define host{

use                     linux-server

host_name               ds-mon11t

alias                   Server monitoring for compuware project

hostgroups              linux-servers

address                 10.216.43.43

display_name            ds-mon11t

icon_image              redhat.png

}

define service{

use                      generic-service

host_name                ds-mon11t

service_description      PING

contacts                 Naumen_nag_request_email

contact_groups           linux-admins

check_interval           5

max_check_attempts       1

check_command            check_ping!100.0,20%!500.0,60%

}

define service{

use                      generic-service

host_name                ds-mon11t

service_description      /Root Partition

contacts                 Naumen_nag_request_email

contact_groups           linux-admins

check_command            check_nrpe!check_root_fs

}

define service{

use                      generic-service

host_name                ds-mon11t

service_description      /Opt Partition

contacts                 Naumen_nag_request_email

contact_groups           linux-admins

check_command            check_nrpe!check_opt_fs

}

define service{

use                      generic-service

host_name                ds-mon11t

service_description      /Tmp Partition

contacts                 Naumen_nag_request_email

contact_groups           linux-admins

check_command            check_nrpe!check_tmp_fs

}

define service{

use                      generic-service

host_name                ds-mon11t

service_description      Current Memory Load

contacts

contact_groups           linux-admins

check_command            check_nrpe!check_memory_load_linux

}

define service{

use                      generic-service

host_name                ds-mon11t

service_description      Current Load

contacts

contact_groups           linux-admins

check_command            check_nrpe!check_load

}

define service{

use                      generic-service

host_name                ds-mon11t

service_description      Zombie procs

contacts

contact_groups           linux-admins

check_command            check_nrpe!check_zombie_procs

}

define service{

use                      generic-service

host_name                ds-mon11t

service_description      NTP Time

contacts

contact_groups           linux-admins

check_command            check_nrpe!check_ntp_time

}

You have new mail in /var/mail/root

[root@nag2 /usr/local/etc/nagios/objects/servers/dc-ds/linux]#

[root@nag2 /usr/local/etc/nagios/objects/servers/dc-ds/linux]# cat /usr/local/etc/nagios/objects/groups/contact-group/linux-admins.cfg

define contactgroup{

contactgroup_name       linux-admins

alias                   Linux Administrators

members                 a.terentyev_sms, a.terentyev_email, a.kuritcyn_email, a.kuritcyn_sms, Naumen_nag_request_email, a.aleksandrov_email, a.aleksandrov_sms, n.makhnev_email, n.makhnev_sms, s.belogrud_email, s.belogrud_sms, e.zbiranik_email, e.zbiranik_sms

}

define contactgroup{

contactgroup_name       linux-admins-non-critical

alias                   Linux Administrators

members                 a.terentyev_email, a.aleksandrov_email, s.belogrud_email, e.zbiranik_email

}

[root@nag2 /usr/local/etc/nagios/objects/servers/dc-ds/linux]#

*. Backup

[root@ds-mon11t backup]# df -h

Filesystem            Size  Used Avail Use% Mounted on

/dev/sda1              16G  1.9G   14G  13% /

tmpfs                 142G     0  142G   0% /dev/shm

/dev/sda4             511G  573M  485G   1% /opt

/dev/sda3             7.9G  147M  7.4G   2% /tmp

/dev/sda2              16G  282M   15G   2% /var

[root@ds-mon11t backup]#

[support@centos00 backup.ds-mon11t]$ ssh root@ds-mon11t "( tar czspf - --one-file-system --exclude=/proc --exclude=/dev --exclude=/sys --exclude=/tmp --exclude=/var/tmp --exclude=/media /etc )" | cat - > ./20140205-1454.ds-mon11t.etc.fs.tgz

root@ds-mon11t's password:

tar: Removing leading `/' from member names

[support@centos00 backup.ds-mon11t]$ ssh root@ds-mon11t "( tar czspf - --one-file-system --exclude=/proc --exclude=/dev --exclude=/sys --exclude=/tmp --exclude=/var/tmp --exclude=/media / )" | cat - > ./20140205-1454.ds-mon11t.root.fs.tgz

root@ds-mon11t's password:

tar: Removing leading `/' from member names

tar: Removing leading `/' from hard link targets

[support@centos00 backup.ds-mon11t]$

[support@centos00 backup.ds-mon11t]$ ls -Alh

total 608M

-rwxrwxrwx 1 root root 7.6M Feb  5 14:59 20140205-1454.ds-mon11t.etc.fs.tgz

-rwxrwxrwx 1 root root 601M Feb  5 15:01 20140205-1454.ds-mon11t.root.fs.tgz

[support@centos00 backup.ds-mon11t]$

*. Clear history file.

[root@ds-mon11t ~]# history -c

[root@ds-mon11t ~]# history

1  2014-02-05 15:05:18 history

[root@ds-mon11t ~]#

*. In several cases it maybe require to create and configure usage of swap file.

For example it could be possible to do like this.

http://www.cyberciti.biz/faq/linux-add-a-swap-file-howto/

[root@rhel64-template ~]# dd if=/dev/zero of=/swapfile bs=1024 count=2097152

2097152+0 records in

2097152+0 records out

2147483648 bytes (2.1 GB) copied, 18.6019 s, 115 MB/s

[root@rhel64-template ~]#

[root@rhel64-template ~]# mkswap /swapfile

mkswap: /swapfile: warning: don't erase bootbits sectors

on whole disk. Use -f to force.

Setting up swapspace version 1, size = 2097148 KiB

no label, UUID=135236fd-4202-4b34-8347-b1a0e163abd0

[root@rhel64-template ~]#

[root@rhel64-template ~]# chmod 0600 /swapfile

[root@rhel64-template ~]#

[root@rhel64-template ~]# vim /etc/fstab

/swapfile               swap                    swap    defaults        0 0

[root@rhel64-template ~]# swapon /swapfile

[root@rhel64-template ~]# swapon -s

Filename                                Type            Size    Used    Priority

/swapfile                               file            2097144 0       -1

[root@rhel64-template ~]#

*. I remember about one thing. During boot sequence I had see one multiple errors. I think - I will fix it.

[root@ds-mon11t ~]# chkconfig --list cpuspeed

cpuspeed        0:off   1:on    2:on    3:on    4:on    5:on    6:off

[root@ds-mon11t ~]#

[root@ds-mon11t ~]# service cpuspeed status

Frequency scaling enabled using ondemand governor

[root@ds-mon11t ~]#

[root@ds-mon11t ~]# ps aux | grep -i cpu

USER       PID %CPU %MEM    VSZ   RSS TTY      STAT START   TIME COMMAND

root     48433  0.0  0.0 103248   876 pts/0    S+   10:46   0:00 grep -i cpu

[root@ds-mon11t ~]#

[root@ds-mon11t ~]# vim /etc/rc3.d/S13cpuspeed

...

84 adjust_cpufreq() {

85   # First arg is a param under $cpu/cpufreq/

86   # Second arg is the value you want to set that param to

87   if $(echo ${1} | grep -qE '(threshold|ignore_nice_load)'); then

88     echo $2 > $cpufreq/$1

89   else

90     for cpu in ${cpus}; do

91       echo $2 > $cpu/cpufreq/$1

92     done

93   fi

94 }

...

[root@ds-mon11t ~]# yum info cpuspeed

Loaded plugins: product-id, security, subscription-manager

This system is not registered to Red Hat Subscription Management. You can use subscription-manager to register.

Installed Packages

Name        : cpuspeed

Arch        : x86_64

Epoch       : 1

Version     : 1.5

Release     : 19.el6

Size        : 64 k

Repo        : installed

From repo   : anaconda-RedHatEnterpriseLinux-201301301459.x86_64

Summary     : CPU frequency adjusting daemon

URL         : http://carlthompson.net/Software/CPUSpeed

License     : GPLv2+

Description : cpuspeed is a daemon that dynamically changes the speed

: of your processor(s) depending upon its current workload

: if it is capable (needs Intel Speedstep, AMD PowerNow!,

: or similar support).

:

: This package also supports enabling cpu frequency scaling

: via in-kernel governors on Intel Centrino and AMD

: Athlon64/Opteron platforms.

[root@ds-mon11t ~]#

[root@ds-mon11t ~]# rpm -qf cpuspeed

error: file /root/cpuspeed: No such file or directory

[root@ds-mon11t ~]# rpm -ql cpuspeed

/etc/rc.d/init.d/cpuspeed

/etc/sysconfig/cpuspeed

/usr/sbin/cpuspeed

/usr/share/doc/cpuspeed-1.5

/usr/share/doc/cpuspeed-1.5/CHANGES

/usr/share/doc/cpuspeed-1.5/CONTRIBUTORS

/usr/share/doc/cpuspeed-1.5/EXAMPLES

/usr/share/doc/cpuspeed-1.5/FEATURES

/usr/share/doc/cpuspeed-1.5/README

/usr/share/doc/cpuspeed-1.5/USAGE

/usr/share/doc/cpuspeed-1.5/license.txt

/usr/share/man/man8/cpuspeed.8.gz

[root@ds-mon11t ~]#

[root@ds-mon11t ~]# cat /etc/sysconfig/cpuspeed

# /etc/sysconfig/cpuspeed

#

# This configuration file controls the behavior of both the

# cpuspeed daemon and various cpufreq modules.

# For the vast majority of users, there shouldn't be any need to

# alter the contents of this file at all. By and large, frequency

# scaling should Just Work(tm) with the defaults.

### DRIVER ###

# Your CPUFreq driver module

# Note that many drivers are now built-in, rather than built as modules,

# so its usually best not to specify one.

# default value: empty (try to auto-detect/use built-in)

DRIVER=

### GOVERNOR ###

# Which scaling governor to use

# Details on scaling governors for your cpu(s) can be found in

# cpu-freq/governors.txt, part of the kernel-doc package

# NOTES:

# - The GOVERNOR parameter is only valid on centrino, powernow-k8 (amd64)

#   and acpi-cpufreq platforms, other platforms that support frequency

#   scaling always use the 'userspace' governor.

# - Using the 'userspace' governor will trigger the cpuspeed daemon to run,

#   which provides said user-space frequency scaling.

# default value: empty (defaults to ondemand on centrino, powernow-k8,

#                       and acpi-cpufreq systems, userspace on others)

GOVERNOR=

### FREQUENCIES ###

# NOTE: valid max/min frequencies for your cpu(s) can be found in

# /sys/devices/system/cpu/cpu*/cpufreq/scaling_available_frequencies

# on systems that support frequency scaling (though only after the

# appropriate drivers have been loaded via the cpuspeed initscript).

# maximum speed to scale up to

# default value: empty (use cpu reported maximum)

MAX_SPEED=

# minimum speed to scale down to

# default value: empty (use cpu reported minimum)

MIN_SPEED=

### SCALING THRESHOLDS ###

# Busy percentage threshold over which to scale up to max frequency

# default value: empty (use governor default)

UP_THRESHOLD=

# Busy percentage threshold under which to scale frequency down

# default value: empty (use governor default)

DOWN_THRESHOLD=

### NICE PROCESS HANDLING ###

# Let background (nice) processes speed up the cpu

# default value: 0 (background process usage can speed up cpu)

# alternate value: 1 (background processes will be ignored)

IGNORE_NICE=0

#####################################################

########## HISTORICAL CPUSPEED CONFIG BITS ##########

#####################################################

VMAJOR=1

VMINOR=1

# Add your favorite options here

#OPTS="$OPTS -s 0 -i 10 -r"

# uncomment and modify this to check the state of the AC adapter

#OPTS="$OPTS -a /proc/acpi/ac_adapter/*/state"

# uncomment and modify this to check the system temperature

#OPTS="$OPTS -t /proc/acpi/thermal_zone/*/temperature 75"

[root@ds-mon11t ~]#

Hello.

I received multiple errors messages during fresh-installed server was rebooted.

You can see it on attached file with a screenshot.

I want ask you several question in according to that.

The first of all - how to fix it safely?

What you recommend? Maybe I should turn off cpuspeed service?

Is it good idea to use it on servers?

Is it safe to use it on high-load servers?

With best regards,

Sergey Belogrud

[root@ds-mon11t ~]# chkconfig --list cpuspeed

cpuspeed        0:off   1:on    2:on    3:on    4:on    5:on    6:off

[root@ds-mon11t ~]# chkconfig --level 3 cpuspeed off

[root@ds-mon11t ~]# service cpuspeed stop

[root@ds-mon11t ~]# chkconfig --list cpuspeed

cpuspeed        0:off   1:on    2:on    3:off   4:on    5:on    6:off

[root@ds-mon11t ~]#

[root@ds-mon11t ~]# yum update cpuspeed

Loaded plugins: product-id, security, subscription-manager

This system is not registered to Red Hat Subscription Management. You can use subscription-manager to register.

Setting up Update Process

No Packages marked for Update

[root@ds-mon11t ~]# yum list duplicates cpuspeed

Loaded plugins: product-id, security, subscription-manager

This system is not registered to Red Hat Subscription Management. You can use subscription-manager to register.

Installed Packages

cpuspeed.x86_64                                           1:1.5-19.el6                                            @anaconda-RedHatEnterpriseLinux-201301301459.x86_64/6.4

[root@ds-mon11t ~]#

After I received recommendation from RedHat technical support. I update this package.

[support@centos00 20140206]$ scp ~/mnt/share/redhat/cpuspeed-1.5-20.el6_4.x86_64.rpm bsv@ds-mon11t:/tmp

bsv@ds-mon11t's password:

cpuspeed-1.5-20.el6_4.x86_64.rpm                                                                                                       100%   37KB  36.8KB/s   00:00

[support@centos00 20140206]$

[root@ds-mon11t ~]# yum localinstall /tmp/cpuspeed-1.5-20.el6_4.x86_64.rpm

Loaded plugins: product-id, security, subscription-manager

This system is not registered to Red Hat Subscription Management. You can use subscription-manager to register.

Setting up Local Package Process

Examining /tmp/cpuspeed-1.5-20.el6_4.x86_64.rpm: 1:cpuspeed-1.5-20.el6_4.x86_64

Marking /tmp/cpuspeed-1.5-20.el6_4.x86_64.rpm as an update to 1:cpuspeed-1.5-19.el6.x86_64

Resolving Dependencies

--> Running transaction check

---> Package cpuspeed.x86_64 1:1.5-19.el6 will be updated

---> Package cpuspeed.x86_64 1:1.5-20.el6_4 will be an update

--> Finished Dependency Resolution

Dependencies Resolved

=========================================================================================================================================================================

Package                           Arch                            Version                                  Repository                                              Size

=========================================================================================================================================================================

Updating:

cpuspeed                          x86_64                          1:1.5-20.el6_4                           /cpuspeed-1.5-20.el6_4.x86_64                           64 k

Transaction Summary

=========================================================================================================================================================================

Upgrade       1 Package(s)

Total size: 64 k

Is this ok [y/N]: y

Downloading Packages:

Running rpm_check_debug

Running Transaction Test

Transaction Test Succeeded

Running Transaction

Updating   : 1:cpuspeed-1.5-20.el6_4.x86_64                                                                                                                        1/2

Cleanup    : 1:cpuspeed-1.5-19.el6.x86_64                                                                                                                          2/2

Verifying  : 1:cpuspeed-1.5-20.el6_4.x86_64                                                                                                                        1/2

Verifying  : 1:cpuspeed-1.5-19.el6.x86_64                                                                                                                          2/2

Updated:

cpuspeed.x86_64 1:1.5-20.el6_4

Complete!

[root@ds-mon11t ~]#

[root@ds-mon11t ~]# chkconfig --level 3 cpuspeed on

[root@ds-mon11t ~]#

[root@ds-mon11t ~]# service cpuspeed start

Enabling ondemand cpu frequency scaling:                   [  OK  ]

[root@ds-mon11t ~]#

-----Original Message-----

From: Red Hat Support [mailto:support@redhat.com]

Sent: Thursday, February 06, 2014 4:28 PM

To: Aleksandrov Aleksey Valerievich; Belogrud Sergey Vladimirovich

Subject: (Waiting on Customer) Case #01035836 (Errors while service cpuspeed starts during boot sequence) [ ref:00DA0000000HxWHMA0.500A000000HnO0YIAV:ref ]

---------------------------------------

|     	Case Information        	|

---------------------------------------

https://access.redhat.com/support/cases/01035836

Case Title   	: Errors while service cpuspeed starts during boot sequence

Case Number  	: 01035836

Case Open Date   : 2014-02-06 10:58:57

Most recent comment: On 2014-02-06 16:27:33, Dutta, Ratul commented:

"Hello Sergey,

Welcome to Red Hat Technical Support. My name is Ratul and I have taken ownership for Kernel team. I have gone through the case description and also checked the provided error messages in provided screen-shot.

After checking the error messages, it looks like the errors does not produce any negative impact on the system.

Reason: The cpuspeed script needs to first check for the existence of ./cpufreq/scaling_available_frequencies. If that sysfs file does not exist, it should use the value of ./cpufreq/cpuinfo_max_freq but this is not happening and hence the error message.

We would suggest you to Upgrade the cpuspeed package to cpuspeed-1.5-20.el6_4 from below errata:

+++

http://rhn.redhat.com/errata/RHBA-2013-1153.html

+++

**I also would like to provide some information about "cpuspeed":

This daemon monitorss the system's idle percentage and reduces or raises the CPU's clock speeds and voltages accordingly to minimize power comsumption when idle and maximize performance when needed.

Refer the manual page for more detailed information

# man cpuspeed

Following link gives better idea on Power Management on Red Hat Enterprise Linux 6:

+++

https://access.redhat.com/site/documentation/en-US/Red_Hat_Enterprise_Linux/6/html-single/Power_Management_Guide/index.html

+++

This "cpuspeed" daemon decide the value based on the cpufreq governor. The cpufreq governor determines what mechanism that the cpufreq driver uses to adjust the cpu frequency. There are different policies available for cpufreq governor like:

- Performance: The CPUfreq governor "performance" sets the CPU statically to the highest frequency within the borders of scaling_min_freq and scaling_max_freq.

- Userspace: The CPUfreq governor "userspace" allows the user, or any userspace program running with UID "root", to set the CPU to a specific frequency by making a sysfs file "scaling_setspeed" available in the CPU-device directory.

- Ondemand: The CPUfreq governor "ondemand" sets the CPU depending on the current usage. To do this the CPU must have the capability to switch the frequency very quickly.

If you want  your CPU always run on maximum speed, you can disable the "cpuspeed" daemon. Once "cpuspeed" daemon is disabled the processor will always run on maximum speed.

If you haven't changed any configuration on the server then the default policy is "Ondemand".

"Most of the customers don't make it enabled/disabled as it doesn't cause any overhead in OS. And its enabled by default which is recommendation."

"Actually this service is enabled for those who are more power-conscious and who don't want their servers consuming power when it isn't necessary. After disabling it, it will consume power everytime independent of the state (whether idle, loaded) of the server."

The recommendation is keeping on this service on server/client/laptops/desktops as it doesn't keep any overhead on operating system. But if you need to disable it, you can disable it."

Let us know if you have any further questions.

Regards,

Ratul Dutta

Red Hat Global Support."

---------------------------------------

-----Original Message-----

From: Red Hat Support [mailto:support@redhat.com]

Sent: Friday, February 07, 2014 11:23 AM

To: Aleksandrov Aleksey Valerievich; Belogrud Sergey Vladimirovich

Subject: (Closed) Case #01035836 (Errors while service cpuspeed starts during boot sequence) [ ref:00DA0000000HxWHMA0.500A000000HnO0YIAV:ref ]

---------------------------------------

|     	Case Information        	|

---------------------------------------

https://access.redhat.com/support/cases/01035836

Case Title   	: Errors while service cpuspeed starts during boot sequence

Case Number  	: 01035836

Case Open Date   : 2014-02-06 10:58:57

Most recent comment: On 2014-02-07 11:23:15, Administrator, TCS commented:

"Hello.

Thank you for your advices.

I have updated package cpuspeed. I have downloaded package cpuspeed-1.5-20.el6_4.x86_64.rpm and then install it.

I hope it will close question about errors during boot process.

Your answer was very helpful for me.

Thanks again.

Now I suppose this case should be closed.

With best regards,

Sergey Belogrud"

---------------------------------------