From: Belogrud Sergey Vladimirovich

Sent: Wednesday, February 11, 2015 6:06 PM

To: 'Grigory Nikonov'

Cc: Pavlov Dmitriy Stanislavovich; Ovchinnikov Dmitriy Sergeevich

Subject: Информация по свободным PCIe разъёмам в контурах

Добрый день Григорий.

Ниже информация по свободным PCIe разъёмам в серверах, задействованных для работы ПО GreenPlum и в серверах, использовавшихся в тестировании ssd-дисков.

Для работы ПО GreenPlum используются серверы модели HP ProLiant DL380p Gen8 и серверы модели HP ProLiant DL380 G7. Все серверы имеют идентичную своему типу конфигурацию.

В серверах типа Gen8 имеются два свободных PCIe разъёма и в серверах типа G7 имеется один свободный PCIe разъём.

[root@sdw7 ~]# dmidecode -t system | grep -E Manufacturer\|"Product Name"

Manufacturer: HP

Product Name: ProLiant DL380p Gen8

[root@sdw7 ~]# dmidecode -t slot | grep -E "DMI type"\|Designation\|Type\|"Current Usage"\|"Bus Address"

Handle 0x0901, DMI type 9, 17 bytes

Designation: PCI-E Slot 1

Type: x16 PCI Express 3

Current Usage: Available

Bus Address: 0000:04:00.0

Handle 0x0902, DMI type 9, 17 bytes

Designation: PCI-E Slot 2

Type: x8 PCI Express 3

Current Usage: In Use

Bus Address: 0000:07:00.0

Handle 0x0903, DMI type 9, 17 bytes

Designation: PCI-E Slot 3

Type: x4 PCI Express 2 x8

Current Usage: Available

Bus Address: 0000:0a:00.0

[root@sdw7 ~]#

[root@sdw8 ~]# dmidecode -t system | grep -E Manufacturer\|"Product Name"

Manufacturer: HP

Product Name: ProLiant DL380 G7

[root@sdw8 ~]# dmidecode -t slot | grep -E "DMI type"\|Designation\|Type\|"Current Usage"\|"Bus Address"

Handle 0x0901, DMI type 9, 17 bytes

Designation: PCI-E Slot 1

Type: x8 PCI Express

Current Usage: In Use

Bus Address: 0000:08:00.0

Handle 0x0902, DMI type 9, 17 bytes

Designation: PCI-E Slot 2

Type: x4 PCI Express

Current Usage: Available

Bus Address: 0000:0b:00.0

Handle 0x0903, DMI type 9, 17 bytes

Designation: PCI-E Slot 3

Type: x4 PCI Express

Current Usage: In Use

Bus Address: 0000:0e:00.0

[root@sdw8 ~]#

Для тестирования ssd-дисков использовались серверы модели HP  ProLiant DL580 G7.

Из особенностей, на одном из серверов были доступны два разъёма «Type: x8 PCI Express 2 x16» (он и показал лучшую производительность и на нём проводилось фактическое тестирование), а на другом два разъёма «Type: x4 PCI Express 2 x8» (на этом сервере оказался не рабочим один из ssd-дисков).

[root@m1-sasetl01t ~]# dmidecode -t system | grep -E Manufacturer\|"Product Name"

Manufacturer: HP

Product Name: ProLiant DL580 G7

[root@m1-sasetl01t ~]#

[root@m1-sasetl01t ~]# dmidecode -t slot | grep -E "DMI type"\|Designation\|Type\|"Current Usage"\|"Bus Address"

Handle 0x0907, DMI type 9, 17 bytes

Designation: PCI-E Slot 7

Type: x4 PCI Express 2 x8

Current Usage: In Use

Bus Address: 0000:05:00.0

Handle 0x0908, DMI type 9, 17 bytes

Designation: PCI-E Slot 8

Type: x4 PCI Express 2 x8

Current Usage: In Use

Bus Address: 0000:08:00.0

Handle 0x0909, DMI type 9, 17 bytes

Designation: PCI-E Slot 9

Type: x8 PCI Express 2 x16

Current Usage: Available

Bus Address: 0000:0b:00.0

Handle 0x090A, DMI type 9, 17 bytes

Designation: PCI-E Slot10

Type: x4 PCI Express 2 x8

Current Usage: In Use

Bus Address: 0000:0e:00.0

Handle 0x090B, DMI type 9, 17 bytes

Designation: PCI-E Slot11

Type: x8 PCI Express 2 x16

Current Usage: Available

Bus Address: 0000:11:00.0

[root@m1-sasetl01t ~]#

[root@m1-sasfront02p ~]# dmidecode -t system | grep -E Manufacturer\|"Product Name"

Manufacturer: HP

Product Name: ProLiant DL580 G7

[root@m1-sasfront02p ~]# dmidecode -t slot | grep -E "DMI type"\|Designation\|Type\|"Current Usage"\|"Bus Address"

Handle 0x0907, DMI type 9, 17 bytes

Designation: PCI-E Slot 7

Type: x4 PCI Express 2 x8

Current Usage: Available

Bus Address: 0000:05:00.0

Handle 0x0908, DMI type 9, 17 bytes

Designation: PCI-E Slot 8

Type: x4 PCI Express 2 x8

Current Usage: Available

Bus Address: 0000:08:00.0

Handle 0x0909, DMI type 9, 17 bytes

Designation: PCI-E Slot 9

Type: x8 PCI Express 2 x16

Current Usage: In Use

Bus Address: 0000:0b:00.0

Handle 0x090A, DMI type 9, 17 bytes

Designation: PCI-E Slot10

Type: x4 PCI Express 2 x8

Current Usage: Available

Bus Address: 0000:0e:00.0

Handle 0x090B, DMI type 9, 17 bytes

Designation: PCI-E Slot11

Type: x8 PCI Express 2 x16

Current Usage: In Use

Bus Address: 0000:11:00.0

[root@m1-sasfront02p ~]#

С уважением,

Сергей Белогруд

Старший системный администратор

1-й Волоколамский проезд, дом 10, стр. 1

Москва, 123060, Россия

Тел.: +7 (495) 648 1000 (вн. 2173)

Моб.: +7 (915) 046 4278

Факс: +7 (495) 645 5909

E-mail: s.belogrud@tinkoff.ru

www.tinkoff.ru

Это электронное письмо строго конфиденциально (включая все вложения). Данная информация предназначена только для получателя(ей). Если Вы не являетесь получателем, Вы не вправе каким бы то ни было образом читать, копировать, использовать или хранить данное сообщение. Просьба немедленно его удалить. Если Вы получили его по ошибке, пожалуйста, свяжитесь с Тинькофф Банком по т. +7 495 648 1000.

http://www.linuxquestions.org/questions/linux-hardware-18/how-can-i-check-pci-slots-remotely-293068/