After server was installed and after package hponcfg was installed too, we can tune iLO to best suite for our purposes.

Linux server installation

HPONCFG is installed automatically when Service Pack for ProLiant is installed. Download the

HPONCFG RPM package for Linux distributions from the HP website. Install the appropriate

package using the RPM installation utility.

For example, for a package installation, install the HPONCFG RPM package on Red Hat Enterprise

Linux 5 by entering the following command:

rpm -ivh hponcfg-4.0.1-2.linux.rpm

If you have an older version of the HPONCFG RPM package installed on the system, run the

following command to remove the older version before installing the new version of HPONCFG:

rpm –e hponcfg

The hp-ilo rpm package and the hp-health rpm package must be installed on the system

before installing the hponcfg rpm package.

After installation, the HPONCFG executable is located in the /sbin directory. Be sure that the

appropriate Management Interface Driver is installed. For details about where to obtain this driver

and file, see “HPONCFG requirements” (page 24).

[root@smdw_prod tmp]# hponcfg -w ./ilo.base.tmp

[root@smdw_prod tmp]# vim ./ilo.base.tmp

<!-- HPONCFG VERSION = "4.1.0" -->

<!-- Generated 5/8/2014 15:27:55 -->

<RIBCL VERSION="2.1">

<LOGIN USER_LOGIN="Administrator" PASSWORD="password">

<DIR_INFO MODE="write">

<MOD_DIR_CONFIG>

<DIR_AUTHENTICATION_ENABLED VALUE = "Y"/>

<DIR_LOCAL_USER_ACCT VALUE = "Y"/>

<DIR_SERVER_ADDRESS VALUE = "ldap.mgmt.tcsbank.ru"/>

<DIR_SERVER_PORT VALUE = "636"/>

<DIR_OBJECT_DN VALUE = ""/>

<DIR_OBJECT_PASSWORD VALUE = ""/>

<DIR_USER_CONTEXT_1 VALUE = "dn=tcsbank,dn=ru"/>

<DIR_USER_CONTEXT_2 VALUE = ""/>

<DIR_USER_CONTEXT_3 VALUE = ""/>

</MOD_DIR_CONFIG>

</DIR_INFO>

<RIB_INFO MODE="write">

<MOD_NETWORK_SETTINGS>

<SPEED_AUTOSELECT VALUE = "Y"/>

<NIC_SPEED VALUE = "10"/>

<FULL_DUPLEX VALUE = "N"/>

<IP_ADDRESS VALUE = "10.219.8.154"/>

<SUBNET_MASK VALUE = "255.255.255.0"/>

<GATEWAY_IP_ADDRESS VALUE = "10.219.8.254"/>

<DNS_NAME VALUE = "CZ314824F9"/>

<PRIM_DNS_SERVER value = "10.216.40.4"/>

<DHCP_ENABLE VALUE = "N"/>

<DOMAIN_NAME VALUE = "ilo.tcsbank.ru"/>

<DHCP_GATEWAY VALUE = "N"/>

<DHCP_DNS_SERVER VALUE = "N"/>

<DHCP_STATIC_ROUTE VALUE = "N"/>

<DHCP_WINS_SERVER VALUE = "N"/>

<REG_WINS_SERVER VALUE = "N"/>

<PRIM_WINS_SERVER value = "0.0.0.0"/>

<STATIC_ROUTE_1 DEST = "0.0.0.0" GATEWAY = "0.0.0.0"/>

<STATIC_ROUTE_2 DEST = "0.0.0.0" GATEWAY = "0.0.0.0"/>

<STATIC_ROUTE_3 DEST = "0.0.0.0" GATEWAY = "0.0.0.0"/>

</MOD_NETWORK_SETTINGS>

</RIB_INFO>

<USER_INFO MODE="write">

<ADD_USER

USER_NAME = "tan"

USER_LOGIN = "tan"

PASSWORD = "%user_password%">

<ADMIN_PRIV value = "Y"/>

<REMOTE_CONS_PRIV value = "Y"/>

<RESET_SERVER_PRIV value = "Y"/>

<VIRTUAL_MEDIA_PRIV value = "Y"/>

<CONFIG_ILO_PRIV value = "Y"/>

</ADD_USER>

</USER_INFO>

</LOGIN>

</RIBCL>

[root@smdw_prod tmp]# hponcfg -g

HP Lights-Out Online Configuration utility

Version 4.1.0 Date 1/3/2013 (c) Hewlett-Packard Company, 2013

Firmware Revision = 1.61 Device type = iLO 3 Driver name = hpilo

Host Information:

Server Name: smdw

Server Number:

[root@smdw_prod tmp]#

[root@smdw_prod tmp]# hponcfg -?

HP Lights-Out Online Configuration utility

Version 4.1.0 Date 1/3/2013 (c) Hewlett-Packard Company, 2013

Firmware Revision = 1.61 Device type = iLO 3 Driver name = hpilo

USAGE:

hponcfg  -?

hponcfg  -h

hponcfg  -m minFw

hponcfg  -r [-m minFw ]

hponcfg  [-a] -w filename [-m minFw]

hponcfg  -g [-m minFw]

hponcfg  -f filename [-l filename] [-s namevaluepair] [-v] [-m minFw]

hponcfg  -i [-l filename] [-s namevaluepair] [-v] [-m minFw]

-h,  --help           Display this message

-?                    Display this message

-r,  --reset          Reset the Management Processor to factory defaults

-f,  --file           Get/Set Management Processor configuration from "filename"

-i,  --input          Get/Set Management Processor configuration from the XML input

received through the standard input stream.

-w,  --writeconfig    Write the Management Processor configuration to "filename"

-a,  --all            Capture complete Management Processor configuration to the file.

This should be used along with '-w' option

-l,  --log            Log replies to "filename"

-v,  --xmlverbose     Display all the responses from Management Processor

-s,  --substitute     Substitute variables present in input config file

with values specified in "namevaluepairs"

-g,  --get_hostinfo   Get the Host information

-m,  --minfwlevel     Minimum firmware level

[root@smdw_prod tmp]#

Obtaining the basic configuration

Use HPONCFG to obtain a basic configuration from iLO 4 by executing the utility from the command

line without specifying an input file. You must provide the name of the output file on the command

line.

For example:

hponcfg /w config.xml

In this example, the utility indicates that it obtained the data successfully and wrote the data to the

output file.

The following is an example of a typical output file:

<!-- HPONCFG VERSION = "4.2.0.0" -->

<!-- Generated 08/20/13 20:14:12 -->

<RIBCL VERSION="2.1">

<LOGIN USER_LOGIN="Administrator" PASSWORD="password">

<DIR_INFO MODE="write">

<MOD_DIR_CONFIG>

<DIR_AUTHENTICATION_ENABLED VALUE = "N"/>

<DIR_LOCAL_USER_ACCT VALUE = "Y"/>

<DIR_SERVER_ADDRESS VALUE = ""/>

<DIR_SERVER_PORT VALUE = "636"/>

<DIR_OBJECT_DN VALUE = ""/>

<DIR_OBJECT_PASSWORD VALUE = ""/>

<DIR_USER_CONTEXT_1 VALUE = ""/>

<DIR_USER_CONTEXT_2 VALUE = ""/>

<DIR_USER_CONTEXT_3 VALUE = ""/>

</MOD_DIR_CONFIG>

</DIR_INFO>

<RIB_INFO MODE="write">

<MOD_NETWORK_SETTINGS>

<SPEED_AUTOSELECT VALUE = "Y"/>

<NIC_SPEED VALUE = "10"/>

<FULL_DUPLEX VALUE = "N"/>

HPONCFG utility 27 <DHCP_ENABLE VALUE = "Y"/>

<DHCP_GATEWAY VALUE = "Y"/>

<DHCP_DNS_SERVER VALUE = "Y"/>

<DHCP_STATIC_ROUTE VALUE = "Y"/>

<DHCP_WINS_SERVER VALUE = "Y"/>

<REG_WINS_SERVER VALUE = "Y"/>

<IP_ADDRESS VALUE = "192.168.1.3"/>

<SUBNET_MASK VALUE = "255.255.255.0"/>

<GATEWAY_IP_ADDRESS VALUE = "192.168.1.1"/>

<DNS_NAME VALUE = "ILODNSNAME"/>

<DOMAIN_NAME VALUE = "hp.com"/>

<PRIM_DNS_SERVER value = "192.168.1.2"/>

<SEC_DNS_SERVER value = "0.0.0.0"/>

<TER_DNS_SERVER value = "0.0.0.0"/>

<PRIM_WINS_SERVER value = "0.0.0.0"/>

<SEC_WINS_SERVER value = "0.0.0.0"/>

<STATIC_ROUTE_1 DEST = "0.0.0.0" GATEWAY = "0.0.0.0"/>

<STATIC_ROUTE_2 DEST = "0.0.0.0" GATEWAY = "0.0.0.0"/>

<STATIC_ROUTE_3 DEST = "0.0.0.0" GATEWAY = "0.0.0.0"/>

</MOD_NETWORK_SETTINGS>

</RIB_INFO>

<USER_INFO MODE="write">

<ADD_USER

USER_NAME = "admin"

USER_LOGIN = "admin"

PASSWORD = "%user_password%">

<ADMIN_PRIV value = "Y"/>

<REMOTE_CONS_PRIV value = "Y"/>

<RESET_SERVER_PRIV value = "Y"/>

<VIRTUAL_MEDIA_PRIV value = "Y"/>

<CONFIG_ILO_PRIV value = "Y"/>

</ADD_USER>

</USER_INFO>

</LOGIN>

</RIBCL>

NOTE: For security reasons, user passwords are not returned.

Obtaining a specific configuration

Obtain a specific configuration using the appropriate XML input file.

For example, the following is the contents of a typical XML input file:

get_global.xml

:

<!-- Sample file for Get Global command -->

<RIBCL VERSION="2.0">

<LOGIN USER_LOGIN="x" PASSWORD="x">

<RIB_INFO MODE="read">

<GET_GLOBAL_SETTINGS />

</RIB_INFO>

</LOGIN>

</RIBCL>

The XML commands are read from the input file get_global.xml and are processed by the

device:

hponcfg /f get_global.xml /l log.txt > output.txt

The requested information is returned in the log file, which, in this example, is named log.txt.

<GET_GLOBAL_SETTINGS>

<!-- A session timeout value of zero means that the timeout is set to infinite. -->

<SESSION_TIMEOUT VALUE="0"/>

28 HPONCFG online configuration utility <F8_PROMPT_ENABLED VALUE="Y"/>

<F8_LOGIN_REQUIRED VALUE="N"/>

<HTTPS_PORT VALUE="443"/>

<HTTP_PORT VALUE="80"/>

<REMOTE_CONSOLE_PORT VALUE="17990"/>

<VIRTUAL_MEDIA_PORT VALUE="17988"/>

<SNMP_ACCESS_ENABLED VALUE="Y"/>

<SNMP_PORT VALUE="161"/>

<SNMP_TRAP_PORT VALUE="162"/>

<SSH_PORT VALUE="22"/>

<SSH_STATUS VALUE="Y"/>

<SERIAL_CLI_STATUS VALUE="Enabled-Authentication Required"/>

<SERIAL_CLI_SPEED VALUE="9600"/>

<VSP_LOG_ENABLE VALUE="N"/>

<MIN_PASSWORD VALUE="8"/>

<AUTHENTICATION_FAILURE_LOGGING VALUE="Enabled-every 3rd failure"/>

<RBSU_POST_IP VALUE="Y"/>

<ENFORCE_AES VALUE="N"/>

<IPMI_DCMI_OVER_LAN_ENABLED VALUE="Y"/>

<REMOTE_SYSLOG_ENABLE VALUE="N"/>

<REMOTE_SYSLOG_PORT VALUE="514"/>

<REMOTE_SYSLOG_SERVER_ADDRESS VALUE=""/>

<ALERTMAIL_ENABLE VALUE="N"/>

<ALERTMAIL_EMAIL_ADDRESS VALUE=""/>

<ALERTMAIL_SENDER_DOMAIN VALUE=""/>

<ALERTMAIL_SMTP_PORT VALUE="25"/>

<ALERTMAIL_SMTP_SERVER VALUE=""/>

<PROPAGATE_TIME_TO_HOST VALUE="Y"/>

</GET_GLOBAL_SETTINGS>

Setting a configuration

Set a specific configuration by using the command format:

hponcfg /f add_user.xml /l log.txt

In this example, the input file has contents:

<RIBCL VERSION="2.0">

<LOGIN USER_LOGIN="x" PASSWORD="x">

<USER_INFO MODE="write">

<ADD_USER

USER_NAME="Landy9"

USER_LOGIN="mandy8"

PASSWORD="floppyshoes">

<ADMIN_PRIV value ="No"/>

<REMOTE_CONS_PRIV value ="Yes"/>

<RESET_SERVER_PRIV value ="No"/>

<VIRTUAL_MEDIA_PRIV value ="No"/>

<CONFIG_ILO_PRIV value="Yes"/>

</ADD_USER>

</USER_INFO>

</LOGIN>

</RIBCL>

The specified user is added to the device.

Using variable substitution

HPONCFG enables you to specify variables in the XML RIBCL script and to assign values to those

variables when you run HPONCFG. This feature helps to avoid rewriting the XML script file every

time with different values. Anything enclosed by two percent sign (%) characters in the XML file is

considered a variable.

In this example, %username%, %loginname%, and %password% are variables:

HPONCFG utility 29

<!-- Add user with minimal privileges to test default setting of

assigned privileges to 'N' -->

<RIBCL version="1.2">

<LOGIN USER_LOGIN="x" PASSWORD="x">

<USER_INFO MODE="write">

<ADD_USER USER_NAME="%username%" USER_LOGIN="%loginname%" PASSWORD="%password%">

<RESET_SERVER_PRIV value="Y" />

<ADMIN_PRIV value="Y" />

</ADD_USER>

</USER_INFO>

</LOGIN>

</RIBCL>

Specify values for the variables when you run HPONCFG by using the substitute option. The

argument must be a string or variable name and value pairs must be separated by a comma (,).

The variable name and its value must be separated by an equal sign (=). For example:

hponcfg /f add_user.xml /s username=testuser,loginname=testlogin,password=testpasswd

In this example, %host_power% is a variable:

<RIBCL VERSION="2.0">

<LOGIN USER_LOGIN="adminname" PASSWORD="password">

<SERVER_INFO MODE="write">

<!-- Modify the HOST_POWER attribute to toggle power on the host server -->

<!-- HOST_POWER="No" (Turns host server power off) -->

<!-- A graceful shutdown will be attempted for ACPI-aware -->

<!-- operating systems configured to support graceful shutdown. -->

<!-- HOST_POWER="Yes" (Turns host server power on) -->

<SET_HOST_POWER HOST_POWER="%host_power%"/>

</SERVER_INFO>

</LOGIN>

</RIBCL>

• To power the system on, enter:

hponcfg /f Set_Host_Power.xml /s host_power=YES

• To power the system off, enter:

hponcfg /f Set_Host_Power.xml /s host_power=NO

Capturing and restoring a configuration

Use HPONCFG to capture basic configuration information in an XML readable file format. Use

this file to set or restore the iLO configuration. This feature is available with HPONCFG version

1.2 and later. HPONCFG writes the configuration information in the HP RIBCL format.

• To capture a configuration, you must specify the name and location of the output file on the

command line. See “Obtaining the basic configuration” (page 27) for details.

For example:

hponcfg /w config.xml

HPONCFG displays a message when it successfully writes the configuration information to

the output file as requested. For security reasons, the default user administrator and user

passwords are not captured in the configuration file or returned in the response. A variable

is provided in its place to use with the substitute option to provide a default password

for all users when restoring a configuration. Manually change the password before using the

file to restore the configuration.

• To restore a configuration, the file must be sent to HPONCFG as input using the /f or -f

option. Add a default password for all users using the substitute or s option.

For example:

30 HPONCFG online configuration utilityhponcfg /f config.xml /s user_password=password