20150827

How ot turn off hard drive with software.

hpssacli -> thelp modify

<target> modify disablepd [forced]

Disables (temporarily fails) the specified physical drive until either a hot

plug event or a reset of the controller. The target can be any valid

physical drive.

controller slot=5 physicaldrive 1E:1:1 modify disablepd

[root@hq-msk-gp-tst-sdw1 ~]# hpssacli controller all show config

Smart Array P410 in Slot 1                (sn: PACCRID11320CXP)

Internal Drive Cage at Port 1I, Box 1, OK

Internal Drive Cage at Port 2I, Box 1, OK

array A (SAS, Unused Space: 0  MB)

logicaldrive 1 (3.8 TB, RAID 5, OK)

physicaldrive 1I:1:1 (port 1I:box 1:bay 1, SAS, 600 GB, OK)

physicaldrive 1I:1:2 (port 1I:box 1:bay 2, SAS, 600 GB, OK)

physicaldrive 1I:1:3 (port 1I:box 1:bay 3, SAS, 600 GB, OK)

physicaldrive 1I:1:4 (port 1I:box 1:bay 4, SAS, 600 GB, OK)

physicaldrive 2I:1:5 (port 2I:box 1:bay 5, SAS, 600 GB, OK)

physicaldrive 2I:1:6 (port 2I:box 1:bay 6, SAS, 600 GB, Predictive Failure)

physicaldrive 2I:1:7 (port 2I:box 1:bay 7, SAS, 600 GB, OK)

physicaldrive 2I:1:8 (port 2I:box 1:bay 8, SAS, 600 GB, OK)

SEP (Vendor ID PMCSIERA, Model  SRC 8x6G) 250  (WWID: 50014380167EDDEF)

[root@hq-msk-gp-tst-sdw1 ~]# hpssacli controller slot=1 physicaldrive 2I:1:6 modify disablepd

Warning: The physical drive will be disabled until the next reboot, if the

drive is replaced, or if the drive is hot plugged. The failed drive

LED will display amber while the drive is in this state. Continue?

(y/n)y

Error: This command did not complete properly. Please check your configuration

for possible hardware or configuration problems. Use the "show" command

on devices to show additional details about the configuration.

[root@hq-msk-gp-tst-sdw1 ~]#

[root@hq-msk-gp-tst-sdw1 ~]# hpssacli controller slot=1 show config

Smart Array P410 in Slot 1                (sn: PACCRID11320CXP)

Internal Drive Cage at Port 1I, Box 1, OK

Internal Drive Cage at Port 2I, Box 1, OK

array A (SAS, Unused Space: 0  MB)

logicaldrive 1 (3.8 TB, RAID 5, Interim Recovery Mode)

physicaldrive 1I:1:1 (port 1I:box 1:bay 1, SAS, 600 GB, OK)

physicaldrive 1I:1:2 (port 1I:box 1:bay 2, SAS, 600 GB, OK)

physicaldrive 1I:1:3 (port 1I:box 1:bay 3, SAS, 600 GB, OK)

physicaldrive 1I:1:4 (port 1I:box 1:bay 4, SAS, 600 GB, OK)

physicaldrive 2I:1:5 (port 2I:box 1:bay 5, SAS, 600 GB, OK)

physicaldrive 2I:1:6 (port 2I:box 1:bay 6, SAS, 600 GB, Failed)

physicaldrive 2I:1:7 (port 2I:box 1:bay 7, SAS, 600 GB, OK)

physicaldrive 2I:1:8 (port 2I:box 1:bay 8, SAS, 600 GB, OK)

SEP (Vendor ID PMCSIERA, Model  SRC 8x6G) 250  (WWID: 50014380167EDDEF)

[root@hq-msk-gp-tst-sdw1 ~]#

[root@hq-msk-gp-tst-sdw1 ~]# hpssacli controller slot=1 physicaldrive 2I:1:6 show detail

Smart Array P410 in Slot 1

array A

physicaldrive 2I:1:6

Port: 2I

Box: 1

Bay: 6

Status: Failed

Last Failure Reason: Fail drive command

Drive Type: Data Drive

Interface Type: SAS

Size: 600 GB

Native Block Size: 512

Rotational Speed: 10000

Firmware Revision: HPD6

Serial Number: EA01PB80W9P81135

Model: HP      EG0600FBDSR

Current Temperature (C): 31

Maximum Temperature (C): 41

PHY Count: 2

PHY Transfer Rate: 6.0Gbps, Unknown

[root@hq-msk-gp-tst-sdw1 ~]

#	Here is how it looks for regular drive failure (it’s other server in other time).

[root@hq-msk-gp-prd-sdw2 ~]# hpssacli controller slot=0 physicaldrive 1I:1:4 show detail

Smart Array P410i in Slot 0 (Embedded)

array A

physicaldrive 1I:1:4

Port: 1I

Box: 1

Bay: 4

Status: Failed

Last Failure Reason: Deferred write error

Drive Type: Data Drive

Interface Type: SAS

Size: 600 GB

Native Block Size: 512

Rotational Speed: 10000

Firmware Revision: HPD6

Serial Number: EA01PB80WBVN1135

Model: HP      EG0600FBDSR

Current Temperature (C): 28

Maximum Temperature (C): 36

PHY Count: 2

PHY Transfer Rate: 6.0Gbps, Unknown

[root@hq-msk-gp-prd-sdw2 ~]#