#

#	I wanted to geit iLO ip addres via cli utility on Gen10 server.

#	But it was unsuccessful.

#

#	I find that, for working hponcfg need CHIF driver hpilo.

#	But where is it and how it could be received… I couldn’t find.

#

[root@ds-tms-kvm01p ~]# dmidecode -t system

# dmidecode 2.12-dmifs

SMBIOS 3.1 present.

# SMBIOS implementations newer than version 2.8 are not

# fully supported by this version of dmidecode.

Handle 0x008D, DMI type 1, 27 bytes

System Information

Manufacturer: HPE

Product Name: ProLiant DL360 Gen10

Version: Not Specified

Serial Number: CZJ812004C

UUID: 39373638-3935-5A43-4A38-313230303443

Wake-up Type: Power Switch

SKU Number: 867959-B21

Family: ProLiant

Handle 0x00AE, DMI type 32, 11 bytes

System Boot Information

Status: No errors detected

[root@ds-tms-kvm01p ~]#

[root@ds-tms-kvm01p ~]# cat > /etc/yum.repos.d/nexus-tcs-hp.repo

[nexus-tcs-hp]

name=Nexus - HPE Software Delivery Repository for spp

baseurl=http://repo-linux.tcsbank.ru/repository/hp/repo/spp-gen10/rhel/$releasever/$basearch/current

enabled=1

gpgcheck=0

repo_gpgcheck=0

skip_if_unavailable=true

proxy=_none_

[root@ds-tms-kvm01p ~]#

[root@ds-tms-kvm01p ~]# yum install hponcfg

...

Installed:

hponcfg.x86_64 0:5.4.0-0

...

[root@ds-tms-kvm01p ~]# rpm -ql hponcfg

/sbin/hponcfg

/usr/lib64/libhponcfg64.so

/usr/share/doc/hponcfg

/usr/share/doc/hponcfg/readme.txt

/usr/share/smartupdate/hponcfg/component.xml

[root@ds-tms-kvm01p ~]#

[root@ds-tms-kvm01p ~]# hponcfg

HP Lights-Out Online Configuration utility

Version 5.4.0 Date 8/6/2018 (c) 2005,2018 Hewlett Packard Enterprise Development LP

ERROR: CpqCiCreateFunc() 0 time failed.

Driver Error Code:(1,1h).

Driver Error Message: CPQCIDRV driver is not loaded.

ERROR: CpqCiCreateFunc() 1 time failed.

Driver Error Code:(1,1h).

Driver Error Message: CPQCIDRV driver is not loaded.

ERROR: A general system error occurred while detecting Management Processor.

ACTION REQUIRED: Check if iLO and iLO driver are up and running.

[root@ds-tms-kvm01p ~]#

https://community.hpe.com/t5/ProLiant-Servers-ML-DL-SL/DL-gen10-conrep-error/td-p/7011436#.XRXYP4gzYwE

https://community.hpe.com/t5/ProLiant-Servers-ML-DL-SL/Driver-Error-Message-CPQCIDRV-driver-is-not-loaded/td-p/6968577#.XRXVwYgzYwE

https://support.hpe.com/hpsc/doc/public/display?docId=a00018323en_us

https://downloads.linux.hpe.com/SDR/project/mcp/