From: Alekhin, Dmitry [mailto:dmitry.alekhin@hp.com]

Sent: Wednesday, June 18, 2014 1:49 AM

To: Belogrud Sergey Vladimirovich; Velichkin, Vladimir; Support Desk

Cc: Stepina, Lilia; Aleksandrov Aleksey Valerievich; Chernyh Aleksandr Vladimirovich; Leonov Aleksey Vladimirovich; Terentev Aleksey; Zbiranik Evgeniy Aleksandrovich; Kartsev Konstantin; Novikov Matvey Sergeevich; Polyakov Valeriy; Ryabov Aleksey; Sotnichenko Sergey; Varakuta Sergey; Gus Konstantin Mikhaylovich

Subject: RE: Подтверждение запланированных работ

Добрый вечер, Сергей,

Проблема с батарейками на серверах, выглядит как аномальная, и я не наблюдал полностью повторяющегося сценария, с не заряжающимися батареями нигде, кроме как в данных заявках.

Было бы странно, если бы в каждой замене суперконденсатора, еще бы менялся модуль FBWC, в придачу с контроллером, так как вся точечная диагностика в таком случае сводится на нет .

Из общего, что я мог найти в двух заявках – это прошивка контроллера(p420) 5.22. в новой-же прошивке 5.24, есть исправление, которое решает проблему заряда модуля.

| Version:5.42 (31 Mar 2014) |  |
| --- | --- |
|  | Fixes |
|  | Upgrade Requirement: Recommended - HP recommends users update to this version at their earliest convenience. Problems Fixed: The following issues are resolved with firmware version 5.42: · Communication between the HP Integrated Lights-Out (iLO) and the HP Smart Array controller may be lost resulting in an IML entry indicating a failed controller. · In some cases, incorrect drive temperatures were reported in an HP D2220sb enclosure, resulting in a system shutdown. · VMware might hang when a VM with 2 or more RDMs used a Smart Array controller to access an HP P2000 G3 or HP MSA 2040 SAN storage device. · Improved RAID 1 performance for small block random write workloads on SSD configurations. · A controller lockup could occur if the controller was configured with a SmartCache smaller than 8GB. · On some power cycles, the supercap would always report that it was charging, even though charging had completed.. |

Есть большая вероятность того, что данная чехарда с батареями есть не что иное, как уже исправленный баг.

Поэтому, как основная рекомендация – обновление микрокода контроллера до версии 5.42.

После обновления потребуется перезагрузить сервер и подождать около 10-15 минут для полного заряда, далее собрать ADU-Report.

Дополнительно у меня есть рекомендации по поводу использования утилиты hpssa, как замены  hpacu, это немного удобнее, т.к. ADU не сохраняет Controller-Debug log на контроллерах для gen8  серверов.

В последующих заявках я обязательно учту Ваши рекомендации, большое спасибо за информацию.

Best Regards,

Dmitry Alekhin

RSAA

+79104995717

From: Belogrud Sergey Vladimirovich [mailto:s.belogrud@tcsbank.ru]

Sent: 18 июня 2014 г. 0:00

To: Alekhin, Dmitry; Velichkin, Vladimir; Support Desk

Cc: Stepina, Lilia; Aleksandrov Aleksey Valerievich; Chernyh Aleksandr Vladimirovich; Leonov Aleksey Vladimirovich; Terentev Aleksey; Zbiranik Evgeniy Aleksandrovich; Kartsev Konstantin; Novikov Matvey Sergeevich; Polyakov Valeriy; Ryabov Aleksey; Sotnichenko Sergey; Varakuta Sergey; Gus Konstantin Mikhaylovich

Subject: RE: Подтверждение запланированных работ

Здравствуйте.

Случилось то, что уже многократно имело место быть и чего я и просил Вас избежать.

Была заменена аккумуляторная батарея интегрированного в материнскую плату raid-контроллера на сервере hq-msk-gp-tst-sdw8 (HP CASE 4648607120).

К сожалению батарея через несколько минут сменила своё состояние на Recharging и, как уже многократно было замечено, в этом состоянии она и остаётся не зависимо от того сколько дней ждать.

Напротив, в случаях удачных замен, обычно, когда сразу меняется и кэш и батарея, состояние батареи после замены устанавливается в “OK” и остаётся таковым. При этом нет ни какой нужды в ожидании конца зарядки батареи в течении 12 часов или более, на что часто ссылаются как на информацию из документации.

Систематическая практика такого подхода HP к замене относительно не дорогих этих конкретных комплектующих только возмущает своих заказчиков. Трудно понять на чём HP пытается сэкономить гоняя своих инженеров по нескольку раз менять батарейки. Скорее всего это очевидное упущение в методике и логистике работ по ремонту неисправности батареи raid-контроллера. И я полагаю, что чем раньше об этом просигнализировать, тем в большем выигрыше все окажутся. Если это возможно, то я бы попросил Вас, передать моё возмущение в службу HP, которая определяет порядок и состав замены запчастей. Я надеюсь они очень быстро заметят имеющее место упущение.

[root@sdw8 ~]# hpacucli controller all show status

Smart Array P420 in Slot 2

Controller Status: OK

Cache Status: OK

Battery/Capacitor Status: OK

Smart Array P420i in Slot 0 (Embedded)

Controller Status: OK

Cache Status: OK

Battery/Capacitor Status: Recharging

[root@sdw8 ~]#

http://downloads.linux.hp.com/repo/spp/rhel/6.2/x86_64/current/

HP Smart Storage Administrator CLI for Linux environments

Version 1.60-17.0 (linuxX64)

12/5/2013

Description

-----------

The HP Smart Storage Administrator CLI (HPSSACLI) is a commandline-based

disk configuration program for Smart Array Controllers.

* All other product names mentioned herein may be trademarks of their

respective companies.

Support

-------

Supported Operating Systems

SuSE Linux Enterprise Server 10 (x86_64)

SuSE Linux Enterprise Server 11 (x86_64)

Red Hat Enterprise Linux 5 (x86_64)

Red Hat Enterprise Linux 6 (x86_64) with the "Compatibility Package"

installed to include the 32-bit libraries.

Debian Sarge 64 bit

Debian Etch (amd64)

* All these platforms need to have the following driver upgrades:

CISS driver version 2.4.50 or later

These drivers and Fibre Channel HBA drivers are available from

http://www.hp.com.

Supported Controllers

Smart Array P410i Controller

Smart Array P411 Controller

Smart Array P212 Controller

Smart Array P712m Controller

Smart Array B110i SATA RAID

Smart Array P812 Controller

Smart Array P220i Controller

Smart Array P222 Controller

Smart Array P420 Controller

Smart Array P420i Controller

Smart Array P421 Controller

Smart Array P822 Controller

Smart Array P721m Controller

Dynamic Smart Array B320i RAID

Dynamic Smart Array B120i RAID

Smart Array P430 Controller

Smart Array P431 Controller

Smart Array P731m Controller

Smart Array P230i Controller

Smart Array P530 Controller

Smart Array P531 Controller

Smart Array P830 Controller

Smart Array P830i Controller

Installing & Running the HP Smart Storage Administrator CLI

--------------------------------------------------------------

Installation:

HPSSACLI is distributed using Red Hat Package Manager (RPM) packages and

.deb packages for Debian.

If there is an older version of HPSSACLI installed on the system, please

use "rpm -e hpssacli" to remove it first. Use "rpm -i <file name>" to

install the application.

Please type "man rpm" for other details regarding RPM.

NOTE: On Debian systems, a ".deb" package file can be installed using the

"dpkg -i hpssacli.xxx.deb" command.  The dpkg -r hpssacli command

is used to remove the package.

The software resides in /opt/hp/hpssacli and the executable

name is hpssacli which is located in /usr/sbin.

When attempting to run HPSSACLI on any HP-supported version of Linux, it is

highly recommended that you install the latest version of the HP ProLiant

Smart Array Controller driver for your intended Linux operating system.

Drivers may be obtained by clicking on "Support & Drivers" from HP's

website at:  http://www.hp.com

For Debian 64 bit:

dpkg -i --force-architecture <file name>. hpssacli requires the libunwind

bit package to be installed.

Run the HPSSACLI:

Make sure that you have gone through the following checklist:

. You run hpssacli as root.

To start the application:

If /usr/sbin is already added in your path, just type:

"hpssacli".

More options can be found by typing "hpssacli help".  Also see the "Using

HPSSACLI" section of this document, located after Feedback.

Exiting HPSSACLI:

1. To exit the HPSSACLI, type "exit" while at the CLI command console.

Additional Notes

----------------

1.  After creating or deleting logical drives with HPSSACLI, under some

special circumstances, the device name mapping might change after you

first reboot the system. Also, Linux might see previously not visible

logical drives after rebooting. This name mapping problem is common to

all SCSI devices on Linux.

2.  Do not insert or remove a controller driver module when hpssacli is

running.

3.  Please stop the HP Storage Agents when deleting logical drives

since HP Storage Agents keeps all logical drives open.

4.  If all physical drives on a controller fail or are removed, software keys

will not be saved to the controller.

5.  HPSSACLI may appear unresponsive during heavy array controller I/O

conditions, such as during a partition format or a logical drive rebuild.

6.  HPSSACLI will not prevent the deletion of logical drives on fibre

controllers that have mounted partitions.

7.  When using Linux as the host operating system, if a Smart Array

controller has 16 or more volumes configured, deleting any volume in

HPSSACLI will not update the /proc/partitions to reflect the deletion of

the device node associated with that volume. Rebooting the server will

cause the update to happen. Note that there is also a limitation that

only the first 16 volumes configured will be available to the operating

system.

8.  On RHEL5 systems, for this version of the HPSSACLI version 3.6.18-10 or

newer of the HP ProLiant Smart Array Controller Driver for Red Hat

Enterprise Linux 5 (x86, x86-64) is required. Drivers may be obtained by

clicking on �Support & Drivers� from HP�s website at:  http://www.hp.com

Notes for HPSSA Scripting

-----------------------

* HPSSA scripting (HPSSASCRIPTING) now uses a new engine to auto configure

volumes. When using the AUTO mode, it will attempt to configure two volumes;

an OS volume and a Data volume. The OS volume will be RAID1, while the Data

volume will be the highest possible RAID up to 6, depending on the number of

disks and RAID modes supported by the controller. Spares may be automatically

allocated. All user input in the input config file is ignored in AUTO mode.

The actual volumes created will depend on the controller, the number and size

of disks and the status of license keys in the controller.

* In scripting input/output, 2 disk mirrors are considered RAID1, 4 disk and

higher mirrors are RAID10. For compatibility, input scripts still accept

RAID1 for 4+ disk mirrors, however we recommend moving to the new argument

terminology as soon as possible.

* ARRAY and VOLUME arguments in configure mode can be specified as "Next",

which will use the next available ID.

* DRIVE arguments can be a number or wildcard, e.g. "*" or "2". In the first

example, all available drives will be used, in the second two drives will be

used.

* The CONTROLLER argument can be FIRST. Controllers are sorted after discovery,

the order being: internal before external, in ascending slot order. This

ensures that the input CONTROLLER=FIRST will target an internal controller

(SLOT=0) or the controller in the lowest PCI slot.

* To improve performance, use the REPEAT keyword when creating multiple

identical logical drives.

Feedback

--------

For support for HPSSACLI or Smart Array controllers, please visit the web at

http://support.hp.com.

HP Smart Storage Administrator (HPSSA)

Version 1.60-17.0 (linuxX64)

12/5/2013

Description

-----------

The HP Smart Storage Administrator (HPSSA) is the storage configuration,

diagnostics, and management utility for ProLiant servers.

* All other product names mentioned herein may be trademarks of their

respective companies.

Support

-------

Supported Operating Systems

64bit Platforms:

SuSE Linux Enterprise Server 10 (x86_64)

SuSE Linux Enterprise Server 11 (x86_64)

Red Hat Enterprise Linux 5 (x86_64)

Red Hat Enterprise Linux 6 (x86_64)

* All these platforms need to have the following driver upgrades:

CISS driver version 2.4.50 or later

These drivers and Fibre Channel HBA drivers are available from http://www.hp.com.

Supported Client Side Browsers

Mozilla Firefox 9.0 or later

Microsoft Internet Explorer 8.0 or above

Google Chrome

* Please make sure the following browser options are enabled:

1. Automatically load images and other data types.

2. Javascript.

3. Style sheets.

Supported Controllers

Smart Array P410i Controller

Smart Array P411 Controller

Smart Array P212 Controller

Smart Array P712m Controller

Smart Array B110i SATA RAID

Smart Array P812 Controller

Smart Array P220i Controller

Smart Array P222 Controller

Smart Array P420 Controller

Smart Array P420i Controller

Smart Array P421 Controller

Smart Array P822 Controller

Smart Array P721m Controller

Dynamic Smart Array B320i RAID

Dynamic Smart Array B120i RAID

Smart Array P430 Controller

Smart Array P431 Controller

Smart Array P731m Controller

Smart Array P230i Controller

Smart Array P530 Controller

Smart Array P531 Controller

Smart Array P830 Controller

Smart Array P830i Controller

Installing & Running the HP Smart Storage Administrator

--------------------------------------------------------------

Installation:

HPSSA is distributed using Red Hat Package Manager (RPM) packages.

If there is an older version of HPSSA installed on the system,

please use "rpm -e hpssa" to remove it first. Use "rpm -i <file name>"

to install the service.

Please type "man rpm" for other details regarding RPM.

The software resides in /opt/hp/hpssa and the executable

name is hpssa which is located in /usr/sbin.

HPSSA can be accessed through the HP System Management Homepage.  If you

have not installed "HP's System Management Homepage", please download and

install it from hp.com.

Run the HPSSA client on the local server as an application:

Requirements:

. You must run hpssa as root

. You must be running on a

. The background process will create a port on the loopback interface

which is not visible to external network connections.

1. Run the hpssa command (/usr/sbin/hpssa) with the "-local" parameter. The

background program will start and launch the firefox browser.

2. To change the default browser that is started, or to specify the command

to use to launch the browser, modify the browserstart.sh script

(under /opt/hp/hpssa/bin) with the correct path.

3. To exit the application, close the main application window.

Run the HPSSA remotely through the HP System Management Homepage:

Requirements:

. Only run HPSSA on servers that are on a local intranet

or a secure network.

. Secure the management port (port 2301) on your network, or run

using the secure port (port 2381).

. Run hpssa as root.

To start the server daemon:

1. Run the hpssa command with the "-start" parameter

/usr/sbin/hpssa -start

2. If this is the first time that HPSSA is started in this mode, you must

restart the SMH server for HPSSA to appear in the SMH page. To restart

SMH type (as root):

/etc/init.d/hpsmhd restart

3. Access the System Management Homepage from the client browser.

URL:        http://<server name>:2301/

Secure URL: https://<server name>:2381/

4. HPSSA can be launched from the Storage section by clicking on the

HP Smart Storage Administrator link.

To stop the server daemon:

1. Run the hpssa command with the "-stop" parameter

/usr/sbin/hpssa -stop

Additional Notes

----------------

1. When running under the System Management Homepage, users will

automatically be logged out after a predetermined period of inactivity

when using HPSSA and will have to re-login.

2. System Management Homepage login timeouts will not work properly while

running HPSSA unless the "Anonymous Access" option is checked on the

System Management Homepage under "Settings->Options".

If you decide not to check "Anonymous Access" and a login timeout

occurs, the System Management Homepage login screen will appear in an

HPSSA frame and will not work correctly. If this happens, close the

HPSSA browser and go back to the System Management Homepage and re-login

as administrator.

Click on the Smart Storage Administrator link to restart the HPSSA

session.

3. To properly exit out of the HPSSA session, click on the Exit (X) link in

the HPSSA window.

4. Please stop the HP Storage Agents when deleting logical drives

since HP Storage Agents keeps all logical drives open.

5. If all physical drives on a controller fail or are removed, software

keys will not be saved to the controller.

6. HPSSA may appear unresponsive during heavy array controller I/O

conditions, such as during a partition format or a logical drive

rebuild.

7. Some browsers allow pop-ups to be turned off or blocked.  For HPSSA to

run, pop-ups must be enabled in the browser.

8. When using Linux as the host operating system, if a Smart Array

controller has 16 or more volumes configured, deleting any volume in the

HPSSA will not update the /proc/partitions to reflect the deletion of

the device node associated with that volume. Rebooting the server will

cause the update to happen. Note that there is also a limitation that

only the first 16 volumes configured will be available to the operating

system.

9. When using Internet Explorer 9.0 to run HPSSA from a server using HPSMH

it may not be possible to save the diagnostic report. To allow the

report to be dowloaded do the following: Under Internet Options, go to

the Advanced tab and clear the "Do not save encrypted pages to disk"

checkbox.

10. When using Internet Explorer 8.0 to run HPSSA remotely, choosing the

"Save Report" link to download a copy of the diagnostic report may cause

the information bar to appear with the following message:

"To help protect your security, Windows Internet Explorer blocked this

site from downloading files to your computer. Click here for more

options..."

Even if the "download file" action is clicked in the information bar,

the file download is blocked and the report is not downloaded.

In order to successfully download the file:

* Choose to view the report first and then save the file, or

* Disable automatic download blocking by enabling automatic prompting

for file downloads. To access the setting that controls automatic

download blocking:

a) Click Tools, and then click Internet Options. The Internet Options

dialog box appears.

b) Click the Security tab. The Internet Options dialog box shows the

security levels and zones. Select the current security zone for the

server that is running HPSSA.

c) Click Custom Level. The Security Settings dialog box appears.

d) Scroll down to Downloads, and then select the Enable option for

Automatic prompting for file downloads.

Feedback

--------

For support for HPSSA or Smart Array controllers, please visit the web at

http://support.hp.com.

From: Alekhin, Dmitry [mailto:dmitry.alekhin@hp.com]

Sent: Saturday, June 21, 2014 1:08 AM

To: Belogrud Sergey Vladimirovich; Velichkin, Vladimir; Support Desk

Cc: Stepina, Lilia; Aleksandrov Aleksey Valerievich; Chernyh Aleksandr Vladimirovich; Leonov Aleksey Vladimirovich; Terentev Aleksey; Zbiranik Evgeniy Aleksandrovich; Kartsev Konstantin; Novikov Matvey Sergeevich; Polyakov Valeriy; Ryabov Aleksey; Sotnichenko Sergey; Varakuta Sergey; Gus Konstantin Mikhaylovich

Subject: RE: Подтверждение запланированных работ

Здравствуйте, Сергей.

Отвечу на Ваши вопросы, также проинформирую о статусе заявки 4648517532;

Правильно-ли я понимаю, что новая прошивка v. 5.42 для raid-конторллеров не включена в последний SPP?

Могли бы Вы пояснить или прислать ссылку на статью, поясняющую как можно добавить в SPP (в iso-образ, например) пакет, обновляющий firmware на свежую версию, не вошедшую в SPP.

Я в курсе, что инсталлировать firmware можно и непосредственно запуская из ОС файл-установщик вида CP022978.scexe, например.

Да, конечно, можно инсталлировать онлайн, в приведенной Вами ссылке:

https://h20565.www2.hp.com/portal/site/hpsc/template.PAGE/public/psi/swdDetails/?sp4ts.oid=426826&spf_p.tpst=swdMain&spf_p.prp_swdMain=wsrp-navigationalState%3Didx%253D2%257CswItem%253DMTX_7903b59b20034c6d920a68f808%257CswEnvOID%253D%257CitemLocale%253D%257CswLang%253D%257Cmode%253D4%257Caction%253DdriverDocument&javax.portlet.begCacheTok=com.vignette.cachetoken&javax.portlet.endCacheTok=com.vignette.cachetoken

При переходе в раздел Installation Instruction Вы найдете инструкцию добавлению данного пакета в записанный образ SPP:

Installation:

To update firmware from Linux operating system on target server:

·         Login as root. (You must be root in order to apply the ROM update.)

·         Place the Smart Component in a temporary directory.

·         From the same directory, execute the Smart Component. For example: ./CPXXXXXX.scexe

·         Follow the directions given by the Smart Component.

·         Reboot your system if you would like the update to take effect immediately.

To use HP Smart Update Manager from the HP Service Pack for ProLiant:

Offline:

·         Place the HP Service Pack for ProLiant on a USB key using the HP USB Key Creator Utility.

·         Place the desired firmware to be updated in the directory, /hp/swpackages on the USB key.

·         Boot from the newly created HP Service Pack for Proliant USB key.

·         Follow HP Smart Update Manager steps to complete firmware update.

Online:

·         Login as root. (You must be root in order to apply the ROM update.)

·         Place the desired firmware to be updated in the directory, /hp/swpackages

·         Locate Start.html which is found in the root of the ISO

·         Open the html file in a supported browser

·         Follow the instructions on the screen

Почему-то на странице «Drivers, Software & Firmware for HP ProLiant DL380p Gen8 Server» среди списка я не обнаружил обновлений firmware для «Smart Array P420».

https://h20565.www2.hp.com/portal/site/hpsc/template.PAGE/public/psi/swdHome/?sp4ts.oid=5177958&spf_p.tpst=swdMain&spf_p.prp_swdMain=wsrp-navigationalState%3DswEnvOID%253D4103%257CswLang%253D%257Caction%253DlistDriver&javax.portlet.begCacheTok=com.vignette.cachetoken&javax.portlet.endCacheTok=com.vignette.cachetoken

Могли бы Вы описать или выслать ссылку на руководство по использованию утилиты hpssa.

Ссылка на RPM:

http://downloads.linux.hp.com/SDR/repo/spp/redhat/6.2/x86_64/2014.02.0_B/hpssacli-1.60-17.0.x86_64.txt

Все команды обратно совместимы с hpacucli, добавлены команды работы с новыми функциями контроллеров P-серии ( SmartPath,SmartCache).

Для генерации диагностики следует выполнить следующую команду:

[root@B6608PN3S tmp]# hpssacli ctrl all diag file=/tmp/hpssa_diag.zip

Generating diagnostic report...done

[root@B6608PN3S tmp]# unzip hpssa_diag.zip

Archive:  hpssa_diag.zip

inflating: ADUReport.htm

inflating: ADUReport.txt

inflating: ADUReport.xml

inflating: slot.0

inflating: report.checksum

В данном случае, это удобно, т.к. SSA собирает serial-debug с контроллеров и помещает их в файлы slot.X в соответствии с их PCI-слотом.

Описанное Вами предпочтение в использовании утилиты hpssa касается только серверов (raid-контроллеров Smart Array P420 и Smart Array P420i , установленных на серверах) модели «ProLiant DL380p Gen8» или модель «ProLiant DL380 G7» (Smart Array P410i) тоже попадает в этот список?

На данный момент пакет утилит HPACU полностью заменен HPSSA, и развиваться он в дальнейшем не будет.

HPSSA поддерживает работу с контроллерами P-серии от P400 до P830 (т.е практически все в линейке HPSA-контроллеров).

#######

Что касается нашей изначальной заявки 4648517532, то я передал лог vmcore и sosreport L2 для анализа, выполнив первичный просмотр дампа.

Пока теория о перезагрузке в следствии неисправного контроллера была отметена, и я считаю исходя из стектрейса и характера паники, что проблема в ядре, и поведение практически идентично описанному в статье:

https://access.redhat.com/site/solutions/54544

Для подтверждения дамп еще передадут в команду ERT (L3) для дополнительного анализа.

Спасибо.

Best Regards,

Dmitry Alekhin

RSAA

+79104995717

[root@centos00 ~]# scp /media/sf_share/hp/hpssa/*.rpm bsv@hq-msk-gp-tst-sm:/tmp/

bsv@hq-msk-gp-tst-sm's password:

hpssa-1.60-17.0.x86_64.rpm                                                                                            100% 8756KB   2.1MB/s   00:04

hpssacli-1.60-17.0.x86_64.rpm                                                                                         100%   10MB   2.0MB/s   00:05

[root@centos00 ~]#

[root@smdw_test ~]# rpm -Uvh /tmp/hpssa-1.60-17.0.x86_64.rpm /tmp/hpssacli-1.60-17.0.x86_64.rpm

warning: /tmp/hpssa-1.60-17.0.x86_64.rpm: Header V3 RSA/SHA1 Signature, key ID 5ce2d476: NOKEY

Preparing...                ########################################### [100%]

1:hpssacli               ########################################### [ 50%]

2:hpssa                  ########################################### [100%]

[root@smdw_test ~]#

[root@smdw_test ~]# scp /tmp/{hpssa-1.60-17.0.x86_64.rpm,hpssacli-1.60-17.0.x86_64.rpm} sdw4:/tmp/

hpssa-1.60-17.0.x86_64.rpm                                                                                            100% 8756KB   8.6MB/s   00:01

hpssacli-1.60-17.0.x86_64.rpm                                                                                         100%   10MB   9.8MB/s   00:00

[root@smdw_test ~]#

[root@sdw4_test ~]# rpm -Uvh /tmp/hpssa-1.60-17.0.x86_64.rpm /tmp/hpssacli-1.60-17.0.x86_64.rpm

warning: /tmp/hpssa-1.60-17.0.x86_64.rpm: Header V3 RSA/SHA1 Signature, key ID 5ce2d476: NOKEY

Preparing...                ########################################### [100%]

1:hpssacli               ########################################### [ 50%]

2:hpssa                  ########################################### [100%]

[root@sdw4_test ~]#

[root@sdw4_test ~]#  hpssacli ctrl all diag file=/tmp/20140626-1559.hpssa.diag.zip

Generating diagnostic report...done

[root@sdw4_test ~]#

[root@sdw4_test ~]# ls -Alh /tmp/20140626-1559.hpssa.diag.zip

-rw-r--r-- 1 root root 267K Jun 26 16:00 /tmp/20140626-1559.hpssa.diag.zip

[root@sdw4_test ~]#

[root@sdw4_test ~]# zipinfo /tmp/20140626-1559.hpssa.diag.zip

Archive:  /tmp/20140626-1559.hpssa.diag.zip

Zip file size: 273407 bytes, number of entries: 4

-rw----     0.0 fat  1102543 t- defN 80-000-00 00:00 ADUReport.htm

-rw----     0.0 fat  2205924 t- defN 80-000-00 00:00 ADUReport.txt

-rw----     0.0 fat  3203210 t- defN 80-000-00 00:00 ADUReport.xml

-rw----     0.0 fat      102 t- defN 80-000-00 00:00 report.checksum

4 files, 6511779 bytes uncompressed, 272973 bytes compressed:  95.8%

[root@sdw4_test ~]#

[root@ds-mon11t ~]# rpm -Uvh /tmp/hpss*

warning: /tmp/hpssa-1.60-17.0.x86_64.rpm: Header V3 RSA/SHA1 Signature, key ID 5ce2d476: NOKEY

Preparing...                ########################################### [100%]

1:hpssacli               ########################################### [ 50%]

2:hpssa                  ########################################### [100%]

[root@ds-mon11t ~]# hpssacli

HP Smart Storage Administrator CLI 1.60.17.0

Detecting Controllers...Done.

Type "help" for a list of supported commands.

Type "exit" to close the console.

=> help

CLI Syntax

A typical HPSSACLI command line consists of three parts: a target device,

a command, and a parameter with values if necessary. Using angle brackets to

denote a required variable and plain brackets to denote an optional

variable, the structure of a typical HPSSACLI command line is as follows:

<target> <command> [parameter=value]

<target> is of format:

[controller all|slot=#|wwn=#|chassisname="AAA"|

serialnumber=#|chassisserialnumber=#|ctrlpath=#:# ]

[array all|<id>]

[physicaldrive all|allunassigned|[#:]#:#|[#:]#:#-[#:]#:#]

[ssdphysicaldrive all|allunassigned|[#:]#:#|[#:]#:#-[#:]#:#]

[logicaldrive all|#]

[enclosure all|#:#|serialnumber=#|chassisname=#]

[licensekey all|<key>]

[ssdinfo]

Note 1: The #:#:# syntax is only needed for systems that

specify port:box:bay. Other physical drive targeting

schemes are box:bay and port:id.

Note 2: The chassisserialnumber is known in HPSSA as the

RAID Array Serial Number. The chassisname is known

in HPSSA as the RAID Array ID.

Note 3: ctrlpath=#:# maps to "smart enclosure hooked up to

host bus adapter slot:host bus adapter port"

Example targets:

controller slot=5

controller chassisname="Lab C"

controller serialnumber=P21DA2322S

controller chassisserialnumber=9J3CJN71XDCH

controller wwn=500308B300701011

controller slot=7 array A

controller slot=5 logicaldrive 5

controller slot=5 physicaldrive 1:5

controller slot=5 physicaldrive 1E:2:3

controller slot=5 ssdphysicaldrive all

controller slot=5 enclosure 4E:1 show

controller slot=5 licensekey XXXXX-XXXXX-XXXXX-XXXXX-XXXXX

For detailed command information type any of the following:

help add

help create

help delete

help diag

help modify

help remove

help shorthand

help show

help target

help rescan

help version

Encryption related commands:

help ctrlpasswdstate

help clearencryptionconfig

help encode

help encrypt

help enableencryption

help encryption

help eula

help fwlock

help import

help instantsecureerase

help localkeymanagermode

help login

help logout

help recoverpassword

help rekey

help remotekey

help removectrlpasswd

help rescankeys

help setmasterkeycache

help setctrlpasswd

help setpasswd

help setrecoveryparams

help unlockvolumes

Help also accepts commonly used CLI parameters and HPSSA keywords. Adding

additional keywords will further filter the help output. Examples:

help ssp        (shows all ssp help including show and modify commands)

help ssp modify (restricts ssp help to only modify commands)

help migrate

help expand

help extend

help <keyword> <keyword> ... <keyword>

=>

Save raid-controller configuration!

[root@ds-mon11t ~]# hpssascripting -c /tmp/hpssascripting.dat

[root@ds-mon11t ~]# cat /tmp/hpssascripting.dat

; Date captured: Fri Jul  4 18:31:27 2014

; Version: 1.60.17.0

Action= Configure

Method= Custom

; Controller Specifications

; Controller HP Smart Array P420i, FirmwareVersion 4.68, License Keys Supported

; SerialNumber 001438024105D10

; DriverName hpsa

; DriverVersion 3.2.0

; SSDSmartPath Supported

Controller= SLOT 0

ReadCache= 10

WriteCache= 90

RebuildPriority= Medium

ExpandPriority= Medium

SurfaceScanMode= Idle

SurfaceScanDelay= 15

DriveWriteCache= Enabled

NoBatteryWriteCache= Disabled

MNPDelay= 60

IRPEnable= Disabled

DPOEnable= Disabled

ElevatorSortEnable= Enabled

QueueDepth= Automatic

PredictiveSpareActivation= Disable

; Array Specifications

Array= A

; Array Drive Type is SAS

; Array Free Space 0 KB

; 1I:2:1 (300 GB, SAS), 1I:2:2 (300 GB, SAS), 1I:2:3 (300 GB, SAS), 1I:2:4 (300 GB, SAS)

Drive= 1I:2:1, 1I:2:2, 1I:2:3, 1I:2:4

OnlineSpare= No

; Logical Drive Specifications

LogicalDrive= 1

RAID= 10

Size= 572140

; SizeBlocks= 1171743836

Sectors= 32

StripSize= 256

Caching= Enabled

; VolumeUniqueID= 600508B1001C453B9BC30FFE9A6A1342

[root@ds-mon11t ~]#

[root@ds-mon11t ~]# hpssascripting

usage: ( [-c filename] | [ -i filename] ) [-e filename ] [-internal | -external] [ -reset ]

-c      Captures configuration to filename. Default output file is ssaoutput.ini

-i      Creates configuration from filename. Default input file is ssainput.ini

-e      Sets the error filename. Default error file is error.ini

-internal       Restricts operations to internal controllers

-external       Restricts operations to external controllers

-reset  Resets the controller configuration before applying a new one. Used with -i option

[root@ds-mon11t ~]#