20160913

#	Here is how to fix troubles with stucked job in iDRAC.

#	One way is to remove all jobs.

[support@centos00 20160815-1714]$ SSHPASS="$(pass common/windows-current)" sshpass -e ssh -t -oStrictHostKeyChecking=no tcsbank.ru\\s.belogrud@hg2h8d2.mgmt.tcsbank.ru racadm getsysinfo | tee -a /tmp/$(date +%Y%m%d-%H%M).getsysinfo.hg2h8d2.log

[support@centos00 20160815-1714]$ SSHPASS="$(pass common/windows-current)" sshpass -e ssh -t -oStrictHostKeyChecking=no tcsbank.ru\\s.belogrud@hg2h8d2.mgmt.tcsbank.ru racadm jobqueue delete -i JID_CLEARALL_FORCE | tee -a /tmp/$(date +%Y%m%d-%H%M).getsysinfo.hg2h8d2.log

Created Job Identifier:

RAC1032: JID_CLEARALL_FORCE job(s) was cancelled by the user.

Connection to hg2h8d2.mgmt.tcsbank.ru closed.

[support@centos00 20160815-1714]$

[support@centos00 20160815-1714]$ SSHPASS="$(pass common/windows-current)" sshpass -e ssh -t -oStrictHostKeyChecking=no tcsbank.ru\\s.belogrud@hg2h8d2.mgmt.tcsbank.ru racadm jobqueue view | tee -  a /tmp/$(date +%Y%m%d-%H%M).getsysinfo.hg2h8d2.log

-------------------------JOB QUEUE------------------------

Connection to hg2h8d2.mgmt.tcsbank.ru closed.

[support@centos00 20160815-1714]$

#	The second way is try to do racreset. It could be done after the first method too.

[support@centos00 20160815-1714]$ ssh -t -oStrictHostKeyChecking=no root@10.219.9.140 racadm racreset | tee -a /tmp/$(date +%Y%m%d-%H%M).getsysinfo.40zj8d2.log

root@10.219.9.140's password:

RAC reset operation initiated successfully. It may take a few

minutes for the RAC to come online again.

Connection to 10.219.9.140 closed.

[support@centos00 20160815-1714]$

20160407

#	Several of useful commands for Dell servers to get some information about server.

#	20181026

[root@ds-pr-onlacq01 ~]# cat /etc/yum.repos.d/tcs-dell.repo

[tcs-dell]

gpgcheck=0

name=DLR

enabled=1

baseurl=http://repo-linux.tcsbank.ru/dell/rhel/$releasever/$basearch

[root@ds-pr-onlacq01 ~]#

[root@ds-pr-onlacq01 work]# yum install srvadmin-idracadm7

cat > $(date +%Y%m%d-%H%M).script.get.info.about.dell.server.sh

svctag="$(racadm getsvctag)" ; racadm getsysinfo | tee -a $(pwd)/$(date +%Y%m%d-%H%M).${svctag}.racadm.getsysinfo.log

svctag="$(racadm getsvctag)" ; racadm getsensorinfo | tee -a $(pwd)/$(date +%Y%m%d-%H%M).${svctag}.racadm.getsensorinfo.log

svctag="$(racadm getsvctag)" ; racadm hwinventory | tee -a $(pwd)/$(date +%Y%m%d-%H%M).${svctag}.racadm.hwinventory.log

svctag="$(racadm getsvctag)" ; racadm getraclog | tee -a $(pwd)/$(date +%Y%m%d-%H%M).${svctag}.racadm.getraclog.log

svctag="$(racadm getsvctag)" ; racadm getsel | tee -a $(pwd)/$(date +%Y%m%d-%H%M).${svctag}.racadm.getsel.log

svctag="$(racadm getsvctag)" ; omreport system summary | tee -a $(pwd)/$(date +%Y%m%d-%H%M).${svctag}.omreport.system.summary.log

svctag="$(racadm getsvctag)" ; omreport system alertlog | tee -a $(pwd)/$(date +%Y%m%d-%H%M).${svctag}.omreport.system.alertlog.log

svctag="$(racadm getsvctag)" ; omreport chassis | tee -a $(pwd)/$(date +%Y%m%d-%H%M).${svctag}.omreport.chassis.log

svctag="$(racadm getsvctag)" ; omreport chassis biossetup | tee -a $(pwd)/$(date +%Y%m%d-%H%M).${svctag}.omreport.chassis.biossetup.log

#	This way helps collect all files in one directory.

[root@m1-pr-onlacq01 20181026-1952]# mkdir 20181026-1952/

[root@m1-pr-onlacq01 20181026-1952]# cd 20181026-1952/

[root@m1-pr-onlacq01 20181026-1952]# . ../20181026-1959.script.get.info.about.dell.server.sh

[root@ds-pr-onlacq01 20181026-1952]# filename="/tmp/$(date +%Y%m%d-%H%M).$(hostname).$(racadm getsvctag).hardware.info.tgz" ; tar -cvzf ${filename} ./* ; chmod 644 ${filename} ; ls -Alh ${filename}

./20181026-2001.BB76C72.racadm.getraclog.log

./20181026-2001.BB76C72.racadm.getsel.log

./20181026-2001.BB76C72.racadm.getsensorinfo.log

./20181026-2001.BB76C72.racadm.getsysinfo.log

./20181026-2001.BB76C72.racadm.hwinventory.log

./20181026-2002.BB76C72.omreport.chassis.biossetup.log

./20181026-2002.BB76C72.omreport.chassis.log

./20181026-2002.BB76C72.omreport.system.alertlog.log

./20181026-2002.BB76C72.omreport.system.summary.log

-rw-r--r-- 1 root root 23K Oct 26 20:13 /tmp/20181026-2013.ds-pr-onlacq01.BB76C72.hardware.info.tgz

[root@ds-pr-onlacq01 20181026-1952]#

[s.belogrud@TCS4357]# scp bsv@ds-pr-onlacq01:/tmp/20181026-2005.ds-pr-onlacq01.BB76C72.hardware.info.tgz /drives/c/Users/s.belogrud/Downloads/

20181026-2005.ds-pr-onlacq01.BB76C72.hardware.info.tgz                                                                                                                        100%   22KB  22.3KB/s   00:00

[s.belogrud@TCS4357]#

#	If I use only this command, I receive follow error...

[root@ds-pr-onlacq01 ~]# yum install srvadmin-idracadm

[root@ds-pr-onlacq01 work]# racadm getsvctag

NOTE:

The current version of Integrated Dell Remote Access Controller (iDRAC)

that do not support the execution of RACADM commands.

Contact Dell customer service to upgrade to iDRAC Express or iDRAC Enterprise

The installed version of RACADM client executable that do not match the

generation of the server.

[root@ds-pr-onlacq01 work]#

#

#	Here is an example how I have collected and publish information about servers’s hardware and config.

#	“20181026 Трудности в работе приложения на onl4acq”

https://wiki.tcsbank.ru/pages/viewpage.action?pageId=307548121&focusedCommentId=307548157&refresh=1540570275082#comment-307548157

#

#	20210301

#	Get server name, service tag, iDRAC ip address.

root@ds-prod-search-es-10:~#  echo -e "Server: $(hostname)" ; ipaddress="$(racadm getniccfg | egrep -e "^IP Address.*[0-9].[0-9].[0-9].[0-9]" | awk "{print \$4}")" ; servicetag="$(racadm getsvctag)" ;  echo -e "$servicetag.mgmt.tcsbank.ru\t\t$ipaddress"

Server: ds-prod-search-es-10.tcsbank.ru

33JP873.mgmt.tcsbank.ru         10.219.116.7

root@ds-prod-search-es-10:~#

#	20170703

#	Get server name, service tag, iDRAC ip address.

[root@m1-2-gp-prd-mdw ~]# for var1_host in m1-2-gp-prd-sdw{1..36} ; do ssh root@${var1_host} '( echo ; echo -e "Server: $(hostname)" ; ipaddress="$(racadm getniccfg | grep -e "^IP Address.*10.219.108" | awk "{print \$4}")" ; servicetag="$(racadm getsvctag)" ;  echo -e "$servicetag.mgmt.tcsbank.ru\t\t$ipaddress" )' ; done

Server: m1-2-gp-prd-sdw1

D8RHXJ2.mgmt.tcsbank.ru         10.219.108.101

...

#	Get name, iDRAC ip address, servicetag. Do it via sshpass.

[support@centos ~]$ list_of_hosts="" ; while read var1_hosts ; do list_of_hosts="${list_of_hosts} ${var1_hosts}" ; done << 'EOF' ; for var1_hosts in ${list_of_hosts} ; do SSHPASS="$(pass common/linux)" sshpass -e timeout 5 ssh -t -oStrictHostKeyChecking=no -oPubkeyAuthentication=no bsv@${var1_hosts}.tcsbank.ru "( echo '$(pass common/linux)' | sudo -S -i sh -c 'echo ; echo ; hostname ; /opt/dell/srvadmin/sbin/racadm getniccfg | grep -e "^IP.*Address.*=.*10.*" ; /opt/dell/srvadmin/sbin/racadm getsvctag ' )" ; done #

ds-gp-prod-sdw01

EOF

20160330

[support@centos00 ~]$ SSHPASS="$(pass common/windows-current)" sshpass -e ssh -t -oStrictHostKeyChecking=no tcsbank.ru\\s.belogrud@cblfc72.mgmt.tcsbank.ru racadm getsysinfo | tee -a /tmp/$(date +%Y%m%d-%H%M).getsysinfo.cblfc72.log

20160329

#	Get service tag of Dell server.

[support@centos00 ~]$ SSHPASS="$(pass common/linux)" sshpass -e ssh -t -oStrictHostKeyChecking=no bsv@m1-hadoop-wk01t "( echo '$(pass common/linux)' | sudo -S -i sh -c ' echo ; hostname ; racadm getsvctag ' )" ; SSHPASS=''

m1-hadoop-wk01t.tcsbank.ru

1J7V762

Connection to m1-hadoop-wk01t closed.

[support@centos00 ~]$

2015XXXX

[root@vm-vol-nag01t ~]# rpm -qf /usr/lib64/nagios/plugins/check_openmanage

nagios-plugins-openmanage-3.7.12-1.el6.x86_64

[root@vm-vol-nag01t ~]# rpm -q nagios-plugins-openmanage

nagios-plugins-openmanage-3.7.12-1.el6.x86_64

[root@vm-vol-nag01t ~]# rpm -qi nagios-plugins-openmanage

Name        : nagios-plugins-openmanage    Relocations: (not relocatable)

Version     : 3.7.12                            Vendor: Fedora Project

Release     : 1.el6                         Build Date: Mon 28 Jul 2014 07:20:48 PM MSK

Install Date: Wed 05 Aug 2015 04:06:18 PM MSK      Build Host: buildhw-06.phx2.fedoraproject.org

Group       : Applications/System           Source RPM: nagios-plugins-openmanage-3.7.12-1.el6.src.rpm

Size        : 279140                           License: GPLv3+

Signature   : RSA/8, Tue 29 Jul 2014 09:50:39 PM MSK, Key ID 3b49df2a0608b895

Packager    : Fedora Project

URL         : http://folk.uio.no/trondham/software/check_openmanage.html

Summary     : Nagios plugin to monitor hardware health on Dell servers

Description :

check_openmanage is a plugin for Nagios which checks the hardware

health of Dell servers running OpenManage Server Administrator

(OMSA). The plugin can be used remotely with SNMP or locally with

NRPE, check_by_ssh or similar, whichever suits your needs and

particular taste. The plugin checks the health of the storage

subsystem, power supplies, memory modules, temperature probes etc.,

and gives an alert if any of the components are faulty or operate

outside normal parameters.

[root@vm-vol-nag01t ~]#

#	Nice way to get usefull and brief information.

[root@vm-vol-nag01t nagios]# /usr/lib64/nagios/plugins/check_openmanage -H m1-hadoop-wk01t -C tcspub -P 1 -p -d

System:      PowerEdge R730xd         OMSA version:    8.1.0

ServiceTag:  1J7V762                  Plugin version:  3.7.12

BIOS/date:   1.3.6 06/03/2015         Checking mode:   SNMPv1 UDP/IPv4

-----------------------------------------------------------------------------

Storage Components

=============================================================================

STATE  |    ID    |  MESSAGE TEXT

---------+----------+--------------------------------------------------------

OK |        0 | Controller 0 [PERC H730P Mini] is Ready

OK |  0:0:1:0 | Physical Disk 0:1:0 [SAS-HDD 4.0TB] on ctrl 0 is Non-RAID

OK |  0:0:1:1 | Physical Disk 0:1:1 [SAS-HDD 4.0TB] on ctrl 0 is Non-RAID

OK |  0:0:1:2 | Physical Disk 0:1:2 [SAS-HDD 4.0TB] on ctrl 0 is Non-RAID

OK |  0:0:1:3 | Physical Disk 0:1:3 [SAS-HDD 4.0TB] on ctrl 0 is Non-RAID

OK |  0:0:1:4 | Physical Disk 0:1:4 [SAS-HDD 4.0TB] on ctrl 0 is Non-RAID

OK |  0:0:1:5 | Physical Disk 0:1:5 [SAS-HDD 4.0TB] on ctrl 0 is Non-RAID

OK |  0:0:1:6 | Physical Disk 0:1:6 [SAS-HDD 4.0TB] on ctrl 0 is Non-RAID

OK |  0:0:1:7 | Physical Disk 0:1:7 [SAS-HDD 4.0TB] on ctrl 0 is Non-RAID

OK |  0:0:1:8 | Physical Disk 0:1:8 [SAS-HDD 4.0TB] on ctrl 0 is Non-RAID

OK |  0:0:1:9 | Physical Disk 0:1:9 [SAS-HDD 4.0TB] on ctrl 0 is Non-RAID

OK | 0:0:1:10 | Physical Disk 0:1:10 [SAS-HDD 4.0TB] on ctrl 0 is Non-RAID

OK | 0:0:1:11 | Physical Disk 0:1:11 [SAS-HDD 4.0TB] on ctrl 0 is Non-RAID

OK | 0:0:1:12 | Physical Disk 0:1:12 [SAS-HDD 500GB] on ctrl 0 is Online

OK | 0:0:1:13 | Physical Disk 0:1:13 [SAS-HDD 500GB] on ctrl 0 is Online

OK |      0:0 | Logical Drive '/dev/sdm' [RAID-1, 465.25 GB] is Ready

OK |      0:0 | Cache Battery 0 in controller 0 is Ready

OK |      0:0 | Connector 0 [SAS] on controller 0 is Ready

OK |    0:0:1 | Enclosure 0:0:1 [Backplane] on controller 0 is Ready

-----------------------------------------------------------------------------

Chassis Components

=============================================================================

STATE  |  ID  |  MESSAGE TEXT

---------+------+------------------------------------------------------------

OK |    0 | Memory module 0 [A1, 16384 MB] is Ok

OK |    1 | Memory module 1 [A2, 16384 MB] is Ok

OK |    2 | Memory module 2 [A3, 16384 MB] is Ok

OK |    3 | Memory module 3 [A4, 16384 MB] is Ok

OK |    4 | Memory module 4 [A5, 16384 MB] is Ok

OK |    5 | Memory module 5 [A6, 16384 MB] is Ok

OK |    6 | Memory module 6 [A7, 16384 MB] is Ok

OK |    7 | Memory module 7 [A8, 16384 MB] is Ok

OK |    8 | Memory module 8 [B1, 16384 MB] is Ok

OK |    9 | Memory module 9 [B2, 16384 MB] is Ok

OK |   10 | Memory module 10 [B3, 16384 MB] is Ok

OK |   11 | Memory module 11 [B4, 16384 MB] is Ok

OK |   12 | Memory module 12 [B5, 16384 MB] is Ok

OK |   13 | Memory module 13 [B6, 16384 MB] is Ok

OK |   14 | Memory module 14 [B7, 16384 MB] is Ok

OK |   15 | Memory module 15 [B8, 16384 MB] is Ok

OK |    0 | Chassis fan 0 [System Board Fan1 RPM] reading: 6840 RPM

OK |    1 | Chassis fan 1 [System Board Fan2 RPM] reading: 6840 RPM

OK |    2 | Chassis fan 2 [System Board Fan3 RPM] reading: 6960 RPM

OK |    3 | Chassis fan 3 [System Board Fan4 RPM] reading: 6480 RPM

OK |    4 | Chassis fan 4 [System Board Fan5 RPM] reading: 7200 RPM

OK |    5 | Chassis fan 5 [System Board Fan6 RPM] reading: 6600 RPM

OK |    0 | Power Supply 0 [AC]: Presence detected

OK |    1 | Power Supply 1 [AC]: Presence detected

OK |    0 | Temperature Probe 0 [System Board Inlet Temp] reads 24 C (min=3/-7, max=42/47)

OK |    1 | Temperature Probe 1 [System Board Exhaust Temp] reads 48 C (max=70/75)

OK |    2 | Temperature Probe 2 [CPU1 Temp] reads 62 C (min=8/3, max=82/87)

OK |    3 | Temperature Probe 3 [CPU2 Temp] reads 65 C (min=8/3, max=82/87)

OK |    0 | Processor 0 [Intel Xeon E5-2640 v3 2.60GHz] is Present

OK |    1 | Processor 1 [Intel Xeon E5-2640 v3 2.60GHz] is Present

OK |    0 | Voltage sensor 0 [CPU1 VCORE PG] is Good

OK |    1 | Voltage sensor 1 [CPU2 VCORE PG] is Good

OK |    2 | Voltage sensor 2 [System Board 3.3V PG] is Good

OK |    3 | Voltage sensor 3 [System Board 5V AUX PG] is Good

OK |    4 | Voltage sensor 4 [CPU2 M23 VPP PG] is Good

OK |    5 | Voltage sensor 5 [CPU1 M23 VPP PG] is Good

OK |    6 | Voltage sensor 6 [System Board 1.05V PG] is Good

OK |    7 | Voltage sensor 7 [System Board BP0 5V PG] is Good

OK |    8 | Voltage sensor 8 [CPU1 M23 VDDQ PG] is Good

OK |    9 | Voltage sensor 9 [CPU1 M23 VTT PG] is Good

OK |   10 | Voltage sensor 10 [System Board 5V SWITCH PG] is Good

OK |   11 | Voltage sensor 11 [System Board DIMM PG] is Good

OK |   12 | Voltage sensor 12 [System Board VCCIO PG] is Good

OK |   13 | Voltage sensor 13 [CPU2 M01 VDDQ PG] is Good

OK |   14 | Voltage sensor 14 [CPU1 M01 VDDQ PG] is Good

OK |   15 | Voltage sensor 15 [CPU2 M23 VTT PG] is Good

OK |   16 | Voltage sensor 16 [CPU2 M01 VTT PG] is Good

OK |   17 | Voltage sensor 17 [System Board NDC PG] is Good

OK |   18 | Voltage sensor 18 [CPU2 M01 VPP PG] is Good

OK |   19 | Voltage sensor 19 [CPU1 M01 VPP PG] is Good

OK |   20 | Voltage sensor 20 [CPU2 M23 VDDQ PG] is Good

OK |   21 | Voltage sensor 21 [System Board 1.5V PG] is Good

OK |   22 | Voltage sensor 22 [PS2 PG Fail] is Good

OK |   23 | Voltage sensor 23 [System Board PS1 PG Fail] is Good

OK |   24 | Voltage sensor 24 [System Board BP1 5V PG] is Good

OK |   25 | Voltage sensor 25 [System Board 1.5V AUX PG] is Good

OK |   26 | Voltage sensor 26 [CPU1 M01 VTT PG] is Good

OK |   27 | Voltage sensor 27 [PS1 Voltage 1] reads 230 V

OK |   28 | Voltage sensor 28 [PS2 Voltage 2] reads 230 V

OK |   29 | Voltage sensor 29 [CPU1 FIVR PG] is Good

OK |   30 | Voltage sensor 30 [CPU2 FIVR PG] is Good

OK |   31 | Voltage sensor 31 [System Board 2.5V AUX PG] is Good

OK |    0 | Battery probe 0 [System Board CMOS Battery] is Presence Detected

OK |    0 | Amperage probe 0 [PS1 Current 1] reads 1.4 A

OK |    1 | Amperage probe 1 [PS2 Current 2] reads 0.2 A

OK |    2 | Amperage probe 2 [System Board Pwr Consumption] reads 322 W

OK |    0 | Chassis intrusion 0 detection: Ok (Not Breached)

OK |    0 | SD Card 0 [vFlash] is Absent

-----------------------------------------------------------------------------

Other messages

=============================================================================

STATE  |  MESSAGE TEXT

---------+-------------------------------------------------------------------

OK | ESM log health is Ok (less than 80% full)

OK | Chassis Service Tag is sane

[root@vm-vol-nag01t nagios]#

#	To print only storage configuration.

[root@vm-vol-nag4-tst etc]# for var1 in {1..36} ; do echo ; echo m1-2-gp-prd-sdw${var1} ; /usr/lib64/nagios/plugins/check_openmanage -H m1-2-gp-prd-sdw${var1} -C tcspub -P 1 -p -d | sed -n "/Storage Components/,/Chassis Components/p" ; done

...

m1-2-gp-prd-sdw36

Storage Components

=============================================================================

STATE  |    ID    |  MESSAGE TEXT

---------+----------+--------------------------------------------------------

OK |        0 | Controller 0 [PERC H730P Mini] is Ready

OK |        1 | Controller 1 [PERC H730P Adapter] is Ready

OK |  0:0:1:1 | Physical Disk 0:1:1 [SAS-HDD 600GB] on ctrl 0 is Online

OK |  0:0:1:2 | Physical Disk 0:1:2 [SAS-HDD 600GB] on ctrl 0 is Online

OK |  0:0:1:3 | Physical Disk 0:1:3 [SAS-HDD 600GB] on ctrl 0 is Online

OK |  0:0:1:4 | Physical Disk 0:1:4 [SAS-HDD 600GB] on ctrl 0 is Online

OK |  0:0:1:5 | Physical Disk 0:1:5 [SAS-HDD 600GB] on ctrl 0 is Online

OK |  0:0:1:6 | Physical Disk 0:1:6 [SAS-HDD 600GB] on ctrl 0 is Online

OK |  0:0:1:7 | Physical Disk 0:1:7 [SAS-HDD 600GB] on ctrl 0 is Online

OK |  0:0:1:8 | Physical Disk 0:1:8 [SAS-HDD 600GB] on ctrl 0 is Online

OK |  0:0:1:9 | Physical Disk 0:1:9 [SAS-HDD 600GB] on ctrl 0 is Ready (Global HS)

OK | 1:0:1:12 | Physical Disk 0:1:12 [SAS-SSD 480GB] on ctrl 1 is Non-RAID

OK | 1:0:1:13 | Physical Disk 0:1:13 [SAS-HDD 600GB] on ctrl 1 is Online

OK | 1:0:1:14 | Physical Disk 0:1:14 [SAS-HDD 600GB] on ctrl 1 is Online

OK | 1:0:1:15 | Physical Disk 0:1:15 [SAS-HDD 600GB] on ctrl 1 is Online

OK | 1:0:1:16 | Physical Disk 0:1:16 [SAS-HDD 600GB] on ctrl 1 is Online

OK | 1:0:1:17 | Physical Disk 0:1:17 [SAS-HDD 600GB] on ctrl 1 is Online

OK | 1:0:1:18 | Physical Disk 0:1:18 [SAS-HDD 600GB] on ctrl 1 is Online

OK | 1:0:1:19 | Physical Disk 0:1:19 [SAS-HDD 600GB] on ctrl 1 is Online

OK | 1:0:1:20 | Physical Disk 0:1:20 [SAS-HDD 600GB] on ctrl 1 is Online

OK | 1:0:1:21 | Physical Disk 0:1:21 [SAS-SSD 480GB] on ctrl 1 is Ready

WARNING |  0:0:1:0 | Physical Disk 0:1:0 [Dell AL14SEB060N, 600GB] on ctrl 0 is Foreign

OK |      0:0 | Logical Drive '/dev/sdb' [RAID-1, 558.38 GB] is Ready

OK |      0:1 | Logical Drive '/dev/sdc' [RAID-10, 1675.12 GB] is Ready

OK |      1:0 | Logical Drive '/dev/sde' [RAID-10, 2233.50 GB] is Ready

OK |      0:0 | Cache Battery 0 in controller 0 is Ready

OK |      1:0 | Cache Battery 0 in controller 1 is Ready

OK |      0:0 | Connector 0 [SAS] on controller 0 is Ready

OK |      1:0 | Connector 0 [SAS] on controller 1 is Ready

OK |    0:0:1 | Enclosure 0:0:1 [Backplane] on controller 0 is Ready

OK |    1:0:1 | Enclosure 1:0:1 [Backplane] on controller 1 is Ready

-----------------------------------------------------------------------------

Chassis Components

[root@vm-vol-nag4-tst etc]#

#	Same as above, olnly with selection of two HDD.

[root@vm-vol-nag4-tst etc]# for var1 in {1..36} ; do echo ; echo m1-2-gp-prd-sdw${var1} ; /usr/lib64/nagios/plugins/check_openmanage -H m1-2-gp-prd-sdw${var1} -C tcspub -P 1 -p -d | sed -n "/Storage Components/,/Chassis Components/p" | egrep -e "0:0:1:0|1:0:1:21" ; done

#	To get information about properly configured power supply on server.

[support@centos00 ~]$ SSHPASS="$(pass common/windows-current)" sshpass -e ssh -t -oStrictHostKeyChecking=no tcsbank.ru\\s.belogrud@cblfc72.mgmt.tcsbank.ru racadm get system.power.HotSpare.Enable

Warning: Permanently added 'cblfc72.mgmt.tcsbank.ru,10.219.10.234' (RSA) to the list of known hosts.

Disabled

Connection to cblfc72.mgmt.tcsbank.ru closed.

[support@centos00 ~]$ SSHPASS="$(pass common/windows-current)" sshpass -e ssh -t -oStrictHostKeyChecking=no tcsbank.ru\\s.belogrud@cblfc72.mgmt.tcsbank.ru racadm get system.power.RedundancyPolicy

Input Power Redundant

Connection to cblfc72.mgmt.tcsbank.ru closed.

[support@centos00 ~]$

#	Here is a way how to set this parameter to properly value.

[support@centos00 20160315-1129]$ SSHPASS="$(pass common/windows-current)" sshpass -e ssh -t -oStrictHostKeyChecking=no tcsbank.ru\\s.belogrud@bb5bc72.mgmt.tcsbank.ru racadm set system.power.HotSpare.Enable Disabled

Object value modified successfully

Connection to bb5bc72.mgmt.tcsbank.ru closed.

[support@centos00 20160315-1129]$

20160406

#	Get details about hardware.

[support@centos00 ~]$ SSHPASS="$(pass common/windows-current)" sshpass -e ssh -t -oStrictHostKeyChecking=no -oPubkeyAuthentication=no tcsbank.ru\\s.belogrud@HS6V762.mgmt.tcsbank.ru racadm hwinventory | tee -a /home/support/mnt/share/temp/$(date +%Y%m%d-%H%M).HS6V762.hwinventory.txt ; SSHPASS=''

-------------------------HARDWARE INVENTORY------------------------

[InstanceID: CPU.Socket.1]

Device Type = CPU

Cache3Location = 0

Cache2Location = 0

Cache1Location = 0

Model = Intel(R) Xeon(R) CPU E5-2640 v3 @ 2.60GHz

Manufacturer = Intel

TurboModeCapable = 1

TurboModeEnabled = 1

ExecuteDisabledCapable = 1

ExecuteDisabledEnabled = 1

VirtualizationTechnologyCapable = 1

VirtualizationTechnologyEnabled = 1

HyperThreadingCapable = 1

HyperThreadingEnabled = 1

NumberOfProcessorCores = 8

NumberOfEnabledThreads = 16

...

-------------------------------------------------------------------

[InstanceID: iDRAC.Embedded.1-1#IDRACinfo]

Device Type = iDRACCard

DNSDomainName = tcsbank.ru

DNSRacName = idrac-HS6V762

URLString = https://10.219.11.66:443

GUID = 3236374f-c0c8-5680-3610-00534c4c4544

PermanentMACAddress = 44:a8:42:35:f5:0e

ProductDescription = This system component provides a complete set of remote management functions for Dell PowerEdge servers

Model = Enterprise

FirmwareVersion = 2.21.21.21

IPMIVersion = 2.0

DeviceDescription = iDRAC

FQDD = iDRAC.Embedded.1-1

InstanceID = iDRAC.Embedded.1-1#IDRACinfo

LastSystemInventoryTime = 2016-04-04T15:52:16

LastUpdateTime = 2016-04-06T15:15:42

LANEnabledState = Enabled

SOLEnabledState = Enabled

-------------------------------------------------------------------

Connection to HS6V762.mgmt.tcsbank.ru closed.

[support@centos00 ~]$

#	Some info about firmware verison could be found there.

[support@centos00 ~]$ grep -i -e firmware /home/support/mnt/share/temp/20160406-1815.HS6V762.hwinventory.txt

ControllerFirmwareVersion = 25.3.0.0016

FirmwareVersion = 00.10.37

FirmwareVersion = 00.10.37

FirmwareVersion = 2.21.21.21

[support@centos00 ~]$

20160323

#

#	Clear ESM log can be done with CLI racadm utility.

#

[support@centos00 20160315-1129]$ ssh tcsbank.ru\\s.belogrud@CBLFC72.mgmt.tcbank.ru

ssh: Could not resolve hostname CBLFC72.mgmt.tcbank.ru: Name or service not known

[support@centos00 20160315-1129]$ ssh tcsbank\\s.belogrud@cblfc72.mgmt.tcsbank.ru

The authenticity of host 'cblfc72.mgmt.tcsbank.ru (10.219.10.234)' can't be established.

RSA key fingerprint is 95:cd:08:ca:fe:3b:ca:88:b3:25:7f:d4:37:5a:0e:60.

Are you sure you want to continue connecting (yes/no)? yes

Warning: Permanently added 'cblfc72.mgmt.tcsbank.ru,10.219.10.234' (RSA) to the list of known hosts.

tcsbank\s.belogrud@cblfc72.mgmt.tcsbank.ru's password: ****

#	To be sure that we connect to right server.

/admin1-> racadm getsysinfo

...

Host Name               = DS-AIC-VOICE16

OS Name                 = Microsoft Windows Server 2012 R2, Standard x64 Edition

...

/admin1-> racadm

racadm>>help

...

clrsel               -- clear the System Event Log (SEL)

...

racadm>>clrsel

racadm clrsel

The SEL was cleared successfully

racadm>>quit

/admin1-> exit

CLP Session terminated

Connection to cblfc72.mgmt.tcsbank.ru closed.

[support@centos00 20160315-1129]$

#	How to get “ESM” log or same, just with other name “SEL” log (System Event Log)

[support@centos00 ~]$ ssh tcsbank.ru\\s.belogrud@cblfc72.mgmt.tcsbank.ru racadm getsel

tcsbank.ru\s.belogrud@cblfc72.mgmt.tcsbank.ru's password:

Record:      1

Date/Time:   03/23/2016 18:39:54

Source:      system

Severity:    Ok

Description: Log cleared.

-------------------------------------------------------------------------------

Record:      2

Date/Time:   03/23/2016 18:41:07

Source:      system

Severity:    Non-Critical

Description: The system board battery is low.

-------------------------------------------------------------------------------

...

-------------------------------------------------------------------------------

Record:      10

Date/Time:   03/23/2016 18:48:46

Source:      system

Severity:    Ok

Description: The system board battery is operating normally.

-------------------------------------------------------------------------------

[support@centos00 ~]$

#	Get info about sensors.

[support@centos00 ~]$ ssh tcsbank.ru\\s.belogrud@HS6V762.mgmt.tcsbank.ru racadm getsensorinfo

tcsbank.ru\s.belogrud@hs6v762.mgmt.tcsbank.ru's password:

Sensor Type : POWER

<Sensor Name>                   <Status>                 <Type>

PS1 Status                      Present                  AC

PS2 Status                      Present                  AC

Sensor Type : TEMPERATURE

<Sensor Name>            <Status>    <Reading> <lc> <uc>  <lnc>[R/W]  <unc>[R/W]

[Key = iDRAC.Embedded.1#SystemBoardInletTemp]

System Board Inlet Temp       Ok      23C      -7C  47C     3C [Y]      42C [Y]

...

#	Get some info about server.

[support@centos00 ~]$ ssh tcsbank.ru\\s.belogrud@HS6V762.mgmt.tcsbank.ru racadm getsysinfo

tcsbank.ru\s.belogrud@hs6v762.mgmt.tcsbank.ru's password:

RAC Information:

RAC Date/Time           = Thu Mar 24 12:32:25 2016

Firmware Version        = 2.21.21.21

Firmware Build          = 12

Last Firmware Update    = 03/04/2016 17:57:28

Hardware Version        = 0.01

...

System Model            = PowerEdge R730xd

System Revision         = I

System BIOS Version     = 1.2.10

Service Tag             = HS6V762

Express Svc Code        = 38709885962

Host Name               = ds-hadoop-wk14p.tcsbank.ru

OS Name                 = Red Hat Enterprise Linux Server

OS Version              = release 6.6 (Santiago) Kernel 2.6.32-504.el6.x86_64 (x86_64)

Power Status            = ON

Fresh Air Capable       = Yes

...

[support@centos00 ~]$ SSHPASS="$(pass common/windows-current)" sshpass -e ssh -t -oStrictHostKeyChecking=no tcsbank.ru\\s.belogrud@cblfc72.mgmt.tcsbank.ru racadm get bios.sysinformation

[Key=BIOS.Setup.1-1#sysinformation]

#SysMfrContactInfo=www.dell.com

#SystemBiosVersion=1.6.2

#SystemCpldVersion=1.0.3

#SystemManufacturer=Dell Inc.

#SystemMeVersion=3.0.7.173

#SystemModelName=PowerEdge R430

#SystemServiceTag=CBLFC72

#UefiComplianceVersion=2.3.1

Connection to cblfc72.mgmt.tcsbank.ru closed.

[support@centos00 ~]$

#	Get Lifecycle log.

[support@centos00 ~]$ ssh tcsbank.ru\\s.belogrud@HS6V762.mgmt.tcsbank.ru racadm getraclog | tee -a /tmp/20160324-1233.getraclog.ds-hadoop-wk14p.log

SeqNumber       = 1481826

Message ID      = USR0030

Category        = Audit

AgentID         = RACLOG

Severity        = Information

Timestamp       = 2016-03-24 12:34:21

Message         = Successfully logged in using s.belogrud@tcsbank.ru, from 172.16.28.74 and SSH.

Message Arg   1 = s.belogrud@tcsbank.ru

Message Arg   2 = 172.16.28.74

Message Arg   3 = SSH

FQDD            = iDRAC.Embedded.1

--------------------------------------------------------------------------------

...

#	To see state of server’s LED.

[support@centos00 ~]$ ssh tcsbank.ru\\s.belogrud@HS6V762.mgmt.tcsbank.ru racadm getled

tcsbank.ru\s.belogrud@hs6v762.mgmt.tcsbank.ru's password:

LED State :    Not-Blinking

[support@centos00 ~]$

#	LED on server can be turned on this way.

20151126

[root@ds-gp-prd-sdw6 ~]# yum install srvadmin-omacore

[root@ds-gp-prd-sdw6 ~]# omreport -?

omreport         Reports component properties.

The available command(s) are:

Command          Description

about            Product and version properties.

licenses         Displays the digital licenses of the installed hardware devices.

preferences      Report system preferences.

system           System component properties.

chassis          Chassis component properties.

storage          Display storage component properties.

Usage: omreport <command...> [name=value...] ... [option...]

Valid command line options are:

-?               Print available command(s) or command help.

-fmt <lst|tbl|ssv|cdv|xml> Format for output results, default is lst.

Where: lst  List format.

tbl  Table format.

ssv  Semicolon separated format.

cdv  Custom delimiter separated format.

xml  Raw XML format.

-outc <file>     Redirect output to file, delete old if exists.

OR

-outa <file>     Redirect output to file, append to old if exists.

OR

Output may also be redirected using Operating System facilities (e.g. |more).

[root@ds-gp-prd-sdw6 ~]#

#	Here is how to get idrac ip-address info from local server.

[root@ds-gp-prd-sdw6 ~]# omreport chassis remoteaccess

Remote Access Information

Remote Access Device

Device Type                        : iDRAC8 Enterprise

iDRAC Ports                        : Present

IPMI Version                       : 2.0

System GUID                        : 3236374f-c0b3-5680-4310-004b4c4c4544

Number of Possible Active Sessions : 5

Number of Current Active Sessions  : 0

Enable IPMI Over LAN               : No

SOL Enabled                        : Yes

MAC Address                        : 18-FB-7B-9D-AD-FB

IPv4 Address

IP Address Source : Static

IP Address        : 10.219.11.79

IP Subnet         : 255.255.255.0

IP Gateway        : 10.219.11.254

[root@ds-gp-prd-sdw6 ~]#

[root@ds-gp-prd-sdw6 ~]# omreport chassis info

Chassis Information

Index                                    : 0

Chassis Name                             : Main System Chassis

Host Name                                : ds-gp-prd-sdw6

iDRAC8 Version                           : 2.15.10.10 (Build 8)

Lifecycle Controller Version             : 2.15.10.10

Chassis Model                            : PowerEdge R730xd

Chassis Lock                             : Present

Chassis Service Tag                      : 3KCV762

Express Service Code                     : 7761281546

Chassis Asset Tag                        : Unknown

Flash chassis identify LED state         : Off

Flash chassis identify LED timeout value : 300

[root@ds-gp-prd-sdw6 ~]#

[root@ds-gp-prd-sdw6 ~]# omreport chassis

Health

Main System Chassis

SEVERITY : COMPONENT

Ok       : Fans

Ok       : Intrusion

Ok       : Memory

Ok       : Power Supplies

Ok       : Power Management

Ok       : Processors

Ok       : Temperatures

Ok       : Voltages

Ok       : Hardware Log

Ok       : Batteries

For further help, type the command followed by -?

[root@ds-gp-prd-sdw6 ~]#

[root@ds-gp-prd-sdw6 ~]# omreport chassis firmware

Firmware Information

Version Information

iDRAC8               : 2.15.10.10 (Build 8)

Lifecycle Controller : 2.15.10.10

[root@ds-gp-prd-sdw6 ~]#

[root@ds-gp-prd-sdw6 ~]# omreport chassis bios

BIOS Information

Manufacturer : Dell Inc.

Version      : 1.3.6

Release Date : 06/03/2015

[root@ds-gp-prd-sdw6 ~]#

[root@ds-gp-prd-sdw6 ~]# omreport system summary

System Summary

------------------

Software Profile

------------------

Systems Management

Name                       : Dell OpenManage Server Administrator

Version                    : 8.1.0

Description                : Systems Management Software

Contains                   : A library for parsing GNU style command line arguments 8.1.0

: Data Engine 8.1.0

: Data Engine SNMP 8.1.0

: Disney SNMP 8.1.0

: Hardware Abstraction Programming Interface 8.1.0

: Instrumentation Services 8.1.0

: Meta package for installing the Server Administrator snmp component 8.1.0

: Meta package for installing the Server Administrator storage snmp component 8.1.0

: Meta package for installing the Server Agent 8.1.0

: NVMe Library 8.1.0

: OpenManage Inventory Collector 8.1.0

: RealSSD package for storage management 8.1.0

: Remote Access Card Data Populator 8.1.0

: Server Administrator CLI 8.1.0

: Server Administrator Common Framework 8.1.0

: Server Administrator Common Services 8.1.0

: Server Administrator Core 8.1.0

: Server Administrator XML Support SDK 8.1.0

: Storage Management SNMP component 5.0.1

: Storage Management System Utilities Package / Libsysfs 1.3.0

: Storage Management accessors package 5.0.1

: Storage Management common files for GUI and CLI 5.0.1

: StoreLib package for storage management 8.1.0

: System Utilities Package/Libsysfs for LSI storage libraries 2.1.0

: iDRAC SNMP components 8.1.0

: iDRAC6 Command Interface 8.1.0

: iDRAC7 Command Interface 8.1.0

Operating System

Name                       : Red Hat Enterprise Linux Server

Version                    : release 6.6 (Santiago) Kernel 2.6.32-504.el6.x86_64 (x86_64)

System Time                : Thu Nov 26 12:15:00 2015

System Bootup Time         : Wed Nov 25 18:03:25 2015

--------

System

--------

System

Host Name                  : ds-gp-prd-sdw6

System Location            : Please set the value

Lifecycle Controller       : Enabled

---------------------

Main System Chassis

---------------------

Chassis Information

Chassis Model              : PowerEdge R730xd

Chassis Service Tag        : 3KCV762

Express Service Code       : 7761281546

Chassis Lock               : Present

Chassis Asset Tag          : Unknown

Node Id                    : 3KCV762

Remote Access Information

Remote Access Device       : iDRAC8 Enterprise

vFlash Media               : Absent

Processor 1

Processor Brand            : Intel(R) Xeon(R) CPU E5-2650 v3 @ 2.30GHz

Processor Version          : Model 63 Stepping 2

Voltage                    : 1300 mV

Processor 2

Processor Brand            : Intel(R) Xeon(R) CPU E5-2650 v3 @ 2.30GHz

Processor Version          : Model 63 Stepping 2

Voltage                    : 1300 mV

Memory

Total Installed Capacity   : 196608  MB

Memory Available to the OS : 193587  MB

Total Maximum Capacity     : 1572864  MB

Memory Array Count         : 1

Memory Array 1

Location                   : System Board or Motherboard

Use                        : System Memory

Installed Capacity         : 196608  MB

Maximum Capacity           : 1572864  MB

Slots Available            : 24

Slots Used                 : 12

ECC Type                   : Multibit ECC

Slot PCIe Slot 1

Adapter                    : [Not Occupied]

Type                       : PCI E Gen 3 X16

Data Bus Width             : 8x or x8

Speed                      : [Not Obtained, see card documentation]

Slot Length                : Long

Voltage Supply             : 3.3 Volts

Slot PCIe Slot 2

Adapter                    : [Not Occupied]

Type                       : PCI E Gen 3 X16

Data Bus Width             : 8x or x8

Speed                      : [Not Obtained, see card documentation]

Slot Length                : Long

Voltage Supply             : 3.3 Volts

Slot PCIe Slot 3

Adapter                    : PERC H730P Adapter

Type                       : PCI E Gen 3 X16

Data Bus Width             : 8x or x8

Speed                      : [Not Obtained, see card documentation]

Slot Length                : Long

Voltage Supply             : 3.3 Volts

Slot PCIe Slot 4

Adapter                    : [Not Occupied]

Type                       : PCI E Gen 3

Data Bus Width             : 16x or x16

Speed                      : [Not Obtained, see card documentation]

Slot Length                : Long

Voltage Supply             : 3.3 Volts

Slot PCIe Slot 5

Adapter                    : [Not Occupied]

Type                       : PCI E Gen 3 X16

Data Bus Width             : 8x or x8

Speed                      : [Not Obtained, see card documentation]

Slot Length                : Long

Voltage Supply             : 3.3 Volts

Slot PCIe Slot 6

Adapter                    : [Not Occupied]

Type                       : PCI E Gen 3

Data Bus Width             : 16x or x16

Speed                      : [Not Obtained, see card documentation]

Slot Length                : Long

Voltage Supply             : 3.3 Volts

BIOS Information

Manufacturer               : Dell Inc.

Version                    : 1.3.6

Release Date               : 06/03/2015

Firmware Information

Name                       : iDRAC8

Version                    : 2.15.10.10 (Build 8)

Firmware Information

Name                       : Lifecycle Controller

Version                    : 2.15.10.10

--------------------------

Remote Access Controller

--------------------------

Remote Access Controller Information

Product                    : iDRAC8 Enterprise

IP Address                 : 10.219.11.79

IP Subnet                  : 255.255.255.0

IP Gateway                 : 10.219.11.254

IPv6 Address 1             : ::

IPv6 Address 2             : ::

IPv6 Gateway               : ::

--------------

Network Data

--------------

Network Interface 0

IP Address                 : 10.216.2.206

Subnet Mask                : 255.255.255.0

Default Gateway            : 10.216.2.254

MAC Address                : 44:A8:42:32:06:3E

Network Interface 1

IP Address                 : 10.216.3.206

Subnet Mask                : 255.255.255.0

Default Gateway            : 10.216.3.254

MAC Address                : 44:A8:42:32:06:40

Network Interface 2

IP Address                 : [No Value]

MAC Address                : 44:A8:42:32:06:42

Network Interface 3

IP Address                 : [No Value]

MAC Address                : 44:A8:42:32:06:44

[root@ds-gp-prd-sdw6 ~]#

[root@ds-gp-prd-sdw6 ~]# omreport chassis biossetup

Operation is under progress, it may take few seconds to complete...

Note: use display=shortnames optional parameter to see attribute names and setting values in short form.

BIOS Setup

System Information

------------------------------------------

System Model Name                                   : PowerEdge R730xd

System BIOS Version                                 : 1.3.6

System Management Engine Version                    : 3.0.7.164

System Service Tag                                  : 3KCV762

System Manufacturer                                 : Dell Inc.

System Manufacturer Contact Information             : www.dell.com

System CPLD Version                                 : 1.0.1

UEFI Compliance Version                             : 2.3.1

Memory Settings

------------------------------------------

System Memory Size                                  : 192 GB

System Memory Type                                  : ECC DDR4

System Memory Speed                                 : 2133 Mhz

System Memory Voltage                               : 1.20 V

Video Memory                                        : 16 MB

System Memory Testing                               : Disabled

Memory Operating Mode                               : Optimizer Mode

Node Interleaving                                   : Disabled

Snoop Mode                                          : Early Snoop

Correctable Memory ECC SMI                          : Enabled

Processor Settings

------------------------------------------

Logical Processor                                   : Enabled

QPI Speed                                           : Maximum data rate

Alternate RTID (Requestor Transaction ID) Setting   : Disabled

Virtualization Technology                           : Enabled

Address Translation Services (ATS)                  : Enabled

Adjacent Cache Line Prefetch                        : Enabled

Hardware Prefetcher                                 : Enabled

DCU Streamer Prefetcher                             : Enabled

DCU IP Prefetcher                                   : Enabled

Logical Processor Idling                            : Disabled

Configurable TDP                                    : Nominal

X2Apic Mode                                         : Disabled

Dell Controlled Turbo                               : Disabled

Number of Cores per Processor                       : All

Processor 64-bit Support                            : Yes

Processor Core Speed                                : 2.30 GHz

Processor Bus Speed                                 : 9.60 GT/s

Family-Model-Stepping                               : 6-3F-2

Brand                                               : Intel(R) Xeon(R) CPU E5-2650 v3 @ 2.30GHz

Level 2 Cache                                       : 10x256 KB

Level 3 Cache                                       : 25 MB

Number of Cores                                     : 10

Family-Model-Stepping                               : 6-3F-2

Brand                                               : Intel(R) Xeon(R) CPU E5-2650 v3 @ 2.30GHz

Level 2 Cache                                       : 10x256 KB

Level 3 Cache                                       : 25 MB

Number of Cores                                     : 10

SATA Settings

------------------------------------------

Embedded SATA                                       : AHCI Mode

Security Freeze Lock                                : Enabled

Write Cache                                         : Disabled

Port A                                              : Auto

Model                                               : Not Enumerated

Drive Type                                          : Not Enumerated

Capacity                                            : N/A

Port B                                              : Auto

Model                                               : Not Enumerated

Drive Type                                          : Not Enumerated

Capacity                                            : N/A

Boot Settings

------------------------------------------

Boot Mode                                           : BIOS

Boot Sequence Retry                                 : Enabled

Hard-Disk Failover                                  : Disabled

BIOS Boot Settings

------------------------------------------

Boot Sequence

: 0. Enabled: Integrated NIC 1 Port 1 Partition 1: QLogic MBA Slot 0100 v7.12.6

: 1. Enabled: Hard drive C:

Hard-Disk Drive Sequence

: 0. Integrated RAID Controller 1: PERC H730P Mini(bus 02 dev 00)

: 1. RAID Controller in Slot 3: PERC H730P Adapter(bus 83 dev 00)

One-Time Boot

------------------------------------------

One-Time Boot Device List                       : Disabled

BIOS Boot Sequence Device                       : Integrated NIC 1 Port 1 Partition 1: QLogic MBA Slot 0100 v7.12.6

BIOS Hard-Disk Drive Sequence Device            : Integrated RAID Controller 1: PERC H730P Mini(bus 02 dev 00)

Integrated Devices

------------------------------------------

USB 3.0 Setting                                     : Disabled

User Accessible USB Ports                           : All Ports On

Internal USB Port                                   : On

Integrated RAID Controller                          : Enabled

Integrated Network Card 1                           : Enabled

Integrated Network Card 1 Port 1 Boot Protocol      : PXE

Integrated Network Card 1 Port 2 Boot Protocol      : None

Integrated Network Card 1 Port 3 Boot Protocol      : None

Integrated Network Card 1 Port 4 Boot Protocol      : None

I/OAT DMA Engine                                    : Disabled

Embedded Video Controller                           : Enabled

Current State of Embedded Video Controller          : Enabled

SR-IOV Global Enable                                : Disabled

OS Watchdog Timer                                   : Disabled

Memory Mapped I/O above 4GB                         : Enabled

Slot Disablement

------------------------------------------

Slot 1                                          : Enabled

Slot 2                                          : Enabled

Slot 3                                          : Enabled

Slot 4                                          : Enabled

Slot 5                                          : Enabled

Slot 6                                          : Enabled

Serial Communication

------------------------------------------

Serial Communication                                : Auto

Serial Port Address                                 : Serial Device1=COM2,Serial Device2=COM1

External Serial Connector                           : Serial Device 1

Failsafe Baud Rate                                  : 115200

Remote Terminal Type                                : VT100/VT220

Redirection After Boot                              : Enabled

System Profile Settings

------------------------------------------

System Profile                                      : Performance

CPU Power Management                                : Maximum Performance

Memory Frequency                                    : Maximum Performance

Turbo Boost                                         : Enabled

Energy Efficient Turbo                              : Disabled

C1E                                                 : Disabled

C States                                            : Disabled

Collaborative CPU Performance Control               : Disabled

Memory Patrol Scrub                                 : Standard

Memory Refresh Rate                                 : 1x

Uncore Frequency                                    : Maximum

Energy Efficient Policy                             : Performance

Number of Turbo Boost Enabled Cores for Processor 1 : All

Number of Turbo Boost Enabled Cores for Processor 2 : All

Monitor/Mwait                                       : Enabled

Workload Profile                                    : Not Available

Power Saver                                         : Disabled

System Security

------------------------------------------

Intel(R) AES-NI                                     : Enabled

System Password                                     :

Setup Password                                      :

Password Status                                     : Unlocked

TPM Security                                        : Off

TPM Information                                 : No TPM present

TPM Status                                      : Unknown

TPM Command                                     : None

Intel(R) TXT                                        : Off

AC Power Recovery                                   : Last

AC Power Recovery Delay                             : Immediate

User Defined Delay (60s to 240s)                    : 0

UEFI Variable Access                                : Standard

Secure Boot                                         : Disabled

Secure Boot Policy                                  : Standard

Miscellaneous Settings

------------------------------------------

Asset Tag                                           :

Keyboard NumLock                                    : On

F1/F2 Prompt on Error                               : Enabled

Load Legacy Video Option ROM                        : Disabled

In-System Characterization                          : Enabled - No Reboot

[root@ds-gp-prd-sdw6 ~]#

[root@ds-gp-prd-sdw6 ~]# omreport chassis fru

System Components (FRU) Information

Device           : System Planar (iDRAC8)

Serial No.       : CN7016354M00AB

Part No.         : 0H21J3

Revision         : A07

Manufacturer     : DELL

Manufacture Date : Thu Apr 23 12:15:00 2015

Device           : Power Supply 1 (PS1)

Serial No.       : PH1629856A007V

Part No.         : 0TPJ2X

Revision         : A02

Manufacturer     : DELL

Manufacture Date : Wed Jun 10 12:26:00 2015

Device           : Power Supply 2 (PS2)

Serial No.       : PH1629856A007W

Part No.         : 0TPJ2X

Revision         : A02

Manufacturer     : DELL

Manufacture Date : Wed Jun 10 12:27:00 2015

Device           : Disk Drive Bay (BP1)

Serial No.       : CN779214CM00I3

Part No.         : 0DTCVP

Revision         : A00

Manufacturer     : DELL

Manufacture Date : Fri Jan 09 23:06:00 2015

Device           : Add-in Card (NDC)

Serial No.       : CN7792155P018E

Part No.         : 0165T0

Revision         : A00

Manufacturer     : DELL

Manufacture Date : Wed May 25 02:00:00 2005

Device           : Add-in Card (PERC1)

Serial No.       : CN7792155L01BE

Part No.         : 07H4CN

Revision         : A01

Manufacturer     : DELL

Manufacture Date : Thu May 28 00:33:00 2015

Device           : A1

Serial No.       : 40177310

Part No.         : M393A2G40DB0-CPB

Revision         : N/A

Manufacturer     : Samsung (00CE00B300CE)

Manufacture Date : Mon Feb 23 15:00:00 2015

Device           : A2

Serial No.       : 401772FD

Part No.         : M393A2G40DB0-CPB

Revision         : N/A

Manufacturer     : Samsung (00CE00B300CE)

Manufacture Date : Mon Feb 23 15:00:00 2015

Device           : A3

Serial No.       : 40177301

Part No.         : M393A2G40DB0-CPB

Revision         : N/A

Manufacturer     : Samsung (00CE00B300CE)

Manufacture Date : Mon Feb 23 15:00:00 2015

Device           : A4

Serial No.       : 40177339

Part No.         : M393A2G40DB0-CPB

Revision         : N/A

Manufacturer     : Samsung (00CE00B300CE)

Manufacture Date : Mon Feb 23 15:00:00 2015

Device           : A5

Serial No.       : 40177164

Part No.         : M393A2G40DB0-CPB

Revision         : N/A

Manufacturer     : Samsung (00CE00B300CE)

Manufacture Date : Mon Feb 23 15:00:00 2015

Device           : A6

Serial No.       : 40177163

Part No.         : M393A2G40DB0-CPB

Revision         : N/A

Manufacturer     : Samsung (00CE00B300CE)

Manufacture Date : Mon Feb 23 15:00:00 2015

Device           : B1

Serial No.       : 40177118

Part No.         : M393A2G40DB0-CPB

Revision         : N/A

Manufacturer     : Samsung (00CE00B300CE)

Manufacture Date : Mon Feb 23 15:00:00 2015

Device           : B2

Serial No.       : 4017729A

Part No.         : M393A2G40DB0-CPB

Revision         : N/A

Manufacturer     : Samsung (00CE00B300CE)

Manufacture Date : Mon Feb 23 15:00:00 2015

Device           : B3

Serial No.       : 40177321

Part No.         : M393A2G40DB0-CPB

Revision         : N/A

Manufacturer     : Samsung (00CE00B300CE)

Manufacture Date : Mon Feb 23 15:00:00 2015

Device           : B4

Serial No.       : 401772FB

Part No.         : M393A2G40DB0-CPB

Revision         : N/A

Manufacturer     : Samsung (00CE00B300CE)

Manufacture Date : Mon Feb 23 15:00:00 2015

Device           : B5

Serial No.       : 40177329

Part No.         : M393A2G40DB0-CPB

Revision         : N/A

Manufacturer     : Samsung (00CE00B300CE)

Manufacture Date : Mon Feb 23 15:00:00 2015

Device           : B6

Serial No.       : 411DF5F4

Part No.         : M393A2G40DB0-CPB

Revision         : N/A

Manufacturer     : Samsung (00CE00B300CE)

Manufacture Date : Mon Aug 24 15:00:00 2015

[root@ds-gp-prd-sdw6 ~]#

#	Get detail about memory modules.

[root@ds-gp-prd-sdw7 ~]# omreport chassis memory

Memory Information

Health : Critical

Attributes of Memory Array(s)

Attributes of Memory Array(s)

Location           : System Board or Motherboard

Use                : System Memory

Installed Capacity : 196608  MB

Maximum Capacity   : 1572864  MB

Slots Available    : 24

Slots Used         : 12

Error Correction   : Multibit ECC

Total of Memory Array(s)

Total Installed Capacity                     : 196608  MB

Total Installed Capacity Available to the OS : 193587  MB

Total Maximum Capacity                       : 1572864  MB

Details of Memory Array 1

Index          : 0

Status         : Ok

Connector Name : A1

Type           : DDR4 - Synchronous Registered (Buffered)

Size           : 16384  MB

Index          : 1

Status         : Ok

Connector Name : A2

Type           : DDR4 - Synchronous Registered (Buffered)

Size           : 16384  MB

Index          : 2

Status         : Ok

Connector Name : A3

Type           : DDR4 - Synchronous Registered (Buffered)

Size           : 16384  MB

Index          : 3

Status         : Ok

Connector Name : A4

Type           : DDR4 - Synchronous Registered (Buffered)

Size           : 16384  MB

Index          : 4

Status         : Ok

Connector Name : A5

Type           : DDR4 - Synchronous Registered (Buffered)

Size           : 16384  MB

Index          : 5

Status         : Ok

Connector Name : A6

Type           : DDR4 - Synchronous Registered (Buffered)

Size           : 16384  MB

Index          :

Status         : Unknown

Connector Name : A7

Type           : [Not Occupied]

Size           :

Index          :

Status         : Unknown

Connector Name : A8

Type           : [Not Occupied]

Size           :

Index          :

Status         : Unknown

Connector Name : A9

Type           : [Not Occupied]

Size           :

Index          :

Status         : Unknown

Connector Name : A10

Type           : [Not Occupied]

Size           :

Index          :

Status         : Unknown

Connector Name : A11

Type           : [Not Occupied]

Size           :

Index          :

Status         : Unknown

Connector Name : A12

Type           : [Not Occupied]

Size           :

Index          : 6

Status         : Ok

Connector Name : B1

Type           : DDR4 - Synchronous Registered (Buffered)

Size           : 16384  MB

Index          : 7

Status         : Critical

Connector Name : B2

Type           : DDR4 - Synchronous Registered (Buffered)

Size           : 16384  MB

Index          : 8

Status         : Ok

Connector Name : B3

Type           : DDR4 - Synchronous Registered (Buffered)

Size           : 16384  MB

Index          : 9

Status         : Ok

Connector Name : B4

Type           : DDR4 - Synchronous Registered (Buffered)

Size           : 16384  MB

Index          : 10

Status         : Ok

Connector Name : B5

Type           : DDR4 - Synchronous Registered (Buffered)

Size           : 16384  MB

Index          : 11

Status         : Ok

Connector Name : B6

Type           : DDR4 - Synchronous Registered (Buffered)

Size           : 16384  MB

Index          :

Status         : Unknown

Connector Name : B7

Type           : [Not Occupied]

Size           :

Index          :

Status         : Unknown

Connector Name : B8

Type           : [Not Occupied]

Size           :

Index          :

Status         : Unknown

Connector Name : B9

Type           : [Not Occupied]

Size           :

Index          :

Status         : Unknown

Connector Name : B10

Type           : [Not Occupied]

Size           :

Index          :

Status         : Unknown

Connector Name : B11

Type           : [Not Occupied]

Size           :

Index          :

Status         : Unknown

Connector Name : B12

Type           : [Not Occupied]

Size           :

[root@ds-gp-prd-sdw7 ~]#

[root@ds-gp-prd-sdw7 ~]# omreport chassis memory index=7

Memory Device Information

Health : Critical

Status      : Critical

Device Name : B2

Size        : 16384 MB

Type        : DDR4 Synchronous Registered (Buffered)

Speed       : 0.47 ns

Rank        : Dual

Failures    : Single-bit warning error rate exceeded.Single-bit failure error rate exceeded.

[root@ds-gp-prd-sdw7 ~]#

[root@ds-gp-prd-sdw7 ~]# omreport chassis fru | grep -A 6 B2

Device           : B2

Serial No.       : 40179555

Part No.         : M393A2G40DB0-CPB

Revision         : N/A

Manufacturer     : Samsung (00CE00B300CE)

Manufacture Date : Mon Feb 23 15:00:00 2015

[root@ds-gp-prd-sdw7 ~]#

#	Other way to use grep. This way is more prefferable.

[root@m1-2-gp-prd-sdw36 ~]# omreport chassis fru | grep -A 6 -e "Device *: *A3"

Device           : A3

Serial No.       : 1626BB21

Part No.         : 18ASF2G72PDZ-2G3B1

Revision         : N/A

Manufacturer     : Micron Technology (002C00B3002C)

Manufacture Date : Mon Mar 06 15:00:00 2017

[root@m1-2-gp-prd-sdw36 ~]#

20151203

#   After memory module have replaced we can see, that module still in Critical state.

#   But after cleaning procedure - everything become Ok.

[root@ds-gp-prd-sdw7 ~]# omreport chassis memory index=7

Memory Device Information

Health : Critical

Status      : Critical

Device Name : B2

Size        : 16384 MB

Type        : DDR4 Synchronous Registered (Buffered)

Speed       : 0.47 ns

Rank        : Dual

Failures    : Single-bit failure error rate exceeded.

[root@ds-gp-prd-sdw7 ~]# dcicfg command=clearmemfailures failures=all

Clearing failures using mask: 31

A1: ok

A2: ok

A3: ok

A4: ok

A5: ok

A6: ok

B1: ok

B2: ok

B3: ok

B4: ok

B5: ok

B6: ok

[root@ds-gp-prd-sdw7 ~]# omreport chassis memory index=7

Memory Device Information

Health : Ok

Status      : Ok

Device Name : B2

Size        : 16384 MB

Type        : DDR4 Synchronous Registered (Buffered)

Speed       : 0.47 ns

Rank        : Dual

Failures    : None

[root@ds-gp-prd-sdw7 ~]#

https://www.dell.com/support/article/ru/ru/rubsdc/sln131078/%D0%BA%D0%B0%D0%BA-%D0%BE%D1%87%D0%B8%D1%81%D1%82%D0%B8%D1%82%D1%8C-%D0%B6%D1%83%D1%80%D0%BD%D0%B0%D0%BB-sbe-%D0%B4%D0%BB%D1%8F-%D0%BC%D0%BE%D0%B4%D1%83%D0%BB%D1%8F-%D0%BF%D0%B0%D0%BC%D1%8F%D1%82%D0%B8-dimm-%D0%BD%D0%B0-%D1%81%D0%B5%D1%80%D0%B2%D0%B5%D1%80%D0%B5-dell-poweredge?lang=ru

20151125

#	Turn on LED indication on server.

[root@ds-gp-prd-sdw6 ~]#

[root@ds-gp-prd-sdw6 ~]# racadm setled -l 1

RAC0907 : System ID LED blink on.

[root@ds-gp-prd-sdw6 ~]#

[root@ds-gp-prd-sdw6 ~]# racadm getled

LED State :    Blinking

[root@ds-gp-prd-sdw6 ~]#

[root@ds-gp-prd-sdw6 ~]# racadm help setled

setled -- Set the state of the LED on a module.

Usage:

racadm setled [-l <ledState>]

Options:

-l <ledState>    0 = No blinking, 1 = Blinking

Examples:

From the iDRAC make the LED blink

$ racadm setled -l 1

LED state was set successfully

From the iDRAC stop the LED from blinking

$ racadm setled -l 0

LED state was cleared successfully

For help on viewing the properties of a group, run the command - racadm help get

For help on configuring the properties of a group, run the command - racadm help set

-----------------------------------------------------------------------

[root@ds-gp-prd-sdw6 ~]#

http://www.dell.com/support/manuals/us/en/19/integrated-dell-remote-access-cntrllr-8-with-lifecycle-controller-v2.00.00.00/racadm_idrac_pub-v1/storage?guid=GUID-9E3676CB-B71D-420B-8C48-C80ADD258E03&lang=en-us

20160725

#	Turn on LED on hard drive.

[root@ds-gp-prd-sdw24 20160722-1746]# racadm storage blink:Disk.Bay.2:Enclosure.Internal.0-1:RAID.Integrated.1-1

STOR095 : Storage operation is successfully completed.

[root@ds-gp-prd-sdw24 20160722-1746]#

#	Turn off LED indication on hard drive.

[root@ds-gp-prd-sdw24 20160722-1746]# racadm storage unblink:Disk.Bay.2:Enclosure.Internal.0-1:RAID.Integrated.1-1

STOR095 : Storage operation is successfully completed.

[root@ds-gp-prd-sdw24 20160722-1746]#

20151201

#	Display only records started from defined number.

[root@ds-gp-prd-sdw7 ~]# racadm getsel -s 5

Record:      5

Date/Time:   10/08/2015 19:23:00

Source:      system

Severity:    Ok

Description: Drive 13 is installed in disk drive bay 1.

-------------------------------------------------------------------------------

Record:      6

Date/Time:   12/01/2015 11:48:01

Source:      system

Severity:    Non-Critical

Description: Correctable memory error rate exceeded for DIMM_B2.

-------------------------------------------------------------------------------

Record:      7

Date/Time:   12/01/2015 11:48:09

Source:      system

Severity:    Critical

Description: Correctable memory error rate exceeded for DIMM_B2.

-------------------------------------------------------------------------------

[root@ds-gp-prd-sdw7 ~]#

#	Same as above, but with other utility.

[root@ds-gp-prd-sdw7 ~]# omreport system alertlog | head -n 11

Alert Log

Alert Log contains...

Severity      : Critical

ID            : 1404

Date and Time : Tue Dec  1 11:48:16 2015

Category      : Instrumentation Service

Description   : Memory device status is critical

Memory device location: B2

Possible memory module event cause:Single bit warning error rate exceeded,Single bit failure error rate exceeded

omreportUnsuccessful command execution!

[root@ds-gp-prd-sdw7 ~]#

20151124

#	Get messages from log about some out of order details.

[root@ds-gp-prd-sdw6 ~]# racadm getsel

Record:      1

Date/Time:   08/20/2015 15:42:26

Source:      system

Severity:    Ok

Description: Log cleared.

-------------------------------------------------------------------------------

Record:      2

Date/Time:   11/24/2015 10:13:38

Source:      system

Severity:    Non-Critical

Description: Correctable memory error rate exceeded for DIMM_B6.

-------------------------------------------------------------------------------

[root@ds-gp-prd-sdw6 ~]#

#	Here is detail about hardware state.

[root@ds-gp-prd-sdw6 ~]# racadm getsensorinfo

…

Sensor Type : MEMORY

<Sensor Name>                   <Status>    <State>             <lc>        <uc>

DIMM A1                         Ok          Presence Detected   NA          NA

DIMM A2                         Ok          Presence Detected   NA          NA

DIMM A3                         Ok          Presence Detected   NA          NA

DIMM A4                         Ok          Presence Detected   NA          NA

DIMM A5                         Ok          Presence Detected   NA          NA

DIMM A6                         Ok          Presence Detected   NA          NA

DIMM A7                         N/A         Absent              NA          NA

DIMM A8                         N/A         Absent              NA          NA

DIMM A9                         N/A         Absent              NA          NA

DIMM A10                        N/A         Absent              NA          NA

DIMM A11                        N/A         Absent              NA          NA

DIMM A12                        N/A         Absent              NA          NA

DIMM B1                         Ok          Presence Detected   NA          NA

DIMM B2                         Ok          Presence Detected   NA          NA

DIMM B3                         Ok          Presence Detected   NA          NA

DIMM B4                         Ok          Presence Detected   NA          NA

DIMM B5                         Ok          Presence Detected   NA          NA

DIMM B6                         Warning     Presence Detected   NA          NA

DIMM B7                         N/A         Absent              NA          NA

DIMM B8                         N/A         Absent              NA          NA

DIMM B9                         N/A         Absent              NA          NA

DIMM B10                        N/A         Absent              NA          NA

DIMM B11                        N/A         Absent              NA          NA

DIMM B12                        N/A         Absent              NA          NA

…

#	Get idrac ip-address and other usefull info (Service Tag, Model and so on).

[root@ds-gp-prd-sdw6 ~]# racadm getsysinfo

…

IPv4 settings:

Enabled                 = 1

Current IP Address      = 10.219.11.79

Current IP Gateway      = 10.219.11.254

Current IP Netmask      = 255.255.255.0

DHCP Enabled            = 0

Current DNS Server 1    = 0.0.0.0

Current DNS Server 2    = 0.0.0.0

DNS Servers from DHCP   = Disabled

...

System Information:

System Model            = PowerEdge R730xd

System Revision         = I

System BIOS Version     = 1.3.6

Service Tag             = 3KCV762

Express Svc Code        = 7761281546

Host Name               = sdw6

OS Name                 = Red Hat Enterprise Linux Server

OS Version              = release 6.6 (Santiago) Kernel 2.6.32-504.el6.x86_64 (x86_64)

Power Status            = ON

Fresh Air Capable       = Yes

…

#	Here we can see LED status on this server.

[root@ds-gp-prd-sdw6 ~]# racadm getled

LED State :    Not-Blinking

[root@ds-gp-prd-sdw6 ~]#

Normal,"Thu Aug 20 2015 15:42:26","Log cleared."

Warning,"Tue Nov 24 2015 10:13:38","Correctable memory error rate exceeded for DIMM_B6."

#	Get last ten logged messages of idrac.

[root@ds-gp-prd-sdw6 ~]# racadm getraclog -c 10

SeqNumber       = 162

Message ID      = ISM0004

Category        = Audit

AgentID         = Unknown

Severity        = Information

Timestamp       = 2015-08-25 10:12:09

Message         = The iDRAC Service Module has successfully started communication with iDRAC.

--------------------------------------------------------------------------------

...

20151008

#	Operating with Dell hard drives.

[root@ds-gp-prd-sdw7 ~]# racadm storage get vdisks -o -p layout,status

Disk.Virtual.0:RAID.Integrated.1-1

Layout                           = Raid-1

Status                           = Ok

Disk.Virtual.0:RAID.Slot.3-1

Layout                           = Raid-10

Status                           = Warning

Disk.Virtual.1:RAID.Integrated.1-1

Layout                           = Raid-10

Status                           = Ok

[root@ds-gp-prd-sdw7 ~]#

[root@ds-gp-prd-sdw7 ~]# racadm storage get pdisks -o -p status

Disk.Bay.0:Enclosure.Internal.0-1:RAID.Integrated.1-1

Status                           = Ok

Disk.Bay.1:Enclosure.Internal.0-1:RAID.Integrated.1-1

Status                           = Ok

Disk.Bay.2:Enclosure.Internal.0-1:RAID.Integrated.1-1

Status                           = Ok

Disk.Bay.3:Enclosure.Internal.0-1:RAID.Integrated.1-1

Status                           = Ok

Disk.Bay.4:Enclosure.Internal.0-1:RAID.Integrated.1-1

Status                           = Ok

Disk.Bay.5:Enclosure.Internal.0-1:RAID.Integrated.1-1

Status                           = Ok

Disk.Bay.6:Enclosure.Internal.0-1:RAID.Integrated.1-1

Status                           = Ok

Disk.Bay.7:Enclosure.Internal.0-1:RAID.Integrated.1-1

Status                           = Ok

Disk.Bay.8:Enclosure.Internal.0-1:RAID.Integrated.1-1

Status                           = Ok

Disk.Bay.12:Enclosure.Internal.0-1:RAID.Slot.3-1

Status                           = Ok

Disk.Bay.13:Enclosure.Internal.0-1:RAID.Slot.3-1

Status                           = Failed

Disk.Bay.14:Enclosure.Internal.0-1:RAID.Slot.3-1

Status                           = Ok

Disk.Bay.15:Enclosure.Internal.0-1:RAID.Slot.3-1

Status                           = Ok

Disk.Bay.16:Enclosure.Internal.0-1:RAID.Slot.3-1

Status                           = Ok

Disk.Bay.17:Enclosure.Internal.0-1:RAID.Slot.3-1

Status                           = Ok

Disk.Bay.18:Enclosure.Internal.0-1:RAID.Slot.3-1

Status                           = Ok

Disk.Bay.19:Enclosure.Internal.0-1:RAID.Slot.3-1

Status                           = Ok

Disk.Bay.9:Enclosure.Internal.0-1:RAID.Integrated.1-1

Status                           = Ok

[root@ds-gp-prd-sdw7 ~]#

[root@ds-gp-prd-sdw7 ~]# racadm storage get pdisks:Disk.Bay.13:Enclosure.Internal.0-1:RAID.Slot.3-1

Disk.Bay.13:Enclosure.Internal.0-1:RAID.Slot.3-1

Status                           = Failed

DeviceDescription                = Disk 13 in Backplane 1 of RAID Controller in Slot 3

RollupStatus                     = Failed

Name                             = Physical Disk 0:1:13

State                            = Failed

OperationState                   = Not Applicable

PowerStatus                      = Spun-Up

Size                             = 558.38 GB

FailurePredicted                 = NO

RemainingRatedWriteEndurance     = Not Applicable

SecurityStatus                   = Not Capable

BusProtocol                      = SAS

MediaType                        = HDD

UsedRaidDiskSpace                = 558.38 GB

AvailableRaidDiskSpace           = 558.38 GB

Hotspare                         = NO

Manufacturer                     = TOSHIBA

ProductId                        = AL13SEB600

Revision                         = DE11

SerialNumber                     = 6590A03LFRD3

PartNumber                       = PH05TFDD7557156CFKXJA06

NegotiatedSpeed                  = 6.0 Gb/s

ManufacturedDay                  = 6

ManufacturedWeek                 = 24

ManufacturedYear                 = 2015

SasAddress                       = 0x500003965808297E

FormFactor                       = 2.5 Inch

RaidNominalMediumRotationRate    = 10000

T10PICapability                  = Not Capable

BlockSizeInBytes                 = 512

MaxCapableSpeed                  = 6 Gb/s

[root@ds-gp-prd-sdw7 ~]#

#	When hard drive has been removed from server.

[root@ds-gp-prd-sdw7 ~]# racadm storage get pdisks:Disk.Bay.13:Enclosure.Internal.0-1:RAID.Slot.3-1

Disk.Bay.13:Enclosure.Internal.0-1:RAID.Slot.3-1

Status                           = Failed

DeviceDescription                = Disk 13 in Backplane 1 of RAID Controller in Slot 3

RollupStatus                     = Failed

Name                             = Physical Disk 0:1:13

State                            = Removed

OperationState                   = Not Applicable

PowerStatus                      = Spun-Up

Size                             = 558.38 GB

FailurePredicted                 = NO

RemainingRatedWriteEndurance     = Not Applicable

SecurityStatus                   = Not Capable

BusProtocol                      = SAS

MediaType                        = HDD

UsedRaidDiskSpace                = 558.38 GB

AvailableRaidDiskSpace           = 558.38 GB

Hotspare                         = NO

Manufacturer                     = TOSHIBA

ProductId                        = AL13SEB600

Revision                         = DE11

SerialNumber                     = 6590A03LFRD3

PartNumber                       = PH05TFDD7557156CFKXJA06

NegotiatedSpeed                  = 6.0 Gb/s

ManufacturedDay                  = 6

ManufacturedWeek                 = 24

ManufacturedYear                 = 2015

SasAddress                       = 0x500003965808297E

FormFactor                       = 2.5 Inch

RaidNominalMediumRotationRate    = 10000

T10PICapability                  = Not Capable

BlockSizeInBytes                 = 512

MaxCapableSpeed                  = 6 Gb/s

[root@ds-gp-prd-sdw7 ~]#

#	After 30 seconds when new hard drive have been inserted.

[root@ds-gp-prd-sdw7 ~]# racadm storage get pdisks:Disk.Bay.13:Enclosure.Internal.0-1:RAID.Slot.3-1

Disk.Bay.13:Enclosure.Internal.0-1:RAID.Slot.3-1

Status                           = Ok

DeviceDescription                = Disk 13 in Backplane 1 of RAID Controller in Slot 3

RollupStatus                     = Ok

Name                             = Physical Disk 0:1:13

State                            = Online

OperationState                   = Rebuilding

Progress                         = 1 %

PowerStatus                      = Spun-Up

Size                             = 558.38 GB

FailurePredicted                 = NO

RemainingRatedWriteEndurance     = Not Applicable

SecurityStatus                   = Not Capable

BusProtocol                      = SAS

MediaType                        = HDD

UsedRaidDiskSpace                = 558.38 GB

AvailableRaidDiskSpace           = 0.00 GB

Hotspare                         = NO

Manufacturer                     = SEAGATE

ProductId                        = ST600MM0006

Revision                         = LS0A

SerialNumber                     = S0M5SQW4

PartNumber                       = CN07YX587262256T00QHA02

NegotiatedSpeed                  = 6.0 Gb/s

ManufacturedDay                  = 4

ManufacturedWeek                 = 27

ManufacturedYear                 = 2015

SasAddress                       = 0x5000C50089D7B789

FormFactor                       = 2.5 Inch

RaidNominalMediumRotationRate    = 10000

T10PICapability                  = Not Capable

BlockSizeInBytes                 = 512

MaxCapableSpeed                  = 6 Gb/s

[root@ds-gp-prd-sdw7 ~]#

[root@ds-gp-prd-sdw7 ~]# racadm storage get pdisks:Disk.Bay.13:Enclosure.Internal.0-1:RAID.Slot.3-1 -p Status,OperationState,Progress

Disk.Bay.13:Enclosure.Internal.0-1:RAID.Slot.3-1

Status                           = Ok

OperationState                   = Rebuilding

Progress                         = 5 %

[root@ds-gp-prd-sdw7 ~]#

[root@ds-gp-prd-sdw7 ~]# racadm storage get pdisks -o -p name,status

Disk.Bay.0:Enclosure.Internal.0-1:RAID.Integrated.1-1

Name                             = Physical Disk 0:1:0

Status                           = Ok

Disk.Bay.1:Enclosure.Internal.0-1:RAID.Integrated.1-1

Name                             = Physical Disk 0:1:1

Status                           = Ok

Disk.Bay.2:Enclosure.Internal.0-1:RAID.Integrated.1-1

Name                             = Physical Disk 0:1:2

Status                           = Ok

Disk.Bay.3:Enclosure.Internal.0-1:RAID.Integrated.1-1

Name                             = Physical Disk 0:1:3

Status                           = Ok

Disk.Bay.4:Enclosure.Internal.0-1:RAID.Integrated.1-1

Name                             = Physical Disk 0:1:4

Status                           = Ok

Disk.Bay.5:Enclosure.Internal.0-1:RAID.Integrated.1-1

Name                             = Physical Disk 0:1:5

Status                           = Ok

Disk.Bay.6:Enclosure.Internal.0-1:RAID.Integrated.1-1

Name                             = Physical Disk 0:1:6

Status                           = Ok

Disk.Bay.7:Enclosure.Internal.0-1:RAID.Integrated.1-1

Name                             = Physical Disk 0:1:7

Status                           = Ok

Disk.Bay.8:Enclosure.Internal.0-1:RAID.Integrated.1-1

Name                             = Physical Disk 0:1:8

Status                           = Ok

Disk.Bay.12:Enclosure.Internal.0-1:RAID.Slot.3-1

Name                             = Physical Disk 0:1:12

Status                           = Ok

Disk.Bay.13:Enclosure.Internal.0-1:RAID.Slot.3-1

Name                             = Physical Disk 0:1:13

Status                           = Failed

Disk.Bay.14:Enclosure.Internal.0-1:RAID.Slot.3-1

Name                             = Physical Disk 0:1:14

Status                           = Ok

Disk.Bay.15:Enclosure.Internal.0-1:RAID.Slot.3-1

Name                             = Physical Disk 0:1:15

Status                           = Ok

Disk.Bay.16:Enclosure.Internal.0-1:RAID.Slot.3-1

Name                             = Physical Disk 0:1:16

Status                           = Ok

Disk.Bay.17:Enclosure.Internal.0-1:RAID.Slot.3-1

Name                             = Physical Disk 0:1:17

Status                           = Ok

Disk.Bay.18:Enclosure.Internal.0-1:RAID.Slot.3-1

Name                             = Physical Disk 0:1:18

Status                           = Ok

Disk.Bay.19:Enclosure.Internal.0-1:RAID.Slot.3-1

Name                             = Physical Disk 0:1:19

Status                           = Ok

Disk.Bay.9:Enclosure.Internal.0-1:RAID.Integrated.1-1

Name                             = Physical Disk 0:1:9

Status                           = Ok

[root@ds-gp-prd-sdw7 ~]#

[root@ds-gp-prd-sdw7 ~]# racadm storage get controllers:RAID.Slot.3-1

RAID.Slot.3-1

Status                           = Ok

DeviceDescription                = RAID Controller in Slot 3

RollupStatus                     = Warning

Name                             = PERC H730P Adapter (PCI Slot 3)

PciSlot                          = 3

FirmwareVersion                  = 25.3.0.0016

DriverVersion                    = --NA--

RebuildRate                      = 30

BgiRate                          = 30

CheckConsistencyRate             = 30

ReconstructRate                  = 30

PatrolReadRate                   = 30

PatrolReadMode                   = Automatic

PatrolReadState                  = Stopped

CheckConsistencyMode             = Normal

LoadBalanceSetting               = Auto

CopybackMode                     = ON

PreservedCache                   = Not Present

CacheMemorySize                  = 2048 MB

PersistHotspare                  = Disabled

SpindownUnconfiguredDrives       = Disabled

SpindownHotspare                 = Disabled

Timeintervalforspindown          = 30 (Minutes)

SecurityStatus                   = Encryption Capable

EncryptionMode                   = None

SasAddress                       = 0x544A842031B66100

PciDeviceId                      = 0x5d

PciSubdeviceId                   = 0x1f42

PciVendorId                      = 0x1000

PciSubvendorId                   = 0x1028

PciBus                           = 0x83

PciDevice                        = 0x0

PciFunction                      = 0x0

BusWidth                         = 8x or x8

SlotLength                       = Long Length

SlotType                         = PCI Express Gen3 x16

MaxCapableSpeed                  = 12.0 Gb/s

LearnMode                        = Not supported

T10PICapability                  = Capable

SupportRAID10UnevenSpans         = Supported

SupportEnhancedAutoForeignImport = Supported

EnhancedAutoImportForeignConfig  = Disabled

SupportControllerBootMode        = Supported

ControllerBootMode               = Continue Boot On Error

RealtimeConfigurationCapability  = Capable

CurrentControllerMode            = RAID

[root@ds-gp-prd-sdw7 ~]#

[root@ds-gp-prd-sdw7 ~]# racadm storage get controllers -o -p name,status

RAID.Integrated.1-1

Name                             = PERC H730P Mini (Embedded)

Status                           = Ok

RAID.Slot.3-1

Name                             = PERC H730P Adapter (PCI Slot 3)

Status                           = Ok

[root@ds-gp-prd-sdw7 ~]#

[root@ds-gp-prd-sdw7 ~]# racadm help raid

Storage monitoring, inventory and configuration of hardware RAID connected to the server.

Usage :

racadm storage get status

racadm storage get <Object type I>

racadm storage get <Object type I> -o

racadm storage get <Object type I> -o -p <property names separated by comma>

racadm storage get <Object type I>:<FQDD's of Object type separated by comma>

-p <property names separated by comma>

racadm storage get <Object type I>:<FQDD's of Object type separated by comma>

racadm storage get <Object type II> --refkey <reference keys separated by comma>

racadm storage get <Object type II> --refkey <reference keys separated by comma> -o

racadm storage get <Object type II> --refkey <reference keys separated by comma> -o

-p <property names separated by comma>

racadm storage <Object type III>:<FQDD>

racadm storage hotspare:<Physical Disk FQDD> -assign yes  -type dhs -vdkey:<FQDD of VD>

racadm storage init:<VD FQDD> -speed fast }

racadm storage createvd:<Controller FQDD> -rl {r0|r1|r5|r6|r10|r50|r60} [-wp {wt|wb}] [-rp {nra|ra|ara}]

[-ss {1k|2k|4k|8k|16k|32k|64k|128k|256k|512k|1M|2M|4M|8M|16M}] -pdkey:<comma separated PD FQDD>

[-dcp {enabled|disabled|default}] [-name <VD name>] [-size <VD size>{b|k|m|g|t}]

racadm storage createsecuritykey:<Controller FQDD> -key <Key id> -passwd <passphrase>

racadm storage modifysecuritykey:<Controller FQDD> -key <Key id>

-oldpasswd <old passphrase> -newpasswd <new passphrase>

racadm storage blink:<PD FQDD>

racadm storage unblink:<PD FQDD>

racadm storage preparetoremove:<PCIeSSD FQDD>

racadm storage secureerase:<PCIeSSD FQDD>

racadm storage patrolread:<Controller FQDD> -state start

--------------------------------------------------------------------------------

Valid Options:

Object type I      : controllers, batteries, vdisks, pdisks, fans, emms, temppro

bes, psus, enclosures.

Object type II     : batteries, vdisks, pdisks, fans, emms, tempprobes, psus, enclosures.

Object type III    : resetconfig, clearconfig, importconfig, deletesecuritykey, encryptvd,

deletevd, converttoraid, converttononraid, ccheck, cancelcheck

-o                 : Displays all the properties of the selected Key or Object.

-p                 : Displays the property names with filter.

FQDD's             : Displays all the properties of the FQDD's Key.

NOTE: Maximum Property names can be specified in -p option is = 10.

NOTE: Maximum FQDD's can be specified is = 3.

--------------------------------------------------------------------------------

Usage Examples :

racadm storage get controllers

racadm storage get controllers -o

racadm storage get controllers -o -p name,status

racadm storage get vdisks -o -p layout,status

racadm storage get controllers:RAID.Integrated.1-1

racadm storage get controllers:RAID.Integrated.1-1 -p status

racadm storage resetconfig:RAID.Integrated.1-1

racadm storage deletevd:Disk.Virtual.0:RAID.Integrated.1-1

racadm storage clearconfig:RAID.Integrated.1-1

racadm storage patrolread:RAID.Integrated.1-1 -state start

racadm storage importconfig:RAID.Integrated.1-1

racadm storage hotspare:Disk.Bay.0:Enclosure.Internal.0-0:RAID.Integrated.1-1 -assign yes -type dhs -vdkey:Disk.Virtual.0:RAID.Integrated.1-1

racadm storage init:Disk.Virtual.0:RAID.Integrated.1-1 -speed fast }

racadm storage createvd:RAID.Integrated.1-1 -rl r0 -wp wt -rp nra -name raid_0 -pdkey:Disk.Bay.0:Enclosure.Internal.0-0:RAID.Integrated.1-1,Disk.Bay.1:Enclosure.Internal.0-0:RAID.Integrated.1-1

racadm storage createsecuritykey:RAID.Integrated.1-1 -key <Key id> -passwd <passphrase>

racadm storage modifysecuritykey:RAID.Integrated.1-1 -key <Key id> -oldpasswd <old passphrase> -newpasswd <new passphrase>

racadm storage deletesecuritykey:RAID.Integrated.1-1

racadm storage encryptvd:Disk.Virtual.0:RAID.Integrated.1-1

racadm storage converttoraid:Disk.Bay.0:Enclosure.Internal.0-0:RAID.Integrated.1-1

racadm storage converttononraid:Disk.Bay.0:Enclosure.Internal.0-0:RAID.Integrated.1-1

racadm storage blink:Disk.Bay.0:Enclosure.Internal.0-0:RAID.Integrated.1-1

racadm storage unblink:Disk.Bay.0:Enclosure.Internal.0-0:RAID.Integrated.1-1

racadm storage preparetoremove:Disk.Bay.6:Enclosure.Internal.0-1:PCIeExtender.Slot.1

racadm storage secureerase:Disk.Bay.6:Enclosure.Internal.0-1:PCIeExtender.Slot.1

racadm storage ccheck:Disk.Virtual.0:RAID.Integrated.1-1

racadm storage cancelcheck:Disk.Virtual.0:RAID.Integrated.1-1

For help on viewing the properties of a group, run the command - racadm help get

For help on configuring the properties of a group, run the command - racadm help set

-----------------------------------------------------------------------

[root@ds-gp-prd-sdw7 ~]#

#

#	20181026

#	Brief info about HDDs.

#

[root@ds-pr-onlacq01 20181026-1952]# racadm storage get pdisks -o -p Status,Size,BusProtocol,MediaType,NegotiatedSpeed,FormFactor

Disk.Bay.0:Enclosure.Internal.0-1:RAID.Integrated.1-1

Status                           = Ok

Size                             = 278.88 GB

BusProtocol                      = SAS

MediaType                        = HDD

NegotiatedSpeed                  = 12.0 Gb/s

FormFactor                       = 2.5 Inch

Disk.Bay.1:Enclosure.Internal.0-1:RAID.Integrated.1-1

Status                           = Ok

Size                             = 278.88 GB

BusProtocol                      = SAS

MediaType                        = HDD

NegotiatedSpeed                  = 12.0 Gb/s

FormFactor                       = 2.5 Inch

Disk.Bay.2:Enclosure.Internal.0-1:RAID.Integrated.1-1

Status                           = Ok

Size                             = 278.88 GB

BusProtocol                      = SAS

MediaType                        = HDD

NegotiatedSpeed                  = 12.0 Gb/s

FormFactor                       = 2.5 Inch

Disk.Bay.3:Enclosure.Internal.0-1:RAID.Integrated.1-1

Status                           = Ok

Size                             = 278.88 GB

BusProtocol                      = SAS

MediaType                        = HDD

NegotiatedSpeed                  = 12.0 Gb/s

FormFactor                       = 2.5 Inch

[root@ds-pr-onlacq01 20181026-1952]#

20161226

#	Here is how to looks predicted failure for disks.

#	We can see it in nagios as a warning.

#	We can get alert email from Dell.

#	In status of virtual disks we see nothing.

[root@m1-splunk-nidx01 ~]# racadm storage get vdisks -o -p layout,status

Disk.Virtual.0:RAID.Integrated.1-1

Layout                           = Raid-10

Status                           = Ok

[root@m1-splunk-nidx01 ~]#

#	In status list of hard drives we see warning.

[root@m1-splunk-nidx01 ~]# racadm storage get pdisks -o -p status

Disk.Bay.0:Enclosure.Internal.0-1:RAID.Integrated.1-1

Status                           = Ok

Disk.Bay.1:Enclosure.Internal.0-1:RAID.Integrated.1-1

Status                           = Ok

Disk.Bay.2:Enclosure.Internal.0-1:RAID.Integrated.1-1

Status                           = Warning

Disk.Bay.3:Enclosure.Internal.0-1:RAID.Integrated.1-1

Status                           = Ok

...

#	And in details of hard drive we can see warning and row about predicted failure.

[root@m1-splunk-nidx01 ~]# racadm storage get pdisks:Disk.Bay.2:Enclosure.Internal.0-1:RAID.Integrated.1-1

Disk.Bay.2:Enclosure.Internal.0-1:RAID.Integrated.1-1

Status                           = Warning

DeviceDescription                = Disk 2 in Backplane 1 of Integrated RAID Controller 1

RollupStatus                     = Warning

Name                             = Physical Disk 0:1:2

State                            = Online

OperationState                   = Not Applicable

PowerStatus                      = Spun-Up

Size                             = 1117.25 GB

FailurePredicted                 = YES

RemainingRatedWriteEndurance     = Not Applicable

SecurityStatus                   = Not Capable

BusProtocol                      = SAS

MediaType                        = HDD

UsedRaidDiskSpace                = 1117.25 GB

AvailableRaidDiskSpace           = 1117.25 GB

Hotspare                         = NO

Manufacturer                     = HGST

ProductId                        = HUC101812CSS204

Revision                         = FJ23

SerialNumber                     = 0EGJY8KD

PartNumber                       = TH09XNF6HGT006AK11V4A01

NegotiatedSpeed                  = 12.0 Gb/s

ManufacturedDay                  = 5

ManufacturedWeek                 = 43

ManufacturedYear                 = 2016

SasAddress                       = 0x5000CCA0801ECF59

FormFactor                       = 2.5 Inch

RaidNominalMediumRotationRate    = 10000

T10PICapability                  = Not Capable

BlockSizeInBytes                 = 512

MaxCapableSpeed                  = 12 Gb/s

SelfEncryptingDriveCapability    = Not Capable

[root@m1-splunk-nidx01 ~]#

#	Get info about CPU.

[root@m1-pr-onlacq01 ~]# omreport chassis processors

Processors Information

Health : Ok

Index             : 0

Status            : Ok

Connector Name    : CPU1

Processor Brand   : Intel(R) Xeon(R) CPU E5-2630 v3 @ 2.40GHz

Processor Version : Model 63 Stepping 2

Current Speed     : 2400  MHz

State             : Present

Core Count        : 8

Index             : 1

Status            : Ok

Connector Name    : CPU2

Processor Brand   : Intel(R) Xeon(R) CPU E5-2630 v3 @ 2.40GHz

Processor Version : Model 63 Stepping 2

Current Speed     : 2400  MHz

State             : Present

Core Count        : 8

[root@m1-pr-onlacq01 ~]#

#	Get info about network cards.

[root@ds-pr-onlacq01 ~]# omreport chassis nics

Network Interfaces Information

Physical NIC Interface(s)

Index             : 0

Interface Name    : em1

Vendor            : Broadcom Corporation

Description       : NetXtreme BCM5720 Gigabit Ethernet PCIe

Connection Status : Connected

Slot              : Embedded

Index             : 1

Interface Name    : em2

Vendor            : Broadcom Corporation

Description       : NetXtreme BCM5720 Gigabit Ethernet PCIe

Connection Status : Connected

Slot              : Embedded

Index             : 2

Interface Name    : em3

Vendor            : Broadcom Corporation

Description       : NetXtreme BCM5720 Gigabit Ethernet PCIe

Connection Status : Disabled

Slot              : Embedded

Index             : 3

Interface Name    : em4

Vendor            : Broadcom Corporation

Description       : NetXtreme BCM5720 Gigabit Ethernet PCIe

Connection Status : Disabled

Slot              : Embedded

Team Interface(s)

Index             : 0

Interface Name    : bond0

Vendor            : Linux

Description       : Ethernet Channel Bonding

Redundancy Status : Full

[root@ds-pr-onlacq01 ~]#

[root@ds-pr-onlacq01 ~]# omreport chassis -?

chassis          Chassis component properties.

The available command(s) are:

Command          Description

pwrmonitoring    Power monitoring properties.

pwrmanagement    Power management properties.

hwperformance    Hardware performance information.

acswitch         AC switch settings.

bios             BIOS properties.

biossetup        BIOS setup configuration properties.

bmc              Remote Access information.

SUBCOMMAND DEPRECATION WARNING:

This subcommand is deprecated and will be discontinued in

a subsequent release.

Use the subcommand "remoteaccess" instead.

fans             Fan probe(s) properties.

fru              System Components (FRU) information.

leds             Chassis LED settings.

firmware         Firmware properties.

info             Chassis information.

intrusion        Chassis intrusion status.

memory           System memory configuration.

nics             Network interface card(s) and Team Interface(s) properties.

ports            Port(s) properties.

frontpanel       Front panel button control settings.

processors       Processor(s) properties.

pwrsupplies      Power supply(s) properties.

slots            Slot(s) properties.

temps            Temperature probe(s) properties.

volts            Voltage probe(s) properties.

remoteaccess     Remote Access information.

batteries        Battery(s) properties.

removableflashmedia        Removeable flash Media properties.

Usage: omreport <command...> [name=value...] ... [option...]

Valid command line options are:

-?               Print available command(s) or command help.

-fmt <lst|tbl|ssv|cdv|xml> Format for output results, default is lst.

Where: lst  List format.

tbl  Table format.

ssv  Semicolon separated format.

cdv  Custom delimiter separated format.

xml  Raw XML format.

-outc <file>     Redirect output to file, delete old if exists.

OR

-outa <file>     Redirect output to file, append to old if exists.

OR

Output may also be redirected using Operating System facilities (e.g. |more).

[root@ds-pr-onlacq01 ~]#

#

#	20200219

#	A way to get info about free PCI-E slots.

#	It helps to understand abilities to install additional NVME disks, for example.

#

[14:53:23 root@ds-gp-prod-sdw01 ~]# omreport storage controller controller=0 info=pdslotreport

Details of Enclosure BP14G_           on Controller PERC H730P Adapter

ID                   : 0:1

Slot Count           : 8

Occupied Slot Count  : 2

Empty Slot Count     : 6

Occupied Slots       : 0,1

Empty Slots          : 2,3,4,5,6,7

Occupied Physical Disk Details

ID                              : 0:1:0

Status                          : Ok

Name                            : Physical Disk 0:1:0

State                           : Online

Power Status                    : Not Applicable

Bus Protocol                    : SAS

Media                           : SSD

Part of Cache Pool              : Not Applicable

Remaining Rated Write Endurance : 100%

Failure Predicted               : No

Revision                        : DWL7

Driver Version                  : Not Applicable

Model Number                    : Not Applicable

T10 PI Capable                  : No

Certified                       : Yes

Encryption Capable              : No

Encrypted                       : Not Applicable

Progress                        : Not Applicable

Mirror Set ID                   : Not Applicable

Capacity                        : 744.63 GB (799535005696 bytes)

Used RAID Disk Space            : 744.63 GB (799535005696 bytes)

Available RAID Disk Space       : 0.00 GB (0 bytes)

Hot Spare                       : No

Vendor ID                       : DELL(tm)

Product ID                      : MZILS800HEHP0D3

Serial No.                      : S3FVNX0K701207

Part Number                     : CN0HF06WSSX0087C013KA00

Negotiated Speed                : 12.00 Gbps

Capable Speed                   : 12.00 Gbps

PCIe Negotiated Link Width      : Not Applicable

PCIe Maximum Link Width         : Not Applicable

Sector Size                     : 512B

Device Write Cache              : Not Applicable

Manufacture Day                 : Not Available

Manufacture Week                : 27

Manufacture Year                : 2018

SAS Address                     : 5002538A4876A1D2

Non-RAID HDD Disk Cache Policy  : Not Applicable

Disk Cache Policy               : Not Applicable

Sub Vendor                      : Not Available

Available Spare                 : Not Available

Cryptographic Erase Capable     : Yes

ID                              : 0:1:1

Status                          : Ok

Name                            : Physical Disk 0:1:1

State                           : Online

Power Status                    : Not Applicable

Bus Protocol                    : SAS

Media                           : SSD

Part of Cache Pool              : Not Applicable

Remaining Rated Write Endurance : 100%

Failure Predicted               : No

Revision                        : DWL7

Driver Version                  : Not Applicable

Model Number                    : Not Applicable

T10 PI Capable                  : No

Certified                       : Yes

Encryption Capable              : No

Encrypted                       : Not Applicable

Progress                        : Not Applicable

Mirror Set ID                   : Not Applicable

Capacity                        : 744.63 GB (799535005696 bytes)

Used RAID Disk Space            : 744.63 GB (799535005696 bytes)

Available RAID Disk Space       : 0.00 GB (0 bytes)

Hot Spare                       : No

Vendor ID                       : DELL(tm)

Product ID                      : MZILS800HEHP0D3

Serial No.                      : S3FVNX0K701223

Part Number                     : CN0HF06WSSX0087C0142A00

Negotiated Speed                : 12.00 Gbps

Capable Speed                   : 12.00 Gbps

PCIe Negotiated Link Width      : Not Applicable

PCIe Maximum Link Width         : Not Applicable

Sector Size                     : 512B

Device Write Cache              : Not Applicable

Manufacture Day                 : Not Available

Manufacture Week                : 27

Manufacture Year                : 2018

SAS Address                     : 5002538A4876A2D2

Non-RAID HDD Disk Cache Policy  : Not Applicable

Disk Cache Policy               : Not Applicable

Sub Vendor                      : Not Available

Available Spare                 : Not Available

Cryptographic Erase Capable     : Yes

[14:54:57 root@ds-gp-prod-sdw01 ~]#

20160406

#	Below is a experience of how to not to do.

#	Here I have created joqueue to apply some changes, but I have used one key, that imedietly start reboot server!

#

#   !!! After this option "-r pwrcycle" server starts REBOOT.

#   The server doesn't wait until it will be rebooted. It starts reboot imedietely.

#

racadm>>set bios.IntegratedDevices.UsbPorts AllOff

racadm set bios.IntegratedDevices.UsbPorts AllOff

[Key=BIOS.Setup.1-1#IntegratedDevices]

RAC1017: Successfully modified the object value and the change is in

pending state.

To apply modified value, create a configuration job and reboot

the system. To create the commit and reboot jobs, use "jobqueue"

command. For more information about the "jobqueue" command, see RACADM

help.

racadm>>

racadm>>get  bios.IntegratedDevices.UsbPorts

racadm get bios.IntegratedDevices.UsbPorts

[Key=BIOS.Setup.1-1#IntegratedDevices]

UsbPorts=AllOn (Pending Value=AllOff)

racadm>>

racadm>>jobqueue create BIOS.Setup.1-1 -r pwrcycle

racadm jobqueue create BIOS.Setup.1-1 -r pwrcycle

RAC1024: Successfully scheduled a job.

Verify the job status using "racadm jobqueue view -i JID_xxxxx" command.

Commit JID = JID_597847944208

Reboot JID = RID_597847946200

racadm>>

#   This job looks like follow.

racadm>>jobqueue view

...

----------------------------------------------------------

[Job ID=RID_597847946200]

Job Name=Reboot: Power cycle

Status=Reboot Completed

Start Time=[Now]

Expiration Time=[Not Applicable]

Message=[RED030: Reboot is complete.]

Percent Complete=[NA]

----------------------------------------------------------

racadm>>

#   After unexpected reboot. Required option is set.

#

racadm>>get  bios.IntegratedDevices.UsbPorts

racadm get bios.IntegratedDevices.UsbPorts

[Key=BIOS.Setup.1-1#IntegratedDevices]

UsbPorts=AllOff

racadm>>

#	This way should be used to create job.

#	After this, server is not rebooting. It’s waiting for when we will command to reboot.

racadm>>jobqueue create BIOS.Setup.1-1

racadm jobqueue create BIOS.Setup.1-1

RAC1024: Successfully scheduled a job.

Verify the job status using "racadm jobqueue view -i JID_xxxxx" command.

Commit JID = JID_597841280716

racadm>>

http://www.dell.com/support/manuals/us/en/19/poweredge-r730xd/iDRAC_RACADM_Pub/jobqueue?guid=GUID-F9C9CD97-B586-4FB9-95F5-B33FEA44C71D&lang=en-us

## jobqueue

| Description | Enables you to view and delete a job or jobs in the current Job Queue. - NOTE: - To run this subcommand, you must have the Server control privilege. - If an unexpected error message is displayed for any operation, ensure you delete some jobs in the jobqueue and retry the operation. |
| --- | --- |
| Synopsis | racadm jobqueue view -i<jobid> racadm jobqueue delete [-i<jobid>][--all] where valid options are -i and --all. racadm jobqueue create <fqdd> [-r <reboot type> ] [-s <start time> ] [-e <expiry time>] racadm jobqueue create <fqdd> [-r <reboot type>] [-s <start time>] [-e <expiration time>] [--realtime] |

| Input | - -i — Specifies a job ID that is displayed or deleted. - NOTE: The value JID_CLEARALL will force delete all the possible jobs in the queue. - --all — The job IDs that are not applicable are deleted. - -fqdd — Specifies an FQDD for which a job should be created. - —r <reboot type> — Specifies a reboot type. - none — No Reboot Job. This option is the default value. - pwrcycle — Power cycle. - graceful — Graceful Reboot without forced shut down. - forced — Graceful Reboot with forced shut down. - start time — Specifies a start time for job scheduled in the yyyymmddhhmmss format. TIME_NOW means immediate. Next Reboot means job is in scheduled state until the next manual restart. - expiry time — Specifies expiry time for the job execution in the yyyymmddhhmmss format. The job must start by this time. TIME_NA means expiry time is not applicable. - --realtime — Specifies the real time job. - NOTE: - --realtime is applicable for storage configuration commands run on systems with PERC 9 cards with firmware version 9.1 and later. - -r option is not valid for real time configuration. |
| --- | --- |

20160419

#	Command to reboot (reload) iDRAC.

[root@ds-hadoop-wk14p ~]# racadm racreset hard

RAC reset operation initiated successfully. It may take a few

minutes for the RAC to come online again.

[root@ds-hadoop-wk14p ~]#

http://www.gnali.org/?p=435

### iDRAC 7 CheatSheet

Filed under: Day to day sys administration — Tags: dell, idrac, racadm — Tin' @ 3:11 pm

# Get all iDRAC settings in a file

?

| 1 | racadm get -f config.txt |
| --- | --- |

# Set password for root user

?

| 1 | racadm set iDRAC.Users.2.Password "password" |
| --- | --- |

# List all ssh key for root user

?

| 1 | racadm sshpkauth -i 2 -v -k all |
| --- | --- |

# Add ssh key to root user

?

| 1 | racadm sshpkauth -i 2 -k 1 "contents of the public key file" |
| --- | --- |

# Delete ssh key for root user

?

| 1 | racadm sshpkauth -i 2 -d -k 1 |
| --- | --- |

# Get iDRAC IP config

?

| 1 2 | racadm getniccfg racadm get iDRAC.NIC |
| --- | --- |

# Set iDRAC DNS Name

?

| 1 | racadm set iDRAC.NIC.DNSRacName rougerie-rsc |
| --- | --- |

# Set iDRAC domain Name

?

| 1 | racadm set iDRAC.NIC.DNSDomainName gnali.org |
| --- | --- |

# Set iDRAC DNS server

?

| 1 2 | racadm config -g cfgLanNetworking -o cfgDNSServer1 191.254.31.175 racadm config -g cfgLanNetworking -o cfgDNSServer2 184.7.12.44 |
| --- | --- |

## Set FrontLCD to hostname

?

| 1 | racadm set System.LCD.Configuration 16 |
| --- | --- |

# Reset iDRAC to factory defaults

?

| 1 | racadm racresetcfg |
| --- | --- |

#Reset iDRAC

?

| 1 | racadm racreset |
| --- | --- |

# Get serial number (service tag)

?

| 1 | racadm getsvctag |
| --- | --- |

# Get current system information

?

| 1 | racadm getsysinfo |
| --- | --- |

# Configure One-Time-Boot

?

| 1 2 | racadm set BIOS.OneTimeBoot.OneTimeBootMode OneTimeBootSeq racadm set BIOS.OneTimeBoot.OneTimeBootSeqDev NIC.Integrated.1-1-1 |
| --- | --- |

# Create Commit and Reboot jobqueue to apply BIOS or NIC Configuration

?

| 1 | racadm jobqueue create BIOS.Setup.1-1 -r pwrcycle -s TIME_NOW -e TIME_NA |
| --- | --- |

# Create Commit jobqueue without Reboot

?

| 1 | racadm jobqueue create BIOS.Setup.1-1 |
| --- | --- |

# Delete a jobqueue

?

| 1 | racadm jobqueue delete -i &lt;job_ID&gt; |
| --- | --- |

# List all Jobqueue

?

| 1 | racadm jobqueue view |
| --- | --- |

# Check Boot Order List

?

| 1 | racadm get BIOS.BiosBootSettings.bootseq |
| --- | --- |

# Disable HyperThreadings

?

| 1 | racadm set BIOS.ProcSettings.LogicalProc Disabled |
| --- | --- |

# Disabled OS to iDRAC Pass-through for iDRAC Service Module (automatically create a pseudo NIC in OS)

?

| 1 | racadm set iDRAC.OS-BMC.AdminState Disabled |
| --- | --- |

# Change SNMP public community

?

| 1 | racadm set iDRAC.SNMP.AgentCommunity unixplop |
| --- | --- |

# Disable ASR

?

| 1 | racadm config -g cfgRacTuning -o cfgRacTuneAsrEnable 0 |
| --- | --- |

# Configure Serial redirection

?

| 1 2 3 4 5 | racadm config -g cfgSerial -o cfgSerialConsoleEnable 1 racadm config -g cfgSerial -o cfgSerialBaudRate 115200 racadm config -g cfgSerial -o cfgSerialCom2RedirEnable 1 racadm config -g cfgSerial -o cfgSerialTelnetEnable 0 racadm config -g cfgSerial -o cfgSerialSshEnable 1 |
| --- | --- |

# Access ssh console

?

| 1 2 | ssh root@rouerie-rsc console com2 |
| --- | --- |

# Disable Serial On Lan

?

| 1 | racadm config -g cfgIpmiSol -o cfgIpmiSolEnable 0 |
| --- | --- |

# Help on specific section

?

| 1 | racadm help BIOS.SysProfileSettings |
| --- | --- |

# Change Power Profile

?

| 1 | racadm set BIOS.SysProfileSettings PerfPerWattOptimizedOs |
| --- | --- |

# Set AC Power Recovery

?

| 1 2 | racadm set BIOS.SysSecurity.AcPwrRcvry Last racadm set BIOS.SysSecurity.AcPwrRcvryDelay Immediate |
| --- | --- |

# Get RAID physical Disk information

?

| 1 2 3 | racadm raid get pdisks racadm raid get pdisks -o (all information) racadm raid get pdisks -o -p state,size (specific information) |
| --- | --- |

# Get RAID Virtual Disk Information

?

| 1 | racadm raid get vdisks |
| --- | --- |

# Configuration Web Virtual console to use JAVA

http://virtual-grind.com/2012/10/09/restarting-drac-interface/

9OCT/120

I recently ran in to a few situations where the DRAC web interface was acting funny.

In one event, when I would log in to the DRAC web interface, despite having the correct username and password, the web interface would just bring me back to the main login page. The second scenario would let me in the main DRAC admin page, but I could not launch a remote console.

Anyway, there is a very simple fix. Simply ssh into your DRAC ip address with the same username and password you use to access the web interface. Once you ssh in, you will be at the main DRAC admin prompt. To reset DRAC and NOT lose any settings, simply enter the command:

racadm racreset sof

20160824

#	How to find mount point that used to hard drive or vice versa.

root@ds-gp-prd-sdw17 ~]# racadm storage get vdisks -o -p layout,status && racadm storage get controllers -o -p Status,DeviceDescription,Name

Disk.Virtual.0:RAID.Integrated.1-1

Layout                           = Raid-1

Status                           = Ok

Disk.Virtual.1:RAID.Integrated.1-1

Layout                           = Raid-10

Status                           = Ok

Disk.Virtual.0:RAID.Slot.3-1

Layout                           = Raid-10

Status                           = Ok

RAID.Integrated.1-1

Status                           = Ok

DeviceDescription                = Integrated RAID Controller 1

Name                             = PERC H730P Mini (Embedded)

RAID.Slot.3-1

Status                           = Ok

DeviceDescription                = RAID Controller in Slot 3

Name                             = PERC H730P Adapter (PCI Slot 3)

AHCI.Embedded.1-1

Status                           = Unknown

DeviceDescription                = Embedded AHCI 1

Name                             = C610/X99 series chipset sSATA Controller [AHCI mode] (Embedded)

AHCI.Embedded.2-1

Status                           = Unknown

DeviceDescription                = Embedded AHCI 2

Name                             = C610/X99 series chipset 6-Port SATA Controller [AHCI mode] (Embedded)

[root@ds-gp-prd-sdw17 ~]#

#

#   Now we know, "RAID.Slot.3-1" has "Name"="PERC H730P Adapter (PCI Slot 3)".

#

#   We remember, that problem disk has identification: Disk.Bay.19:Enclosure.Internal.0-1:RAID.Slot.3-1.

#   That means, that this disk is placed on "PERC H730P Adapter (PCI Slot 3)".

#

#   Then we define, that "PERC H730P Adapter (PCI Slot 3)" has block device "/dev/sdc".

#

[root@ds-gp-prd-sdw13 ~]# lsscsi -g

[0:2:0:0]    disk    DELL     PERC H730P Mini  4.26  /dev/sda   /dev/sg0

[0:2:1:0]    disk    DELL     PERC H730P Mini  4.26  /dev/sdb   /dev/sg1

[1:2:0:0]    disk    DELL     PERC H730P Adp   4.26  /dev/sdc   /dev/sg2

[root@ds-gp-prd-sdw13 ~]#

#   Here we can see, that the block device "/dev/sdc" mounted to "/data2" mount point.

#

[root@ds-gp-prd-sdw13 ~]# lsblk

NAME    MAJ:MIN RM   SIZE RO TYPE MOUNTPOINT

nvme0n1 259:0    0   1.5T  0 disk /ssd1

sda       8:0    0 558.4G  0 disk

├─sda1    8:1    0     1G  0 part /boot

├─sda2    8:2    0     6G  0 part [SWAP]

├─sda3    8:3    0 251.4G  0 part /

├─sda4    8:4    0     1K  0 part

└─sda5    8:5    0   300G  0 part /duet

sdc       8:32   0   2.2T  0 disk

└─sdc1    8:33   0   2.2T  0 part /data2

sdb       8:16   0   2.2T  0 disk

└─sdb1    8:17   0   2.2T  0 part /data1

[root@ds-gp-prd-sdw13 ~]#

#   Now we know, that disk "Disk.Bay.19:Enclosure.Internal.0-1:RAID.Slot.3-1" belong to mount point "/data2".

#	How to get details about defined raid controller.

[root@ds-gp-prd-sdw13 ~]# racadm storage get controllers:RAID.Integrated.1-1 -p Status,DeviceDescription,Name,SlotType

RAID.Integrated.1-1

Status                           = Ok

DeviceDescription                = Integrated RAID Controller 1

Name                             = PERC H730P Mini (Embedded)

SlotType                         = Unknown

[root@ds-gp-prd-sdw13 ~]#

#	For PCI-E cards we can get info, where it placed.

[root@ds-gp-prd-sdw13 ~]# racadm storage get controllers:RAID.Slot.3-1 -p Status,DeviceDescription,Name,PciSlot,SlotType

RAID.Slot.3-1

Status                           = Ok

DeviceDescription                = RAID Controller in Slot 3

Name                             = PERC H730P Adapter (PCI Slot 3)

PciSlot                          = 3

SlotType                         = PCI Express Gen3 x16

[root@ds-gp-prd-sdw13 ~]#

k

[root@ds-gp-prd-sdw13 ~]# racadm storage get controllers -o

...

RAID.Slot.3-1

Status                           = Ok

DeviceDescription                = RAID Controller in Slot 3

RollupStatus                     = Ok

Name                             = PERC H730P Adapter (PCI Slot 3)

PciSlot                          = 3

FirmwareVersion                  = 25.4.0.0017

DriverVersion                    = --NA--

RebuildRate                      = 30

BgiRate                          = 30

CheckConsistencyRate             = 30

ReconstructRate                  = 30

PatrolReadRate                   = 30

PatrolReadMode                   = Automatic

PatrolReadState                  = Stopped

CheckConsistencyMode             = Normal

LoadBalanceSetting               = Auto

CopybackMode                     = ON

PreservedCache                   = Not Present

CacheMemorySize                  = 2048 MB

PersistHotspare                  = Disabled

SpindownUnconfiguredDrives       = Disabled

SpindownHotspare                 = Disabled

Timeintervalforspindown          = 30 (Minutes)

SecurityStatus                   = Encryption Capable

EncryptionMode                   = None

SasAddress                       = 0x544A842031B8D200

PciDeviceId                      = 0x5d

PciSubdeviceId                   = 0x1f42

PciVendorId                      = 0x1000

PciSubvendorId                   = 0x1028

PciBus                           = 0x32

PciDevice                        = 0x0

PciFunction                      = 0x0

BusWidth                         = 8x or x8

SlotLength                       = Long Length

SlotType                         = PCI Express Gen3 x16

MaxCapableSpeed                  = 12.0 Gb/s

LearnMode                        = Not supported

T10PIC   apability                  = Capable

SupportRAID10UnevenSpans         = Supported

SupportEnhancedAutoForeignImport = Supported

EnhancedAutoImportForeignConfig  = Disabled

SupportControllerBootMode        = Supported

ControllerBootMode               = Continue Boot On Error

RealtimeConfigurationCapability  = Capable

CurrentControllerMode            = RAID

#   Here is how to see some details about disk

[root@ds-gp-prd-sdw13 ~]# racadm storage get pdisks:Disk.Bay.19:Enclosure.Internal.0-1:RAID.Slot.3-1

[root@ds-gp-prd-sdw13 ~]# lspci -v | less

...

02:00.0 RAID bus controller: LSI Logic / Symbios Logic MegaRAID SAS-3 3108 [Invader] (rev 02)

Subsystem: Dell PERC H730P Mini

Flags: bus master, fast devsel, latency 0, IRQ 26

I/O ports at 2000 [size=256]

Memory at 96300000 (64-bit, non-prefetchable) [size=64K]

Memory at 96200000 (64-bit, non-prefetchable) [size=1M]

Expansion ROM at 91a00000 [disabled] [size=1M]

Capabilities: [50] Power Management version 3

Capabilities: [68] Express Endpoint, MSI 00

Capabilities: [d0] Vital Product Data

Capabilities: [a8] MSI: Enable- Count=1/1 Maskable+ 64bit+

Capabilities: [c0] MSI-X: Enable+ Count=97 Masked-

Capabilities: [100] Advanced Error Reporting

Capabilities: [1e0] #19

Capabilities: [1c0] Power Budgeting <?>

Capabilities: [148] Alternative Routing-ID Interpretation (ARI)

Kernel driver in use: megaraid_sas

Kernel modules: megaraid_sas

...

84:00.0 RAID bus controller: LSI Logic / Symbios Logic MegaRAID SAS-3 3108 [Invader] (rev 02)

Subsystem: Dell PERC H730P Adapter

Flags: bus master, fast devsel, latency 0, IRQ 64

I/O ports at 8000 [size=256]

Memory at c8100000 (64-bit, non-prefetchable) [size=64K]

Memory at c8000000 (64-bit, non-prefetchable) [size=1M]

Expansion ROM at c8300000 [disabled] [size=1M]

Capabilities: [50] Power Management version 3

Capabilities: [68] Express Endpoint, MSI 00

Capabilities: [d0] Vital Product Data

Capabilities: [a8] MSI: Enable- Count=1/1 Maskable+ 64bit+

Capabilities: [c0] MSI-X: Enable+ Count=97 Masked-

Capabilities: [100] Advanced Error Reporting

Capabilities: [1e0] #19

Capabilities: [1c0] Power Budgeting <?>

Capabilities: [148] Alternative Routing-ID Interpretation (ARI)

Kernel driver in use: megaraid_sas

Kernel modules: megaraid_sas

#

#	20171102

#

#	Some commands to get info about drives.

[root@ds-hadoop-wk10p ~]# omreport storage vdisk

List of Virtual Disks in the System

Controller PERC H730P Mini (Embedded)

ID                                : 0

Status                            : Ok

Name                              : Virtual Disk 0

State                             : Ready

Hot Spare Policy violated         : Not Assigned

Encrypted                         : Not Applicable

Layout                            : RAID-1

Size                              : 465.25 GB (499558383616 bytes)

T10 Protection Information Status : No

Associated Fluid Cache State      : Not Applicable

Device Name                       : /dev/sdm

Bus Protocol                      : SAS

Media                             : HDD

Read Policy                       : Read Ahead

Write Policy                      : Write Back

Cache Policy                      : Not Applicable

Stripe Element Size               : 64 KB

Disk Cache Policy                 : Unchanged

[root@ds-hadoop-wk10p ~]#

[root@ds-hadoop-wk10p ~]# omreport storage pdisk controller=0

List of Physical Disks on Controller PERC H730P Mini (Embedded)

Controller PERC H730P Mini (Embedded)

ID                              : 0:1:0

Status                          : Ok

Name                            : Physical Disk 0:1:0

State                           : Non-RAID

Power Status                    : Spun Up

Bus Protocol                    : SAS

Media                           : HDD

Part of Cache Pool              : Not Applicable

Remaining Rated Write Endurance : Not Applicable

Failure Predicted               : No

Revision                        : GS11

Driver Version                  : Not Applicable

Model Number                    : Not Applicable

T10 PI Capable                  : No

Certified                       : Yes

Encryption Capable              : No

Encrypted                       : Not Applicable

Progress                        : Not Applicable

Mirror Set ID                   : Not Applicable

Capacity                        : 3,725.50 GB (4000225165312 bytes)

Used RAID Disk Space            : 0.00 GB (0 bytes)

Available RAID Disk Space       : 3,725.50 GB (4000225165312 bytes)

Hot Spare                       : No

Vendor ID                       : DELL(tm)

Product ID                      : ST4000NM0023

Serial No.                      : Z1Z5QE85

Part Number                     : TH0529FG2123349B06EKA02

Negotiated Speed                : 6.00 Gbps

Capable Speed                   : 6.00 Gbps

PCIe Negotiated Link Width      : Not Applicable

PCIe Maximum Link Width         : Not Applicable

Sector Size                     : 512B

Device Write Cache              : Not Applicable

Manufacture Day                 : 07

Manufacture Week                : 37

Manufacture Year                : 2014

SAS Address                     : 5000C500626E25DD

Form Factor                     : Not Available

Sub Vendor                      : Not Available

…

ID                              : 0:1:13

Status                          : Non-Critical

Name                            : Physical Disk 0:1:13

State                           : Online

Power Status                    : Spun Up

Bus Protocol                    : SAS

Media                           : HDD

Part of Cache Pool              : Not Applicable

Remaining Rated Write Endurance : Not Applicable

Failure Predicted               : Yes

Revision                        : AS0B

Driver Version                  : Not Applicable

Model Number                    : Not Applicable

T10 PI Capable                  : No

Certified                       : Yes

Encryption Capable              : No

Encrypted                       : Not Applicable

Progress                        : Not Applicable

Mirror Set ID                   : Not Applicable

Capacity                        : 465.25 GB (499558383616 bytes)

Used RAID Disk Space            : 465.25 GB (499558383616 bytes)

Available RAID Disk Space       : 0.00 GB (0 bytes)

Hot Spare                       : No

Vendor ID                       : DELL(tm)

Product ID                      : ST9500620SS

Serial No.                      : 9XF3KW1L

Part Number                     : TH055RMX2123354600Q4A08

Negotiated Speed                : 6.00 Gbps

Capable Speed                   : 6.00 Gbps

PCIe Negotiated Link Width      : Not Applicable

PCIe Maximum Link Width         : Not Applicable

Sector Size                     : 512B

Device Write Cache              : Not Applicable

Manufacture Day                 : 04

Manufacture Week                : 15

Manufacture Year                : 2015

SAS Address                     : 5000C5008378DA15

Form Factor                     : Not Available

Sub Vendor                      : Not Available

[root@ds-hadoop-wk10p ~]#

#	One drive above is in Predictive failure state, by the way.

20160824

#	How to highlight drives at server and server too.

[root@ds-gp-prd-sdw17 ~]# at 18:00 today <<< 'racadm storage blink:Disk.Bay.2:Enclosure.Internal.0-1:RAID.Integrated.1-1'

job 1 at 2016-08-23 18:00

[root@ds-gp-prd-sdw17 ~]# at 18:00 today <<< 'racadm storage blink:Disk.Bay.15:Enclosure.Internal.0-1:RAID.Slot.3-1'

job 2 at 2016-08-23 18:00

[root@ds-gp-prd-sdw17 ~]# logout

[root@ds-gp-prd-sdw13 ~]# at 18:00 today <<< 'racadm storage blink:Disk.Bay.19:Enclosure.Internal.0-1:RAID.Slot.3-1'

job 1 at 2016-08-23 18:00

[root@ds-gp-prd-sdw13 ~]#

[root@ds-gp-prd-sdw13 ~]# at 18:00 today <<< 'racadm setled -l 1'

job 2 at 2016-08-23 18:00

[root@ds-gp-prd-sdw13 ~]# logout

Connection to sdw13 closed.

[root@ds-gp-prd-smdw ~]# ssh sdw17

Last login: Tue Aug 23 17:00:13 2016 from mdw

[root@ds-gp-prd-sdw17 ~]# at 18:00 today <<< 'racadm setled -l 1'

job 3 at 2016-08-23 18:00

[root@ds-gp-prd-sdw17 ~]#

20160824

#	This is attempt to do some check on drives or whole raid controller.

From: Vladimir_Markus@dell.com [mailto:Vladimir_Markus@dell.com]

Sent: Friday, August 19, 2016 5:55 PM

To: Belogrud Sergey Vladimirovich

Cc: Piotr_Kretkiewicz@Dell.com

Subject: RE: Dell Support Case: 932140477 [R730xd] HDDs performance down

Dell - Internal Use - Confidential

Добрый день, Сергей,

Если мне не изменяет память, то тест занимает несколько часов и проводится как background operation.

...

Документ, по которому не работала ссылка прилагаю, но можете запустить чек через команду racadm.

Mit freundlichen Grüßen / Best Regards / С уважением

Vladimir Markus

Enterprise Resolution Manager

From: Vladimir_Markus@dell.com [mailto:Vladimir_Markus@dell.com]

Sent: Friday, August 19, 2016 1:18 PM

To: Belogrud Sergey Vladimirovich

Cc: Leonov Aleksey Vladimirovich; Piotr_Kretkiewicz@Dell.com

Subject: RE: Dell Support Case: 932140477 [R730xd] HDDs performance down

...

Consistency Check можно запустить через OMSA в WebGUI, racadm с помощию команды- racadm storage ccheck:<vdisk fqdd> или PERC CLI (инструкция на странице 42 в мануале - https://topics-cdn.dell.com/pdf/poweredge-rc-h730_Reference%20Guide_en-us.pdf )

Mit freundlichen Grüßen / Best Regards / С уважением

Vladimir Markus

Enterprise Resolution Manager

From: Vladimir_Markus@dell.com [mailto:Vladimir_Markus@dell.com]

Sent: Friday, August 19, 2016 12:13 PM

To: Leonov Aleksey Vladimirovich; Piotr_Kretkiewicz@Dell.com

Cc: Belogrud Sergey Vladimirovich

Subject: RE: Dell Support Case: 932140477 [R730xd] HDDs performance down

...

Заказы мы оформили, в понедельник диски должны быть у Вас. Наши коллеги L3 попросили еще запустить Consistency Check до и после замены дисков.

Mit freundlichen Grüßen / Best Regards / С уважением

Vladimir Markus

Enterprise Resolution Manager

[root@ds-gp-prd-sdw13 ~]# racadm raid ccheck:Disk.Virtual.0:RAID.Slot.3-1

STOR094 : The storage configuration operation is successfully completed

and the change is in pending state.

To apply the configuration operation immediately, create a configuration job

using the --realtime option.

To apply the configuration after restarting

the server, create a configuration job using the -r option.

To create the necessary real-time and restart jobs, run the jobqueue command.

For more information about jobqueue command, run the

'racadm help jobqueue' command.

[root@ds-gp-prd-sdw13 ~]#

20161003

#	Change default root password for iDRAC.

#	This command line allow insert ip-address (10.219.11.101) or fqdn 488v762.mgmt.tcsbank.ru) of idrac interface.

[support@centos00 20161003-1206]$ while read -p "Enter iDRAC FQDN: " var1_servicetag ; do var2_username="$( SSHPASS="$(pass common/windows-current)" sshpass -e ssh -t -oStrictHostKeyChecking=no tcsbank.ru\\s.belogrud@${var1_servicetag} racadm get idrac.users.2.username | grep UserName | cut -d "=" -f2 | tr -d '\r' )" ; if [ "${var2_username}" = "root" ] ; then echo -e "Server: ${var1_servicetag}\nUser: ${var2_username}" ; var3_newpassword="$(pwgen -c -n -s -C 10 1)" ; echo "Password: ${var3_newpassword}" ; SSHPASS="$(pass common/windows-current)" sshpass -e ssh -t -oStrictHostKeyChecking=no tcsbank.ru\\s.belogrud@${var1_servicetag} racadm set idrac.users.2.password "${var3_newpassword}" | tee -a ./$(date +%Y%m%d-%H%M).racadm.set.${var1_servicetag}.txt ; else echo "No, we mightn't change pasword for user: ${var2_username}." ; fi ; done

Enter iDRAC FQDN: 10.219.11.101

Warning: Permanently added '10.219.11.101' (RSA) to the list of known hosts.

Connection to 10.219.11.101 closed.

Server: 10.219.11.101

User: root

Password: h9NOr8ZS7r

[Key=idrac.Embedded.1#Users.2]

Object value modified successfully

Connection to 10.219.11.101 closed.

Enter iDRAC FQDN: 10.219.10.223

Warning: Permanently added '10.219.10.223' (RSA) to the list of known hosts.

Connection to 10.219.10.223 closed.

Server: 10.219.10.223

User: root

Password: KixNk9EoRa

[Key=idrac.Embedded.1#Users.2]

Object value modified successfully

Connection to 10.219.10.223 closed.

Enter iDRAC FQDN: ^C

[support@centos00 20161003-1206]$

#	Here we can insert only Service tag.

[support@centos00 20161003-1206]$ while read -p "Enter service tag: " var1_servicetag ; do var2_username="$( SSHPASS="$(pass common/windows-current)" sshpass -e ssh -t -oStrictHostKeyChecking=no tcsbank.ru\\s.belogrud@${var1_servicetag}.mgmt.tcsbank.ru racadm get idrac.users.2.username | grep UserName | cut -d "=" -f2 | tr -d '\r' )" ; if [ "${var2_username}" = "root" ] ; then echo -e "Service tag: ${var1_servicetag}\nServer: ${var1_servicetag}.mgmt.tcsbank.ru\nUser: ${var2_username}" ; var3_newpassword="$(pwgen -c -n -s -C 10 1)" ; echo "Password: ${var3_newpassword}" ; SSHPASS="$(pass common/windows-current)" sshpass -e ssh -t -oStrictHostKeyChecking=no tcsbank.ru\\s.belogrud@${var1_servicetag}.mgmt.tcsbank.ru racadm set idrac.users.2.password "${var3_newpassword}" | tee -a ./$(date +%Y%m%d-%H%M).racadm.set.${var1_servicetag}.txt ; else echo "No, we mightn't change pasword for user: ${var2_username}." ; fi ; done

Enter service tag: 5VJL8D2

Warning: Permanently added '5vjl8d2.mgmt.tcsbank.ru,10.219.9.109' (RSA) to the list of known hosts.

Connection to 5VJL8D2.mgmt.tcsbank.ru closed.

Service tag: 5VJL8D2

Server: 5VJL8D2.mgmt.tcsbank.ru

User: root

Password: KB8Qawnwtm

[Key=idrac.Embedded.1#Users.2]

Object value modified successfully

Connection to 5VJL8D2.mgmt.tcsbank.ru closed.

...

From: Leonov Aleksey Vladimirovich

Sent: Friday, January 13, 2017 11:06 AM

To: Belogrud Sergey Vladimirovich

Subject: RE: dell srvadmin sudden segfault

на m1-arc-manager01

в нагиос выглядело как отвал Server Health по snmp.

Алексей Леонов

Администратор серверных комплексов

Тел.: +7 495 648-10-00, доб. 2536

From: Belogrud Sergey Vladimirovich

Sent: Friday, January 13, 2017 10:55 AM

To: Leonov Aleksey Vladimirovich

Subject: RE: dell srvadmin sudden segfault

Добрый день.

А это на где так стало?

И как оно себя предъявляло в виде ошибок? От nagios что-то сыпалось?

Сергей Белогруд

Старший системный администратор

Тел.: +7 495 648-10-00, доб. 2173

From: Leonov Aleksey Vladimirovich

Sent: Friday, January 13, 2017 9:27 AM

To: unix-admins

Subject: dell srvadmin sudden segfault

Доброго времени суток, Коллеги.

как говорится, я просто оставлю это здесь:

симптом:

OMSA, всех мастей, внезапно начинает сегфолтится.

вся проблема в семафорах. их кончилось. надо увеличить:

sysctl -w kernel.sem="250 32000 100 1184"

и в /etc/sysctl.conf

kernel.sem=250 32000 100 1184

ну и рестарт служб.

все сразу начнет работать

С уважением,

Алексей Леонов

Администратор серверных комплексов

From: Piotr.Kretkiewicz@dell.com [mailto:Piotr.Kretkiewicz@dell.com]

Sent: Wednesday, February 01, 2017 12:01 PM

To: Leonov Aleksey Vladimirovich; Belogrud Sergey Vladimirovich

Subject: RE: Dell Support Case: 942816195 [R730xd]

Добрый день,

Конечно Вам заменим, но если я бы сразу получил логи из контроллера жестких дисков, может я бы раньше нашел информацию о сбое этого диска.

У меня вопрос: потому что не очень есть отправлять запчасти два раза за одну неделю на данном сервис таге, могу я Вам выписать вызов в пятницу?

Это помогло бы избежать сложностей

Кроме того, высылаю Вам ссылку на утилиту которая собирает только логи контроллера RAID. Может в случаях когда вылетает диск, Вам будет проще чем собирать TSR

Extracting the RAID-Controller Logs via MegaCLI (Windows + Linux)

Прямая ссылка на скачивание megacli:

http://www.avagotech.com/docs-and-downloads/raid-controllers/raid-controllers-common-files/8-07-14_MegaCLI.zip

To export information about the status of the RAID controller and the connected hard drives, use the MegaCLI tool by LSI.

Note: This article is aimed at Dell PowerEdge servers with the following operating systems:

·         Windows (Windows Server 2003/2008/2012)

·         Linux (SLES, RHEL, XenServer, other distributions)

To create the appropriate diagnostic files, perform the following steps:

1.    Download the appropriate version of the MegaCli tool for Windows Operating systems from the Download section of the LSI support site.

For the most popular Linux operating systems, you also need the following file to ensure a smooth execution: - Lib_Utils-1.00-09.noarch.rpm

2.    Unzip the downloaded files and copy them in any directory on the server or host system according your installed operating system (for example: Windows – C:\temp\megacli, Linux - /tmp/megacli). Verify whether the above mentioned libraries are installed in your Linux operating system.

3.    Open a terminal in Linux or a command prompt in Windows operating systems and navigate to the newly created directory.

4.    Now run the following commands:

o	For Windows:

1.    To create the RAID controller log: MegaCli.exe -FwTermLog -Dsply –aALL > ttylog.txt

2.    To create the EventLog since commissioning of the controller: MegaCli.exe -AdpEventLog -IncludeDeleted -f deleted.txt -aALL

Note: the deleted.txt is always stored in the MegaCli folder. Unfortunately, this can not be changed.

o	For Linux:

1.    To create the RAID controller log: ./MegaCli64 -FwTermLog -Dsply –aALL > /tmp/ttylog.txt

2.    To create the EventLog since commissioning of the controller: ./MegaCli64 -AdpEventLog -IncludeDeleted -f deleted.txt -aALL

Note: the deleted.txt is always stored in the MegaCli folder. Unfortunately, this can not be changed.

С уважением,

Пётр Креткевич

Enterprise Engineer

Dell EMC | Remote Services

piotr.kretkiewicz@dell.com

Dell Halle GmbH in Vertretung der Dell GmbH. Raffineriestraße 28, 06112 Halle (Saale), Germany

Geschäftsführer: Doris Albiez, Sanjay Tyagi

Vorsitzender des Aufsichtsrates: Jürgen Renz

Eingetragen beim Amtsgericht Stendal unter HRB 215298, USt.-ID: DE242553389

WEEE-Reg.Nr. der Dell GmbH: DE 49515708

From: Piotr.Kretkiewicz@dell.com [mailto:Piotr.Kretkiewicz@dell.com]

Sent: Wednesday, February 01, 2017 2:45 PM

To: Leonov Aleksey Vladimirovich; Belogrud Sergey Vladimirovich

Subject: RE: Dell Support Case: 942816195 [R730xd]

Извините, выслал Вам утилиту для старших серверов. Для 13 генерации (R630, R730, R830, R930) есть другая утилита – PERCCLI

http://www.dell.com/support/article/us/en/19/SLN295890/en#DI

## 13G PowerEdge Servers – RAID controller log export using PercCLI tool on Windows and Linux hosts

Article summary: This article discusses how to export the RAID controller log using the PercCLI tool on Windows and Linux hosts.

### Issue:

This article provides detailed information about the export of RAID controller logs via the PERCCLI tool. The tool is designed for Dell PowerEdge servers of the 13th generation with Windows, Linux or ESXi installed. The logs contain data, which alllow the Dell Tech Support to determine the status of RAID controllers and the connected disk devices to narrow down possible sources of error. ​

### Table of Contents:

- Advance Information

- Directions

- Download and Installation

- Video: Using PERCCLI in Windows

#### Advance Information

To export information about the status of the RAID controller and its attached hard drives, you can use the PERCCLI tool. Below you will find all the necessary steps to create the appropriate diagnostic files.

The tool only captures information about the hardware status and operations, but not about about any data written on the devices. Furthermore it can be used without a restart of the server and it delivers the results instantly.

#### Directions

Supported OS

- Windows (Windows Server 2003/2008/2012)

- Linux (SLES, RHEL, XenServer, other distributions)

- ESXi (5.x and above)

Supported Controllers:

- PERC H330

- PERC H730

- PERC H730P

- PERC H830

#### Download and Installation

- Download the PERCCLI tool in the latest version from the Dell support website via the following link PERCCLI

http://www.dell.com/support/home/us/en/04/Drivers/DriversDetails?driverId=3XDPP

- Unzip the downloaded file and copy the contents to a directory on the server or host system.

Example: Windows - C:\temp\perccli or Linux - /tmp/perccli 1. Installation on Windows: § on Windows systems, no additional installation is required 2. Installation on Linux: § on Linux systems the installation of < perccli-x.x.x.x.noarch.rpm > RPM is required, which is part of the perccli-1.x.x.x.x.tar.gz in the perccli-1.x.x.x.x.tar file. § The following steps are required for installation: 1. To install, run the following command: rpm -ivh < perccli-x.x.x.x.noarch.rpm > 2. to upgrade the PERCCLI RPM, execute the following command: rpm -Uvh < perccli-x.x.x.x.noarch.rpm >

Attention: PERCCLI must be run with administrator privileges!

- Open a terminal in Linux or a command prompt in Windows operating systems and navigate to the folder created before.

Info: Under Linux PERCCLI is installed by default in the following directory: /opt/MegaRAID/perccli

- Now run the following commands sequentially:

/cx = Controller | x=ID (usually 0)

Windows

| Command | Description |
| --- | --- |
| perccli /cx show termlog > termlog.txt | Creates the RAID controller log (ttylog). |
| perccli /cx /eall /sall show all > disks.txt | Creates a list with information about the RAID controllers, virtual disks and hard disks installed. |
| perccli /cx/bbu show all > battery.txt | Creates a detailed list of the battery status of the RAID controller (state of charge, learning cycle, etc.) |

Linux

| Command | Description |
| --- | --- |
| ./perccli /cx show termlog > termlog.txt | Creates the RAID controller log (ttylog). |
| ./perccli /cx /eall /sall show all > disks.txt | Creates a list with information about the RAID controllers, virtual disks and hard disks installed. |
| ./perccli /cx/bbu show all > battery.txt | Creates a detailed list of the battery status of the RAID controller (state of charge, learning cycle, etc.) |

A list of all other commands can be found in the reference guide.

- Create a ZIP archive out of the text files and send it to Dell Support for further diagnosis.

С уважением,

Пётр Креткевич

Enterprise Engineer

From: Leonov Aleksey Vladimirovich

Sent: Monday, February 06, 2017 4:27 PM

To: Belogrud Sergey Vladimirovich

Subject: FW: Dell Support - 6ZQG8D2 / 942816195

хорошая утилитка. рекомендую.

Алексей Леонов

Администратор серверных комплексов

Тел.: +7 495 648-10-00, доб. 2536

From: Piotr.Kretkiewicz@dell.com [mailto:Piotr.Kretkiewicz@dell.com]

Sent: Monday, February 06, 2017 2:52 PM

To: Leonov Aleksey Vladimirovich

Subject: RE: Dell Support - 6ZQG8D2 / 942816195

Добрый день Алексей,

Коллега мне передал информации что еще один диск вылетел на сервере.

Я проверил все логи и на одном из контроллеров (конкретно на этом, который стоит в слоте нр. 3) повторяются эти самые ошибки. Их есть там очень много, что контроллер удаляет ее и данные в них доступные только из  4 Февраля.

Блогадорю утилиты PercCLI есть возможность собрать все логи, даже удаленные, по этому у меня вопрос:

Вы можете собрать эти логи и прислать мне по электронной почте? Мы просто бы хотели одновременно, полностью решить проблему и эти данные бы нам очень помогли

http://www.dell.com/support/home/us/en/04/Drivers/DriversDetails?driverId=3XDPP

- Unzip the downloaded file and copy the contents to a directory on the server or host system.

Example: Windows - C:\temp\perccli or Linux - /tmp/perccli 1. Installation on Windows: § on Windows systems, no additional installation is required 2. Installation on Linux: § on Linux systems the installation of < perccli-x.x.x.x.noarch.rpm > RPM is required, which is part of the perccli-1.x.x.x.x.tar.gz in the perccli-1.x.x.x.x.tar file. § The following steps are required for installation: 1. To install, run the following command: rpm -ivh < perccli-x.x.x.x.noarch.rpm > 2. to upgrade the PERCCLI RPM, execute the following command: rpm -Uvh < perccli-x.x.x.x.noarch.rpm >

Attention: PERCCLI must be run with administrator privileges!

- Open a terminal in Linux or a command prompt in Windows operating systems and navigate to the folder created before.

Info: Under Linux PERCCLI is installed by default in the following directory: /opt/MegaRAID/perccli

- Now run the following commands sequentially:

perccli -AdpEventLog -IncludeDeleted -f deleted.txt -aALL

С уважением,

Пётр Креткевич

Enterprise Engineer

#	20170803

#	Aleksandrov has changed DNS sets via this method.

From: Aleksandrov Aleksey Valerievich

Sent: Thursday, August 3, 2017 4:44 PM

To: unix-admins <unix-admins@tinkoff.ru>

Subject: Настройка iDRAC через racadm и XML

Возможно кому-то будет полезно.

Настройка iDRAC через racadm и XML

3 августа 2017 г.

16:05

Для начала получаем конфигурацию iDRAC в XML формате

racadm.exe -r 10.219.110.89 -u root -p calvin get -f .\Desktop\74WXMK2.xml -t xml

Далее полученный файл нужно отредактировать и залить обратно в iDARC

racadm.exe -r 10.219.110.89 -u root -p calvin set -f .\Desktop\74WXMK2.xml -t xml

…

Please wait while racadm transfers the file.

File transferred successfully. Initiating the import operation.

RAC977: Import configuration XML file operation initiated.

Use the "racadm jobqueue view -i JID_017835160128" command to view the status

of the operation.

Смотрим статус Job

racadm.exe -r 10.219.110.89 -u root -p calvin jobqueue view -i JID_017835160128

…

---------------------------- JOB -------------------------

[Job ID=JID_017835160128]

Job Name=Configure: Import system configuration XML file

Status=Completed

Start Time=[Not Applicable]

Expiration Time=[Not Applicable]

Message=[SYS053: Successfully imported and applied system configuration XML file.]

Percent Complete=[100]

----------------------------------------------------------

#

#	I’ve downloaded distributive.

http://www.dell.com/support/home/us/en/19/Drivers/DriversDetails?driverId=CNX3N

#	20170803

#	Using DSU utility to Firmware update.

#

[root@ds-pr-onltraces01p ~]# curl http://repo-linux/tcs-dell-dsu-rhel7.repo.sh | sh

% Total    % Received % Xferd  Average Speed   Time    Time     Time  Current

Dload  Upload   Total   Spent    Left  Speed

100   342  100   342    0     0  15695      0 --:--:-- --:--:-- --:--:-- 16285

[root@ds-pr-onltraces01p ~]#

[root@ds-pr-onltraces01p ~]# yum install dsucatalog

[root@ds-pr-onltraces01p ~]# yum install dell-system-update

[root@ds-gp-prd-smdw ~]# for var1_host in ds-gp-prd-sdw14 ; do ssh -t ${var1_host} '( echo ; hostname ; dsu )' ; done

ds-gp-prd-sdw14

Dell System Update 1.4.2

Copyright (C) 2014 Dell Proprietary.

Verifying catalog installation ...

Installing catalog from repository ...

Fetching dsucatalog ...

Reading the catalog ...

Fetching invcol_M18HC_LN64_17.06.000.1171_A00 ...

Verifying inventory collector installation ...

Getting System Inventory ...

Determining Applicable Updates ...

|-----------Dell System Updates-----------|

[ ] represents 'not selected'

[*] represents 'selected'

[-] represents 'Component already at repository version (can be selected only if -e option is used)'

Choose:  q - Quit without update, c to Commit, <number> - To Select/Deselect, a - Select All, n - Select None

[ ]1  iDRAC

Current Version : 2.30.30.30 Upgrade to : 2.41.40.40

[ ]2 BCM57800 1-Gigabit Ethernet rev 10 (em4)

Current Version : 08.07.25 Upgrade to : 10.00.15

[ ]3 BCM57800 1-Gigabit Ethernet rev 10 (em3)

Current Version : 08.07.25 Upgrade to : 10.00.15

[ ]4 BCM57800 10-Gigabit Ethernet rev 10 (em2)

Current Version : 08.07.25 Upgrade to : 10.00.15

[ ]5 BCM57800 10-Gigabit Ethernet rev 10 (em1)

Current Version : 08.07.25 Upgrade to : 10.00.15

[ ]6 OpenManage | iDRAC Service Module

Current Version : 0 Upgrade to : 2.5

[ ]7 BIOS

Current Version : 2.1.6 Upgrade to : 2.4.3

[-]8 Firmware for  - Disk 0 in Backplane 1 of PERC H730P Mini Controller 0

Current Version : DE11 same as : DE11

[ ]9 Dell 32 Bit uEFI Diagnostics, version 4239, 4239A33, 4239.41

Current Version : 4239A33 Upgrade to : 4239A36

[ ]10 OS COLLECTOR 1.1, OSC_1.1, A00

Current Version : OSC_1.1 Downgrade to : 2.1

[ ]11 Firmware for  - Disk 20 in Backplane 1 of PERC H730P Adapter Controller 1

Current Version : DL2B Upgrade to : DL2D

[ ]12 PERC H730P Adapter Controller 1 Firmware

Current Version : 25.4.0.0017 Upgrade to : 25.5.2.0001

[ ]13 PERC H730P Mini Controller 0 Firmware

Current Version : 25.4.0.0017 Upgrade to : 25.5.2.0001

[ ]14 13G Backplane Expander FW

Current Version : 3.03 Upgrade to : 3.32

Enter your choice : a

|-----------Dell System Updates-----------|

[ ] represents 'not selected'

[*] represents 'selected'

[-] represents 'Component already at repository version (can be selected only if -e option is used)'

Choose:  q - Quit without update, c to Commit, <number> - To Select/Deselect, a - Select All, n - Select None

[*]1  iDRAC

Current Version : 2.30.30.30 Upgrade to : 2.41.40.40

[*]2 BCM57800 1-Gigabit Ethernet rev 10 (em4)

Current Version : 08.07.25 Upgrade to : 10.00.15

[*]3 BCM57800 1-Gigabit Ethernet rev 10 (em3)

Current Version : 08.07.25 Upgrade to : 10.00.15

[*]4 BCM57800 10-Gigabit Ethernet rev 10 (em2)

Current Version : 08.07.25 Upgrade to : 10.00.15

[*]5 BCM57800 10-Gigabit Ethernet rev 10 (em1)

Current Version : 08.07.25 Upgrade to : 10.00.15

[*]6 OpenManage | iDRAC Service Module

Current Version : 0 Upgrade to : 2.5

[*]7 BIOS

Current Version : 2.1.6 Upgrade to : 2.4.3

[-]8 Firmware for  - Disk 0 in Backplane 1 of PERC H730P Mini Controller 0

Current Version : DE11 same as : DE11

[*]9 Dell 32 Bit uEFI Diagnostics, version 4239, 4239A33, 4239.41

Current Version : 4239A33 Upgrade to : 4239A36

[*]10 OS COLLECTOR 1.1, OSC_1.1, A00

Current Version : OSC_1.1 Downgrade to : 2.1

[*]11 Firmware for  - Disk 20 in Backplane 1 of PERC H730P Adapter Controller 1

Current Version : DL2B Upgrade to : DL2D

[*]12 PERC H730P Adapter Controller 1 Firmware

Current Version : 25.4.0.0017 Upgrade to : 25.5.2.0001

[*]13 PERC H730P Mini Controller 0 Firmware

Current Version : 25.4.0.0017 Upgrade to : 25.5.2.0001

[*]14 13G Backplane Expander FW

Current Version : 3.03 Upgrade to : 3.32

Enter your choice : c

Fetching Systems-Management_Application_DT30R_LN64_2.5_A00 ...

Installing Systems-Management_Application_DT30R_LN64_2.5_A00

Collecting inventory...

.

Running validation...

OpenManage || iDRAC Service Module

No version of this Update Package is currently installed.

Software application name: OpenManage | iDRAC Service Module

Package version: 2.5

Executing update...

WARNING: DO NOT STOP THIS PROCESS OR INSTALL OTHER DELL PRODUCTS WHILE UPDATE IS IN PROGRESS.

THESE ACTIONS MAY CAUSE YOUR SYSTEM TO BECOME UNSTABLE!

.

Update Failed

Systems-Management_Application_DT30R_LN64_2.5_A00 could not be installed

Fetching Diagnostics_Application_NNPW9_LN_4239A36_4239.44 ...

Installing Diagnostics_Application_NNPW9_LN_4239A36_4239.44

Collecting inventory...

...............................................

Running validation...

Dell 32 Bit uEFI Diagnostics, version 4239, 4239A33, 4239.41

The version of this Update Package is newer than the currently installed version.

Software application name: Dell 32 Bit uEFI Diagnostics, version 4239, 4239A33, 4239.41

Package version: 4239A36

Installed version: 4239A33

Executing update...

WARNING: DO NOT STOP THIS PROCESS OR INSTALL OTHER DELL PRODUCTS WHILE UPDATE IS IN PROGRESS.

THESE ACTIONS MAY CAUSE YOUR SYSTEM TO BECOME UNSTABLE!

......................................................................................................................................

The update completed successfully.

Fetching Diagnostics_Application_VX3V5_LN64_2.1_A00 ...

Installing Diagnostics_Application_VX3V5_LN64_2.1_A00

Collecting inventory...

................................................

Running validation...

OS COLLECTOR 1.1, OSC_1.1, A00

The version of this Update Package is older than the currently installed version.

Software application name: OS COLLECTOR 1.1, OSC_1.1, A00

Package version: 2.1

Installed version: OSC_1.1

Executing update...

WARNING: DO NOT STOP THIS PROCESS OR INSTALL OTHER DELL PRODUCTS WHILE UPDATE IS IN PROGRESS.

THESE ACTIONS MAY CAUSE YOUR SYSTEM TO BECOME UNSTABLE!

..............................................................................................................................................

The update completed successfully.

Fetching iDRAC-with-Lifecycle-Controller_Firmware_4950Y_LN_2.41.40.40_A00 ...

Installing iDRAC-with-Lifecycle-Controller_Firmware_4950Y_LN_2.41.40.40_A00

Collecting inventory...

.

Running validation...

iDRAC

The version of this Update Package is newer than the currently installed version.

Software application name: iDRAC

Package version: 2.41.40.40

Installed version: 2.30.30.30

Executing update...

WARNING: DO NOT STOP THIS PROCESS OR INSTALL OTHER DELL PRODUCTS WHILE UPDATE IS IN PROGRESS.

THESE ACTIONS MAY CAUSE YOUR SYSTEM TO BECOME UNSTABLE!

..............................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................

The update completed successfully.

Fetching Network_Firmware_98GPG_LN_10.00.15_A00-00 ...

Installing Network_Firmware_98GPG_LN_10.00.15_A00-00

Collecting inventory...

.

Running validation...

BCM57800 10-Gigabit Ethernet rev 10 (em1)

The version of this Update Package is newer than the currently installed version.

Software application name: BCM57800 10-Gigabit Ethernet rev 10 (em1)

Package version: 10.00.15

Installed version: 08.07.25

BCM57800 10-Gigabit Ethernet rev 10 (em2)

The version of this Update Package is newer than the currently installed version.

Software application name: BCM57800 10-Gigabit Ethernet rev 10 (em2)

Package version: 10.00.15

Installed version: 08.07.25

BCM57800 1-Gigabit Ethernet rev 10 (em3)

The version of this Update Package is newer than the currently installed version.

Software application name: BCM57800 1-Gigabit Ethernet rev 10 (em3)

Package version: 10.00.15

Installed version: 08.07.25

BCM57800 1-Gigabit Ethernet rev 10 (em4)

The version of this Update Package is newer than the currently installed version.

Software application name: BCM57800 1-Gigabit Ethernet rev 10 (em4)

Package version: 10.00.15

Installed version: 08.07.25

Executing update...

WARNING: DO NOT STOP THIS PROCESS OR INSTALL OTHER PRODUCTS WHILE UPDATE IS IN PROGRESS.

THESE ACTIONS MAY CAUSE YOUR SYSTEM TO BECOME UNSTABLE!

...........................................................................................................................................................

The system should be restarted for the update to take effect.

Fetching Serial-ATA_Firmware_KCXYV_LN_DL2D_A00 ...

Installing Serial-ATA_Firmware_KCXYV_LN_DL2D_A00

Collecting inventory...

..

Running validation...

INTEL SSDSC2BA40

The version of this Update Package is newer than the currently installed version.

Software application name: Firmware for - Disk 20 in Backplane 1 of PERC H730P Adapter Controller 1

FQDD: Disk.Bay.20:Enclosure.Internal.1:RAID.Integrated.1-1

Package version: DL2D

Installed version: DL2B

INTEL SSDSC2BA40

The version of this Update Package is newer than the currently installed version.

Software application name: Firmware for - Disk 21 in Backplane 1 of PERC H730P Adapter Controller 1

FQDD: Disk.Bay.21:Enclosure.Internal.1:RAID.Integrated.1-1

Package version: DL2D

Installed version: DL2B

Executing update...

WARNING: DO NOT STOP THIS PROCESS OR INSTALL OTHER PRODUCTS WHILE UPDATE IS IN PROGRESS.

THESE ACTIONS MAY CAUSE YOUR SYSTEM TO BECOME UNSTABLE!

..................................................................................................................................

The update completed successfully.

Fetching SAS-RAID_Firmware_NH55C_LN_25.5.2.0001_A09_02 ...

Installing SAS-RAID_Firmware_NH55C_LN_25.5.2.0001_A09_02

Collecting inventory...

..

Running validation...

PERC H730P Adapter Controller 1

The version of this Update Package is newer than the currently installed version.

Software application name: PERC H730P Adapter Controller 1 Firmware

Package version: 25.5.2.0001

Installed version: 25.4.0.0017

PERC H730P Mini Controller 0

The version of this Update Package is newer than the currently installed version.

Software application name: PERC H730P Mini Controller 0 Firmware

Package version: 25.5.2.0001

Installed version: 25.4.0.0017

Executing update...

WARNING: DO NOT STOP THIS PROCESS OR INSTALL OTHER PRODUCTS WHILE UPDATE IS IN PROGRESS.

THESE ACTIONS MAY CAUSE YOUR SYSTEM TO BECOME UNSTABLE!

...................................................................................................................

The system should be restarted for the update to take effect.

Fetching Firmware_V23C6_LN_3.32_A00-00_03 ...

Installing Firmware_V23C6_LN_3.32_A00-00_03

Collecting inventory...

......................

Running validation...

13G Backplane Expander Firmware

The version of this Update Package is newer than the currently installed version.

Software application name: 13G Backplane Expander Firmware

Package version: 3.32

Installed version: 3.03

Executing update...

WARNING: DO NOT STOP THIS PROCESS OR INSTALL OTHER PRODUCTS WHILE UPDATE IS IN PROGRESS.

THESE ACTIONS MAY CAUSE YOUR SYSTEM TO BECOME UNSTABLE!

........................................................................

The update completed successfully.

Fetching BIOS_6YDCM_LN_2.4.3 ...

Installing BIOS_6YDCM_LN_2.4.3

Collecting inventory...

Running validation...

PowerEdge BIOS

The version of this Update Package is newer than the currently installed version.

Software application name: BIOS

Package version: 2.4.3

Installed version: 2.1.6

Executing update...

WARNING: DO NOT STOP THIS PROCESS OR INSTALL OTHER DELL PRODUCTS WHILE UPDATE IS IN PROGRESS.

THESE ACTIONS MAY CAUSE YOUR SYSTEM TO BECOME UNSTABLE!

.........................................................

The system should be restarted for the update to take effect.

Please reboot the system for update(s) to take effect

Done! Please run 'dsu --inventory' to check the inventory

Exiting DSU!

Connection to ds-gp-prd-sdw14 closed.

[root@ds-gp-prd-smdw ~]#

#

#	20190603

#	How to get mac addresses of embedded network cards.

#

[s.belogrud@TCS4357]# ssh tcsbank.ru\\s.belogrud@10.219.109.178 racadm getsysinfo | egrep -ie "Host Name|Service Tag|NIC.*1-1-1|NIC.*1-2-1"

Service Tag             = 778V762

Host Name               = m1-gp-prod-sdw20

NIC.Integrated.1-1-1    Ethernet                = 44:A8:42:2F:AB:C5

NIC.Integrated.1-2-1    Ethernet                = 44:A8:42:2F:AB:C7

[s.belogrud@TCS4357]#

[s.belogrud@TCS4357]# ssh tcsbank.ru\\s.belogrud@10.219.109.178

...

/admin1-> racadm get

iDRAC

System

LifeCycleController

BIOS

NIC

STORAGE

FC

/admin1->

/admin1-> racadm get nic.nicconfig

NIC.nicconfig.1 [Key=NIC.Integrated.1-1-1#nicconfig]

NIC.nicconfig.2 [Key=NIC.Integrated.1-2-1#nicconfig]

NIC.nicconfig.3 [Key=NIC.Integrated.1-3-1#nicconfig]

NIC.nicconfig.4 [Key=NIC.Integrated.1-4-1#nicconfig]

/admin1-> racadm get nic.nicconfig.1

[Key=NIC.Integrated.1-1-1#nicconfig]

BannerMessageTimeout=5

BootOptionROM=Enabled

BootRetryCnt=NoRetry

BootStrapType=AutoDetect

HideSetupPrompt=Disabled

LegacyBootProto=PXE

LnkSpeed=10Gbps

#VLanId=1

VLanMode=Disabled

WakeOnLan=Disabled

/admin1->

https://rowty.net/view/84/1/Power-on--off-Dell-server-from-idrac

Power on, off Dell server from idrac

rated 3 times [    3] [  0]  / comments: 3 / hits: 3155  / 2 yaers ago, fri, mar 3, 2017, 09:32:34

You can use the following commands after ssh to idrac to power off, on, cycle, etc.. your Dell server

# power server off

racadm serveraction powerdown

# power server on

racadm serveraction powerup

# perform server power cycle

racadm serveraction powercycle

# force hard server power reset

racadm serveraction hardreset

# display current power status of server

racadm serveraction powerstatus

#

#	20190821

#	Failure on Backplane.

#	And only nagios show this failure.

#	In iDRAC I see everything is “Green”.

#

[root@vm-vol-nag4-tst objects]# /usr/local/nagios/libexec/check_openmanage -H 10.216.2.32 -C tcspub -P 2c -p

Enclosure 0:0:1 [Backplane] on controller 0 is Failed

Enclosure 1:0:1 [Backplane] on controller 1 is Failed|T0_System_Board_Inlet=23C;42;47 T1_System_Board_Exhaust=36C;70;75 T2_CPU1=55C;85;90 T3_CPU2=51C;85;90 W2_System_Board_Pwr_Consumption=336W;0;0

A0_PS1_Current_1=0.8A;0;0 A1_PS2_Current_2=0.6A;0;0 V26_PS1_Voltage_1=228V;0;0 V27_PS2_Voltage_2=228V;0;0 F0_System_Board_Fan1=6360rpm;0;0 F1_System_Board_Fan2=6240rpm;0;0 F2_System_Board_Fan3=6240

rpm;0;0 F3_System_Board_Fan4=6120rpm;0;0 F4_System_Board_Fan5=6240rpm;0;0 F5_System_Board_Fan6=6240rpm;0;0

[root@vm-vol-nag4-tst objects]#

[root@vm-vol-nag4-tst objects]# /usr/local/nagios/libexec/check_openmanage -H 10.216.2.32 -C tcspub -P 2c -p -d

System:      PowerEdge R730xd         OMSA version:    9.3.0

ServiceTag:  2FBV762                  Plugin version:  3.7.12

BIOS/date:   2.9.1 12/04/2018         Checking mode:   SNMPv2c UDP/IPv4

-----------------------------------------------------------------------------

Storage Components

=============================================================================

STATE  |    ID    |  MESSAGE TEXT

---------+----------+--------------------------------------------------------

OK |        0 | Controller 0 [PERC H730P Mini] is Ready

...

CRITICAL |    0:0:1 | Enclosure 0:0:1 [Backplane] on controller 0 is Failed

CRITICAL |    1:0:1 | Enclosure 1:0:1 [Backplane] on controller 1 is Failed

-----------------------------------------------------------------------------

[14:30:50 root@m1-gp-prod-sdw32 ~]# omreport storage enclosure

List of Enclosures in the System

Enclosure(s) on Controller PERC H730P Mini (Embedded)

ID                          : 0:1

Status                      : Critical

Name                        : Backplane

State                       : Failed

Connector                   : 0

Target ID                   : Not Applicable

Configuration               : Not Applicable

Firmware Version            : 3.35

Downstream Firmware Version : Not Applicable

Service Tag                 : Not Applicable

Express Service Code        : Not Applicable

Asset Tag                   : Not Applicable

Asset Name                  : Not Applicable

Backplane Part Number       : Not Applicable

Split Bus Part Number       : Not Applicable

Enclosure Part Number       : Not Applicable

SAS Address                 : 500056B3841934FD

Enclosure Alarm             : Not Applicable

Enclosure(s) on Controller PERC H730P Adapter (Slot 3)

ID                          : 0:1

Status                      : Critical

Name                        : Backplane

State                       : Failed

Connector                   : 0

Target ID                   : Not Applicable

Configuration               : Not Applicable

Firmware Version            : 3.35

Downstream Firmware Version : Not Applicable

Service Tag                 : Not Applicable

Express Service Code        : Not Applicable

Asset Tag                   : Not Applicable

Asset Name                  : Not Applicable

Backplane Part Number       : Not Applicable

Split Bus Part Number       : Not Applicable

Enclosure Part Number       : Not Applicable

SAS Address                 : 500056B3841934FD

Enclosure Alarm             : Not Applicable

[14:31:09 root@m1-gp-prod-sdw32 ~]#

#

#	But here - there are no problems.

#

[14:16:03 root@m1-gp-prod-sdw32 ~]# omreport chassis

Health

Main System Chassis

SEVERITY : COMPONENT

Ok       : Fans

Ok       : Intrusion

Ok       : Memory

Ok       : Power Supplies

Ok       : Power Management

Ok       : Processors

Ok       : Temperatures

Ok       : Voltages

Ok       : Hardware Log

Ok       : Batteries

For further help, type the command followed by -?

[14:16:19 root@m1-gp-prod-sdw32 ~]#

#

#	20190919

#	Found problems with raid controller cache battery.

#	https://www.dell.com/support/manuals/ru/ru/19/integrated-dell-remote-access-cntrllr-8-with-lifecycle-controller-v2.00.00.00/racadm_idrac_pub-v1/storage?guid=guid-9e3676cb-b71d-420b-8c48-c80add258e03&lang=en-us

#

#	To see the latest log messages.

#

[16:04:28 root@m1-gp-prod-sdw33 ~]# racadm storage get batteries -o

Battery.Integrated.1:RAID.Slot.3-1

Name                             = Battery

DeviceDescription                = Battery on RAID Controller in Slot 3

Status                           = Ok

State                            = Ready

Battery.Integrated.1:RAID.Integrated.1-1

Name                             = Battery

DeviceDescription                = Battery on Integrated RAID Controller 1

Status                           = Failed

State                            = Removed

[16:04:30 root@m1-gp-prod-sdw33 ~]#

[15:55:22 root@m1-gp-prod-sdw33 ~]# racadm getsel | tail -n 20

-------------------------------------------------------------------------------

Record:      109

Date/Time:   09/19/2019 15:45:21

Source:      system

Severity:    Critical

Description: The PERC2 battery has failed.

-------------------------------------------------------------------------------

Record:      110

Date/Time:   09/19/2019 15:50:24

Source:      system

Severity:    Ok

Description: The PERC2 battery is operating normally.

-------------------------------------------------------------------------------

Record:      111

Date/Time:   09/19/2019 15:53:20

Source:      system

Severity:    Critical

Description: The PERC2 battery has failed.

-------------------------------------------------------------------------------

[15:55:39 root@m1-gp-prod-sdw33 ~]#

#

#	To see count of messages.

[15:52:24 root@m1-gp-prod-sdw33 ~]# racadm getsel -c 5

Record:      1

Date/Time:   09/18/2019 23:02:01

Source:      system

Severity:    Ok

Description: Log cleared.

-------------------------------------------------------------------------------

Record:      2

Date/Time:   09/18/2019 23:06:53

Source:      system

Severity:    Ok

Description: The PERC2 battery is operating normally.

-------------------------------------------------------------------------------

Record:      3

Date/Time:   09/18/2019 23:11:08

Source:      system

Severity:    Critical

Description: The PERC2 battery has failed.

-------------------------------------------------------------------------------

Record:      4

Date/Time:   09/18/2019 23:17:19

Source:      system

Severity:    Ok

Description: The PERC2 battery is operating normally.

-------------------------------------------------------------------------------

Record:      5

Date/Time:   09/18/2019 23:17:39

Source:      system

Severity:    Critical

Description: The PERC2 battery has failed.

-------------------------------------------------------------------------------

[15:53:39 root@m1-gp-prod-sdw33 ~]#

#	Here is how looks battery problems.

[16:00:28 root@m1-gp-prod-sdw33 ~]# racadm storage get controllers:RAID.Integrated.1-1

RAID.Integrated.1-1

Status                           = Ok

DeviceDescription                = Integrated RAID Controller 1

RollupStatus                     = Warning

Name                             = PERC H730P Mini (Embedded)

FirmwareVersion                  = 25.5.5.0005

DriverVersion                    = --NA--

RebuildRate                      = 30

BgiRate                          = 30

CheckConsistencyRate             = 30

ReconstructRate                  = 30

PatrolReadRate                   = 30

PatrolReadMode                   = Automatic

PatrolReadState                  = Stopped

CheckConsistencyMode             = Normal

LoadBalanceSetting               = Auto

CopybackMode                     = ON

PreservedCache                   = Not Present

CacheMemorySize                  = 2048 MB

PersistHotspare                  = Disabled

KeyID                            = null

SpindownUnconfiguredDrives       = Disabled

SpindownHotspare                 = Disabled

Timeintervalforspindown          = 30 (Minutes)

SecurityStatus                   = Encryption Capable

EncryptionMode                   = None

SasAddress                       = 0x544A8420347BFD00

PciDeviceId                      = 0x5d

PciSubdeviceId                   = 0x1f47

PciVendorId                      = 0x1000

PciSubvendorId                   = 0x1028

PciBus                           = 0x2

PciDevice                        = 0x0

PciFunction                      = 0x0

BusWidth                         = Unknown

SlotLength                       = Unknown

SlotType                         = Unknown

MaxCapableSpeed                  = 12.0 Gb/s

LearnMode                        = Not supported

T10PICapability                  = Capable

SupportRAID10UnevenSpans         = Supported

SupportEnhancedAutoForeignImport = Supported

EnhancedAutoImportForeignConfig  = Disabled

SupportControllerBootMode        = Supported

ControllerBootMode               = Continue Boot On Error

RealtimeConfigurationCapability  = Capable

bootVD                           = Disk.Virtual.0:RAID.Integrated.1-1

CurrentControllerMode            = RAID

[16:00:48 root@m1-gp-prod-sdw33 ~]#

[17:20:37 root@m1-gp-prod-sdw33 ~]# lsscsi --long -v 0:2:1:0

[0:2:1:0]    disk    DELL     PERC H730P Mini  4.29  /dev/sdb

state=running queue_depth=256 scsi_level=6 type=0 device_blocked=0 timeout=90

dir: /sys/bus/scsi/devices/0:2:1:0  [/sys/devices/pci0000:00/0000:00:01.0/0000:02:00.0/host0/target0:2:1/0:2:1:0]

[17:21:26 root@m1-gp-prod-sdw33 ~]#

[17:22:51 root@m1-gp-prod-sdw33 ~]# lspci -s 0000:02:00.0

02:00.0 RAID bus controller: LSI Logic / Symbios Logic MegaRAID SAS-3 3108 [Invader] (rev 02)

[17:23:00 root@m1-gp-prod-sdw33 ~]#

#

#	20190925

#	Some info how to automate iDRAC configuration changes with Power shell scripting.

#

https://wiki.tcsbank.ru/pages/viewpage.action?pageId=530187136

Для настройки параметров через PS, можно воспользоваться следующим способом:

Устанавливаем модуль через NuGet: Find-Module Posh-SSH | Install-Module

Сохраняем связку логин-пароль в переменную $cred = Get-Credential s.barkov@tcsbank.ru

Подключаемся по SSH: $sshsession = New-SSHSession -ComputerName $h -Credential $cred -Verbose -AcceptKey

Создаем поток, который можем читать/писать: $ssh = $sshsession | New-SSHShellStream

Настраиваем DNS: $ssh.WriteLine("racadm set iDRAC.IPv4.DNS1 10.219.50.4")

$ssh.WriteLine("racadm set iDRAC.IPv4.DNS2 10.219.50.5")

Если нужно прочитать ответ, то $ssh.Read()

Закрываем SSH сессию: $sshsession | Remove-SSHSession

Если хотим заскриптовать, то можно сделать так:

Read-host -assecurestring | convertfrom-securestring | out-file 'C:\tasks\dell\p.pwd'

И в скрипте:

$password = get-content C:\tasks\dell\p.pwd | convertto-securestring

$cred = new-object -typename System.Management.Automation.PSCredential -argumentlist "s.barkov@tcsbank.ru",$password

param ($h)

if ($h){

$password = get-content C:\tasks\dell\p.pwd | convertto-securestring

$cred = new-object -typename System.Management.Automation.PSCredential -argumentlist "s.barkov@tcsbank.ru",$password

#$cred = Get-Credential s_barkov@tcsbank.ru

$sshsession = New-SSHSession -ComputerName $h -Credential $cred -Verbose -AcceptKey

start-sleep 3

$ssh = $sshsession | New-SSHShellStream

start-sleep 3

$ssh.WriteLine("racadm set iDRAC.IPv4.DNS1 10.219.50.4")

start-sleep 5

$ssh.Read()

start-sleep 5

$ssh.WriteLine("racadm set iDRAC.IPv4.DNS2 10.219.50.5")

start-sleep 5

$ssh.Read()

start-sleep 5

$sshsession | Remove-SSHSession

}

Вызов: скрипт.ps1 Z8RHXJ2.mgmt.tcsbank.ru

#

#	https://wiki.tcsbank.ru/pages/viewpage.action?pageId=526489297

#

#

#   http://fibrevillage.com/storage/605-how-to-find-hard-drive-model-and-serial-number-on-linux

#

#   scsi_id

#   Use scsi_id command to querys a SCSI device via the SCSI INQUIRY vital product data (VPD) page 0x80 or 0x83 and uses the resulting data to generate a value that is unique across all SCSI devices that properly support page 0x80 or page 0x83.

#

Belogrud Sergey Vladimirovich

Если известен серийный номер проблемного диска, как понять к какому разделу файловой системы он относится?

Нажмите здесь для раскрытия...

Относительно быстрый метод на примере сервера у которого есть диски непосредственно смонтированные в файловую систему и есть диски объединённые в raid и уже этот логический диск смонтирован в файловую систему.

Блочные устройства /dev/sg0, /dev/sg1 - это логическое объединение наборов физических дисков в raid-массив.

Блочные устройства /dev/sg2 - /dev/sg11 - это физические диски смонтированные в файловую систему, каждый в свой каталог.

[root@ds-gp-prd-bkp1 ~]# lsscsi -g

[0:0:2:0]    disk    SEAGATE  ST8000NM0075     PS26  /dev/sdc   /dev/sg2

[0:0:3:0]    disk    SEAGATE  ST8000NM0075     PS26  /dev/sdd   /dev/sg3

[0:0:4:0]    disk    HGST     HUS728T8TAL5200  RS01  /dev/sde   /dev/sg4

[0:0:5:0]    disk    SEAGATE  ST8000NM0075     PS26  /dev/sdf   /dev/sg5

[0:0:6:0]    disk    SEAGATE  ST8000NM0075     PS26  /dev/sdg   /dev/sg6

[0:0:7:0]    disk    SEAGATE  ST8000NM0075     PS26  /dev/sdh   /dev/sg7

[0:0:8:0]    disk    SEAGATE  ST8000NM0075     PS26  /dev/sdi   /dev/sg8

[0:0:9:0]    disk    SEAGATE  ST8000NM0075     PS26  /dev/sdj   /dev/sg9

[0:0:10:0]   disk    SEAGATE  ST8000NM0075     PS26  /dev/sdk   /dev/sg10

[0:0:11:0]   disk    SEAGATE  ST8000NM0075     PS26  /dev/sdl   /dev/sg11

[0:2:0:0]    disk    DELL     PERC H730P Mini  4.29  /dev/sda   /dev/sg0

[0:2:2:0]    disk    DELL     PERC H730P Mini  4.29  /dev/sdb   /dev/sg1

[root@ds-gp-prd-bkp1 ~]#

Посмотреть серийные номера дисков, непосредственно смонтированных в файловую систему можно так.

Но эта команда не покажет серийные номера дисков, объединённых в raid-массив. Это блочные устройства /dev/sg0, /dev/sg1.

[root@ds-gp-prd-bkp1 ~]# for var1 in {0..11} ; do echo -e "\nCount: ${var1}: /dev/sg${var1}" ; scsi_id -g -p 0x80 /dev/sg${var1} ; done

Count: 0: /dev/sg0

SDELL    PERC H730P Mini00ab3758133602ad1f0068967da06d86

Count: 1: /dev/sg1

SDELL    PERC H730P Mini00b109aa0af1e367200068967da06d86

Count: 2: /dev/sg2

SSEAGATE ST8000NM0075   ZA13F1GL

Count: 3: /dev/sg3

SSEAGATE ST8000NM0075   ZA147348

Count: 4: /dev/sg4

SHGST    HUS728T8TAL5200VAJ2XV5L

Count: 5: /dev/sg5

SSEAGATE ST8000NM0075   ZA13K0Z0

Count: 6: /dev/sg6

SSEAGATE ST8000NM0075   ZA13JZV9

Count: 7: /dev/sg7

SSEAGATE ST8000NM0075   ZA145HXV

Count: 8: /dev/sg8

SSEAGATE ST8000NM0075   ZA13KMVH

Count: 9: /dev/sg9

SSEAGATE ST8000NM0075   ZA14DT6T

Count: 10: /dev/sg10

SSEAGATE ST8000NM0075   ZA145GQJ

Count: 11: /dev/sg11

SSEAGATE ST8000NM0075   ZA13KLRH

[root@ds-gp-prd-bkp1 ~]#

Пусть для примера серийный номер диска находится среди найденных - допустим это "ZA145HXV".

Здесь, просто так для примера, использованы два способа показать серийный номер диска по известному идентификатору диска. Утилитой scsi_id или smartctl.

[root@ds-gp-prd-bkp1 ~]# scsi_id -g -p 0x80 /dev/sg7

SSEAGATE ST8000NM0075   ZA145HXV

[root@ds-gp-prd-bkp1 ~]#

[root@ds-gp-prd-bkp1 ~]# smartctl -a /dev/sg7 | grep -e "Serial number:" -e "Device type:" -e "Transport protocol:"

Serial number:        ZA145HXV

Device type:          disk

Transport protocol:   SAS

[root@ds-gp-prd-bkp1 ~]#

Теперь по известной идентификации блочного устройства: "/dev/sg7" и выводу команды "lsscsi -g" можно - сопоставляем, что /dev/sg7 это /dev/sdh.

В файловой системе linux искомое блочное устройство определяется, как /dev/sdh.

С помощью следующей команды можно понять в какой каталог смонтировано блочное устройство /dev/sdh - это /data06.

[root@ds-gp-prd-bkp1 ~]# lsblk -o NAME,TYPE,MOUNTPOINT,FSTYPE,SIZE /dev/sdh

NAME TYPE MOUNTPOINT FSTYPE  SIZE

sdh  disk /data06    ext4    7.3T

[root@ds-gp-prd-bkp1 ~]#

Для случая, когда требуется посмотреть серийные номера дисков, логически объединённых в raid-массив.

Используется утилита smartctl для идентификатора блочного устройства /dev/sg0 (аналогично и для /dev/sg1). Количество индексов для дисков в этой утилите берётся с запасом, чтобы точно быть уверенным, что собрана информация обо всех дисках объединённых в данный raid-массив. Для последних не использующихся индексах - информация о диске будет пустой - диск отсутствует.

[root@ds-gp-prd-bkp1 ~]# for var1 in {0..20} ; do echo -e "\nCount: ${var1}" ; smartctl -a -d megaraid,${var1} /dev/sg0 | grep -e "Serial number:" -e "Device type:" -e "Transport protocol:" ; done

Count: 0

Serial number:        X6J0A03PT2XD

Device type:          disk

Transport protocol:   SAS

Count: 1

Serial number:        X6J0A03QT2XD

Device type:          disk

Transport protocol:   SAS

Count: 2

Serial number:        ZA13F1GL

Device type:          disk

Transport protocol:   SAS

Count: 3

Serial number:        ZA147348

Device type:          disk

Transport protocol:   SAS

Count: 4

Serial number:        VAJ2XV5L

Device type:          disk

Transport protocol:   SAS

Count: 5

Serial number:        ZA13K0Z0

Device type:          disk

Transport protocol:   SAS

Count: 6

Serial number:        ZA13JZV9

Device type:          disk

Transport protocol:   SAS

Count: 7

Serial number:        ZA145HXV

Device type:          disk

Transport protocol:   SAS

Count: 8

Serial number:        ZA13KMVH

Device type:          disk

Transport protocol:   SAS

Count: 9

Serial number:        ZA14DT6T

Device type:          disk

Transport protocol:   SAS

Count: 10

Serial number:        ZA145GQJ

Device type:          disk

Transport protocol:   SAS

Count: 11

Serial number:        ZA13KLRH

Device type:          disk

Transport protocol:   SAS

Count: 12

Serial number:        S422JYQ8

Device type:          disk

Transport protocol:   SAS

Count: 13

Serial number:        S422L95K

Device type:          disk

Transport protocol:   SAS

Count: 14

Count: 15

Count: 16

Count: 17

Count: 18

Count: 19

Count: 20

[root@ds-gp-prd-bkp1 ~]#

И, как и ранее, с помощью следующей команды можно понять какие каталоги размещены на дисках объединённых логически в raid-массив устройстве /dev/sda.

[root@ds-gp-prd-bkp1 ~]# lsblk -o NAME,TYPE,MOUNTPOINT,FSTYPE,SIZE /dev/sda

NAME                              TYPE MOUNTPOINT FSTYPE        SIZE

sda                               disk                        278.9G

├─sda1                            part            vfat          500M

├─sda2                            part            vfat           40M

├─sda3                            part            vfat            2G

├─sda4                            part /boot      ext2          500M

└─sda5                            part            LVM2_member 275.9G

├─vg_dsgpprdbkp1-lv_root (dm-0) lvm  /          ext4           50G

├─vg_dsgpprdbkp1-lv_swap (dm-1) lvm  [SWAP]     swap            4G

└─vg_dsgpprdbkp1-lv_opt (dm-2)  lvm  /opt       xfs         221.9G

[root@ds-gp-prd-bkp1 ~]#