#

#	20190304

#

#	Show info about memory on server.

[root@m1-pr-iris02p ~]# hpasmcli -s "SHOW DIMM"

DIMM Configuration

------------------

Processor #:                     1

Module #:                     1

Present:                      Yes

Form Factor:                  9h

Memory Type:                  DDR4(1ah)

Size:                         16384 MB

Speed:                        2133 MHz

Supports Lock Step:           No

Configured for Lock Step:     No

Status:                       Ok

Processor #:                     1

Module #:                     2

Present:                      Yes

Form Factor:                  9h

Memory Type:                  DDR4(1ah)

Size:                         16384 MB

Speed:                        2133 MHz

Supports Lock Step:           No

Configured for Lock Step:     No

Status:                       Ok

...

[root@m1-pr-iris02p ~]#

[root@m1-pr-iris02p ~]# dmidecode -t system

# dmidecode 3.0

# SMBIOS entry point at 0x788db000

Found SMBIOS entry point in EFI, reading table from /dev/mem.

SMBIOS 2.8 present.

Handle 0x0009, DMI type 1, 27 bytes

System Information

Manufacturer: HP

Product Name: ProLiant DL360 Gen9

Version: Not Specified

Serial Number: CZJ44804QH

UUID: 32353537-3236-5A43-4A34-343830345148

Wake-up Type: Power Switch

SKU Number: 755262-B21

Family: ProLiant

Handle 0x00BF, DMI type 32, 11 bytes

System Boot Information

Status: No errors detected

[root@m1-pr-iris02p ~]#

[root@smdw_test snmp]# yum install hp-health

Loaded plugins: downloadonly, product-id, subscription-manager

Updating Red Hat repositories.

Setting up Install Process

Package matching hp-health-9.1.0.42-54.rhel6.x86_64 already installed. Checking for update.

Nothing to do

[root@smdw_test snmp]# rpm -ql hp-health

/etc/init.d/hp-asrd

/etc/init.d/hp-health

/opt/hp/hp-health

/opt/hp/hp-health/addons

/opt/hp/hp-health/addons/libcpqci64.a

/opt/hp/hp-health/bin

/opt/hp/hp-health/bin/IrqRouteTbl

/opt/hp/hp-health/bin/hp-asrd

/opt/hp/hp-health/bin/hpasmlited

/opt/hp/hp-health/bin/hpasmpld

/opt/hp/hp-health/bin/hpasmxld

/opt/hp/hp-health/examples

/opt/hp/hp-health/examples/lite-mgmt

/opt/hp/hp-health/examples/lite-mgmt/COPYING.txt

/opt/hp/hp-health/examples/lite-mgmt/README.txt

/opt/hp/hp-health/examples/lite-mgmt/cma.conf.nosnmp

/opt/hp/hp-health/examples/lite-mgmt/hpsysmon.sh

/opt/hp/hp-health/hp-health.license

/opt/hp/hp-health/hprpm.xpm

/opt/hp/hp-health/sh

/opt/hp/hp-health/sh/hpasmxld_reset.sh

/sbin/hpasmcli

/sbin/hpbootcfg

/sbin/hplog

/sbin/hpuid

/usr/lib64/libcpqci64.so

/usr/lib64/libcpqci64.so.3

/usr/lib64/libcpqci64.so.3.0

/usr/lib64/libhpasmintrfc64.so

/usr/lib64/libhpasmintrfc64.so.3

/usr/lib64/libhpasmintrfc64.so.3.0

/usr/lib64/libhpev64.so

/usr/lib64/libhpev64.so.1

/usr/lib64/libhpev64.so.1.0

/usr/share/man/man4/hp-health.4.gz

/usr/share/man/man4/hpasmcli.4.gz

/usr/share/man/man7/hp_mgmt_install.7.gz

/usr/share/man/man8/hp-asrd.8.gz

/usr/share/man/man8/hpbootcfg.8.gz

/usr/share/man/man8/hplog.8.gz

/usr/share/man/man8/hpuid.8.gz

/var/log/hp-health

/var/spool/compaq

/var/spool/hp-health

[root@smdw_test snmp]# /sbin/hpasmcli

HP management CLI for Linux (v2.0)

Copyright 2008 Hewlett-Packard Development Group, L.P.

--------------------------------------------------------------------------

NOTE: Some hpasmcli commands may not be supported on all Proliant servers.

Type 'help' to get a list of all top level commands.

--------------------------------------------------------------------------

hpasmcli> help

CLEAR  DISABLE  ENABLE  EXIT  HELP  NOTE  QUIT  REPAIR  SET  SHOW

hpasmcli> show

Invalid Arguments

SHOW ASR

SHOW BOOT

SHOW DIMM [ SPD ]

SHOW F1

SHOW FANS

SHOW HT

SHOW IML

SHOW IPL

SHOW NAME

SHOW PORTMAP

SHOW POWERMETER

SHOW POWERSUPPLY

SHOW PXE

SHOW SERIAL [ BIOS | EMBEDDED | VIRTUAL ]

SHOW SERVER

SHOW TEMP

SHOW TPM

SHOW UID

SHOW WOL

hpasmcli> show asr

ASR timeout is 10 minutes.

ASR is currently enabled.

hpasmcli> show boot

First boot device is: CDROM.

One time boot device is: Not set.

hpasmcli> show server

System        : ProLiant DL380 G7

Serial No.    : CZ314824F7

ROM version   : P67 07/02/2013

iLo present   : Yes

Embedded NICs : 4

NIC1 MAC: 3c:d9:2b:04:7e:7e

NIC2 MAC: 3c:d9:2b:04:7e:80

NIC3 MAC: 3c:d9:2b:04:7e:82

NIC4 MAC: 3c:d9:2b:04:7e:84

Processor: 0

Name         : Intel Xeon

Stepping     : 2

Speed        : 2800 MHz

Bus          : 133 MHz

Core         : 6

Thread       : 12

Socket       : 1

Level1 Cache : 192 KBytes

Level2 Cache : 1536 KBytes

Level3 Cache : 12288 KBytes

Status       : Ok

Processor: 1

Name         : Intel Xeon

Stepping     : 2

Speed        : 2800 MHz

Bus          : 133 MHz

Core         : 6

Thread       : 12

Socket       : 2

Level1 Cache : 192 KBytes

Level2 Cache : 1536 KBytes

Level3 Cache : 12288 KBytes

Status       : Ok

Processor total  : 2

Memory installed : 131072 MBytes

ECC supported    : Yes

hpasmcli>

[root@smdw_test snmp]# /usr/local/nagios/libexec/check_hpasm --hostname 127.0.0.1 --community tcspub -vvv --perfdata=short

CRITICAL - could not find Net::SNMP module, wrong device

[root@smdw_test snmp]# service hp-health status

Using Proliant Standard

IPMI based System Health Monitor

(hpasmlited) is running...                               [  OK  ]

[root@smdw_test snmp]# service hp-health restart

Using Proliant Standard

IPMI based System Health Monitor

Shutting down NIC Agent Daemon (cmanicd):                [  OK  ]

Shutting down Storage Event Logger (cmaeventd):          [  OK  ]

Shutting down FCA agent (cmafcad):                       [  OK  ]

Shutting down SAS agent (cmasasd):                       [  OK  ]

Shutting down IDA agent (cmaidad):                       [  OK  ]

Shutting down IDE agent (cmaided):                       [  OK  ]

Shutting down SCSI agent (cmascsid):                     [  OK  ]

Shutting down Health agent (cmahealthd):                 [  OK  ]

Shutting down Standard Equipment agent (cmastdeqd):      [  OK  ]

Shutting down Host agent (cmahostd):                     [  OK  ]

Shutting down Threshold agent (cmathreshd):              [  OK  ]

Shutting down RIB agent (cmasm2d):                       [  OK  ]

Shutting down Performance agent (cmaperfd):              [  OK  ]

Shutting down SNMP Peer (cmapeerd):                      [  OK  ]

Shutting down Proliant Standard

IPMI based System Health Monitor (hpasmlited):     [  OK  ]

Starting Proliant Standard

IPMI based System Health Monitor (hpasmlited):

Starting Health agent (cmahealthd):                      [  OK  ]

Starting Standard Equipment agent (cmastdeqd):           [  OK  ]

Starting Host agent (cmahostd):                          [  OK  ]

Starting Threshold agent (cmathreshd):                   [  OK  ]

Starting RIB agent (cmasm2d):                            [  OK  ]

Starting Performance agent (cmaperfd):                   [  OK  ]

Starting SNMP Peer (cmapeerd):                           [  OK  ]

Starting Storage Event Logger (cmaeventd):               [  OK  ]

Starting FCA agent (cmafcad):                            [  OK  ]

Starting SAS agent (cmasasd):                            [  OK  ]

Starting IDA agent (cmaidad):                            [  OK  ]

Starting IDE agent (cmaided):                            [  OK  ]

Starting SCSI agent (cmascsid):                          [  OK  ]

Starting NIC Agent Daemon (cmanicd):                     [  OK  ]

[root@smdw_test snmp]#

[root@smdw ~]# hpasmcli

HP management CLI for Linux (v2.0)

Copyright 2008 Hewlett-Packard Development Group, L.P.

--------------------------------------------------------------------------

NOTE: Some hpasmcli commands may not be supported on all Proliant servers.

Type 'help' to get a list of all top level commands.

--------------------------------------------------------------------------

hpasmcli> help

CLEAR  DISABLE  ENABLE  EXIT  HELP  NOTE  QUIT  REPAIR  SET  SHOW

hpasmcli> show

Invalid Arguments

SHOW ASR

SHOW BOOT

SHOW DIMM [ SPD ]

SHOW F1

SHOW FANS

SHOW HT

SHOW IML

SHOW IPL

SHOW NAME

SHOW PORTMAP

SHOW POWERMETER

SHOW POWERSUPPLY

SHOW PXE

SHOW SERIAL [ BIOS | EMBEDDED | VIRTUAL ]

SHOW SERVER

SHOW TEMP

SHOW TPM

SHOW UID

SHOW WOL

hpasmcli> show iml

Event: 0 Added: 09/03/2012 20:06

INFO: Maintenance Note - Maintenance note: IML cleared through hpasmcli.

Event: 1 Added: 05/05/2011 00:04

CAUTION: POST Messages - POST Error: 1785-Drive Array not Configured.

Event: 2 Added: 05/05/2011 00:12

CAUTION: Power Subsystem - System Power Supply: General Failure (Power Supply 1).

Event: 3 Added: 05/05/2011 00:12

CAUTION: Power Subsystem - System Power Supplies Not Redundant.

Event: 4 Added: 08/17/2011 16:37

CAUTION: POST Messages - POST Error: 1704-Slot X Drive Array - Array Accelerator Super-Cap is charging. The Array Accelerator Cache will be enabled once Super-Cap has been charged. No action is required..

Event: 5 Added: 08/25/2011 15:13

CAUTION: POST Messages - POST Error: 1704-Slot X Drive Array - Array Accelerator Super-Cap is charging. The Array Accelerator Cache will be enabled once Super-Cap has been charged. No action is required..

Press q to quit, c to continue without pagination

or any other key to see the next screen.:

Event: 6 Added: 08/25/2011 15:20

INFO: Maintenance Note - Maintenance note: Updated Power Management Controller firmware to version 1.6.

Event: 7 Added: 08/25/2011 15:21

INFO: System Revision - ROM flashed (New version: 05/05/2011).

Event: 8 Added: 12/10/2011 14:26

CAUTION: POST Messages - POST Error: 1704-Slot X Drive Array - Array Accelerator Super-Cap is charging. The Array Accelerator Cache will be enabled once Super-Cap has been charged. No action is required..

Event: 9 Added: 12/11/2011 15:26

CAUTION: POST Messages - POST Error: 1729-Slot X Drive Array - Disk Performance Optimization Scan in Progress - RAID 5/6 performance may be higher after completion..

Event: 10 Added: 06/06/2014 16:29

INFO: System Revision - ROM flashed (New version: 07/02/2013).

Event: 11 Added: 06/07/2014 13:30

REPAIRED: Power Subsystem - System Power Supply: General Failure (Power Supply 2).

Press q to quit, c to continue without pagination

or any other key to see the next screen.:

Event: 12 Added: 06/07/2014 13:30

REPAIRED: Power Subsystem - System Power Supplies Not Redundant.

hpasmcli>

hpasmcli> SHOW POWERSUPPLY

Power supply #1

Present  : Yes

Redundant: Yes

Condition: Ok

Hotplug  : Supported

Power    : 80 Watts

Power supply #2

Present  : Yes

Redundant: Yes

Condition: Ok

Hotplug  : Supported

Power    : 90 Watts

hpasmcli>

hpasmcli> show f1

The POST F1 prompt is currently delayed.

hpasmcli>

hpasmcli> show asr

ASR timeout is 10 minutes.

ASR is currently enabled.

hpasmcli>

hpasmcli> show dimm

DIMM Configuration

------------------

Processor #:                     1

Module #:                     1

Present:                      Yes

Form Factor:                  9h

Memory Type:                  DDR3(18h)

Size:                         8192 MB

Speed:                        1333 MHz

Supports Lock Step:           Yes

Configured for Lock Step:     No

Status:                       Ok

Processor #:                     1

Module #:                     2

Present:                      Yes

Form Factor:                  9h

Memory Type:                  DDR3(18h)

Size:                         8192 MB

Speed:                        1333 MHz

Supports Lock Step:           Yes

Configured for Lock Step:     No

Status:                       Ok

Processor #:                     1

Module #:                     3

Present:                      Yes

Form Factor:                  9h

Memory Type:                  DDR3(18h)

Size:                         8192 MB

Speed:                        1333 MHz

Supports Lock Step:           Yes

Configured for Lock Step:     No

Status:                       Ok

Processor #:                     1

Module #:                     4

Present:                      Yes

Form Factor:                  9h

Memory Type:                  DDR3(18h)

Size:                         8192 MB

Speed:                        1333 MHz

Supports Lock Step:           Yes

Configured for Lock Step:     No

Status:                       Ok

Processor #:                     1

Module #:                     5

Present:                      Yes

Form Factor:                  9h

Memory Type:                  DDR3(18h)

Size:                         8192 MB

Speed:                        1333 MHz

Supports Lock Step:           Yes

Configured for Lock Step:     No

Status:                       Ok

Processor #:                     1

Module #:                     6

Present:                      Yes

Form Factor:                  9h

Memory Type:                  DDR3(18h)

Size:                         8192 MB

Speed:                        1333 MHz

Supports Lock Step:           Yes

Configured for Lock Step:     No

Status:                       Ok

Processor #:                     1

Module #:                     8

Present:                      Yes

Form Factor:                  9h

Memory Type:                  DDR3(18h)

Size:                         8192 MB

Speed:                        1333 MHz

Supports Lock Step:           Yes

Configured for Lock Step:     No

Status:                       Ok

Processor #:                     1

Module #:                     9

Present:                      Yes

Form Factor:                  9h

Memory Type:                  DDR3(18h)

Size:                         8192 MB

Speed:                        1333 MHz

Supports Lock Step:           Yes

Configured for Lock Step:     No

Status:                       Ok

Processor #:                     2

Module #:                     1

Present:                      Yes

Form Factor:                  9h

Memory Type:                  DDR3(18h)

Size:                         8192 MB

Speed:                        1333 MHz

Supports Lock Step:           Yes

Configured for Lock Step:     No

Status:                       Ok

Processor #:                     2

Module #:                     2

Present:                      Yes

Form Factor:                  9h

Memory Type:                  DDR3(18h)

Size:                         8192 MB

Speed:                        1333 MHz

Supports Lock Step:           Yes

Configured for Lock Step:     No

Status:                       Ok

Processor #:                     2

Module #:                     3

Present:                      Yes

Form Factor:                  9h

Memory Type:                  DDR3(18h)

Size:                         8192 MB

Speed:                        1333 MHz

Supports Lock Step:           Yes

Configured for Lock Step:     No

Status:                       Ok

Processor #:                     2

Module #:                     4

Present:                      Yes

Form Factor:                  9h

Memory Type:                  DDR3(18h)

Size:                         8192 MB

Speed:                        1333 MHz

Supports Lock Step:           Yes

Configured for Lock Step:     No

Status:                       Ok

Processor #:                     2

Module #:                     5

Present:                      Yes

Form Factor:                  9h

Memory Type:                  DDR3(18h)

Size:                         8192 MB

Speed:                        1333 MHz

Supports Lock Step:           Yes

Configured for Lock Step:     No

Status:                       Ok

Processor #:                     2

Module #:                     6

Present:                      Yes

Form Factor:                  9h

Memory Type:                  DDR3(18h)

Size:                         8192 MB

Speed:                        1333 MHz

Supports Lock Step:           Yes

Configured for Lock Step:     No

Status:                       Ok

Processor #:                     2

Module #:                     8

Present:                      Yes

Form Factor:                  9h

Memory Type:                  DDR3(18h)

Size:                         8192 MB

Speed:                        1333 MHz

Supports Lock Step:           Yes

Configured for Lock Step:     No

Status:                       Ok

Processor #:                     2

Module #:                     9

Present:                      Yes

Form Factor:                  9h

Memory Type:                  DDR3(18h)

Size:                         8192 MB

Speed:                        1333 MHz

Supports Lock Step:           Yes

Configured for Lock Step:     No

Status:                       Ok

hpasmcli>

hpasmcli> show dimm spd

DIMM Configuration

------------------

Processor #:                     1

Module #:                     1

Present:                      Yes

Form Factor:                  9h

Memory Type:                  DDR3(18h)

Size:                         8192 MB

Speed:                        1333 MHz

Supports Lock Step:           Yes

Configured for Lock Step:     No

Status:                       Ok

SPD BYTES:

_________

00: 92 11 0b 01 03 1a 00 08 0b 52 01 08 0c 00 7c 00

10: 69 78 69 30 69 11 20 89 00 05 3c 3c 00 f0 83 05

20: 80 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00

30: 00 00 00 00 00 00 00 00 00 00 00 00 0f 11 04 09

40: 00 04 b3 21 00 00 50 55 00 00 00 00 00 00 00 00

50: 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00

60: 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00

70: 00 00 00 00 00 80 ad 01 12 31 12 13 cc ff 06 07

80: 48 4d 54 33 31 47 52 37 43 46 52 34 43 2d 48 39

90: 20 20 54 33 80 ad 00 57 44 32 37 4e 39 30 32 33

a0: 49 31 38 00 01 03 02 04 03 02 05 03 00 00 00 00

b0: 00 00 00 00 00 00 00 00 00 00 00 00 00 00 24 0e

c0: 11 01 43 5a 33 32 33 36 56 53 54 39 00 00 00 00

d0: 00 00 0e 11 07 ad 01 00 01 01 00 00 00 00 ff ff

e0: ff ff ff ff ff ff ff ff ff ff ff ff ff ff ff ff

f0: ff ff ff ff ff ff 00 00 00 00 00 00 00 00 00 00

Processor #:                     1

Module #:                     2

Present:                      Yes

Form Factor:                  9h

Memory Type:                  DDR3(18h)

Size:                         8192 MB

Speed:                        1333 MHz

Supports Lock Step:           Yes

Configured for Lock Step:     No

Status:                       Ok

SPD BYTES:

_________

00: 92 11 0b 01 03 1a 00 08 0b 52 01 08 0c 00 7c 00

10: 69 78 69 30 69 11 20 89 00 05 3c 3c 00 f0 83 05

20: 80 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00

30: 00 00 00 00 00 00 00 00 00 00 00 00 0f 11 04 09

40: 00 04 b3 21 00 00 50 55 00 00 00 00 00 00 00 00

50: 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00

60: 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00

70: 00 00 00 00 00 80 ad 01 12 31 12 63 cc fe 06 07

80: 48 4d 54 33 31 47 52 37 43 46 52 34 43 2d 48 39

90: 20 20 54 33 80 ad 00 57 44 32 37 4e 39 30 32 33

a0: 49 31 38 00 01 03 02 04 03 02 05 03 00 00 00 00

b0: 00 00 00 00 00 00 00 00 00 00 00 00 00 00 24 0e

c0: 11 01 43 5a 33 32 33 36 56 53 54 39 00 00 00 00

d0: 00 00 0e 11 07 ad 01 00 01 02 00 00 00 00 ff ff

e0: ff ff ff ff ff ff ff ff ff ff ff ff ff ff ff ff

f0: ff ff ff ff ff ff 00 00 00 00 00 00 00 00 00 00

...

hpasmcli> show fans

Fan  Location        Present Speed  of max  Redundant  Partner  Hot-pluggable

---  --------        ------- -----  ------  ---------  -------  -------------

#1   SYSTEM          Yes     NORMAL  29%     Yes        0        Yes

#2   SYSTEM          Yes     NORMAL  29%     Yes        0        Yes

#3   SYSTEM          Yes     NORMAL  48%     Yes        0        Yes

#4   SYSTEM          Yes     NORMAL  57%     Yes        0        Yes

#5   SYSTEM          Yes     NORMAL  57%     Yes        0        Yes

#6   SYSTEM          Yes     NORMAL  13%     Yes        0        Yes

hpasmcli>

hpasmcli> show ht

Processor hyper-threading is currently enabled.

hpasmcli>

hpasmcli> show ipl

IPL (Standard Boot Order)

-------------------------

#0      CDROM

#1      Floppy

#2      USBKEY

#3      HDD

#4      PXE

hpasmcli>

hpasmcli> show name

Server name is: ds-gp-tst-sm.tcsbank.ru

hpasmcli>

hpasmcli> show portmap

WARNING: No rack information available from SMBIOS system data.  This is probably NOT a blade system.

hpasmcli>

hpasmcli> show powermeter

Power Meter #1

Power Reading  : 170

hpasmcli>

hpasmcli> show pxe

PXE boot status (4 Embedded NICs):

NIC1: PXE enabled

NIC2: PXE disabled

NIC3: PXE disabled

NIC4: PXE disabled

hpasmcli>

hpasmcli> show serial bios

BIOS console redirection port is currently set to COM1/9600.

hpasmcli>

hpasmcli> show serial embedded

Embedded serial port A: COM1

Embedded serial port B: Disabled

hpasmcli>

hpasmcli> show serial virtual

The virtual serial port is currently COM2.

hpasmcli>

hpasmcli> show server

System        : ProLiant DL380 G7

Serial No.    : CZ3236VST9

ROM version   : P67 07/02/2013

iLo present   : Yes

Embedded NICs : 4

NIC1 MAC: 00:9c:02:a8:d8:5a

NIC2 MAC: 00:9c:02:a8:d8:5c

NIC3 MAC: 00:9c:02:a8:d8:5e

NIC4 MAC: 00:9c:02:a8:d8:60

Processor: 0

Name         : Intel Xeon

Stepping     : 2

Speed        : 2800 MHz

Bus          : 133 MHz

Core         : 6

Thread       : 12

Socket       : 1

Level1 Cache : 192 KBytes

Level2 Cache : 1536 KBytes

Level3 Cache : 12288 KBytes

Status       : Ok

Processor: 1

Name         : Intel Xeon

Stepping     : 2

Speed        : 2800 MHz

Bus          : 133 MHz

Core         : 6

Thread       : 12

Socket       : 2

Level1 Cache : 192 KBytes

Level2 Cache : 1536 KBytes

Level3 Cache : 12288 KBytes

Status       : Ok

Processor total  : 2

Memory installed : 131072 MBytes

ECC supported    : Yes

hpasmcli>

hpasmcli> show temp

Sensor   Location              Temp       Threshold

------   --------              ----       ---------

#1        AMBIENT              19C/66F    41C/105F

#2        CPU#1                40C/104F   82C/179F

#3        CPU#2                40C/104F   82C/179F

#4        MEMORY_BD            30C/86F    87C/188F

#5        MEMORY_BD            30C/86F    87C/188F

#6        MEMORY_BD            31C/87F    87C/188F

#7        MEMORY_BD            32C/89F    87C/188F

#8        SYSTEM_BD            35C/95F    90C/194F

#9        SYSTEM_BD            31C/87F    65C/149F

#10       SYSTEM_BD            39C/102F   90C/194F

#11       SYSTEM_BD            32C/89F    70C/158F

#12       SYSTEM_BD            40C/104F   90C/194F

#13       I/O_ZONE             27C/80F    70C/158F

#14       I/O_ZONE             33C/91F    70C/158F

#15       I/O_ZONE             34C/93F    70C/158F

#16       I/O_ZONE              -         70C/158F

#17       I/O_ZONE              -         70C/158F

#18       I/O_ZONE              -         70C/158F

#19       SYSTEM_BD            24C/75F    70C/158F

#20       SYSTEM_BD            26C/78F    70C/158F

#21       SYSTEM_BD            28C/82F    80C/176F

#22       SYSTEM_BD            28C/82F    80C/176F

#23       SYSTEM_BD            33C/91F    77C/170F

#24       SYSTEM_BD            32C/89F    70C/158F

#25       SYSTEM_BD            29C/84F    70C/158F

#26       SYSTEM_BD            30C/86F    70C/158F

#27       I/O_ZONE             35C/95F    70C/158F

#28       I/O_ZONE              -         70C/158F

#29       SCSI_BACKPLANE_ZONE  35C/95F    60C/140F

#30       SYSTEM_BD            60C/140F   110C/230F

hpasmcli>

hpasmcli> show tpm

Trusted-Platform-Module is absent.

hpasmcli>

hpasmcli> show uid

UID is currently off.

hpasmcli>

hpasmcli> show wol

Wake-On-Lan is currently enabled.

hpasmcli>