From: Kurakin, Andrey [mailto:andrey.kurakin@hp.com]

Sent: Friday, October 24, 2014 9:58 PM

To: Belogrud Sergey Vladimirovich

Subject: 4649742657

Сергей, добрый день,

Для анализа проблемы перезагрузки сервера DL980 G7 прошу Вас предоставить информацию:

- после чего произошло зависание и последующая перезагрузка сервера, например, проводились ли на сервере работы,  были ли недавно произведены обновления?

Также мне необходимы следующие логи:

1.  	- IML log из iLO сохранить в CSV формате

IML log

o   Log in to ILO3 and click “integrated Management Log”

o   Press CSV button (highlighted)

o   Highlight the Text and copy them into a text editor (notepad)

2.      - ILO log - сохранить в CSV формате

Use the same procedure used with IML Logs (View CSV and copy into a text editor)

3.       ELOGs

Capture the contents of the following url, where “serverILOAddress” is the ip address of the DL980 ILO:

https://serverILOAddress/elogext.txt

4.      BIOS settings: conrep

That can be done online with the conrep utility that is included in the HP Scripting Toolkit for Linux 9.30: http://goo.gl/AviMLd

(The utility is under .\hp-scripting-toolkit-linux-9.30\hp-scripting-toolkit-linux-9.30\utilities and must be executed with the –s option in order to save the output to a file:  “./conrep –s”)

Example:

[root@DL980G7K24U16 hp-scripting-tools]# pwd

/opt/hp/hp-scripting-tools

[root@DL980G7K24U16 hp-scripting-tools]# ./bin/conrep -s -x /opt/hp/hp-scripting-tools/etc/conrep.xml -f /tmp/DL980bios_23_05_2013.xml

5.      sosreport из ОС

6.  	ADU report

7.  	Screenshots from I-lo:

information>overview

information>diagnostics

Всю информацию можно сохранить на FTP

HTTPS Access   : https://ftp.usa.hp.com/hprc

Login      	: es742657

Password   	: RT2zh+AR

OR

FTP Access 	: ftp://es742657:RT2zh+AR@ftp.usa.hp.com

Также прошу проверить, что в BIOS –RBSU выставлены обязательные настройки для DL980 (ниже). Если такие настройки не устанавливались ранее, то их необходимо установить, в ином случае сервер ведет себя не стабильно, что может выражаться в зависаниях и перезагрузках:

RBSU settings:

Power Management Options - RECOMMENDATIONS

-> HP power profile

-> Custom

-> HP Power Regulator

-> HP Static High Performance Mode or OS Control Mode (for Windows)

-> Advanced Power Management Options

-> Minimum Processor Idle Power Package State

-> No Package

-> Minimum Processor Idle Power State

-> No C-states

-> Memory Power Capping

-> Disabled

-> Collaborative Power Control

-> Disabled

Regards,

Andrey Kurakin

Remote Support Account Advocate

HP ATP

Customer Solution Center

Global Support Delivery

Technology Services, Enterprise Group

17@hp.com

M +7 916 054 66 34

T  +7 495 797 35 00

ZAO “Hewlett-Packard A.O.”

Leningradskoe sh. 16A 3 bld.

123171 Moscow

Russia

From: Kurakin, Andrey [mailto:andrey.kurakin@hp.com]

Sent: Monday, October 27, 2014 12:31 PM

To: Terentev Aleksey; Belogrud Sergey Vladimirovich

Cc: Aleksandrov Aleksey Valerievich; Chernyh Aleksandr Vladimirovich; Leonov Aleksey Vladimirovich; Zbiranik Evgeniy Aleksandrovich

Subject: RE: 4649742657

Алексей, добрый день,

Мы проверили логи. Следующие детали могли привести к ASR:

·         Настройки BIOS выставлены не корректно, как упоминалось ранее. Для ОС RedHat должен стоять параметр “NO-C-States” во вкладке Intel_Minimum_Processor_Idle_Power_State. В настоящий момент стоит настройка C6_State.

В логах есть ошибки ядра и они могут быть связаны с настройками BIOS.

22:29:52,477 INFO kernel:[Hardware Error]: Machine check events logged

22:29:52,477 INFO kernel:[Hardware Error]: Machine check events logged

·         Кроме того известно, что к неожиданны перезагрузкам может приводить определенные версии прошивки iLO. Смотрите приложенный файл: Linux - Integrated Lights-Out 3 (iLO 3) Unresponsiveness and Subsequent Kernel Panics May Occur on HP ProLiant G7-Series Servers With iLO 3 Firmware Version 1.57 (or Later) When Running Linux and the hp-health.

Для дальнейшего расследования нам нужны следующие файлы:

1.      /var/log/messages

2.      /var/log/mcelog

3.      And output of hpdiags:

HP Diagnostics is obtained by running:

# /opt/hp/hpdiags/hpdiags -v 5 -o /tmp/hpdiag_<hostname>.txt -p

Regards,

Andrey Kurakin

Remote Support Account Advocate

HP ATP

Customer Solution Center

Global Support Delivery

Technology Services, Enterprise Group

17@hp.com

M +7 916 054 66 34

T  +7 495 797 35 00

ZAO “Hewlett-Packard A.O.”

Leningradskoe sh. 16A 3 bld.

123171 Moscow

Russia

20141027

[root@m1-db08-prod ~]# yum install hp-scripting-tools

Loaded plugins: product-id, security, subscription-manager

Updating certificate-based repositories.

Setting up Install Process

Resolving Dependencies

There are unfinished transactions remaining. You might consider running yum-complete-transaction first to finish them.

--> Running transaction check

---> Package hp-scripting-tools.x86_64 0:9.60-39.rhel6 will be updated

---> Package hp-scripting-tools.x86_64 0:10.01-1.rhel6 will be an update

--> Finished Dependency Resolution

Dependencies Resolved

==========================================================================================================================================================================

Package                                        Arch                               Version                                   Repository                              Size

==========================================================================================================================================================================

Updating:

hp-scripting-tools                             x86_64                             10.01-1.rhel6                             tcs-hp-stk                             3.9 M

Transaction Summary

==========================================================================================================================================================================

Upgrade       1 Package(s)

Total download size: 3.9 M

Is this ok [y/N]: y

Downloading Packages:

hp-scripting-tools-10.01-1.rhel6.x86_64.rpm                                                                                                        | 3.9 MB     00:00

Running rpm_check_debug

Running Transaction Test

Transaction Test Succeeded

Running Transaction

Warning: RPMDB altered outside of yum.

Updating   : hp-scripting-tools-10.01-1.rhel6.x86_64                                                                                                                1/2

Please read the Licence Agreement for this software at

/opt/hp/hp-scripting-tools/hp-scripting-tools.license

By not removing this package, you are accepting the terms

of the "HP Proliant Essentials Software End User License Agreement".

The hp-scripting-tools RPM has installed successfully.

Cleanup    : hp-scripting-tools-9.60-39.rhel6.x86_64                                                                                                                2/2

Installed products updated.

Updated:

hp-scripting-tools.x86_64 0:10.01-1.rhel6

Complete!

[root@m1-db08-prod ~]#

[root@m1-db08-prod ~]# /opt/hp/hpdiags/hpdiags -v 5 -p -o /tmp/20141027-1513.hpdiag.m1-db08-prod.txt

HP Insight Diagnostics Version 9.64.1262A (x86_64)

Copyright (c) 2002, 2013 Hewlett-Packard Development Company, L.P.

../../src/frontend/hpdiagstc/hpdiagstc.cpp(line 386), OpenDynamicLib failed with /opt/hp/hpdiags/libtcrib.so

../../src/frontend/hpdiagstc/hpdiagstc.cpp(line 386), OpenDynamicLib failed with /opt/hp/hpdiags/libtcvideo.so

sh: mono: command not found

sh: mono: command not found

sh: mono: command not found

sh: mono: command not found

sh: mono: command not found

sh: mono: command not found

Cannot open /etc/hba.conf

System configuration capture completed (100%)

[root@m1-db08-prod ~]#

[root@m1-db08-prod ~]# less /tmp/20141027-1513.hpdiag.m1-db08-prod.txt

#HP Insight Diagnostics Version 9.64.1262A (x86_64)

#Copyright (c) 2002, 2013 Hewlett-Packard Development Company, L.P.

Overview

System

Product Name: ProLiant DL980 G7

Serial Number: CZ3333LLNN

Processor Package 1: Intel(R) Xeon(R) CPU E7- 4870 @ 2.40GHz

Processor Package 2: Intel(R) Xeon(R) CPU E7- 4870 @ 2.40GHz

Processor Package 3: Intel(R) Xeon(R) CPU E7- 4870 @ 2.40GHz

Processor Package 4: Intel(R) Xeon(R) CPU E7- 4870 @ 2.40GHz

Processor Package 5: Intel(R) Xeon(R) CPU E7- 4870 @ 2.40GHz

Processor Package 6: Intel(R) Xeon(R) CPU E7- 4870 @ 2.40GHz

Processor Package 7: Intel(R) Xeon(R) CPU E7- 4870 @ 2.40GHz

Processor Package 8: Intel(R) Xeon(R) CPU E7- 4870 @ 2.40GHz

Operating system environment: Red Hat Enterprise Linux Server release 6.2 (Santiago)

Total memory - Test all available memory

Board 1

Card 1 Status: Online

DIMM 1A (DDR3): 16384 Mbytes

DIMM 2C: Not Installed

DIMM 3B (DDR3): 16384 Mbytes

DIMM 4D: Not Installed

DIMM 5D: Not Installed

DIMM 6B (DDR3): 16384 Mbytes

DIMM 7C: Not Installed

DIMM 8A (DDR3): 16384 Mbytes

Board 2

Card 2 Status: Online

DIMM 1A (DDR3): 16384 Mbytes

DIMM 2C: Not Installed

...

3.5"/2.88 MB floppy services support (int 13h): No

Print screen service support (int 5h): Yes

8042 keyboard services support: Yes

Serial services support: Yes

Printer services support: Yes

CGA/mono video services support: Yes

NEC PC-98: No

ACPI support: Yes

USB legacy support: Yes

AGP support: No

I2O boot support: No

LS-120 boot support: No

ATAPI Zip drive boot support: No

IEEE 1394 boot support: No

Smart battery support: No

BIOS boot specification support: Yes

Function key-initiated network boot support: Yes

Targeted content distribution support: Yes

Vendor: HP

BIOS Version: P66

BIOS Release Date: 07/29/2013

Internal speaker - Device

Port address: 61h

PCI Bus 0 - Device

PCI Bus 1 - Device

PCI Bus 2 - Device

PCI Bus 3 - Device

PCI Bus 4 - Device

PCI Bus 11 - Device

PCI Bus 17 - Device

[root@m1-db08-prod ~]#