https://wiki.tcsbank.ru/pages/viewpage.action?pageId=23177397

маркировка трафика в NFS-сетях

для корректной работы в смешанных сетях, где присутствует выделенный трафик NFS, надо включать маркировку трафика на хосте.

Для Linux-систем это настраивается через IPTABLES и выглядит так -

root@ds-db07-prod ~]# /etc/init.d/iptables status

Table: filter

Chain INPUT (policy ACCEPT)

num target prot opt source destination

Chain FORWARD (policy ACCEPT)

num target prot opt source destination

Chain OUTPUT (policy ACCEPT)

num target prot opt source destination

Table: mangle

Chain PREROUTING (policy ACCEPT)

num target prot opt source destination

Chain INPUT (policy ACCEPT)

num target prot opt source destination

Chain FORWARD (policy ACCEPT)

num target prot opt source destination

Chain OUTPUT (policy ACCEPT)

num target prot opt source destination

1 DSCP all -- 10.217.76.0/24 0.0.0.0/0 DSCP set 0x12

2 ACCEPT all -- 10.217.76.0/24 0.0.0.0/0

3 DSCP all -- 10.217.64.0/24 0.0.0.0/0 DSCP set 0x12

4 ACCEPT all -- 10.217.64.0/24 0.0.0.0/0

5 DSCP all -- 0.0.0.0/0 0.0.0.0/0 DSCP set 0x0c

Chain POSTROUTING (policy ACCEPT)

num target prot opt source destination

[root@ds-db07-prod ~]# more /etc/sysconfig/iptables

....

.....

:POSTROUTING ACCEPT [70002648:52273375754]

-A OUTPUT -s 10.217.76.0/24 -j DSCP --set-dscp 0x12

-A OUTPUT -s 10.217.76.0/24 -j ACCEPT

-A OUTPUT -s 10.217.64.0/24 -j DSCP --set-dscp 0x12

-A OUTPUT -s 10.217.64.0/24 -j ACCEPT

-A OUTPUT -j DSCP --set-dscp 0x0c

COMMIT

[root@ds-db07-prod ~]#

СЕТЕВАЯ КОНФИГУРАЦИ ХОСТА -

[tan@ds-db07-prod ~]$ ifconfig

bond0 Link encap:Ethernet HWaddr D4:C9:EF:75:9B:B0

inet addr:10.217.50.45 Bcast:10.217.50.255 Mask:255.255.255.0

inet6 addr: fe80::d6c9:efff:fe75:9bb0/64 Scope:Link

UP BROADCAST RUNNING MASTER MULTICAST MTU:1500 Metric:1

bond1 Link encap:Ethernet HWaddr D4:C9:EF:75:9B:B0

inet addr:10.217.76.210 Bcast:10.217.76.255 Mask:255.255.255.0

inet6 addr: fe80::d6c9:efff:fe75:9bb0/64 Scope:Link

UP BROADCAST RUNNING MASTER MULTICAST MTU:9000 Metric:1

bond2 Link encap:Ethernet HWaddr D4:C9:EF:75:9B:B0

inet addr:10.217.64.211 Bcast:10.217.64.255 Mask:255.255.255.0

inet6 addr: fe80::d6c9:efff:fe75:9bb0/64 Scope:Link

UP BROADCAST RUNNING MASTER MULTICAST MTU:9000 Metric:1