https://access.redhat.com/solutions/24474

Configuring a VLAN device over a bonded interface on Red Hat Enterprise Linux

ВОПРОС

Need to configure a VLAN device over a bonded interface

ОКРУЖЕНИЕ

Red Hat Enterprise Linux 5

Red Hat Enterprise Linux 6

РЕШЕНИЕ

Any necessary bonding devices must be configured before continuing with the steps below. Check the output of ifconfig to be sure. Please also note that BONDING_OPTS goes in ifcfg-bondX.

For example, bond0 with mode1.

# cat /etc/sysconfig/network-scripts/ifcfg-bond0

DEVICE=bond0

ONBOOT=yes

BOOTPROTO=none

USERCTL=no

BONDING_OPTS="mode=1 miimon=100"

Configure the VLAN interface in /etc/sysconfig/network-scripts. The configuration filename should be the bond interface (dot) VLAN ID number. So, if the VLAN ID is 2, and bond interface is bond0, then the configuration filename should be ifcfg-bond0.2:

# cat /etc/sysconfig/network-scripts/ifcfg-bond0.2

DEVICE=bond0.2

BOOTPROTO=none

IPADDR=192.168.1.1

NETMASK=255.255.255.0

USERCTL=no

VLAN=yes

ONBOOT=yes

Restart networking:

# service network restart

КОММЕНТАРИИ

Jeff Ewing's picture Jeff Ewing

Newbie

5 points

IPADDRE=192.168.1.1

This looks wrong to me

13.08.2010

ответить

Bryan Yount's picture Bryan Yount

Red HatGuru

4698 points

Hey Jeff. The article has been majorly overhauled and only pertains to configuring VLANs on top of existing bond devices. We now link to the much better "How to configure bonding in RHEL" article instead.

25.04.2013

ответить

Svenska Handelsbanken's picture Svenska Handelsbanken

Newbie

5 points

21.02.2011 - Edited

ответить

Sergey Guzenkov's picture Sergey Guzenkov

GuruPro

2378 points

Good document.

Last example would be even better if you changed the value of the ip.forwarding rather than rewrite '1' with '1'.

Item 6. modprobe.conf is a deprecated way of setting bonding parameters. According to the deployment guide bonding options should go to the BONDING_OPTS section of the ifcfg-<interfaceName> file.

https://access.redhat.com/knowledge/docs/en-US/Red_Hat_Enterprise_Linux/5/html/Deployment_Guide/s2-networkscripts-interfaces-chan.html

12.10.2012

ответить

Sorin Serban's picture Sorin Serban

Red HatPro

432 points

Hi Sergey,

last example: i see your point, it would be more clear, as it's meant to demonstrate which value is being changed using the "/" separator.

modprobe.conf: you are correct, we no longer recommend that in rhel5 and 6, it might have been correct for very early versions of rhel5 (need to double check that), but i agree this needs to be updated to avoid confusion.

thanks a lot for your feedback!

--

sorin

15.10.2012

ответить

Sergey Guzenkov's picture Sergey Guzenkov

GuruPro

2378 points

Item 5. There is no such an option as BOOTPROTO=static. Please check the /usr/share/doc/initscripts-8.45.33/sysconfig.txt

BOOTPROTO=none|bootp|dhcp

Item 4. There is no such option as TYPE=Ethernet.

Please check the /usr/share/doc/initscripts-8.45.33/sysconfig.txt

Item 7. Unlike provided code output that has <physical interface>.<vlan> in the /proc/net/vlan, on my machines RHEL5.6 I see <bonding interface>.<vlan> devices. Can you please explain the difference with your output?

# ls -l /proc/net/vlan/

total 0

-rw------- 1 root root 0 Oct 12 20:26 bond0.142

-rw------- 1 root root 0 Oct 12 20:26 bond1.279

-rw------- 1 root root 0 Oct 12 20:26 config

# cat /proc/sys/net/ipv4/conf/

all/ bond0.142/ bond1.279/ default/ lo/

To summarize what configuration files should have what options:

ifcfg-eth0 should have DEVICE,ONBOOT,BOOTPROTO,MASTER,SLAVE parameters defined.

ifcfg-bond0 should have DEVICE,ONBOOT,BOOTPROTO,BONDING_OPTS parameters defined.

ifcfg-bond0.<vlan> should have DEVICE,ONBOOT,BOOTPROTO,VLAN parameters defined.

Could you please acknowledge suggested corrections or/and correct me where I am wrong?

12.10.2012

ответить

Sorin Serban's picture Sorin Serban

Red HatPro

432 points

Hi, Sergey,

BOOTPROTO: you are correct (as we discussed). basically "static" is equivalent to "none" or "we_dont_need_dhcp_here", or whatever you like. In fact, other values except bootp/dhcp are not being actively acted upon (though they might be in the future). Agreed, it can be a bit confusing, and only the documented supported option should be recommended to avoid confusion (documenting "static" as an alternative to "none" might be considered, as it's become quite popular in use).

TYPE: you are correct, and it's similar to the above: the "useful" values are "Bridge" and "Tap" (for bridge and tunnel interfaces, respectively), using "Ethernet" only serves to help with clarity in the configuration, and make sure there's no "residual" value left (resetting the variable). It is used a lot, and i think we might consider documenting this as a "dummy" value.

"physical interface": you are correct, this is not right, and it only makes sense for independent, non-bonded interface. the way you showed from your machine is the correct way, the vlan interface must be attached to the bond interface, not the physical interface.

I agree with you summary, and plan to use it when correcting this, if you're okay with that.

Thank you again for your excellent feedback

I will double check all the above and correct where needed.

--

sorin

15.10.2012

ответить

Sorin Serban's picture Sorin Serban

Red HatPro

432 points

Just to clarify: the eth0.10 part is only a note which applies to vlan interfaces in general, but it is clear it can cause confusion, even if it's just a "note:"

15.10.2012

ответить

Sergey Guzenkov's picture Sergey Guzenkov

GuruPro

2378 points

some observations:

for all network configuration files specifying DEVICE is mandatory. If you don't specify DEVICE in a configuration file, your configuration file is not regarded as configuration and not processed.

Your file name can be whatever you want, and your DEVICE name can be whatever you want. There is no relationship between the two (unless its a bonding interface, which is confusing :))

For example you can rename your ifcfg-eth0 to ifcfg-johny with 'DEVICE=xp5'. And in the ifconfig output you will see xp5 device.

SLAVE=YES will not work! 'yes' must be typed in low characters.

In my opinion the SLAVE parameter is unnecessary, redundant. MASTER is necessary and already implies that the interface is slave. But at the moment if you don't specify both, your bonding will not work.

05.11.2012

ответить

RedHatAdmin Parsons's picture RedHatAdmin Parsons

Active Contributor

129 points

Why not one article? is that too much to ask? I appreciate the information, but it's spread across multiple posts in the knowledgebase. Some of us are stuck with older RHEL releases for a while due to vendor application requirements, and as such will be referencing this information from some time to come.

One thing not made clear via this article or the main bonding article pertains to the IP address assignment when dealing with VLAN.

If you created the bond0.x VLAN bond and assign the static IP address, I assume therefore you should remove if from bond0. So, in an example where you have two nics, you will end up with four config files, correct?

ifcfg-eth0

ifcfg-eth1

ifcfg-bond0

ifcfg-bond0.x (where x is VLAN id, and static IP address appears only in this config file)

Please advise.

25.03.2014

ответить

Sorin Serban's picture Sorin Serban

Red HatPro

432 points

Hi,

why not one article: the decision was made, some time ago, to keep the bonding configuration part in one document, this helps keep this short and to the point, and makes sure that any corrections needed would not have to be maintained in multiple places; sorry if this was very inconvenient for you

regarding your configuration: you are correct, but keep in mind there is nothing, technically speaking, to prevent you from having an ip address (normally in a separate subnet) on bond0 itself too. The corresponding traffic would be untagged though, of course.

please, if it's still not clear or have further questions, ideally open a ticket (via access.redhat.com, or by phone), so we can assist you.

Thanks again for your feedback, we will review your concerns and see if we can change the article according that.

Regards,

--

sorin

25.03.2014

https://access.redhat.com/solutions/483803

Whether configuring bond on top of VLAN as slave interfaces is a valid configuration ?

ВОПРОС

Can bond on top of vlan interfaces be configured ?

Why is bond on top of VLAN tagged slaves, not a supported configuration?

Below is the configuration in which the vlan interfaces are configured on slave interfaces of bond.

[root@host]# cat /etc/sysconfig/network-scripts/ifcfg-bond0

DEVICE=bond0

IPADDR=X.X.X.X

NETWORK=X.X.X.X

BONDING_OPTS="mode=<any-mode> miimon=100"

[...]

[root@host]# cat /etc/sysconfig/network-scripts/ifcfg-eth4

# QLogic Corp. cLOM8214 1/10GbE Controller

DEVICE=eth4

BOOTPROTO=none

ONBOOT=no

USERCTL=no

TYPE=Ethernet

[root@host]# cat /etc/sysconfig/network-scripts/ifcfg-eth4.28

# QLogic Corp. cLOM8214 1/10GbE Controller

DEVICE=eth4.28

BOOTPROTO=none

ONBOOT=yes

MASTER=bond0

SLAVE=yes

VLAN=yes

[root@host]# cat /etc/sysconfig/network-scripts/ifcfg-eth5

# QLogic Corp. cLOM8214 1/10GbE Controller

DEVICE=eth5

BOOTPROTO=none

ONBOOT=no

USERCTL=no

TYPE=Ethernet

[root@host]# cat /etc/sysconfig/network-scripts/ifcfg-eth5.28

# QLogic Corp. cLOM8214 1/10GbE Controller

DEVICE=eth5.28

BOOTPROTO=none

ONBOOT=yes

MASTER=bond0

SLAVE=yes

Is above configuration supported by Red Hat?

ОКРУЖЕНИЕ

Red Hat Enterprise Linux (Any version)

РЕШЕНИЕ

That type of configuration is not the supported configuration, technically it may work but is not the supported configuration from Red Hat side.

The supported configuration is to create VLAN device over bonded interface as mentioned in following article: Configuring a VLAN device over a bonded interface on RHEL

ЭТАПЫ ДИАГНОСТИКИ

Check the current bonding and VLAN configuration done in the system.

https://access.redhat.com/support/cases/#/case/01309570

Описание

TCS AdministratorЗдравствуйте!

Мы столкнулись с проблемой, когда после перезагрузки системы не работают агрегированные интерфейсы. Сами они, и их "slave" интерфейсы, поднимаются корректно, линки тоже есть,

но пинги до "соседей" не идут пока не выполнить следующую последовательность команд: "/sbin/ifconfig bondX down && /sbin/ifup bondX". Причем, при наличии в системе трех подобных интерфейсов (bond0, bond1 и bond2),

некорректно работает только "bond1" и "bond2". С "bond0" таких проблем нет.

Подробнее о интерфейсах. Это агрегация типа "mode=1" (active-backup) из двух "slave" интерфейсов в VLAN:

$ ls /proc/net/bonding/

bond0  bond1  bond2

$ cat /proc/net/bonding/bond1

Ethernet Channel Bonding Driver: v3.6.0 (September 26, 2009)

Bonding Mode: fault-tolerance (active-backup)

Primary Slave: None

Currently Active Slave: eth0.1776

MII Status: up

MII Polling Interval (ms): 100

Up Delay (ms): 0

Down Delay (ms): 0

Slave Interface: eth0.1776

MII Status: up

Speed: 10000 Mbps

Duplex: full

Link Failure Count: 0

Permanent HW addr: 2c:27:d7:53:bd:00

Slave queue ID: 0

Slave Interface: eth2.1776

MII Status: up

Speed: 10000 Mbps

Duplex: full

Link Failure Count: 0

Permanent HW addr: 2c:27:d7:54:0b:b8

Slave queue ID: 0

$ cat /etc/modprobe.conf

alias bond0 bonding

options bond0 mode=1 miimon=100 primary=eth0.1750

alias bond1 bonding

options bond1 mode=1 miimon=100 primary=eth2.1776

alias bond2 bonding

Помогите, пожалуйста, разобраться с проблемой!

Спасибо!

Алексей.

Pathan, Mehmoodkha

Dec 4, 2014

11:47:20 AM +0300

Hello,

Thank you for the update. I'm closing this service request as per your update.

Please feel free to get back to us in case of any further queries.

After this ticket has been closed, you may receive an email request to fill out a survey on your experience with this service request.

I encourage you to fill this survey out, as your feedback is very important to Red Hat and it helps us to determine what we are doing correctly as well as highlight what we can do to assist you better.

Thanks again for choosing Red Hat Support.

Thanks and regards,

Mehmoodkha Pathan

Reply

Administrator, TCS

Dec 4, 2014

10:57:27 AM +0300

Спасибо за ответ!

Вопросов больше по этой теме у нас нет. Заявку мохжно закрывать.

Алексей.

Reply

Pathan, Mehmoodkha

Dec 2, 2014

8:14:37 PM +0300

Greetings,

Thank you for contacting Red Hat Technical Support.

My name is Mehmoodkha Pathan and I have taken the ownership of this case.

From the given configuration I observe that you configured bond on top of vlan interface :

-------------

$ cat /proc/net/bonding/bond1

Ethernet Channel Bonding Driver: v3.6.0 (September 26, 2009)

Bonding Mode: fault-tolerance (active-backup)

Primary Slave: None

Currently Active Slave: eth0.1776

MII Status: up

MII Polling Interval (ms): 100

Up Delay (ms): 0

Down Delay (ms): 0

Slave Interface: eth0.1776   <<<=====

MII Status: up

Speed: 10000 Mbps

Duplex: full

Link Failure Count: 0

Permanent HW addr: 2c:27:d7:53:bd:00

Slave queue ID: 0

Slave Interface: eth2.1776   <<<=====

MII Status: up

Speed: 10000 Mbps

Duplex: full

Link Failure Count: 0

Permanent HW addr: 2c:27:d7:54:0b:b8

Slave queue ID: 0

-------------

This type of configuration is not the supported configuration, technically it may work but is not the supported configuration from Red Hat side.

Refer following kbase article to know more details :

-----------------------------------------------------

https://access.redhat.com/solutions/483803

-----------------------------------------------------

Hope above helps.

Thanks and Regards,

Mehmoodkha Pathan