Oct 24, 2021

*. 07:00 Ночные работы по задаче "обновить прошивку на сервере" (https://jira.tcsbank.ru/browse/IN-1674) [Третьяков Артём].

Возникли проблемы при загрузке ОС. В опциях старта ОС в grub2 было указано смонтировать не существующий swap раздел. При попытке смонтировать его загрузка обрывалась с попаданием в консоль dracut. Смонтировал загрузочный раздел /boot в /mnt/boot, убрал опцию монтирования swap. Сервер загрузился.

Перед этим проверил на ошибки раздел /root и создал swap-файл в нём.

I updated firmware as I did it before (here are my detail notes 20200415 Mellanox network card spam messages on server console.docx).

Then I sent server to reboot. And during boot process I seen follow messages.

There were lot of similar messages. And at the end I received dracut prompt.

#	I found this article with advices how to help myself under dracut environment during boot problems.

#	https://fedoraproject.org/wiki/How_to_debug_Dracut_problems

#	I decided to mount /root partition, that was under LVM.

lvm vgscan

lvm vgchange -ay

blkid

#	But problem was not linked with root partition.

#	I need only mount /boot partition and remove this parameter "rd.lvm.lv=centos_ds-pr-paym-es-metal-01/swap" from boot section in /boot/grub2/grub.cfg file.

#	To did that I found previously saved information from /etc/fstab - UUID of boot partition.

#	And then I saw in blkid output which device has that UUID. And mount it.

mkdir -p /mnt/boot

blkid

mount /dev/sda1 /mnt/boot

vi /mnt/boot/grub2/grub.cfg

#	Then I saved my edition of grub.cfg file. Unmouted /boot partition and rebooted server.

#	Thise command was executed after server successfully booted.

#

root@ds-pr-paym-es-metal-01 ~$ sed '87,107!d' /boot/grub2/grub.cfg

### BEGIN /etc/grub.d/10_linux ###

menuentry 'CentOS Linux (3.10.0-957.27.2.el7.x86_64) 7 (Core)' --class centos --class gnu-linux --class gnu --class os --unrestricted $menuentry_id_option 'gnulinux-3.10.0-957.el7.x86_64-advanced-bc524cad-958d-47a0-9bf8-d16fd0fa4482' {

load_video

set gfxpayload=keep

insmod gzio

insmod part_msdos

insmod xfs

set root='hd0,msdos1'

if [ x$feature_platform_search_hint = xy ]; then

search --no-floppy --fs-uuid --set=root --hint-bios=hd0,msdos1 --hint-efi=hd0,msdos1 --hint-baremetal=ahci0,msdos1 --hint='hd0,msdos1'  ffc36d80-dfcd-4aa5-9793-8b8dd88a628a

else

search --no-floppy --fs-uuid --set=root ffc36d80-dfcd-4aa5-9793-8b8dd88a628a

fi

linux16 /vmlinuz-3.10.0-957.27.2.el7.x86_64 root=/dev/mapper/centos_ds--pr--paym--es--metal--01-root ro crashkernel=auto rd.lvm.lv=centos_ds-pr-paym-es-metal-01/root rhgb quiet LANG=en_US.UTF-8

initrd16 /initramfs-3.10.0-957.27.2.el7.x86_64.img

}

#

#       linux16 /vmlinuz-3.10.0-957.27.2.el7.x86_64 root=/dev/mapper/centos_ds--pr--paym--es--metal--01-root ro crashkernel=auto rd.lvm.lv=centos_ds-pr-paym-es-metal-01/root rd.lvm.lv=centos_ds-pr-paym-es-metal-01/swap rhgb quiet LANG=en_US.UTF-8

#

root@ds-pr-paym-es-metal-01 ~$

root@ds-pr-paym-es-metal-01 ~$ parted /dev/sda -s p

Model: HPE LOGICAL VOLUME (scsi)

Disk /dev/sda: 300GB

Sector size (logical/physical): 512B/512B

Partition Table: msdos

Disk Flags:

Number  Start   End     Size    Type     File system  Flags

1      1049kB  1075MB  1074MB  primary  xfs          boot

2      1075MB  300GB   299GB   primary               lvm

root@ds-pr-paym-es-metal-01 ~$ pvs

/dev/sdc: open failed: No medium found

PV         VG                            Fmt  Attr PSize   PFree

/dev/sda2  centos_ds-pr-paym-es-metal-01 lvm2 a--  278.36g 213.36g

/dev/sdb   paym_vg                       lvm2 a--   <5.24t   1.64t

root@ds-pr-paym-es-metal-01 ~$ vgs

VG                            #PV #LV #SN Attr   VSize   VFree

centos_ds-pr-paym-es-metal-01   1   2   0 wz--n- 278.36g 213.36g

paym_vg                         1   1   0 wz--n-  <5.24t   1.64t

root@ds-pr-paym-es-metal-01 ~$ lvs

LV           VG                            Attr       LSize  Pool Origin Data%  Meta%  Move Log Cpy%Sync Convert

lv_home      centos_ds-pr-paym-es-metal-01 -wi-ao---- 15.00g                                                                                             root         centos_ds-pr-paym-es-metal-01 -wi-ao---- 50.00g

paym_es_h_lv paym_vg                       -wi-ao---- <3.60t

root@ds-pr-paym-es-metal-01 ~$

#	This server has no swap.

root@ds-pr-paym-es-metal-01 ~$ swapon

root@ds-pr-paym-es-metal-01 ~$ lsblk

NAME                                           MAJ:MIN RM   SIZE RO TYPE MOUNTPOINT

sda                                              8:0    0 279.4G  0 disk

├─sda1                                           8:1    0     1G  0 part /boot

└─sda2                                           8:2    0 278.4G  0 part

├─centos_ds--pr--paym--es--metal--01-root    253:0    0    50G  0 lvm  /

└─centos_ds--pr--paym--es--metal--01-lv_home 253:1    0    15G  0 lvm  /home

sdb                                              8:16   0   5.2T  0 disk

└─paym_vg-paym_es_h_lv                         253:2    0   3.6T  0 lvm  /mnt/hot_01

root@ds-pr-paym-es-metal-01 ~$

#

#	Before I understood how to fix difficulties, I did follow things.

#	I mounted /root partition, created swap-file, then unmount /root partition and run xfs_repair for it.

#	I used follow commnads, but they were useless because there were not /dev/... devices under chroot environment.

mkdir -p /mnt/root

mount /dev/mapper/centos_ds-pr-paym-es-metal-01-root /mnt/root

chroot /mnt/root

#	I found a way how to create swap file, but it was not helped me, so it was deleted by me after the server booted correctly.

/mnt/root/sbin/dd if=/dev/zero of=/mnt/root/swapfile bs=1M count=2048

#	Next my idea was to change mount option in /etc/fstab, where set option to run filesystemc check after lot time it used.

#	But in /etc/fstab was "0 0" options at the end of all lines.

...

exit

umount /mnt/root

#	There was not found errors on /root partition.

#	I'm not sure - was it in /bin or it was not, but xfs utilities exist under dracut - that's the fact.

/bin/xfs_repair /dev/mapper/centos_ds-pr-paym-es-metal-01-root

20140828 #354104 Hard drive failure on ds-gp-prod-sdw6.docx - here I found my notes about xfs_repair utility.

mkswap,swapon,swapoff - here I wrote how create swapfile.