HP Insight Diagnostics Version 9.50.1009A (x86_64)

Computer Name: ds-afs-prod

### Diagnosis Log

| Passed | - Denotes passed tests |
| --- | --- |
| Warning | - Denotes tests that passed with warnings |
| Failed | - Denotes failed tests |

| Device, Test | Number of Times Diagnosed | Failed Count | Passed with warnings Count | Diagnosis Time |  |
| --- | --- | --- | --- | --- | --- |
| Logical Drive 1, Storage Controller in Slot 0 | 1 | 0 | 0 | 00:00:01. | Passed. |
| Logical Drive 2, Storage Controller in Slot 0 | 1 | 0 | 0 | 00:00:01. | Passed. |
| Power Supply 1 | 1 | 0 | 0 | 0:00:09. | Passed. |
| Power Supply 2 | 1 | 0 | 0 | 0:00:01. | Passed. |

### Test Log

Test Log is empty

### Error Log

Error Log is empty

### Integrated Management Log

| Severity | Class | Last Update | Initial Update | Count | Description |
| --- | --- | --- | --- | --- | --- |
| Informational | Maintenance | 03/02/2013 09:54 | 03/02/2013 09:54 | 1 | Maintenance note: IML cleared through hpasmcli |
| Caution | Power | [NOT SET] | [NOT SET] | 1 | System Power Supply: Input Power Loss or Unplugged Power Cord, Verify Power Supply Input (Power Supply 2) |
| Caution | Power | [NOT SET] | [NOT SET] | 1 | System Power Supplies Not Redundant |
| Caution | Power | 12/14/2012 00:09 | 12/14/2012 00:06 | 2 | System Power Supply: Input Power Loss or Unplugged Power Cord, Verify Power Supply Input (Power Supply 2) |
| Caution | Power | 12/14/2012 00:09 | 12/14/2012 00:06 | 2 | System Power Supplies Not Redundant |
| Informational | System Revision | 12/20/2012 13:59 | 12/20/2012 13:59 | 1 | Firmware flashed (iLO 4 1.20) |
| Informational | Maintenance | 12/20/2012 14:02 | 12/20/2012 14:02 | 1 | Maintenance note: Updated Power Management Controller firmware to version 3.1 |
| Caution | Main Memory | 04/27/2013 07:33 | 04/27/2013 07:33 | 1 | Corrected Memory Error threshold exceeded ((Processor 2, Memory Module 2)) |
| Critical | Drive Array | 05/01/2013 10:49 | 05/01/2013 10:49 | 1 | Drive Array Controller Failure (Slot 0) |
| Caution | Main Memory | 05/07/2013 08:52 | 05/07/2013 08:52 | 1 | Corrected Memory Error threshold exceeded ((Processor 2, Memory Module 2)) |
| Caution | Main Memory | 05/15/2013 15:14 | 05/15/2013 15:14 | 1 | Corrected Memory Error threshold exceeded ((Processor 2, Memory Module 2)) |
| Caution | Main Memory | 05/21/2013 14:32 | 05/21/2013 14:32 | 1 | Corrected Memory Error threshold exceeded ((Processor 2, Memory Module 2)) |
| Repaired | Power | 05/28/2013 15:10 | 05/28/2013 15:10 | 1 | System Power Supply: Input Power Loss or Unplugged Power Cord, Verify Power Supply Input (Power Supply 1) |
| Repaired | Power | 05/28/2013 15:10 | 05/28/2013 15:10 | 1 | System Power Supplies Not Redundant |
| Critical | Main Memory | 08/06/2013 12:04 | 08/06/2013 12:04 | 1 | Uncorrectable Memory Error ((Processor 2, Memory Module 11)) |
| Critical | Main Memory | 08/06/2013 11:04 | 08/06/2013 11:04 | 1 | Uncorrectable Memory Error detected by ROM-based memory validation (Board 2, Memory Module 2) |
| Critical | Main Memory | 08/06/2013 13:39 | 08/06/2013 13:39 | 1 | Uncorrectable Memory Error ((Processor 2, Memory Module 3)) |
| Critical | Main Memory | 08/06/2013 12:39 | 08/06/2013 12:39 | 4 | Uncorrectable Memory Error detected by ROM-based memory validation (Board 2, Memory Module 2) |
| Critical | Main Memory | 09/13/2013 11:56 | 09/13/2013 11:56 | 2 | Uncorrectable Memory Error detected by ROM-based memory validation (Board 2, Memory Module 2) |
| Critical | Main Memory | 09/13/2013 13:01 | 09/13/2013 13:01 | 1 | Uncorrectable Memory Error ((Processor 2, Memory Module 4)) |
| Critical | Main Memory | 09/13/2013 12:01 | 09/13/2013 12:01 | 1 | Uncorrectable Memory Error detected by ROM-based memory validation (Board 2, Memory Module 2) |

### System Event Log

System Event Log is empty