From: Gus Konstantin Mikhaylovich

Sent: Wednesday, September 18, 2013 3:52 PM

To: Terentev Aleksey

Cc: Varakuta Sergey; Aleksandrov Aleksey Valerievich; Belogrud Sergey Vladimirovich; greenplum_admins

Subject: RE: Техническое задание на инсталляцию промышленного Greenplum в DS.

Проверьте , может я где неправ ))

From: Gus Konstantin Mikhaylovich

Sent: Wednesday, September 18, 2013 3:49 PM

To: Terentev Aleksey

Cc: Varakuta Sergey; Aleksandrov Aleksey Valerievich; Belogrud Sergey Vladimirovich; greenplum_admins

Subject: RE: Техническое задание на инсталляцию промышленного Greenplum в DS.

Дополнение :

На всех серверах выставить :

·         Максимальную производительность  процов в профиле

·         Максимальную частоту шины фиксированно

·         Write-back кеширование для всех контроллеров

·         Asr –  10 минут

·         Включить принудительное охлаждение

·         Отключить опцию выключения при перегреве

·         Если есть возможность зафиксировать частоту памяти на максимум .

·         (подумать что еще включить можно , чтобы работало побыстрее , может что еще можно подкрутить )

Запланировать проверку и установку опций BIOS  на всех серверах  тестового и продового контуров.

Проверить нет ли где установленных опций в профилях питания оборудования ( в OS и оборудовании , которые бы не были выставлены в приоритете производительности )

From: Gus Konstantin Mikhaylovich

Sent: Wednesday, September 18, 2013 3:41 PM

To: Terentev Aleksey

Cc: Varakuta Sergey; Aleksandrov Aleksey Valerievich; Belogrud Sergey Vladimirovich

Subject: Техническое задание на инсталляцию промышленного Greenplum в DS.

Техническое задание на инсталляция  промышленного Greenplum в DS.

Инсталляция мастер-серверов (mdw, smdw) .

Для мастер серверов  необходимо создать  следующие  разделы :

Для  единственного контроллера   следует создать одну  дисковую группу  raid 5 куда включить диски , с резервированием одного диска для hotspare.

Cоздать след. разделы :

Файловая система  	Разм  Исп  Дост  Исп% смонтирована на

/dev/sda4              60Gb  /root

/dev/sda1            1Gb  /boot

/dev/sda2             1,3T  /data  (xfs)

/dev/sda3             300G   /work (xfs)

/dev/sdw4           130 gb (swap)

Для серверов установить настройки ядра :

(Параметры для всех серверов одинаковы)

# ------------------------------ GreeenPlum Area ------------------------------

kernel.sem = 250 512000 100 2048

kernel.sysrq = 1

kernel.core_uses_pid = 1

kernel.msgmnb = 65536

kernel.msgmax = 65536

kernel.msgmni = 2048

net.ipv4.tcp_syncookies = 1

net.ipv4.ip_forward = 0

net.ipv4.conf.default.accept_source_route = 0

net.ipv4.tcp_tw_recycle = 1

net.ipv4.tcp_max_syn_backlog = 4096

net.ipv4.conf.all.arp_filter = 1

net.ipv4.ip_local_port_range = 1025 65535

net.core.netdev_max_backlog = 10000

vm.overcommit_memory = 2

kernel.shmmax = 500000000

kernel.shmall = 4000000000

kernel.shmmni = 4096

vm.dirty_ratio = 50

vm.dirty_background_ratio = 50

vm.dirty_writeback_centisecs = 3000

vm.dirty_expire_centisecs = 5000

vm.swappiness = 10

vm.vfs_cache_pressure = 100

# ------------------------------ GreeenPlum Area ------------------------------

И настройки лимитов

# ------------------------------ GreeenPlum Area ------------------------------

gpadmin         soft    nofile      	262544

gpadmin         hard	nofile          262544

gpadmin         soft    nproc       	196608

gpadmin         hard    nproc       	196608

*               soft    nofile      	65536

*               hard    nofile      	65536

*               soft    nproc       	131072

*               hard    nproc       	131072

# ------------------------------ GreeenPlum Area ------------------------------

# End of file

Так же перенастроить профили оборудования (вставить в автозапуск при загрузке )

modprobe speedstep-lib

modprobe pcc-cpufreq

modprobe acpi-cpufreq

modprobe cpufreq_stats

modprobe freq_table

modprobe cpufreq_ondemand

modprobe cpufreq_conservative

modprobe cpufreq_powersave

echo never > /sys/kernel/mm/redhat_transparent_hugepage/enabled

echo never > /sys/kernel/mm/redhat_transparent_hugepage/defrag

echo deadline > /sys/block/sda/queue/scheduler

echo deadline > /sys/block/sdb/queue/scheduler

/sbin/blockdev --setra 16385 /dev/sdb

/sbin/blockdev --setra 16385 /dev/sdb

echo ondemand > /sys/devices/system/cpu/cpu0/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu1/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu2/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu3/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu4/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu5/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu6/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu7/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu8/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu9/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu10/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu11/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu12/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu13/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu14/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu15/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu16/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu17/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu18/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu19/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu20/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu21/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu21/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu22/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu23/cpufreq/scaling_governor

echo 72000 > /proc/sys/fs/xfs/xfssyncd_centisecs

echo 2000 > /proc/sys/fs/xfs/xfsbufd_centisecs

echo 72000 > /proc/sys/fs/xfs/age_buffer_centisecs

(да добавить в rc.local)

IP Адреса и развязку сети оставить прежнюю.

Инсталляция  сегмент-серверов (mdw, smdw) .

Для двух контроллеров создать  для каждого дисковую группу raid5.

Cоздать след. разделы :

Для первой дисковой группы первого контроллера :

Файловая система  	Разм  Исп  Дост  Исп% смонтирована на

/dev/sda4              60Gb /  ( да для / )

/dev/sda1            1Gb  /boot

/dev/sda2             3.6T  /data1  (xfs)

/dev/sda3             530G   /work1 (xfs)

/dev/sda4            130 gb  (swap)

Для первой дисковой группы второго  контроллера :

Файловая система  	Разм  Исп  Дост  Исп% смонтирована на

/dev/sda2             3.6T  /data2  (xfs)

/dev/sda3             530G   /work2 (xfs)

Для серверов установить настройки ядра :

# ------------------------------ GreeenPlum Area ------------------------------

kernel.sem = 250 512000 100 2048

kernel.sysrq = 1

kernel.core_uses_pid = 1

kernel.msgmnb = 65536

kernel.msgmax = 65536

kernel.msgmni = 2048

net.ipv4.tcp_syncookies = 1

net.ipv4.ip_forward = 0

net.ipv4.conf.default.accept_source_route = 0

net.ipv4.tcp_tw_recycle = 1

net.ipv4.tcp_max_syn_backlog = 4096

net.ipv4.conf.all.arp_filter = 1

net.ipv4.ip_local_port_range = 1025 65535

net.core.netdev_max_backlog = 10000

vm.overcommit_memory = 2

kernel.shmmax = 500000000

kernel.shmall = 4000000000

kernel.shmmni = 4096

vm.dirty_ratio = 50

vm.dirty_background_ratio = 50

vm.dirty_writeback_centisecs = 3000

vm.dirty_expire_centisecs = 5000

vm.swappiness = 10

vm.vfs_cache_pressure = 100

# ------------------------------ GreeenPlum Area ------------------------------

И настройки лимитов

# ------------------------------ GreeenPlum Area ------------------------------

gpadmin         soft    nofile      	262544

gpadmin         hard    nofile      	262544

gpadmin         soft    nproc       	196608

gpadmin         hard    nproc       	196608

*               soft    nofile      	65536

*               hard    nofile      	65536

*               soft	nproc           131072

*               hard    nproc       	131072

# ------------------------------ GreeenPlum Area ------------------------------

# End of file

Так же перенастроить профили оборудования (вставить в автозапуск при загрузке )

modprobe speedstep-lib

modprobe pcc-cpufreq

modprobe acpi-cpufreq

modprobe cpufreq_stats

modprobe freq_table

modprobe cpufreq_ondemand

modprobe cpufreq_conservative

modprobe cpufreq_powersave

echo never > /sys/kernel/mm/redhat_transparent_hugepage/enabled

echo never > /sys/kernel/mm/redhat_transparent_hugepage/defrag

echo deadline > /sys/block/sda/queue/scheduler

echo deadline > /sys/block/sdb/queue/scheduler

/sbin/blockdev --setra 16385 /dev/sdb

/sbin/blockdev --setra 16385 /dev/sdb

echo ondemand > /sys/devices/system/cpu/cpu0/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu1/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu2/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu3/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu4/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu5/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu6/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu7/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu8/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu9/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu10/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu11/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu12/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu13/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu14/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu15/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu16/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu17/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu18/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu19/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu20/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu21/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu21/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu22/cpufreq/scaling_governor

echo ondemand > /sys/devices/system/cpu/cpu23/cpufreq/scaling_governor

echo 72000 > /proc/sys/fs/xfs/xfssyncd_centisecs

echo 2000 > /proc/sys/fs/xfs/xfsbufd_centisecs

echo 72000 > /proc/sys/fs/xfs/age_buffer_centisecs

(да добавить в rc.local

IP Адреса и развязку сети оставить прежнюю.

На всех узлах создать папки /app  с правами пользователю 777 gpadmin:gpadmin

Дать права /work* /data*  777 для gpadmin:gpadmin

( да именно таких комманд )

Konstantin M. Gus

GreenPlum Administrator

Skype - neshwill

39A, bld. 1  Leningradskoe Shosse

Moscow, Russia, 125212

Tel:  +7 (495) 648 1000

Fax: +7 (495) 645 5909

E-mail: k.gus@tcsbank.ru

http://www.tcsbank.ru

This e-mail (including attachments) is confidential and is intended for the addressee(s) only. Unless you are the addressee, you may not read, copy, use or store this e-mail in any way, and delete immediately.  If you have received it in error, please contact ‘Tinkoff Credit Systems’ (Bank ‘TCS’ ZAO) on +7 495 648 1000.

Это электронное письмо строго конфиденциально (включая все вложения). Данная информация предназначена только для получателя(ей). Если Вы не являетесь получателем, Вы не вправе каким бы то ни было образом читать, копировать, использовать или хранить данное сообщение. Просьба немедленно его удалить. Если Вы получили его по ошибке, пожалуйста свяжитесь с нашей компанией «Тинькофф Кредитные Системы» Банк (закрытое акционерное общество) по т. +7 495 648 1000.